National Instruments NI-488.2 Routines and NI-488 Functions for IBM CSet,

Borland C and Microsoft C Language Interface.

NI-488.2M Software for MC-GPIB and OS/2.

Version 2.2.

Copyright 1994 National Instruments Corporation.

All Rights Reserved.

After installing the NI-488.2M software, your C directory should

contain the following files.

README.DOC   -  Information file.

NI488.DLL    -  The dynamic link library for the 32-bit IBM CSet

language interface.

NIBOR.DLL    -  The dynamic link library for the 32-bit Borland C

language interface.

NI488_16.DLL -  The dynamic link library for the 16-bit Microsoft C

language interface.

NI488.LIB    -  The import library for IBM CSet language interface.

NIBOR.LIB    -  The import library for Borland C language interface.

NI488_16.LIB -  The import library for Microsoft C language interface.

DECL.H       -  32-bit C program declarations.

DECL_16.H    -  16-bit C program declarations.

ASYNCH.C     -  Source file illustrating the asynchronous functions.

CLR_TRG.C    -  Source file illustrating the device clear and trigger

functions.

EASY4882.C   -  Source file illustrating simple NI-488.2 routines.

EOS.C        -  Source file illustrating the use of the end-of-string

character.

NON_CIC.C    -  Source file illustrating communication from the

standpoint of a non-controller.

PPOLL.C      -  Source file illustrating parallel polls using the

NI-488.2 routines.

RQS4882.C    -  Source file illustrating serial polls with the

NI-488.2 routines.

RQS.C        -  Source file illustrating serial polls with the

NI-488 functions.

SIMPLE.C     -  Source file illustrating simple communication between

a computer and a GPIB device.

Running Your Application Program

--------------------------------

DECL.H  and DECL_16.H contain function prototypes for 32-bit and 16-bit

language interfaces respectively, complete with parameter specifications,

to aid in type checking at compile time.  You must include the respective

file at the beginning of your application program.

If you are using IBM CSet, compile and link your application

program with NI488.DLL as explained in the section "Using your 32-bit

Language Interface" in the Getting Started manual.

If you are using Borland C for OS/2, compile and link your application

program with NIBOR.DLL as explained in the section "Using your 32-bit

Language Interface" in the Getting Started manual.

If you are using Microsoft C Version 6.0, compile and link your application

program with NI488_16.DLL as explained in the section "Using your 16-bit

Language Interface" in the Getting Started manual.