(tm)

SNOOPER

Version 3

Copyright 1989-96 Vias and Associates

All rights reserved.

Here's how to reach us:

Mail:     Vias and Associates

PO Box 470805

San Francisco, CA 94147-0805

Phone:    800-332-8234 (orders only, please)

415-921-6262 (tech support and other queries)

Monday through Friday, 9AM to 6PM, Pacific time

Fax:      415-922-3197 (24 hours)

NOTE: If you find the above information out-of-date, please contact

the Association of Shareware Professionals to get current contact

information. You may reach the ASP at the contact points listed on the

following page.

Internet  72260.1601@compuse               :                   rve.com

WWW:      http://ourworld.compuserve.com/homepages/jvias

CompuServe:  72260,1601

You can find the latest copy of Snooper (SNOOPR.ZIP) in the PC

Hardware Forum (GO PCHW), General Hardware library (4). This was

formerly known as the IBM Hardware Forum.

Also, you can register Snooper in the Shareware Registration forum (GO

SWREG). Snooper's ID is 2535.

BBS:      216-381-3320. PC-Ohio (33.6Kbps, 8N1), Norm Henke, sysop.

Enter your real name and a password when prompted. At the command

prompt, enter "z snoop" and you will see a list of files whose

filenames and/or descriptions include "snoop." When you see Snooper,

press the spacebar until it is highlighted and it will be marked for

downloading. Cancel the "z" command and enter "d" for download. The

PC-Ohio BBS is an excellent, well run system with many interesting

features for PC users, including low cost Internet access. Give it a

call!

VIAS & ASSOCIATES

This program and its documentation were created by Vias & Associates,

a computer consulting, programming, and technical writing firm based

in San Francisco, California. If you, or someone you know, needs

software or documentation written, edited, or designed, please contact

us.

Do you need a customized system information program? For example,

one that writes comma-delimited files? Call us! Our rates are

affordable. Whether it's a computer program, documentation, or

something in between, we look forward to working with you!

_______

____|__     |                (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

This program is produced by a member of the Association of Shareware

Professionals (ASP). The ASP wants to make sure that the shareware

principle works for you. If you are unable to resolve a shareware-

related problem with an ASP member by contacting the member directly,

ASP may be able to help. The ASP Ombud Member can help you resolve a

dispute or problem with an ASP member, but does not provide technical

support for members' products. Please write to the ASP Ombud Member at

545 Grover Road, Muskegon, MI USA 49442-9427, send a FAX to 616-788-

2765, or send a CompuServe message via CompuServe Mail to ASP Ombud

Member 70007,3536.

TABLE OF CONTENTS

VIAS & ASSOCIATES ...................................................2

TABLE OF CONTENTS ...................................................3

WHAT IS SNOOPER? ....................................................6

WHO NEEDS SNOOPER? ..................................................6

You ...............................................................6

MIS Managers and Network Administrators ...........................6

Consultants, Technicians, Support Personnel .......................6

Hardware and Software Vendors .....................................6

Computer Dealers ..................................................7

REQUIREMENTS ........................................................7

FILES ON THIS DISK ..................................................7

INSTALLATION ........................................................8

DOS Installation ..................................................8

Windows Installation ..............................................8

Upgrading from a Previous Version .................................9

THE PHILOSOPHY BEHIND SNOOPER .......................................9

LEGALITIES .........................................................10

Warranty, Copyright, Liability, Trademarks .......................10

License ..........................................................10

WHAT IS SHAREWARE? .................................................11

REGISTERING/ORDERING ...............................................11

Registration Benefits ............................................12

Upgrades .........................................................12

Pricing ..........................................................12

How To Register ..................................................13

SYNTAX .............................................................13

Help Switches (H|?) ..............................................14

Mono Switch (M) ..................................................14

Desqview Mode Switch (D) .........................................14

Logging ..........................................................15

Log Filename Parameter (Lfilename) ...............................15

Log File Environment Variable ....................................15

Auto-Logging Switch (A) ..........................................16

Log DOS System Files Switch (F) ..................................17

Log Windows System Files Switch (W) ..............................18

Quiet Mode Switch (Q) ............................................18

Disk Drive Switch (D:) ...........................................18

Chip Detection Bypass Switches (C, N) ............................18

OtherIRQ Switch (O) ..............................................19

IDE Skip Switch (I) ..............................................19

Tune Skip Switch (T) .............................................19

Configuration File Parameter (Sfilename) .........................19

Config File Environment Variable .................................19

OPERATION ..........................................................20

Windows Operation ................................................21

Keys .............................................................21

ERRORLEVEL .........................................................25

BUGS ...............................................................25

Known Bugs and Anomalies .........................................25

WISH LIST ..........................................................26

MAIN SCREEN ........................................................26

Computer .........................................................26

Advanced Power Management (APM) ..................................26

Central Processing Unit (CPU) ....................................27

CPU Speed ........................................................27

Virtual-86 Mode ..................................................27

Numeric Data Processor (NDP) .....................................27

Bus ..............................................................28

Memory ...........................................................28

Drives ...........................................................30

Video ............................................................31

Ports ............................................................32

ROM BIOS .........................................................33

Mouse ............................................................34

Keyboard .........................................................34

Environment ......................................................35

DOS ..............................................................36

Disk Cache .......................................................37

Network ..........................................................37

Disk Information .................................................38

DIAGNOSTICS SCREEN .................................................41

Serial Ports .....................................................41

Parallel ports ...................................................43

Sound Card Address ...............................................44

Interrupt Request (IRQ) ..........................................44

Direct Memory Access (DMA) .......................................44

BUS SCREEN .........................................................45

NETWORK SCREEN .....................................................45

Network Type .....................................................45

Network Address ..................................................45

Node Address .....................................................45

Socket ...........................................................45

User Name ........................................................45

Hardware .........................................................46

Software .........................................................46

Server ...........................................................46

Connections ......................................................46

CMOS SCREEN ........................................................46

Navigating the CMOS Screen .......................................47

CMOS Status ......................................................47

Date and Time ....................................................47

Memory ...........................................................48

Floppy Drives ....................................................48

Hard Drives ......................................................48

Display ..........................................................49

Coprocessor ......................................................49

Saving Your Changes ..............................................49

BENCHMARK SCREEN ...................................................49

CPU Throughput ...................................................50

Video Throughput .................................................50

Disk Throughput ..................................................50

SETUP SCREEN .......................................................51

Navigating the Setup Screen ......................................51

Mono Mode ........................................................51

Desqview Mode ....................................................51

Editor ...........................................................51

Drive ............................................................51

Sounds ...........................................................52

Skip IDE .........................................................52

Other IRQ ........................................................52

Allow CMOS Editing ...............................................52

Colors ...........................................................52

License Number ...................................................53

Saving Your Changes ..............................................53

A LITTLE BIT ABOUT SNOOPER'S CREATION ..............................53

A LITTLE BIT ABOUT SNOOPER'S AUTHOR ................................54

ACKNOWLEDGMENTS ....................................................54

INDEX ..............................................................56

WHAT IS SNOOPER?

Snooper is a system information utility. It "snoops around" your

computer to report its configuration and operating characteristics.

WHO NEEDS SNOOPER?

You

You can use Snooper to keep an eye on your memory and disk usage.

Snooper can help you when you are installing new peripherals or

software by showing you what resources are already in use. Also, when

you talk to technical support personnel, Snooper can help you answer

many of their questions about your computer, and can let you edit your

CONFIG.SYS and AUTOEXEC.BAT files. You can even determine the

configuration of a used computer you're thinking of buying. Snooper

has many uses!

MIS Managers and Network Administrators

If you are in charge of keeping a large installation of PCs up and

running, Snooper can help. For example, it's well-suited for

collecting hardware inventory information. Add Snooper to a user's

login script with appropriate command line switches and it will

collect information on that node into a file on the server. (See

"Auto-Logging," below, for details on how to do this.) You can then

assemble the files into a database of configuration information.

Afterward, you can tell at a glance which users need DOS upgrades

simply by referring to the database, without visiting each site or

calling each user. Major corporations are using this method to collect

inventory data with Snooper.

Also, we would be glad to create a customized version of Snooper to

assemble all the information you need, in the format you specify.

Consultants, Technicians, Support Personnel

You'll find Snooper helpful for showing at a glance what kind of

machine you are dealing with when you must troubleshoot or upgrade it.

Also, you can use Snooper to help you ensure the computer recognizes

peripherals you install. If you installed a mouse, for example, ensure

Snooper's display shows mouse information. If it doesn't, the mouse

may not work. Snooper also can help you maintain a log of machines

you've worked on. Then, the next time a customer calls, you already

have a good idea of his or her computer's configuration.

Hardware and Software Vendors

With the appropriate license, you may distribute copies of Snooper

with each computer or software package you sell. You'll receive

several benefits:

Snooper, the System Information Program for You!       6

*First, Snooper will make troubleshooting over the telephone faster,

easier, and less expensive. As you know, many computer problems are

caused by incompatible DOS versions, incorrectly installed hardware,

and the like. You can find out much about a customer's system by

asking her to run Snooper and relay the relevant information.

Second, we can create customized versions of Snooper to integrate       *

into your software's installation routine, to ensure the user's

computer complies with the minimum requirements.

And finally, because we can make your company's name appear on       *

Snooper's screen, every time your customers run it, they will

remember it was you who provided them this useful utility.

Computer Dealers

Snooper's display is useful in providing a continuous at-a-glance

display of a machine's specifications for potential customers. No more

scrambling to recall a machine's specs. Simply run Snooper. You can

even determine the configuration of a used computer you're thinking of

buying to resell.

REQUIREMENTS

You may want to postpone reading the rest of this manual to run

Snooper. Go ahead! Snooper normally doesn't affect your computer in

any way except to write a log file if you ask it. Here are its

requirements:

IBM PC or 100% compatible       *

256K RAM       *

*DOS 3.1 or higher

*a video card

In short, Snooper should run on your computer. Make sure Snooper is in

the current directory or on the path. Then just enter "snooper" at the

DOS prompt. Press <Esc> to exit Snooper.

FILES ON THIS DISK

All of the following files should be on this disk or in this

compressed file. If any are missing, or if they don't all have the

same date and time, please don't redistribute the remaining files.

Contact one of the distribution points listed on the front page for a

complete (and possibly newer) version.

Copyright 1989-1996 Vias & Associates    7

Filename       Description

SNOOPER.EXE    Snooper, the System Checker, executable program.

SNOOPER.DOC    Snooper's main manual (this file).

SNOOPER.FAQ    Frequently Asked Questions (and answers) about Snooper.

SNOOPER.HST    Snooper's revision history.

VENDOR.DOC     Information for shareware vendors and sysops.

VENDINFO.DIZ   Information about installing Snooper, its license,

warranty, and more. The first section is human-

readable, the latter is compressed data useful for

shareware vendors and sysops.

FILE_ID.DIZ    Description file some BBSs and vendors will use

automatically. Thanks for helping distribute Snooper!

READ.ME        Brief introductory matierial concernng installation,

etc.

ORDER.FRM      Order form ready to be filled out and sent in.

INSTALLATION

DOS Installation

Installation couldn't be easier. If your copy includes an installation

program, simply enter "a:\install" (or "b:\install"). If not, simply

copy all files to a subdirectory on your hard or floppy disk. Switch

to that subdirectory. You can print this manual by copying it to your

printer. The command:

copy snooper.doc prn

usually works, assuming SNOOPER.DOC is in the current directory and

your printer is attached to your first parallel port. Of course,

Snooper shows you how many parallel ports you have on its Main screen.

Windows Installation

You may not want to run Snooper under Windows. Some of Snooper's

reports must be disabled and you may get erroneous or unpredictable

results or even lockups. Remember, Snooper was designed as a DOS

application and Windows resists programs such as Snooper that try to

access the hardware directly. If you have problems running Snooper

under Windows, just run it from DOS (not from a "DOS Prompt" in

Windows).

If you want to live dangerously by disregarding the above warnings,

follow the instructions under "DOS Installation." When all files have

been copied:

1. In Windows, select an appropriate program group or create a new

one called Snooper.

2. Select New from the File menu.

3. Click on New Program Item, then OK.

4. For Description, type "Snooper."

8    Snooper, the System Information Program for You!

5. For Command Line, type Snooper's full path. For example, if

Snooper resides on your C: drive in your "\SNOOPER" directory, use

the command line "C:\SNOOPER\SNOOPER.EXE" here. (If you want to

use a .PIF file, create one with PIFEDIT to point to SNOOPER.EXE.

Then follow these instructions but make Command Line point to, for

example: "C:\SNOOPER\SNOOPER.PIF" instead.)

6. Click on Change Icon and choose an icon.

7. Last, click the OK buttons until you return to the Program

Manager's main screen.

Snooper's icon should appear in the selected program group. You can

set up other icons for Snooper's documentation, if you wish. To run

Snooper, double click on its icon.

Upgrading from a Previous Version

If you are using an older version of Snooper, simply run the

installation program, if included, or copy the new files over the old.

You may want to retain your .CFG files (see below). Read the history

file, SNOOPER.HST, to find out what we've changed since we released

the version you are using. If you need more details, refer to

appropriate sections of the manual on disk.

Future configuration files are likely to be backward-compatible

with older ones. This means you can likely retain the old

configuration files if you wish. The documentation will state if you

must make new ones.

THE PHILOSOPHY BEHIND SNOOPER

As we designed and wrote Snooper, we kept several design

considerations in mind:

We wanted it to be useful to both beginners and experienced users,       *

for the casual user and the technocrat. If you don't know what "NDP"

means, don't worry. This manual can help you.

We wanted Snooper to run quickly, and to show everything it knows on       *

one (well, maybe a few) screens. With Snooper, you needn't wait for

lengthy searches and you needn't pull down six menus to find what

you want.

*We strived for accuracy in its reports. This fact sometimes can

explain differences between Snooper's reports and those of similar

but vastly inferior programs.

We tried to make Snooper as aesthetically pleasing as possible. Too       *

often an otherwise excellent program is ruined by a garish (and

permanent) color scheme. Snooper's display is, we think, attractive

and easy to read. If you don't like its color scheme, you can change

it. To reduce clutter, when a report would be zero, none, or not

applicable, Snooper simply leaves that report area blank.

*We wanted to make Snooper intuitive and easy to use. We made the

keystrokes and command line switches similar to other programs so

you wouldn't have to learn yet another interface. For example,

Copyright 1989-1996 Vias & Associates    9

typing "snooper/?" at the DOS prompt or pressing <F1> from the Main

screen displays help screens, just as you'd expect.

LEGALITIES

Here's the nasty part. Please bear with us while we assault you with

the usual barrage of disclaimers and legal mumbo-jumbo. It is an

unfortunate but necessary addition to every software manual published

in our litigious society. End of lecture.

Warranty, Copyright, Liability, Trademarks

SNOOPER IS SUPPLIED AS-IS. IT IS NOT GUARANTEED FOR FITNESS OR       *

SUITABILITY FOR ANY PARTICULAR PURPOSE.

USE SNOOPER AT YOUR OWN RISK. NEITHER JOHN VIAS, NOR VIAS AND       *

ASSOCIATES, WILL BE HELD LIABLE FOR ANY DAMAGES--ACTUAL,

CONSEQUENTIAL, OR OTHERWISE--FROM THE USE OF, OR THE INABILITY TO

USE, SNOOPER, OR FOR ERRORS IN ITS DOCUMENTATION.

VIAS AND ASSOCIATES WARRANTS THE MEDIUM UPON WHICH WE DISTRIBUTE       *

SNOOPER, IF GIVEN REASONABLE CARE, TO BE FREE OF DEFECTS IN

MATERIALS AND WORKMANSHIP FOR AS LONG AS YOU USE THE PRODUCT.

ALL OTHER WARRANTIES, EXPRESS OR IMPLIED, ARE HEREBY EXPLICITLY       *

DISCLAIMED. YOUR RIGHTS MAY VARY FROM STATE TO STATE.

SNOOPER, THE SYSTEM CHECKER, A       *                              ND ITS DOCUMENTATION ARE COPYRIGHTED.

ALL RIGHTS RESERVED.

THE BRAND NAMES USED IN THIS MANUAL ARE TRADEMARKS OR REGISTERED       *

TRADEMARKS OF THEIR RESPECTIVE OWNERS.

ANY DISAGREEMENTS ABOUT SNOOPER WILL BE SUBJECT TO THE LAWS OF THE       *

STATE OF CALIFORNIA.

License

We encourage individuals, sysops, user groups, shareware vendors, and

others to use, copy, and distribute Snooper in compliance with these

restrictions:

You are hereby granted a limited license to use Snooper fo       *                                                          r an

evaluation period not to exceed thirty (30) days. After that period,

you must either discontinue use of Snooper or register it with the

author (see below).

*If you wish to use Snooper to gather inventory on a network or use

it in any other way such that more than one person may be running

Snooper at the one time, you must purchase a site- or distribution-

license. Contact us for information and pricing.

*You are encouraged to pass Snooper along, but you must distribute

the program and its related files together, intact and unmodified.

Don't reproduce the printed documentation in any way.       *

Don't disassemble or otherwise reverse-engineer Snooper or any part       *

of it.

*Don't "bundle" Snooper with another product or service without our

prior written permission. This restriction applies but is not

Snooper, the System Information Program for You!       10

limited to: systems dealers, consultants, VARs, and book and

magazine publishers. Please do contact us, however, as we are

delighted to have Snooper included with other quality products.

Shareware vendors, rack vendors, CD-ROM and book publishers, and other

businesses are also subject to the following restrictions:

*Your advertisements, catalogs, and other literature must clearly

explain that users must pay authors for shareware they use. Also,

that money paid the vendor for the disk does not satisfy the user's

obligation to the author.

If the version you have is over six months old, ensure you have the       *

most current version by downloading Snooper from CompuServe or its

distribution BBS. See the first page of this manual for distribution

points.

If we ask you to stop distributing Snooper, do so.       *

OK, enough legal stuff.

WHAT IS SHAREWARE?

Snooper is distributed by a marketing method called shareware. Those

of us who distribute software via this technique believe:

*People should be able to evaluate a program in its actual operating

environment, which a computer store is not;

They should have a reasonable time to evaluate it, which is longer       *

than a computer salesperson's patience will allow (trust us, we

know);

*Users are honest enough to register the program with the author. The

user not only will gain a clear conscience, but also will encourage

the programmer to improve the software, which in turn is good for

the user;

Commercial software is often overpriced. Because many shareware       *

authors don't have to pay for high salaries, advertising, fancy

packaging, and other commercial marketing necessities, we are able

to keep costs down.

Keep in mind that shareware is not free. We shareware authors expect

those of you who use our programs to pay us for our efforts just as

you pay makers of commercial software for theirs. We're just nicer

about collecting.

REGISTERING/ORDERING

As stated above, you are granted a limited license to evaluate

Snooper. If you continue to use Snooper after a 30-day evaluation

period, you must pay the author for it. Although you may have paid a

shareware vendor a few dollars for a disk, you should know the author

gets none of that money. Just as you have to pay for commercial

software you use, you must pay for shareware you use. This is called

Copyright 1989-1996 Vias & Associates    11

registering. You can think of it as ordering the registered version of

the program.

NOTE: If you plan to use Snooper to gather system information from

computers on a network, or by sending copies of it to clients, branch

offices, etc., or in any other way such that Snooper might be used by

more than one person at a time, you must purchase a corporate license

or site license.

Volume discounts and customized versions are available and

encouraged. Please contact us for details. We will need to know how

you wish to use or redistribute Snooper and how many copies you will

need.

Registration Benefits

Your registration fee entitles you to:

*A printed 70-page manual.

The most recent version of Snooper on disk.       *

Notification b       *              y mail of the next major upgrade.

A significant discount on upgrades.       *

Lifetime free technical support (see the first page of this manual       *

for contact information). This is a toll call.

A license number to make the Registration Reminder Screen go away.       *

The peace of mind in knowing you have upheld the law, legitimized       *

your use of Snooper, and supported the shareware concept.

Our undying gratitude.       *

Upgrades

Upgrades are available at a significant discount to registered users

of any previous Snooper version. Upgrades include the latest version

on disk and a printed manual. The disk will include the history file,

so you can see what changes have been made. You can order an upgrade

any time, and a newer version will be sent to you. We will delay

shipment until a newer version is available, if necessary.

Pricing

Here is Snooper's pricing, current as of publication date:

Full version                    Upgrade

Users          Price per user   Users     Price per user

1-9            $39.00           1-9       $15.00

10+            Please call      10+       Please call

Remember, if you are using Snooper to gather hardware inventory across

a network, or are sending out disks to clients, branch offices, etc.,

you must purchase a site license or distribution license. We negotiate

these licenses on an individual basis, because your needs are

different from those of others. If you need multiple disks or manuals,

12    Snooper, the System Information Program for You!

or you have some other special need, please contact us for further

information.

How To Register

It's very quick, easy, and flexible.

Remember, if you are using Snooper in any manner that may allow more

than one person to use Snooper at a time (e.g., gathering hardware

inventory across a network, sending disks to clients, branch offices,

etc.), you must purchase a site license or distribution license.

To register by mail or fax: If you have it, print the ORDER.FRM file.

If not, from Snooper's Main screen press <F1> for help, then <O> for

ordering information, then <P> for printing an order form/invoice.

Make sure your printer is ready. Finally, select the port to which you

want Snooper to print the order form. There is a list of choices at

the bottom of your screen (usually, <1> will work).

Fill out the form completely and either mail it or fax it. Our

address and fax number are on the first page of this manual. You must

send your registration in US funds drawn on a US bank. You can pay

with a business, personal, travelers, or cashier's check; a money

order, Postal Money Order, or American Express International Money

Order. Or you can send cash if that's easiest for you. Make checks

payable to "Vias and Associates."

To register by phone: Have your Visa, MasterCard, Discover, or

American Express card handy and call our tollfree order line listed on

the first page of this manual.

To register by purchase order: If you're ordering Snooper for a

business, just send a copy of the order form/invoice to your

Purchasing department and have them forward a purchase order to us.

When we receive it, we'll send your manual, disk, and license number,

and send an invoice to your Accounts Payable department. This is

faster than your having to contact us first to send you an invoice.

To register by CompuServe: Go SWREG and order item # 2535. Snooper's

price will be added to your CompuServe bill.

Thank you for registering! We think you will enjoy and benefit from

Snooper for years to come.

SYNTAX

There are many switches and options you can use to change Snooper's

operation. Enter them after Snooper's name at the DOS prompt. They are

all optional and all case-insensitive. Each must be separated by at

least one space. You may preface each with a hyphen or a slash if you

wish.

Some have long-name forms, so in a batch file, you can easily see

what the switch is for. They all have single-letter forms, as well,

Copyright 1989-1996 Vias & Associates    13

for faster entry at the command line. In fact, Snooper only notices

the first character (except where it expects filenames, and with the

Auto-Logging switch), so you can create your own long-name forms

(e.g., "/IDESkipped" instead of just "/I").

If you enter an invalid parameter, Snooper will display its first

help screen (command line syntax), and show you the invalid

characters. Here is a diagram of Snooper's command line options. All

options are in brackets to indicate they are optional. Each is

described fully in the following sections.

SNOOPER [H(elp) | ?] [M(ono)] [D(esqview)] [A[B/D/E/L/N/O/S]]

[Q(uiet)] [D:] [C(PUSkip)] [N(DPSkip)] [O(therIRQ)]

[I(DESkip)] [T(uneSkip)] [F] [W] [Cfilename] [Lfilename]

NOTE: The batch mode switch (B) has been removed. Instead, to run

Snooper and have it return to the DOS prompt immediately, use the

Auto-Logging switch by itself (i.e., "snooper a"). It's handy when you

want to quickly run Snooper from a batch file and don't want to have

to press <ESC> to exit. Snooper will get information from only one

disk drive and return to DOS. Used in this way, this switch will not

log any screens.

Help Switches (H|?)

"H" and "?" are help switches. If a help switch is specified, Snooper

displays help screens that show command line syntax, examples, and

keystrokes you can use while in the program. Press <O> from a help

screen to see ordering information. Snooper ignores certain command

line switches when you specify a help switch.

Mono Switch (M)

Normally, Snooper senses if it is being run on a machine with a

monochrome card and automatically uses its monochrome mode background

color. However, Snooper can't tell if your PC has a color card but a

monochrome, color composite, or LCD display, such as most laptops. If

you use such a machine, you should try using the monochrome switch.

Usually, Snooper's display will be more legible, but it depends on the

particular display. Also, you can configure Snooper's colors to create

the most readable color combination, then save it, eliminating the

need to use this switch (see Setup key, below).

Desqview Mode Switch (D)

When Snooper detects Desqview, it uses the BIOS to write to the screen

instead of writing directly to video memory. If you have an old CGA

adapter (on which you may see "snow" when Snooper draws its screen) or

if something interferes with Snooper's detection of Desqview, you may

want to use this switch. Screen displays will be slower.

Snooper, the System Information Program for You!       14

Logging

Logging is handy to use in batch files for getting printouts of

Snooper's output for later reference. If no log file exists, Snooper

will create one. If one already exists, Snooper will append the new

data to the end of the old file. In this way, you can create one file

with system information for all the computers at your site.

The log file grows by 2000 bytes each time Snooper copies a screen

to it. Thus, you easily can tell by looking at the file's size how

many screens you have already logged. For example, if the log file is

8000 bytes, you've logged four screens (8000/2000 = 4). Of course, if

you log system files, which copies CONFIG.SYS and AUTOEXEC.BAT,

WIN.INI and SYSTEM.INI files to the log file, it will no longer follow

this convention.

Snooper first checks for a log filename on the command line. Simply

type it immediately following the "L" (with no intervening space) like

this:

snooper /Lc:\files\snoop.dat

If you want Snooper's output to go to your printer, just specify the

port it's connected to (omit the trailing colon). So if your printer

is on LPT1:, type:

snooper -Llpt1

In these examples, you would have to press the Logging key (Alt-L) or

Log System Files Key (Alt-F) while in Snooper for any logging to

occur. But see Auto-Logging, below.

Log Filename Parameter (Lfilename)

"L" is the log filename parameter. Use this parameter to override

Snooper's default log filename (SNOOPER.LOG). You may specify a full

pathname, filename, or a port (e.g., "prn"). See "Logging," above, for

more information on Snooper's powerful logging features.

Log File Environment Variable

If you don't specify a log filename, Snooper searches the DOS

environment for a variable called SNOOPLOG. (This variable used to be

called SNOOPER. In this release, you may still use the old name, which

was retained for compatibility. However, it probably will not be

recognized in future versions, so please edit your batch files, if

necessary.) You can set the SNOOPLOG variable by typing:

set snooplog=variable

at the DOS prompt. Variable can be any valid filename or port, such as

PRN:

set snooplog=prn

Copyright 1989-1996 Vias & Associates    15

If you don't specify a log file on the command line or with SNOOPLOG,

Snooper writes a file named SNOOPER.LOG in the current directory.

Because not all printers can print line-drawing characters, Snooper

translates its borders to ordinary (low ASCII) characters (hyphens,

vertical bars, and plus signs). This way, virtually any printer can

quickly and accurately print Snooper's display. Therefore, you should

use the logging keystroke or switch and not the <PrtScr> key, which

doesn't translate the line-drawing characters.

NOTE: If you have an early Color Graphics Adapter, you may see "snow"

(interference) on your monitor during the logging process. Don't be

alarmed. This is normal and will not hurt your machine.

NOTE: Snooper must find the screen buffer to log its display. It will

find it even if it has been moved by a program adhering to the

Relocatable Screen Interface Specification (e.g., Desqview, Topview,

and Memory Commander).

Auto-Logging Switch (A)

The Auto-Logging feature automatically copies, or logs, one or more of

Snooper's screens to a file or printer, without human intervention.

You can specify where you want the resultant file written with the Log

Filename parameter (see above), or let Snooper use its default log

filename, SNOOPER.LOG.

The Auto-Logging switch is designed for quickly and easily

gathering hardware inventory. Specify the switch ("A") followed by

letters representing the screens you want logged. They will be logged

in the order you specify. When Snooper is done Auto-Logging, it

returns to DOS.

The Auto-Logging switch follows the rules for specifying log

filenames. It defaults to writing to SNOOPER.LOG. If you want to use a

different filename, use the "L" switch. Or you can use the SNOOPLOG

environment variable. You can cause Auto-Logging to print each screen

instead of logging to a file if you set SNOOPLOG=PRN. See "Examples"

below.

Each letter specified after the "A" corresponds to the Alt- key you

would press in Snooper to access that screen. For example, to log the

Diagnostics screen in Snooper you would press Alt-D. To Auto-Log the

Diagnostics screen, you would specify "AD" on the command line ("A"

for Auto-Log and "D" for the Diagnostics screen). Here are all the

screens and the letters you use to Auto-Log them:

16    Snooper, the System Information Program for You!

Screen to Auto-Log   Letter to       Notes

specify after

the "A"

Benchmark            E               CPU and Hard Disk benchmarks

run. Video is run unless quiet

mode is active.

Bus                  B

CMOS                 O

Diagnostics          D               This screen may take several

seconds.

Main                 L

Network              N

Setup                S

Examples:

To Auto-Log the Main, Diagnostics, and Network screens, in that order,

to SNOOPER.LOG, enter:

snooper aldn

To Auto-Log the Benchmark, CMOS, and Setup screens to

F:\LOGS\1234.LOG, enter:

snooper aeos lf:\logs\1234.log

To Auto-Log the Main and CMOS screens to a printer on PRN, enter:

snooper alo Lprn

or

set snooplog=prn

snooper alo

NOTE: The presence of either or both of the following command line

switches will disable the <Alt-F> keystroke in Snooper, preventing the

user from accidentally logging system files twice.

Log DOS System Files Switch (F)

Specify this switch and Snooper will, upon exit, copy your CONFIG.SYS

and AUTOEXEC.BAT files to the log file, with markers identifying the

beginning and end of each system file. You can use this with the

logging switch, above; it will use the same log file.

Snooper will search the root directory of the drive whose info is

shown in Snooper's main screen for CONFIG.SYS and AUTOEXEC.BAT, so be

sure you're looking at the right disk's info.  Snooper will warn you

via the message box if it can't find the files.

Copyright 1989-1996 Vias & Associates    17

Log Windows System Files Switch (W)

Specify this switch and Snooper will, upon exit, copy your WIN.INI and

SYSTEM.INI files to the log file, with markers identifying the

beginning and end of each system file. You can use this with the

logging switch, above; it will use the same log file.

Snooper searches the current directory and the path for WIN.INI and

SYSTEM.INI.

Quiet Mode Switch (Q)

When Snooper runs in quiet mode, it suppresses all screen output. ANSI

detection and video benchmarks are disabled because to work properly,

they need to display characters on the screen. The ERRORLEVEL

variable, Logging, and Auto-Logging features operate as usual.

Use quiet mode when you don't want users to see Snooper's display,

such as during hardware inventory-gathering or hardware or software

installation. These tasks are easily accomplished via batch files or

login scripts and the Auto-Logging switch.

NOTE: Snooper will let you run it interactively in quiet mode, but do

so only with extreme caution! You run the risk of becoming lost and

confused in Snooper's screens and perhaps inadvertently changing the

CMOS or otherwise running amuck.

Disk Drive Switch (D:)

D: represents a disk drive letter followed by a colon. Use it to

specify the disk drive whose information you want to see first. You

can, of course, see information from any of your drives, as explained

under "keystrokes" below. If you don't specify a drive, or if you

specify an invalid one, Snooper will use the current drive.

Chip Detection Bypass Switches (C, N)

A few machines lock up during Snooper's microprocessor or math

coprocessor detection routines. When this happens, Snooper will draw

its background and then stop. The message box in the lower right

corner of the display will indicate which switch to use. If you have

this problem, all you need to do is rerun Snooper, specifying one or

both of these switches.

The "C" switch prevents Snooper from checking the CPU

(microprocessor) type (it also keeps Snooper from identifying certain

computer types and other information).

The "N" switch prevents Snooper from determining which Numerical

Data Processor (NDP, or math coprocessor) your computer has. It

sometimes can guess anyway, based on the CPU type (assuming the "/C"

switch was not specified, of course).

Snooper, the System Information Program for You!       18

OtherIRQ Switch (O)

A few computers aren't compatible with the method Snooper uses to

detect IRQs in the Diagnostics screen. If you find your computer hangs

in that screen with the message "Getting port IRQ," specify this

switch. Unfortunately, this method isn't as accurate as the default

method, so the results may not be as useful.

IDE Skip Switch (I)

A few computers aren't compatible with the method Snooper uses to

detect the model name of IDE hard drives. If you find your computer

hangs with a "Getting IDE model" message in the message box, specify

this switch. Snooper will then no longer try to detect the drive

model.

Tune Skip Switch (T)

The method Snooper uses to detect sound cards, game ports, and SCSI

host adapters can interfere with some network cards residing at I/O

addresses in the 200-390 range, especially if network drivers are

loaded. If your computer has a network card and Snooper hangs with a

"Getting sound" or "If locked, use /t" message in the message box,

specify this switch. Snooper will then no longer try to detect these

devices.

Configuration File Parameter (Sfilename)

A configuration file contains options that specify Snooper's defaults.

Colors, license number, monochrome mode and other options can be

specified in the configuration file. To create one, see the

instructions under Setup key, below.

When it starts, Snooper looks for the environment variable SNOOPCFG

(see below), which points to a configuration file. If SNOOPCFG hasn't

been defined, Snooper looks in its home directory (i.e., the directory

in which SNOOPER.EXE resides) for a file called SNOOPER.CFG.

Use the Configuration File parameter to load a different

configuration file, which is useful if you have multiple files, as on

a network. Note that specifying a filename is necessary if you use

this switch. If Snooper can't find the file you specified, it will say

so in the message box. Also, if the file's size is incorrect (as may

happen with an old configuration file and a new version of Snooper),

it will report the file to be invalid. To maintain its integrity,

don't attempt to modify the configuration file manually. Use the Setup

screen.

Config File Environment Variable

Snooper searches the environment for a variable called SNOOPCFG. You

can set the SNOOPCFG variable by typing:

Copyright 1989-1996 Vias & Associates    19

set snoopcfg=variable

at the DOS prompt. Variable can be any valid DOS filename.

Command line parameters that follow the specified configuration

filename override the config file's options. Those that precede it are

overridden by the config file's options. In the following example,

Snooper would load a configuration file called C:\UTILS\SNOOPER.BOB.

snooper /Sc:\utils\snooper.bob /m

Regardless of the configuration file's contents, Snooper would run in

monochrome mode because the monochrome mode command line switch was

specified after the configuration filename.

Examples

Some examples will help clarify Snooper's options.

snooper /h

Snooper will show its help screen and examples.

snooper sd:\utils\snoop.cnf e:

Snooper will load the configuration file named C:\UTILS\SNOOP.CNF,

then show information from drive E: and wait for keystrokes. <Esc>

quits.

snooper -M c: a

Snooper will use its monochrome display colors, get drive information

from drive C:, and immediately return to DOS.

set snooplog=prn

snooper -l\snoop.dat al

Snooper will show information for the current drive, then write a log

file in the root directory of the current drive called SNOOP.DAT.

Recall that a log filename on the command line overrides the SNOOPER

environment variable, so Snooper does not log to "prn." There will be

a quiz later.

OPERATION

After the following brief account of Snooper's operation, we will

describe a few parts in greater detail.

When you first invoke Snooper, it looks for its default

configuration file and configures itself accordingly. Then it reads

its command line, looking for switches. It loads a configuration file

if you specified one. Recall that command line switches that follow a

specified config file override that config file's options.

20    Snooper, the System Information Program for You!

Because of the EGA's notoriously buggy cursor routines, we thought

it would be wise to leave the EGA cursor alone, so it stays on. On all

other video systems, Snooper turns off the cursor to make the display

a bit cleaner.

It then gets country-dependent information from DOS, based on the

COUNTRY command in your CONFIG.SYS file. (You don't have to use it

unless you live outside the US.) The information tells Snooper how to

arrange and punctuate the date, time, and numbers.

At the bottom of its display, Snooper shows the day, date, and time

according to DOS. This feature comes in handy when you want to view a

log file of Snooper's output as it lets you know when it was made. It

also allows you to check your computer's date and time for accuracy

(some computer clocks run slowly).

After Snooper shows you its display, it awaits certain keystrokes.

Invalid keystrokes cause Snooper to beep. This feature, added partly

to aid visually impaired users, can be turned off via the Setup

screen.

When you press <ESC>, Snooper writes a log file if you told it to,

turns the cursor back on, sets the ERRORLEVEL variable, and returns

you to DOS.

Windows Operation

Snooper performs somewhat differently under Windows. If Snooper is

idle, it gives up its time slice immediately. What this means is

Snooper won't slow down your other running programs. The only drawback

is that the time display freezes. Just press the spacebar when

Snooper's window is active to update the time. Ignore the beep if you

hear one.

In Windows 386 enhanced mode, Snooper turns off IDE model checking,

which interferes with 32-bit disk access. Also, if you invoke the

Diagnostics screen (in Windows or any other detected multitasker or

network), a message pops up warning you that what the Diagnostics

screen must do may disrupt other currently running applications. You

are given the option of continuing or returning to the Main screen.

Benchmark results under Windows will vary widely due to

multitasking. To gather accurate benchmarks, run Snooper from DOS.

You can paste Snooper's display, as text, into the Clipboard (see

Paste to Clipboard key, below).

NOTE: Sometimes, Snooper will not run properly under Windows. If you

find this to be the case, simply exit Windows completely and run

Snooper from the DOS prompt.

Keys

On the last line of each of Snooper's displays, there is a list of

available keystrokes to jog your memory. Note that on the Main screen,

because of space limitations, not all the available keys are listed.

However, they are all listed on the help screen (press <F1> from the

Main screen). Explanations of Snooper's secondary screens are given

after those for the Main screen, below.

Copyright 1989-1996 Vias & Associates    21

Exit Key

The <Esc> key returns you to the Main screen if you're not already

there. If you are, it exits Snooper and returns you to DOS.

Help Key

The <F1> key, when pressed from Snooper's Main screen, displays the

second of two help screens: keys available while you're in Snooper.

The first help screen, accessible with <PgUp>, <Up>, or <Home>, shows

Snooper's command line syntax, just as if you had used Snooper's help

switch at the DOS prompt (i.e., "snooper /?").

Order/Registration Key

Pressing the letter "O" from a help screen displays Snooper's ordering

screen. Once there, if you'd like to print an order form/invoice to

fill out and send to us, press <P>. For your convenience, along the

bottom of your screen, Snooper lists every parallel and serial port in

your system. You print to any listed port (assuming there is a printer

attached), or you may print to a file, which you can then edit and

print yourself. Press a number and Snooper will print the order

form/invoice to the corresponding port. Normally, you would type <1>

for LPT1. This is just one of the easy and fast ways to register

Snooper, and we hope you will find it convenient (soon!).

Disk Drive Keys

On the right side of Snooper's Main screen, details about a single

disk are shown. You can use the <Left>, <Up>, and <PgUp> keys to tell

Snooper to get disk information for the next lower-lettered disk. For

example, if you're looking at C:'s info, press <PgUp> to see drive B:.

The <Right>, <Down>, and <PgDn> keys do the opposite. <Home> takes you

to drive A: and <End> takes you to the last valid drive in your

computer.

Alternatively, you can press the letter corresponding to the drive

you want to see. If you want the A: drive's data, for example, just

press <A>.

The list at the top right of the display shows you which drives are

available on your system, and which drive's info you are viewing. If

you press an invalid drive letter, Snooper will beep (if sound is

enabled) and display a message in the message box.

File Editing

From the Main screen you can invoke a text editor to edit your

CONFIG.SYS, AUTOEXEC.BAT, and SNOOPER.LOG files.

Snooper has a simple but capable editor built in. Press <F1> when

you're in it to view its help screen. The obvious advantage of using

the internal editor is it's always available. If you wish, however,

22    Snooper, the System Information Program for You!

you can have Snooper load an external editor, perhaps one with special

features you like. The Setup screen lets you choose which editor

Snooper will use, internal or external.

Whatever editor you choose, Snooper ensures the file you want to

edit exists before attempting to load the editor. The message box will

alert you if the file is missing.

After you exit the editor, Snooper asks if you want to reboot your

computer, necessary for changes to CONFIG.SYS and AUTOEXEC.BAT to take

effect. If you choose to reboot, Snooper will delay several seconds to

try to ensure the file is actually written to the disk. Otherwise, a

disk cache with write-delayed caching may not have time to save the

edited file.

An external editor must be name  EDIT.COM, EDIT.EXE, or EDIT.BAT.                                         d

It must be in the current directory or on the path. If it's not

already named EDIT, you can simply rename it or create a batch file

called EDIT.BAT that calls it. You can, of course, add other commands

in the batch file. Use a replaceable parameter for the filename.

Here's a sample EDIT.BAT file that works for all three files Snooper

lets you edit:

echo off

copy %1 c:\backup\%1

ed /r %1

In this example, the original file is copied to another directory for

safe keeping. Also, a hypothetical switch ("/r") is passed to an

editor named ED. This should give you an idea about what can be put in

EDIT.BAT.

Edit CONFIG.SYS and AUTOEXEC.BAT Keys

Press <Alt-C> to edit CONFIG.SYS or <Alt-A> to edit AUTOEXEC.BAT.

Snooper looks for the file in the root directory of whatever drive

you're looking at on its Main screen. So if you want to load

C:\AUTOEXEC.BAT, ensure drive C:'s information is showing before you

press <Alt-A>. Check the highlighted letter in the drive list to be

sure. Snooper will tell you if the file you want to edit doesn't exist

in the root directory of the drive you're looking at. Remember, you

can log these files with the Log System Files Switch (F) or Key (<Alt-

F>).

Edit SNOOPER.LOG Key

From Snooper's Main screen press <Alt-V> and Snooper will invoke an

editor so you can edit an existing SNOOPER.LOG file. You may want to

do this to compare earlier screen dumps with recent ones, or to add

comments to the log file. See above for editor-naming conventions.

Log Key

Use <Alt-L> to write a log file to disk or to a port, such as a

printer port (see also Log to Printer Key, below). You may log any

Copyright 1989-1996 Vias & Associates    23

screen with this key. It works similarly to the logging command line

switch except that it works immediately, not after you exit. If you

also specified the logging switch with a filename, it will use that

filename. If not, it will use the SNOOPLOG environment variable if it

exists; otherwise, the default: SNOOPER.LOG.

The information you're looking at when you press the logging key is

the information that will be logged. Because of this, you can press

<Alt-L> in the Main screen, then <Alt-D>,<Alt-L>,<Esc> and so on until

you have a log file containing snapshots of all the screens you're

interested in. If you wish to do this automatically, simply use the

Auto-Logging Switch (/A, see above). This log file can be very useful

when you need to remember what kind of computer someone has. You can

even log help screens for future reference.

Log to Printer Key

<Alt-P> will print the screen you're looking at. It sends its output

to PRN. If your printer is on LPT2:, simply set the SNOOPLOG

environment variable  or the /L parameter to LPT2 and use the <Alt-L>

keystroke instead.

Log System Files Key

Pressing <Alt-F> causes Snooper to copy the CONFIG.SYS, AUTOEXEC.BAT,

WIN.INI, and SYSTEM.INI files to the log file. The CONFIG.SYS and

AUTOEXEC.BAT files on the drive whose information you're viewing are

the ones that will be added to the log file. The message line will

tell you if neither file was found. Snooper will look for the Windows

files in the current directory and along the path and will log

whichever files it finds.

NOTE: If either or both Log System Files switches (F and W) are

specified on the command line, this keystroke will be disabled,

preventing the user from accidentally logging the system files twice.

Log to Windows Clipboard Key

When Snooper is running under Windows, you can press <Shift-Delete> to

paste a copy of Snooper's display to the Windows Clipboard. (<Ctrl-

Insert>, the copy key, would have been more appropriate, but it's only

available on enhanced keyboards.) Because many Windows fonts don't

include high-ASCII characters, Snooper will translate its line-drawing

characters into low-ASCII characters as usual. You can then paste the

display into a Windows document. Of course, for the characters to

align properly, you must use a monospaced font such as Courier or OEM.

24    Snooper, the System Information Program for You!

ERRORLEVEL

If certain errors occur, upon exit Snooper sets the ERRORLEVEL batch

file variable and shows the value of ERRORLEVEL on the screen in the

message box. Here are the error conditions ERRORLEVEL reports:

Errorlevel Meaning

Successful completion: there were no errors.                 0

Drive door was open or there was no disk in a requested                 1

(or default) disk drive, or a disk was bad or unformatted.

2 User specified an invalid drive on the command line.

4 DOS version is older than 3.1.

8 Error occurred while Snooper was writing a log file.

16 User specified an invalid command line parameter.

32 Abnormal termination: Snooper encountered an unanticipated

problem and terminated.

Notice the numbers aren't consecutive. This numbering scheme allows

Snooper to add the numbers to indicate multiple errors.

For example, if you specified an invalid drive and Snooper

encountered an error writing a log file, it would set ERRORLEVEL to

ten, the sum of two and eight. The ERRORLEVEL variable can be tested

in batch files. For example:

if ERRORLEVEL 10 goto BadDrv_and_LogError

See your DOS manual for details and usage of the ERRORLEVEL variable.

BUGS

Snooper has been under development for several years, but it may still

have a bug or two lurking in it. To debug Snooper thoroughly, we would

need access to a huge array of computers and peripherals. We don't. It

has been used successfully on hundreds of machines but such testing

can never be comprehensive.

If you think you've found a bug, please let us know by sending the

invoice (don't forget your phone number) with a specific description

of the bug. Include a printed log file if you can. Of course, an

inaccurate report may mean your hardware, BIOS, DOS, TSRs, or

something else is incompatible or is interfering with Snooper,

especially on older machines. We'd like to hear from you anyway.

Known Bugs and Anomalies

A few machines lock up when running Snooper. The culprit is       *

sometimes the CPU, NDP, IDE, sound card, or IRQ detection routines.

Simply specify the "C," "N," "S," "T," or "O" command line switches

(the message box may tell you which, or try each). Plus, see the

SNOOPER.FAQ file for more details. We've fixed this problem more

than once but there's always one more machine out there....

Copyright 1989-1996 Vias & Associates    25

*Because of the way NDOS and 4DOS allocate memory in their non-

swapping modes, Snooper can't find the environment, and will give an

incorrect Environment report such as: "Free 65,536 Total 0."

WISH LIST

Snooper will never be complete, of course. We have continually found

new reports and features to add to make it an easy-to-use and powerful

utility. There are a few things we still want from Snooper:

Brand detection of more no       *                          n-Intel CPUs and NDPs

Detection of VL local bus       *

*Detection of tape drives

Differentiation of ST506, IDE, ESDI, and SCSI hard drives       *

If you think you can help us with the above challenges, please call,

write, or leave E-mail. With your help, Snooper can become an even

more powerful utility. Also, we'd be glad to mention your help in the

acknowledgments if you'd like (all together: "Oooh, ahhh!").

MAIN SCREEN

Following, roughly in the order they appear on-screen, is a detailed

list of Snooper's Main screen reports, followed by the reports on the

other screens.

Computer

The type of computer Snooper is being run on. On some XT and AT

clones, Snooper can't tell if it's running on an actual IBM machine or

a compatible, so it will report the IBM equivalent (e.g. "PC AT").

Snooper can recognize over a hundred machines by name, including many

ATTs, Dells, IBMs, Olivettis, and Toshibas.

NOTE: If you are not skipping CPU detection and Snooper reports

Computer Type as "Unknown ID: . . ." please drop us a note with the ID

numbers, and the exact model and brand computer Snooper was running

on. Thanks.

Advanced Power Management (APM)

If Advanced Power Management (APM), which implements energy

conservation, is active, Snooper will say so after the computer type.

26    Snooper, the System Information Program for You!

Central Processing Unit (CPU)

The computer's microprocessor chip. Snooper can report the presence

of: 8088/86, V20/V30, 80286, 80386SX, 80386DX, 80486SX, 80486DX, and

Pentium CPUs.

If you have a math coprocessor, and you disable CPU detection but

leave math coprocessor detection on, Snooper will try to guess what

CPU you have by the NDP type.

NOTE: If Snooper reports "80386," it means your 386SX or 386DX CPU is

in virtual-86 mode (explained below). Some operating environments and

expanded memory managers (e.g. EMM386) would report an error if

Snooper tried to determine which of the two chips is present. Instead,

Snooper skips the determination and shows you have one of the two

chips. If you unload the program that puts your CPU into virtual-86

mode, Snooper can then tell you which CPU you have.

CPU Speed

The number after the hyphen (e.g., the "90" in "Pentium-90") is an

estimate of your computer's speed in Megahertz. This benchmark is very

accurate, given that it doesn't induce a noticeable delay in

execution.

Virtual-86 Mode

If "V86" appears in the CPU report, your computer's CPU is operating

in virtual-86 mode, instead of real mode. The 80286 and newer CPUs

implement protected mode, which is often used by memory managers,

multitaskers (including Microsoft Windows), and other software.

Multitaskers that put the CPU in protected mode actually run programs

in virtual-86 mode, which tricks each program into believing it has

total access to the machine. In truth, the multitasker controls the

programs' access.

Numeric Data Processor (NDP)

The Numeric Data Processor (NDP), also called the math coprocessor, or

Floating Point Unit (FPU). Math coprocessors significantly speed

calculations involving floating point numbers (i.e., numbers with

decimal portions). Snooper can detect 8087, 80287, 80387SX, 80387, and

Weitek 1167 math chips. Snooper will report "internal" for 80486DX and

better CPUs, since they have math coprocessors built into them.

Some computers have a switch or configuration option that the user

sets to reflect the presence (or absence) of a math coprocessor.

Because these are often set incorrectly, Snooper's report is not

dependent on this setting. But Snooper does check the setting. If

Snooper finds its NDP report and the switch setting don't agree, it

displays a check mark in the NDP report. This lets you know you should

check (and reset) the switch (and/or system configuration, if you have

an AT-class computer). If you don't see a check mark, the switch or

configuration is set properly.

Copyright 1989-1996 Vias & Associates    27

Bus

The architecture of the bus your computer uses. The bus is the part

you plug expansion cards into: the slots. Most computers report "ISA"

(pronounced "ICE-uh"), which stands for Industry Standard

Architecture, the bus in PCs, XTs, ATs, and most clones.

Many IBM PS/2 computers will report "MCA," or Micro Channel

Architecture. The MCA is entirely different and cards based on it are

incompatible with those for the ISA standard, but MCA does add

features and increase performance in some cases. Some computers have

both buses, in which case Snooper reports "MCA+ISA." For a list of the

MCA adapter cards in your system, invoke the Bus screen by pressing

<Alt-B>.

A consortium of companies worked out a different standard. It is

called "EISA," (pronounced "EES-uh") meaning Enhanced ISA. It retains

compatibility with the older ISA bus, but like the MCA, it adds

features and power. Still, it is losing popularity in favor of the PCI

bus.

Local buses provide increased throughput over traditional buses

such as ISA. Apparently, there is no software detection available at

this writing for the VESA Local bus, or VL-Bus, so Snooper can't

detect it. However, Snooper can detect the PCI bus, developed by

Intel. It will also report if your system has both PCI and EISA buses.

Memory

Conventional Memory

Bytes of conventional RAM (the first 640K) in your computer.

NOTE: A few computers use more RAM than most for their video displays,

so Snooper may not report all the memory the computer has (e.g., it

may report 624K instead of 640K on some Tandys).

Free Memory

Bytes of conventional RAM still available for programs and data. This

report allows you to see the effect of, for example, loading and

unloading memory-resident programs.

Used Memory

Bytes of conventional memory DOS and memory-resident programs are

using (total minus free memory).

Extended Memory

Extended memory is memory beyond the 1M boundary. It is available only

on AT-type machines (that is, those with 80286 or newer

Snooper, the System Information Program for You!       28

microprocessors). (If you have an XT-type computer, this report will

always be blank.) Programs written to do so can store data in extended

memory. This report shows the total amount of extended memory

installed, despite how you've configured it. Even if a program is

using it all, Snooper will still show the total amount installed. This

report is taken from the CMOS. See the CMOS screen section below for

an explanation of the CMOS.

Ext Free

This report shows how much extended memory you still have available.

Snooper gets this report from the computer's BIOS. This memory is only

available to mostly older programs that don't use XMS (see below).

XMS Memory

As we mentioned above, programs written to do so can store data in

extended memory. The trouble is, different programs handle extended

memory differently. This is because until the eXtended Memory

Specification (or "XMS"), there was no standard for accessing extended

memory, and the computer's built-in facilities are crude at best. Some

programs are polite enough to decrease the amount of extended memory

they report to other programs by the amount they themselves are using.

Some aren't so polite, leaving themselves open to having their memory

overwritten by another program. As you might imagine, this leads to

problems.

If you have loaded an eXtended Memory Manager, or XMM (e.g.,

Microsoft's HIMEM.SYS)--that is, if you have XMS memory, Snooper will

report how much is still available.

eXtended Memory Specification Version

The number displayed after "XMS" is the version of the eXtended Memory

Specification the loaded XMM supports. This is different from the

internal version number of the XMM program itself. Newer versions

support added features, so be sure your XMM supports the program you

want to run.

High Memory Area (HMA) status

The high memory area is the 64K (less 16 bytes) block beginning at the

1M boundary, immediately above the ROM BIOS. It, like all types of

extended memory, can be found only on AT-type computers, and only by

programs written to use it. This report shows you if a HMA exists (you

must have an XMM loaded to provide it), and if it is available for use

or is being used by a program. If the HMA is in use, the information

this report provides depends on the DOS version the machine is running

under. If DOS 5 or newer is loaded, Snooper will tell you how many

bytes of the original 64K are still free. If not, it simply lets you

know the HMA is in use. Normally, only one program at a time uses the

HMA, but it can be shared.

Copyright 1989-1996 Vias & Associates    29

A20 Line Status

Shown after "HMA" on the same line if you're running DOS 4 or lower,

this report shows if the CPU address line called A20 is enabled.

Snooper shows "(A20)" is the A20 is enabled, nothing if it's not.

Usually, the A20 is handled automatically by memory management

software. At times, you may need to know the A20's status, perhaps

while investigating an obscure memory management problem.

Upper Memory Block (UMB)

With DOS 5 and some third party memory managers, you can load some of

your device drivers and other memory-resident programs above

conventional memory, into what are called upper memory blocks. Snooper

reports the largest available UMB. You must have the line "DOS=UMB" in

your CONFIG.SYS file for this report to work.

Expanded Memory Specification Total

The amount of expanded memory (EMS) installed. Lotus, Intel, and

Microsoft developed EMS to break DOS's 640K memory barrier (sort of).

Some programs, which have been written to do so, can use EMS to store

data. Not surprisingly, many spreadsheet programs use EMS. Unlike

extended memory, EMS is available on both XT- and AT-class machines.

It is not used by Windows applications.

Expanded Memory Manager Level

Shown after "EMS," the version of Expanded Memory Specification the

hardware and/or software is implementing. As of this writing, this

report probably should be either 3.2 or 4.0. Some programs require EMS

4.0, which has enhanced capabilities. Like the XMM version, this

reflects the specification version, not the EMM program's version.

EMS Memory Free

The amount of EMS still available for programs and data.

Drives

Floppy

The number of floppy drives installed. Despite what some people

believe, 3.5" diskettes are floppy disks, not hard disks, despite

their hard plastic shells. Thus, they will be counted in this report.

30    Snooper, the System Information Program for You!

Hard

The number of hard disks (also called "fixed disks") installed. Each

physical hard disk adds to the total, regardless of its partitioning.

That is, if you have one hard disk partitioned into C: and D: drives,

it will count as one hard disk.

Physical

Simply the sum of floppy and hard disks. These are physical drives

attached to your computer.

Logical

The total number of disk drives DOS recognizes. These include floppy,

hard, RAM, CD-ROM, and network drives. Also included are simulated

disk drives made with the DOS ASSIGN and SUBST commands, all hard disk

partitions, and possibly other types. This report includes the drives

identified in the Physical Drives report (above). Not included are

tape drives, which are not directly recognized by DOS.

Sometimes software splits a hard disk into two or more "partitions"

(usually C: and D:). A hard disk like this will count as two. If you

have a single floppy disk drive, the number given will not include

drive B:, as it's simply another name for drive A:. But all other

available drive letters count.

Video

Video Type

The type of video adapter, and, mostly for VGAs, either the adapter

brand or the type of monitor used. Snooper can detect:

Report     Video card

MDA        Monochrome Display Adapter, probably a text-only card (one

that doesn't provide graphics capability).

Hercules   Hercules Graphics Card (a monochrome card with graphics

capability) or a Hercules-compatible; Snooper can detect

Hercules Graphics, InColor, and Plus cards by name.

CGA        Color Graphics Adapter

EGA        Enhanced Graphics Adapter

PGC        Professional Graphics Controller

MCGA       Multi-Color Graphics Array

VGA        Video Graphics Array

SVGA       VGA card with more than 256K

XGA(-2)    eXtended Graphics Adapter, on Micro Channel PCs

Copyright 1989-1996 Vias & Associates    31

VGA Card Brand

Snooper recognizes several VGA cards by brand, and many specific

models.

VESA

A few years ago, several video hardware manufacturers formed the Video

Electronics Standards Association, or VESA (pronounced "VEE suh" or

"VEH suh"). Their purpose is to standardize the PC video marketplace,

making recommendations for standardized resolutions, frequencies, and

so on. If the Video report begins with "VESA," your VGA card seems to

comply with their recommendations. Snooper also shows you the VESA

specification version your card complies with.

Monitor Type

Snooper can sometimes tell what type of monitor you are using. If

Snooper has room on the screen, it shows which of three kinds of

monitor you have: monochrome (usually green or amber), color (digital

with EGAs or analog with VGAs or MCGAs), or digital color (for some

MCGAs).

Video Memory

On EGA and some VGA cards, Snooper reports how much video memory is

installed on the card. EGAs can have 64K to 256K; VGA cards, 256K to

4M and beyond. The amount of video memory, the video card, and the

monitor together determine the maximum resolution and colors you can

see on your computer. With some early VESA cards, Snooper may show a

little less memory than is actually installed (e.g., 1.5M versus 2M),

as there was no way to tell for sure.

Ports

Serial Ports

The addresses of all serial, or RS-232, ports installed. Usually, you

would use these ports for modems, pointing devices (such as mice),

some printers, plotters, and a variety of less common equipment. More-

advanced serial port information is available in the Diagnostics

screen via <Alt-D>.

Parallel Ports

The addresses of all parallel ports (also called printer ports)

installed. As you would think, parallel ports are usually used for

printers, but are now increasingly used for scanners, network

adapters, and other peripherals. More-advanced parallel port

32    Snooper, the System Information Program for You!

information is available in the Diagnostics screen, accessed via the

<Alt-D> key.

NOTE: Netware tricks software (including Snooper) into thinking there

are more parallel ports than are actually present. If your system is

part of a Netware LAN, don't be surprised to see three parallel ports,

some with the same address. To find out how many parallel ports there

really are, take your machine off the network and rerun Snooper.

Game Port

The presence of a game port, or joystick adapter. The word "Game" will

appear next to "Ports" if Snooper detects a joystick. The joystick

detection methods seems to work well, but they are imperfect, as the

PC provides no foolproof way to detect if a game port is present.

Also, a joystick must be plugged into the port for the report to work.

Sound Cards

The presence of Sound Blaster, Adlib, or Roland MPU-401 MIDI sound

cards, or their compatibles. The Diagnostics screen will show the port

address of some sound cards. Press <Alt-D> for this screen.

ROM BIOS

Brand

The manufacturer of the computer's ROM BIOS. Snooper can recognize

several BIOS brands: American Megatrends, Inc. ("AMI"); Award; Chips

and Technology ("C & T"), Compaq; DTK; IBM; Phoenix; and Zenith.

How well the BIOS was written has much to do with how compatible

your computer is. A poorly written BIOS plagues its owner with

compatibility problems: programs won't run or they lock the computer,

new hardware refuses to install properly, etc. A well-written BIOS is

a joy to behold.

Date

The date stored in the ROM BIOS, which provides an indication of your

computer's age. The computer was built since that date.

Plug-N-Play

With limited hardware resources such as IRQ lines and DMA channels

(described in the Diagnostics screen section below) to go around, many

users endure hours of configuration nightmares when installing a new

adapter card. Although the Micro Channel and EISA buses solve this

problem with semi-automatic configuration, nothing has helped the ISA

bus. Plug-N-Play hopes to do just that. With Plug-N-Play extensions in

Copyright 1989-1996 Vias & Associates    33

your system's BIOS, your computer can arbitrate potential hardware

conflicts for you, automatically.

Extensions

The segment addresses (places in memory) of any BIOS extensions in the

computer. These extensions, which supplement the computer's built-in

BIOS, are usually found on add-in cards. An EGA or VGA BIOS, for

example, adds routines not found in the computer's own ROM BIOS video

routines, and are often found at C000. An XT's hard drive BIOS is

usually found at C800. Snooper searches for an extension every 256

bytes from C000 to FE00 (i.e., C000, C100, C200, etc.). This report

comes in handy for telling your memory manager to avoid addresses used

by adapter cards.

Mouse

Brand and Driver Version

Shows what brand of mouse is installed (Microsoft, Logitech, Z-NIX,

and Mouse Systems), and the driver version.

A mouse usually requires a software-based driver (program). Its

file is usually named MOUSE.COM or MOUSE.SYS. If a driver is loaded,

Snooper will report its version. This report is useful for debugging,

because if you're having trouble with your mouse, you may find that a

new driver solves the problem. Also, forcing the driver to load into

conventional, and not high, memory also can solve other problems.

Drivers are often available free or at low cost from the mouse vendor.

Some drivers, such as Genius and Logitech, report a Microsoft-

equivalent version rather than their own internal version. Snooper can

report a Logitech mouse driver's true version.

Port

Possible reports are: "bus" (the mouse connects to a specialized

expansion card inside the computer), "Microsoft Inport" (a Microsoft-

brand bus mouse), "serial" (the mouse plugs into a serial port),

"Hewlett Packard," and "PS/2."

If you have a serial mouse, Snooper tries to guess which serial

port it is connected to by knowing what resources the mouse is using.

If it isn't using IRQ4 (COM1 or COM 3) or IRQ3 (COM2 or COM4), Snooper

displays which IRQ it is using. The Diagnostics screen shows which IRQ

the mouse is using, and if it's a serial mouse, which port it's on.

This can aid in troubleshooting.

Keyboard

Type

The first number in the Keyboard report. Shows what kind of keyboard

you have attached to your machine. Possible answers are "84" (non-

Snooper, the System Information Program for You!       34

enhanced, or XT), "101" (enhanced, or AT, including keyboards with

even more keys), or, if your computer only supports 84-key keyboards,

"N/A" (if your computer doesn't support enhanced keyboards, Snooper

can't test for one).

Support

Reports what type of keyboard your computer's BIOS supports. If "101"

appears after the keyboard type, your computer's ROM BIOS supports an

enhanced keyboard, often with function keys along the top and a

separate cursor keypad. It recognizes the keys an enhanced keyboard

adds to the standard keyboard, such as F11, F12 and certain cursor key

combinations.

Environment

The DOS "environment" is an area of memory in which DOS keeps certain

information it needs to run. Some examples of information in the

environment are the format of the DOS prompt and the series of

directories DOS searches for executable files. These are the PROMPT

and PATH strings, and there are others. A string consists of the

variable name (such as PATH), an equal sign, and the value of the

string (often a path or list of paths such as C:\UTILS;C:\DOS;C:\).

This sample PATH string looks like this:

path=c:\utils;c:\dos;c:\

You can view and edit the environment with the SET command. Because of

the way 4DOS and NDOS allocate memory for the environment in their

non-swapping modes, the environment reports will be inaccurate when

Snooper is run under those environments.

Free

The number of bytes available in the environment. To calculate this

report, for each string, Snooper counts each character of the variable

name (such as "PATH"), the equal sign, each character of the value

(such as "C:\DOS;C:\"), and one hidden character.

Total

The number of bytes of environment space allocated by DOS. You can

change the amount of memory DOS allocates to the environment with the

/E switch of the SHELL command in your CONFIG.SYS file. Different DOS

versions use different memory units for the extra environment space.

Check your DOS manual.

Copyright 1989-1996 Vias & Associates    35

DOS

Brand and Version

Snooper can detect: PC-DOS (from IBM), MS-DOS (from Microsoft,

Phoenix, and others), DR DOS (from Digital Research), HP-DOS (from

Hewlett Packard), DEC-DOS (from Digital Equipment Corp.), or Zen-DOS

(not Eastern mysticism, just DOS from Zenith). Snooper also will

report if it is running under OS/2 1.x or 2.x. A letter appears after

the version on systems running DOS 5.0 or newer. This is the DOS

revision, a sub-version of sorts.

Versions of DOS since 5.0 can load most of themselves into the HMA,

freeing the conventional memory they would normally use. If "HMA"

appears after the DOS type, DOS is loaded there. If "ROM" appears, DOS

is stored in the computer's ROM, as with some laptops.

NOTE: Both MS- and PC-DOS 3.30 report themselves as PC-DOS; Snooper

reports that version as "MS/PC-DOS," indicating it can't

differentiate.

Shell

A DOS shell is a program that provides features DOS doesn't or makes

DOS easier to use, or both. Snooper can report the presence of:

Microsoft Windows Real, Standard, or 386 Enhanced modes; Concurrent

DOS; DoubleDOS; 4DOS and NDOS (only in swapping mode); Desqview;

Taskview; Topview, DOS 5's task switcher, the Virtual Control Program

Interface (VCPI), the DOS Protected Mode Interface (DPMI), and

ANSI.SYS. It can often report the DOS shell's version number as well.

If Snooper detects Desqview, it uses the BIOS to write to the screen.

If Snooper detects 4DOS or NDOS, it tells what shell level it is

running under. This kind of shell has a different meaning. When you

shell from a program (it can be called something else, such as "DOS

prompt"), you leave the program in memory but return to the DOS

prompt. You type "exit" to return to your program. This is faster than

exiting, then reloading the program, and lets you perform a simple

task such as formatting a diskette. "Root" means you're not shelled

out of a program; "1" means you've shelled out of one program; "2"

means you shelled out of one program, loaded and shelled out of

another; and so on. Snooper doesn't have a shell feature.

Files

The number of files DOS will allow to be open at once, as defined by

the command "FILES=nn" in your CONFIG.SYS file. DOS always reserves

some files for itself. Usually, your computing activity will require

several files to be used at once, especially if you use a multitasking

environment or a database. Most software vendors recommend you make at

least 20 files available, requiring the line "FILES=20" to be in your

CONFIG.SYS file. A few programs, notably Windows, may increase this

number, so don't be surprised if the number is higher in Windows than

in DOS.

36    Snooper, the System Information Program for You!

Buffers

Number of buffers DOS uses for disk operations, as defined by the

command "BUFFERS=p[,s]" in the CONFIG.SYS file. When your computer

requests data from a disk, DOS transfers the data from the disk into

RAM, where the program can access it. Subsequent requests for the same

data are read from RAM, not from the disk. The reason is simple: RAM

is fast, the disk is comparatively slow, so disk operations are sped

up. Snooper detects the number of primary (the p above) and secondary

buffers, if defined (the s above). See your DOS manual for more

details.

Break

Break status (on or off). Pressing Ctrl-Break can force your computer

to stop what it's doing in an emergency. Normally, DOS only checks for

a Ctrl-Break keystroke when it's writing to the screen or reading from

the keyboard, but if break is on, DOS checks more often.

Verify

Disk verify status (on or off). If verify is on, DOS uses a checksum

method to confirm (in theory) what it writes to a disk is valid. Of

course, when verify is on, disk operations are a bit slower.

Disk Cache

Reports the presence of a disk cache, and often, its version.

Recognized caches (and vendors) include: Smartdrive (which comes with

DOS and Windows); Norton Cache (Norton Utilities); PC-Cache 6.0 and

above (PC Tools); IBMCache (from you-know-whom); Super PC-Kwik 3.20

and above, Hyperdisk, and QuickCache II (shareware products), and

possibly others.

NOTE: Because PC-Cache and Qualitas's QCache are versions of Super PC-

Kwik, they respond to the same detection method and so may be reported

as Super PC-Kwik. PC-Cache 5.1 would be reported as Super PC-Kwik

3.20, PC-Cache 5.5 as Super PC-Kwik 3.27. QCache 4.00 would be

reported as Super PC-Kwik 4.00.

Network

A network is a combination of hardware and software that enables users

to share peripherals and data. Snooper can detect several networks:

Netware, LANtastic, Microsoft, Invisible, PC LAN, and Easy-Net. It

also detects SHARE.EXE, which is often loaded in networking

environments. Sometimes this report only tells you the computer has a

LAN card, not that a network is up and running. On LANtastic, it also

shows the version. For more information about your Netware network,

invoke the Network screen by pressing <Alt-N>.

Copyright 1989-1996 Vias & Associates    37

Disk Information

Drive List

Snooper reports the letters of all valid disk drives in the upper

right corner of Snooper's display, enclosed in brackets. The current

drive's letter is highlighted and capitalized. If you have a single-

floppy system, the list will not include drive B:, as it merely

references drive A:. If your system has at least 20 logical drives,

Snooper will list them in two rows.

Label

The volume label of the current drive. You can change the label with

the LABEL program, supplied with DOS.

Directory

Shows the default directory of the selected disk. If the first

character shown is a plus sign, Snooper has omitted the first part of

the directory to make it fit.

IDE Drive Model/SCSI Host Adapter

Snooper can report the model of most IDE hard drives. Sometimes you

can see the model, sometimes the brand. This report can save you the

trouble of opening the computer to check the drive model. Recall that

Snooper skips detection of the IDE model under Windows 386 Enhanced

mode.

Snooper also can recognize several Adaptec SCSI host adapters, and

will report the model number in this report. It also will report the

slot the card resides in if it's an EISA adapter.

NOTE: The next reports (Drive Type through Cylinders), if the drive is

a floppy drive, concern the disk drive itself, not the diskette in it.

For example, if you have a 1.2M floppy disk drive and you have a

double density (360K) diskette in the drive, Snooper will report 15

Sectors Per Cylinder. That's because high density drives can handle

disks with 15 sectors per cylinder, although 360K disks have only

nine.

The report works this way so you can tell if the drive is high

density without needing a high density diskette to be in it. You can

determine the total capacity of a diskette (and thus its type) from

the Total Space report.

38    Snooper, the System Information Program for You!

Drive Type

Reports what type of disk drive you're examining. Snooper usually

bases this report on a BIOS report for the drive, but it also uses

other methods. Snooper can detect the following capacities for 5.25"

drives: 1.2M, 360K, 320K, 180K, and 160K. For 3.5" drives, it can

detect: 2.8M, 1.4M, and 720K drives. Other possibilities are: "fixed

disk" (probably a hard disk), "CD-ROM" (if it's local), "RAM disk,"

"Bernoulli," or "ID: nnh" (meaning Snooper doesn't recognize the disk

type and shows the actual disk type byte instead).

CMOS Type

Snooper queries the CMOS to find out what hard drive type (expressed

as a number), or what size and capacity floppy drive is installed.

Status

Shows if the disk is being compressed by DoubleSpace (available with

some recent version of MS-DOS) or Stacker. If Stacker is present, its

version is reported. Other reports are "local" and "network." If the

DOS commands SUBST or JOIN are affecting the disk, they are reported.

Heads

The number of heads a disk drive has. This number is often the same as

the number of sides the disk has. Most floppy drives will report two;

hard drives usually report several.

Sectors/cylinder

The number of sectors per cylinder on the disk drive. Sectors are "pie

slices" of the disk; cylinders are concentric rings. Normally, floppy

drives report 9, 15, 18, or 36; hard drives, 17, 26, 34, or more.

Cylinders

The number of cylinders on a disk drive. Cylinders are concentric

rings on the disk. Typically, floppy drives report 40 or 80; hard

drives, a few hundred to over one thousand.

NOTE: Original PCs and some XTs can't report heads, sectors/cylinder,

and cylinders. Neither can simulated disk drives (that is, logical but

not physical drives, such as RAM drives). If they can't, Snooper will

usually leave the appropriate areas blank.

Copyright 1989-1996 Vias & Associates    39

For the following disk reports, if the drive is empty, if the disk is

defective or unformatted, or the drive door isn't closed, Snooper will

report "Drive not ready."

Sector Size

The number of bytes stored in each sector. This number is almost

always 512.

Cluster Size

The number of bytes in each cluster. Recent DOS versions call clusters

"allocation units" (as brevity takes another slap in the face). A

cluster is the smallest space a file can take. Thus, even if the "DIR"

command reports a file is only three bytes, if the disk's cluster size

is 2048 bytes, the file will take 2048 bytes of disk space. A floppy

disk may report 1024, an XT's hard disk usually 8192, an AT's hard

disk from 2048 to 32,768, and a CD-ROM, 2048.

Total Space

The capacity of the disk in bytes. This includes all bytes, even if

the FORMAT program has marked some areas unusable. Snooper will even

report accurately the total data recorded on a CD-ROM, which virtually

no other system information program can do.

Free Space

The number of bytes still available for use.

Used Space (bytes)

The number of bytes in use by files, subdirectories, and any areas

marked unusable. One use of this report is computing how many floppy

disks you will need to back up your hard disk: floppy disks needed =

used space / floppy disk capacity + 1.

Used Space (%)

The percentage of disk space being used. Even on an empty disk, this

number may not be zero because of bad sectors or empty subdirectories.

Used Space (graph)

Provides an easily absorbed way to see how much disk space is in use.

The Used Space graph is one of Snooper's handiest features and

provides a quick way to monitor disk use.

40    Snooper, the System Information Program for You!

DIAGNOSTICS SCREEN

This screen, accessed via <Alt-D>, shows you details about your

computer's hardware status. It can help you diagnose interrupt

conflicts and other problems. If you have a multitasker or network

running, Snooper will pop up a warning and a prompt to continue before

letting you into this screen. The reason is these routines may disrupt

serial or parallel communications going on in other tasks. For

example, if you were transferring a file in one window and Snooper is

running in another and you tried to access the Diagnostics screen,

Snooper would likely disrupt your transfer or even hang up, forcing

you to start over. None of us would want that.

Also, networks and multitaskers sometimes can interfere with

Snooper's ability to gather accurate information. Unload the network

or multitasker and you may view the Diagnostics screen safely. In

fact, this screen is most accurate when you boot your computer from

plain DOS: no TSRs, no network. A mouse driver is OK.

If you try the Diagnostics screen and your computer locks up,

simply reboot with minimum CONFIG.SYS and AUTOEXEC.BAT files and it

should work fine.

Serial Ports

Address

Here Snooper ensures that an actual serial port exists by testing the

serial port chip (see UART, below). It doesn't just rely on a likely

port address; it makes sure the address points to a working port. If

fewer ports appear on this screen than on the main one, you may have a

faulty port.

UART

The Universal Asynchronous Receiver/Transmitter chip handles the

receiving and transmitting of data through the serial port. Snooper

detects which model is in each serial port in your computer (you may

have more than one kind). It detects the 8250 (found in most XTs),

16450 (found in many ATs), 16550 (found in early PS/2 models 50, 60,

and 80), 16550A (the minimum required for high-speed communications),

and Type 3 (supports DMA, found in latter PS/2s and others).

The last two chips add 16-byte first in, first out buffers (FIFOs)

to store characters being sent or received from the serial port. These

are necessary for high-speed communications at 9600 bits-per-second or

faster. The 16550 had defective FIFOs, rendering the buffers useless.

The problem was corrected in the 16550A. Internal modems provide their

own serial ports, so if yours is internal, Snooper will tell you what

UART is supplied by the modem itself.

Snooper displays an asterisk next to the UART if the FIFO buffers

are open, a condition that may be harmless but normally shouldn't

occur.

Copyright 1989-1996 Vias & Associates    41

Speed

Shows the speed at which the serial port has been initialized. This

speed bears no relation to the speed of a modem that may be attached.

Format

Data bits: The number of bits (per byte) the port treats as a

character. It can be 5, 7, or 8, with 8 being the most common.

Transmitting 7 data bits means 7 of the 8 bits will be treated as a

character; the eighth is ignored or treated as a parity bit.

Parity: Parity provides a crude method of error-detection but is

largely ignored today. Nearly all BBSs use No parity. Other reports

are: Even, Odd, Mark, and Space.

Stop bits: The stop bit or bits are sent after each character.

NOTE: Although the values of the above parameters are important, what

is crucial is that they be identical on both ends of the

communications link (your modem and the one you're calling).

Otherwise, communications can't occur. This is a common source of

frustration among modem users.

IRQ

Snooper performs a test to determine what IRQ (see Interrupt Request

Lines, below) each serial port would use. These are not just the

default values, but the actual IRQs the port would use. If software

(e.g., a mouse driver) configured the port to respond with an

interrupt upon, for example, receipt of data, Snooper displays the IRQ

number alone. If no interrupt would be triggered, Snooper displays the

IRQ number in parentheses. This just means no program is loaded that

uses the port. Occasionally, a port becomes dissociated from its usual

IRQ, and Snooper can't tell what IRQ that port would use. In that

case, Snooper leaves the IRQ report blank.

Device

Modems and mice are two common peripherals attached to serial ports.

Snooper can usually detect the presence of Hayes-compatible modems and

fax/modems, and fully Microsoft-compatible mice, and report which is

connected to each port. It also can sometimes show if both are

connected to one port (two ports sharing one address). If the modem is

external, it must be turned on for this report to work. A mouse driver

must be loaded for mouse detection to work.

Fax Info

If Snooper finds a fax/modem, it asks it what classes it supports.

Classes are specifications that define what capabilities a fax/modem

has. Class zero is simply a data modem. Class one adds fax

Snooper, the System Information Program for You!       42

capabilities. Classes are shown as a series of digits under the

appropriate port, except class zero, which is assumed (e.g., "1,2").

Snooper also will check for the presence of two fax interfaces, the

Communicating Applications Specification (CAS) and FaxBIOS. If either

is found, Snooper will show its name and version on the line below Fax

Info.

Parallel ports

Address

The same as the addresses on Snooper's Main screen.

IRQ

Snooper performs a test to determine what IRQ (see Interrupt Request

Lines, below) each parallel port would use. Again, these are the

actual IRQs the port would use, not just defaults. If software (e.g.,

a print spooler) configured the port to respond with an interrupt

when, for example, the printer is ready, Snooper displays the IRQ

number alone. If no interrupt would be triggered, Snooper displays the

IRQ number in parentheses.

Some parallel ports require a loopback plug to be inserted to

respond to Snooper's IRQ test. A loopback plug connects two pins in

the parallel port and allows the IRQ to be detected. If you find

Snooper can't detect the parallel port IRQs on your system, get a

parallel port loopback plug (you can find them at electronics parts

stores) and try it.

Occasionally, a port becomes dissociated from its IRQ, and Snooper

can't tell what IRQ that port would use. Also, some parallel ports

don't generate interrupts correctly. In these cases, Snooper leaves

the IRQ report blank.

NOTE: Your printer does not need to be on, or even attached, for this

report to work. You may hear it make a noise when Snooper checks the

port's IRQ. This is normal.

Status

Snooper also indicates the status of the parallel ports. The error

conditions change with different printers, so little can be said about

what each line really indicates. The only universal is your printer is

ready to print only when the Selected report alone is active.

NOTE: To help you debug a printer problem quickly, the parallel port

status is constantly monitored, so if, for example, you turn on your

printer or press its On-line button, the status indicators will

change. However, you should never plug in or unplug any peripheral

until you have turned off power both to the system and peripheral.

Copyright 1989-1996 Vias & Associates    43

Sound Card Address

Displays the sound card that appears on Snooper's Main screen, and may

show the port address to aid in diagnosing port conflicts.

For the IRQ and DMA reports which follow, "Available" appears on the

screen if the resource seems to be unused, "In use" if it has been

reserved by a hardware device.

Interrupt Request (IRQ)

On the right of the Diagnostics screen, Snooper shows which interrupt

request (IRQ) lines are currently active (indicated by the asterisks).

The PC had eight IRQ lines, the AT has fifteen. These lines are used

to get the CPU's attention when a hardware device needs servicing.

Some devices have assigned IRQs. For example, COM1 is normally set to

trigger IRQ4. Snooper displays these default assignments, although

they may not coincide with your system's, which may be configured

differently.

A device doesn't necessarily permanently reserve the IRQ it uses.

For example, you may have a scanner that uses IRQ 5, but only when

you're scanning. Snooper has no way to know this because you're not

likely to be scanning while Snooper is running. On the other hand, you

may be trying to add a device that also doesn't permanently reserve

its IRQ, and will never be used when the scanner is being used. Then

it's probably all right to assign IRQ 5 to the new device. Thus, you

can use Snooper's IRQ list as a starting point in determining which

IRQs are safe to use.

If a mouse driver is loaded, "Mouse" appears in the IRQ list,

indicating which IRQ your mouse is using. If your driver is new

enough, Snooper also will tell you if the driver is a TSR loaded in

your AUTOEXEC.BAT file (Snooper will display "MOUSE.COM"), or a device

driver loaded in your CONFIG.SYS file ("MOUSE.SYS").

Direct Memory Access (DMA)

Next to the IRQ report is the Direct Memory Access report. DMA

channels are used to transfer data without the aid of the CPU,

speeding transfers. The PC had four DMA channels; AT-class computers

have seven.

Snooper tries to show which DMA channels may be used by hardware

devices in your computer, but because of the PC's design, there is no

way to tell for sure. Even so, Snooper's method is sometimes close

enough. If, however, all DMA channels in your system seem to be in

use, you may have to disregard Snooper's DMA report. Snooper will

alert you to this case.

Snooper, the System Information Program for You!       44

BUS SCREEN

With the Micro Channel bus architecture comes the ability to detect

adapter cards by name. Snooper can recognize over 1,100 cards.

Snooper will list slots zero through nine, and identify what cards

are in which slots. A slot with no corresponding text is empty.

NOTE: If you see a message similar to: "Unknown card, please contact

author," please make a note of the four-digit number and the actual

card installed (your Reference Diskette should tell you), and let us

know. We'll add it to our Micro Channel adapter database.

NETWORK SCREEN

Press <Alt-N> when you're running Snooper on a Novell Netware network

(we hope to add other network types), and the Network screen will show

you details about your network configuration. See your Netware

documentation for more details about items Snooper reports on.

Network Type

This will remind you of the network type from Snooper's Main screen.

Network Address

The network address uniquely identifies the part of the network you're

on.

Node Address

The node address uniquely identifies the workstation you're on. ARCnet

cards need to have their node addresses set at installation; Ethernet

cards come with a preset node address.

Socket

Users aren't typically aware of this low-level resource, but it may

come in handy in troubleshooting network installations.

User Name

The name with which the user logged into the default server. Note you

can log onto different servers with different names.

Copyright 1989-1996 Vias & Associates    45

Hardware

Network Card

Shows what network card is installed in your system if you have an ODI

driver loaded. You do not need to be logged into the network. Snooper

also shows hardware resources the network card is using: memory

addresses, IRQ lines, port addresses, and DMA channels.

Software

These reports show version numbers, levels, and interrupts in use by

drivers and other network software. Software listed in the second

column is running on the server.

Server

Default

The name of the default server.

Connection Number

The connection the workstation is using to connect to the default

server.

Connections

These reports show the maximum connections (users) allowed on the

default server, the number of users currently logged in, and the most

users connected at once since the server was last booted.

CMOS SCREEN

Every computer based on the PC-AT standard (i.e., all 286s and newer)

has a small area of memory called the CMOS (pronounced "SEE-moss"),

used to store configuration information. The letters in "CMOS" stand

for the material the configuration chip is made of (Complementary

Metal Oxide Semiconductor, if that helps).

CMOS memory is non-volatile, meaning its contents are retained even

when the computer is turned off. This is accomplished with a battery

that constantly supplies power to the CMOS chip.

Each time the computer is booted, it checks that the CMOS

configuration accurately reflects the actual configuration. If not,

the computer warns you the configuration has changed. This may occur

because the CMOS battery is dead, or (more rarely) a renegade program

has wiped out the CMOS. After replacing the failing battery with a

fresh one, you are likely required to change the CMOS information to

reflect the actual configuration. Snooper's CMOS screen lets you do

just that.

46    Snooper, the System Information Program for You!

Older computers required a setup program that came on a floppy disk

with the computer. Many users misplace this disk or never received

one. These users especially will find Snooper's CMOS screen

invaluable. But most computers now come with a built-in setup

facility, accessible with a keystroke when the system boots. So why

duplicate that functionality in Snooper?

First, Snooper's Setup screen is more friendly. Some built-in setup

screens, especially older ones, are user-hostile. For example, you may

have to choose a hard drive type without knowing the parameters for

that type. You would have to hunt down the computer's documentation,

and hope the drive table was included and accurate. Snooper shows you

the parameters for the drive type you are about to choose, and you can

easily view each drive type to select the most appropriate one. Also,

some built-in setup programs don't let you abandon your changes if you

want to. Snooper does.

Second, Snooper provides a fast, attractive, consistent way for

technicians to edit CMOS data without having to learn each setup

screen's user interface. You can use Snooper's screen with point-and-

shoot ease, or press a single letter to quickly access the intended

option. Some built-in setups make you change or confirm all the

options in sequence, instead of letting you choose just the one you

want to change.

Third, you can log the CMOS data, along with Snooper's other

screens, into a file for later reference. You can't do that with

built-in setup programs.

Navigating the CMOS Screen

To move around this screen, press the highlighted letter for the

option you want. Or use <Tab> and <Shift-Tab>, or the arrow keys, to

move the highlight bar to the desired option. Press <+> and <-> or

<PgUp> and <PgDn> to change the option.

WARNING: Be very careful when editing CMOS data. You can render your

system temporarily un-bootable by specifying the wrong drive type. If

you're not absolutely sure what you're doing, stop! Seek professional

help.

CMOS Status

Snooper will list up to seven errors that can occur with the CMOS.

Most often, you'll see "Incorrect configuration" when you've added or

removed a drive or some memory. "CMOS battery is dead" means you

probably should replace it. If there are no CMOS errors, Snooper

reports "OK."

Date and Time

These options change the date and time. To aid you in setting the

date, the day of the week is also displayed, but because it's not

stored in the CMOS, you can't set it directly.

Copyright 1989-1996 Vias & Associates    47

NOTE: Unlike all other options on this screen, changing the date and

time changes the CMOS date and time as you change them on-screen. So

even if you exit the CMOS screen without saving your other changes,

the CMOS date and time will still reflect any changes you made to

them.

Memory

Tells the computer how much conventional and extended memory is

present. These options can be changed in 128K increments.

Floppy Drives

Tells the computer the capacity and size of the installed floppy

drives. Choices range from the 360K floppy to the 2.88M.

Hard Drives

Here you are allowed to choose the hard drive types. Choose type zero

if you have no drive, or a SCSI drive. Otherwise, choose the type that

matches the parameters of the hard drive you have. "Pre-comp" is short

for Write Pre-Compensation, often abbreviated "WPC." "L-zone" means

Landing Zone, the cylinder the heads will come to rest on when the

system is turned off.

Snooper finds the drive parameters table in your computer's ROM

BIOS. Snooper can find the beginning of the table, but there's no

foolproof way to tell where it ends. In some computers, the table has

as few as a couple dozen entries; in others, a couple hundred. If you

see some strange drive types that don't make sense, you've gone past

the end of the table.

If you see "User-defined" after the drive type, it means Snooper

has run out of pre-defined types. The type you're looking at can be

defined by the user to fit a hard drive whose parameters don't appear

in the list. If you need to define a custom drive type, you can't use

Snooper to do so. BIOS makers haven't standardized on a way to store

user-defined drive parameters in the CMOS, so Snooper wouldn't know

where to put them. Instead, you must use the computer's own setup

program (often stored in the ROM BIOS and accessed via <Ctrl><Alt>-

<S>, <Ctrl><Alt>-<Enter>, or <Ctrl><Alt>-<Esc>, or with <Delete>,

<F1>, <F2>, or <F10> during boot-up).

NOTE: Some memory managers have a feature that provides more UMB space

by moving the ROM BIOS to another memory location. If this feature is

in use on your system, Snooper may not be able to find your computer's

hard drive table because the memory manager moved it. If so, disable

the memory manager's feature or use your computer's built-in setup

utility.

Snooper, the System Information Program for You!       48

Display

Here you indicate what kind of video adapter is installed. If you have

a text-only monochrome or a Hercules Graphics monochrome adapter,

select "monochrome." For CGA cards, you should choose "CGA 80 columns"

(the normal number of columns). If you have an EGA, VGA, SVGA, XGA, or

PGC card, choose "VGA and EGA."

Coprocessor

If your system has a math coprocessor, choose "Installed." Snooper's

Main screen will tell you if your system has a math chip (NDP).

Saving Your Changes

Press <Esc> when you are finished with the CMOS screen. If you have

made changes, Snooper will prompt you in the message box to save your

changes permanently into the CMOS chip. If you want to abandon your

changes, press <N>, or <Enter>. Or press <Y> to save them. For your

changes to take full effect, you may have to reboot your computer.

Snooper will do this for you, with your permission. If you pressed

<Y>, Snooper will then ask if you want it to reboot your system. If

so, press <Y> and your computer will reboot.

NOTE: If you reboot, and you are using a multitasker such as Microsoft

Windows, any unsaved work you may have been doing in another window

will be lost. Pause and think carefully before you decide to reboot.

BENCHMARK SCREEN

Snooper's Benchmark screen provides three speed indexes: CPU, Video,

and Hard drive throughput. They are displayed as numbers and bar

graphs.

The CPU benchmark is run continually when you're viewing this

screen. Since the other benchmarks may take several seconds to run,

they must be invoked by you by pressing a key. If you run benchmarks,

then return to the Main screen, only to return to the Benchmark

screen, the benchmark scores will reappear so you needn't run them

again. You may, of course, run them as many times as you like.

The scales for the bar graphs adjust automatically for different

CPUs. They also adjust themselves if the score would exceed the

default scale for that CPU (i.e., if the system were particularly fast

for its CPU class). The default scales and graphs are displayed in

green, in blue if the scales were adjusted to accommodate fast

hardware. Snooper's automatic scaling is effectively infinite, so you

needn't worry that the next generation of CPUs will exceed Snooper's

capabilities.

Copyright 1989-1996 Vias & Associates    49

CPU Throughput

This benchmark test is run continually, a fact you can demonstrate by

switching your computer in and out of turbo mode. You will see the

speed in Megahertz, and the benchmark score change. This test is an

estimate of the speed at which an AT (e.g., a 286 computer) would have

to run to be as fast as your computer. For example, if the CPU

throughput score is 150, an AT would have to run at 150 Megahertz to

keep up with your system.

Also, during normal operation, you may see a slight fluctuation in

speed as TSRs briefly gain control of your system (for example, a disk

cache writing data to a hard drive). You can hold down a key and see

the key repeat feature slow the system a bit. If you're running

Snooper under a multitasker such as Microsoft Windows, you may see

large fluctuations in speed as other programs gain control and Windows

performs housekeeping functions. All this speed changing is normal,

and doesn't indicate a problem.

Video Throughput

When you press the <V> key, Snooper performs a video benchmark test.

This figure shows how fast your computer can display text under DOS,

in thousands of characters per second. This figure is heavily reliant

on your system's CPU and video card. This may have little correlation

to how fast your system draws graphics, or how well it would perform

under Microsoft Windows. It measures DOS text speed. Of course, if

you're measuring the speed of a system to be used mostly in DOS, this

figure is quite relevant.

Disk Throughput

Press <D> to perform the hard drive benchmark on drive one (normally

C:). This test usually takes just a few seconds, or it may take a bit

longer, depending on the hard drive's speed.

Much ado is made of a hard drive's "average access time," or

"average seek time." This is a measure of how fast the drive's

read/write heads can move from a randomly selected cylinder of the

disk to another. It provides a rough indication of the drive's speed

at accessing requested data. Another commonly quoted benchmark is

"data throughput," a measure of how fast data can get from drive to

computer. Unfortunately, this test is often performed in an optimized

setting, reading the same data from the same place on the disk over

and over. But although each of these tests is helpful, neither

measures how fast a drive will perform the way you use it.

Snooper tries to approximate just that. It exercises the read/write

heads and reads data at each stop, providing a benchmark that combines

the best of both tests. Of course, other factors are important to

overall disk speed: CPU speed, disk caching, the disk controller being

on a local bus, and so on.

For the safety of your data, this test never writes to your drive,

it only reads, so there's no chance of it damaging your data.

If you have two hard drives, press <I> to test the second; its

score will replace the first's. At the left of the graph, a number

50    Snooper, the System Information Program for You!

will show which drive's speed is being shown. Note you must have two

separate hard drives, not just two partitions on the same drive (they

would have the same score anyway).

SETUP SCREEN

You may change Snooper's colors and other defaults by pressing <Alt-S>

from the Main screen. You will see the Setup screen options and a

"fake" display to show you what Snooper will look like with the colors

you specify.

Navigating the Setup Screen

To move directly to a field, press the highlighted letter for the

field you want. Or use <Tab> and <Shift-Tab> or the arrow keys to move

the highlight bar. Press <+> and <-> or <PgUp> and <PgDn> to change

the selected option. For the license number and filename options, type

the entry and press <Enter>.

Mono Mode

You can force Snooper to use its Mono mode by using this option.

Snooper detects monochrome cards automatically. But you may have a

reason to override the default. For example, your laptop  may have a

monochrome LCD display that emulates a VGA card (making Snooper think

you have a color monitor) but the display is more readable in

monochrome mode. Set this option to Yes and Snooper will use its mono

background color. The default is "Auto," meaning Snooper will try to

determine if Mono mode is necessary.

Desqview Mode

Again, Snooper normally knows when to use Desqview mode, but you may

have a reason to specify this option. For example, if you have an old

CGA card and you see "snow" when Snooper draws its display. The

default is "Auto," meaning Snooper decides if Desqview mode is needed.

Editor

This option tells Snooper if you want to use its internal editor or an

external one called EDIT. The default is "Internal."

Drive

You may want Snooper to give you information about a certain drive

first, without having to specify it on the command line. Perhaps

you're a systems administrator and although your users are running

Snooper from the server's driver, you want it to show information for

Copyright 1989-1996 Vias & Associates    51

their C: drive. Use the default disk drive option to specify it.

Snooper will cycle through all the letters of the alphabet and the

word "default." Of course, once you're in Snooper, you can change to

any drive in the system. If Snooper doesn't find the drive you

specified when it's run, it will show information for the current

drive. The default for this option is, of course, "Default."

Sounds

When you press a wrong key, try to select an invalid drive, or an

error occurs, Snooper usually beeps. This option is on by default but

you may turn off all sounds with this option.

Skip IDE

Some computers aren't compatible with Snooper's method of detecting an

IDE hard drive's model name. This option will cause Snooper to skip

detection of the IDE drive model. It corresponds to the /IDESkip

command line switch, and is off by default (i.e., it will try to get

the drive's model).

Other IRQ

Some computers aren't compatible with Snooper's method of detecting

port IRQs in the Diagnostics screen. This option will cause Snooper to

use an alternative method of detecting IRQs. The alternative method

isn't as accurate, however, so use this option, and its corresponding

command line switch (/OtherIRQ) only if necessary. It is off by

default.

Allow CMOS Editing

This hidden option allows you to toggle the function of the CMOS

screen from editing to simply viewing. If you're a network

administrator and you want to prevent non-technical users from editing

their CMOS configurations, simply press <A> once and save the

configuration file. Subsequent invocations of Snooper will provide a

CMOS viewer. The CMOS screen will look almost identical, and it can be

logged like all other screens, but no editing can be performed. To

revert to CMOS editing mode, return to the Setup screen and press <A>

once again. Don't forget to save your changes. This option is

invisible so users can't change the option to use the editor.

This topic does not appear in the table of contents or index so you

just need to be careful about letting them see this section of the

manual.

Colors

You can change Snooper's colors to suit your taste. Snooper will cycle

through the available colors. You can go through them in either order

Snooper, the System Information Program for You!       52

by using the <+> and <-> keys. Snooper's "fake" display will show you

how the real display will look. Remember, you must save your changes,

exit, then reinvoke Snooper for the changes to take effect.

License Number

To make that annoying Registration Reminder Screen (RRS) go away

forever, press <L> and enter the license number we sent you when you

registered. Be sure to enter the number exactly as it appears or

Snooper won't accept it (it's awfully picky). After you save your work

(see below) and exit, Snooper will no longer show its RRS. Your

license number is shown at the top of the order information screen

(from the Main screen, press <F1>, <O>).

NOTE: After you enter your license number, you should save your config

file under the default name, so it always will be available to

Snooper. Otherwise, the registration reminders might start popping up

again. Of course, it would be foolish to give your license number away

to anyone. They should pay, too!

Saving Your Changes

If you want to quit and not save your changes, press <Esc> and no new

configuration file will be written.

If you've made changes you want to keep, you must save them to a

configuration file. The default configuration filename is always

displayed. This is the file that was loaded when Snooper was invoked,

or would have been if it existed. To accept the default, simply press

<S>, then <Enter>. To specify a new file, type a new filename and

press <Enter>.

Snooper will write a config file with the new options, overwriting

any old file of the same name. To create multiple files, simply enter

different names at the prompt. Snooper will report if the file was

successfully saved, or show an error message if it wasn't. If the save

is successful, the next time you run Snooper, the new options will

take effect. That's all there is to it!

A LITTLE BIT ABOUT SNOOPER'S CREATION

Snooper was developed over thousands of hours, many of which we

probably should have spent asleep. Snooper began in February 1987 on

an 8 MHz AT compatible (yuck!), moved on to a 386SX-16 (ick!), and now

to a Pentium-90 (ahhh!). It was tested along the way on hundreds of

machines. It originally was written in Turbo Pascal version 3.01A, and

has since been expanded under every version through 6.0. Its source

code as of this writing consists of over 14,000 lines of Pascal,

including some BASM assembler.

Remember, the SNOOPER.HST file will list the changes that have been

made since Snooper's inception. When you get a new version, check this

file so you can take advantage of new features.

Copyright 1989-1996 Vias & Associates    53

A LITTLE BIT ABOUT SNOOPER'S AUTHOR

We'll take the liberty of assuming some of you want to know a little

more about Snooper's author. We'll keep it short. The following

paragraph was written by John Vias, Snooper's principal author.

"I'm originally from Evanston, Illinois (suburban Chicago), but

have lived most of my life in Florida. At the University of Florida, I

earned a degree in English, a fact I hope is reflected in this manual.

I now own a computer consulting, programming, and technical writing

business called Vias and Associates (pretty catchy, huh?). Finally, in

August 1994, Vias and Associates was moved to San Francisco (it's the

City by the Bay, you know), fulfilling a longtime dream of mine to

live in the Bay Area. Our eternal gratitude to all Snooper users for

your continued support."

ACKNOWLEDGMENTS

Snooper was not created in a vacuum, of course, although John's head

has been likened to one. Here are a few sources of information we

used, and for which we are very grateful. We apologize to those we

forgot to mention. Without them, Snooper would be only . . . well, we

hate to think about it.

PC Magazine (by Ziff-Davis Publishing Company)

An invaluable source of information about the PC family. A great

deal can be learned from its pages. It has helped many a

programmer.

Turbo Pascal 6.0: Techniques and Utilities (by Neil J. Rubenking:

Ziff-Davis Press, 1991)

With the help of this book, Snooper's code grows better and more

efficient by following Mr. Rubenking's advice. Highly recommended

for anyone who wants to write Turbo Pascal programs.

Advanced MS-DOS Programming (by Ray Duncan: MS Press, sec. ed., 1989)

Written by one of PC Magazine's stable of PC wizards, this book was

our source for documented BIOS and DOS calls.

Programmer's Guide to the IBM PC (by Peter Norton: MS Press, 1985)

The Programmer's Guide was a handy reference in Snooper's early

years.

Ralf Brown (INTERvvx.ZIP)

(The "vv" is the volume of release and the "x" is "A," "B," or "C."

The list comes in three files.) An enormous list of DOS, BIOS, and

program-specific interrupts, many otherwise undocumented. Quite

simply, some of Snooper's reports wouldn't exist without it.

Invaluable in that it reports bugs and incompatibilities books

rarely mention. And it's free! Check out the book Undocumented DOS,

cowritten by Ralf.

Snooper, the System Information Program for You!       54

Andrew Rossman (InfoPlus)

Mr. Rossman used to maintain a multi-page system information

utility. From it we got snippets of code and programming

techniques.

Peter Volpa (Circuit Systems)

Peter helped us with some disk routines. He's the author of CD-

QUICK, an award-winning CD-ROM cache.

John Fox (EDIT2.PAS)

John wrote the code Snooper uses in its internal editor. Thanks,

John!

Yuval Tal (TP6XMS.ZIP)

Author of a Turbo Pascal 6.0 unit for using an XMM.

Borland International, Inc. (Turbo Pascal)

Thanks and hats off to Borland for creating Turbo Pascal.

Greg Barton (sysop, ACCUG BBS)

Special thanks to Greg Barton, who kindly set up a Snooper

conference on his BBS, and a special menu to make it easy for you

to get the newest version. ACCUG info is on the first page of this

manual.

Greg Wrey (former sysop, TimeSlice BBS)

Thanks to Greg Wrey, who was sysop of Snooper's original support

BBS, and who beta-tested Snooper on his systems many times.

Our friends (courtesy of their parents)

With more patience than we deserved, they helped debug Snooper by

running it on their PCs and reporting what happened before it

crashed.

All those we forgot to mention

Those folks who left E-mail or wrote letters, reporting we forgot

something or shouldn't this be such-and-such.

And most of all, Snooper's Registered Users

Without the encouragement and support of Snooper's registered

users, John would have given up in disgust and moved to a tiny

cottage in the hills, living off the land and forever writing

programs for his own amusement. Occasionally, when registrations

are slow, he still threatens to--so please keep those registrations

coming!

Copyright 1989-1996 Vias & Associates    55

INDEX

navigating, 37

A20 line status, 23                    overview, 36

Acknowledgments, 42                   Colors, setting, 41

Address                               Command line syntax, 11

parallel ports, 34                  CompuServe, 1, 2

serial ports, 32                    Computer type, 21

sound cards, 34                     Config files

Address, Vias and Associates, 1        environment variable, 16

Advanced Power Management. See         loading, 16

APM                                 CONFIG.SYS

APM, 21                                editing, 19

ASP, 2                                 logging, 14, 19

Association of Shareware              Configuring Snooper, 39

Professionals.     ASP                        See                  Connection, 36

AUTOEXEC.BAT                          Connections, 36

editing, 19                         Conventional memory

logging, 14, 19                      free, 22

total, 22

BBS                                    used, 23

PC-Ohio, 1                          Copyright, 9

Benchmark screen, 38                  COUNTRY command, 17

Benchmarks                            CPU

CPU, 39                              benchmark, 39

disk, 39                             model, 21

video, 39                            speed (MHz), 21

Break, 29                              virtual-86 mode, 22

Buffers, 29                           CPU speed, 21

Bugs                                  Current directory, 30

disclaimer, 20                      Customization, 2

known, 20                           Cylinders, 31

Bus, 22

Bus screen, 35                        Date

CMOS, 37

Cache, 29                              current, 17

CAS, 33                                ROM BIOS, 26

CD-ROM, 30, 31                        Desqview mode.     Setup screen                                                            See

Central Processing Unit.     CPU                                See           or Switches

Clipboard, logging to, 19             Diagnostics screen, 32

Cluster size, 31                      Direct Memory Access. See DMA

CMOS                                  Directory, 30

conventional memory, 37             Disks. See also Drives

coprocessor, 38                      benchmark, 39

date/time, 37                        cache, 29

definition, 36                       cluster size, 31

display, 38                          compression, 31

extended memory, 37                  directory, 30

floppy drives, 37                    free space, 32

hard drives, 38                      label, 30

saving changes to, 38                local/remote, 31

status, 37                           sector size, 31

CMOS drive type, 30                    sectors/cylinder, 31

CMOS screen                            status, 31

caveat, 37                           total space, 31

used space, 32                      FaxBIOS, 33

Distributing Snooper with your        Files in the Snooper package, 7

products, 6                         Files, editing, 18

DMA                                   FILES=, 29

list, 35                            Floating Point Unit. See FPU

overview, 35                        Floppy drives, 24

DOS                                   FPU, 22

brand, 28                           Free space (disk), 32

break, 29

BUFFERS=, 29                        Game port, 26

FILES=, 29

location, 28                        Hard drives, 24

revision, 28                        Hardware requirements, to run

shell, 28                            Snooper, 7

verify, 29                          Heads, 31

version, 28                         High Memory Area.     HMA                                                               See

DOS installation, 7                   HMA, 23

Drives. See also Disks

CMOS type, 30                       IDE drive model, 30

cylinders, 31                       Installation

floppy, 24                           DOS, 7

hard, 24                             Windows, 8

heads, 31                           Internet, 1

IDE drive model, 30                 IRQs

list, 30                             list, 34

logical, 24                          parallel port, 34

physical, 24                         serial port, 33

SCSI host adapter, 30               ISA bus, 22

type, 30

Joystick adapter.     Game port                                                               See

EDIT.BAT, 18

Editing                               Keyboard

AUTOEXEC.BAT, 19                     support for, 27

CONFIG.SYS, 19                       type, 27

editor setup, 40                    Keys

overview, 18                         AUTOEXEC.BAT editing, 19

EISA bus, 22                           CONFIG.SYS editing, 19

EMS                                    disk drive, 18

EMM level, 24                        exit, 17

free, 24                             help, 18

overview, 24                         log system files, 19

total, 24                            log to printer, 19

Environment                            log to Windows clipboard, 19

definition, 28                       logging, 19

free space, 28                       ordering, 18

total space, 28                      overview, 17

ERRORLEVEL, 15, 17, 20                 registering, 18

Expanded Memory Specification.         SNOOPER.LOG editing, 19

See EMS

Extended memory                       Label (disk), 30

free, 23                            Landing zone, 38

total, 23                           Legal stuff, 9

Extensions, 27                        Liability, 9

License, 9

Fax/modems, 33                        License number

Copyright 1989-1996 Vias & Associates    57

saving, 41                           server, 36

using discretion with, 41            socket, 36

Logging                                software, 36

Auto-Logging, 13                     type, 29, 35

Auto-Logging syntax examples,        user name, 36

14                                 Network screen, 35

character translation, 13           Node address, 35

DOS system files, 14                Novell, 35

environment variable, 13            Numeric Data Processor. See NDP

keys, 19

log filename parameter, 13          Operation

log to printer key, 19               under DOS, 17

overview, 12                         under Windows, 17

quietly, 15                         Ordering. See Registering

system files, 19                    Other IRQ.     Setup screen                                                         See              or

to Windows clipboard, 19             Switches

Windows system files, 14

Logical drives, 24                    Parallel ports

Loopback plug, 34                      address, 26, 34

IRQ, 34

Main screen, 21                        status, 34

Math coprocessor, 22                  Philosophy of Snooper, 8

MCA                                   Phone numbers, 1

bus, 22                             Physical drives, 24

cards, 35                           PIF file, 8

Memory.     Conventional, EMS,               See                           Planning to register soon?, 11

Extended, HMA, UMB, Video, and      Please register now, 11

XMS                                     -N                                             Plug  -Play, 26

Mice.     Mouse             See                             Ports.     Game port; Mouse                                                    See

Micro Channel Architecture. See        port; Parallel ports; and

MCA                                  Serial ports

Microprocessor.     CPU                       See                   Pricing, 11

Modem, on serial port, 33             Printer ports. See Parallel

Monitor type, 25                       ports

Mono mode. See Setup screen or

Switches                            Registering, 10

Mouse                                  benefits of, 10

brand, 27                            by check or money order, 11

driver version, 27                   by CompuServe, 11

on serial port, 33                   by phone, 11

port, 27                             by purchase order, 11

Mouse port, 27                         definition, 10

MOUSE.COM, 35                          how to, 11

MOUSE.SYS, 35                          reasons to, 10

Relocatable Screen Interface

NDP, 22                                Specification, 13

Netware, 35                           Requirements to run Snooper, 7

Network                               ROM BIOS

address, 35                          brand, 26

card (NIC), 36                       date, 26

connection number, 36                extensions, 27

connections, 36

default server, 36                  San Francisco, 2, 42

hardware resources, 36              Screens

node address, 35                     Benchmark, 38

CMOS, 36                             batch mode, 12

Diagnostics, 32                      chip detection bypass, 15

Main, 21                             config file, 16

Network, 35                          config file env variable, 16

Setup, 39                            Desqview mode, 12

SCSI host adapter, 30                  disk drive, 15

Sector size, 31                        help, 12

Sectors/cylinder, 31                   IDE skip, 15

Serial ports                           log DOS system files, 14

address, 26, 32                      log filename, 13

data bits, 33                        log Windows system files, 14

device, 33                           mono mode, 12

fax, 33                              other IRQ, 15

IRQ, 33                              quiet mode, 15

parameters, 33                       tune skip, 15

parity, 33                          Syntax

speed, 33                            diagram, 12

stop bits, 33                        examples, 16

UART, 32                            Syntax (See also Switches), 11

Server, 36                            SYSTEM.INI, logging, 14, 19

Setup screen

colors, 41                          Technical writing, 2

Desqview mode, 40                   Telephone numbers, 1

drive, 40                           Time

editor, 40                           CMOS, 37

license number, 41                   current, 17

mono mode, 40                       Total space (disk), 31

navigating, 40                      Trademarks, 9

other IRQ, 41

overview, 39                        UMB, 24

saving changes to, 41               University of Florida (Go

skip IDE, 40                         Gators!), 42

sounds, 40                          Upgrading

Shareware, 10                          how to, 10

Shell, 28                              procedure, 8

Shouldn't you register now?, 11       Upper Memory Block.     UMB                                                                 See

Skip IDE. See Setup screen or         Used space (disk), 32

Switches                            User name, 36

SNOOPCFG, 16                          User-defined hard drive types,

Snooper                                38

capsule bio, 41

Snooper's author                      Verify, 29

capsule bio, 42                     VESA, 25

SNOOPER.LOG                           VGA, 25

editing, 19                         Vias and Associates, 2, 42

SNOOPLOG, 13                          Video, 25

Snow, 13, 40                           benchmark, 39

Socket, 36                             memory, 25

Sound cards                            monitor type, 25

address, 34                          type, 25

model, 26                            VESA, 25

Sounds, 40                             VGA card model, 25

Status, parallel port, 34             Virtual-86 mode, 22

Switches

Auto-Logging, 13                    Warranty, 9

Copyright 1989-1996 Vias & Associates    59

WIN.INI, logging, 14, 19

Windows

installation, 8

Snooper's operation under, 17

Windows clipboard, logging to,

19

Wishes, 20

WPC, 38

Write Pre-Compensation. See WPC

XMS memory

free, 23

version, 23