AWS2703: 2703/2540 Emulator

===========================

AWS2703 allows devices to connect to the P/390 via the asynchronous port;

i.e.  via the COM ports.  Currently supported devices are Modems and the

2540 card punch.

When a modem is attached to the asynchronous port the AWS2703 device manager

acts as a 2703 Control Unit.  This allows remote users to connect to the

P/390 via an ASCII terminal.

DEVICE TYPE: 2703, ICA, 2540P

============

CONFIGURING a 2703 or 2540

==========================

For each 2703 or 2540 device, you need to specify the parameters governing its

communications.  To do this, go to the P/390 Configurator UPDATE SYSTEM DEVICES

screen.  Enter the address and the manager number.  Then in the FN/P field,

enter the following parameters:

devname:baud,parity,data,stop,flow,dcd

Default Values:

COM1:1200,N,7,2,OFF,DCD

where

devname  - Name of the device to be associated with this port

For example: COM1 or COM2

baud     - The line speed for the port.  This can be any standard

rate between 110-19200 baud.

parity   - N(one), E(ven), O(dd), M(ark), S(pace) are the supported values.

(specify only the first character of each value)

data     - 7 or 8 (number of data bits)

stop     - 0 or 1 (number of stop bits)

flow     - OFF : No flow control

XON : XON/XOFF flow control

DTR : Hardware (DTR/DSR) handshaking

dcd      - DCD : A separate thread will be started to monitor the CD

signal to detect when it has been dropped.

OFF : The CD thread will not be started; if CD is dropped, the

OS/2 COM driver will insert a break character (0xFF) into

the input stream.

VM REQUIREMENTS

===============

A CP nucleus generated with 2703 TELE2 lines and/or 2540 punches.

Sample DMKRIO entries:

PUN      RDEVICE ADDRESS=00D,DEVTYPE=2540P,CLASS=(A)

RDEVICE ADDRESS=(024,8),DEVTYPE=2703,ADAPTER=TELE2

RCTLUNIT ADDRESS=020,CUTYPE=2703

Note : An ICA TELE2 line will be recognized and treated as a 2703 TELE2 line

The punch emulation is provided primarily for driving plotters.  Plot

files are often punched to RSCS or other virtual machines which drive a

plotter.  On a P/390, the extra virtual machine may not be necessary.

If the plot file is generated as 80 byte records and contains all the

necessary data for the plot, you can define a real CP PUNCH in your VM

nucleus and have the 2703 emulator respond to VM like a punch, reading

the plot (punch) records and driving the plotter.  This method offers an

easy way to support attended plotter operations.  It is not recommended

for unattended operation because little in the way of recovery or error

reporting is available.

INSTALLATION AND DEBUG INFORMATION

==================================

The device manager is started in the IPL.CMD file.

Optional Device Manager Parameters

----------------------------------

These parameters should be specified using the P/390 Configurator "Update

System Devices" menu, "Device Manager Parm Edit/Display" submenu (F2/F12).

The parms MUST be in double quotes to prevent OS/2 from splitting the line

if an '&' character is used.  These parms include:

/I=init_string   Use this to specify an alternate Modem Init string.  The

The default init string is "ATE0Q1M1S0=1"

Example:  'AWSSTART AWS2703.EXE n "/I=ATE0Q1M1S1=2"'

This will override the default INIT string and

set up the COM port to answer on the second ring

instead of the first ring.

/IO              This will create a file called AWS2703.DAT into which

all outbound and inbound data streams will be recorded.

For this option to be enabled, I/O tracing for the specified

2703 device address must also be enabled. Use MANOPS Device

Debug menu to enable I/O tracing.

MISCELLANEOUS INFORMATION

=========================

Only one copy of this program should be running at any time.  It supports

multiple ports simultaneously, including mixed device types.

Interface information:

In order to recognize a device as being online, the carrier-detect line is

monitored.  If the carrier drops, an intervention required error is sent to CP.

When a device is disabled by VM, DTR on the port is dropped.  It is expected

that any attached devices (modems, etc) respond properly (ie., hang up the

phone, not allowing a connection to be made until DTR is once again raised.

Modems:

If you are attaching a 'smart' modem as a device, it is recommended that

result codes be disabled (ATQ1 on Hayes-compatibles) and autoanswer be

enabled (ATS0=1).  If everyone plays fair, the modem will not autoanswer

when DTR is down.

TRANSLATE TABLES UNDER VM

=========================

If the AWS2703 device is owned by the CP, it is considered a TTY device.  As

such the CP will translate outbound and inbound data, via Module DMKTTX, as

follows:

To translate ASCII --> EBCDIC InBound from the Device:

1 Find the Hex representation of the character.  ( > is '3E' in ASCII )

2 Then the bit representation                    ( 0011 1110 )

3 Reverse the order of the bits                  ( 0111 1100 )

4 Find the Hex representation of the bits        (  7    C   )

5 Use the hex representation as entry into the

TTY USASCII-8 LTC -> EBCDIC Translation Table

* The first byte is the vertical component     (  7        )

* The second byte is the horizontal component  (       D   )

* The intersection of 7 and D is 0x6E          ( '6E' is > in EBCDIC )

NOTE: The TTY-->EBCDIC translation table is set up such that the same EBCDIC

value will result regardless of the high order ASCII bit being 1 or 0.

To translate EBCDIC --> ASCII OutBound from the Device:

1 Find the Hex representation of the bits        ( '6E' is > in EBCDIC )

2 Use the hex representation as entry into the

EBCDIC --> TTY USASCII-8 LTC Translation Table

* The first byte is the vertical component     (  6        )

* The second byte is the horizontal component  (       E   )

* The intersection of 6 and E is 0x7D          (  7    D   )

3 Then the bit representation                    ( 0111 1101 )

4 Reverse the order of the bits                  ( 1011 1110 )

5 Convert back to HEX                            ( '3E' is > in ASCII )

NOTE: Notice that high order ASCII bit is still active.  This is the

'parity' bit.  The EBCDIC-->TTY translation table stores all target

ASCII values with MARK Parity (assuming 7 data bits).  It actually

stores the parity bit in as part of the 'reversed-ASCII' value.

This is why the AWS2703 defaults all COM ports to Noparity and

7 data bits; i.e. to strip off the parity bit before transmission.

The tables used for this translation are located in module DMKTBN and are

as follows:

*

*        EBCDIC -> TTY UASCII-8 LEVEL TERMINAL CODE (1977)

*

DMKTBNEA EQU   *

*                 0 1 2 3 4 5 6 7 8 9 A B C D E F

DC    X'018141C1FF91FFFFFFFFFFD131B171F1' 00

DC    X'098949C9FF5111011999FFFF39B979F9' 10

DC    X'FFFFFFFFFF51E9D9FFFFFFFFFFA161E1' 20

DC    X'FFFF69FFFFFFFF21FFFFFFFF29A9FF59' 30

DC    X'05FFFFFFFFFFFFFFFFFFFF753D15D53F' 40

DC    X'65FFFFFFFFFFFFFFFFFF85255595DD7B' 50

DC    X'B5F5FFFFFFFFFFFFFFFF3F35A5FB7DFD' 60

DC    X'FFFFFFFFFFFFFFFFFF075DC503E5BD45' 70

DC    X'FF8747C727A767E71797FFFFFFFFFFFF' 80

DC    X'FF57D737B777F70F8F4FFFFFFFFFFFFF' 90

DC    X'FF7FCF2FAF6FEF1F9F5FFFFFFFDBFFFF' A0

DC    X'FFFFFFFFFFFFFFFFFFFFFFFFFFBBFFFF' B0

DC    X'DE8343C323A363E31393FFFFFFFFFFFF' C0

DC    X'BF53D333B373F30B8B4BFFFFFFFFFFFF' D0

DC    X'3BFFCB2BAB6BEB1B9B5BFFFFFFFFFFFF' E0

DC    X'0D8D4DCD2DAD6DED1D9DFFFFFFFFFFFF' F0

*                 0 1 2 3 4 5 6 7 8 9 A B C D E F

EJECT

*

*        TTY UASCII-8 LEVEL TERMINAL CODE -> EBCDIC (1977)

*

DMKTBNAE EQU   *

*                 0 1 2 3 4 5 6 7 8 9 A B C D E F

DC    X'00007C7C404079791010D7D7F0F09797' 00

DC    X'1616C8C84D4D88881818E7E7F8F8A7A7' 10

DC    X'3737C4C45B5B84843C3CE3E3F4F4A3A3' 20

DC    X'0C0CD3D36B6B93931C1CE0E04C4C4F4F' 30

DC    X'0202C2C27F7F82821212D9D9F2F29999' 40

DC    X'2525D1D15C5C91913F3FE9E97A7AA9A9' 50

DC    X'2E2EC6C6505086863232E5E5F6F6A5A5' 60

DC    X'0E0ED5D54B4B95951E1E5F5F6E6EA1A1' 70

DC    X'0101C1C15A5A81811111D8D8F1F19898' 80

DC    X'0505C9C95D5D89891919E8E8F9F9A8A8' 90

DC    X'2D2DC5C56C6C85853D3DE4E4F5F5A4A4' A0

DC    X'0D0DD4D4606094941D1DBDBD7E7ED0D0' B0

DC    X'0303C3C37B7B83831313E2E2F3F3A2A2' C0

DC    X'0B0BD2D24E4E92922727ADAD5E5EC0C0' D0

DC    X'2F2FC7C77D7D87872626E6E6F7F7A6A6' E0

DC    X'0F0FD6D6616196961F1F6D6D6F6F0707' F0

*                 0 1 2 3 4 5 6 7 8 9 A B C D E F

If an AWS2703 device is ATTACHed directly by the an application, the all

OutBound and InBound data transmission is controled by that application.

In this case, the above translation process does not apply.