AWS9346 SCSI Quarter Inch Tape Drive Manager

============================================

AWS9346 has limited support for the 1/4 inch tape format known as

'QIC120'.  The drive supported is the IBM 3450 model 001 1.2GB 1/4 Inch

Tape Cartridge Drive.

AWS9346 works with the TAPE, VMFPLC2, MOVEFILE, and DDR commands.  It

reads and records data in buffered mode (data streaming).  The

performance is greatly improved if you use a large buffer size.

For example, to use 32K buffer on the TAPE command:

TAPE DUMP * * A ( BLK 32K

TAPE LOAD * * A ( BLK 32K

Before using this in production, you should thoroughly test it with your

application.

DEVICE TYPE: 9346

====================

CONFIGURING the 9346

====================

To configure a 9346 tape drive, go to the UPDATE SYSTEM DEVICES (F2) screen

and fill in the address and manager number fields appropriately.  Leave the

parameter/filename field blank.

You must specify AWS9346.SYS (which is provided in the P/390

installation diskettes) in your CONFIG.SYS and install the OS2SCSI.DMD

driver (provided with OS/2).  See the sample CONFIG.SYS file provided

during P/390 installation.  If you are using SCSI3480 or SCSI3420, then

the AWS9346.SYS device driver statement MUST be after them in

config.sys.

To use a 9346 with VM/ESA 370 feature, your VM system must have been

generated with an address for it.  In other words, there must be an

RDEVICE macro for a 9346 at some address in DMKRIO.  You cannot use a

9346 where 3420 or 3480 tape drives are defined.  If your VM system does

not include a 9346, you must have it add one to DMKRIO and regen your CP

nucleus.

VSE supports 9346 in both 370 and ESA modes; as with VM, using larger

block sizes keeps the drive in streaming mode and improves performance.

RESTRICTIONS:

============

VM/ESA ESA feature does not support the 9346.

AWS9346 is not supported when two S/370 Channel Emulator/A cards are

installed.

AWS9346 is provided primarily for data interchange with 9371 processors.

The preferred SCSI tape media is 4mm DAT (SCSI3420 or SCSI3480 device

managers).