AWSICA: Integrated Communications Adapter support

=================================================

***** NOTE:  The following information is preliminary and subject to  *****

*****        change as the Wide Area Connector card support evolves.  *****

AWSICA provides Synchronous Data Link Control (SDLC) Integrated Communications

Adapter (ICA) support using Multiprotocol, SDLC Modem/A, or Wide Area

Connector (WAC) cards.  In addition, Binary Synchronous Communication

(BSC) Control ICA support is provided using WAC cards.

DEVICE TYPE: ICA

============

CONFIGURING AN ICA ADAPTER

==========================

To configure an ICA, go to the configurator UPDATE SYSTEM DEVICES (F2) screen

and enter an address in the Addr field, a manager number in the Mgr field, and

one or more of the following options in the Filename/Parameters field (see

"Usage guidelines for AWSICA options" below for additional information):

<<PRI>  | <ALT> | <S=<* | s><i>>

<<SDLC> | <BSCA>>

<<SW>   | <LL>>   <<PRTS> | <NPRTS>> <<NRZI> | <NNRZI>>

<<DL>   | <NDL>>  <<LSN>  | <NLSN>>

<<DSR>  | <NDSR>> <<DCD>  | <NDCD>>

<A=xx> <T=t1t2> <NPR=nnn> <ICLK=bbbbbbb> <DD=ddnme>

Where

PRI   -- This device is to use the "primary" multiprotocol card ("SDLC_1").

This is the default and can only be specified for, at most, one

device.  If neither PRI nor ALT is specified, devices will be

assigned to cards in the order they are encountered in the

device mapping file (the output of the configurator), the primary

assigned first followed by the alternate.  See the usage note

below regarding selecting arbitration levels.  Specifying this

parameter requires use of a multiprotocol card.

ALT   -- This device is to use the "alternate" multiprotocol card ("SDLC_2").

This can be specified for, at most, one device.  If neither PRI

nor ALT is specified, devices will be assigned to cards in the

order they are encountered in the device mapping file (the

output of the configurator), the primary assigned first followed

by the alternate.  See the usage note below regarding selecting

arbitration levels.  Specifying this parameter requires use of

a multiprotocol card.

si    -- This device is to use the "i" interface of the Wide Area Connector

(WAC) card located in slot "s".  The WAC supports up to two

interfaces, designated "A" or "B".  It can be installed in any

available slot, "1" through "8".  If only one WAC is installed,

an "*" can be used as the slot number to indicate "use lowest

number slot containing a WAC card".  Specifying this parameter

requires use of a wide area connector card.  Examples:  "S=*A"

indicates to use the "A" interface of the first WAC card found

when scanning the slots in numerical order.  "S=7B" indicates

to use the "B" interface of the WAC card installed in slot 7.

SDLC  -- This device is to emulate a SDLC ICA.  This is the default.

For VM, be sure the address used is genned as a SDLC ICA.

BSCA  -- This device is to emulate a Bisync ICA.  This parameter can

*only* be specified on lines using a WAC card.  For VM, be

sure the address used is genned as a BSC ICA.

SW    -- This device is using a switched line.  VTAM checks for this so if

DIAL=YES for VTAM, specify this option.  This is the default.

LL    -- This device is using a leased line.  VTAM checks for this

so if DIAL=NO for VTAM, specify this option.

PRTS  -- Continuously assert Request To Send for this device.  This is the

default and generally can be used in all but multidropped

configurations.

NPRTS -- Assert Request To Send only when transmitting.  Required

for multidropped configurations.

NRZI  -- Use Non-Return to Zero Inverted encoding for the bit stream.

Specify if the remote end is using NRZI encoding.

NNRZI -- Do not use Non-Return to Zero Inverted encoding.  This is

the default and must be specified or defaulted if the remote

end is not using NRZI encoding.

DL    -- This device is attached to a modem supporting the V.25 bis

synchronous dial commands.  This option enables "autodial"

support (i.e. the dial CCW) and is the default.  The

IBM 5853, 7855, and SDLC Modem/A all support V.25 bis

protocol.  There is no harm in specifying this option

even if your modem does not support V.25 bis as long

as the application does not use the dial CCW.  You

*must* specify SW (or allow it to default) for this

option to be effective.  If using the multiprotocol card,

it *must* be P/N 92G7516 or P/N 6451114 for V.25 bis

protocol to work.  If it is not, and this option is

specified or defaulted, an error message will be given

as the needed hardware support for autodial is not available.

In that case, you must explicitly specify "NDL" to disable

the autodial support.  For autodial to work you must

have *both* a V.25 bis capable modem *and* a multiprotocol

card type which supports V.25 bis (P/N 92G7516, P/N 6451114,

or the SDLC Modem/A or WAC).

NDL   -- This device is not attached to a V.25 bis modem.  This will

cause command rejects to be returned on any dial CCWs.

LSN   -- Inform the application program that this device is attached

to a modem supporting the V.25 bis synchronous dial commands.

This option enables "listen" support (i.e. the listen CCW)

and is the default.  If supported by the application program,

this will allow V.25 bis modem error responses to be retrieved

and analyzed.  You *must* specify DL (or allow it to default)

for this option to be effective.  See the discussion of DL

(above) for additional details.

NLSN  -- This device is not attached to a V.25 bis modem.  This will

cause command rejects to be returned on any listen CCWs.

DSR   -- Monitor DataSet Ready (DSR) at all times for leased lines.  This is

the default and does not apply to switched lines (which always

monitor DSR at all times).  DSR is also referred to as CCITT signal

107 in the V.24 recommendation and is pin 6 on the RS-232 connector.

NDSR  -- Do not begin monitoring DSR for leased lines until the first

receive event has occurred.  This does not apply to switched lines

(which always monitor DSR at all times).  The intent of this option

is to allow a certain amount of "slack" checking for DSR when using

null modem cables or dataset eliminators which may make the local

DSR state dependent on the readiness of the other "remote" end.

In such cases it can be quite tricky to get some applications to

connect.  A prominent example is VTAM which enables, disables, and

re-enables on the secondary side of a leased subarea connection.

These transitions can cause the loss of DSR on the primary side

resulting in VTAM stopping the connection attempt.  It is

recommended to use this option *only* when absolutely necessary as

it defeats the normal AWSICA interface checks.  For example, it is

much better to use a null modem cable or dataset eliminator which

makes the local DSR state dependent only on the local DTR state,

and not on the *remote* DTR state.

DCD   -- Monitor Data Carrier Detect (DCD).  This is the default and provides

the most efficient ICA operation.  Use DCD if at all possible.  DCD

is also referred to as CCITT signal 109 in the V.24 recommendation

and is pin 8 on the RS-232 connector.  It is sometimes called RLSD

(Receive Line Signal Detector) or just Carrier Detect (CD).

NDCD  -- Do not monitor DCD.  DSR will be used exclusively for determining

the DCE (e.g. modem) state.  On the WAC, this is more expensive

as the hardware does not provide for DSR transition interrupts.

So, when NDCD is specified for WAC ICA lines, the DSR state will

be polled, which causes more overhead than waiting for a DCD

transition interrupt.  However, if your modem or dataset eliminator

does not provide a DCD signal to indicate the connection is ready,

you must use NDCD to allow operation.

xx    -- SDLC link address to use (by default, can be overridden by

application program).  This parameter is optional.  It is

specified as a hexadecimal value.  Normally, this value

does not need to be specified as the application program

will supply it.

t1t2  -- BSC tributary addresses when operating as a tributary station.

These are the addresses used by the address prepare CCW when

monitoring the link for a polling or selection sequence.  The

value is specified as two one-byte hexadecimal values.  The

default is x'4040'.  The supplied value cannot be a BSC control

character (< x'20') and it cannot have the x'20' bit set in the

address byte (e.g. x'6060' is invalid).  The master station uses

the x'20' bit to distinguish selection from polling.  The second

address can be used as a group address.  If only one address is

needed, use the same value for each address (e.g. x'C1C1').

Unless it is known that the software being used employs the

BSC address prepare CCW, this parameter is not needed.

nnn   -- SDLC nonproductive receive timeout to use (applicable only

when running in SDLC primary mode).  The default is 32

seconds.  This parameter is optional.  It is specified

as a decimal value between 1 and 255 seconds.  Normally,

the default value is acceptable.  Specify a smaller value

if you wish a quicker realization that the remote end has

not responded.  For BSC lines, this parameter can be used

to change the default 3 second read timeout.

ddnme -- Name of device driver which will operate this device.

The value specified must be a valid device driver name

installed in the system.  Maximum length is 8 characters.

For MPCA devices, use "MPADD$".  For WAC devices, use

"WACDD$".  This parameter should not need to be used in

the preliminary version as the correct device driver will

be selected based on the parameter used to identify the

hardware card associated with this device.

b...b -- Use internal clocking as close to the bit rate specified

as permitted by the hardware.  Do *not* specify this

parameter in a traditional DCE (modem) clocked environment.

It is intended to be used with a "null" modem cable of

the appropriate type or a dataset eliminator which will

redrive the clock supplied by the local DTE (AWSICA).

See "Null modem cable information (ICLK option)" below.

The value specified is a decimal value between 1 and

2048000 bits per second.  The parameter is not supported

on the MPCA.  The actual bit rate used for the clock may

differ from the value specified.  The hardware has a fixed

clock rate that is divided by an integer to a value as

close as possible to the specified bit rate.

Usage notes:

============

Up to two multiprotocol cards are supported, each providing the appearance of a

single 9221-type SDLC ICA device.  Switched and nonswitched RS-232 lines are

supported, with a maximum data rate of 19.2 Kbps.  No BSC support is available

with the multiprotocol cards.

Up to four wide area connector (WAC) cards are supported and each may have up

to two line interfaces installed.  You must specify the desired electrical

interface when ordering.  If you order just the WAC card, only *one* interface

will be installed.  See "PS/2 feature information" below for additional details

on ordering WACs.  The maximum supported configuration is eight 9221-type

SDLC/BSC ICA devices.  Switched and nonswitched RS-232 and V.35 lines and

nonswitched X.21 lines are supported (no switched X.21 support), with a maximum

data rate of 500 Kbps (V.35).  This is the fastest speed tested, higher rates

*should* also work.  It is not yet known whether actual systems will support

up to eight lines all running at the maximum speed.  For example, the active

polling loop used by VTAM may very well impose lower speed limits.  The RS-422

interface should work (switched and nonswitched) but has not been extensively

tested.

Define the line to VTAM as you would a 9221 SDLC ICA (but note restrictions

below).  For more information about the 9221 SDLC ICA, see "ES/9000

Telecommunications Subsystem Description and Reference" (SA24-4198).  If

connecting to a NCP, keep in mind that ICAs do *not* support two way

simultaneous SDLC (ADDRESS=(lnbr,FULL)) or more than 7 outstanding SDLC

frames (MAXOUT=7).

Autodial assumes a V.25 bis capable modem is attached and that a multiprotocol

card capable of V.25 bis operation is being used (P/N 92G7516 or P/N 6451114).

Other older multiprotocol cards do NOT support V.25 bis operation.  The SDLC

Modem/A and Wide Area Connector support V.25 bis.

For BSC autodial, the ICA is operated in SDLC mode during the dial sequences

to permit use of SDLC V.25 bis modems.  The switching into and out of SDLC

mode is transparent to the BSC application.  Because it is expected that most

BSC applications will assume a V.25 implementation (*not* V.25 bis), AWSICA

will supply the V.25 bis CRN command prefix.  The BSC application only supplies

the telephone number (as it normally would in a V.25 external autocall unit

environment).  This is unlike the SDLC autodial mode wherein the application

is expected to pass the entire V.25 bis modem command, including the CRN

command prefix itself.  This is the same as the 9221 where the BSC autodial

support is V.25 and the SDLC autodial support is V.25 bis.  The difference

between the 9221 implementation and AWSICA is that AWSICA permits the use

of a SDLC V.25 bis modem for *both* SDLC and BSC autodial.

It is recommended that a primary card (see PRI option above) be configured

to use arbitration level 1.  An alternate card (see ALT option above) should

be configured to use arbitration level 7.  These use dedicated DMA channels

and are compatible with Communications Manager.  If an alternative arbitration

level must be used, select from those that use dedicated DMA channels (levels

1, 3, 5, 6, and 7).  While use of other than one of these levels should

work, AWSICA does not attempt to acquire and release a programmable DMA

channel for each I/O operation.  It will therefore monopolize use of a

programmable DMA channel if assigned an arbitration level requiring one.

The Wide Area connector does not use any arbitration levels so has none

of these ramifications.

For VM/SP or VM/ESA 370-feature, the DMKRIO entry corresponding to the device

should be of the form:

RDEVICE ADDRESS=cuu,DEVTYPE=ICA,ADAPTER=SDLC

or, for a BSC line:

RDEVICE ADDRESS=cuu,DEVTYPE=ICA,ADAPTER=BSCA

AWSMPADD device driver (MPCA):

==============================

The control unit requires the AWSMPADD.SYS device driver to be installed in

CONFIG.SYS when using a multiprotocol card or SDLC Modem/A:

DEVICE=d:\path\AWSMPADD.SYS <~P | ~A> <32K | 64K> <DP>

The first optional parm indicates whether one card in a two-card system

should be ignored (i.e., able to be used by other OS/2 software such as

Communications Manager).  The default (no parm) is that all cards present

should be used by AWSICA.  "~P" indicates to ignore the primary card (SDLC_1);

"~A" indicates to ignore the alternate card (SDLC_2).  The "~" character is

displayed on the PS/2 as a "tilde".

It is not useful to exclude *both* cards (will result in an error message;

do not load AWSMPADD.SYS if both cards are to be excluded).  Similarly, there

is no point to excluding one card in a one-card system; again, just do not

load AWSMPADD.SYS in this case.

The second optional parm indicates whether AWSMPADD should allocate a 32K

or 64K I/O buffer for each card used by AWSICA.  The default of 32K allows

for SDLC frames 4,096 bytes in size to be received in the worst case (8

outstanding frames).  This is more than adequate in all normal situations.

Specifying 64K increases the maximum SDLC frame size in the worst case to

8,192 bytes.

The third optional parm indicates whether dynamic POS support is desired.

This causes AWSMPADD to tolerate the absence of the MPCA at system bringup

and causes it to release all MPCA resources when AWSICA terminates.  When

used with the companion device driver, AWSPOSDD, the MPCA can then be

toggled between ASYNC and SYNC modes, if care is taken, without running

the PS/2 configuration program and rebooting the system.  Most errors

which normally are detected at system IPL are detected at AWSICA bringup

when this option is selected.  It is not recommended this option be used

unless development has been consulted.  When trying to isolate an

AWSICA problem, dynamic POS *must* be disabled.

AWSWACDD device driver (WAC):

=============================

The control unit requires the AWSWACDD.SYS device driver to be installed in

CONFIG.SYS when using a wide area connector (WAC) card:

DEVICE=d:\path\AWSWACDD.SYS <M=aaa <M=bbb <M=ccc <M=ddd>>>>

No parameters need be specified on this invocation for microchannel WACs.

For microchannel systems, there are no concerns regarding interrupt sharing

and other resource choices.  Just use the normal microchannel configuration

process and it will assign the resources needed by the WAC card.

For ISA WAC installations, the optional "M=" parameter specifies up to

four 16K memory windows to be used to map WAC on-card memory into system

memory.  While the ISA WAC IRQ and I/O port addresses are determined by

the user during hardware installation (DIP switches), the memory window

address is assigned by the device driver during system initialization.

If no "M=" parameters are provided, the device driver will attempt to

automatically assign a 16K memory window to each WAC card found.  If greater

control over this process is desired, specify the "M=" parameter.  The

operand must be a hexadecimal 4K page number on a 16K boundary in the.

range 0C0 - 0DC.  That is, 0C0, 0C4, 0C8, 0CC, 0D0, 0D4, 0D8, or 0DC.

The starting memory address used is the supplied value times 4096.  For

example, M=0D4 means that an assignable address for the WAC is 0D4000-0D7FFF.

This is a 16K window starting at "page" 0D4.  If you specify the "M="

parameter, specify one for each WAC card in the system (up to four cards

total).  Use of the "M=" parameter should not normally be needed and is

provided to allow customization flexibility.  Try your installation without

it and only specify it if a memory conflict occurs.

Microchannel systems use level sensitive interrupt requests and the

interrupt level can be shared between disparate devices.  Industry standard

architecture (ISA) systems use edge triggered interrupt requests and this

precludes interrupt level sharing between device drivers.  So, in general

configure each ISA WAC card to use its own *exclusive* interrupt level.

The interrupt level is determined by DIP switches 1-3 on the ISA WAC card

IRQ 10 and 11 would be the first two choices to try on most systems.  If

AWSWACDD is not able to hook the interrupt exclusively, AWSICA108E ("Error

establishing interrupt handlers") will be issued at OS/2 boot time and the

device driver will fail to install.  Review your ISA WAC DIP switch settings

and select another available interrupt level for the card.

Because the need for exclusive interrupt levels may be hard to meet when

using multiple WAC cards, AWSWACDD does support a limited form of interrupt

sharing for ISA WAC cards.  That is, so long as nothing but AWSWACDD-owned

WAC cards are configured to use an interrupt level, multiple ISA WAC cards

can be set to the same level.  AWSWACDD will still hook this interrupt

exclusively (meaning no other device drivers can use it) but it will accept

and process all interrupts on this level.  In this way, you can run all of

the installed WAC cards from just a single interrupt level.

Extended industry standard architecture (EISA) systems support the use of

configuration files to assist in keeping track of the resources in use by

the current configuration of installed cards.  At the present time, it is

not clear if an EISA configuration file can be made which would allow for

the choices listed above.  That is, a file which would allow both an interrupt

to be exclusively assigned to each WAC card *and* would allow for the form

of limited interrupt sharing supported by AWSWACDD.  For that reason, no

EISA configuration file for the WAC is being supplied with AWSICA.  Use the

EISA procedures to learn which resources are available (I/O ports and IRQs)

and use this information in setting the ISA WAC DIP switches.

Note that you do *not* use any of the LAPS (LAN Adapter and Protocol Support)

installation procedures described on the WAC installation diskette.  All

AWSICA run-time support is provided by AWSWACDD.SYS.  At present, WAC adapters

*cannot* be shared between LAPS and AWSWACDD.SYS.  Be sure that the LAPS WAC

device driver, IBMWAC.OS2, is *not* installed in CONFIG.SYS.  If you have

configured a WAC card with LAPS that is to be used by AWSWACDD.SYS, you *must*

use LAPS and remove its configuration for that card.  Failure to do so will

generally result in difficult-to-diagnose problems with AWSICA.

AWSICA control unit emulation manager:

======================================

The control unit is expected to be invoked via the AWSSTART utility without

a session:

AWSSTART AWSICA.EXE N </CPU=dddd> </NODDTRC>

This is normally handled by the supplied IPL.CMD REXX procedure.

Optional Device Manager Parameters

----------------------------------

These parameters should be specified using the P/390 Configurator "Update

System Devices" menu "Device Manager Parm Edit/Display" submenu (F2/F12).

The CPU optional parameter permits specifying the CPU model number returned

on the Sense-ID CCW response.  It should be a valid 4-digit decimal value

(each digit being in the range 0-9).  If not specified, AWSICA returns the

actual CPU model number being used within the Sense-ID response:

x'FF dddd 00CA vvvv'

The last two bytes are determined by the configured line discipline

and options.  See "ES/9000 Telecommunications Subsystem Description

and Reference" (SA24-4198) for more information.  The intent of this

option is to allow correct operation with applications and control

programs which depend on the CPU model number returned via Sense-ID.

For example, VM/ESA ESA-feature requires the CPU model to be 9221 to

automatically bring SDLC ICA devices online at IPL.

The NODDTRC optional parameter is used to suppress trace entries normally

made by the device drivers when tracing is enabled for a device.  When

specified, only control-unit level tracing will be made for devices which

are being traced.  This can be used to minimize the burden to the system

of having both the CU and DD making trace entries.  It also will lengthen

(somewhat) the collective memory of the system trace table (as fewer entries

are being made).  This parameter has no effect if tracing is not turned on

(the normal case).  It applies globally, to all AWSICA devices, unlike

the device-level granularity of the system tracing function.

Utility mode options

--------------------

A utility mode is also available to assist in defining the devices:

AWSICA /qcfg=ddname

where "ddname" is the "internal" DD name of one of the supported AWSICA device

drivers.  Use "MPADD$" for AWSMPADD and "WACDD$" for AWSWACDD.  The "$"

character is x'24' in hex, upper case "4" on US keyboards.  This will list

the hardware configuration currently available to AWSICA.

AWSICA /qres=ddname

where "ddname" is as above.  This will list the hardware resources (interrupt

levels, I/O port range, and memory range) in use by the cards owned by the

specified device driver.

The utility mode of AWSICA can be run whether or not AWSICA is already loaded

as a control unit manager.  Note that AWSSTART is *not* used when invoking

AWSICA in utility mode.  The intent is to assist the user in identifying the

installed hardware in the manner needed by the configurator.

Restrictions and known deviations from the 9221 ICA:

====================================================

Multidropped configurations have undergone limited testing to date.

The WAC X.21 interface is only supported for leased lines (no switched line

support).

The WAC RS-422 interface has only undergone limited testing to date.

Link Problem Determination Aid-level 2 (LPDA-2) protocol support is not

provided.

PU T2.1 support was not implemented on the original 9370 ICA but was introduced

on the 9371 and 9221 SDLC ICAs.  This support has been added to AWSICA but has

only undergone limited testing to date.

A recent marketing flash suggested that both the SDLC Modem/A and the

multiprotocol card may not work reliably on PS/2 systems with clocks in excess

of 33Mhz.  Until more testing can be done with AWSICA on these systems and more

experience gained, be sure to carefully evaluate its operation on any system

running at 33Mhz or more.

PS/2 feature information:

=========================

Order item (A), item (B), item (C), or item (D):

(A)  FC 0960, PN 93F0973 -- IBM Asynchronous/SDLC V.32 Modem/A

FC 0960 avoids the need for both a multiprotocol card and a separate

external modem. It also requires ONE of the following cables:

- FC 6661, PN 93F0975 -- 4-wire leased  line cable

- FC 6662, PN 93F0976 -- RJ-11 switched line cable

- FC 6663, PN 93F0977 -- RJ-45 switched line cable

(B)  FC 3043, PN 92G7516 -- Personal System/2 Multi-Protocol Adapter/A

FC 3043 requires a separate RS-232 synchronous modem and connecting cable.

The previous version of this feature, PN 6451114, is equivalent to

PN 92G7516.  There are no functional differences between the two.

The earliest version of this feature, PN 6451003, can be used but will not

work with autodial support due to lack of support for V.25 bis protocol.

Note:  Announcement 996-068, dated March 19, 1996, indicated that PN

92G7516 will be withdrawn from marketing effective June 21, 1996.  If

you cannot obtain this feature, the RS-232 Wide Area Connector (see

below) should be used.

(C)  FC 2872, PN 33G8465 -- Wide Area Connector, microchannel, RS-232D.

FC 2873, PN 33G8466 -- Wide Area Connector, microchannel, V.35.

FC 2875, PN 33G8467 -- Wide Area Connector, microchannel, X.21.

FC 2878, PN 33G8468 -- Wide Area Connector, microchannel, RS-422.

These features come with a cable of the appropriate type to attach

to the data communication equipment (DCE).  The WAC supports up to

two interfaces but is shipped with only one installed.  The interface

types can be mixed (one RS-232D and one V.35 for example).  To order

a second interface:

PN 33G8461 -- WAC additional RS-232 electrical interface board (EIB)

PN 33G8462 -- WAC additional V.35   electrical interface board (EIB)

PN 33G8463 -- WAC additional X.21   electrical interface board (EIB)

PN 33G8464 -- WAC additional RS-422 electrical interface board (EIB)

The EIBs come with an appropriate cable for DCE attachment.  The X.21

EIB is only supported for leased lines (no switched line support).

(D)  FC 2880, PN 33G8469 -- Wide Area Connector, ISA bus,      RS-232D.

FC 2884, PN 33G8470 -- Wide Area Connector, ISA bus,      V.35.

FC 2888, PN 33G8471 -- Wide Area Connector, ISA bus,      X.21.

FC 2890, PN 33G8472 -- Wide Area Connector, ISA bus,      RS-422.

These features come with a cable of the appropriate type to attach

to the data communication equipment (DCE).  The WAC supports up to

two interfaces but is shipped with only one installed.  The interface

types can be mixed (one RS-232D and one V.35 for example).  To order

a second interface:

PN 33G8461 -- WAC additional RS-232 electrical interface board (EIB)

PN 33G8462 -- WAC additional V.35   electrical interface board (EIB)

PN 33G8463 -- WAC additional X.21   electrical interface board (EIB)

PN 33G8464 -- WAC additional RS-422 electrical interface board (EIB)

The EIBs come with an appropriate cable for DCE attachment.  The X.21

EIB is only supported for leased lines (no switched line support).

Notes:

FC 3042, PN 6450348 -- Personal System/2 Multi-Protocol Adapter/A

FC 3042 will not work if the PS/2 clock rate is greater than 16Mhz.  This

product is no longer available from IBM and has been replaced by FC 3043.

It is not supported for use with AWSICA.

If ordering the Wide Area Connector, choose between (C) and (D) above

based on your system bus.  Select (C) for microchannel-based systems

and (D) for PCI/ISA-based systems.  Note that the additional EIB feature

is common between the MCA and ISA WACs.  These features can be freely

swapped between differing WAC base cards.

***** WARNING  WARNING  WARNING  WARNING  WARNING  WARNING *****

For PCI/ISA-based systems, you *must* order the ISA-bus WACs listed

above (D).  Do *not* order:

FC 8270, PN 85H7204 -- Wide Area Connector, PCI bus,      RS-232D.

FC 8271, PN 85H7205 -- Wide Area Connector, PCI bus,      V.35.

FC 8279, PN 85H7882 -- Wide Area Connector, PCI bus,      X.21.

FC 8272, PN 85H7206 -- Wide Area Connector, PCI bus,      RS-422.

While these sound like a plausible card to order, they are not

supported by AWSWACDD as they are a completely different hardware

design.  For example, the PCI WAC only supports a single port per card.

The only thing in common between the PCI WAC and the MCA/ISA WACs is

the name "WAC"!  There is no plan to add support to AWSWACDD for the

PCI WAC card.

Usage guidelines for AWSICA options:

====================================

At first glance, the number of line options may appear daunting.  However,

many are only used for "peculiar" conditions.  The options below are listed

in two groups.  The first group contains the "common" options.  You will

almost certainly have to consider each of these and select those needed

when the AWSICA defaults are not appropriate for your configuration.  The

second group contains options that are much less commonly needed.  You may

be able to successfully ignore them as they provide flexibility for unusual

situations.

Another reason the option list is complicated is that some options are used

only for SDLC, some only for BSC, and some are used for either SDLC or BSC.

So, in the list below, each is specifically listed with its line discipline

applicability.

In the list below, the default of each opposable option (indicated by the

presence of "vs.") is listed first.  The full set of defaults are:

PRI (if MPA), SDLC, SW, PRTS, NNRZI, DL, LSN, DSR, DCD, T=4040,

NPR=32 (if SDLC), NPR=3 (if BSCA), DD=MPADD$ (if PRI or ALT),

DD=WACDD$ (if S=si).

Common options:

AWSICA option  � Usage     � Comments

���������������ݞ����������ݞ���������������������������������������������

PRI  vs. ALT   � SDLC-only � As it only applies to the SDLC-only MPA card

vs. S=si  � either    � Specifies WAC PS/2 slot location

���������������ݞ����������ݞ���������������������������������������������

SDLC vs.       � SDLC-only � This is a SDLC line

BSCA  � BSC-only  � This is a BSC  line

���������������ݞ����������ݞ���������������������������������������������

SW   vs. LL    � either    � Switched or leased lines

���������������ݞ����������ݞ���������������������������������������������

PRTS vs. NPRTS � either    � Controls request-to-send behavior, CCITT 105

���������������ݞ����������ݞ���������������������������������������������

NNRZI vs. NRZI � SDLC-only � NRZI encoding not used with BSC

Options for special situations:

AWSICA option  � Usage     � Comments

���������������ݞ����������ݞ���������������������������������������������

DL   vs. NDL   � either    � Controls usage of BSC or SDLC Dial CCW

���������������ݞ����������ݞ���������������������������������������������

LSN  vs. NLSN  � SDLC-only � Controls usage of SDLC Listen CCW

���������������ݞ����������ݞ���������������������������������������������

DSR  vs. NDSR  � either    � Controls DSR checking on leased lines

���������������ݞ����������ݞ���������������������������������������������

DCD  vs. NDCD  � either    � Controls DCD checking

���������������ݞ����������ݞ���������������������������������������������

A=xx           � SDLC-only � Default hexadecimal SDLC link address

���������������ݞ����������ݞ���������������������������������������������

T=t1t2         � BSC-only  � Tributary addresses used by BSC Adprep CCW

���������������ݞ����������ݞ���������������������������������������������

NPR=nnn        � either    � Sets SDLC primary non-productive timeout or

�           � changes BSC default 3-second read timeout

���������������ݞ����������ݞ���������������������������������������������

ICLK=bbbbbbb   � either    � Sets baud rate if using DTE-clocking

���������������ݞ����������ݞ���������������������������������������������

DD=ddname      � either    � Device driver name to use with this line

Null modem cable information (ICLK option):

===========================================

AWSICA can be used with a null modem cable (no modems or dataset eliminators).

When modems are used, the modem supplies transmit and receive clocks to AWSICA.

When a null modem cable is used, AWSICA *must* supply these clocks to itself

and to the "remote" device.  The AWSICA ICLK option is used to indicate that

AWSICA is to supply a clock of the specified rate.  This option is not

supported on MPCA devices so null modem cables can *not* be used in that

configuration.  In addition, the WAC V.35 EIB does not support supplying a

clock signal to the interface so ICLK can *not* be used there either.  If

using a null modem cable with AWSICA on each end (two P/390s), only *one*

end should specify ICLK.  The other end will be clocked by the ICLK system.

That is, it is no different than when using a modem.  The ICLK end supplies

the clock to the non-ICLK end.

WAC RS-232 null modem cable:

RS-232 uses a 25-pin D-shell electrical connector.  The numbers below refer

to the pin numbers of this connector.  The "left side" of this cable is

connected to the ICLK system.  AWSICA then drives a clock on pin 24

which the cable then supplies to pins 15 (transmit clock) and 17 (receive

clock) of both sides.  Each side's RTS is reflected back as its RFS.

Each side's DTR is reflected back as its DSR and CD signals.  This

avoids intervention required errors that would otherwise occur if one

side is started before the other.  ICLK values up to about 64,000 bps

should work, although speeds of that magnitude may be marginal.

2 TD     -------> RD      3

3 RD     <------- TD      2

4 RTS    ->|  |<- RTS     4

5 RFS    <-|  |-> RFS     5

6 DSR    <-|  |-> DSR     6

8  CD    <-|  |->  CD     8

20 DTR    ->|  |<- DTR    20

1 Shield -------- Shield  1

7 SG     -------- SG      7

15 TCLK   <-|----> TCLK   15

17 RCLK   <-|----> RCLK   17

24 ECLK   ->|

WAC X.21 null modem cable:

X.21 uses a 15-pin D-shell electrical connector.  The numbers below refer

to the pin numbers of this connector.  The cable below is as recommended

by the WAC documentation.  Unlike the RS-232 cable above, this is a

symmetric cable.  Either end can be attached to the ICLK system.  The

ICLK end will internally clock itself and drive a clock on the B(A)/B(B)

circuits.  The other end will use S(A)/S(B) as its clock input, just as

if a modem was attached.  The transmit and receive circuits are swapped

symmetrically as are the control and indicate circuits.  One result of

this is that it is possible to get intervention required errors if the

remote end is not started (which means its control circuit pair will not

be driving the local end's indicate circuits).  It can be tricky to get

such a connection going, especially if the application disables the line

as part of its start-up procedure!  Use of the NDSR option may help this

situation.  ICLK values up to about 2.048 Mbps should work, although

speeds of that magnitude may be marginal.

2 T(A)   -------> R(A)    4

9 T(B)   -------> R(B)   11

4 R(A)   <------- T(A)    2

11 R(B)   <------- T(B)    9

3 C(A)   -------> I(A)    5

10 C(B)   -------> I(B)   12

5 I(A)   <------- C(A)    3

12 I(B)   <------- C(B)   10

1 Shield -------- Shield  1

8 SG     -------- SG      8

6 S(A)   <------- B(A)    7

13 S(B)   <------- B(B)   14

7 B(A)   -------> S(A)    6

14 B(B)   -------> S(B)   13

WAC RS-422 null modem cable:

RS-422 uses a 37-pin D-shell electrical connector.  The numbers below refer

to the pin numbers of this connector.  The cable below is as recommended

by the WAC documentation.  Unlike the RS-232 cable above, this is a

symmetric cable.  Either end can be attached to the ICLK system.  The

ICLK end will internally clock itself and drive a clock on the ST(A)/ST(B)

circuits.  The other end will use RT(A)/RT(B) as its clock input, just as

if a modem was attached.  The transmit and receive circuits are swapped

symmetrically as are the RTS and receiver ready circuits.  One result of

this is that it is possible to get intervention required errors if the

remote end is not started (which means its RTS circuit pair will not be

driving the local end's receiver ready circuits).  It can be tricky to get

such a connection going, especially if the application disables the line

as part of its start-up procedure!  Use of the NDSR option may help this

situation.  ICLK values up to about 2.048 Mbps should work, although

speeds of that magnitude may be marginal.

4 SD(A)  -------> RD(A)   6

22 SD(B)  -------> RD(B)  24

6 RD(A)  <------- SD(A)   4

24 RD(B)  <------- SD(B)  22

7 RS(A)  -------> RR(A)  13

25 RS(B)  -------> RR(B)  31

13 RR(A)  <------- RS(A)   7

31 RR(B)  <------- RS(B)  25

1 Shield -------- Shield  1

19 SG     -------- SG     19

8 RT(A)  <------- ST(A)   5

26 RT(B)  <------- ST(B)  23

5 ST(A)  -------> RT(A)   8

23 ST(B)  -------> RT(B)  26

Some common sense code meanings:

================================

ICA sense codes are quite specific and often point directly at the problem.

If VTAM is the ICA application program, these sense codes will be displayed

as the last field of message IST446I.  These are two byte codes.  If the

error code indicates a particular aspect of modem operation (for example,

the way DSR is being handled), review your modem configuration and insure

it is set up for "V.25 bis rules".  Note:  Not all possible sense codes

are listed here!  See "ES/9000 Telecommunications Subsystem Description

and Reference, SA24-4198" for complete details.

4020 -- DSR is off during a read-type or write-type command

4021 -- DSR is off at the start of a read-type or write-type command

4022 -- CTS is off at the start of a write-type command

4023 -- CTS is off during a write-type command

4024 -- Apparent loss of transmit clock during a write-type command

4025 -- For NPRTS mode, CTS on before RTS or CTS did not drop after RTS

402E -- DSR did not come up before enable timeout expired

4031 -- DSR is on too soon during enable (e.g. permanently on)

403D -- CTS is on too soon, did not come up at all, or dropped during Dial

403E -- Apparent loss of transmit clock during a dial command

403F -- DSR did not come up before dial timeout expired

02C5 -- DSR is on too soon during dial (e.g. permanently on)

01EC -- For switched lines, DSR did not drop before disable timeout expired

01F0 -- Non-productive receive timeout during a read-type command

01F1 -- Idle timeout during a read-type command on a primary station

01F2 -- Fill character timeout during a write-type command

01FF -- General catch-all for a "should not occur" error

The following are caused by various V.25 bis modem responses.  See your

modem manual for a more complete description.

4027 -- V.25 bis error:  INV       (invalid)

4028 -- V.25 bis error:  CFI       (call failure indication)

402F -- V.25 bis error:  CFICB     (modem busy)

4030 -- V.25 bis error:  CFIDE     (dial tone error)

4032 -- V.25 bis error:  CFIET     (busy signal detected)

4036 -- V.25 bis error:  CFIFC     (forbidden call)

4037 -- V.25 bis error:  CFINS     (number not stored)

4038 -- V.25 bis error:  CFINT     (no answer, timed out)

4039 -- V.25 bis error:  CFIRT     (no answer, ringing)

403A -- V.25 bis error:  DLC       (delayed call)

403B -- V.25 bis error:  VAL | LS  (valid or list)

403C -- V.25 bis error:  INC       (call collision)