AWSC370: S/ 370 Channel Emulator

===============================

The AWSC370 manager allows real S/370 I/O control units to be attached

to a P/390 system via the S/ 370 Channel Emulator/A card.

Important Notes:

1. The use of hardware device address 00 is not allowed.

2. You cannot read or write records greater than 65535 bytes (64K - 1).

3. The P/390 I/O subsystem does not support the READ BACKWARDS CCW unless

the SKIP bit is on.  This may affect certain tape utility programs.

It applies to all tapes, whether emulated, SCSI-attached, or Bus & Tag

Channel-attached.

CONFIG.SYS

==========

You must add the following statements to the BOTTOM of CONFIG.SYS:

If they are already present, but commented out, remove the "rem"

from the front of each line you wish to make active.

DEVICE=D:\P390\chan370.sys tbuf=2040 pollcount=10000

RUN=D:\P390\chan370.exe

Device Manager Input Parameters

===============================

The Channel Emulator can be started at three different speeds and in three modes

The default is to start all cards in Block, DCI mode. These parameters

can be specified with the P/390 Configurator "Update System Devices"

menu "Device Manager Parm Edit/Display" submenu (F2/F12).

Channel Speed

-------------

/[s]Dn       Turns on high-speed Data Streaming (default = DCI mode).

"s" is optional PS/2 slot number of card (default = all slots).

"n" is data streaming speed:

/D3 gives 3 MB/sec

/D4 gives 4.5 MB/sec

Note: If /D is not specified, the channel speed of all S/ 370 Emulator cards

will be DC Interlock (DCI).

Channel Mode

------------

/[s]S        Run channel in Selector mode

"s" is optional PS/2 slot number of card (default = all slots).

/[s]B        Run channel in Byte Multiplexor mode

"s" is optional PS/2 slot number card (default = all slots).

Note: If neither /S nor /B are specified, all S/ 370 Emulator cards will run

in Block Multiplexor mode.

Miscellaneous Settings

----------------------

/Wttttt      Set timeout of "ttttt" seconds to wait for interrupts

(default is 10 seconds).

/W65535 shuts off timeout (useful for tracing).

/Z           activates trace debugging, including SNAP tracing support.

Examples:

--------

1. To start the S/ 370 Emulator manager in 4.5 MB/sec Data Streaming mode:

call dmstart "AWSC370.EXE N /D4"

2. To start two S/ 370 Emulator cards in different modes:

call dmstart "AWSC370.EXE N /5B /1D4"

This will run the card in PS/2 slot 5 in Byte Multiplexor mode

and the card in PS/2 slot 1 in Block Multiplexor mode at 4.5MB/sec.

Configuring C370 devices

========================

There are two situations where you need to add parameters to a CHAN370 device

entry in DEVMAP using the P/390 Configurator "UPDATE SYSTEM DEVICES" menu:

1. If the hardware unit address does not match the device address used by

your P/390 operating system.

IMPORTANT: The use of hardware device address 00 is not allowed.

2. If there are multiple S/ 370 Emulator cards in the PS/2.

These parameters may optionally be specified in the FN/P field of any devices

that are assigned to the CHAN370 MGR code:

(blank)  P/390 device address is the same as the hardware unit address.

Assign to S/ 370 Emulator card in lowest numbered PS/2 slot.

*      Same as blank.

*dd    Remap P/390 device address to hardware unit address "dd".

Assign to S/ 370 Emulator card in lowest numbered PS/2 slot.

c      Don't remap device address.

Assign to S/ 370 Emulator card in PS/2 slot "c".

cdd    Remap P/390 device address to hardware unit address "dd".

Assign to S/ 370 Emulator card in PS/2 slot "c".

Note:  If "c" is specified, it is checked. If there is no S/ 370 Emulator card

in slot "c" then AWSC370.EXE terminates with an error message.

Examples:

--------

1. This DEVMAP entry remaps P/390 address 580 to hardware unit address 60.

Type    Addr    Mgr   FN/P

----    ----    ---   ------------

3420    580      x    *60

2. Assign device 580 to S/ 370 Emulator card in slot 5 and 590 to card in slot 1.

Type    Addr    Mgr   FN/P

----    ----    ---   ------------

3420    580      x    5

3480    590      x    1

3. Assign device 580 to S/ 370 Emulator card in slot 5 and remap to unit addr 60.

Type    Addr    Mgr   FN/P

----    ----    ---   ------------

3420    580      x    560

Ordering Information:

=====================

S/ 370 Channel Emulator/A is ONLY available from an IBM Marketing Branch

as RPQ 8B3927 for a 3820 printer.

Utilities

=========

C370MAP     will show the status of the S/ 370 Emulator channel

and display the device address mapping.

CLRIO       issues a CLRIO or Clear Channel to hardware devices.

TRCC370     starts a trace of S/ 370 Emulator channel activity.