LCS3172 3172 Lan Channel Station for TCP/IP

``````` ```````````````````````````````````

LCS3172 emulates a 3172 LAN Channel Station and provides the mechanism

used by MainFrame TCP/IP products to interface to Local Area Networks.

DEVICE TYPE - 3088 (even/odd SubChannel Address pair)

`````` ````

CONFIGURING LCS3172

``````````` ```````

LCS3172 requires a DeviceDriver (AWSLCSDD.SYS) so insure that the following

line is present, uncommented (remove "REM") in CONFIG.SYS:

DEVICE=d:\P390\AWSLCSDD.SYS

Where d:\P390 is where you installed the P/390 programs.

There are no parameters for AWSLCSDD.SYS

NOTE: Since AWSLCSDD calls the ProtocolManager (PROTMAN.OS2) during

system initialization, the CONFIG.SYS statement for AWSLCSDD

needs to be after ALL statements for the ProtocolManager, e.g.

DEVICE=C:\IBMCOM\PROTMAN.OS2 /I:C:\IBMCOM

DEVICE=C:\IBMCOM\PROTOCOL\LANDD.OS2

DEVICE=C:\IBMCOM\PROTOCOL\LANDLLDD.OS2

*----> AFTER you run MPTS (or LAPS), check your CONFIG.SYS, and move

EVERY new statement from after the P/390 block of statements to

before them. The P/390 statements begin and end as shown below:

REM Make sure these statements are ALWAYS at the bottom of CONFIG.SYS !

REM *******************************************************************

REM                P/370 & P/390

REM *******************************************************************

. . . . . . . P/390 Statements . . . . . . . . . . . . .

REM This must ALWAYS be the LAST line of CONFIG.SYS !!

To configure LCS3172, go to the P/390 Configuration "Update System Devices"

(F2) screen.  Configure an even/odd pair of consecutive addresses as device

type 3088 for each logical 3172.  Use the manager code for LCS3172 in the Mgr

field.  An optional comment may be placed in the FN/P field, but it will be

ignored by LCS3172.

Example:

������������������������������ Available disk space in Kbytes: 35,598 ��������ͻ

�Addr Device  Label  Atype  Size    Mgr   FN/P     PATH:>F:\                  <�

���Ŀ-------------------------------�Ŀ----------------------------------------�

�440�|      >       >     >         �A�>                                       �

�����                               ���

�440  3088                           A                                         �

�441  3088                           A                                         �

������������������������������������������������������������������������������ͼ

MGR Codes: 1=AWSFBA   2=AWSCKD   3=AWS3274  4=LAN3274  5=AWS3215  6=AWS2821

7=AWS2540  8=LAN3088  9=AWS3172  A=LCS3172  B=AWS2703  C=AWSICA   D=AWSFEP

���Ŀ                          �Ŀ

�440� is the channel address   �A� is the LCS3172 manager code

�����                          ���

Up to four logical 3172 attachments (four consecutive even/odd address pairs,

eight channel addresses total) may be defined. But in general, you should only

configure a single LCS3172 that can be used with multiple LAN adapters.

�� HARDWARE LIMITATIONS ��������������������������������������������������Ŀ

�                                                                          �

�    FOR SERVER/330 WITH BUILTIN AMD 10MBIT ETHERNET ADAPTER, DO NOT USE   �

�    ANY OF THE IBM PCI ETHERNET ADAPTERS THAT USE THE IBMEANDI.OS2 NDIS   �

�    DRIVER (IBM PCI ETHERNET ADAPTER 13H9237) SINCE THE IBMEANDI.OS2      �

�    NDIS DRIVER WILL NOT CO-EXIST WITH THE PCNTND.OS2 DRIVER USED BY      �

�    THE SERVER/330 BUILT-IN 10MBIT ETHERNET ADAPTER                       �

�                                                                          �

�    DO NOT USE THE 3COM EL90X.OS2 NDIS DRIVER (3COM ETHERLINK XL 3C905    �

�    10/100 PCI ADAPTER) SINCE THE EL90X.OS2 DRIVER (AT LEAST THRU         �

�    RELEASE 3.0F 01/23/1998) IS SUBJECT TO DEVICE DRIVER TRAPS            �

�                                                                          �

����������������������������������������������������������������������������

����������������������

�                    �

� LANA CONFIGURATION �

�                    �

����������������������

The Local Area Network Adapter (LANA) configuration should be coordinated with

the P/390 TCP/IP (VM|MVS) PROFILE.  Additionally, if you are using EtherNet,

then the EtherNet Frame Type (DIX|IEEE) must be defined to the OS/2 802.2

Protocol Driver (LANDD.OS2).  The OS/2 LANA configuration is usually done by

a configuration aid (either LAPS (Lan Adapter and Protocol Support) or MPTS

(Multiple Protocol Transport System)) but also can be done by direct

modification of the CONFIG.SYS and PROTOCOL.INI files.

At the time of this writing, there are two packages which include LANDD.OS2

which LCS3172 uses to drive the LAN Adapters.  LAPS is used with base OS/2

versions while MPTS is used with the CONNECT and SERVER packages.  OS/2 LAN

Server also includes MPTS and will replace LAPS if installed over LAPS on

a base OS/2 system.

To configure, use the "LAPS.EXE" program found in the "IBMCOM" SubDirectory

(always on the boot drive) or the "MPTS.EXE" program found in "IBMCOM" (if

you have the MPTS package). If your LANA is not listed in the "Network

Adapters" window, then you will need to install the NDIS driver from the

diskette that came with the LANA.  Any LANA that has an OS/2 NDIS driver

OS/2 can be used.

You must add 802.2 protocol support to LANA used by LCS3172 (LCS3172 does not

use 802.2 data link protocol but does use the 802.2 Direct Interface which is

provided as a subset of the LANDD 802.2 protocol).

If you are using an EtherNet Adapter, then configure the 802.2 protocol option

"Type of EtherNet driver support" to match your EtherNet network.  Choose "D"

for DIX (the most common) or "I" for 802.3.  Whatever configuration method

you use, what actually governs this is the ETHERAND_TYPE parameter in the

LANDD$ section of PROTOCOL.INI.  AWSLCSDD obtains this "Type of EtherNet

driver support" from the ProtocolManager at system initialization time and

uses this selection as the default format for EtherNet frames on this

LANA.  All EtherNet frames received before P/390 TCP/IP sends LCS3172 an

EtherNet frame for transmission (LCS3172 has no knowledge of the P/390 TCP/IP

EtherNet frame format until LCS3172 actually receives an EtherNet frame from

P/390 TCP/IP) are presented to P/390 TCP/IP in this default format.

When (or if) LCS3172 learns the P/390 TCP/IP frame format and if that format

is different from the LANDD "Type of EtherNet driver support" configured,

then LCS3172 will attempt to "make" the IP connection by translating the ARP

frames.

LCS3172 LIMITATIONS

``````` ```````````

Since LCS3172 uses LANDD to actually transmit and receive the frames, you

must configure the LANDD ETHERAND_TYPE (DIX|IEEE) correctly.  Unlike the

P/390 TCP/IP situation discussed above, LCS3172 cannot make an adjustment

if the LANDD ETHERAND_TYPE is configured incorrectly.  More than 99% of the

EtherNets in use today will require that you change this value from the

LAPS/MPTS default "I" to "D".

NOTE: The LANDD ETHERAND_TYPE is a global parameter which applies to all

users of the LANDD 802.2 protocol.  If a "D|DIX" LANA is shared with

a P/390 VTAM/SNA 3172 (LSApLAN|LSApBMGR), then all of the EtherNet

SNA clients must use the DIX EtherNet frame format to communicate

with that P/390 VTAM/SNA 3172.

Since LCS3172 uses the LANDD 802.2 Direct Interface, and because LANDD allows

only one application at a time to use the Direct Interface, the LANA used by

LCS3172 cannot be simultaneously shared with other applications that use the

LANDD 802.2 Direct INTERFACE (eg., OS2 TCP/IP). If you want to use OS/2 TCP/IP

simultaneously with LCS3172, then you must install two LANA in your system --

one for OS/2 TCP/IP and one for LCS3172.

LANDD assigns each LANA a sequential number, beginning with zero, in the

order in which the LANA are listed in the BINDINGS= parameter in the LANDD$

section of PROTOCOL.INI.  Additionally, configuration of LANA should be done

by LANA slot order (lowest LANA slot number first, then proceed by increasing

LANA slot order).

LCS3172 support up to 4 LANA per system (numbers may range from 0 to 15).

In Systems with more than 4 LANA, LCS3172 will use the first 4 listed in the

BINDINGS= parameter in the LANDD$ section of PROTOCOL.INI.  Unless there is a

compelling reason to do otherwise, use a single LCS3172 Attachment with

multiple LANA.  In other words, define a single 3172 with multiple LANA instead

of multiple 3172's each with a single LANA (single 3172 with multiple LANA has

less overhead and will give better performance).

������������������������

�                      �

� TCP/IP CONFIGURATION �

�                      �

������������������������

The LANDD Number assigned during LANA configuration is used to select the

LANA in the TCP/IP PROFILE.  LCS3172 does not verify that the LANA Type

(IBMTR|ETHERNET) specified in the TCP/IP PROFILE matches the actual LANA

referenced by LANDD Number.  It is up to the user to coordinate the LANDD and

TCP/IP PROFILE configurations.

Example:

Token-Ring definition (TCPIP PROFILE)

; Hardware Definitions

; 3172 @ 440/441 TokenRing

DEVICE UNITY LCS 440 #��� this is the Even (READ) Channel Address

LINK TR440 IBMTR 0 UNITY

#

�������� LANDD Number

Example:

EtherNet definition (TCPIP PROFILE)

; Hardware Definitions

; 3172 @ 440/441 EtherNet

DEVICE UNITY LCS 440 #��� this is the Even (READ) Channel Address

LINK EN440 ETHERNET 1 UNITY

#

�������� LANDD Number

DEBUG AND TRACE OPTIONS

````` ``` ````` ```````

For debug and trace entries, add a debug parameter to the device manager

using the P/390 Configurator "Update System Devices" menu "Device Manager

Parm Edit/Display" submenu (F2/F12):

A  LCS3172   > W /DEBUG X|XX Y|YY RD TD NONE

#

�       ������Ŀ

�       � X    �    XSIDE (CHANNEL) AWSTRACE SUMMARY

�       �      �

�       � XX   �    XSIDE (CHANNEL) AWSTRACE DETAIL

�       �      �

�       � Y    �    YSIDE (LANA) AWSTRACE SUMMARY

�       �      �

�       � YY   �    YSIDE (LANA) AWSTRACE DETAIL

�       �      �

�       � RD   �    AWSTRCSP ReceiveFrameData

�       �      �

�       � TD   �    AWSTRCSP TransmitFrameData

�       �      �

�       � NONE �    NO AWSTRACE/AWSTRCSP

�       �      �

�       ����������  /DEBUG OPTIONS

���Ŀ

�   �

� N �   NO WINDOW

�   �

� T �   WATCH TEXT WINDOW

�   �

������� WINDOW OPTION

DEBUG NOTES: SUMMARY TRACE includes all ERROR/MAJOR EVENTS (MINIMUM/DEFAULT)

DETAIL TRACE adds MINOR EVENTS

/DEBUG NONE does not prevent posting ERROR/MAJOR to WATCH WINDOW

EXAMPLE TRACE DISPLAY

``````` ````` ```````

WATCH WINDOW is displayed when LCS3172 is started with TextWindow option ...

������������������������������������������������������������������������������Ŀ

� R/W 0440/0441 BLK 00000EAE/00000C7A FR 00001452/00001EDE XC 004C8410/00A17B9A�

� � LTLN LANA ADDRESS RECV FRAME 000004A0 LOST/LOW 00000000 LOST/OUT 00000000 ��

� � TR00 4000749000C8 XMIT FRAME 0000030D LOST/ERR 00000000 RCVBUF 00F8:00DD �ٳ

� � LTLN LANA ADDRESS RECV FRAME 00000FB2 LOST/LOW 00000000 LOST/OUT 00000000 ��

� � DE01 4000749001C8 XMIT FRAME 00001BD1 LOST/ERR 00000000 RCVBUF 0108:00E8 �ٳ

� ����������������������������������������������������������������������������ĳ

� 07/14 10:19:21 (XSIDE COMMAND 07/STARTUP) CCUU=0441                          �

� 07/14 10:19:21 (XSIDE COMMAND 01/STARTLAN) CMD LTLN=0200                     �

� 07/14 10:19:21 (XSIDE COMMAND 01/STARTLAN) CMD LTLN=0101                     �

� 07/14 10:19:21 (XSIDE RESPONSE 04|0D/LANSTAT|LANSTAT2) LANA=00/802           �

� 07/14 10:19:21 (XSIDE RESPONSE 04|0D/LANSTAT|LANSTAT2) LANA=01/DIX           �

� 07/14 10:34:22 (BLOCK FRAME XSIDE PRECONFIGURED DIX) LTLN=0101               �

� 07/14 12:09:19 (XSIDE HALT) READY->NOTREADY CCUU=0440                        �

� 07/14 12:09:19 (XSIDE RESET) YSIDE RECEIVE CANCELED LTLN=0200                �

� 07/14 12:09:19 (XSIDE RESET) YSIDE RECEIVE CANCELED LTLN=0101                �

� 07/14 12:09:19 (XSIDE HALT) READY->NOTREADY CCUU=0441                        �

� 07/14 15:31 ET 000045BA  BLOCK/S 0000  FRAME/S 0000  BYTE/S 00000369 LCS97195�

��������������������������������������������������������������������������������

�������������������Ŀ

�CHANNEL STATUS LINE�

���������������������

��Ŀ READ CHANNEL ADDRESS

��Ŀ WRITE CHANNEL ADDRESS

�����������������������������������������������������������������������������Ŀ

�R/W 0440/0441 BLK 00000EAE/00000C7A FR 00001452/00001EDE XC 004C8410/00A17B9A�

�������������������������������������������������������������������������������

READ BLOCK COUNT ��������

WRITE BLOCK COUNT ��������

READ FRAME COUNT ��������

WRITE FRAME COUNT ��������

READ BYTE COUNT ��������

WRITE BYTE COUNT ��������

�����������������Ŀ

�LANA STATUS LINES�

�������������������

RECEIVE OUT-OF-BUFFER CONDITIONS ������Ŀ

RECEIVE FRAMES LOST (LOW BUFFER) ������Ŀ

RECEIVE FRAME COUNT ������Ŀ

�����������������������������������������������������������������������������Ŀ

�� LTLN LANA ADDRESS RECV FRAME 000004A0 LOST/LOW 00000000 LOST/OUT 00000000 ��

�� TR00 4000749000C8 XMIT FRAME 0000030D LOST/ERR 00000000 RCVBUF 00F8:00DD �ٳ

�������������������������������������������������������������������������������

RECEIVE BUFFER POOL FREE BUFFERS (CURRENT:MINIMUM) ���������

�������� TRANSMIT ERRORS

�������� TRANSMIT FRAME COUNT

������������ FUNCTIONAL ADDRESS

�� LANDD NUMBER (TCP/IP PROFILE FORMAT)

�� LAN TYPE (TR=TOKENRING)(DE=DIX ETHERNET)(IE=IEEE 802.3 ETHERNET)

�����������������Ŀ

�TRACE INFORMATION�

�������������������

�07/14 10:19:21 (XSIDE COMMAND 07/STARTUP) CCUU=0441                          �

�07/14 10:19:21 (XSIDE COMMAND 01/STARTLAN) CMD LTLN=0200                     �

�07/14 10:19:21 (XSIDE COMMAND 01/STARTLAN) CMD LTLN=0101                     �

�07/14 10:19:21 (XSIDE RESPONSE 04|0D/LANSTAT|LANSTAT2) LANA=00/802           �

�07/14 10:19:21 (XSIDE RESPONSE 04|0D/LANSTAT|LANSTAT2) LANA=01/DIX           �

�������������������������������������������������������������������������������

The WATCH WINDOW has three parts.  The top displays LANA status by LCS3172

Channel Attachment and grows from the top down.  The top portion gets refreshed

every 10 seconds or after an entry is written to the bottom.

The bottom (except last line) contains MAJOR and ERROR Event information.  This

display grows from the bottom of the display towards the bottom of the top

portion. This display is useful for debugging initial installations and for

monitoring exception conditions.

�������������������Ŀ

�LANDD CONFIGURATION�  The third part of the WATCH WINDOW (the last line)

���������������������  BEFORE any STARTLAN Commands are received lists the

LANDD LAN (up to four) adapters found.  This is a

list of all possible LANDD LANA available for use

with LCS3172.  AFTER LCS3172 receives a STARTLAN

Command this line is replaced with the Rate

Information line listed below.

ڿ TOTAL NUMBER OF LANDD LANA DETECTED (AVAILABLE FOR USE)

03 LANA LANDDNUMBER/ADAPTERTYPE TR 00/0040 EN 01/0100 TR 0C/0010

LANA LIST ���������� ���������� ���������� ����������

�� LANA TYPE (TR=TOKENRING)(EN=ETHERNET)

�� LANDD LANA NUMBER (00:0F)

���� LANDD ADAPTERTYPE

the following is a partial list of possible LANDD Adaptertypes:

'0004'x IBM TokenRing Adapter/A

'0010'x unknown or OEM TokenRing Adapter

'0012'x IBM LANStreamer MC 32 Adapter

'0040'x IBM TokenRing 16/4 Adapter/A

'0100'x EtherNet Adapters

����������������Ŀ

�RATE INFORMATION�  The third part of the WATCH WINDOW (the last line) AFTER

������������������  a STARTLAN is received contains overall rate statistics

for LCS3172 and is refreshed every ten seconds.  BEFORE

a STARTLAN Command is received this line contains a list

of available LANDD LANA (see above).

DATA TRANSFER RATE (BYTES/SECOND) ������Ŀ

FRAME RATE (FRAMES/SECOND) ��Ŀ

BLOCK RATE (BLOCKS/SECOND) ��Ŀ

�07/14 15:31 ET 000045BA  BLOCK/S 0000  FRAME/S 0000  BYTE/S 00000369 LCS97195�

�������� ELAPSED TIME (IN SECONDS SINCE LCS3172 STARTED)*

�������� CURRENT TIME

LCS97195 (in the bottom right corner) is the LCS3172.EXE version number

* unless using GENSTAT2 command to reset the Channel statistics when LCS3172

is running with a single SubChannel address pair, then elapsed time is the

time since Channel statistics were last reset with a GENSTAT2 command.

DEFAULT RECEIVE BUFFER POOL CONFIGURATIONS

``````` ``````` `````` ```` ``````````````

�� ETHERNET ����������������������������������������������������������������Ŀ

�                                                                            �

� The following EtherNet ReceiveBufferPool configuration was selected based  �

� upon the operation of the AMD PCNET (on PC Server/330 planar) and 3COM     �

� OS/2 NDIS drivers which are able to only use 7 LANDD (SAP) buffers per     �

� EtherNet frame (1492 byte maximum I-field length) ...                      �

�                                                                            �

�   ETHERNET RECEIVE BUFFER POOL - 264 BUFFERS OF 240 BYTES                  �

�                                                                            �

�                                   33 BUFFERS SHORTAGE THRESHOLD            �

�                                                                            �

�                                   66 BUFFERS RECOVERY THRESHOLD            �

�                                                                            �

�   For EtherNet, the formula is as follows ...                              �

�                                                                            �

�   FrameSize = (BufferSize-62) + (BufferSize-12)*(NDISBuffers-1)            �

�                                                                            �

�   NumberOfBuffers = 63488 / BufferSize                                     �

�                                                                            �

�   so for /RBEN 240 and an NDIS driver that uses up to 7 buffers            �

�                                                                            �

�   FrameSize = 1546 = (240-62) + (240-12)*(7-1)                             �

�                                                                            �

�   NumberOfBuffers = 264 = 63488 / 240                                      �

�                                                                            �

� The IBM MicroChannel Streamer EtherNet adapters (PC Server/500) are able   �

� to use up to 9 LANDD (SAP) buffers per EtherNet frame (1492 byte maximum   �

� I-field length) and are able to use a 192 byte buffer.                     �

�                                                                            �

������������������������������������������������������������������������������

�� TOKENRING ���������������������������������������������������������������Ŀ

�                                                                            �

� The following TokenRing ReceiveBufferPool configuration was selected based �

� upon the operation of the IBM Streamer TokenRing adapters (PC Server/330   �

� and PC Server/500) OS/2 NDIS drivers which are able to use 9 LANDD (SAP)   �

� buffers per TokenRing frame (2048 byte maximum I-field length) ...         �

�                                                                            �

�   TOKENRING RECEIVE BUFFER POOL - 248 BUFFERS OF 256 BYTES                 �

�                                                                            �

�                                    31 BUFFERS SHORTAGE THRESHOLD           �

�                                                                            �

�                                    62 BUFFERS RECOVERY THRESHOLD           �

�                                                                            �

� TokenRing permits frame sizes up to about 18000 bytes.  Users wanting to   �

� experiment with TokenRing frame sizes larger than about 2150 bytes will    �

� need to increase the ReceiveBufferSize (see method and WARNING below).     �

�                                                                            �

�   For TokenRing, the formula is as follows ...                             �

�                                                                            �

�   FrameSize = (BufferSize-24) + (BufferSize-16)*(NDISBuffers-1)            �

�                                                                            �

�   NumberOfBuffers = 63488 / BufferSize                                     �

�                                                                            �

�   so for /RBTR 256 and an NDIS driver that uses up to 9 buffers            �

�                                                                            �

�   FrameSize = 2152 = (256-24) + (256-16)*(9-1)                             �

�                                                                            �

�   NumberOfBuffers = 248 = 63488 / 256                                      �

�                                                                            �

������������������������������������������������������������������������������

RECEIVE BUFFER MANAGEMENT

``````` `````` ``````````

LCS3172 ReceiveBufferPools configuration may vary by LAN type. When the

number of free buffers in the ReceiveBufferPool falls below a minimum number

(SHORTAGE TRESHOLD) a ReceiveBufferLow condition occurs.  All frames received

during a ReceiveBufferLow condition are counted and immediately discarded (are

not passed from the LANA task to the BLOCKER task).  If the LANA task (using

the remaining ReceiveBuffers as a ReceiveCountingPool) is unable to keep up

with the receive traffic a ReceiveOutOfBuffers condition occurs and the LANA

task suspends all receive frame processing until the ReceiveBufferPool is

recovered (more than RECOVERY THRESHOLD ReceiveBuffers become available by

blocking them as ReadData across the ReadSubChannel).  Once the ReceiveBuffer

Pool is recovered, the receive process is restarted.

The LANA receive counts in the WATCH WINDOW will be accurate as long as no

ReceiveOutOfBuffer conditions occur.  Each ReceiveOutOfBuffer condition is

counted once but always represents an unknown number of lost receive frames.

ReceiveBufferLow processing is done at Ring0 (a LANDD appendage at interrupt

time) so as to make every attempt to avoid a ReceiveOutOfBuffer condition.

Two FreeBuffer counts are given after the RCVBUF label on the second LANA

status line.  The first value is a sample of the current ReceiveBufferPool

FreeBuffer count and usually will reflect the maximum buffer count.  If this

value is consistently less than the maximum value, then that may be an

indication that some ReveiveBuffers have been "lost".  The second value is

the minimum FreeBuffer count observed since LCS3172 was started and provides

an indication of worst case ReceiveBufferPool capacity.

Both ReceiveBufferPool configurations (EtherNet and TokenRing) may be

specified as parameters to the LCS3172 device manager.

WARNING: at the time of this writing, there are bugs in LANDD in handling

ReceiveOutOfFBuffer conditions and there is another bug in LANDD

which causes traps in LANDD.OS2 if certain values (the ones

selected as defaults seem to work) are used for ReceiveBufferSize.

SO PLEASE CONTACT P/390 SUPPORT IF YOU HAVE A REQUIREMENT FOR

DIFFERENT RECEIVEBUFFERPOOL CONFIGURATIONS

IF, IN LIGHT OF THE ABOVE WARNING, YOU STILL WANT TO EXPERIMENT

WITH THE RECEIVEBUFFERPOOL CONFIGURATION THEN THE FOLLOWING MAY

BE PASSED AS PARAMETERS ON THE AWSSTART LCS3172.EXE LINE ...

for EtherNet .... /RBEN SIZE NUMBER SHORTAGE RECOVERY

for TokenRing ... /RBTR SIZE NUMBER SHORTAGE RECOVERY

and the following are PositionalParameters ...

SIZE (ReceiveBufferSize) is a number

which is a multiple of 16 from 128 to 2048

NUMBER (Number of ReceiveBuffers) is a number

from 16 to 63488/SIZE

SHORTAGE (ReceiveBufferPool Shortage Threshold) is a number

from 1 to NUMBER  (default is NUMBER/8)

RECOVERY (ReceiveBufferPool Recovery Threshold) is a number

from 1 to NUMBER (default is NUMBER/4)

subject to the following rules ...

the parameters are positional and only the following three

configurations are permitted ...

(SIZE) (SIZE NUMBER) (SIZE NUMBER SHORTAGE RECOVERY)

if just (SIZE) is specified then NUMBER=63488/SIZE

SHORTAGE=NUMBER/8

RECOVERY=NUMBER/4

if just (SIZE NUMBER) are specified then SHORTAGE=NUMBER/8

RECOVERY=NUMBER/4

if (SIZE NUMBER SHORTAGE RECOVERY) are all specified

then SHORTAGE < RECOVERY (both must be specified together)

Since anything other than a DecimalNumber(0:9) is parsed as a

delimiter, the following are the default specifications ...

/RBEN (SIZE 240 COUNT 264 SHORT 33 RECOVER 66)

/RBTR (SIZE 256 COUNT 248 SHORT 31 RECOVER 62)

END LCS3172.DOC 17MAR1998/JLH