AWSOMA:  Optical Media Attach

=============================

AWSOMA reads a CDROM in OMA format.  IBM uses this format to ship program

products, fixes, publications, etc.  AWSOMA reads a "Tape Descriptor File"

(TDF) which describes the format and location of files on the CDROM and the

logical organization of those files on the "tape."  For more information the on

the TDF and Optical Media Attach, see the following publications:

SC52-1201-00 Optical Media Attach/2 Technical Reference

SC53-1200-00 Optical Media Attach/2 User's Guide

DEVICE TYPE:  3420 (Control unit 3803) or 3422 (Control unit 3422)

============

Note:  for VSE and VM, 3422's are supported as device type 3430.

CONFIGURING AWSOMA

==================

To configure the OMA, go to the Configurator F2 "UPDATE SYSTEM DEVICES"

menu and enter the real address of a 3422/3420 tape device in the Addr

field, the MGR code for AWSOMA in the Mgr field, and the fully qualified

name of the tape descriptor file (TDF) on the CDROM.

Addr  Device                         Mgr    FN/P

------------------------------------------------------------------

>580   >3420                         >D   >K:\TAPES.\UAA001.TDF

THE VM MOUNT COMMAND

====================

To change the TDF filename from VM while it is running, use the CMS MOUNT

command. Attach the tape to your userid and issue the mount command:

MOUNT 181 (QUERY        to display the name of the current tape file

MOUNT 181 (STACK        to put the name of the current file on the stack

MOUNT 181 pcfilename    to change the name

Enter HELP MOUNT from a P/390 CMS session for additional information.

THE VSE TMOUNT COMMAND

======================

To change the TDF filename while VSE is running, use the VSE TMOUNT

program with the following JCL:

To query the TDF file associated with device 500:

//ASGN SYS004,500

//EXEC TMOUNT,SIZE=AUTO,PARM='500 (QUERY'

To assign a new TDF file to be associated with device 340:

//ASGN SYS004,500

//EXEC TMOUNT,SIZE=AUTO,PARM='340 K:\CDROM\XYZ.TDF'

THE OS/2 AWSMOUNT COMMAND

=========================

You can use the OS/2 AWSMOUNT command to change the filename that is

associated with the emulated tape or to issue Rewind and Rewind Unload

commands. See AWSMOUNT.DOC or enter AWSMOUNT ? from an OS/2 prompt.

OTHER CONSIDERATIONS

====================

Although AWSOMA was designed to read CDROMS, you can create your own TDF

file on a normal hard drive and use that to load data into the P/390

system.  However, AWSOMA only supports reading data, so you cannot

use it to move data from the P/390 to the PC.

NOTE: All characters in the TDF file must be in UPPERCASE!

The P/390 I/O subsystem does not support the READ BACKWARDS CCW unless

the SKIP bit is on.  This may affect certain tape utility programs.  It

applies to all tapes, whether emulated, SCSI-attached, or Bus & Tag

Channel-attached.