DISPLAY Modes:

bios              adapter num    buffer char

mode description  types  colors  start  size

0   40x 25 A/N  C/E     16,    B8000, 8x8

1   40x 25 A/N  C/E     16,    B8000, 8x8

2   80x 25 A/N  C/E     16,    B8000, 8x8

3   80x 25 A/N  C/E     16,    B8000, 8x8

4  320x200 APA  C/E      4,    B8000, 8x8

5  320x200 APA  C/E      4,    B8000, 8x8

6  640x200 APA  C/E      2,    B8000, 8x8

7   80x 25 A/N  C/E/M   4M,    B0000, 9x14

D  320x200 APA  E	  16,    A0000, 8x8

E  640x200 APA  E	  16,    A0000, 8x8

F  640x350 APA  E       4M,    B0000, 9x14

10  640x350 APA  E	   4,    A0000, 8x14

11  720x350 APA  H   	   4,    A0000, 8x14

MF  640x350 APA  E       4M,    B0000, 9x14

M10  640x350 APA  E	  16,    A0000, 8x14

E0   40x 25 A/N  E       16,    B8000, 8x14

E1   40x 25 A/N  E       16,    B8000, 8x14

E2   80x 25 A/N  E       16,    B8000, 8x14

E3   80x 25 A/N  E       16,    B8000, 8x14

Notes:

A/N	Alphanumeric

APA	All Points Addressable

(alphanumeric display is not presently supported)

C	Adapter type = CGA

E	Adapter type = EGA

M	Adapter type = Mono adapter

H	Adapter type = Hercules (not BIOS compatible)

Mx	Same mode with ega having more than 64k memory.

Ex	Same mode when an Enhanced Color Display is attached.

Num Colors = 4M refers to different mono attributes:

Color	Attribute

0x0	Black

0x3	White

0xC	Blinking

0xF	Intensified

Standard Color Mapping for Mode 10:

Character	Monochrome	Color Display	Color Display

Attribute	Display		64Kb memory	> 64Kb memory

---------	-------		-----------	-------------

0x00		Black		Black		Black

0x01		White		Blue		Blue

0x02		Black		Black		Green

0x03		White		Blue		Cyan

0x04		Blinking	Red		Red

0x05		Intensified	White		Magenta

0x06		Blinking	Red		Brown

0x07		Intensified	White		White

0x08		Black		Black		Dark Grey

0x09		White		Blue		Light Blue

0x0A		Black		Black		Light Green

0x0B		White		Blue		Light Cyan

0x0C		Blinking	Red		Light Red

0x0D		Intensified	White		Light Magenta

0x0E		Blinking	Red		Yellow

0x0F		Intensified	White		Intensified White