README.DOC

==========

FHDX 3.4 currently supports the following drives:

(1) STACY

i) CON3021 (Conner 20Mb - CP3020)

ii) CON3042 (Conner 40Mb - CP3040)

(2) MEGAFILE 60 -

i) MCS5068 (Microscience 65Mb)

ii) MSB5064 (Miniscribe 60Mb)

iii) SEA5065 (Seagate 60Mb)

iv) NO LABEL 60 (Miniscribe 60Mb)

(3) MEGAFILE 44 (Syquest 44Mb)

(4) MEGAFILE 30 (Seagate 30Mb)

(5) MEGAFILE 20 -

i) SEA5020 (Seagate 20Mb)

ii) NO LABEL 20

(6) SH205

(7) SH204

(8) SH104

The correct sequence of the test procedure is :

Select Unit (optional) --> Select Disk Model (optional)

--> Controller Test --> Daisy-chain Test --> Format

--> Input Defect List (optional) --> Burn-in Test --> Ship

as displayed on the menu bar.  But, each function  can  also

be run independently in no specific order.

You only need to SELECT UNIT or SELECT DISK MODEL  when  the

current one is not what you want.  Both the current unit and

current disk model are displayed on the right hand  side  of

the menu bar.

You only need to INPUT DEFECT LIST when the  vendor  of  the

disks provide a defect list.

WARNING!!

ALL functions, EXCEPT Daisy-Chain Test and  Ship,  are  des-

tructive.   Please  be sure to back up everything (if possi-

ble) that you may want before you proceed.

October 2, 1989

- 2 -

Functional description of each program:

---------------------------------------

(1)  CONTROLLER TEST --

This program tests if the controller  interacting  with

the  drive  is functioning correctly.  If the unit is a

SCSI drive, it will do a test unit ready on the  drive.

If  the unit is a non-SCSI drive, it will try to format

the drive as a 2-head, 20-cylinder and 17  sectors  per

track device (to make it efficient).  The format param-

eters for this test are specified in  the  WINCAP  file

under the 'ct' entries.

(2)  DAISY-CHAIN TEST --

This program tests if the daisy-chain port  works.   It

will  do  a non- destructive markbad on unit 1 (for 128

sectors, to be efficient).  The program will first save

the  data the 128 sectors on unit 1, WRITE a worst case

test pattern to that 128 sectors, READ data  back  from

that  128  sectors,  VERIFY  that the data read back is

exactly the same as what was written onto it, and WRITE

the  original  data  back onto those 128 sectors.   The

parameters for this test are specified  in  the  WINCAP

file under the 'dc' entry.

(3)  FORMAT --

This program formats the entire surface  of  the  drive

and  creates  a  default partition when format is done.

The default partition schemes for the different  drives

are  specified  in the WINCAP file.  There is no FORMAT

track command available.  This FORMAT program will  NOT

delete  existing defective sector information logged on

the drive earlier  (those  entered  manually  and  also

those found during the burn-in test).  The program will

save the defective sector information from  the  begin-

ning  of  the  drive, format it, partition it, and then

write the defective sector information  back  onto  the

drive.  The FORMAT program and the PARTITION program in

HDX 2.x (and  onwards)  will  also  save  the  existing

defective  sector  information  logged  on the drive by

FHDX.

October 2, 1989

- 3 -

(4)  INPUT DEFECT LIST --

This program logs at the beginning of  the  drive,  the

defects entered manually from the defect list generated

by the vendors.  These defects  inputted  will  NOT  be

deleted by the FORMAT program or the BURN-IN program in

FHDX, and will NOT be deleted by the FORMAT program  or

the PARTITION program in HDX 2.x (and onwards).

(5)  BURN-IN TEST --

This program  tests  all  sectors  of  the  drive,  one

cylinder at a time, in the following order -

0, (N/4), 1, (N/4)+1, ... (N/4)-1, (N/2),

(N/2), (3N/4), (N/2)+1, (3N/4)+1, ... (3N/4)-1, N.

where N is the number of cylinders on the drive.   This

order is much faster than the 0, N, 1, (N-1), 2, (N-2),

... order.

The program was designed to test the cylinders  in  the

order  specified above, so as to do seek tests combined

with  write/read/verify  tests.   When   testing   each

cylinder, the program will SEEK to that cylinder, WRITE

a worst case test pattern (as specified in  the  WINCAP

file)  to every sector on that cylinder, READ data back

from that cylinder, and VERIFY that the data read  back

is  exactly the same as what was written to it.  A sec-

tor will be logged on the drive as bad if  there  is  a

write  error, a read error or a data error on that sec-

tor.  The program also does a  4-bit  rotation  on  the

test  pattern  for each cycle of the burn-in test (e.g.

For RLL drives, test pattern for  cycle  1  is  0x6333,

test pattern for cycle 2 is 0x3633, etc...).

All bad sectors found on a  drive  during  the  burn-in

test will be logged at the beginning of the drive.  The

logged defective sector information will  be  preserved

by the FORMAT program and the INPUT DEFECT LIST program

in FHDX, and also by the FORMAT program and the  PARTI-

TION program in HDX 2.x (and onwards).

October 2, 1989

- 4 -

(6)  SHIP --

This program parks the heads  of  a  drive.   You  MUST

switch  off  the power of the drive once its heads have

been parked.

October 2, 1989