- Parameter Consistency Check using MoShell

1	Introduction	1

1.1	Revision history	1

1.2	Purpose	1

1.3	Prerequisite	2

2	Procedure	2

2.1	Check validity of parameter reference file	3

2.2	Perform the parameter audit	5

2.3	Analyse the result	8

Appendix	Creating a reference file from Winnow	6

# Introduction

Radio Access Network for 3G includes a lot of parameters that have to be configured. Many different people might have been involved in this process. Finding errors or non-optimal behaviour is facilitated if the current parameter settings are known and can be compared to a set of reference values. This is an essential part of a consistency check. This document describes how to achieve this using the diff command in MoShell.

## Revision history

| Revision | Date | Description |
| --- | --- | --- |
| A | 2008-01-14 | First version with new document number. |
| B | 2008-02-13 | Added ref to 32/1551-AXD10503/1 in chap 2.3 |
| C | 2009-04-04 | Updated for P7 |
| D | 2010-03-04 | Updated for W10 |
| E | 2011-12-22 | Updated for W11 |
| F | 2012-08-26 | Updated for W12 |

## Purpose

This document is intended to describe the procedure for checking the actual parameter settings against a reference list with recommended values.

## Prerequisite

IMPORTANT:  Always make sure to use the latest MoShell version. The audit may not be accurate if an older MoShell version is used !!!! 
The latest MoShell version can be downloaded from http://utran01.au.ao.ericsson.se/moshell or run directly from /vobs/iov/rnc/bin/moshell , using the following rule in the config spec:
element /vobs/iov/rnc/bin/moshell/... /main/LATEST

# Procedure

The procedure has the following steps:

- Check that the parameter files stored in moshell/commonjars/pm are up to date

- Perform the parameter audit. Two methods are described, a one-step method and a two-step method.

- Analyse the result

The different steps are described in the following sections.

Note: for more information, please refer to the moshell/mobatch documentation. 
 - For mobatch information, type “mobatch” on its own, at the unix prompt. 
 - For information about the MoShell “diff” command, just type “h diff” at the MoShell prompt.

## Check validity of parameter reference file

The parameter reference file used by diff consists of a tabbed ASCII file with 3 columns:

- MO class name or RDN/LDN of the MO

- Parameter (=Attribute) name

- Required value

These files are stored in the directory moshell/commonjars/pm, under the following names:

| RAN Release | RNC | RBS | RXI |
| --- | --- | --- | --- |
| P3 | PARAM_RNC_E_1_0.txt | PARAM_RBS_C_1_0.txt | PARAM_CPP_3.txt |
| P4 | PARAM_RNC_F_1_0.txt | PARAM_RBS_H_1_0.txt | PARAM_CPP_4.txt |
| P5 | PARAM_RNC_G_1_0.txt | PARAM_RBS_J_1_0.txt | PARAM_CPP_5.txt |
| P6 | PARAM_RNC_H_1_0.txt | PARAM_RBS_K_1_0.txt | PARAM_CPP_6.txt |
| P6.1 | PARAM_RNC_J_1_0.txt | PARAM_RBS_K_1_0.txt | PARAM_CPP_6.txt |
| P7 | PARAM_RNC_K_1_0.txt | PARAM_RBS_L_1_0.txt | PARAM_CPP_7.txt |
| W10 | PARAM_RNC_L_1_0.txt | PARAM_RBS_M_1_0.txt | PARAM_CPP_7.txt |
| W11 | PARAM_RNC_M_1_0.txt | PARAM_RBS_N_1_0.txt | PARAM_CPP_8.txt |
| W12 | PARAM_RNC_N_1_0.txt | PARAM_RBS_S_1_0.txt | PARAM_CPP_9.txt |

Open the file(s) relevant for the node(s) you are going to audit and check the revision information stored at the beginning of the file. 
Note: the MOM version of the node can be checked with MoShell command “pv mom”.

Compare that this corresponds to the latest file version stored in CDM.
The corresponding document numbers in CDM are listed here:

| RAN | RNC | RBS | RXI |
| --- | --- | --- | --- |
| P3 | 3/190 59-HSD 101 02/3 | 4/190 59-HSD 101 02/3 | 5/190 59-HSD 101 02/3 |
| P4 | 3/190 59-HSD 101 02/4 | 4/190 59-HSD 101 02/4 | 5/190 59-HSD 101 02/4 |
| P5 | 3/190 59-HSD 101 02/5 | 4/190 59-HSD 101 02/5 | 5/190 59-HSD 101 02/5 |
| P6 | 3/190 59-HSD 101 02/6 | 4/190 59-HSD 101 02/6 | 5/190 59-HSD 101 02/6 |
| P7 | 3/190 59-HSD 101 02/7 | 4/190 59-HSD 101 02/7 | 5/190 59-HSD 101 02/7 |
| W10 | 3/190 59-HSD10102/10 | 4/190 59-HSD10102/10 | 5/190 59-HSD10102/10 |
| W11 | 3/190 59-HSD10102 B | 4/190 59-HSD10102 B | 5/190 59-HSD10102 B |
| W12 | 3/190 59-HSD10102 C | 4/190 59-HSD10102 C | 5/190 59-HSD10102 C |

If the PARAM file in moshell/commonjars/pm is not up to date, then update it by copy-pasting the contents of the excel sheet into the text file.

Note that from W11, there is no more suffix at the end of the document number. The RAN release is given by the revision. Example rev B=W11A, rev B1=W11B

## Perform the parameter audit

Note: all types of nodes and sw versions can be audited at the same time, ie, RNC with RBS, P4 with P5, etc.

### Two-step method

1) First read the parameters from the node(s)

From the unix prompt (shown with the % sign), do:

    1a) If one node only

% moshell <ipaddress> ‘lt all ; kget’

==> the result is saved to a logfile, let’s call it kget_file

1b) If many nodes

% mobatch <sitefile> ‘lt all ; kget’

==> the result logs are saved in a directory, let’s call it kget_dir

2) Then perform the audit

The audit can either be performed against one of the default reference files located in moshell/commonjars/pm. In this case, it is not necessary to specify a reference file since MoShell will automatically choose the appropriate for the specific node type and node SW. 
Alternatively, it is possible to specify a different reference file.

    2a) Using the default reference file(s)

From the MoShell prompt (shown with the > sign), do:

If one node only:

> diff kget_file

If many nodes:

> diff kget_dir/

Note: if the result files need to be put in a specific place, this can be specified as the fourth word on the command line, in which case, the third word shall be “default”. Eg:

> diff kget_file default output_dir/

or, if many nodes:

> diff kget_dir/ default output_dir/

2b) Using a specific reference file

From the MoShell prompt, do:

If one node only:

> diff kget_file myreferencefile.txt

If many nodes:

> diff kget_dir/ myreferencefile.txt

Note: if the result files need to be put in a specific place, this can be specified as the fourth word on the command line. Eg:

> diff kget_file myreferencefile.txt output_dir/

or, if many nodes:

> diff kget_dir/ myreferencefile.txt output_dir/

### One-step method

a) Using the default reference file(s)

From the unix prompt, do:

If one node only:

% moshell <ipaddress> ‘lt all ; diff . ‘

If many nodes:

% mobatch <sitefile> ‘lt all ; diff . ‘

Note: if the result files shall be put in a specific place, this can be specified as the fourth word on the command line, in which case the third word shall be default. E.g:

% moshell <ipaddress> ‘lt all ; diff . default output_dir/’

or:

% mobatch <sitefile> ‘lt all ; diff . default output_dir/’

b) Using a specific reference file

From the unix prompt, do:

If one node only:

% moshell <ipaddress> ‘lt all ; diff . myreferencefile.txt‘

If many nodes:

% mobatch <sitefile> ‘lt all ; diff . myreferencefile.txt‘

Note: if the result files shall be put in a specific place, this can be specified as the fourth word on the command line. E.g:

% moshell <ipaddress> ‘lt all ; diff . myreferencefile.txt output_dir/’

or, if many nodes:

% mobatch <sitefile> ‘lt all ; diff . myreferencefile.txt output_dir/’

## Analyse the result

The result is shown on screen with space-separated fields and also saved in the result directory as a CSV file with 4 fields, separated by semicolons.

- MO identity

- Attribute

- Current Value

- Required Value

Each row shows a parameter with a different value than the reference.

The CSV file can be opened in excel for easier viewing.

A command file is also generated to correct the discrepancies. The command file can be run using the MoShell command “run” or can be converted to EMAS format using the “u!” command, in order to be run in EMAS.

If the audit was done on many nodes in parallel, the command files will be stored in the output  directory with the extension “.cmd” and can be run with mobatch. Eg:

mobatch <sitefile> output_dir/

IMPORTANT: Always go through the result file before running the command file(s) and correct the command file(s) if necessary before running:

- Some discrepancies may just be due to syntax differences between the reference file and the node.

- Some discrepancies in RNC UeRc parameters may appear when the document 3/19059-HSD10102 is not up to date with the document 32/1551-AXD10503/1.

Appendix: format of the reference file

The format of the reference values depends on the data type of the attribute.

Example:

- integer: 

Aal2Ap	timerErq	5000

 - struct:

Aal2QosProfile	profileClassA.boundOnNodeDelay	10000

 - enum:

AtmPort	hecCorrectionMode	TRUE(1)

 - struct containing an enum:

Mtp3bSpTtc	spPriority.prioCb	HIGHEST(3)

 - sequence of integers:

UeRc=1,UeRcTrCh=1	maxTfSubset	[4] = -1 -1 -1 -1

 - sequence of enums:

UtranCell	accessClassesBarredCs	[16] = FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0) FALSE(0)

 - sequence of structs:

EthernetSwitch      pbitQueueMap [8] = 0,1;1,0;2,1;3,1;4,2;5,2;6,3;7,3
EthernetSwitchPort  pbitQueueMap [8] = 0,0;0,1;1,2;1,3;2,4;2,5;3,6;3,7
EthernetSwitchModulePort  pbitQueueMap [8] = 1,0;0,1;2,2;3,3;4,4;5,5;6,6;7,7
GigaBitEthernet dscpPbitMap [64] = 0,0;1,0;2,0;3,0;4,0;5,0;6,0;7,0;8,0;9,0;10,1;11,0;12,1;13,0;14,1;15,0;16,0;17,0;18,3;19,0;20,3;21,0;22,3;23,0;24,0;25,0;26,4;27,0;28,4;29,0;30,4;31,0;32,0;33,0;34,5;35,0;36,5;37,0;38,5;39,0;40,0;41,0;42,0;43,0;44,0;45,0;46,6;47,0;48,0;49,0;50,0;51,0;52,0;53,0;54,0;55,0;56,0;57,0;58,0;59,0;60,0;61,0;62,0;63,0
InternalEthernetPort dscpPbitMap [64] = 0,0;1,0;2,0;3,0;4,0;5,0;6,0;7,0;8,0;9,0;10,1;11,0;12,1;13,0;14,1;15,0;16,0;17,0;18,3;19,0;20,3;21,0;22,3;23,0;24,0;25,0;26,4;27,0;28,4;29,0;30,4;31,0;32,0;33,0;34,5;35,0;36,5;37,0;38,5;39,0;40,0;41,0;42,0;43,0;44,0;45,0;46,6;47,0;48,0;49,0;50,0;51,0;52,0;53,0;54,0;55,0;56,0;57,0;58,0;59,0;60,0;61,0;62,0;63,0
ManagedElement healthCheckSchedule [2] = 00:00,1 (SUNDAY);02:30,4 (WEDNESDAY)

- sequence of moRef:

Synchronization syncReference [8] = Equipment=1,Subrack=MS,Slot=4,PlugInUnit=1,TimingUnit=1,TuSyncRef=1 Equipment=1,Subrack=MS,Slot=6,PlugInUnit=1,ExchangeTerminal=1,Os155SpiTtp=1

- Appendix: Creating a reference file from Winnow

Currently all the reference files are stored in CDM as described in chapter 2.1.

It is also possible to generate a reference file directly from Winnow, using the following procedure. The reference file will contain all parameters with a recommended value.

Enter http://winnow.wrn.ki.sw.ericsson.se in a web browser.

Login – no password needed.

Select Product Release.

Select an appropriate Product Delivery.

Select Search Parameter from the menu.

Check the box for Advanced search.

Select MOC Name, Parameter Name and Recommended Value to be included in the result list.

In the filtering text box for Recommended Value, enter the text is not null.

Submit

Save to file – give the file an explanatory name and store it in a local directory.