Notes on Harris RT-1446 Command Syntax when v601C Firmware is installed.

By

Ken Fowler, KO6NO

January 19, 2011

The response to “?<” is the Status String: (< is LF>

!0<#0<$0<&0<*0<"1<'1<(1<,0<A2<D1<H1<I0<M1<R1<S0<V0<Y0<Z0<E0<O1<T1<U<C;;<F1234567<B100<X000<.<

The response is the same for both Adaptive and Non-Adaptive Baud Rates.

1. Meter shows Transmit Power when keyed by PTT while in REMOTE.  However, there are no Power Values displayed. While Transceiver is keyed by PTT, Response to "U" is "K".

2. RF Gain can be set by “Gxx<”. 00 = Min, 63 = Max, No Response.  No Gxx< in status.  If xx is greater than 63 and less than 100, it is ignored. If xx = 100, Gain corresponds to xx = 10.  Only the first two digits are used.

3. Mode command is sent by either upper or lower case “[M|m]<” with no value. Response is “Mx<”.  The value x steps in sequence 1 –> 2 -> 3 -> 4 -> 1 … while  Mode changes from USB -> LSB -> AME -> CW -> LSB … When Mode steps to [USB,LSB, CW] AGC goes to SLOW (A2).  When Mode steps to AME, AGC goes to FAST (A4). When Mode goes to CW, Audio Source goes to OFF (H4 in status).  When Mode steps to USB, Audio Source goes back to original value. Sidetone status is reported in response to Mode command. When Mode goes to CW, Sidetone goes to ON (S1). When Mode goes to (USB, LSB, AME), Sidetone goes to OFF (S0) but may be toggled to ON. When Mode goes to CW, PTT goes to OFF.  PTT does not return to ON when Mode goes from CW to USB.

4. AGC command is sent by either upper or lower case “[A|a]<”.  Response is “Ax<”.  The value of x steps in sequence 2 -> 3 -> 4 -> 2 … while AGC changes from SLOW -> MEDIUM -> FAST -> SLOW … .  AGC can not be set to OFF.

5. Audio Source command is sent by either upper or lower case “[H|h]<”.  Response is “Hx<”.  The value of x steps in sequence 1 -> 2 -> 3 -> 1 … while the Audio Source changes from MIC -> AUD2 -> PATCH -> MIC …. Note the change to “H4” in CW Mode.

6. Mute command may be sent as “(<” (No value) and response is “(x<”.  Muting toggles.  “(0” means Mute is ON, “(1” means Mute is OFF.

7. BPF on/off command may be sent as “,<” and response is “,x<”. BPF condition toggles. “,0” means BPF is Enabled, “,1” means BPF is Bypassed.

8. BFO control and response are same as v604M firmware. When BFO is on, “+ BFO ON” or “- BFO ON” appears at right end of Frequency.

9. Clipping goes to OFF “I0” when Audio Source goes to AUD2.  Clipping will not turn on if AUD2 is selected.  Clipping may be toggled between On and Off if MIC or PATCH are selected.

10. PTT, as noted, changes to OFF when Mode goes to CW and does not return to ON when Mode goes to USB.  PTT may be toggled ON when in any Mode but command is sent as “’”<, no value. Response is “’1”< or “’0”<. ON is 1, OFF is 0. (This is different than v604M.)

11. Toggle between Remote and Local Audio is commanded by “””, a single quote with no value.  Echo is “”0”< or “”1”<.  Remote is “”0”, Local is “”1”.