NAME

RLC - Radio List Compiler for Wookie field programmer.

SYNOPSIS

RLC [[<in>] <out>]

where in defaults to MODEL.LST and out defaults to MODEL.DAT

DESCRIPTION

Builds model list for the wookie field programmer. Input is taken from

the ascii file *in* and translated to the binary file *out*. The syntax

for the compiler is:

Each model is defined in a type block:

TYPE <radio_type>

.

(here goes the model definition)

.

END

The model definition is the following:

DESC <string>       : String description. Can be anything up to 40 chars.

DEF <file_name>     : Default codeplug file: must reside in LIB.

DEF1 <file_name>    : Default codeplug file: must reside in LIB.

DEF2 <file_name>    : Default codeplug file: must reside in LIB.

FREQ <a> <b> <c>    : Min/Max frequency (kHz) and base frequency of radio.

NFRQ <a>            : Maximum simultanous defined frequencys (default 250)

SPAC [123]          : Number of channel spacings:

1: 12.5kHz 3: 12.5/20/25kHz (default 3)

POW [1234]          : Number of power levels (default 4)

HEAD <h>            : Control head: (C, E, I, J, no default)

DISP <r> <c> [C]    : Rows and colums of display (default 2X14), and

case support ('C': supports lower case)

And the flags

MOBILE              : For mobiles

PORTABLE            : For portables

KEYPAD              : For radios with keypad

ROTARY              : For radios with rotary

CROTARY             : For radios with continous rotary

The default codeplug can thus be defined with one or two default files:

DEF file.cp

or

DEF1 file1.cp

DEF2 file2.cp

DEF1 and DEF are here equivalent. If two files are used, the files are

read sequentially: file2.cp thus overwrites the data of file1.cp. The

two file definition is thus only usefull if one of the files only contains

part of the codeplug (ie. the internal or the external).

EXAMPLE

An example input file may contain the following block:

TYPE H01KDH9AN7AN

DESC Portable 1-5W

FREQ 136000.0 178000.0 103000.0

POW  2

SPAC 1

HEAD E

PORTABLE

KEYPAD

DISP 1 14

END

VERSION

1.0

FILES

RLC.DOC          This document

LIB\RLC.EXE      The compiler executable

LIB\MODEL.LST    Supplied model list

LIB\MODEL.DAT    Compiled list used by Wookie

BUGS

Current version does not handle errors very well: don't stress it.

COPYRIGHT

(c) Motorola Inc. 1994