==================================================================

DOS / PC / RSS Information                  For Motorola RSS Users

DOS_INFO.DOC   15sep97                  Copyright(c) Motorola Inc.

Radio Software Technology Center        LMPS, Ft. Lauderdale,  FLA

==================================================================

This information is intended to aid our RSS Users in understanding

how the PC, DOS and the RSS work together to program and service

Motorola Radio Products.  No representation is made that this

information is complete or accurate.  Motorola expressly disclaims

any liability for the accuracy or completeness of the information, or

for any damages resulting from the use or misuse of the information.

Please refer to the manufacturer's documents for your particular

system.  Additional information on the use of PCs and DOS can be

found in commonly-available DOS/PC references.  Later versions of

DOS (5.x and 6.x) include the "HELP [command]" command.

Copyrights, Trademarks, and Service Marks for various companies and

products mentioned below are the sole property of the manufacturers,

and are hereby acknowledged as such.

==================================================================

This file contains the following topics:

(1)     DOS / PC (Troubleshooting) Setup Checklist

(2)     Multiple DOS Configurations

(Multiple CONFIG.SYS / AUTOEXEC.BAT files)

(3)     Attempting to Execute RSS Under WINDOWS

(4)     CAUTIONS When Using FLOPPY DISKETTES for Archiving Codeplugs

(5)     "Sorting" Codeplug-File Directory Entries, for Faster Finding

(6)     DOS Extended Error Codes

==================================================================

==================================================================

==================================================================

SECTION (1)

DOS / PC (Troubleshooting) Setup Checklist

==================================================================

If you are having difficulty running RSS products on newer PCs -or-

Laptop PCs, especially older versions of the RSS products, the tips

below may help you.  We strongly suggest trying these items

-PRIOR- to calling Motorola Customer Service for aid.

Because PCs and DOS are rapidly changing and expanding their

architectures, it may not be possible for older RSS versions to

successfully run on the latest machines.  If these tips don't work

for your old versions, we suggest contacting Motorola Americas Parts

Division for an RSS upgrade.  If you have the latest version and

still have problems after trying these tips, please contact Motorola

Customer Service.

The items below are suggested as a starting place.  You must consult

your manufacturer's manuals for details on how to access your CMOS

setup, and other PC-specific operations.  Your DOS manual must be

consulted for DOS version-specific information.  Also, consult the

README.TXT file on the RSS installation disk for the most current RSS

information.

-------------------------------------------------

(-A-)   VERIFY RIB, CABLES and SETUP are OK:

-------------------------------------------------

Using your older PC, verify that the RIB, CABLE and Radio setup is

working correctly.  After this step, move the PC-to-RIB cable from

your original PC to you new PC's Comm. Port, remembering that the

Comm. Port number (COM1 or COM2) may be different. See RSS Manual for

Comm. Port selection and the "COMMUNICATIONS TEST" function.

If no older PC is available, try changing the RIB, CABLES and Radio

to verify proper hardware operation; also, ensure proper Comm. Port

selection.

----------------------------

(-B-)  MEMORY CONFIGURATION:

----------------------------

Modern PCs and DOS can support four types of memory;

(1) CONVENTIONAL - The lowest 640 KB of memory address space. This

is the primary memory of all PCs.  Those RSS products that do

-not- use EXTENDED memory need as much of this space as possible.

(2) UPPER - The next-highest 384 KB memory address space.

Application programs normally do not access this memory.  This space

contains VIDEO/DISPLAY and other device data.  It also can be used to

load certain device drivers "HIGH", to free-up some CONVENTIONAL

memory.  !! CAUTION !! Some driver programs do not run in the UPPER

memory area. On 80386 (and above) PCs, access to UPPER memory requires

use of the EMM386.EXE memory manager.

(3) EXTENDED/HIGH - Together, CONVENTIONAL and UPPER memory take up

the first 1 MB of memory.  EXTENDED memory is all memory above the

1 MB address.  The HIGH memory area is actually the first 64KB of

EXTENDED memory space.  Loading DOS "HIGH" puts most of DOS in this

HIGH area.  On 80286 (and above) PCs, the EXTENDED/HIGH memory can be

accessed using an extended memory manager such as HIMEM.SYS.  Many

RSS products require EXTENDED memory.

(4) EXPANDED - This type of memory is not directly accessable to

programs.  It normally was installed using an expanded-memory board,

and required an expanded-memory manager to "map" the physical memory

into the program's address space.  Today, EXPANDED memory can be

"emulated" by managers such as EMM386.EXE, which use EXTENDED memory

to emulate the expanded-memory card.  EMM386.EXE can also be used

to load certain device drivers "HIGH", into the UPPER memory.

Memory management begins with statements placed in the CONFIG.SYS

and AUTOEXEC.BAT files.  Memory configuration should ideally

maximize the amount of CONVENTIONAL and EXTENDED memory available, by

using HIMEM.SYS and loading "DOS=HIGH" (in CONFIG.SYS).

To repeat for clarity, loading DOS "HIGH" actually puts DOS into the

HIGH portion of EXTENDED memory.  Loading device drivers "HIGH" puts

them into the UPPER memory area (requires EMM386.EXE).  Both techniques

can be used to free-up CONVENTIONAL memory, if needed.

!! NOTE !! The DOS 5.x/6.x command "MEM /C | MORE" can be used to profile

your memory configuration, and monitor changes in memory allocation when

CONFIG.SYS and AUTOEXEC.BAT are changed.

RSS products do -NOT- require EXPANDED memory, and if EMM386.EXE is

used for either EXPANDED -or- UPPER memory access, enough EXTENDED

memory must be left free for RSS allocation.  Consult the RSS User's

Guide or the README.TXT file on the RSS installation disk to

determine how much EXTENDED memory is required.

Additional CONVENTIONAL memory can be freed by using EMM386.EXE to

load selected device drivers into the UPPER area, using the "ram 64"

switch (see DOS "HELP EMM386.EXE").  The "noems" switch can prevent

some RSS products from executing properly on some machines, under

some DOS versions.

----------------------------

(-C-)   CONFIG.SYS

----------------------------

As part of your troubleshooting, remember that DOS does -NOT- require

CONFIG.SYS and AUTOEXEC.BAT to be present at startup.  When these two

files are absent, this gives you a "clean start" (e.g. -NO-

conflicting drivers, and -NO- "Terminate, Stay Memory Resident" (TSR)

programs). Any RSS -not- requiring EXTENDED memory should be able to

run with -no- CONFIG.SYS / AUTOEXEC.BAT.

!! CAUTION !! There are some PC "security" programs that don't allow

the CONFIG.SYS and/or AUTOEXEC.BAT files to be missing or altered.

Check with your PC System Administrator if this is the case.

!! CAUTION !! For ultimate safety, create a "bootable" floppy

diskette, using the DOS "FORMAT /S" command, and copy your original

CONFIG.SYS / AUTOEXEC.BAT files to the formatted floppy diskette.

Do NOT delete your original CONFIG.SYS and AUTOEXEC.BAT files; instead,

rename them (DOS "REN" command) to CONFIG.001 and AUTOEXEC.001.  Then,

later, you can "COPY" them back to the standard names, and edit them

for changes.

For PCs with EXTENDED memory, the following -minimal- CONFIG.SYS

should work for -all- RSS products, and HIMEM.SYS is REQUIRED for RSS

products that need EXTENDED memory;

DEVICE=C:\DOS\HIMEM.SYS /V

DOS=HIGH

BUFFERS=20

FILES=20

STACKS=9,512

( Device=C:\DOS\.... no entries at start of troubleshooting )

!! CAUTION !! It is (Microsoft) recommended practice to always load

HIMEM.SYS as the  -1st-  line of the CONFIG.SYS;   -IF-   EMM386.EXE

is used, it should be the  -2nd-  line, with "DOS=HIGH,UMB" as the

-3rd-  line.  ALWAYS put the "memory manager" entries -prior- to the

"memory user" entries.  Reversing them may appear to work, but is not

a stable configuration.  HIMEM should be -prior- to EMM386.

For maximum availability of CONVENTIONAL memory, EMM386.EXE can be used

to load device drivers "HIGH" into Upper Memory Blocks:

DEVICE=C:\DOS\HIMEM.SYS /V

DEVICE=C:\DOS\EMM386.EXE RAM 64

DOS=HIGH,UMB

BUFFERS=20

FILES=20

STACKS=9,512

( DeviceHIGH=C:\DOS\.... no entries at start of troubleshooting )

If you are able to find the simplest CONFIG.SYS (with -NO- AUTOEXEC.BAT)

that enables RSS execution, then lines from the original CONFIG.SYS can

be restored one at a time to resolve which element is in conflict with

the RSS execution. Use DOS 5.x/6.x "MEM /C | MORE" to view memory area

allocation, as you make changes.

Once a stable CONFIG.SYS is established, begin building back the

AUTOEXEC.BAT, starting with the "SET PATH=C:\DOS;C:\....." line.

Some drivers/utilities to watch for in CONFIG.SYS and AUTOEXEC.BAT;

1)  The DOS "SHARE" Program causes problems with some RSS products.

2)  The DOS "GRAPHICS" Program causes problems with some RSS products.

3)  The DOS "EMM386" Expanded Memory Mgr. "noems" switch can conflict

with some RSS products.

4)  The DOS "POWER.EXE Program can interfere with some LapTop power

management systems (see below).

----------------------------

(-D-)   CMOS SETUP

----------------------------

Most modern PCs have CPU speed selection and "caching" options as

part of their CMOS setup.  To begin your troubleshooting, set CPU to

the lowest possible speed, and turn -all- cache (internal and

external) off.  Both the high speed and caching can interfere with

timing when reading/writing the Radio through the RIB box.

----------------------------

(-E-)   POWER MANAGEMENT

----------------------------

LAPTOPS (and some newer desktops as well) have power management

features to conserve operating power. The interrupt used in this

process cannot be turned off by applications, and typically takes a

-long- time to service the various devices, and this can interfere

with timing when reading/writing the Radio through the RIB box.  Turn

the power management OFF, or to the least-active mode allowed.

There are some PC manufacturers who are advising that the Microsoft

POWER.EXE driver be REMOVED from the CONFIG.SYS file, because it may

interfere with the PC's internal power management functions.  Consult

your manufacturer's literature.

In some cases, power management must be disabled at several different

levels; the PC's CMOS setup, the CONFIG.SYS/AUTOEXEC.BAT files, and

the Windows environemnt.

-------------------------------------------------

(-F-)  8250/16450/16550 Communications Port UARTs

-------------------------------------------------

The 16550 UART (Universal Async. Receive/Transmit) is functionally

equivalent to the 8250 and 16450 UARTs, with the addition of 16-byte

XMIT/RECV FIFO buffers.  To conserve power and chip count, many

manufacturers are using custom-ICs to -emulate- the 16550 UART.  Some

emulations are variants, such as powering up with the FIFO enabled (a

true 16550 powers up with FIFO -disabled-).

The symptom would be repeated errors while trying to READ/WRITE the

Radio Codeplug via the RIB box.  This is most likely to be seen on

LAPTOP PCs, but can occur on some DESKTOPs as well.

Some manufacturers are supplying a utility program that allows the

user to -disable- the FIFOs, for backwards compatibility.  For

example, COMPAQ supplies "COMFIFO.EXE" (c) for their "Contura Aero (c)"

Laptop series.  Consult your manufacturer's literature and/or THEIR

Customer Service.

Motorola LMPS Product Services can supply a utility called COMMCHEK.EXE

which can report/change the FIFO state if the UART(s) are 16550 or

compatible.

==================================================================

==================================================================

SECTION (2)

Multiple DOS Configurations (Multiple CONFIG.SYS / AUTOEXEC.BAT files)

==================================================================

It may be that you will need to keep more than one set of CONFIG.SYS and

AUTOEXEC.BAT files, switching to one set for a specific applications'

use, then switching to another set for use with RSS programs. A sample

"batch" file, SWITCH.BAT, is shown below, and can be used to quickly

switch to a desired pair of CONFIG/AUTOEXEC files:

e.g. '\SWITCH RSS' switches to the RSS pair of configuration files, then

the PC is re-booted with CTL-ALT-DEL.

@echo OFF

echo   -SWITCH-    autoexec + config   to  %1

if %1- == rss-  GOTO DO_RSS

if %1- == act-  GOTO DO_ACT

echo ERROR !!   arg1 value -NOT- equal to;

echo   rss      Motorola Radio Service Software

echo   act      For Accounting/Payroll Applications

GOTO   END

:DO_RSS

copy   C:\autoexec.RSS   C:\*.bat

copy   C:\config.RSS     C:\*.sys

echo   READY FOR --RSS--

GOTO   END

:DO_ACT

copy   C:\autoexec.ACT   C:\*.bat

copy   C:\config.ACT     C:\*.sys

echo   READY FOR --ACCOUNTING--

GOTO   END

rem    additional applications can be added....

:END

echo   READY FOR -- SYSTEM RE-BOOT --

echo   END of -SWITCH-

A more general (and cryptic) method is the following non-specific

version which would accept any 3-letter file suffix, but requires

that all users remember which file pair is for a specific

application:

@echo   off

echo    SWITCH_2

echo    Arg_1 = %1

if %1- == - goto ERROR

if NOT EXIST    autoexec.%1   goto ERROR_2

if NOT EXIST    config.%1     goto ERROR_2

goto DOIT

:ERROR

echo ERROR: NEEDS -3- letter suffix argument as Arg1

goto END

:ERROR_2

echo ERROR: CANNOT find autoexec/config  - .%1 -

goto END

:DOIT

echo    copy C:\autoexec.%1   c:\autoexec.BAT

copy    C:\autoexec.%1   c:\autoexec.BAT

echo    copy C:\config.%1     c:\config.SYS

copy    C:\CONFIG.%1     c:\config.SYS

:END

echo    READY FOR -- SYSTEM RE-BOOT --

echo    END of SWITCH_2

Advanced Users may wish to consult DOS 6.xx "HELP MULTI-CONFIG" for

information on using MENU commands within a single CONFIG.SYS and

AUTOEXEC.BAT pair, which allows different DOS configurations by

user's choice from a menu, at boot-up time.

REM    WINDOWS 3.1 / DOS   MENU CONFIG.SYS

[menu]

menuitem=WIN_31, Windows-3.1

menuitem=DOS,    MS-DOS 6.22

menudefault=WIN_31

[WIN_31]

Device=C:\WINDOWS\HIMEM.SYS /V /TESTMEM:OFF

Device=C:\WINDOWS\EMM386.EXE RAM 64

[DOS]

Device=C:\DOS\HIMEM.SYS /V /TESTMEM:OFF

Device=C:\DOS\EMM386.EXE RAM 64

[common]

DOS=HIGH,UMB

STACKS=9,512

LASTDRIVE=E

DeviceHIGH=C:\MOUSE\LMOUSE.COM

--------------------------------------------

@ECHO   OFF

REM     Windows-3.1 -or- DOS 6.22 --- MENU AUTOEXEC.BAT

prompt  $P$G

SET     TEMP=C:\temp

SET     TMP=C:\temp

SET     COPYCMD=/Y

rem CONFIG.SYS sets -menu- Value in DOS ENV. Var. "CONFIG"

GOTO    %CONFIG%

:WIN_31

SET     PATH=C:\WINDOWS;C:\WINDOWS\SYSTEM;.;C:\dos;C:\utils;

goto    END

:DOS

SET     PATH=.;C:\dos;C:\utils;

goto    END

:END

DOSKEY

==================================================================

==================================================================

SECTION (3)

Attempting to Execute RSS Under WINDOWS    [*]=(tm) Microsoft Corp

==================================================================

The original PC/DOS design is based on the assumptions of:

1)  Single User.

2)  Single Application/Task active at one time.

3)  Direct access to hardware by Application software.

Communications applications, such as the RSS, rely on these

assumptions to give them the priority (no multi-user, multi-tasking

interference) and speed (direct hardware access) to maintain the

critical timing needed to sustain a communications protocol.

Windows (and many networked environments) must assume:

1)  Multiple Users possible (e.g. networking).

2)  Multiple Tasks "Active" at same time. (multi-tasking)

3)  Direct access to hardware by applications not allowed.

We cannot guaruntee successful operation of any RSS under Microsoft

Windows[*].  Windows will not allow programs direct access to the

Comm. Port and Timer hardware; these functions are emulated by

Windows, hence they are slower than under DOS, and this could

interfere with critical communications timing.  Higher-speed CPUs/PCs

(80486+ at 66 MHz and above) have the best chance of success.

Any setup for RSS within Windows 3.1 [*] or Windows 95 [*] should be

done in the DOS FULL-SCREEN Mode, as opposed to the DOS WINDOW Mode.

Communications programs (i.e. RSS Reading / Writing the Radio

Codeplug) cannot be executed successfully in WINDOW "background"

mode, and the WINDOW "foreground" mode may not sufficiently support

the critical timing needed to consistently communicate with the

RIB/Radio. FULL-SCREEN Mode gives higher priority (speed) to the RSS

than the DOS WINDOW Mode.

--------------------------------------------

For Windows 3.1, use the PIF EDITOR to open the "DOSPRMPT.PIF" file;

On the "MAIN" menu

Set "Display Usage" to      FULL SCREEN

Set "Execution" to          EXCLUSIVE.

Under "ADVANCED..."

set "Foreground Priority" to 10,000,

set the "Detect Idle Time" box OFF.

SAVE the file before using the "MS-DOS PROMPT" icon.

--------------------------------------------

--------------------------------------------

For Windows 95, create an icon/shortcut.  Under PROPERTIES:

Select PROGRAM Sheet;

set "CLOSE ON EXIT" box TRUE.

Under PROGRAM Sheet "ADVANCED";

set "PREVENT PROGRAM FROM DETECTING WINDOWS" to TRUE.

set "SUGGEST MS-DOS MODE" to FALSE.

set "MS-DOS MODE" to FALSE.

Select SCREEN Sheet;

set "USAGE" to FULL SCREEN.

Select MISC Sheet;

set "ALLOW SCREENSAVER" to FALSE.

set "IDLE SENSITIVITY" to LOW.

SAVE the file before using the "MS-DOS PROMPT" icon.

--------------------------------------------

If the above does not allow the RSS to function properly, then;

a) EXIT Windows 3.1 and use normal DOS.

b) Set Windows 95 PROGRAM/ADVANCED Sheet to "MS-DOS Mode".

==================================================================

==================================================================

SECTION (4)

CAUTIONS When Using FLOPPY DISKETTES for Archiving Codeplugs

==================================================================

If you are using FLOPPY DISKETTES for archiving your codeplug files,

please keep the following in mind;

A)  Due to ambiguities in the (extended) error codes returned to an

RSS from DOS/BIOS (See "DOS Extended Error Codes" below), the RSS

cannot always properly report the precise nature of an error while

reading/writing disk files.

One example is using an UN-FORMATTED Floppy Diskette.  The DOS error

code returned is "31", "GENERAL FAILURE" (type unknown).  This type

of failure usually is interpreted by the RSS as an "immediate exit"

situation, because the error could be from other resources, such as

your -fixed- disk, and continuing could jeapordize your system/data.

B)  Due to (A) above, you should ensure that -all- your diskettes are NEW

and PROPERLY FORMATTED prior to using them for codeplug archives.

C)  Floppy diskettes and -drives- are exposed to DIRT and MOISTURE.  Keep

your diskettes in their sleeves and away from contaminants. Clean your

-drives- periodically with commercially-available cleaning kits.

D)  You should check the condition of your floppy diskettes PERIODICALLY,

by running DOS "CHKDSK (A: -or- B:)" or (DOS 6.x) "SCANDISK (A: -or- B:)".

If these DOS programs detect problems, even if "corrected", we recommend

copying that diskette to a NEWLY FORMATTED diskette before continuing,

and using the NEW diskette for future archiving.

If problems persist, the diskette may be defective, -or- your -drive-

may need cleaning or repair.

E)  Using a floppy diskette that is ALMOST FULL can cause problems,

such as not leaving enough room for renamed backups (the old version

stays and the new version is added), not enough room for the next RSS

database entry (into the *.DBF file), etc.  After each RSS session,

perform a DOS "DIR (A:  -or- B:)"; the last entry of the report tells

how many bytes are "free" on the diskette.  We recommend leaving at

least 10,000 bytes free at all times.

F)  An alternative to floppy diskettes is re-organizing the archive

system on your -FIXED- disk.  The RSS allows multiple CODEPLUG or

ARCHIVE subdirectories, under unique names, such that each sub-dir. can

be organized by model number, customer, department, etc.  Create the

sub-dirs. via DOS "mkdir -name-" command.  Then access each sub-dir.

from the RSS using the "(F2) Change Archive" on the GET or SAVE screen.

This offers the advantages of greater speed in finding files and in

reading / writing files, much quicker access to various archives

without changing diskettes, and higher limits on how many files can

be in a single archive.

These archives need periodic BACKUP to floppy diskettes (or -tapes-).

Make sure you save -all- files in the archive sub-dir., to include some

RSS "database" files (*.DBF).  Large sets of archive files can be backed

up to floppy diskettes via "compression" programs that create "*.zip"

files.  !! CAUTION !! The first file you should save to a compressed

backup diskette is the version-matching "uncompress/unzip" program file.

==================================================================

==================================================================

SECTION (5)

"Sorting" Codeplug-File Directory Entries, for Faster Finding

==================================================================

Many RSS programs were not designed to "sort" the file entries in a

codeplug archive into an "ascending" number order, by serial number.

Quickly finding a particular serial number from 100+ entries can be a

problem. There are utilities available that can sort directory entries

into ascending number order, making it easier to find particular serial

numbers.

One such utility is Peter Norton's (c) "DS" (Directory Sort, Version

4.50).  It will read directory entries, sort them by several

different user-chosen orders, and re-write the directory entries

permanently in the new order.

Another method is the DOS Version 6.x "DEFRAG" command, similar to

Peter Norton's (c) "SPEEDISK" (Version 5.0);  both offer an option to

sort files into ascending order while de-fragmenting your disk files.

All of the above (and others) can be used on both -fixed- and -floppy-

disk archive directories.  Please read the appropriate documents carefully.

!! CAUTION !! Always backup your disk(ette) -prior- to first use of

these utilities.

NOTE !! These utilities will -not- change the order of files

displayed with any RSS that requires a *.DBF file in the Archive

sub-directory.  E.g. MaxTrac Conventional RSS always requires that a

file "MAXTRACM.DBF" be present in any sub-directory that contains

"valid" archive files.  The order of files displayed is determined by

the order in which the files were saved and entered into the *.DBF file.

==================================================================

==================================================================

SECTION (6)

DOS Extended Error Codes

==================================================================

The RSS Program is an application program that must use the PC's DOS

(Disk Operating System) and BIOS Interfaces for access to computer

resources such as memory, disk drives, disk files, printers,

communication ports (e.g.  COM1), etc.

If the RSS performs an operation on a resource, and DOS/BIOS detects an

error, DOS will return an error code to the RSS.  This will most

often (but not always) be reported in the form;

Error Code          - nn    Gives Specific Error Information

Error Class         - xx    Gives General Situation Information

Locus               - yy    Gives General Source (Device) of Error

Recommended Action  - zz    Gives Recommended (Recovery) Action

In some cases, only the Error Code is shown.

To aid users in resolving problems with computer resources, the

following list of DOS Error Codes is given.  These Codes are commonly

documented elsewhere as "DOS Extended Error Codes", under INT 59(hex)

"GET EXTENDED ERROR INFORMATION".  Also shown are the CLASS and LOCUS

Codes.

The ACTION Codes are not shown.  The RSS has limited options to take

any system-level "recovery action" without jeapordizing your data or

your system. Some Codes are ambiguous.  Each new version of DOS can

expand the number of defined Error Codes and/or expand the current

definitions.  Also, various VERSIONS of DOS/BIOS can return different

Codes, depending on their implementation.  E.g., a common return for

attempting access to an un-formatted floppy disk is (CODE-CLASS-LOCUS)

31-13-2 "GENERAL FAILURE",  not the possible  11-9-2  or  26-11-2.

No representation is made that this list is complete or accurate.  Further

information on the codes and terms shown below can be found in

commonly-available DOS references.  Using the three Codes together may

help you to find/correct any resource problems you are having.

The Codes are shown as DECIMAL values.

DOS EXTENDED ERROR CODES:

=========================

001 -   Invalid Function Number

002 -   File Not Found

003 -   (Dir/File) Path Not Found

004 -   No (File) Handles Available (Too Many Files Open)

005 -   Access Denied (e.g. Attempt to Write a Read-Only File)

006 -   Invalid (File) Handle

007 -   Memory Control Blocks Destroyed

008 -   Insufficient (Not Enough) Memory

009 -   Invalid Memory Block Address

010 -   Invalid Environment Block

011 -   Invalid Format

012 -   Invalid (File) Access Code

013 -   Invalid Data

014 -   (RESERVED)

015 -   Invalid (Disk) Drive (Specification)

016 -   Attempt To Remove Current Directory

017 -   Not The Same Device

018 -   No More Files

019 -   Disk Is Write Protected

020 -   Unknown Disk Unit ID

021 -   (Disk) Drive Not Ready

022 -   Unknown Disk Command

023 -   CRC Error (Disk File Data Error)

024 -   Bad Disk Request Structure Length

025 -   (Disk) Seek Error

026 -   Unknown Media Type (e.g. Non-DOS Disk)

027 -   (Disk) Sector Not Found

028 -   (Printer) Out Of Paper

029 -   Write Fault/Error

030 -   Read  Fault/Error

031 -   General Failure (Exact Nature UNKNOWN)

032 -   (File) Sharing Violation (e.g. DOS "SHARE")

033 -   (File) Locking Violation

034 -   Invalid Disk Change

035 -   (File) FCB Unavailable

036 -   Sharing Buffer Overflow

037 -   (RESERVED)

038 -   Unable To Complete (Disk) File Operation

039 -   (RESERVED)

:::

049 -   (RESERVED)

050 -   Network Request Not Supported

051 -   Remote Computer Not Listening

052 -   Duplicate Name On Network

053 -   Network Name Not Found

054 -   Network Busy

055 -   Network Device No Longer Exists

056 -   Net BIOS Command Limit Exceeded

057 -   Network Adapter (Hardware) Error

058 -   Incorrect Network Response

059 -   Unexpected Network Error

060 -   Incompatible Remote Adapter

061 -   Print Queue Full

062 -   Not Enough Space For Print File

063 -   Print File Deleted

064 -   Network Name Deleted

065 -   Access Denied

066 -   Network Device Type Incorrect

067 -   Network Name Not Found

068 -   Network Name Limit Exceeded

069 -   Net BIOS Session Limit Exceeded

070 -   (Sharing) Temporarily Paused

071 -   Network Request Not Accepted

072 -   Print or Disk Redirection Is Paused

073 -   (RESERVED)

:::

079 -   (RESERVED)

080 -   File Already Exists

081 -   (RESERVED)

082 -   Cannot Make Directory Entry

083 -   Fail On INT 24 (Critical Error Interrupt)

084 -   Too Many Redirections (Out Of Network Structures)

085 -   Duplicate Redirection (Network Device Already Assigned)

086 -   Invalid Password

087 -   Invalid Parameter

088 -   Network Data Fault

089 -   Function Not Supported By Network

090 -   Required System Component Not Installed

DOS ERROR CLASS CODES:

======================

01  -   Out Of Resource; No More Left Of Whatever You Asked For

02  -   Temporary Situation (e.g. Locked File); Try Again Later

03  -   Authorization (Access Rights - Permissions)

04  -   Internal DOS Error (Not User/Application);     CAUTION, SERIOUS !!

05  -   Hardware Failure (Whatever Resource You Are Accessing). SERIOUS !!

06  -   System Software Failure; Other DOS Problems;   CAUTION, SERIOUS !!

07  -   Application Program (Software) Error

08  -   Item (Resource) Requested Not Found

09  -   Bad (Disk) Format ( e.g. Non-DOS Disk )

10  -   Item (e.g. File) Locked

11  -   Bad Media (e.g. Disk Sees CRC Errors On File Read, File Corrupted)

12  -   Already Exists

13  -   Error Class Is UNKNOWN

DOS LOCUS CODES:

================

01  -   Locus is UNKNOWN

02  -   Block  (Data Transfer) Device (e.g. Disk Drive/RAM Disk Emulator)

03  -   Network Device/Resource/Operation

04  -   Serial (Data Transfer) Device (e.g. Comm. Port (COM1), Printer)

05  -   Memory Related