DATA FILE CONVERSION

Edited by Sean Flood, Engineering support group.

Date      5th July 1990

Property of MOTOROLA STORNO

(c) Copyright of Motorola Storno 1990.

The field programmers which preceded version R04.03.00 used a smaller format

of data file for storing the radio personality. If you have files which you

need to use with the newer field programmers, then it will be necessary to

convert the older files to the new format, this document discribes the use of

a conversion program called 'fileconv'.

Do you need to convert your files?

Older files have a filename extension of "I80" and are 1150 bytes which is

smaller than the new format of 1950 bytes. If you use the DOS command DIR it

indicates the size of the file in its response shown below :

C>DIR

Volume in drive C is SYSTEM

Volume Serial Number is 1472-1446

Directory of  C:\DATA

.            <DIR>     05-07-90   2:23p

..           <DIR>     05-07-90   2:23p

FILECONV EXE     10208 30-05-90   5:07p

RADIO_A  I80      1150 21-02-89   5:31p      <-- old format file

4 File(s)     192512 bytes free

To convert files.

The program 'FILECONV.EXE' must be in the same directory as the files to be

converted, then using radio_a radio file as an example type the following:

C>FILECONV RADIO_A

MOTOROLA STORNO

(c) Copyright of Motorola Storno 1990.

Revision : R01.00.00    Model : Data file conversion

This program converts 8 channel programmer files to 16 channel

programmer files, only required for up issue's of programmers.

Source filename      : RADIO_A.I80

Destination filename : RADIO_A.I8E

Program complete.

Then using the DOS command DIR we can see that a new file has been generated

with the filename extension of "I8E" and length of 1950 bytes.

C>DIR

Volume in drive C is SYSTEM

Volume Serial Number is 1472-1446

Directory of  C:\DATA

.            <DIR>     05-07-90   2:23p

..           <DIR>     05-07-90   2:23p

FILECONV EXE     10208 30-05-90   5:07p

RADIO_A  I80      1150 21-02-89   5:31p

RADIO_A  I8E      1950 05-07-90   2:28p

5 File(s)     184320 bytes free

If you program a radio with more channels than the original radio the new

channels will be copied from the highest channel used in the original radio,

this program will not convert files from the new format to the old format.

Errors.

There are five errors which could occur are listed below and also discribed

in more detail:

i)     The source file cannot be opened.

The program was unable to find the file in the current directory,

ensure that you DO NOT enter the filename extension when running the

file conversion program or check that the file in the directory has

the correct file name extension.

ii)    The source file is incomplete.      OR

The source file is too long.

The file you have specified is checked before it is used as the source

file for the new file. The source file must be 1150 bytes long and if

the it is any other length then the field programmer or this program

will be unable to used this file.

iii)   The source file has an error.

The computer has read the file and has decided that the file has been

corrupted.

iv)    The destination file already exist.

If the destination filename is the same as a file which already exists

in the current directory then the program will abort without changing

any files. The data file may have been converted once before.

More experienced users.

The program does return an error code of 1(one) if an error is detected while

doing the file conversions, only the errors discribed above will return 1

otherwise the program will return 0.

#