WinFLEX

Pager Decoding Software for Windows95/98

Version 1.0

Table of Contents

General Description	3

Basic Operation	3

Setting up WinFLEX	4

WinFLEX Menu	5

File	5

Logfile (Ctrl + L)	5

Clear Display (Ctrl + D)	5

Exit (Alt + X)	5

Edit	5

Copy (Ctrl + C)	5

Copy Upper Pane (Ctrl + PgUp)	6

Copy Lower Pane (Ctrl + PgDn)	6

Setup	6

Interface	6

Options (Ctrl + O)	6

Display	7

Font	7

Colors	7

ScrollBack	8

Filters (Alt + L)	8

Monitor	10

Statistics (Ctrl + S)	10

Keep Alive	12

Help	13

Help Topics (F1)	13

About WinFLEX	13

# General Description

WinFLEX is a Windows application that allows you to decode RF transmissions from a paging system by using a radio scanner and a data slicer circuit.  The scanner will need to be modified to tap the discriminator and you will need to build a circuit called a data slicer or, optionally, you can build the 4 Level FSK decoder circuit instead.  The data slicer (2 Level FSK circuit) will allow you to decode all three POCSAG speeds (2400, 1200, and 512 baud), Golay (GSC), FLEX 1600bps transmissions and phases A and C of FLEX 6400bps transmissions.  The data slicer will not allow you to decode phases B and D of FLEX 6400bps transmissions or ReFLEX transmissions.  For these you will need the 4 Level FSK decoder circuit.  I would recommend building the data slicer circuit initially and then building the 4 Level FSK decoder circuit later if you need to decode all phases of FLEX 6400bps transmissions or ReFLEX transmissions.

Included in the WinFLEX zip file are schematics for building the data slicer and 4 Level FSK decoder.  The ReadMe.txt file also contains a simple ASCII drawing of a data slicer and information about modifying your scanner to tap the discriminator.

# Basic Operation

The WinFLEX display is divided into two windowpanes.  The upper pane is used to display filter matches (which will be covered later in this document) and the lower pane is used to display messages as they are being decoded from the scanner.  The top edge of the lower pane can be repositioned by clicking and dragging that edge with the mouse.  Move the cursor near the top edge of the lower pane and the cursor will change to an arrow pointing up and down.  When you see this cursor you can click and drag the top of the lower pane.  This allows you to see more or less information in each pane.  WinFLEX will save the size of the upper and lower windowpanes when you exit the program.  This setting will be restored when WinFLEX is started running again.  You can make the WinFLEX window larger or smaller by dragging the edges or corners of the WinFLEX window.  The size and position of the WinFLEX window will be saved when the program exits.  These settings will also be restored when WinFLEX is started running again.

The lower pane cannot be sized smaller than 1 line of text in height.  This is to prevent the upper edge of the lower pane from being so close to the bottom of the WinFLEX window that it cannot be easily “grabbed” to resize the lower pane again.  The top edge of the lower pane can be moved all the way to the top of the WinFLEX window if desired.  This will prevent any filter information from being displayed, but, if you are not using the filters, you may wish to do this to have more messages being displayed in the lower windowpane.

When the windowpanes have more than a “screen full” of information displayed, scroll bars will appear on the right edge of the windowpane.  These scroll bars will allow you to scroll back to previously displayed messages.  If the button in the scroll bar (the “thumb tab”) is not ALL THE WAY at the bottom of the scroll bar, WinFLEX will NOT display any new information in that pane.  This was done so that WinFLEX would not keep scrolling a message out of your view when you have scrolled back to look at it.  Any new messages that are received while the display is “scrolled back” will not be lost.  They will be stored in the display buffer, but will not be displayed until you scroll the pane all the way back to the bottom.  You can use the up or down arrows, the PgUp or PgDn keys and the Home and End keys to scroll the lower pane up or down.

The title bar in WinFLEX will show the current paging format that is being decoded.  For FLEX transmissions, the title bar will show the FLEX speed in bits per second and which phases are being used.  For POCSAG transmissions, the title bar will show the baud rate (2400, 1200, or 512).  For Golay Sequential Code transmissions, the title bar will display “Golay (GSC)”.

The following formats are used to display messages:

POCSAG – The 7-digit capcode will appear within left and right curly braces and will be followed by the function code (from 1 to 4) which will be followed by the baud rate.  This will be followed by the numeric or alphanumeric message.  An example of a POCSAG message would be:

{0473285} 3 1200 This is a test message.

FLEX – The 7-digit or 9-digit capcode will appear within square brackets, which will be followed by the FLEX phase, which will be followed by the FLEX format.  This will be followed by the numeric, alphanumeric, or binary message.  The following FLEX formats are used:

SECURE  – Used for FLEX secure vector messages.

SH/TONE – Used for short numeric or tone-only messages.

ST NUM  – Used for standard numeric messages.

SF NUM  – Used for special format numeric messages.

ALPHA   – Used for alphanumeric messages.

BINARY  – Used for binary format messages.

An example of FLEX messages would be:

[004863547] A ST NUM  1-800-555-1212

[003495623] C  ALPHA  Bob, call Sam when you get the sales figures.

[0953465] A SH/TONE 555-9999

Golay Sequential Code (GSC) – The capcode will appear within left and right parenthesis and will be followed by the function code (from 1 to 4).  Since there is nothing in the Golay protocol to distinguish numeric messages from alphanumeric messages, if the message is 24 characters long or less, the message will be decoded as both numeric and alphanumeric format.  If the message is longer than 24 characters the message will decoded as alphanumeric.  An example of a Golay message would be:

(034513) 1 THIS IS A LONG ALPHANUMERIC TEST MESSAGE.

If the timestamp option is enabled, timestamps will appear in the following format:

--------------- <date> <time> ---------------

The <date> and <time> information will be formatted based on the way your Windows date and time settings are configured.  You can change the date and time format by going to the Windows Control Panel and double clicking on “Regional Settings”.  Click on the Time or Date tab and select the appearance that you wish.  Note that other Windows programs (like Explorer) will use this date and time setting, so changing it will effect how the date and time appears in other applications as well as in WinFLEX.

# Setting up WinFLEX

You will need to have your scanner discriminator tap connected to the input of your data slicer circuit and the output of the data slicer connected to an available COM port on your PC.  You will need to set the scanner to the frequency that you wish to monitor.  You will need to start the WinFLEX application running.  Choose “Setup” from the WinFLEX menu bar and select the “Interface” option to configure WinFLEX for your hardware.

# WinFLEX Menu

The menu items in the WinFLEX menu are described below.  If there is a control key (Ctrl) or Alternate key (Alt) shortcut assigned to a menu, the shortcut will be show in parenthesis beside the menu item.

## File

### Logfile (Ctrl + L)

The Logfile dialog box allows you to capture all messages decoded by WinFLEX to a log file.  This log file is a standard ASCII text file that can be viewed with Notepad.exe, Wordpad.exe or any standard text editor.  The log file dialog gives you the option of flushing messages to disk.  Normally Windows will not write information to disk until the buffer holding data waiting to go to disk becomes full.  How often this happens will depend on how quickly messages are being received and how much memory you have.  The “Flush Log File Output To Disk” option allows you to force data to be written to disk after every message, every 5 seconds, every 10 seconds, every 30 seconds, or every 60 seconds.  You may want to flush the log file to disk if you are using another program to examine the log file while WinFLEX is running.  Note: Flushing the log file to disk after each message is not recommended if you are receiving more than a few messages per minute.  The extra disk activity can severely effect system performance.

### Clear Display (Ctrl + D)

The “Clear Display” menu item allows you to clear the upper and lower windowpanes.

### Exit (Alt + X)

The “Exit” menu item allows you to exit the WinFLEX program.  If you have “Confirm Exit” checked in the Setup -> Options dialog, you will asked to confirm that you wish to exit WinFLEX.  If the “Confirm Exit” option is not checked, the WinFLEX program will terminate immediately.

## Edit

### Copy (Ctrl + C)

The Copy menu item allows you to copy selected text from the WinFLEX upper or lower windowpane.  Select the text you wish to copy using the mouse.  Click on the location for one corner of the selection box using the left mouse button and drag the mouse to the location of the opposite corner.  As you drag the mouse you will see the area displayed using inverse video.  Release the left mouse button to complete the selection process.  Use the “Copy” menu item (or the shortcut “Ctrl+C”) to copy the selected text to the Windows Clipboard.  You can then paste the copied text into other applications.  Note: if you use the “Copy” menu item and no text is currently selected, you will hear a short beep indicating that the copy was not successful.

Note: You can use the “Copy” menu item (or shortcut “Ctrl+C”) to create filters by coping a capcode from one of the WinFLEX windowpanes to the filter dialog box.  Use the mouse to select the capcode (only select the numeric digits) and press Ctrl+C, then press Alt+L to bring up the filter dialog box.  Click on “Add” to create a filter.  Select the capcode filter type, then click in the “Capcode” box.  Press Ctrl+V to paste the copied capcode into the “Capcode” box.  Select “Reject” to reject this capcode, or click on “OK” to capture any future messages to this capcode in the filter windowpane.

### Copy Upper Pane (Ctrl + PgUp)

The Copy Upper Pane menu item allows you to copy the currently displayed contents of the entire upper windowpane to the Windows Clipboard.  You do not need to select any text in the upper windowpane before using the “Copy Upper Pane” menu item.

### Copy Lower Pane (Ctrl + PgDn)

The Copy Lower Pane menu item allows you to copy the currently displayed contents of the entire lower windowpane to the Windows Clipboard.  You do not need to select any text in the lower windowpane before using the “Copy Lower Pane” menu item.

## Setup

### Interface

The Interface dialog box allows you to choose the COM port that is connected to the data slicer.  You can choose from COM1 – COM4 or your can choose a “Custom” COM port.  The COM1 through COM4 selections will automatically change the Port I/O Address and the Port IRQ Vector to the standard values found on most PCs.  You can determine if the I/O Address and IRQ Vector values match your hardware configuration by looking at the Ports section in the Device Manager tab of the Control Panel -> System Properties dialog box.  For more details see the ReadMe.txt file that accompanies this document.

In the Interface dialog box you will also need to select whether you are using a 4 Level FSK decoder circuit or not.  For the simple 2 Level FSK circuit (data slicer) or, for a Hamcomm type interface, you should NOT have the 4 Level FSK box checked.  If you are using the 4 Level FSK decoder circuit, you should check the 4 Level FSK box.

There is also an option in the Interface dialog box that will allow you to invert the data received from your scanner.  At first, try decoding messages without the “Invert Data” box checked.  If you do not see any messages being decoded in the lower WinFLEX window, you may need to come back to the Interface dialog box and check the “Invert Data” box.

The options in the Interface dialog box will not take effect until you click on the “OK” button.  You can cancel any changes made in the Interface dialog box by clicking on the “Cancel” button.  The options that you select will be saved to an INI file when you exit the WinFLEX application.  These options will automatically be restored when WinFLEX starts running again.

### Options (Ctrl + O)

The Options dialog box contains options that can be toggled on or off.  The following options are available in the Options dialog box:

Confirm Exit – This option, when checked, will cause a confirmation dialog box to appear when you exit from WinFLEX.  This allows you to cancel the exit if you did not really mean to exit the application.  If this option is disabled, there will be no confirmation when WinFLEX exits.

Timestamp – This option, when checked, will display a timestamp once per minute between messages as they are decoded.  The timestamp will only appear right before a message is displayed and will not appear again until the start of the next minute.  If no messages are received for several minutes, the timestamp will not appear again until a message is received.

Show Numeric/Tone – This option, when checked, will display numeric messages and tone-only messages when they are received.  If this option is disabled, numeric and tone-only messages will not be displayed.

Enable Filter Beep – This option, when checked, will cause WinFLEX to produce a beep when a filter match occurs.  The sound used for the beep is the “Default sound” in the Control Panel -> Sounds dialog box.  If no sound is specified for the “Default sound”, you will hear a short beep instead.  If the Enable Filter Beep option is not checked, WinFLEX will not produce a sound when a filter match occurs.

Decode POCSAG – This option, when checked, will cause WinFLEX to decode POCSAG format transmissions.  If you do not wish to monitor POCSAG transmissions, you can leave this option disabled.  Leaving this option enabled, when not needed, will cause WinFLEX to use more CPU time than necessary.

Decode FLEX – This option, when checked, will cause WinFLEX to decode FLEX format transmissions.  If you do not wish to monitor FLEX transmissions, you can leave this option disabled. Leaving this option enabled, when not needed, will cause WinFLEX to use more CPU time than necessary.

Decode Golay (GSC) – This option, when checked, will cause WinFLEX to decode Golay Sequential Code (GSC) transmissions.  If you do not wish to monitor Golay transmissions, you can leave this option disabled. Leaving this option enabled, when not needed, will cause WinFLEX to use more CPU time than necessary.

### Display

#### Font

The Font dialog box allows you to choose the font you wish to use for the displayed characters.  All of the fonts presented in the font dialog box are fixed pitch fonts, which means that all of the characters within each font use the same amount of space horizontally.  Fixed pitch fonts are used because WinFLEX will automatically wrap lines longer than 80 characters at the right edge of the window.  If variable pitch fonts were used, the right edge would be “jagged” and would not look as good.  You can select Italic or Bold font styles and can select the size of the font that you wish to use.

#### Colors

The Colors dialog box allows you to change the colors used to display information in WinFLEX.  You can change the background color by selecting the “Background” button.  You can change the capcode color (which is used for capcodes of any format) using the “Capcode” button.  You can change the FLEX phase color by using the “FLEX Phase” button.  You can change the FLEX format color, which describes the type of FLEX message decoded, by using the “FLEX Format” button.  You can change the timestamp color using the “Timestamp” button.  You can change the bit errors color using the “Bit Errors” button.  You can change the color used to display numeric messages by using the “Numeric” button.  You can change the color used to display alphanumeric messages by using the “Alphanumeric” button.  You can change the color used to display FLEX binary format messages by using the “FLEX Binary” button.  You can change the color used to indicate a filter match by using the “Filter Match” button.  You can change the color used for filter labels by using the “Filter Label” button.  You can reset all colors to the default settings by using the “Default Colors” button.

There is a sample window in the Colors dialog box that will show you an example of what the display will look like if you select “OK” to exit the Colors dialog box.  You can adjust the colors and see what things would look like without making these changes permanent by using this sample window.  If you don’t want to save the color changes, you can select “Cancel” to exit the Colors dialog box without changing the current WinFLEX colors.

#### ScrollBack

The Scrollback dialog box allows you to change the number of lines available in the scrollback buffer in the upper windowpane and in the lower windowpane.  The maximum number of lines that can be configured for each of these windowpanes is 9999 lines.

## Filters (Alt + L)

Filters allow you to capture messages sent to specific capcodes, or to capture messages that contains specific text.  The Filter dialog box in WinFLEX allows you to add, delete or edit filters.  You can also log filter matches to a log file which can be view later using any standard ASCII text editor (like Notepad).  WinFLEX also gives you the ability to run another application and provide filter fields as arguments to that application.  For example, you could run a program that would send the capcode and message from a filter match to your pager.  One such program, called TAPP.EXE is included in the WinFLEX zip file.

The Filter dialog box displays any filters that have been previously defined in a scrollable window.  To add a new filter, click on the “Add” button in the Filter dialog box.  To delete an existing filter, select the filter you wish to delete from the filter list and click on the “Delete” button in the Filter dialog box.  WinFLEX will display a confirmation dialog box to make sure that you wish to delete that filter.  To edit an existing filter, you can select the filter you wish to edit from the filter list and click on the “Edit” button in the Filter dialog box, or you can simply double click on a filter in the filter list.

When adding a new filter, you will be presented with a dialog box that allows you to choose the filter type, a capcode (if necessary), an optional function code (for POCSAG and Golay filters), an optional baud rate (for POCSAG), an optional filter label, and an optional text string.  You can also choose whether this filter will cause the filter command file to be executed when a match for this filter occurs.  Here is a description of the fields in the Add/Edit Filter dialog box:

Filter: You can select a Text filter, which will match specific text without regard to the paging format or capcode.  You can select a POCSAG Capcode filter, which requires a 7-digit capcode to match against.  You can select a FLEX Short Capcode filter, which requires a 7-digit capcode to match against.  You can select a FLEX Long Capcode filter, which requires a 9-digit capcode to match against.  You can also select a Golay (GSC) Capcode filter, which requires a 6-digit capcode to match against.

Capcode: This field is only available for capcode filter types.  The capcode may only contain numeric digits (0-9) or a question mark character (?) which is used as a wildcard that will match any single digit.

Reject: This field is only available for capcode filter types.  When reject is selected, any messages that are sent to this capcode will not be displayed or stored in the log file.  This allows you to specify capcodes that you never want displayed.

Function Code: This field is only available for POCSAG and Golay capcode filter types.  The function code can be specified as Any, which will match any function code, or can be specified as function code 1, 2, 3, or 4.

Baud Rate: This field is only available for POCSAG capcode filter types.  The baud rate can be specified as “Any”, “2400”, “1200”, or “512”.  Selecting “Any” will match any baud rate.  Other baud rate selections will only match when the baud rate of the decoded message matches the baud rate setting in the filter.

Label: This field is used to help identify the filter when it is displayed in the WinFLEX upper windowpane.  The label can be 20 characters long and may contain any text you wish to use to identify that filter.  When the filter match is displayed in the upper windowpane, the label will be displayed after the capcode.

Text: This field may contain any text that you wish to match.  The Text field is limited to 40 characters.  Text entered in this field is NOT case sensitive.  For example, if you specify the word “Test” in the Text field, it would match “test”, “TEST”, and “teST”.  Note that if you have a capcode filter AND you have text in the Text field, the filter will not match unless there is a message sent to the specified capcode AND that message contains the text in the Text field.

Enable Filter Command File: If this option is checked, and you have enabled the “Filter Command File” in the WinFLEX Filter dialog box, the filter command file will be executed when a match is found for this filter.

When editing an existing filter, you cannot change the filter type.  If you have a capcode filter with text specified, you cannot edit it to make it just a text type filter and vice versa.  You will have to delete the existing filter and add a new filter if you wish to do this.

In the WinFLEX Filter dialog box, you have the option to log filter matches to a log file.  If you check the “Enable Output File” option, you can also provide a filename by clicking in the box to the right of the “Enable Output File” option, or you can select the “Use Date For Output File Name” option.

Clicking the box to the right of the “Enable Output File” option will bring up an Open File dialog box which will allow you to select or type in the name of the file you wish to use to store filter matches in.  The file used to store filter matches is a standard ASCII text file.  The information is stored in the file exactly as it appears on the screen except that any binary characters (characters whose ASCII value is less than 32) are converted to a period character (“.”) when they are written to the filter log file.  When WinFLEX opens the filter log file, it will preserve any information already in the file and append any new filter matches to the end of the file.  If timestamps are enabled, they will also get written to the filter log file.

Selecting “Use Date For Output File Name” will cause WinFLEX to automatically create a file using the last 2 digits of the year, the 3 letter abbreviation of the current month, and the 2 digits of the current day of the month for a filename.  This file will be appended to if it already exists and will be closed at midnight.  After midnight, a new file with the year, month, and date will be opened and used for output.

If you have the filter output file enabled, you also have the option of flushing the filter output to disk at regular intervals.  To do this, check the “Flush Filter Output To Disk” option and select the rate that you wish to use to flush the output to disk.  You have the option of flushing the output to disk after every message, at 5-second intervals, at 10-second intervals, at 30-second intervals, or at 60-second intervals.  This allows you to force the filter log information to be written to disk so that other programs that might try to access the filter log file while WinFLEX is running will have an up-to-date copy of the filter log.  If you do not enable the “Flush Filter Output To Disk” option, the filter log file will only be updated when the Windows operating system flushes the log file to disk.  The rate at which this happens will vary depending on how much memory you have and how often filter matches occur.  The filter log file will also be flushed to disk when you exit the WinFLEX program.  Note that if you enable the “Flush Filter Output To Disk” option and specify that WinFLEX should flush the output after every message, you may get a very heavy amount of disk activity if you have several filter matches occur very closely together.  This can lead to performance problems.  It is not recommended to flush the output to disk after every message unless your filter matches do not occur very often.

The Filter dialog box has an “Enable Command File” option which allows you to enable an external program that you wish to run when a filter match occurs that has the “Enable Filter Command File” option enabled.  The external program, or command file, can be any executable file (.EXE, .COM, or .BAT filename extension).  When you check the “Enable Command File” option, you must also specify the command file you wish to run by clicking in the box to the right of the “Enable Command File” option.  This will bring up an Open File dialog box that will allow you to choose the command file you wish.  To provide arguments to the filter command file, you can use the “Command File Arguments” option.  Click in this box and type in the arguments that you wish to pass to the command file when it is executed.  These arguments can be any text that you wish and can be the following special character sequences:

%c – Gets expanded to the capcode (for FLEX), capcode, function code and baud rate (for POCSAG) or capcode and function code (for Golay), that caused the filter match.

%l – Gets expanded to the label that was assigned to the filter that matched (if a label was defined).  Note that this is lowercase L, not the digit one.

%m – Gets expanded to the message that was sent when the filter match occurred.

For example, if you create a batch file with the following commands:

ECHO LABEL=%1 CAPCODE=%2 AND MESSAGE=%3 >> FILTER.OUT

Check the “Enable Command File” option and assign this batch file as the command file.  Then place “%l” “%c” “%m” (with the quotes) in the “Command File Arguments” field.  If you had a filter that had the “Enable Filter Command File” option checked and the following message was received that matched that filter:

{0264573} 3 1200 Test message.

The following would appear in the FILTER.OUT file:

LABEL=”” CAPCODE=”{0264573} 3 1200” MESSAGE=”Test message.”

If you wish to use the TAPP.EXE program to send yourself a page when a filter match occurs, you will need to check the “Enable Command File” option, and assign the TAPP.EXE file as the command file.  The following “Command File Arguments” could be used if your modem was on COM3 and the TAP/IXO number for your paging company was 1-800-555-1234 and your PIN number was 1234567:

COM3 1-800-555-1234 1234567 “%l %c %m”

This would send the filter label, capcode, and message text to your pager.

NOTE: Since MS-DOS has a command line limit of 120 characters the length of the command and all arguments must not exceed 120 characters.  WinFLEX will automatically truncate any command longer than 120 characters before the command is executed.  To obtain more command line space for arguments, it is recommended that the command file (or batch file) be located in the root directory (i.e. C:\X.EXE or C:\X.BAT).

## Monitor

### Statistics (Ctrl + S)

The Statistics dialog box shows you how many messages were received during the last hour and during the current day.  The hourly statistics will be reset at the start of each hour and the daily statistics will be reset just after midnight.

If you wish to store statistics information to a disk file, you can do so using 2 different formats.  The first is an ASCII text file with the number of messages and number of characters decoded during the current hour (at the end of each hour) and during the current day (at the end of each day).  The second file format (CSV format) is a comma delimited format that can be imported into spreadsheet programs to allow you to create charts, or graphs, or perform calculations on the statistics information.

If you wish to store statistics information to a text file, you will need to enable the “Write Statistics To Text File” option.  You can click in the box to the right of the “Write Statistics To Text File” option to specify a filename, or you can select the “Use Date For Statistics Text File Name” option.

Clicking on the box to the right of the “Write Statistics To Text File” option will cause an Open Statistics File dialog box will appear and you can select an existing statistics file or type in the name of a new statistics file.  If you open an existing file, the statistics will be appended to the end of the existing file.

Selecting “Use Date For Statistics Text File Name” will cause WinFLEX to automatically create a file using the last 2 digits of the year, the 3 letter abbreviation of the current month, and the 2 digits of the current day of the month for a filename.  This file will be appended to if it already exists and will be closed at midnight.  After midnight, a new file with the year, month, and date will be opened and used for output.

If you wish to store statistics information in the CSV format, you will need to enable the “Write Statistics To CSV File” option.  You can click in the box to the right of the “Write Statistics To CSV File” option to specify a filename, or you can select the “Use Date For Statistics CSV File Name” option.

Clicking on the box to the right of the “Write Statistics To CSV File” option will cause an Open Statistics File dialog box will appear and you can select an existing statistics file or type in the name of a new statistics file.  If you open an existing file, the statistics will be appended to the end of the existing file.

Selecting “Use Date For Statistics CSV File Name” will cause WinFLEX to automatically create a file using the last 2 digits of the year, the 3 letter abbreviation of the current month, and the 2 digits of the current day of the month for a filename.  This file will be appended to if it already exists and will be closed at midnight.  After midnight, a new file with the year, month, and date will be opened and used for output.

When the CSV format file is used, the first line written to the file will be a description of the columns of statistics data.  The descriptions for each of these CSV fields is…

DATE – The date the statistics information was written.

TIME – The time the statistics information was written.

F6400N – Number of FLEX 6400 numeric messages decoded in the current hour.

F3200N – Number of FLEX 3200 numeric messages decoded in the current hour.

F1600N – Number of FLEX 1600 numeric messages decoded in the current hour.

P2400N – Number of POCSAG 2400 baud numeric messages decoded in the current hour.

P1200N – Number of POCSAG 1200 baud numeric messages decoded in the current hour.

P512N – Number of POCSAG 512 baud numeric messages decoded in the current hour.

GOLAYN – Number of Golay numeric messages decoded in the current hour.

F6400A – Number of FLEX 6400 alphanumeric messages decoded in the current hour.

F3200A – Number of FLEX 3200 alphanumeric messages decoded in the current hour.

F1600A – Number of FLEX 1600 alphanumeric messages decoded in the current hour.

P2400A – Number of POCSAG 2400 baud alphanumeric messages decoded in the current hour.

P1200A – Number of POCSAG 1200 baud alphanumeric messages decoded in the current hour.

P512A – Number of POCSAG 512 baud alphanumeric messages decoded in the current hour.

GOLAYA – Number of Golay alphanumeric messages decoded in the current hour.

F6400NC – Number of FLEX 6400 numeric characters decoded in the current hour.

F3200NC – Number of FLEX 3200 numeric characters decoded in the current hour.

F1600NC – Number of FLEX 1600 numeric characters decoded in the current hour.

P2400NC – Number of POCSAG 2400 baud numeric characters decoded in the current hour.

P1200NC – Number of POCSAG 1200 baud numeric characters decoded in the current hour.

P512NC – Number of POCSAG 512 baud numeric characters decoded in the current hour.

GOLAYNC – Number of Golay numeric characters decoded in the current hour.

F6400AC – Number of FLEX 6400 alphanumeric characters decoded in the current hour.

F3200AC – Number of FLEX 3200 alphanumeric characters decoded in the current hour.

F1600AC – Number of FLEX 1600 alphanumeric characters decoded in the current hour.

P2400AC – Number of POCSAG 2400 baud alphanumeric characters decoded in the current hour.

P1200AC – Number of POCSAG 1200 baud alphanumeric characters decoded in the current hour.

P512AC – Number of POCSAG 512 baud alphanumeric characters decoded in the current hour.

GOLAYAC – Number of Golay alphanumeric characters decoded in the current hour.

Note that the CSV file DOES NOT contain a daily total.  You can calculate this yourself using a formula in the spreadsheet.

### Keep Alive

The Keep Alive feature will alert you if no messages have been decoded for some configurable amount of time.  This usually happens when the RF system goes “off the air”.  The Keep Alive dialog box has three time ranges that you can set so that you can have different timeout periods during different parts of the day.  You can select the start time (at the beginning of an hour), the stop time (at the end of an hour), and the timeout (the maximum amount of time that can elapse without a message being decoded).  If the current time is between the start time and the stop time and no messages have been decoded within the timeout period, WinFLEX will begin playing the ALERT.WAV wave file (if you have a sound card) or beeping the speaker (if no sound card is installed, or the sound card is in use by another application).  A dialog box will also appear indicating that the Keep Alive timer expired.  The alert will sound every second until the alert dialog box is closed.

When configuring the time ranges in the Keep Alive dialog box, make sure that time ranges do not overlap.  WinFLEX will not allow you to enter time ranges that overlap each other.  For example, you can’t have one range start at 2:00 a.m. and stop at 9:59 a.m. and have another range start at 9:00 a.m. and stop at 11:59 a.m., since the 9 o’clock hour would be overlapped.  You can have a range go from before midnight to after midnight.  For example, having a range start at 8:00 p.m. and stop at 5:59 a.m. is perfectly valid (as long as no other time ranges overlap this 10 hour period).

If the timeout changes from one time range to another, WinFLEX will reset the Keep Alive timeout when the new time range comes into effect.  For example let’s say you have a time range with a 10-minute timeout and no messages have been decoded for 8 minutes.  If the hour changes into a new time range and the new timeout is 5 minutes, the Keep Alive alarm will not occur for another 5 minutes, even though the RF network has already been off the air for 8 minutes.  The Keep Alive timeout is not reset at the start of an hour as long as that hour is in the same time range as the previous hour (or has the same timeout as the previous time range).  For example, let’s say you have a time range from 8:00 a.m. to 4:59 p.m. with a timeout of 10 minutes and the current time is 9:55 a.m.  If the RF network goes off the air at this point you will get a Keep Alive timeout at 10:05 a.m. (not 10:10 a.m.)

The Keep Alive dialog box has an “Enable Command File” option that allows you to enable an external program that you wish to run if the Keep Alive timeout occurs.  The external program, or command file, can be any executable file (.EXE, .COM, or .BAT filename extension).  When you check the “Enable Command File” option, you must also specify the command file you wish to run by clicking in the box to the right of the “Enable Command File” option.  This will bring up an Open File dialog box that will allow you to choose the command file you wish.  To provide arguments to the command file, you can use the “Command File Arguments” option.  Click in this box and type in the arguments that you wish to pass to the command file when it is executed.

## Help

### Help Topics (F1)

Selecting this menu item brings up the help information describing WinFLEX features and options.

### About WinFLEX

The About dialog box tells you the version number of WinFLEX that you are currently running.