ICOM RESET

| RADIO | # RESET |
| --- | --- |
| 2GAT | ## TURN RADIO ON,HOLD (LIGHT)&(FUN), TURN ON AND RELEASE |
| 02AT | ## S/N <34,000 –PUSH BUTTON ON MAIN BOAD NEXT TO LITIUM BATT S/N >34,000 – HOLD(FUN) AND TURN RADIO ON |
| 2SA | HOLD(LIGHT)&(MON) AND TURN RADIO ON |
| 2SAT | HOLD (FUN) & (A) AND TURN RADIO ON |
| 2SRA | HOLD (FUN) & (A) & (CLR)AND TURN RADIO ON |
| U2AT | HOLD (LIGHT) AND TURN RADIO ON |
| 3SA | HOLD(LIGHT)&(MON) AND TURN RADIO ON |
| 3SAT | HOLD (FUN) & (A) AND TURN RADIO ON |
| 4GAT | TURN RADIO ON, HOLD (LIGHT) & (FUN), TURN RADIO OFF |
| 4SA | HOLD(LIGHT)&(MON) AND TURN RADIO ON |
| 4SAT | HOLD (FUN) & (A) AND TURN RADIO ON |
| 4SRA | HOLD (FUN) & (A) & (CLR)AND TURN RADIO ON |
| U4AT | HOLD (LIGHT) AND TURN RADIO ON |
| 12AT | HOLD (FUN) AND TURN RADIO ON |
| 12GAT | TURN RADIO ON, HOLD (LIGHT) & (FUN), TURN RADIO OFF |
| 24AT | HOLD (FUN) & (A) AND TURN RADIO ON |
| 32AT | HOLD (FUN) & (A) & (LIGHT) AND TURN RADIO ON |
| 228 | HOLD (SQUELCH/MON) &(LOCK) AND TURN RADIO ON |
| 229A | HOLD (SET) & (MW) AND TURN RADIO ON |
| 271 | REPROGRAM RAM CARD |
| 275 | HOLD (M-CL) AND TURN RADIO ON |
| 375 | HOLD (M-CL) AND TURN RADIO ON |
| 448 | HOLD (SQUELCH/MON) &(CLOCK) AND TURN RADIO ON |
| 471 | REPROGRAM RAM CARD |
| 475 | HOLD (M-CL) AND TURN RADIO ON |
| 575 | HOLD (M-CL) AND TURN RADIO ON |
| 725 | HOLD (FUN) & (MW) AND TURN RADIO ON |
| 726 | HOLD (FUN) & (MW) AND TURN RADIO ON |
| 761 | HOLD (M – CLEAR) AND TURN RADIO ON |
| 765 | HOLD (M – CLEAR) AND TURN RADIO ON |
| 781 | HOLD (M – CLEAR) AND TURN RADIO ON |
| 900 | HOLD (MR) AND TURN POWER OFF AND THEN ON AGAIN |
| 901A | TURN RADIO ON, PUCH (CHECK) & (MW) |
| 970 | HOLD (MW) AND TURN RADIO ON |
| 1201 | HOLD (SQUELCH/MON) &(LOCK) AND TURN RADIO ON |
| 1220 | TURN RADIO OFF FOR A FEW MINUTES AND THEN PRESS SWITCH S1 ON LOGIC POWER ON |
| 1271 | REPROGRAM RAM CARD |
| 1275 | HOLD (M-CL) AND TURN RADIO ON |
| 1520 | TURN RADIO OFF FOR A FEW MINUTES AND THEN PRESS SWITCH S1 ON LOGIC POWER ON |
| 1600 | TURN RADIO OFF, WAIT A FEW MINUTES AND TURN POWER ON |
| 2400 | HOLD (SUB VOL) AND (MW) AND TURN POWER ON |
| 2500 | HOLD (SUB VOL) AND (MW) AND TURN POWER ON |
| 3200 | HOLD (FUN) AND TURN RADIO ON |
| 3210 | HOLD (SQUELCH/MON) &(LOCK) AND TURN RADIO ON |