# Field Report for the DCS

| Bug No. |  | DCS/067 |  |  |  | SYSTEM |  |  |  |  |  |  |  |  | VERSION |  |  |  | CLOSED |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date |  | 07/10/02 |  |  |  | DCS |  |  |  |  |  |  |  |  | No. |  |  | Date | Ver |  |  | Date | Type | ## By |  |
| From |  | W. Wood |  |  |  |  |  |  |  |  |  |  |  |  | T7.06 |  |  | 04/09/02 |  |  |  |  | MP |  |  |
|  |  | (NG) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | S/W |  |  |
| Country |  | UK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HW/SW |  |  |  | H/W |  |  |  |  |  | X | S/W |  |  |  |  |  | PCMMC |  |  |  |  |  |  |  |  |
| Feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type |  |  |  | New |  |  |  |  | X |  | Bug/Always |  |  |  |  |  | Others |  |  |  |  |  |  |  |  |
|  |  |  |  | Change |  |  |  |  |  |  | Bug/Intermittent |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Description (Detailed description of a problem or a feature, new or current) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When using a 3rd party voicemail system incorrect integration digits are sent to the VMAA port. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operation (Step by step sequence to reproduce a problem) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Program SLT station 217 as VMAA port using MMC 207 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Program Group 500 to overflow to the VMAA port after 10 secs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. When an extension calls group 500 and the call overflows to the VMAA port, the reason digit sent is for Direct Call e.g 1208 (default) as set in MMC 726 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. When an ISDN line call group 500 and overflows to the VMAA port, the reason digit sent is for a DID Call e.g. 8887300 (default) as set in MMC 726 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. In other words, at no time is the integration digit for an overflowed call from a group sent, what should be sent is 4500 (by default) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. Please note that this bug does not affect the SVMi-8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Configuration (The system environment, installation) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Customer/Site information (Customer/Site where the problem or feature addition of change request is reported) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Version of related software (to be filled in when necessary) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SP : ver 7.1 |  |  |  |  |  |  | AA : 2.04 |  |  |  |  |  |  | PRI ver 3.4 |  |  |  |  |  | BSI : |  |  |  |  |  |
| LP : |  |  |  |  |  |  | VD : |  |  |  |  |  |  | BRI : |  |  |  |  |  | PCMMC : |  |  |  |  |  |
| Programming (programming required to reproduce a problem or required for a new/modified feature) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Remarks (Related problems, history, etc.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Management results |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Duplication |  |  |  |  |  |  | Status |  |  |  |  |  |  | Waiting for more information |  |  |  |  |  |  | Requested By |  |  |  |  |
|  | Success |  |  |  | Failed |  |  | Open |  |  |  |  | Closed |  |  |  |  |  |  |  |  |  |  |  |  |