| To : ALL | Doc. No. | SECYYMM-XX-X |
| --- | --- | --- |
| C.C. : | Country | ALL |
| ## From: Harry Kim | Tel: Fax: E-mail: |  |
| Subject : OfficeServ 12 MCP S/W release notes |  |  |

Overview :

OfficeServ 12 MCP S/W release

12 MCP S/W:	V2.72 (’07.10.18)		OS12V272.zpg

## New Version Changes

’07.10.18. V2.72

(1) MCP S/W changed items

[Added Features & Modified items]

- VoIP Stack License was added.

- According to the VoIP License Policy, VoIP Stack License is added. As for more detailed background of this, please refer to the announcement from Export team or Export team or Marketing team.

- In order to use H.323 stack and SIP stack, you have to put the LICENSE KEY into FEAT LICENSE KEY option in MMC841.

- H.323 Stack and SIP Stack License are added. The available number of H.323 Stack License and SIP Stack License can be shown at the FEATURE LICENSE STS option in MMC841.

- H.323 Stack License is necessary to use H.323 trunk. The customer is allowed to use H.323 trunk as many as the number of H.323 Stack License.

- SIP Stack License is necessary for SIP Trunk and IP-UMS/IVR. SIP STACK ALLOW option is added in MMC841 so the customers can specify the available SIP Stack number per each type that uses SIP protocol.

- The following is detailed description on SIP STACK ALLOW option in MMC841.

- MAX COUNT: The total number of SIP Stack allowed in the license (Read only)

- FREE COUNT: The number of free SIP Stack which is not yet assigned for the each type (Read only).

- SIP TRUNK: The number of SIP Stack for SIP Trunk. It can be assigned up to the number of FREE COUNT

- IP-UMS/IVR: The number of SIP Stack for IP-UMS/IVR. It can be assigned up to the number of FREE COUNT.

- Note) Before users upgrade existing system with the earlier MCP software than V2.72 to MCP V2.72, they need to check whether or not they have been using H.323 TRK and SIP TRK. If they are using those features, they have to get the VoIP Stack License Key before they upgrade the MCP software to V2.72. If not, they are not able to use these features.

- Refer to the attached 1Q Problem List (check 1Q number started with ‘N’)

Refer to the attached 1Q Problem List (check 1Q number started with ‘P’ or ‘D’)

# 1Q Problem List

S/W Compatibility Table :

(1) OfficeServ 12 MCP S/W

| MODEL | MCP | OSM |  |
| --- | --- | --- | --- |
| VERSION | V2.72 | V2.17 |  |
| DATE | 07.10.18 | 07.08.08 |  |

’06.11.20 V2.69

(1) MCP S/W changed items

# Added Features & Modified items

- Refer to the attached 1Q Problem List (check 1Q number started with ‘N’)

# Fixed Bugs

- Refer to the attached 1Q Problem List (check 1Q number started with ‘P’ or ‘D’)

# 1Q Problem List

’06.07.25 V2.66

(1) MCP S/W changed items

# Added Features & Modified items

- Refer to the attached 1Q Problem List (check 1Q number started with ‘N’)

# Fixed Bugs

- Refer to the attached 1Q Problem List (check 1Q number started with ‘P’ or ‘D’)

# 1Q Problem List

’06.03.23 V2.63

(1) MCP S/W changed items

※ 06.03.23 V2.63 change

If system has no MAC address, system is running as default MAC.

This way was applied our previous version.

※ MCP S/W file extension was changed to “.zpg” from ‘.pgm”.

(“.zpg” file is the compressed MP file.)

We will release the compressed MP file only from MCP V2.63.

For using the new compressed MP file, must change startup program of version 1.03.

For the detail, please refer to the attached file and release note for startup V1.06.

# Added Features & Modified items

- Refer to the attached 1Q Problem List (check 1Q number started with ‘N’)

# Fixed Bugs

- Refer to the attached 1Q Problem List (check 1Q number started with ‘P’ or ‘D’)

# 1Q Problem List

’05.11.24 V2.60

(1) MCP S/W changed items

# Added Features & Modified items

- UMS function Enable at OffceServ500 and OfficeServ100. This function same as OfficeServ 7200.

- DUAL AP Function for SMT-R2000 Added. To use this, setting MMC849 Select AP Type to “DUAL AP”.

- MMC845 WLAN SWICH option Added for supporting WLAN SWITCH.

- WIP5000M phone changed to using MMC119 CID DISPLAY. The CID is always stored, but the WIP phone display is following MMC119.

- To support New TEPRI card, new card type(PR2) added. But Old and New TEPRI card’s card type is identical. So there is a rule to update the TEPRI card.

- To update the s/w of Old TEPRI, TEPRI card s/w version should be below 3.xx and file name must be started with “PRI” or “pri”.

- To update the s/w of New TEPRI, TEPRI card s/w version should be above 4.xx and file name must be started with “PR2” or “pr2”.

If above condition doesn’t match, MMC will displays error statement.

- [N051011035]	CLI or DDI as option for permanent dislplay on keysets. UK Only.

MMC119 “CONV DISP” option is added. There are 4 options in CONV DISP, “CID NUM FIRST”, “CID NAME FIRST”, “DID NUM FIRST”, “DID NAME FIRST”.

If MMC110 “NOT.CONT CID” option is set to OFF, CID Number/Name or DID Number/Name that assigned in MMC ”119 CONV DISP”should be displayed on keysets permanently.

- [N051014010]	[ITALY_IIR001] “TRSF RCV” feature in MMC701 is added.

When this feature is set to ON, a user can not receive any transferred extenal call from other stations.

- [N051014010]	[ITALY_IIR003] Translating ISDN call value meaning. It is changed.

[ITALY_IIR006] Change setting value of HOOK FLASH MAX TM from 200ms to 130 ms in MMC501. ITALY Only.

- [N051014010]	[ITALY_IIR004] Displaying the Station Name on the top line of LCD during the internal call.

MMC300 “RBK STN NAME” option is added. When this option is set to ON, the Station Name is displayed on the top line of LCD during the internal call.

- [N051014010]	[ITALY_IIR007] Reducing to ZERO sec the overflow time when all members are out of the group.

MMC601 “ALLOUT NEXT” option is added. When this option is set to ON and station group members are all log-out, the call send to the NEXT PORT immediately.

This option is not allowed to UCD group.

- [N050930008]	DB loading flow is changed. When the system DB breaks at system starts, the DB of Flash memory is loaded.

And the DB of Flash memory is saved automatically at assigned time in MMC815. MMC815 default “AUTO SAVE TM” is changed from “00:00(means not saving)” to “03:30”.  OS100/OS12 Only.

- [D050831020]	[GS_SME_037] OHVA call involving Softphone/ITP5012L to/from other IP phone has 2way speechpath. When an ITP5012L or Softphone is involved in a OHVA call to/from another IP phone, the OHVA call has 2way speech path and loses speech path to the original call that was barged into with OHVA.

It should modify MCP s/w only, following is new spec.

- If IP phone(A) connects outside or PSTN party(B) then another IP phone(C) can OHVA. In this case, new MGI assign for OHVAing IP phone(C).

The voice path connect: C – MGI(1) -> MGI(2) – A, A-MGI(2) -> B

- If IP phone(A) connects IP phone(D) then another IP phone(C) cannot OHVA.

In this case, 3 MGI required.

- [N050902012] [GS_SF_137] Changes to MMC 244. It's modified.

- [D050912013] [GS_SME_040] Allow 4-digit SVMi Sound Prompts to be entered in MMC 748 Assign VM MOH. It's modified with STA request.

# Fixed Bugs

- [OS210] SPNet Forward Problem Fixed.

- [P051011034]	[OS172] Not possible to set Ext Forward from DISA. This problem is fixed.

A user can set ALL, BUSY, NANS external forward after calling in on a trunk set fo DISA.

1. Incoming call from DISA trunk

2. A caller presses forward ID assigned MMC724

3. If forward destination was assigned in MMC102, a caller presses a forward type and *. Forward type is set.

(ex. 60 + 1 + * = all call forward set)

4. If forward destination was not assigned in MMC102, a caller can assign forward destination number after pressing forward type.

(ex. 60 + 1 + 70011234 + * = all call forward set)

- [P051011036]	[OS199] ‘0’ line digital phones prevent incoming calls on 701(PRI port1) from forwarding. It is fixed. OS500-M Only.

If a AOM is connected to DLI card, Some database of 1st port in trunk card inserted in 1st slot was changed. (MMC400 TRUNK INC DND, TRK FORWARDING)

- [P050926058]	[SF044] Centralized voicemail and group overflow in remote node.

It is problem that happen transmitting group member’s number to host node when overflowing to centralized voicemail group. It was fixed so that the call should go to the group mailbox.

- [P050930001]	SPAIN PRS problem is fixed.

If “PRS2” is assigned in MMC414 for SPAIN, this operation will be satisfied for SPAIN PRS spec.

When an outgoing call is made, if the opposite end clears down first, the trunk will be released. When an incoming call is received and the opposite end clears down first, the trunk will be reased. SPAIN Only.

- [P050926056]	[GS_SF_125] Call Record does not update display when Barge In is used.

When a user tries Barge In on station that is using Call Record, Barge-In soft menu does not displayed. But a user can presses “BARGE” key. This is problem.

It was fixed so that a user could not operate Barge-In feature when Call Record is activating. If a user tries to press “BARGE” key, “barge-in failed” message is displayed to LCD and error tone is heard.

- [P050928034]	When Large LCD phone is set has ADMINISTRATOR for HOTEL feature, the SMDR key has not soft key function.

SMDR soft key function was not applied to Large LCD phone. The Soft key function of Large LCD phone’s SMDR key was added. ITALY Only.

- [P051004038]	[SF079] Hotel/Motel room status report printing data twice. This problem is fixed.

- [D051005041]	[Soft_SME_022] Called party Softphone user receive the Call Recording Softkeys display. Remote Softphone caller should not receive Call Record Softkeys. This problem is fixed.

- [P050921055]  [GS_SF_141] UCD report Month wrong. It's fixed.

- [P050921056]  [GS_SF_140] MMC 829 and UCDview option should be deleted. It's fixed.

- [P050921057]  [GS_SF_139] Busy Call Park key problem. It's fixed.

- [P051005036]  [IT060] Recorded MSG cannot be changed the second time when 2 ports are assigned as MOH in AA. It's fixed.

- [P051005037]  [IT061] Some Telecom Italia services for Operator do not work. It's fixed and SEI only.

- [P051011042]  [IT064] Sytem Conference has some problem when ISDN Feature is set ON (in MMC861). An external party hang up at conference mode and other parties hear the congestion tone. System does not release the B-channel because of PI. It was fixed so that system could release the B-channel immediately when an external party hangs up at conference mode. PI don’t apply at conference mode.

- [P051012034]  [OS217] With the country code set to Holland – If an incoming DDI call is not answered at an extension or does not follow a call forward, the DDI call will ring at the destination programmed in MMC 406. If this destination in MMC 406 is a local extension or group, the feature works correctly. If the destination in MMC 406 is a networked extension or group using SPnet, the DDI call will just carry on ringing at the original extension. This problem is fixed.

- [P051012036]  [GS_SF_143] MMC 409 changing trunk data. When a trunk card (TEPRI or TRK) is installed in slot 1, The default status of some of the data entries form the first trunk circuit is changed from default.  The problem is only on the first trunk port (7001). It is fixed.

- [P051012027]  [IT067] OS Operator cannot download the speed dial from the system if 950 SPD are set ON. It is fixed.

- [P051103001]  [GS_SF_144] BD-PMS keep alive pulse missing. It is fixed.

- [P051103003]  [GS_SF_145] BD-PMS not responding to all commands. It is fixed.

’05.09.13 V2.48

(1) MCP S/W changed items

# Added Features & Modified items

- New ITP added (ITP-5112L, 5121E, 5114E, 5107E)

- Changed to decide Port by Conn ID or Channel ID to Option by PRI card

- Group Pickup, My Group Pickup key added to Large LCD Phone.

- ITP-5112L and ITP-51XX added to MMC841 SYS IP OPTN's PHONE VERSION

# Fixed Bugs

- Problem correction that Ring Signal does not happen if do Conference by R/D Trunk

- When it is SPEAKER mode, problem correction that do not become Error Display at ISDN Call Clear

- Problem correction that Navigation Key is processed by QLCR Port

- When it is ACCESS DENY, problem correction that do not become Routing by LCR Next Group

- Port that Tel Number does not exist to MMC606 SPEED BLOCK is modified so that do not displayed.

- WHEN CHANGING THE STATION LANGUAGE, THE FIRST LINE OF THE DISPLAY CHANGES CORRECTLY, BUT THE SECOND LINE OF THE IDLE DISPLAY(DATE AND TIME) STAYS IN ENGLISH. NEEDS TO BE FIXED ON 100 R1/R2 AND 500R1/R2. It’s fixed. (STA Request: 1Q ID- D050719022 )

- [GS_SF_134] Message Waiting to SLTs not working. It’s fixed.

- [GS_SF_135] Call record during conference call. It’s fixed.

- MGI Channel will lock up after receiving a VoIP Network call from remote node to an unplugged ITP Phone. It’s fixed. (LSP: OS0062)

’05.07.13 V2.46b

(1) MCP S/W changed items

# Added Features & Modified items

# Fixed Bugs

- If ABW is active when agent logs in, UCD Agent Status shows Log In and Idle instead of ABW and Idle. It’s fixed. (STA Reported)

’05.06.20 V2.46

(1) MCP S/W changed items

# Added Features & Modified items

- [SEUK PCR112] The default setting of the HOOK FLASH MIN timer for use of the new SLT from Novita. It’s changed from 80msec to 70msec (OS100 only)

- Finish display and E.164 Number 100 processing function addition

- MMC 761 Tax Rate Setup (Hotel/Motel) function modified. It is modified to display third decimal place from second decimal place. (STA Req)

- Analog TRUNK CID RX processing function is applied in the all countries.

- In case of use SVMi by UCD ment, It is modified so that UCD ment can input to 4 seats. (STA Req and Mainly for OS7200)

  - When specify UCD Group's Next Port by VMS UCD Group, is modified so that can input MMC607's UCD First msg & Second msg to 4 seats.

  - When specify MMC224's WAKE-UP AA msg Group by VMS UCD Group, is modified so that can input message num to 4 seats.

- Can not listen imagination tone if apply Net.LCR to use LCR function code better beside LCR CODE. So USER must be able to specify Trunk Group to do route changing a Dial digit.

→ MMC763 SECOND LCR addition (Table number : 200)

1) IN DIGIT : after press E-LCR CODE user a Dial Digit

2) OUT DIGIT : In case of a Dial Digit of user becomes Matching to Table's IN

DIGIT actually, number (if IN DIGIT was not inputted input

prohibition) to rise from obscurity to outside

3) USE LCR NUM : When pressed E-LCR1, E-LCR2, what LCR CODE among

E-LCR3, E-LCR4, specify that use Digit inputted to IN DIGIT in

case it is. If ALL selected, IN DIGIT in relevant Table can be

used with every matching.

4) NEXT ROUTE : Input Trunk Group number to use when digit that become

Matching to IN DIGIT was inputted. When was inputted, digit

that become Matching to relevant Table's IN DIGIT in case it is LCR goes changed OUT DIGIT and does so that follow MMC710's LCR DIGIT.

5) (200)Times Table is no item that input above IN DIGIT, OUT DIGIT, USE LCR

NUM. This Tables specify Trunk(group) number or LCR to use when digit that press E-LCR CODE and do not become Matching to IN DIGIT was inputted.

→ MMC724 FEATURE CODE : E-LCR1, E-LCR2, E-LCR3, E-LCR4 addition

=> To MMC 724 E-LCR specify.

=> If E-LCR Code entered, DIAL TONE attached.

=> If DIGIT that is specified to MMC763's "IN DIGIT" inputed, change as

"OUT DIGIT"

=> Select trunk Group that is specified to "NEXT ROUTE" and dial "OUT DIGIT"

=> In case "NEXT ROUTE" is specified by LCR "OUT DIGIT"to MMC710 LCR

DIGIT Confirm whether become match and does so that do Normal LCR dispatch of a message.

=> When dial DIGIT that is not registered to MMC763's "IN DIGIT", Dial "NEXT ROUTE" that is specified to MMC763's last Table.

# Fixed Bugs

- [GS_SF_111] Chain call forward no Answer with wrong mailbox greeting. It’s fixed.

- [GS_SF_112] MMC 829 has incorrect default data. It’s corrected partially. PRINTER TCP is changed to 9100. But LAN TCP is not changed. Because Port 5100 ~ 5106 is a common port which can be used by other application. So did not change existing LAN TCP default value

- [GS_SF_115] Network DND Display Problem. It’s fixed.

- [GS_SF_116] No speed dial pauses sent to PRI. It’s fixed.

- [OS164] Barge in over network (SPNet) works but the microphone of the extension which is barging is ON instead of OFF. It’s fixed

- [OS190] UCD Agents are put into WRAP-UP once their extension has been rung. It’s fixed.

- [OS191] When set a paired extension as a DND Forward destination of its paired partner, the system ‘freezes/locks up’ and has to be rebooted. It’s fixed.

- Problem correction acting by G729a in case of do MGI2's Codec type by G711.

- Problem correction that do not become Call in WIP in case of input # to MMC846 INSERT DGT.

- [LSP OS BUG0046] SLT voicemail ports drop out on internal calls. It's fixed

- [LSP OS BUG0053/0047] Disconnect signal is not sent to SLT voicemail ports after end of incoming ISDN trunk call. It's fixed.

- [iDCS100 Bug063] Invoking PRS2 in MMC414 stops call duration and call cost getting inserted in the SMDR output. This problem also reproduced in OS100/500. It’s fixed.

- When using Economic keysets(3key), there is call-back problem through the network by using SPNET protocol. It's fixed. To use CB function over the network, press TRSF key and CB Feature code (deafult 44).

- Among ISDN Outgoing Conference, there is a problem that announcement is heard to remainder member if other person ends the call. It's fixed.

’05.06.02 V2.44

(1) MCP S/W changed items

# Added Features & Modified items

# Fixed Bugs

- Certain 800 # (AA/IVR) can not receive DTMF tones. It is fixed.

- ISDN 3PTY problems are fixed.

- SLT callback problem is fixed.

- ISDN Supplementary Service problems as below are fixed.

-> Keypad Facility problem.

-> Forward on/off, view problem.

- [IT054] System wrongs the call from the incoming logs list if LCR is set ON. It is fixed.

- [OS088] If a call into the system is made via an AA card (or VM card set as AA), and the call is directed to an extension that has a “Divert all” set, the call fails. It is fixed.

- [OS122] Cannot transfer a call Externally using the speed dial feature if the extension trying to transfer is barred outgoing access in the toll tables and COS tables. It is fixed.

- [OS144] ‘Missed Call’ displayed when the WIP5000M is either a member of a group or paired to another extension and the call is answered by another member, paired extension or picked up. It is fixed.

- [OS152] When attempting to dial an external number stored in the ‘Incoming Calls’ list on a WIP5000M handset, the call will be unsuccessful as the trunk access code is not inserted. It is fixed.

- [OS171] CLI routing does not display DDI info and always shows NONE. It is fixed.

- [OS178] Changing the Tech password blanks screen. It is fixed.

- [OS180] Dial a group that is overflowed to a UCD group, clear the call and the call key remains lit. It is fixed.

- It is fixed “No audio” problem using inter-node forward under SPnet configuration. (STN of node A -> STN of node B(NOANS FWD to STN of node C) -> STN of node C)

- It is fixed the REQ_ALIVE MSG dose not send to SVMi card periodically.

’OS100 05.03.22 V2.44 / OS500 05.03.16 V2.44

(1) MCP S/W changed items

# Added Features & Modified items

- In Newzealand, CID trunk option is added in MMC 425.

- [IT049] 1 on busy feature is also available at intercom call.

- AA or VMS ports can be assinged in MMC614 USE GROUP station type.

# Fixed Bugs

- System Reset problem is fixed.

- When SVMi port is locked up, the SVMi card send hook-on message to system to clear lock-up status. At that time the system make this port to idle state and clear lock-up status.

- [IT048] Networking transfer does not work after using network diret pickup feature. It is fixed.

- Group listen soft key is removed at Soft phone

- When restarting the system, card LEDs doesn’t off. This problem is fixed.

- System freezing problem is fixed when using CTI conference feature.

- When transfering the call to different node and transferred station is set to external call forward, tranfer does not work. It is fixed

’05.03.07 L2.44, M2.44

(1) MCP2 S/W changed items

# Added Features & Modified items

# Fixed Bugs

- Voice channel not opened when MCP and MGI IP type is PRIVATE w PUBLIC in MMC830 and MMC831. It’s fixed.

- Dialing Problem when using H.323 overlap dialing. It’s fixed.

’05.03.03 L2.44, M2.44

(1) MCP2 S/W changed items

# Added Features & Modified items

- When Speed Blocks are freed, the data in the block is cleared. (UK)

- In MMC835, add more sub option of DTMF TYPE (MGI3).

- MMC 809. System language removed. The German and Portuguese are almost English.

- PCMMC download speed improved.

- Large LCD phone SMS feature changed. Can send to Large LCD phone only.

# Fixed Bugs

- In case of CID trunk, Trunk answer timer doesn't work problem fixed.(SEI)

- PCMMC upload/download incorrect problem fixed.(system side)

-> 860,841,806,705,105 are MCP fixed, others are OSM fixed.

- When keying to Large LCD Phone, it’s often to changed into call display state. It’s fixed.

- When more speed block allocated to station, there is a problem to allocate

other station’s speed block. It’s fixed. (STUK)

- When uploading data from PCMMC to system after memory clear, informations related to IP Phone are not updated. It’s fixed.

- WIP Phone registration fail when work with WBS Combo. It’s fixed. (STA)

- In OS500, fixed MGI available checking problem with MGI at SLOT1.

’05.02.04 L2.43, M2.43 (STA Only)

(1) MCP2 S/W changed items

# Added Features & Modified items

- Add extension auto hold function. If AUTO HOLD in MMC110 is ON and push the DT/DS/Call key, current call changed into HOLD state. ICM AUTOHOLD option added in MMC210 and must be ON to use extension auto hold. (LSP)

- Add MCID processing result (from CO) display on the LCD of DGP when push MCID key. (LSP)

- Menu Key function of 2 Line LCD DGP(DS 5021/14 or ITP 5021/14) changed.

  - If cancel key pushed when destination extension number displayed during FORWARD set, destination extension number cleared.

  - if cancel key pushed when there is no destination number, move to upper menu.

- When PRI to VOIP tandem call and user busy, instead of sending busy tone, tandem cause and release the call promptly (caller process opposite busy). (STA)

- Add Overlap dial function using H.323 and Overlap Dial option in MMC834.

- When incoming call which is made by Q-sig Networking tandem received, node number excluded from call waiting display. (STA)

- When Phone Call forwarded to ITP phone with no MGI channel, Error process was changed : Internal call - error tone & display, External call - transfer to operator (GS_SF_085)

- When Incoming CO call terminating to ITP phone with no MGI channel, Error process was changed : Internal call - error tone & display, External call - transfer to operator. (GS_SF_087)

- When Network call terminating to unplugged ITP phone, Error process was changed : Internal call - error tone & display, External call - transfer to operator. (GS_SF_090)

- When incoming call over VoIP Network received, error process methods changed.

- No MGI channel and No trunk : next route

- No MGI channel and No destination : no next route

- “plug out” message displayed when incoming Networking call is destination out of order

- When incoming Networking call received by Group call, Group information displayed. (STA)

- No CID display option adapted in WIP phone. When RCV option in MMC312 is NO and MMC119 is NO DISPLAY, No CID display in WIP phone.

- Call can be transferred when WIP phone is locked like a DECT Phone. (LSP)

- When WIP phone registered, phone Type (WIP or IPP) check function reinforced.

- MMC 863 removed. Available in Korea only.

- In MMC 825, SPnet Digit send option added. When MCP signaling, not use MGI for DTMF sending.

- In MMC SVMi-12E display change to SVMi-20E.

- In MMC408, can set same MOH source for Hold MOH and AA MOH.

- In case of OfficeServ 500, MMC 813 enabled.(STA only)

- In case of SoftPhone, hold retrieved by hold key when idle state. BGM feature is removed.

- PCMMC 890 port clear added.

# Fixed Bugs

- Daughter board display Error fixed. (STA)

- MMC210 PAIR NO RING option does not work with unconditional group. It’s fixed (STUK_OS116)

- License key display problem in Key MMC. It’s fixed. (STA)

- On DS 5021/14 or ITP 5021/14 telephones: When using the Navigation Menu Key, the FORWARD SET options can only be used to program a forward destination, the forward cannot be enabled from this menu. It’s fixed. (STUK_OS061)

- When the option LCR with CCC is used the tone returned from the carrier cannot be altered with MMC501, MCL Delay Timer. It’s fixed. (STUK_OS083)

- A transferred call to an extension with Auto Answer and Headset mode enabled is presented through the speaker and not the headset. It’s fixed. (STUK_OS098)

- BSI card works but is not recognized in MMC 806. It’s fixed. (STUK_OS107)

- Unable to Camp On a Transferred call over VoIP Network. It’s fixed. (STUK_OS127)

- The 2nd line of the LCD of both ITP-5014D & ITP-5021D keysets goes off and on again approximately 12 seconds after the minute is updated. It’s fixed (STUK_OS128)

- When WIP Phone paired with Keyset, a forward set on the keyset does not immediately update the WIP Phone. It’s fixed. (LSP)

- CALL FWD NO ANS ON CENRALIZED VM FAILS. It's fixed. (GS_SF_077)

- 800 number Conf/hold/trf issue. It's fixed. (GS_SF_082)

- ITP Phones changing Station ID’s. It's fixed. (GS_SF_091)

- No transferring of calls to stations call forwarded all or busy across the network. It's fixed. (GS_SF_093)

- Network trunk ring on second call button issue. It's fixed. (GS_SF_096)

- Network PRI trunk on second call button issue. It's fixed. (GS_SF_097)

- Network Call waiting sending link ID station number. It's fixed. (GS_SF_099)

- Network calls no speech path, “out of order”. It's fixed. (GS_SF_100)

- Network calls wrong display on station group calls. It's fixed. (GS_SF_102)

- Group conference displays Issue. It's fixed. (GS_SF_103)

- Remote IP network node sending out DTMF tones. It's fixed. (GS_SF_104)

- WIP Phone one way speech problem. It’s fixed. (SEI)

- STA in-house switch reset problem fixed when PRI incoming.

- MCP and SVMi state mismatch problem fixed.

- When PCMMC 840 upload, the user ID removed problem fixed.

- In PCMMC 714, can’t set “D” or “A” at destination field problem fixed.

’04.12.30 L2.43, M2.43 (STA Only)

(1) MCP2 S/W changed items

# Wireless LAN Features related items

- Added Calling Waiting Display Function at WIP5000M: confirmed tone & waiting call information displayed at WIP5000M. Maximum 2 call.

- Modifying WIP5000M registration using User ID & Password. Users can WIP5000M registration using random user id & password. (Before, the registration possible in sequence.)

- Modified WIP5000M idle status display. Many status information (including forward, hold, lock, etc) is displayed at idle state.

- Modified Message Function at pushed Message button of WIP5000M navigator. Users can handle the Station messages and VMS messages.

- Added Basic WBS24 Time Zone function to remote WBS24.

# Added Features & Modified items

- AUTO UPDATE TIME function in MMC861 is updated: When enabled, if PRI(TE), use to update Time/Date Chip (excluding AUSTRALIA). If incoming valid time date from QSIG PRI, use to update Time/Date Chip.

- Add SIP phone at MMC740 Virtual Number

- IP of ITP is cleared after 500ms when Station type changed from desktop to mobile in MMC844.

- SoftPhone display changed SAVE&REPEAT => SAVE/REPEAT

- System support CPU load command added in DataView.

- Operator Information sent to DateView when RTO Mode or RING Plan changed.

- Add interface of TRK GROUP MEMBER CHANGE command in DataView, and send result of EXT/ TRK GROUP MEMBER CHANGE to DataView.

- 8DLI2 Card interface is added. (8DLI2 is a new version of 8DLI card)

- SPK-HOT LINE added in MMC300.

  - If option is On and SPK key pressed, automatic dialing with SPEED 00.

  - If option is On and off hook, MMC 306 hot dialing operated.

  - If option is Off, MMC 306 hot dialing operated.

- MMC740 SVMi connect type: Card, DLI, IP-UMS

- Display restriction added in Large LCD Phone interface when display Option which is not displayed in KMMC.

- In case of Russia, Error tone frequency is set to 420Hz same as other tone frequency.

- To support more alarm code in SYSTEM ALARM ASSIGNMENTS, alarm index digits count modified. (2 → 3)

- Change default value of INPUT GAIN and VOICE VOL in MGI DSP OPTIONS.

(31 → 32)

- Change default value of TSW GAIN CONTROL in LEVEL AND GAIN as below.

|  | Before (db) | After (db) |
| --- | --- | --- |
| VOIP → WLAN | 0.0 | 1.9 |
| WLAN → VOIP | 0.0 | 1.9 |

- Change the max value of ISDN INT DGT TM in SYSTEM TIMERS. (15 → 60)

- Also in case of USA when ISDN PROGCON in MMC210 is on, if an outgoing call receives PROGRESS message from ISDN trunk, the call will connect without CONNECT message.

- Timers of all port in SVMi card will be detached when attached timer in SVMi card is detached.

# Fixed Bugs

- In case of Q-sig network wiith PRI trunk, Remote Hold doesn’t work. It’s fixed.

- TEPRI card which is in E1 mode is intermittently out of service after system restart.

It’s fixed.

- MFR DLY TM is not working. It’s fixed.

- In case of incoming Q-sig Networking call into station which has no available call button, Q-sig networking FWD is not working. It’s fixed.

- Call fwd display function on tandem networking using PRI Q-sig is fixed. (GS_SF_084)

- When SoftPhone executes call record, soft key is displayed 2 line format. It’s fixed.

- ITP Registration related problems are fixed.

  - Phone Lockup for 30sec problem when set relocation or regist clear of WIP Phone.

  - User ID change problem when many ITP is in the remote dynamic NAT.

- Fixed the problem not going to next group, when UK LCR not available group. It’s fixed.

- Fixed the problem not operating Internal no answer forward.

- CTI related problems are fixed.

  - In case of Networking Call, Call is piled up after route optimizing. It’s fixed.

  - When UCD group call overflowed, although original UCD group member answer and release the call, overflowed group receive wrap-up status. It’s fixed.

  - Telno is not updated normally when PCMMC uploads number plan which is changed by CTI S/W. It’s fixed.

’04.11.11 L2.41, M2.41

- MCP2 S/W changed items

: This S/W is for general release of WLAN feature. So there is no need to set MMC to view the WLAN related MMC previously. In case of V2.37, there is an option in MMC 861 USE WLAN FEATURE.

# ITP Features related items

- In MMC840, PUBLIC/FIREWALL type added in IP TYPE option. (STA request)

( When there is a call between DGP(or ITP) and Remote ITP in the NAT system which has firewall function, MGI will change the destination port of RTP message which is received from Remote ITP into source port, and send RTP message to Remote ITP )

- In MMC840, PRIVATE IP option added. (STA request)

( When there is a call between Remote ITP in the same NAT system, if PRIVATE IP is used, MGI is not allocate to Remote ITP.

MGI is allocated for Remote ITP which is in the different NAT system )

# Wireless LAN Features related items

- To support Spanish in the WIP Phone, Spanish codes for WIP Phone include in the system.

- Change Default codec from G.711 to G.729 in the COMBO Type AP(same as BASIC Type AP)

- When system changes from COMBO type AP to BASIC type AP or BASIC to COMBO, system clears related informations.

# Added Features & Modified items

- Improve the OfficeServ Operator initial speed. (more faster)

- In MMC 841,  ‘-‘ character automatically inserted when License Key for Soft Phone is inserted. (when input more than 8 characters)

- When S0 station receives incoming call from ISDN trunk, MCP passes only ASCII value of UUI, display data.

- Although ISDN DDI/MSN state, AUTO ANSWER function is enabled.

- In PCMMC840, REGIST CLR and SW UPGRADE option added.

- Change VOIP RE-ROUTE TM(MMC501) default value. (5sec -> 15sec)

- Receiving CLI number from ISDN, “0” is inserted only when NB TYPE (MMC428 BRI OPTION) is NATIONAL. If area code (MMC718 MY AREA CODE) only has “0”, “0” is not inserted.

- OFFHOOK RING (MMC300) style changed. (Italy request)

( OFF -> OFFHOOK RING off, ON -> 2 Beep (external). 1 Beep (internal) )

# Fixed Bugs

- Unable to dial in Overlap mode from an S0 bus. It’s fixed (STUK_OS095)

- Some MSN are duplicated in MMC421 if 2 BRI cards are installed in 2 subsequent slots. It’s fixed (ITALY_IT022)

- OFF HOOK RING of ITP is not deactivate although it is set OFF in MMC300. It’s fixed (ITALY_IT032)

- The system freezes when you confirm cost clearing on SMDR hotel function key. It’s fixed (ITALY_IT034)

- RETRY function doesn't work when the called party is busy. It’s fixed (ITALY_IT035)

- Calls made from a FAX station don’t decrease the phone credit of the paired room. It’s fixed (ITALY_IT037)

- External dialed number is wrong when LCR is enabled depending on DIGITS LENGTH in MMC710. It’s fixed (ITALY_IT039)

- Call Forward doesn't accept an external number destination already set. It’s fixed (ITALY_IT040)

- The problem of PRI name service in Canada. It’s fixed.

- Garbage CLI number inserted when the S0 station receive ISDN call. It’s fixed.

- Both System and MGI’s IP type are PUBLIC ONLY (MMC830, 831), MGI is not allocated. It’s fixed.

- During conference with WIP Phone, there is a problem which first conference member’s call disconnected when conference member increase 3 to 4. It’s fixed.

- CID display problem when Blind transfer. It’s fixed

- When intercom paging from WIP Phone to DGP, there is no broadcasting voice on the DGP. It’s fixed.

- Display problem in the WIP Phone when push the last Paging Zone Number to paging. It’s fixed.

- During intercom between WIP Phones, WIP Phone A which is HOLD state because  WIP Phone B push CONF Key, is disconnected after 30 ~ 40 sec. It’s fixed.

- Dial pulse call has a problem in Spain, Netherlands, Denmark, and Israel. It’s fixed.

- MCID problem in Australia. It’s fixed.

- When s/w upgrade from V2.13, tel number 33xx overwrote by mobile phone problem fixed.

’04.09.09 L2.35, M2.35

(1) MCP2 S/W changed items

# Wireless LAN Features related items

: This feature is implemented from V2.30. But, in this version, the related MMC are changed to hidden because this version is not for WLAN feature. To test WLAN feature, refer to the following items.

- To use the WLAN feature, MMC 861 USE WLAN FEATURE must be set to ENABLE. If not, the WLAN related MMCs will not be shown. In case of USA, this MMC does not appear in MMC 861. To use WLAN feature in case of USA, memory address 0x04000000 must be changed to 0x98762345. Then, MMC 861 USE WLAN FEATURE will be shown.

- The progress tone of WIP will be serviced by self-generating tone source of WIP like as ITP. So. WIP <-> WIP call does not need MGI port because the ringback tone does not serviced through the MGI port. (It needs new WIP, WBS and WLI S/W)

- Some case of 2 line lcd display for WIP is changed.

- Korean language will be serviced for WIP.

- IP Address duplication check is inserted.

- In KMMC mode, easy IP Address input method is added by pressing A ~ E button in MMC 845 and 848.

- Voice path control problem when handover is modified.

- SOFT MENU & Phone Setting MENU processing of WIP is improved.

# Enhanced Networking feature related items

: This feature is implemented from V2.20. In this version, some problems are fixed in addition.

- MMC 501 RECALL DISCONNECT time does not work for Remote Attendant. It’s fixed.

- When the Remote Attendant is invalid or in CID/DID rejection status, the local attendant will be called instead of Remote Attendant.

- Error display for Enhanced Networking feature is modified.

- CO-CO disconnect time does not work when the UCD final destination is a remote system extension. It’s fixed.

- In V2.34, AA cannot transfer calls across networked nodes. It’s fixed. (GS_SF_066)

- In a scenario with multiple nodes connected via IP networking using centralized SVMI, CO calls coming into the remote nodes do not get forwarded to the station’s mail box but goes to the main system greeting.: It’s fixed (GS_SF_065)

# Fixed Bugs

- When the VOIP trunk call is answered by BiVMS port, the DTMF signal from VOIP trunk does not delivered to BiVMS port. This problem is fixed.

- When the call destination of Analog trunk is an extension and the extension is on DND status, the incoming call is ringing to the operator.

- MMC 993 TRACE SIO RESET is set to YES several times, the system has frozen intermittently. It’s fixed.

- Intermittently CLIP review does not work: Fixed (STUK_OS052)

- When using ans S0 BUS and mapping MSN numbers to the circuits. The MSN numbers lock after a short period of time. The system needs to be reset to free the MSN numbers. : Fixed (STUK_OS068)

- UCD & PERI UCD statistics are incorrect. : Fixed (STUK_OS071)

- Calls are intermittently jumping the queue when in the UCD group queue. : This problem can be solved by Call Priority Feature. (STUK_OS072)

- When Set Relocation is used on Virtual SLT the move takes place and the extension can receive incoming calls but extension cannot make any outgoing calls because the extension cannot break Dial Tone. : Fixed (STUK_OS073)

- If a UCD group has only one member and in MMC 607 GROUP BUSY NEXT is enabled for this group, when this group overflows to another normal group, a second call in the queue hears noting but the call rings the group with internal ring tone cadence.: Fixed (STUK_OS077)

- AA version display error in M type is fixed.: Fixed (STUK_OS079)

- Call Costing does not work correctly for incoming calls: Fixed (STA_GS_SF_044)

- Call No Longer Available Display: Fixed (STA_GS_SF_055)

- 1st port on Analog trunk cannot ring on incoming calls: Fixed (STA_GS_SF_063)

- PRI card error rate is improved.

- When service type of S0 station which has DATA type is changed to SPEECH when it dials outgoing calls through the ISDN trunk. It’s fixed/

- Intermittently AA ports are locked-up. It’s is fixed

- Conference call making via LCR dialing by CTI application does not work. It’s fixed

- When the call is screened transferred to busy station and CBK is pressed, the additional busy event is sent to the CTI application. It’s fixed.

- When an ITP is waiting for a MMPG, then any call is not possible from another ITP: Fixed (ITALY_IT030)

- No CID info sent over network during blind transfer: Fixed (STA_EN018)

- Incorrect display when transferring calls across the network: Fixed (STA_SF019)

# Added Features & Modified items

- Loop Trunk Disconnection by Tone Check is added.

(To use this feature, MMC 861 LP TRK TONE DISC option is set to ENABLE and MMC 400 TONECHK DISC option is set to ON)

- License Key for Soft Phone and Spot News is added.

(In MMC 841, you can enter the license key. Then, the system will check the entered license key and show the license status corresponding the entered license key)

- Trunk Auto MOH Connection feature is added.

(To use this feature, MMC 210 TRK AUTO MOH option is set to ON and MMC 400 AUTO ANSWER option is set to ON. The delay time for auto answer can be changed in MMC 503 DISA ANSWR time)

- MMC 210 PAIR NO RING option is newly added. (STUK_PCR107)

- MMC 210 TRSF VT KEY option is newly added.

(If this option is set to ON, it will work like as VT key operation when the user transfer the call to Voice Mail by TRSF key)

- Belgium default is changed (STUK_PCR109)

- Spain default is changed (STUK_PCR70)

- ITALY request are added.

. MMC 206 BARGE IN WITHOUT TONE option is removed.

. MMC 210 ORT COST 0 is added

. MMC 300 PASS COST is added

. MMC 300 RECL TRSF is added

. MMC 300 A-RECL2OPER option is added

. MMC 601 GROUP BUSY option value is changed(OFF/ALL/1oB)

. Vacant Message default is changed to Italian language.

. SMDR key operation is changed as requested.

- MMC 835 SILENCE SUPPRESSION default is changed to DISABLE.

- In PCMMC 806, the power status will be shown like as KMMC 806.

- When VM RESTART is set to YES, the halt process will be done automatically before SVMi card reset.

- When the SVMi card does not have VPM board, the system will not reset the SVMi card. (STA_Request)

- MMC 701 ALM CLR option only will be shown when the HOTEL feature is enabled.

- When the first port of SVMi card is set to MOH, the SVMi card does not alive. It’s fixed.

- When the BI-VMS GRP member does not assigned, the VM Halt does not work. It’s fixed.

- USA default is changed (Programmed Message, MMC 711, MMC 824)

- Message Count display is changed. All of VM, SMS, Mail message count will be displayed

- MMC 324 SLI2 GAIN control is added. It is only for 16SLI2 card.

- MMC 830 CTI SERVER ADDR and NEWS ADDRESS items are newly added.

When the Application is in connected status, new connection request from pre-assigned IP Address is only accepted. The new connection request from other IP Address will be rejected.

- In case of UK, the MCL WAIT time is applied for LCR outgoing call instead of PAUSE TIME.

- In MMC 834 and MMC 837, “CLIR with Number” option is added (Spain Request)

- In MMC 419 and MMC420, “CLIR w NUMB” option is added.. (Spain Request)

- MMC 836 USE MULTI E.164 is added. (Spain Request)

- MMC 836 GK REGISTERED is added. (Spain Request)

- MMC 861 ISDN SS FEATURE is added. (Except USA, KOREA)

: This MMC is for new BRI/PRI card supporting ISDN Supplementary Service. (These are under developing)

- MMC 861 SLI2 PRS SERVICE is added. (Korea Only)

- MMC 718 INS DGT is added (Korea Only)

- MMC 300 USE INS DGT is added (Korea Only)

’04.05.07 L2.33, M2.33

- MCP2 S/W changed items

- In case of KOREA, WLAN -> ATRK tsw gain default is change to 0 dB from –6 dB.

- When the WIP is called which battery is off, the caller hears some cycle of ringback tone and then the call is dropped by system. In this case, the LCD display “No response” will be shown to caller in current version and the next call will be treated as plug-out.

- (BASIC AP) The used codec type for voice path between WIP and ITP is changed to follow the first member MGI codec in MMC 615 LOCAL ITP from assigned ITP DSP TYPE in MMC 840.

’04.04.27 L2.32, M2.32

(1) MCP2 S/W changed items

-  WIP LCD display will be changed to 2 line LCD format from medium font display after following action. Feature activation in handover condition may occurs some problem because of message loss. This problem is under analyzing.

: When system feature key is pressed during conversion.(ex: TRSF, HOLD, CONF…)

: When WIP dial the feature ID .

- In case of VOIP Networking, OUTBAND DTMF type will be served. (MGI3 S/W must be changed)

- MMC 840 PUB TO PUB is added. This option is added to allocate MGI between Remote ITPs in same private zone.

- MMC 840 SIG TYPE is added. This option is added to change UDP/TCP signal type.

(ITP V3.xx only supports TCP signal type. When ITP V2.xx attempt to connection the signal type will be changed to UDP automatically even though user set the TCP signal type)  TCP signal connection is under testing

- When Set Relocate is used on Virtual SLT the move takes place and the extension can receive incoming calls but the extension cannot make any outgoing calls because the extension cannot break Dial Tone. : Fixed (OS/073)

- WLAN related gain default is changed. These changed values applied after DB initial. If you don’t want DB initial, enter these values in manually.

MMC 835 MGI3 INPUT GAIN: 31 -> 38

MMC 841 ITP TX GAIN/HSET: 1:23->29, 2:24->30,…, 8:30->36

ITP TX GAIN/MIC: 1:23->29, 2:24->30,…, 8:30->36

MMC 805 TSW GAIN

| WLAN->DGP | 0 dB |  | DGP->WLAN | 0 dB |
| --- | --- | --- | --- | --- |
| WLAN->SLT | 0 dB |  | SLT->WLAN | 0 dB |
| WLAN->ATRK | -6 dB |  | ATRK->WLAN | 0 dB |
| WLAN->DTRK | 0 dB |  | DTRK->WLAN | 0 dB |
| WLAN->DECT | 0 dB |  | DECT->WLAN | 0 dB |
| WLAN->VOIP | 0 dB |  | VOIP->WLAN | 0 dB |
| WLAN->SVMi | 0 dB |  | SVMi->WLAN | 0 dB |
| WLAN->WLAN | 0 dB |  |  |  |
| VOIP->ATRK | -6 dB |  |  |  |

’04.03.30 L2.31, M2.31

- MCP2 S/W changed items

- Using COMBO WBS, user hear echo of DTMF when press DIGIT during trunk conversation.

: Fixed

- MMC 845 CODEC LIST default is changed: G.711A Only (USA: G.711U Only)

- In case of M type, CID detection by RCM card does not work: Fixed

- In case of Large DGP, SOFT MENU screen goes to blank after some action. (V2.30 Only)

: Fixed

’04.03.22 L2.30, M2.30

(1) MCP2 S/W changed items

# Including V2.13 & V2.20 fixed items

# Wireless LAN Features are added.

- WLAN interface specification is changed

  - WLI card: L-Max 3 Card, M-Max 1 Card

  - WBS24 Combo: L-Max 24, M-Max 8

  - WBS24 Basic: L-Max 80, M-Max 40

  - WIP: L-Max 240(default: 120),M-Max 120(default: 60)

    - The total count of IP Phone plus WIP Phone does not exceed the above max count.

    - The count of IP Phone and WIP phone can be changed in MMC 844 IP STN TYPE

  - Combo Type and Basic Type WBS cannot be used in simulataneously.

(Used AP type can be in MMC 849)

  - The WBS can support maximum 4 channels of simulataneous call.

  - In case of WBS24 Basic, the system tones are provided to WIP Phone by MGI port.

  - The WBS channel is reserved at time of outgoing and ringing.

  - In case of WBS 24 Basic, outgoing and incoming of WIP Phone can be made when the free

mgi port is exist. When the conversion is active and there is no need mgi port, then

system release the allocated mgi port. (WIP <-> WIP, WIP <-> Local IP Phone does not

need a mgi port for conversion)

- WLAN related MMC

  - MMC 202 WLAN REGST: 0000

  - MMC 844 IP STN TYPE

  - MMC 845 WLAN PARA

[WLAN]

SYSTEM ID: if chaned, all WIP’s registartion information are cleared

SYSTEM KEY: if chaned, all WIP’s registartion information are cleared

1ST DNS IP, 2ND DNS IP: Domain Name Server IP

2ND WBS IP: if chaned, all WIP’s registartion information are cleared

CODEC LIST:

RF CHANNEL: if chaned, all WIP’s registartion information are cleared

VERSION: WLAN module S/W version

TFTP SERVR, TFTP FILE

WBS TX PWR: All WBS’s TX Power value change

CLR WBSREG: All WBS’s registration information clear

[CWBS/BWBS]

IP ADDR, GATEWAY, NET MASK: WBS IP address

MAC ADDR: WBS MAC Address

VERSION: WBS Version

STATUS: WBS current status(ON/OFF)

RF CHAN: use rf channel. Selectable in range of pre-assigned WLAN:RF CHANNEL list

TX POWER: WBS Tx Power change

PARA CLR: WBS MAC ADDR and VERSION information clear

[SIP] SIP protocol related timer change

RE-TRANS T1, RE-TRANS T2, RE-TRANS T4, GEN RING TM, INV RING TM,

GEN NO RESP, INV NO RESP, REQ RETRY, PROVISIONAL

  - MMC 846 WIP INFO

REGISTERED: Registered Status View

LOCATED: Current Status View (Attaced or Detached)

PHONE TYPE: WIP Phone Type

WLI NUMBER, WBS NUMBER: WIP location information

IP OFFSET: Offset of MMC 848 IP List

IP ADDRESS, MAC ADDRESS: WIP IP address and Mac Address

USER ID: For Registration Check

PASSWORD: For Registration Check

INSERT DGT: When user dial over 5 digit and the digit does not start trunk, trunk group, or

other feature id, this pre-assigned digit is inserted in front of user dialing digit.

  - MMC 847 WLAN RESET

: WLI card reset, WBS reset

  - MMC 848 WLAN IP/MAC

: IP Address assigned which can be allocated for WIP Phone

(MAC List is for furture. It is not used now)

  - MMC 849 WLAN CONFIG

REGISTER voWLAN: New Registation Available or Not

WIP REGIST CLEAR: WIP Registration Clear(Forced/Normal)

WBS WEP SERVICE: Use of Encryption Key

WEP KEY: Encryption Key Assign

STATIC WBS IP: STATIC WBS IP Assign Use or Not

STATIC WIP IP: STATIC WIP IP Assign Use of Not

SELECT AP TYPE: AP TYPE Assign(COMBO AP/BASIC AP)

# Some bugs are fixed.

- [OS/069] Retry fails to dial on BRI/PRI if LCR is programmed and LCR Allow is on in

MMC 400.: Fixed

- [OS/070] When Review Key is programmed and calls are received, the LED on the

Review key lights and the key can be accessed and used in the normal manner. If

any changes are made to any key-phones' programming in MMC 722, the Review Key

LED goes out on all key-phones. The function still works but there is no longer any

indication that there is a new call logged: Fixed

- [OS/066] In OfficeServ manager V3, MMC 315(station relocate) is not available: Fixed

- [GS_SF_060] When 2 stations are paired and both set to AUTO ANSWER MODE, a call

transferred to the primary gets auto answered by the secondary. : Fixed

’04.03.02 L2.20, M2.20

- MCP2 S/W changed items

# 1) Enhanced Networking Features added

- Centralized Attendant

- Message Key and Light across Networked nodes

- Group Overflow with networking

- Paging across Networked nodes

- VT key across Networked nodes

- NS/BLF between Networked nodes

- Call Coverage "CC" Keys

- Remote Hold across Networked nodes

- AA Card Networking Support

# 2) Additional Enhanced Networking Features added

- Trunk Ring Destination to Networked Nodes

- UCD Final Destination to Networked Nodes

- AA Error Destination to Networked Nodes

- Call Pickup across Network Nodes : Direct pickup, Hold pickup and Page parked pickup

- Enhanced Centralized Voice Mail : Soft Key and LCD display

# 3) Additional Enhanced Features added

- Call Priority : MMC759.CID > MMC714.DID > MMC419.Port

- Enhanced Distinctive Ring : MMC759.CID > MMC714.DID > MMC419.Port

- CID Call Intercept : MMC759.CID Ring Destination and Call Reject

- Flexible BIVMS Group number

- Real SMDR Duration

- Self Features Reset

- When press MMC724. “ALLCLR” ID + Station Password, the following features cleared.

- Station Lock, Forward, Alarm Reminder, Programmed Message, DND, Station Message

- MMC831. MGI IP Type

- MMC737. All AA port MOH

- MMC840. IP phone Time Zone

- MMC 841. ITP TX GAIN/MIC,ITP RX GAIN/SPKR is added

- MMC835. MGI Codec & Multi Frame

- DS-5000S series phone

- Italy ISDN Supplementary Service

- MMC210. AUTH TO ACCT option

- MMC115. Programmed Message 19.,20 default change (GS_SF_057)

- Enhanced PCMMC Administrator : MMC100.DND mode and MMC433.ERROR status added

- Forward Destination clear by pressing HOLD after dialign Forward Feature ID

- DND Forward Destination Clear when Forward set is cleared by dialing Forward Featrye ID and 0

- 2 line LCD Phone is removed in MMC 612 because there is no method of deleting not answered

destination (GS-SF_058)

- Opposite number display when barge-in.

- MMC 813 is removed in case of U.S.A (STA Request)

- RCM2 card interface is added

# 4) Bug Fixed

- When the MFR/CID DSP made busy in MMC853, the feature doesn’t work properly. : Fixed

- After Diagnostics time in MMC854, the Free MFR counts decreased. : Fixed

- When changes Boss/Secretary by PCMMC, the 5012L doesn’t update boss/secretary status properly.: Fixed

- The MGI card slot act LED doesn’t work properly.: Fixed

- When detects answer PRS from analog Trunk, the DNIS network event doesn’t send to CTI link properly.: Fixed

- AME one way speech over Q-sig networking (STA SF055) : Fixed

- Tandem call to busy station over network (STA SF049) : Modified, but couldn’t duplicate.

- When the TEPRI card restart, the conference call doesn’t clear completely. (LSP One touch solution) : Fixed

- If UCD Final destination sets CFU or CFB to same UCD group, when all members log out condition, the system freezing. (STA) : Fixed

- Day & Date not displayed correctly when systems/phones set to a language other than English: Fixed (STUK, OS/056)

- When Keyphone set to Spanish there is corruption on the display during ringing: Fixed (STUK, OS/057)

- Cross talk when call forward is set to restricted number: Fixed (STUK, OS/059)

- Networking trunk call display error fix when incoming calling number matches station number

- Attendant recall does not work when NOA FWD is set: Fixed

- Networking tandem fax call drop problem is fixed.

- Lock-up problem when the station which All/Busy external fwd is set is called in CONF mode

: Fixed

- LCR next port routing & TGR next port access does not work when e1/t1 trunk link is disconnected. : Fixed

- DS key display of Large LCD Phone update when station name or tel number is changed.

- Hold call pick-up possible by INQUIRE key when there is no CID name.

- Trunk lock-up when SVMi port calls a station and hang-up which is set to CFU.: Fixed

- SCP2 & LCP2 S/W changed items

- RCM2 D-BD is added and cid Tx function is enabled.

- DS-5000S series DGP is added.

- Rx volume levels added Falcon DGP is same with old DGP. : Fixed

- When more than 2 AA card are in M system, MCP is restarted. : Fixed

- RCM CID error which is installed 3rd cabinet is fixed

- Large LCD Phone enblock dialing error is fixed

- Intermittent noise and call drop out and corrupted dial tone on DECT handsets when system configured as Denmark

’03.11.20 L2.12, M2.12

(1) MPP S/W changed items

- When the destination of LAN PRINTER is set to BOTH or PRINTER and IP Printer does not connected, the system goes to freeze status.(SEC)

: Fixed

- When the FAX outgoing call is made via the CO trunk of other system which is linked with VOIP qsig trunk, the FAX service does not work.(STA)

: Fixed

- When the large phone is overlab mode and MMC 210 ORG DIAL LOG is set to ON, the call log was not left.(SEC)

: Fixed.

- When the Attribute of SmartMedia file is Read-Only, the pgm upgrade of this file does not work.(STA)

: Fixed.

- MMC 841 ITP handset and mic tx gain’s default value changed. MMC 807 ITP handset and mic tx defaule value changed. This is for voice quality improvement.(SEC)

- When the DDI call destination is in another network node and that destination sets external call forward, the call forward external doesn’t work.(STA)

: Fixed

- When the DECT phone answer callback ring after CBK NOANS TIME, the system reset.(STUK, OS048)

: Fixed

- When all MGI channels are not available, the error message changes “All MGI CHs not available" to "No MGI CHs are available".(STA, MG_SF_022)

- After diagnostics time in MMC 854, the system reset.(STUK,STA)

: Fixed

- When MGI card network error, the system resets that MGI card every 12 hours.(SEC)

- Alarm time in MMC 501 doesn’t work.(STA)

: Fixed

- Improve the H.323 call fail problem caused by H.323 call stack overflow while LCS call test. It happens when call setup messages are left un-processed.

: Fixed

’03.11.01 L2.11, M2.11

(1) MPP S/W changed items

- If the user press ‘*’ or ‘#’ twice when incoming call is ringing, the MCP2 goes the freeze mode or restart mode. To reproduce this problem, the SVMi card must be installed.

: Fixed.

- [STUK:OS/042] Calls from AA Card setn to extension that has External Divert set do not follow the External Divert and fail.

: Fixed

- [STUK:OS/043] When a remote ITP terminal user loses connectin to the OfficeServ, calls directed to this extension fail with the ‘PLUG OUT’ message

: Changed. In this case, the call is forward to the PRESET fwd destination.

- The FAX service via VOIP Networking Trunk is newly added.

- The LCR class can be set for the trunk in MMC 310.

- BD-PMS interface problem is fixed. In previous version, the invalid PMS data is send to the PC from keyphone system.

- When the FWD destination is set to a external number, only the trunk number is shown on the CTI application because the system send the invalid digit length. This problem is fixed.

- In case of VOIP trunk(H323 trunk/SIP trunk/Networking turnk), the caller information does not shown on CTI application. This problem is fixed.

- In case of incoming QSIG Networking call, the DTMF signal does not generated by system. This problem is fixed.

- When the FWD is set by Large Phone, only ALL FWD can be set. This problem is fixed.

- In HOTEL feature, the BILL is added after CHECK-IN, the next day room cost follows the newly added BILL cost. This problem is fixed.

- In QSIG networking, the analog trunk incoming of system A is answered by extension of system A, then the call is transfered to the extension of system B. When the extenstion of system B finish the call after MMC 501 ROUTE OPTIMIZE TIME, the analog trunk of system A does not released. This problem is fixed.

- The FAX service via SIP Trunk problem is fixed.

- When the DTMF type of MGI3 card is OUTBAND, the DTMF signal does not send to the called party after the called party generates the DTMF signal. This problem is fixed.

- In case of SIP trunk, DTMF signal does not delivered via MGI3 port. This problem is fixed.

[Notice] Maximum Channel Use Limitation of MGI3 card

The current MGI3 card does not support 16 calls simulatenously. So, you must set the MMCs to avoid that all port of MGI3 card are used simultaneously. This setting can be made in MMC 615 MGI GROUP. The MGI3 port which cannot be used must be deleted for all types of MGI group in MMC 615.

’03.10.01 L2.10, M2.10

(1) MPP S/W changed items

- [STUK:OS/038] Unable to File Upload SPPV204 Using PCMMC V209.

: Fixed

- [STUK:OS/040] System resets when a number is dialled by an extension that has Barged-in

: Fixed

’03.09.23 L2.10, M2.10

(1) MPP S/W changed items

- In MMC 401, VOIP trunk can be assinged to PBX line to overcome the toll restriction.

- MMC 120 CALENDAR option is added. (0. CALENDAR, 1:PREV SCREEN)

- MMC 120 AOM CURSOR option is added. (PREV POSITION, 01~99)

- 32M SmartMedia interface is modified.

: It is fixed the system cannot read or write files which is located in over 16M area on 32M SmartMedia Card.

- When DECT SYSTEM CODE is changed via PCMMC, the saved dect system code is broken. This problem is fixed.

- QSIG tandem call problems are fixed.

-  LCD display error when the LCR tandem outgoing call is transfered to another

LCR tandem outgoing call

  - DID call wait option does not work in case of QSIG tandem call.

  - Call Forward dose not work in case of QSIG tandem call.

’03.09.16 L2.09b, M2.09b

(1)MPP S/W changed items

- Under the condition the system A and B are linked via H.323 trunk, turn off the system B. Then, extension of system A tries to call via H.323 trunk many times. After that, the system B is turned on again. When the system B is ready to work, the system A will be freezed.

==> This problem is fixed.

’03.09.04 V2.08

(1)MPP S/W changed items

- When MMC 607 GBUSY NEXT is set to ON, the 2nd UCD message is played incorrectly. It is fixed.

- MMC 824 DISP option is newly added. If this is set to Yes, it works same as earlier version.

If this is set to No, it is displayed like as a normal outgoing call.

- If you are attempt to call VOIP Networking call without setting of LINK ID in MMC 820, the MGI port reserved by the VOIP Networking Trunk will not be released. This problem is fixed.

- The Boss/Secr key display of the Large Phone is exchanged each other.

- The extension which is set relocated with the virtual extension does not follow forward Busy/No Answer setting when not active so far. In this case, the extension will be forwarded to All FWD destination.

- MMC 210 VOIP REALRBT option does not worked because MMC display processing has a fault. It is fixed.

- When new unconditional group call is camped on the ITP phone, which is in conversation with other party and is last ITP member of unconditional station group, the ITP phone will loose the voice path. It is fixed.

- [STUK:OS/021] UCD Supervisor Key cannot log in or out Agents by pressing Soft Button. It is fixed.

- The Soft Menu list does not shown when the Large LCD Phone goes to dialing mode from idle status. This problem occurs only the Large LCD Phone is in Overlab Dialing Mode. It is fixed.

- You can assign the AA MOH port by entering the MOH message number in MMC.736 and then select AAMOH port in MMC 308. After this, turn off the cabinet which is installed the AA card and then turn on the cabinet. When the cabinet is alived, you cannot find the preassigned AA MOH port in MMC 308. It is fixed.

’03.08.22 V2.07

(1) MPP S/W changed items

- Rerouting problem in Voip Qsig is fixed when trk group mode is DISTRIBUTE.

- VOIP QSIG route optimise problem is fixed.

- The ‘#’ digit in SLT will operate like as “OK” button during forward setting.

- The VOIP trunk is removed in MMC 401,402,410,425,503.

- ‘#’ during forward setting, the pressed digit will be saved.

- The Minimum Handset Rx level data will be sent to Large LCD Phone

’03.08.14 V2.06

(1) MPP S/W changed items

- When the caller uses LNR feature after outgoing call via LCR feature, the system will restart. This problem is fixed.

- The tone data of ITP phone is newly added for other countries as well as Korea, USA, UK, Italy and Australia. (Holland Request)

- MMC 601 “S0 STN GRP” type is newly added and it is changed the S0 station can be forwarded.(UK)

- It is changed the Large LCD Phone can use Conerence Softkey. (UK)

- The LCD display of Directory Dial for Large LCD phone is changed. (Upper line: Name, Lower line: Number) (UK)

- The Large LCD ITP follows the ITP volume in MMC 807.DGP VOL.CTRL.

- The H.323 and SIP call error rate is reduced less than 0.5% from 3%.

- The DND cancel can be done even though DND limited class.

- The PCMMC V2.07(03.07.31) does not support follow MMC item.

- MMC 601 “S0 STN GRP” type

’03.08.07 V2.05

(1) MPP S/W changed items

- Some Korean Language display for Large LCD Phone is changed.(뉴스/정보->뉴스표시,뉴스 그룹->상세 뉴스)

- When Station B in conversion with Trunk A attempts to transfer the call to Station C(Large LCD Phone) and release the call after Station C is answered, the LCD display of Station C doest not go to idle status. The problem is fixed. (STUK report)

- When MMC 119 is set to Name First and the user press the CID./NXT key during conversion to check the CLI information for camped call which has not CLI name information, the LCD shows CLI number instead of “no CLIP name” from this version. (STUK).

- When Station B in conversion with Station A attempts to transfer the call to Station C, the Station A cannot hear the confirm tone in case of Station A or Station C are IP device. (STUK)

- The cause value sent to the network when the destination of ISDN DDI call is in DND  is changed to 17(user busy) from 47.(STUK)

- When the Forwad COS is blocked, the destination field information for MMC 102 of PCMMC does not send correctly. It is fixed.(Korea)

- When the network send 1,2 or 5 for the destination status in R2 outgoing sinaling, the call will be connected. In Previous version, the call is cleared in these case. (Korea Only)

- Ii is fixed the SVMi message count does not displayed correctly.(STUK,STA)

- The T-switch gain is decreased 6dB when the MGI1/2 card has daughter board for echo cancel. The new MGI1/2 card S/W(After V1.17) must be used with MCP V2.05. If not, the voice quality will to bad because the voice is too high or low.

- It is fixed the some MGI port is blocked during the outgoing call via H.323 or SIP trunk.

- When the H.323 or SIP trunk has an incoming call during waiting the clear_co message, the old call is clear and the new incoming call will be accepted.

- Boss/Secr Message download error for Large LCD DGP is fixed.

- The LCD display error for BOSS key extension in Large LCD Phone is fixed.

’03.07.31 V2.04

(1) MPP S/W changed items

- MMC 805 ALL->VOIP gain changed from 0 to –6dB for echo chacelling.  (MGI1,2 only. Exclude MGI3)

- If “Cancel” key pressed during transfer, the state will return to previous state.

- Smdr print not operated problem in case disconnect of recalled call is fixed.

- In 2 line phone, if long digit key pressed in case of not allocated speed dial block, changed to do normal dial.

- During conversation with A, transfer to B and directory/speed dial menu access may be cause lock-up of B. This problem is fixed to not operated menu in this state.

- During conversation with A, directory/speed dial menu access may be cause lock-up of A. This problem is fixed.

- MMC841 Upgrade time and Phone version wrong display problem fixed.

- In MMC407, MMC 890, changed to input MGI number.

- In ip phone, off hook ring caused speech lost. We changed this tone to beep tone 2 times to overcome speech lost.

- In all EU except Denmark and Norway, if incoming ISDN call with calling number type is national, insert “0” to CLIP and delete in case matched with area code.

- In setting Boss/Secr with PCMMC, there are problem not operated immedeately (only operated with plug out & plug in). This problem is fixed.

- When PCMMC uploading, added data range check of MMC 834,835,836,840,841.

- In setting BRI/PRI oprion or S0 mode with PCMMC, there are problem not operated immedeately. This problem is fixed.

- In hotel funtion, if cost digit table is empty, call clear problem of outgoing with more than 10 digits. This is fixed.

- MMC 201 Option added.

  - ORG DIAL LOG: If this is “on”, all dialed digit inputed include error will be saved.

  - TIE TRSF RCL : If this is “on”, when transfer to TIE trunk, no answer recall operated.

  - VOIP REALRBT: If this is “on”, use real ringback tone from MGI.

- MMC 501 “VOIP RE-ROUTE TM” added. To re-route next group, if H.323, SIP call destination is no answer during this time.

- MMC 120 “CONV DISP” added. Selectable LCD low display during conversion. This will be not adapted yet at the phone s/w.

- MMC 841 “MGI ALIVE PERIOD” added. Moved from MMC 902. (In PCMMC, remained at 902.)

- System restart problem when received 9 or more digits starting with “0” from R2 trunk is fixed.

- When analog trunk mal function, system never recovery without card or system reset. This froblem is fixed.

’03.07.10 V2.03

- MPP S/W changed items

- IP phone’s TX, Rx gain date changed with test data.

- When system restart,  the problem erased the speed, cid, and log block of kdb-d is fixed.

- When H..323 or SIP outgoing using LCR, if no response, changed to rerouted next group.

- Changed to restart in case of set relocation of Large dgp.

- When R2 incoming in Korea, if the first digit is not “0”, automitic inserted “0” if digit length is more than 8.

- When Large LCD phone become idle, there are no spot news on message. This is fixed.

- MMC838 the last private IP initial value changed. This is following the type of system IP. Class A -> x.255.255.255, class B -> x.y.255.255, class C -> x,y,z.255.

- In USA Version, the problem BRI-S call not success is fixed. The reason is that the ISDN timer initalized with invalid value.

- During conference with large LCD phone, there are problem if changing the master twice or more. This is fixed.

- LCR function problem fixed. When all busy of the first trunk group and the next is ISDN group, there are some delay (ISDN inter digit time) to rerout. This delay eleminated.

- MMC616, there are some problem if MGI fixed user input and delete repeatly. This is fixed.

- If IP phone is disconnected, the phone version to PCMMC is changed to 255.255.

- “no cid info” will be not displayed if not cid trunk.

- If enblock redialing repeatly to IDSN or VoIP, there are the problem that “digit too long” is displayed and impossible call. This is fixed.

- In L system, if restarting the H.323 task during online, not possible to activate all channels because memory shortage. In this case, 60 channels will be activated, and the other 20 channels will be out of service.

- MMC841 MGI ALIVE PERIOD option added. MGI link check methed changed from ping test to alive message check (MGI->MCP). To use this function, must upgrade MGI s/w with new version. If not upgrading MGI s/w, must set this option to 0 (disable).

- If SCP restart, there are the problem that ther virtual ports about IP phone, WIP phone, DECT phone and VoIP trunk is operated abnormal. This is fixed.

- There are the problem the MGI port not released. This is fixed.

- During PCMMC all uploading, changed to make MGI paratemer download to MGI. (If not all uploading, must click card reset by user.)

- MMC210 Option added.

-  FRE FWD BUSY : When ON, a call arrives busy station, that is not set forward busy, if preset no answer destination is available, a call re-route to that destination immediately.

- MMC300 Option added.

- FORWARD OVRD : When the station is set to ON, if the station call to forwarded station, the call will not forward.

- RECL TO OPER : When the station is set to ON, if the station transfer a call and the destination doesn’t answer, the call will be recall to operator instead of the station.

- SLT PWR DISC : When the normal phone is set to ON, if the station receives real disconnect signal instead of busy tone or error tone. In case of VMAA ports or DATA RING ports are not reference this option, this type SLI ports are always receives real disconnect signal.

- MMC400 Option added.

- REPEAT CLI : This option determines what kinds of CLI number will be sent to the trunk to trunk call. (Trunk or Received CLI from Trunk)

- MMC419,420 "NB PLAN", "NB TYPE" Option applied to all country except USA and Korea.

- MMC425 CID trunk applied to Italy.

- MMC 501 "TSW CONN. DELAY" time applied to all country.

- MMC601 Option added.

- GRP AUTOANS : When this option is set to ON, a group call follows auto answer option also.

- MMC615 Group added.

- ITP PAGED : This determines what MGI ports can be used with station page to ITP keysets across a private IP network.

- MMC714 "MC" Option added.

- MC : The number of maximum call count.  When this sets 99, this feature will not work. When this sets 0, if a call comes that matched DID digits and DGT field then the system will be reject call. When this sets between 1 to 98, if a call comes that matched DID digits and DGT field then the system counts same DID digits call, and in case of the count is same or higher than this value,  the system sends busy signal to caller.

- MMC724 "Virtual SLT" and "Virtual DGP" changed to "Virtual EXT"

- MMC725 Option added.

- SET RELOCATION : If set to YES then the SMDR record will show set relocated information. If A change to B, displayed A on "EXT" and B on "ACCOUNT".

- ITP REGISTRATION : If set to YES then the SMDR record will show IP-based station to system connect and disconnect. IP address and signaling port displayed on "ACCOUNT".

- MMC727 Version display changed. If test version, displayed like "L2.02a", not "T02.02".

- MMC822 added. Assigns the virtual extension port type. Not separate Virtual SLT and DGP. Default initial is same with old version.

- MMC835 Option added.

- MGI3  : FAX Retry Count

- MGI2  : Silence Suppresion

Echo Cancel --- always enable

- MGI1  : Silence Suppresion

Echo Cancel --- always enable

- MMC840 DSP Parameter, Version, S/W Upgrade Option added.

- FRAME COUNT : This value determines the transmission interval time of VoIP packets. generated by the IP keyset. This data uses case of MMC 841 ITP DSP PARA DOWN = PHONE DATA only. The range is 10~40ms. Default is 10ms.

- JITTER SIZE	 : Decides the minimum time to consider delay for jitter adjustment. This data uses case of MMC 841 ITP DSP PARA DOWN = PHONE DATA only. The range is 10~90ms. Default is 20ms.

- TOS FIELD : An eight-bit binary value that will be utilized by external routers, switches, etc (that optionally support TOS-bit prioritization) – to identify the transport-priority value of data packets generated by the IP keyset. This value can be left at default value (00000) if your network infrastructure does not support this method of bandwidth management. This data uses case of MMC 841 ITP DSP PARA DOWN = PHONE DATA only. Default is all bits 0.

- S/W VERSION : Display IP keyset software version. This read only data.

- S/W UPGRADE : This is used IP keyset software upgrade request command. When this selects YES and press RIGHT soft key, the system requests to IP keyset software upgrade with TFTP IP address.

- MMC841 ITP Parameter, ITP S/W Upgrade option added. The other options are grouped according to its type.

- ITP TX GAIN : Sets IP-based keyset TX gain value. There are 8 levels which is the handset TX level of DGP. Each level have to set 0-63(-32dB ~ +32dB) .

- ITP RX GAIN : Sets IP-based keyset RX gain value. There are 8 levels which is the handset RX level of DGP. Each level have to set 0-63(-32dB ~ +32dB) .

- ITP DSP PARAMETER

* FRAME COUNT : This value determines the transmission interval time of VoIP packets. generated by the IP keyset. This data uses case of MMC 841 ITP DSP PARA DOWN = PHONE DATA only. The range is 10~40ms. Default is 10ms.

* JITTER SIZE : Decides the minimum time to consider delay for jitter adjustment. This data uses case of MMC 841 ITP DSP PARA DOWN = PHONE DATA only. The range is 10~90ms. Default is 20ms.

* TOS FIELD : An eight-bit binary value that will be utilized by external routers, switches, etc (that optionally support TOS-bit prioritization) – to identify the transport-priority value of data packets generated by the IP keyset. This value can be left at default value (00000) if your network infrastructure does not support this method of bandwidth management. This data uses case of MMC 841 ITP DSP PARA DOWN = PHONE DATA only. Default is all bits 0.

* DOWN : Determines data uses system-wide data or each phone data for IP-based keyset DSP control. Default is SYS DATA. If selected "PHONE DATA", applied MMC 840.

- ITP VERS UPGRADE :

* TYPE : MMC COMMAND : use MMC840 S/W UPGRADE menu.

PHONE CONN : automatic upgrade when phone connected.

AUTO TIME : all phone upgrade at set time.

* START(HHMM) : all phone upgrade start time

* INTERVAL (5~99 sec) : IP-based keyset software automatically upgrade interval time between phones. To avoid TFTP connection error.

- MMC861 system option added.

- AUTO UPDATE TIME : Sets enable(1) or disable(0) the system automatically time and date update form ISDN call connection message. If sets enable, when system receives ISDN call connection message, system checks valid data of encapsulated time and date, and update system time and date device.  If once updated, the next update will be more than 1 minute. Only the message from the reference clock activated card applied.

- SYSTEM SPPED BIN : Sets max 500(0) or max 950(1) the maximum number of system speed dial bin. If sets 500 then same in previous system. If sets 950, can assign system speed dial block is up to 950 bins and speed dial bin numbers always 3 digits(000 to 999). Even if station speed dial bin numbers are 3 digits(000 to 049).

- Operations when all MGI channel not available changed.

- In case DGP/SLT call to the group with IP phone and MGI channel not available : IP phone is excluded in the mode of sequential/distribute mode. In the mode of uncondition, if the group member is all IP phone, then displayed "All MGI Chs not avaliable". Otherwise, ringing to idle member(s).

- In case IP phone call to the group with DGP/SLT and MGI channel not available : DGP/SLT is excluded in the mode of sequential/distribute mode. In the mode of uncondition, if the group member is all DGP/SLT phone, then displayed "All MGI Chs not avaliable". Otherwise, ringing to idle member(s).

- In case the answering the call to unconditional group at above cases and MGI channel not available : Error displayed and failed to answer.

- In case the call incomming from voip trunk and and MGI channel not available : The call disconnected.

- In case the call incomming to IP phone from normal trunk and and MGI channel not available : The call re-routed to the operator.

- About Large LCD phone.

- idle status display changed.

- KMMC available. But cursor is not displayed yet because the phone s/w is not supported yet.

- If set to Boss/Serc, SECR MESSAGE soft key display changed.

- The problem is fixed which not displayed "page from 2001" when paged.

- The problem is fixed which not ring-back tone off when dispatch conference.

- UCD SP key changed. "MAIN MENU" soft key is displayed to return the top menu. "AGENT DATA" soft key instead scroll key is displayed to see agent data.

- If headset mode, Transfer LED is flashed. If Forward All, the LED is On.

- About IP phone.

- The problem is fixed which not connected with system when set relocated.

- The problem is fixed which not tone off when paged.

- The problem is fixed which not updated time/date at L system when changed time/date

with MMC 505.

- About Alarm Remort

- Changed the code of some Minor alarms.

MNG01,02 : Phone Disconnect & Connect

MNG03,04 : Off Hook Alarm & On Hook

MNG05    : MGI Packet Loss

MNG06    : MGI Packet Delay

- Changed the code of some alarms about MGI.

MJE01,02 : MGI Restart & Stop

MJE03    : MGI IP Duplicate

MJE04,05 : MGI Network Error & Recovery

MJE06    : MGI DSP Error & Recovery

- MGI Network Error Alarm is generated when MGI link error is continued more than 10 seconds.

- MGI DSP Error Alarm is generated when the DSP of MGI card has problem.

- When MGI Network Error or MGI DSP Error state, the channels to the card will not allocated.

- About R2 incoming

- The external forward all/busy/dnd functions.available about the incoming call from R2 trunk.

- The tandem function available from R2 trunk to other trunk. (R2 tandem method is not signaling pass. But after signaling ended, outgoing started. If no other digit received from R2 tandem during 3.5 seconds, the outging started.)

- In case R2 tandem, if the destination is multi-LCR, after received the digits count at MMC 824 "SZ", and then R2 signalling ended.

- About MGI block

- When MGI blocked, if the MGI channels is used, the "IP network error" is displayed and error tone generated to the user and the opposite. (If the user and opposite are voip trunks,  call cleared. MGI block include Card Restart, Card Stop, Link disconnect, IP Duplicate)

- The noise problem of IP phone is fixed. The noise generated by key touch in off-hook state.

- SMDR print problem is fixed when DND On/Off DND with DND key or CTI.

- One way speech problem is fixed which the tandem call to isdn trunk.

- The call disconnect problem is fixed which the tandem call to isdn trunk.

- The problem which the Trunk locked in trunk error state is fixed.

- MGI channel not release problem is fixed when no dialing and hook-off of seized voip trunk.

- If the incoming call to the virtual DGP, IP phone, WIP phone, DECT phone which is disconnected and no answer fowarded, then fowarded imedeately. But preset forward no answer is not applied.

- In case of trunk tandem, if toll restiction, not disconnected imedeately, but the error tone generated and after 10 seconds the call disconnected.

- In case DECT, WIP phone power off, tel number remove message sent to VDC linker.

- When call record end , the problem not sent conference end message to cti is fixed.

- In 2 line LCD idle display, change to not display sms count except korean language. "2 NEW MESGS" is applied to multi-language.

- In case vitual dgp changed to slt, the call is operated. But otherwize, pluged out.

- When redial, the problem "too digits long..." displayed and not operated redial function is fixed.

’03.06.10 V2.02

- MPP S/W changed items

- About New 2 line DGP

- In SVMi AME state the call answered with SEND key pressed.

- In conversation state, if user press menu key, then the problem of voice rx off problem is fixed.

- About Large LCD phone

- In case of boss phone, added the idle soft menu of “divert”.

- In conversation stae, added the soft menu of “conference”.

- In case of forward setting, added the soft menu to select forward type.

- On AOM, set of “GRP CONF” and “CALL LOG” key avaliable now.

- The BOSS key display of AOM is changed from “BOSS/SECR 1” to “BOSS 2001” or the name.

- About IP Phone

- Problems about Page are fixed. (include MGI control)

- Tone control problems are fixed. (Make use its own tone)

- If No Answer Forward set to IP phone with not registered, the call to the phone forwarded.

- Set relocation for IP phone is available now. In this case IP phone re-registered after 4~5 second.

- In IP phone or Large LCD DGP, Spot news group in/out message always sent to it. (no apected with info display on/off)

- When IP phone is outgoing via the analog or T1/E1 trunk, mic control changed to delete noise.  This is more modification in next version.

- Added MMC 841 options: ITP tx gain, ITP rx gain, multiframe count, jitter buffer size, tos field.  The IP phoneis not available now, available in next version.

- To send volume data to IP phone, use 1~8, not apected with MMC 805 tx level conversion.

- About Trunk Tandem

- MMC 400 REPEAT CLI option added. During trunk tandem call, If option is On, received CLI sent, otherwise CLIP table or CO Tel number sent.

- In case of tandem call , expand digits 16 to 20.

- In case of tandem call to enbloc ISDN line, if received all digits, the delay of 2 sec is deleted.

- In case of tandem call from QSIG to ISDN, node number of CLI deleted.

- About R2 Signalling

- In case of outgoing transfering to R2 trunk, the problem about outgoing trunk lock-uped are fixed. The problem occured when the call unscreened transfered before R2 sigalling completed.

- The max digits of caller number to R2 trunk set to 15, because of the error if 16 or more digits sent.

- In case of outging call to R2 trunk, when the call can’t success (the reason: destination busy…), the busy tone continued during 1 minute, then the call will be disconnected.

- About LCR

- In case of LCR second group outgiong call after account code pressed, the problem of deleted account code is fixed.

- In case of redialing of LCR, there was the delay(ISDN inter digit time) to go to second group. The delay deleted.

- About Normal Call

- In case incomming call from VOIP networking ringing, the problem sometimes not displayed the caller’s name is fixed.

- T-switch gain of VOIP trunk changed to follow from DTRK to VOIP.

- During the incomming call to UCD group member are ringing, if the new incoming call occurred, the new call will be overflowed, not previous call.

- In MMC321(USA case) calling party number, the display order is changed to station, voip trunk, and tunk. Not by Slot order.

- In case of incoming call that the user pressed account code, account code (not did name) displayed in SMDR.

- The CID name from PRI line is in FACILITY message in USA. Added the routine to save CID name when recevied the message.

- Even if the boss set divert, the call from secretary will be not to busy.

- About Virtual DGP

- Virtual DGP line type changed from DS-5021D to Falcon 28B in USA and Australia.

- If No Answer Forward set to virtual DGP, the call to the phone forwarded.

- About MGI

- When MGI unavailable (card stop, ip duplicate, alive check fail), the MGI using call should disconnet without any information. This will be changed next version.

- In case of IP duplication of MGI card, the alive test disabled before card restart.

- About Hotel/Motel

- If check in end time is more high than check out time, the cost is not calculated. This problem is fixed.

- After RSV service with 64B AOM, the problem DND led not recovered is fixed.

- About PC Console and VDC Linker

- In MMC 841, CTI LINK ALIVE Time (30-180 sec) added. If the time set to 0 sec(input with HOLD key), link alive checking disabled.

- If a phone disconnected, the tel number sent to CTI driver.

- When the call transferred is busy, the problem not CTI override is fixed.

- When conversation state, CTI deflect can be operated to the new incoming call from trunk.

- The problem not CTI divert to station group is fixed.

- When pickup of incoming call to group. The problem the incorrect station information sent to CTI is fixed.

- In case of multiple tel number deleted by Key MMC, the deleteded tel number to send CTI only, not all tel number.

’03.05.19 T2.02

(1) MPP S/W changed items

- MGI Card IP Duplication Alarm Added.

- Bugs about alarms about MGI are fixed.

- In large LCD phone, processing method of SVMi soft key changed.  Should be use correct SVMi software.

- In HOTEL function, call cost is applied on incomming call (call rate:8).

- Separated MGI1,2,3 options (MMC835).

- UCD port type checking error fixed.

- During ISDN dialling, the chain dial is available before alerting.

- If Easy Set request message about SMS, the ack response message will be sent in case of SMS open, clear, or buff.

- In large LCD phone, sometimes not downloaded large lcd data.

- In large LCD phone, TFSF Led will be set on or flash even if fwd1/dnd key is exist.

- In LNR function, the saved digits duplicated problem fixed.

- The tos field of MGI option added for QOS.

- In large LCD phone, if data changed, phone parameter will be sent.

- In large LCD phone, not checking speed block during speed no. search.

(2) SCP2 S/W changed items

- DLI link speed changed. Supported both 2k and 16k speed. 16k is only for Large LCD phone.

- The problem about abnormal ringing when SLT hook off.

(3) MCP2 Boot & StartUp S/W changed items

- The 32M byte Smart Card Supported.

’03.04.29 V2.01

(1) MPP S/W changed items

- MMC 816 CONF GAIN is newly added.

- For IP Phone operation under NAT configuration, real port and IP address conversion feature is newly added.

- PRI Calls transferred to group 501 recall to SVMi(529) main greeting and NOT to group 501 MB.

- If 21B keysets are programmed via PCMMC System Key Programming(MMC 723) using 21B selection, (trying to program 21D) the same button on 28 and 12 btn keysets is “fouled” and becomes ususable and illuminates red.

- The problem 12D display number do not match is fixed. VT529 -> VMTRANS 752, In/Out 501 -> I/O 702.

- In CTI blind transfer, not sending establish event when the station answer.

- In MMC102,MMC118, the problem showing illegal number (Trunk Group #..) is fixed.

- In Enblock ISDN line, without “#” we can’t lnr,snr,spd,efwd. The problem is fixed.

- Scrolling of last message from easyset in buttom line of 2line LCD phone.

- Large Phone must use its own alarm fuction. So, block MMC112, MMC116 about large Phone.

- With DECT phone, the problem speed dialing and LNR doesn’t working in stanby mode is fixed.

- Using PCMMC, the problem loading error PGM to Smartmedia is fixed.

- In the large phone with INFODSP on, the idle display cant recovered when SPK button pushed and then SPK button pushed again. The problem fixed.

- In MMC 840, display UDP/RTP port.

- When external foward to digit set lcr digit and qlcr digit, qsig fwd operated.

- In IP phone registration, saperate UID duplication error.

- Add recovery function in OS, when message is full to one not-processing task (maybe error), the other tasks can’t processing because of lack of message.

- In socket sending, checking data length to link shutdown. In abnormal tcp disconnection, curr status on MMC 829 set to OFF.

’03.04.15 V2.00

First release software