| Engineering Change Request (Key Telephone System) |  |  |  |
| --- | --- | --- | --- |
| ECR No | SECECR-1036 | Date issued | February 28, 2006 |
| Proposed | Originated by Approved by ___________ Kyungok Ko Honghan Kim |  |  |
| Model | OfficeServ12 | Unit | OS12 Boot S/W |
| Subject | # OS12 Boot S/W Version Upgrade to V1.02 |  |  |
| Summary of Change | - D060201025> Modification for OS system’s MAC prefix change . - D060209051> Modification of Boot program because the change of 32M NAND makes it long times to load PGM and to write NAND . ※ The detailed contents refer to the following page 2 |  |  |
| Reason of Change | To fix problems. |  |  |
| Details of Change | - Prepare for the change of MAC prefix instead of current MAC prefix(0000F0) - Fix to use other MAC prefix in addition to current prefix(0000F0) because of the lack of MAC address. - Use 32M NAND Flash memory - Up to now use 16M NAND Flash but this chip will be not produced in the future. And will be use 32M NAND Flash instead of this. |  |  |
| Impact of Change | PTT : Not affected Safety/EMC : Not affected. Classification : On a running change basis |  |  |
| DOC Affected | Circuit Diagrams : Not affected |  |  |
| Proposed Implementation Date |  | March, 2006 |  |
| TO | All |  |  |
| Approval of ALE | Please sign your signature here and return to us by FAX. Signature ALE Name ................................................ Approval Authority .......................................... Approval Number ............................................ |  |  |

|  | Btos12.bin | Btos12.pgm |
| --- | --- | --- |
| CHECKSUM | 6A3B | −(PGM type) |
| VERSION | V1.02 |  |
| DATE | 060228 |  |

Change of MAC prefix

- Reason of change

Up to now OfficeServ system use ‘0000F0’ as MAC prefix. But this prefix can’t be used in the future because of the lack of MAC address. So will enable to use other prefix in addition to current prefix(0000F0). But if MAC address is all FF or all 00, system regards this as

invalid value.

- Change of MP related to MAC prefix(above version 2.63)

- If MAC address error happens

- If MAC prefix isn’t ‘0000F0’ in current MP, system set MAC address as default

value. But in this case it happens problem that several system use same MAC

address. So from version 2.63, system regards this case as MAC address error and

keep flickering run led of MCP board at four-minute intervals. And in this case system

doesn’t progress next step anymore.

(2) Delete MMC908

- According to the license policy of OfficeServ system, it isn’t allowed to change MAC

address at discretion. From above version 2.63, system can’t use MMC908 that can

change MAC address.

Change of Boot/Startup program for using 32M NAND

- Reason of change

Up to now OfficeServ12 system use 16M NAND for saving MP file, Startup file, DB, etc. But 16M NAND will be not produced anymore. And will be use 32M NAND instead of this.

But using current Boot/Startup program in 32M NAND makes it long times to load file from NAND and to write into NAND. So we’ll change Boot/Startup program to optimize the time.