| Engineering Change Request (Key Telephone System) |  |  |  |
| --- | --- | --- | --- |
| ECR No | SECECR-1083 | Date issued | May 26, 2006 |
| Proposed | Originated by Approved by ___________ Kyungok Ko Honghan Kim |  |  |
| Model | OfficeServ500 | Unit | OS500 Startup S/W |
| Subject | # OS500 Startup S/W Version Upgrade to V1.07 |  |  |
| Summary of Change | - D060530019> System stop problem while decompressing MP ZPG file. 2. D060530021> Using MultiMedia Card(MMC) because of the lack of Smart Media Card. ※ The detailed contents refer to the following page 2 |  |  |
| Reason of Change | To fix problems. |  |  |
| Details of Change | 1. System stop problem while decompressing MP ZPG file. - System can stop while decompressing some MP ZPG file compressed PGM above 8.1M because of the lack of space for decompressing. 2. Using MultiMediaCard(MMC) - Up to now use SmartMedia but this will be not produced in the future. And will be use MultiMedia Card instead of this. |  |  |
| Impact of Change | PTT : Not affected Safety/EMC : Not affected. Classification : On a running change basis |  |  |
| DOC Affected | Circuit Diagrams : Not affected |  |  |
| Proposed Implementation Date |  | June, 2006 |  |
| TO | All |  |  |
| Approval of ALE | Please sign your signature here and return to us by FAX. Signature ALE Name ................................................ Approval Authority .......................................... Approval Number ............................................ |  |  |

|  | Startup.pre |
| --- | --- |
| CHECKSUM | −(PGM type) |
| VERSION | V1.07 |
| DATE | 060526 |

Using MultiMedia Card instead of Smart Media

- Reason of using MultiMedia Card(MMC)

Up to now OfficeServ500/7200/7400 system use Smart Media for saving MP file, Startup file, DB, etc. But Smart Media will be not produced anymore. So we will use MultiMedia Card(MMC) instead of this.

For using MultiMedia Card, must change boot program of v1.06, startup program of

v1.07 and MP of v2.66. New boot/startup/MP program can use both Smart Media card

and MultiMedia Card.

- Difference between Smart Media card and MultiMedia Card(MMC)

- Reduced Booting time

Booting time takes half times as compared with using Smart Media card in OS500

/7200.(In OS7400, boot time takes 80% times as compared with using Smart Media card because of difference of RTOS)

(2) Increase of Media capacity

We will use MultiMedia Card of 64MB and above. Up to now we have used Smart

Media card of 16MB or 32MB.

Compress MP file for using 16M SmartMedia

- Reason of using compressed MP file

Up to now OfficeServ500/7200 system use 32M SmartMedia for saving MP file, Startup file, DB, etc. But 32M SmartMedia will be not produced anymore. And will be use 16M SmartMedia instead of this.

But in such cases it has one problem. If we need to change MP file, we have to save two MP files into 16M SmartMedia. But we can’t save two MP files into 16M SmartMedia because MP file size increase more than 8M from version 2.63

So we’ll use compressed MP file to solve this problem.

- Difference between current MP file and new compressed MP file

- Change startup program

For using compressed MP file, must change startup program of version 1.06

New startup program(above version 1.06) can load both new compressed MP file

(“zpg” file) and current MP file(“pgm” file).

- Use different file extension

New compressed MP file use “zpg” as file extension instead of  “pgm”.

- Step for using new compressed MP file

[Remark] After change of startup program, must upload new compressed MP file.

While system is running, connect to system through PCMMC

(1) Change Startup program(startup.pre) of version 1.06

Erase current startup program and upload new startup program of version 1.06.

(2) Upload new compressed MP file of version 2.63

Check if the file extension is “zpg”

(3) Check if startup.ini file exist

If startup.ini and MP files above two exist in SmartMedia, system loads MP file

designated in startup.ini. In the various case, which MP file is loaded refer to

the following page 3.

- Restart system and check if MP is running normally

OfficeServ System’s MP file loading process

[Remark] In the following “file name” exclude file extension.

1. If startup.ini doesn’t exist in SmartMedia

- According to MP file name, system loads the latest version.

But if the latest version files are two, the file name is same and extension is different,

system loads “pgm” file.

2. If startup.ini exists in SmartMedia, find the file designated in startup.ini.

(1) If the designated file exist

- System loads the designated file.

(2) If the designated file doesn’t exist

- According to MP file name, system loads the latest version.

But if the latest version files are two, the file name is same and extension is different,

system loads “pgm” file.