| Engineering Change Request (Enterprise Communication System) |  |  |  |
| --- | --- | --- | --- |
| ECR No | SECECR-1116 | Date issued | 08 Sep, 2006 |
| Proposed | Originated by Approved by J.Y. Yoon S.S. Rue |  |  |
| Model | OS 12,100,500,7200 And 7400 | Unit | # WIP-5000M S/W |
| Subject | ## WIP-5000M S/W V03.07.04 release |  |  |
| Summary of Change | - The call answering problem is fixed. - Arp request time is longer than 50ms. - The RTP packets started to send after the call is received. - Wrong call name display bug is fixed. - Deleting call log by menu selection works in normal. |  |  |
| Reason of Change | WIP-5000M S/W bug fix. |  |  |
| Details of Change | - This bug was fixed with restarting the wlan chip - Arp request time is longer enough over than 50ms. - No RTP packet would be sending before the destination answer the call. - There was uninitialized global variable on special condition - Flash write operation is added |  |  |
| Impact of Change | PTT : Not affected Safety/EMC : Not affected. Classification : On a running change basis |  |  |
| DOC Affected | Circuit Diagrams : Not affected |  |  |
| Proposed Implementation Date |  | Sep, 2006 |  |
| TO | STA, SEUK, SEI, LSP |  |  |
| Approval of ALE | Please sign your signature here and return to us by FAX. Signature ................................................ ALE Name ................................................ Approval Authority .............................................… Approval Number ............................................... |  |  |

| MODEL | SYSTEM | #### WBS24 | #### WIP5000M |
| --- | --- | --- | --- |
| VERSION | 2.66 above | V1.41 | OfSr_v03.07.04 |
| DATE | 06.07.dd | 05.10.04 | 06.09.08 |

Remarks:  If the system version is under v2.60, some functions would be restricted.