| To : STA,SEI,LST,etc | Doc. No. | SEC0604-04-3 |
| --- | --- | --- |
| C.C. : | Country | SEUK, STA, SEI, LSP, etc. |
| ## From: | Tel: 82-31-279-4337 Fax: E-mail: |  |
| Subject : WIP5000M S/W Release |  |  |

Overview :

WIP5000M S/W release

WIP5000M S/W:	OfSr_v03.06.00(’06.03.20)

OfSr_v03.06.00_0320_type1.r0 (for SEUK, STA, SEI, LSP, Korea, etc)

OfSr_v03.06.00_0320_type2.r0 (for SEUK)

OfSr_v03.06.00_0320_type3.r0 (for SEUK)

## New Version Changes

’06.03.20 OfSr_v03.06.00

- WIP5000M S/W changed items

    - In a language selection process, English is selected as a default language if a system manager select a wrong country or language that can not be supported by the WIP-5000M

    - Pick up delay is shorten about 1.5 second

    - While the WIP-5000M is on displaying the call duration time, a user can change to the on-hook state by pressing the ‘End’ key without waiting for a while.

    - A lock up condition was fixed.(If a terminal received probe responses from the AP’s –over than 20 APs-, the terminal can not save the all data and locked up.)

    - Hand over condition was improved on some complex barriers condition.

    - Outgoing call will be waiting for a while, if CQ value is under 32.

    - The cancel key operates as one step out on the user registration process.

    - The DND condition on menu item shows a current value instead of default value

    - Some Swedish strings are updated(type2: for SEUK only)

-"Sida" (Page, in English), is changed to "Allanrop"

- "Ingångsmedd."( Input Message, in English), is changed to "Skriv meddelande"

- "Shema" (Schedule, in English), is changed to "Kalender"

S/W Compatibility Table :

| MODEL | MCP2-L/M | WBS | WIP |
| --- | --- | --- | --- |
| VERSION | V2.60 above | V1.41 | OfSr_V03.06.00 |
| DATE | 05.10.17 | 05.10.04 | 06.03.20 |

Previous Version Change History:

’05.10.17 OfSr_v03.05.00

- WIP5000M S/W changed items

    - An error that the phone doesn’t register the system occurs when several phones are registered to the system. This bug is fixed.

    - If users input the group conference name more than 8 characters in Korean,  "Group Name Change Fail " is displayed to the screen ,and then the group name is saved. This bug is fixed.

    - After Leaving the station message on the phone, and the phone turns off and turns on, the message icon is disappeared. This bug is fixed

    - Modify the way the phone saves the call log using the option of the CID display.

    - During the conference, When the phone is ‘mute’and another phone is hung up,

The ‘Mute’ display on screen is disappeared. This bug is fixed

    - Modify the error on screen during the VOIP call.

    - In Phone lock state, user can’t stop the alarm signal. This bug is fixed

    - Add the ‘Data Clear’ fuction in hidden menu.

    - The option of the execution via uni-subnet is cancelled.

    - The Error in call log occurs when the call is disconnected out of the range.

This bug is fixed

’05.10.03 OfSr_v03.04.00

- WIP5000M S/W changed items

    - When ‘Search Number’ function is used., can’t input digit.

- This bug is fixed.

’05.08.05 OfSr_v03.03.00

- WIP5000M S/W changed items

    - User can select German Language. ( Add German string )

    - When ‘Page’ function is started, mobile’s audio path is not opened.

      - This bug is fixed.

’05.07.26 OfSr_v03.02.00

- WIP5000M S/W changed items

    - Add AP Scanning function.

    - Unlock the phone in the auto-lock mode during the conversation.

    - If one combo AP restarts, WIP5000M joins another combo AP. But, WIP5000M sends data with old AP’s MAC address.

’05.06.27 OfSr_v03.01.00

- WIP5000M S/W changed items

    - During phone call, if user selects the Soft menu and presses the button of the number that isn’t displayed on a screen, one-way speech happens. Pressing the button of the number that isn’t displayed on a screen is to turn the tone off. This modifies not to turn off the tone during phone call.

    - Remove unused Flag ‘CALL_STS_DIAL_FLAG’

    - To use the station lock on the phone, the FWD field of DB1, DB2 messages has to be set, But the phone sets the field of DB2 message. This is modified to set the field of both DB1 and DB2.

    - The buffer related to the display on a screen is cleared , but it isn’t displayed. This leads to clear the string on a screen partially. bug is fixed.

    - If the phone fails the periodic SIP REGISTER, the user can’t be able to make a call till the next SIP REGISTER. Therefore the interval time of SIP REGISTER is reduced when SIP REGISTER is failed. ( 50 sec -> 10 sec )

    - While Missed call is displayed on screen, user can press the SEND or OK button to initialize the count of missed calls. This is modified to be able to initialize the count of missed calls when any button is pressed for user’s convenience

    - While Missed call is displayed on screen, if user presses the digit button, Phone shows the initial display. This is modified to be able to display the digit button the user presses on screen while the number of missed call is displayed on screen for user’s convenience

    - During conference call, while a user sets the phone to ‘mute’, if other users hang up the phone, ‘mute’ is cancelled. bug is fixed.

    - the phone regards the incoming call that users don’t answer as missed call. This causes the phone to regard the group incoming call that user can’t answer due to the answer from other users in the same group as the missed call. Bug fixed.

    - The phone can detect the disappearance of the signal of the access point more rapidly.  (it takes 15 seconds)

    - Both Call setup time and Hold retrieve time are reduced.

    - When ‘Page’ function is started, the END button is locked for the moment. bug is fixed.

    - When the phone turns on and the key volumn sets zero, if ‘Page’ function is started, the Handset/Headset microphone doesn’t work. bug is fixed.

    - There is slow operation problem with operating in PSM(Power Save Mode). This problem has fixed by using common PSM APIs.

    - Sometimes key-tone sound is weird when used with ear-set. It was a problem of tone collision. Bug fixed.

    - When registering new phone to the System, if another registered phone has the same IP address, it causes IP collision and does not stop. This bug is fixed by stopping the network module when registration failed.

    - Fix “missed calllog count” problem by initializing it when the phone initializes

    - When the phone title is Korean, if you change the phone language , title is shown with breaking characters. Fix this problem by correcting Korean processing module.

    - If phone restarts in the static IP mode, it can’t be registered again. Bug Fixed.

    - When the caller uses the soft-menu during the outgoing call, Soft menu displays with language different from the current language . Bug Fixed.

    - Fix leap year problem in World Clock function.

    - In Booting Menu (When turn on the phone, press ‘Power off Key’ + ‘5’), press ‘9’. It changes menu to parameter reset. In this menu, if press the button except ‘1’ and ‘2’, the phone is locked up. Bug fixed.

    - When pressing the password to unlock the phone in the auto-lock mode, the last digit of password is displayed. Bug fixed

    - Modified Boot Loader for Auto Upgrade function.

    - When auto S/W upgrading is stopped by unpredictable situation, the phone is only able to upgrade S/W in engineering mode. This is fixed by continue upgrading after above situation.

    - There is noise when upgrading phone S/W. this problem is corrected by initialize audio path.

    - When downloading v3.0 image, there is a problem that ‘SAMSUNG ‘ and ‘Downloader’ string is displayed repeatedly. It is caused by broken Application S/W image. Fixed by checking the information abut TFTP server and setting again TFTP server address.

    - When using internet Call, the phone does not display the call information.

      - When disconnected from Internet, the phone reconnect PSTN network without notification. This is corrected

      - In case of disconnected from both Internet and PSTN network, 3 or 4 second later with blank display, the phone is reconnected to PSTN network. This is corrected

    - If unregistered phone turns off, it sends SIP detach message to the system with illegal IP address. Modified that unregistered phone does not send SIP detach message.

    - Sometimes Battery and antenna icon disappears when receiving the call. Bug Fixed.

    - The phone is locked-UP when register to the system during the advanced registration mode (press Power-Key and 5 when turn-on ). Bug fixed.

’05.01.05 V2.15 Build 0

- WIP5000M S/W changed items

    - __YJY_1127_DBG__

      - Enlarged system message Rx buffer size.(bug fixed)

    - __INKOORI_1216_DBG__

      - Registration bug fixed(Static IP could not be registered)

    - __YJY_050105_DBG__

      - WIP-5000M did not work on IDLE state.(S/W Lock Up) bug fixed.

’04.11.23 V2.11 Build 5

- WIP5000M S/W changed items

    - __INKOORI_1122_DBG__

      - During the conference, there is a problem that terminal in mute mode is automatically out of the mute mode. Fixed.

    - __YJY_1122_DBG__

      - During the terminal initialization process, handoff is blocked.

      - Whenever a new call starts, 2 Line LCD buffer is cleared.

    - __INKOORI_1121_DBG__

      - Show current terminal’s password using the hidden menu at the key locked state.

’04.11.13 V2.11 Build 4

- WIP5000M S/W changed items

    - __YJY_1110_DBG__

      - Call instance will be terminated instead release a call If call instance exists before making a new call,

      - Sometimes PRISM MAC is disabled in out-of-service area. It’s fixed

      - WIP-5000M never goes to sleep mode in out-of-service area. Bug fixed.

’04.11.08 V2.11 Build 3

- WIP5000M S/W changed items

    - __KJW_1104 _DBG__

      - Using cold start instead of warm start

      - When registration fails, animation and letters overlap each other. This problem is fixed.

      - When get WBS ID message, the index(“WIP5000M_ID”) of terminal is not stored

      - If voice path is changed, terminal in MUTE mode is automatically set free from MUTE mode

      - When conference menu is activated, open HEADSET path by EAR MIC check

      - When LCD buffer is controlled only in UI thread

      - When terminal is in mute mode during conversation over phone, RSSI menu is not activated.

    - __CONF_BUSY_DBG__

      - Busy jump is available when a user trying conference call.

    - __INKOORI_1027_TEST__

      - Sometimes a terminal displays “Register fail” when WAP was reset. It’s fixed.

      - Display bug fixed while using C.O. outgoing call.

      - Packetization time is changed to 40ms from 20ms while using G.711 codec

    - __YJY _1028_DBG__

      - wTelnet Thread is disabled. But It’s possible to be enable the service on hidden menu + 7

    - __KJW_1026_DBG__

      - The called party number is disappear when a user try menu display while trying outgoing call. It’s fixed.

      - If key-lock option is set. The menu key is disabled while a terminal is connect state.

      - If SIP registration fails, out-of-service icon is displayed.

      - Unexpected auto-response message problem is fixed.

    - __INKOORI_1026_DBG__

      - Sometimes the signal strength icon shows out-of-service on IDLE state. Try some method to fix it.

      - SIP registration fail handler is added.

    - __YJY_1023_DBG__

      - A call setup after busy jump waiting for 300ms to send a location Indication message first.

    - __KJW_1021_DBG__

      - The back slash icon connect icon are always cleared together.

      - SIP 400 error event handler is added.

      - SIP register fail cause the out-of-service icon is displayed. And it is disappeared when a terminal receive a SIP register success.

      - The call time display error is cleared that never return to IDLE display.

    - __INKOORI_1022_DBG__

      - The audio tone is cleared before making a new call

      - Invalid web-key cause a terminal lock-up. It’s fixed.

      - WIP-5000M sometimes miss a probe response from WAP after the WAP power on reset. It’s fixed.

      - Before detecting a WAP is power off condition, a terminal try to scan 3 times.

    - __YJY_1019_DBG__

      - New SIP message header value is added to indicate a Hand-off call setup.

      - 486_BUSY message cause the call release in CONNECT state

      - Change the default value of echo cancellation function to be enabled.

    - __INKOORI_1011_DBG__

      - Scanning method is changed when the WIP-5000M is located in out of service area.

      - If the associated WAP was not found within 4 times scanning, Prism driver would be reset and rejoin the current WAP

    - __KJW_1011_DBG__

      - The call end time will be displayed when an user end the conference call or end a call in waiting state

      - 2 Line LCD message update the display buffer immediately.

    - __RSS_1006_DBG__

      - Neighbor AP information request/response message added

    - __YJY_1013_DBG__

      - Retransmit the association request frame for every second if a terminal has not received a association response from WAP.

      - rtcpErrorCnt is reset when the call instance is created.

    - __INKOORI_1007_DBG__

      - Add functions that check whether AP is disconnected or shut down.

    - __KJW_1001_DBG__

      - In case that terminal is in station lock, conference, hold and transfer keys are disable.

      - When register terminals, display ‘Searching’ instead of ‘Searching WAP’.

      - KeyTone is disable during On-line : avoid collisions between RBT and KeyTone.

      - When terminals in station lock are unlocked, reduce delay time : delay due to Flash R/W is reduced.

      - To avoid sending useless PA messages, activate initPAQueue() function in rtnToldle() function.

      - Simplify storing Call Log flow : simplify procedures related with Call index.

    - __INKOORI_1005_DBG__

      - In power save mode, terminals can handle register timeout properly.

Change timeout time(20 second) for sending SIP register from UI into 40 second

      - When check whether AP is disconnected or shut down, use beacon frame instead of signal level in probe response.

      - Protect terminals from memory overflow in Prism caused by missing FID during transmissions of packets.

      - When receivers do busy jump function due to channel busy, terminals restart. It’s fixed.

      - When senders call, continue doing busy jump function due to channel busy.

It’s fixed.

    - __YJY_1005_DBG__

      - MediaStream in SIP Instance ,which is yet to be destroyed, is destroyed when new SIP Instance create. Lock-up can be caused by accessing MediaStream which is mentioned above.

      - Fix problems related with display of call time

      - Remove links of clockset.htm and handset.htm from ipp.mk

’04.09.17 V2.11 Build 2

- WIP5000M S/W changed items

    - __YJY_0914_DBG__

      - Add busy jump function that makes terminals find new channel by roaming when  channel is full.

’04.09.16 V2.11 Build 1

- WIP5000M S/W changed items

    - __INKOORI_0913_DBG__

      - When terminals which do not join in any APs scan channels, they make MAC disable/enable once instead of continue changing MAC state(disable/enable).

      - Change all values of register timeout time into 90 seconds

    - __INKOORI_0915_DBG__

      - Protect terminals from lock-ups caused by memory overflow which is caused by getting continuous error response messages when SIP stack is in erroneous state.

    - __KJW_0912_DBG__

      - Erase all information in PA_QUEUE to avoid resending information in PA_QUEUE after terminals are registered

      - Change data Tx Retry Time(3.5 second) in PA thread in 5 second.

      - Set Retry timer 30 second and set the number of tries 5 during register procedure.

      - When register is done, terminals display ‘Forward Type’ menu. It’s fixed by adding flow which stores DB information from System into buffers of terminals.

      - When System changes language setting, this change applies to terminals.

’04.09.13 V2.11 Build 0

- WIP5000M S/W changed items

    - __YJY_0906_DBG__

      - Fix the problem related to pressing the END Key of the terminal during Handoff in Combo type AP environment.

      - Fix the problem related to copying the terminal number of the other side during Handoff Recall in Combo type AP environment.

      - Fix the problem related to destroying the call instance that remains in the part of preDialKey send.

    - __KJW_0912_DBG__

      - When register failure is occurred, erase all information in PA message queue.

      - Protect PA message queue from storing duplicate information during register procedure.

      - During register procedure, if PA message Flow fails, retry PA message Flow every 25 second and set the number of retries 4.

’04.09.10 V2.10 Build 11

- WIP5000M S/W changed items

    - __KJW_0910_DBG__

      - After two items are registered in Group Conference, terminal does not display inserted information and cursor when pull down cursor. This problem is fixed.

      - After push destination number, push quickly ‘Ok’ button twice, the call is connected. This problem is fixed.

    - __INKOORI_0909_DBG__

      - Terminal is not re-registered automatically by the system. This problem is fixed.

’04.09.09 V2.10 Build 10

- WIP5000M S/W changed items

    - __INKOORI_0909_DBG__

      - when terminal obtains authorization for upload traffic, the terminal is disconnected from the AP. Fixed .

’04.09.08 V2.10 Build 8

- WIP5000M S/W changed items

    - __INKOORI_0907_DBG__

      - when increase the traffic data of the upload side, the state of terminal can be out of range. It’s Fixed.

      - After push DIGIT key, push quickly ‘Ok’ button twice, the call setup is done too quickly than intended. Fixed.