| To : | Doc. No. | 20041108 |
| --- | --- | --- |
| C.C. : | Country |  |
| From : | Tel: 82-31-279-4315 Fax: 82-31-279-4289 E-mail: rlaalswl@samsung.com |  |
| Subject: iDCS 500 new S/W (M&L) |  |  |

Overview :

iDCS 500 NEW MCP S/W Release

500-L S/W: L1.40 (2004.11.18)

500-M S/W: M1.40 (2004.11.18)

500-SCP/LCP S/W: V1.16 (2004.11.16)

PCMMC : V3.03 (2004.11.08)

Contents :

- iDCS 500 MCP S/W for testing

- For details, refer to the next page.

S/W Compatibility Table :

|  | MCP-M | MCP-L | SCP/LCP | BSI | LAN | TEPRI | ITM3 | PCMMC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Version | M1.40 | L1.40 | V1.16 | #### V6.0 | V1.08 | V1.06 | V3.03 | V3.03 |
| Date | 041118 | 041118 | 041116 | 020401 | 020930 | 040623 | 030701 | 041108 |

[Changed Items]

- INCOMMING CALLS DO NOT FORWARD WHEN TRANSFERRED FROM SVMi.

: It is fixed.

- MMC 851 WHEN VIEWED VIA PCMMC DISPLAYS UNREADABLE DATA

: It is fixed. To fix this problem, the MCP S/W was changed.

- It appears that if the user setting the wake up (alarm reminder message MMC 116) answers the call on speaker phone then the audio is not heard. It works fine on the handset. When the user answers on speakerphone they are connected to the AA card but no sound is heard.

: It is fixed.

- [WAKE-UP AA Feature when the system does not have AA card]

We are modified the software as the following:

- We are going to be able to set this up using Group Busy option although there is no AA card in the system. Therefore, if three options in MMC 224 are assigned as if there is AA card in the system,

the designated MOH (instead of AA message) can be heard when user answers Alarm.

- AA Group: an AA group should be assigned but this AA group does not have any members because of no AA card.

- Message No: a message number (1-64) should be entered.

- Group Busy: the designated MOH should be assigned.

- When Wake UP AA feature is not programmed but Alarm MOH option (this is added in

MMC 210) is set to ON, the station MOH will be heard when user answers Alarm.

- [SVMi product volume control]

: In MMC 805, TSW GAIN CONTROL for SVMi is added. The below options are added.

SLT --> SVMi, DGP --> SVMi, DTRK --> SVMi, ATRK --> SVMi, VOIP --> SVMi

SVMi --> SLT, SVMi --> DGP, SVMi --> DTRK, SVMi --> ATRK, SVMi --> VOIP

- [VT problem] VT key does not work with an analog voicemail. When the VT key is used to transfer a call to an analog voicemail the system does not send the mailbox information only the call type digit is sent.

: It is fixed.

- MMC 210 REDIAL REVW option is added. If this option is set to ON, outgoing call log review will be appear when the LNR key is pressed.

- Canada French & US Spanish language display is newly added. (MMC 121). Please test it and then inform us what you find out.

- The ITM3 V3.0 S/W interface is added.

MMC 831 VOIP PARAMETER

- VERSION option is added. (V2/V3)

MMC 832 VOIP CODE

- Entry 32~59 will be used to INCOMING ROUTE TABLE for V3 S/W

- GK USE option is added.

MMC 834 VOIP OPTION

- PCM COMPANDING option is removed.

- EARLY H245 MODE is added.

- RING BACK TONE is added.

- Q931 NO ANS TIME is added.

- ISP TYPE is added.

- FACILITY TYPE is added.

- SNMP Server IP is removed.

MMC 835 VOIP DSP OPTION

- VOICE PROMPT SWAP is removed.

- MAX JITTER DELAY is added.

- MIN JITTER DELAY is added.

- DTMF TRANS ROUTE is added.

- FAX ERR CORRECT is added.

MMC 836 VOIP GK OPTION

- RAS MANUAL is added

- GK ROUTING is added..

- KEEP ALIVE is removed.

- SmartMedia Upgrade procedure is changed to prevent the upgrade of wrong program. To do this, LAN S/W, MCP S/W and PCMMC S/W are all changed.

: If you want to upgarde S/W by PCMMC, you can now see the file information(file name, size and date) of SmartMedia and you can delete the file.

- iDCS News (new name of Smart Control News) and iDCS Call (new name of Smart Control Plus) interface is added.

: MMC 722 & 723 INFDSP, CHOICE key is added.

- EasySet interface is added.

- MMC 829 LAN PRINTER is added. (This feature is under testing. And the LAN S/W must be upgraded to test this feature. The LAN S/W is under developing now)

- MW key is added for iDCS 500-M S/W

- MMC 819 SM FILE CONTROL is added. In this MMC, you can see the file name and size of SmartMedia. And you can delete file by pressing HOLD button.

- [HOTEL]  Administrator station cannot change station status to HOLD & NEED CLEAN or HOLD & NEED MAINT via HOTEL key.

: Fixed

- [HOTEL]  System will not allow 911 calls from station whose deposit is exhausted.

: Fixed

- [HOTEL][ALL]  All cost display releated in HOTEL version is united when USE EURO

option is ON.

19.  [HOTEL][ALL]  SMDR & HOTEL cost is not operated by CREDIT LIMIT when the call is

released for excess of call credit.

: Fixed

- [HOTEL][ALL]  CREDIT LIMIT does not work when the call is connected via ISDN MPD

line.

: Fixed

21.  [KOREA_ETC]  Trunk Night Group Service option is added in some countries

- This service is able to use by pressing NIGHT key programmed in MMC 722 & MMC 723

- [ALL]  When a SLI port programmed as VMAA in MMC 217 dials LCR code to access LCR, the system resets.

: Fixed.

- [ALL]  Some port is not overflowed to the next port when many calls are received at the

same time.

: Fixed

24.  [ALL]  The QLCR digit size is changed from maximum 9 digits to maximum 8 digits.

25.  [KOREA]  MMC 725 MIN.OF DIAL VIEW option is added.

(The user can mask dial number except number of dial view counts by using this option )

26.  [USA]  MMC 320 BRANCH GROUP is added in the U.S.A by request of the STA.

( This programming is added to resolve iDCS500 system’s slow processing. When the

“Heavy Call Traffic” occurs the iDCS500 system’s processing power appears to diminish

to the point where the system may “freeze” or lockup where the only way to recover is to

reset the system)

- [ALL]  When creating a conference call via trunk, if the SVMi-8 is called, the caller can record on SVMi-8. This progress is changed that this call is serviced as a direct call mode of SVMi-8, and recorded message is heard to the outside party

28.  [ALL]  Wrong Call Forward ID sent to VMAA.

In a network of 3 systems (A, B & C) all station in B & C are CFNA (Call Forward No

Answer across Network) across the network to the SVMi 8 VMAA in system A.

If a station in system B calls a station in system C when the call follows CFNA to the VMAA

in A the VMAA prompts the caller in B for its password.

- This problem was ocurred by sending a worong call forward ID to VMAA. So It is fixed.

- [ALL]  In a networked system in iDCS500 system, when the called party does not answer for a given time (about 40 sec) this call is released. So this problem is corrected by changing that the call is connected until before the calling party release

- [ALL]  In a networked system in iDCS500 system, when the called party is BUSY or DND mode, the caller does not hear busy tone and then this call is dropped

: Fixed

- [ALL]  When a TEPRI card is inserted to the first slot in the system, virtual port numbers of   TEPRI card are not assigned.

: Fixed

34.  [ALL]  ITM3 card restart problem

- A ITM 3 card diagnostic (Link test) process is changed that not diagnostic on default. And a ITM 3 card can operate link test when MMC 902 TRK SZR TEST TIME is set to 255

: Fixed

- [AUSTRAILIA]  The trunk ports can be assigned to MOH on 8 TRK card.

- [ALL] Calls waiting information is missig from all types of UCD states while a queuing message is played to a caller.

: Fixed.

- [ALL] When an analog VMAA access a PRI to make an outgoing call the dial tone that is

sent to the VMAA is DTMF and not CO dial tone. This also happens when LCR is accessed

and if LCR accesses a PRI trunk. This causes problems because when the VMAA sees the

DTMF dial tone it assumes this is intercom dial tone and that the CO call has been

disconnected and hangs up.

: Fixed. Second DTMF(BA) will not be sent to VMAA and real dial tone will not be

connected also.

- [SMDR] Several SMDR problems are cleared.

- Fixed the discord SMDR header and SMDR data for some countries.

- Print out ‘0’ as call duration time when call is not connected (no answer) on PRS or MPD trunk.

- Modified not to reduct SMDR start time from call duration time on MPD/PRS trunk.

- [Alarm report] Fixed alarm report problem. Following alarm reports were not worked perfectly.

- MJA01 : POR Restart

- MJA02 : Soft Restart

- [DECT] As a consequence function Call Back doesn’t work.

: Fixed.

- [Network] We havs found a problem with networked sites when all the network links are busy. What happens is that the next call will get constant ring back tone even though the call is not ringing. We have duplicated this in our office with a PSTN PRI calling in to a remote site when all the links are busy. The setup we have is as follows.

Main System

701~704 NI2 PRI

725~729 Q-SIG PRI

DID number 5994142 rings at remote site via DID pass through.

725~729 are busy

A call for 5994142 is incoming on 701

As 725~729 are busy 701 should hear busy signal, however the following sequence

happens:

701 rings at operator group for one or two rings and then disconnects. At this time 701 is

hearing silence. When operator stops ringing 701 starts to hear ringback.

After 2 minutes of ringback 701 goes idle and caller is dropped.

I have attached a trace from an ISDN tester for this call sequence if this will help.

This problem can also be duplicated as reported by a dealer today if there are 3 systems

networked together as follows.

system A

701~704 Q-SIG PRI to 701~704 of system B

725~729 Q-SIG PRI to 701~704 of system C

System A has stations 2001 to 2032

System B has stations 2101 to 2116

System C has stations 2201 to 2216

In system A trunks 701~704 are all busy.

station 2202 dials 2111

2202 hears silence and then ringback for 2 minutes then goes idle.

: Modified this problem. In this case calling party will hear busy tone and can see the RETRY

key on LCD. So caller can hook on or try Auto redial by using RETRY key.

- [Networking] When progress in message is received from CO instead of alert in message on

Q-sig tandem, progress in message was not transferred to another system.

: Added new option at MMC 210. If PROG.2 ALERT is set to ON, system will change

progress in message to alert in and transfer the alert in message. But on OFF, progress in

message will be bypassed.

- Program Key Operation Bug (LSP 001)

The Program key operation does not provide the ability to program MMC 705 and 706.

Currently the only MMCs that can be programmed are 100, 102, 104, 115, 116 and 505.

MMC 705 and 706 must be added to the PROG key operation. Previously MMC 705 and 706

were available in MCP version 1.10 software.

: Added MMC 705 and 706.

- DS Key for DECT Extension (LSP015)

DS (Extension)key with DECT handset extender (eg. 8801) does not provide LED indication

on the status of DECT extension ie. idle LED OFF, busy LED ON or ringing LED flashing.

LED is OFF at all times.

: Fixed.

- MOH via Trunk port

Does not operate in MCPL121.pgm. Option is available in MMC 400 and can be set to ON,

the trunk port can then be programmed in MMC308 & 309 as the MOH source however no

MOH is heard.

: Fixed.

46. Hotel Call Billing – ISDN Advice of Charge

ISDN calls with AOC are recorded incorrectly on Hotel Bill. Using the AOC feature the correct

charge is shown on keyset (designated as a Guest Room phone) as $1.08 however the call

is recorded on the Guest Bill as $108.00. PSTN Meter Pulse operates correctly for both hotel

and non hotel billing purposes.

: Fixed.

- Room with expired credit cannot make emergency call.

A room that is Checked Out can make an emergency call. If the station is checked in, the

system does not check if the call is Emergency Call according to MMC709. A room that has

credit limit of $0.00 will be permitted to make the call, but will be cut off on answer. A room

that has a credit will be permitted to make an emergency call, but will be cut off when the

credit limit is exceeded. It does not apply the Toll Override code in MMC709.

: Fixed.

48. What happens is that each station (all keysets are 12 button LCD sets or 24 button LCD

sets) has a SG5105 key to access the analog VMAA. when this key is pressed in

speakerphone mode everything works fine. However if the user lifts the handset and presses

the key aproximately 50% of the time the call will drop when the VMAA answers. This

happens at both sites although the example above is for the main site. The VMAA group

there has 12 members and the group in the remote site is 5205 and has 8 members.

: Fixed.

- Problem with Alarm Message

If the user setting the wake up Alarm Reminder message (MMC116) answers the call

offhook or using [Speaker] key, the message is audible. If the user uses the [Ans/Rls] key, it

is not audible.

: Fixed.

- Message set from Ext paired with DECT does not clear MSG if answered by DECT

: Fixed.

- DID Number/Name in MMC 725 not available.

: Added this option at MMC 725.

- Paired DECT phones does ring with AME switched on.

: Fixed.

- Unconditional UCD grp, all out doesn't go to finale destination.

: Fixed.

- DECT doesn't follow call forward using Q-Sig

: Fixed.

- Not Cont CID in Holland.

: Fixed.

- [Networking] CC SIG CONN : YES option will not work on this version because we found some problem. CC SIG CONN will always work as ‘NO’.

So on callback ringing, ringing system will not check the state of opposite party.

- Voice mail - Some DECT users have notification set on their mailboxes.

The DECT phones are being turned off by some users when not on site this results in calls

going straight to their mailbox which then takes a message and notifies the handset, which

is turned off and goes again into the mailbox taking another message and again causing the

handset to be notified. This will continue as the handset keeps getting notified resulting in

the mailbox becoming full of voicemail notification messages. This can be avoided by not

turning off the DECT. The problem can still arise if they are on the other site with their

DECT. Can the SVMi-8 voicemail recognize notification calls to a voicemail box and

therefore not record a new message everytime it receives a notification message? The

Voice Mail takes 1-2 Hours to initialise as a result of the number of messages in the DECT

mailboxes. The message limit setting for mailboxes does not actually limit the total it only

indicates to the handset user that the pre set number has been exceeded. We have set a

nightly delete to fix the problem in the short term.

: fixed.

- [STUK_BUG041] The option of PERI UCD SIO:ON does not work in this version of software.

: Fixed.

- On connecting PCMMC, if link was disconnected abnormally you could not access PCMMC

or KMMC again.

: fixed.

- [STUK_BUG043] Call transferred by the AA card to a Virtual extension do not follow call forwards over q-sig.

: fixed.

- [STUK_BUG043] The only real point that we would like to raise is the issue with the new feature of AA calls to an extension that is set to forward across the Q-Sig network. If the call is an internal call there is a problem.

: Fixed. When ICM EXT FWD is ON at MMC 210, this feature will work well.

[ L1.24 & M1.24 ]

- [GULF_All] After picking the call up, we could not flash and sending DTMFs to CO.

: Fixed.

- [GULF_All] When a message is left in the mailbox for a DECT handset the handset does

not receive message wait notification. Fixed

- [GULF_All] When pressed the PARK key, HELD event was sent to CTI one more time.

: Fixed.

- [GULF_All] S0 extension ports can be assigned as members of a sequential ringing station

group.

- "S0 Group" type is added in MMC601 STN GROUPS TYPE

< MMC 601: STN GROUPS>

TYPE : S0 GROUP

MEMBER : only available S0 mapping number (7801, 7802...) which is

mapped to a specific S0 bus bearer (701, 702..) in MMC 424.

- [Russia_All] Cleared the one way speech problem when single line phone that assigned

dial pulse type used the DTMF trunk.

- [Russia/CIS] To add the possibility to choose the SUBSCRIBER number type in

outgoing CLIP information. Now iDCS-500 can send the CLIP to provider only in

NATIONAL or UNKNOWN type.

: Added INTERNATIONAL, NETWORK, SUBSCRIBER, ABBREV and

EXTENSION.

- [LSP_All] Cleared system restart problem when assign the LAN printer at PCMMC.

- [LSP_All] When virtual port was assigned it’s mailbox was not created. Clear this problem.

- [LSP_All] Request to change software so ACCxxx keys can be used during an

external call.

: Applied. (possible only ACC001~ACC999 & outgoing call)

- [LSP_All] Transfers to busy stations go to incorrect mailboxes. Fixed.

- [China_All] Miss numbers transferred for R2MFC tandem incoming call. Fixed.

- [Korea_All] Add delay time for connecting T-switch for external forward or LCR

call.(MMC501)

- [Korea_All] For R2MFC DDI call, DNIS was not sometimes sent to CTI. Fixed.

- [Korea_All] Cleared VOIP group lock-up problem. When dialed invalid number or occurred

link fail after seizure the VOIP group, the group was locked up.

- [Korea_All] For DDI incoming call, if caller dialled more 12 digits, the call was not made.

: Fixed.

- [Korea_All] Added GBUSY NEXT option at MMC 607. (default : OFF)

When this option is set and all UCD members are busy, the call will ring at next

group at once.

- [STUK_All] Cleared one way speech on conversation(Conference).

When one call was made and another call was incoming second call at virtual

port, other real port answered the call. In this case one way speech occurred at

first call.

- [STUK_All] Sometimes call stuck was occurred at CTI server. Because device id was not

sent to TSP for some events. Fixed.

- [STUK_All] Hon event for paired port was not sent to CTI. Fixed.

- [STUK_All] Preset Forward Busy

Allowed Preset Forward no answer for Busy status. PRESET BUSY is added at

MMC 210 for this feature. The destination is station or station group pre-assigned at

MMC 320.

- [STUK_All] Forward Ovrrride

Added FWD OVERRIDE at MMC 300. If this option of caller is set YES, the forward

of called party is cancelled temporary.

- [STUK_All] Recall to Operator

Added RCL TO OPER at MMC 300. If this option of transfering station is set YES, the

call will recall to operator instead of transfering station after transfer recall time.

- [STUK_All] Transfer to CO cause value 17 from Q-sig

When PRI call of system A from CO is incoming to system B via Q-sig channel and

called party is busy, cause value 17 will send to CO.

- [STUK_All] During testing with a PRI circuit connected, a phantom call came into a

UCD group and the display showed [NO CLIP INFO] 2020202020202020. This was

an old bug on later DCS levels of software.

: Fixed.

- [STUK_All] Back busy on DDI (PCR004)

Added MC option at MMC 714. This means max. call count for each DDI number.

DDI calls can be waiting up to MC for each DDI number.

(must have CALL buttons more MC number)

- [STUK_All] Forced Auto answer for station group (PCR049)

Should have an option for call to be forced to a headset and answered automatically,

with a set of rules. Options for Internal, External and Group Calls. These options are

available with Smart Control. These need to be options in the system software.

When selling into a Call Centre Environment, our competitors offer this feature and

could result in loss of sales.

: Allowed forced auto answer for station group that assigned ring type as sequential

or distribute.

- [STUK_All] MMC 515 (Daylight assign) which is not available in KMMC, can be opened in

PCMMC. Applied for M version.

- [STUK_All] Call stuck was occurred when paired station was called.

: Fixed this problem.

- [STUK_All] Extension 201 cannot search the personal speed dial directory by name.

: Fixed.

- [STUK040_All] Unsupervised Conferences are combined into a larger conference.

: Fixed.

- [STUK043_All] Call transferred by the AA card to a Virtual extension do not follow call

forwards over q-sig.

: Fixed.

- [STUK046_All] Calls that reach a UCD group when overflowed from another are not

correctly added to the "Calls Answered" statistics.

: Fixed.

- [STUK047_All] Calls answered from a paired handset are not added to UCD stats.

: Fixed.

- [STUK048_All] When a paired DECT handset is i use, UCD calls are still presented to the

keyphone.

: Fixed.

- [STUK052_All] Cannot make trunk to trunk transfers over analogue lines.

: Modified the software for this. In this case CO-CO Disconnect timer will be attached.

- [STUK053_UK] Station messaging information on the LCD was incorrect.

####  MSG19:HHMM, MSG20:DDMM

- [STA_All] Improvement ITM3 features.

- VOIP VIR RBT at MMC 210 : connect virtual ringback tone when set ‘OFF’

- RELAY RBT at MMC 834 : relay the real ringback tone from CO when set ‘ON’

- Warning Dest at MMC 834 : transfer the call to warning destination when

outgoing call is failed

- SNMP SERVER IP at MMC 834 : cleared busy return after dialing invalid number

through ITM3 trunk group

- [STA_All] When all UCD members were logout, ALLOUT  FINAL was ON and final

destination was NONE, the call was transferred to operator. So modified to

release the call.

- [STA_All] Cleared some problems for centralized VMS on networking.

- [STA_All] When a station in one network node calls a busy station in another network

node the call automatically camps on rather than giving a busy signal.

When called party is busy and caller is CO;

PATH RETEN is YES on MMC 823 : connect Busy tone

PATH RETEN is NO on MMC 823 : auto camp-on

When caller is extension of opposite system ;

PATH RETEN is YES : connect Busy tone

PATH RETEN is NO : follow the CW option of DDI table on caller’s system

CW is NO : connect Busy tone

CW is YES : auto camp-on

- [STA_USA] Barge in between nodes has been temporarily disabled due to a system

reset problem.

: Fixed.

- [STA_All] The first digit delay timer currently operates in 300 mS increments even

though it is programmed in 100 mS increments. The timer should follow the

programmed setting. Changed minimum data to 300msec.

- [STA/STUK_All] There needs to be a new setting in MMC 102 to allow the activation

of CFB and CFNR at the same time. STA suggests using 60# as the activation

code for this feature.

: Add CFB/CFNR option at MMC 102.

- [STA_All] There needs to be a new setting added to MMC 110 to allow station to

station calls across the network to follow the auto answer setting in MMC 103.

: Added AUTO ANS NET at MMC 110.

- [STA_USA] We have found in several cases that when an iDCS500 TEPRI is

connected to a NI-2 PRI from a Siemens EWSD central office that the default

setting for timer

T303 is too short. We would like to have the default setting for timer T303 for NI-

2 PRI trunks increased from 4 seconds to 5 seconds.

: Changed default data.

- [STA_All] Calls transferred by SVM do not follow preset forward no answer. When the

SVM transfers a call to a station that has the preset forward no answer set to

549 and BOTH the call will not forward.

: Fixed.

- [STA_All] When a call is screen transferred to a remote system and the remote party

rejects the call the transferring party will be locked up to the remote trunk if the

destination party hangs up before the transferring party returns to the outside trunk.

: Fixed.

- [STA_All] We have had a problem reported where the outside caller can hear the user

during AME if the user is in headset mode.

: Fixed.

- [SEI_All] Applied multiple booth service up to 3.

- [SEI_All] Allowed to assign DECT & S0 port for guest room.

- [SEI_Italy] Cleared cost problem that 1/1000 of cost is print out for HMREPT or PMS.

- [SEI_Italy] Added Wakeup status print option for Hotel.

- [SEI_Italy] Added LCR/ENBLOCK tone and DND & MSGWAIT tone were merged at

MMC 506. (default:200/200/600/1000 ON/OFF)

- [SEI_Italy] Changed display data for cause values.

18,27,127 : non accessibile

34,44   : canale occupato

- [SEI_Italy] Deleted TRANSFER and changed DIAL to DIAL/TRANSFER at MMC 514.

- [SEI_Italy] No Off-Hook ring tone is heard into the handset when a busy user receive

a second call, although the related option is set on (MMC 300). Actually, user

hears a brief interruption during the conversation, but not the tone. Problem

happens always, both external, DDI, group or intercom call.

: In this case, beep ring will be connected instead of Camp-on tone.

- [SEI_All] Station pair in a group

If the call is direct to the primary or secondary extension number, no problem

happens. It means, when one of the paired phone is busy, caller hears the busy tone always. If the primary of the paired numbers is a member of a station group (please note GROUP BUSY option: ON), the primary phone receive the call although the secondary phone is talking. If the secondary of the paired numbers is a member of a station group (please note GROUP BUSY option: ON), the secondary phone receive the call although the primary phone is talking.

: Fixed.

- [SEI_All] Password access violation

If you start the programming forgetting the accessing procedure (800 - 4321, etc.),

the first row of the LCD displays a wrong message (maybe it is not in English

language).

: Fixed.

- [SEI_All] Q-sig feature limitation

After called a party, a SLT cannot use network features as CCBS, CCNR, Call

Intrusion, because the flash key is not working on this type of phone.

: Fixed.

- [SEI_Italy] One of our dealers has installed an iDCS 500 system with 4 TEPRI cards

(total 120 trunks). Unfortunately, Trunk Group (MMC603) has 99 entries only.

Therefore some trunks cannot accessed by means a single trunk code (in our

case "0"). As a temporary method, I suggested him to enable the LCR and LCR

CLASS feature overflowing to a next trunk group in case of all member of first

trunk group are busy. Could you tell me if others possibilities are helpful for that

issue? Could you forward this issue to R&D and asking for a new option in

MMC603 in order to overflow to another group, similar the 'NEXT PORT' in

MMC601.

: Modified the software for this feature.

For this feature, use TRKGRP2MBR at MMC 210. When this option is set ON,

last two trunk groups will be operate as same trunk group. For example,

members of trunk group 847 and 848 are selected when seizure 847 group. If

you want this feature for ‘0’, changed number plan for trunk group.

- [SCP_All] Fixed 16DLI port initial problem.

- [PA_All] Account not verified was not worked.

: Fixed.

- [PA_All] MMC 210 ICM EXT FWD ‘OFF’ was not worked for CFU & CFB when the

call was transferred from AA.

: Fixed.

- [PA_All] For networking, displayed caller number was different between primary and

secondary station.(station pair) Caller’s number was display on primary

extension but trunk number was display on secondary extension.

: Fixed.

[L1.25/M1.25]

- [STUK075_All] Software levels are read incorrectly.

: Fixed.

- [STUK076_All] SSPD name is not displayed when a number is dialled using the

Directory feature.

: Fixed.

- [STA020_All] Auto answer does not work correctly with screen transfers.

For analog trunk, mic. was not opened of called extension that is set auto

answer mode.

: Fixed.

[L1.28/M1.28]

- [KOR_All] Added TRK TMC GAIN for new TMC. (-10dB ~ +20dB)

- [STA_USA] CID name was sometimes not saved at CID REVIEW block.

: Fixed.

- [China_All] CID was not display on LCD for E1 line.

: Fixed.

- [KOR_All] Added STN NO RING at MMC 110 for remove ring for incoming and

intercom call.

- [China_All] E1 line was blocked when incoming call from loop trunk was transferred to

E1 and called party was busy.

: Fixed.

- [China_All] Added SMDR AUT2ACC at MMC 210. When this option is set

Authorization code will be printed out Account code field up to 10 digits.

[L1.31/M1.31]

- [STA_All] Applied Canadian analog CID at SCP.

- [STA_All] SPD key did not work when caller received PROGRESS_IN message from

CO.

: Fixed but need to test.

- [STA_All] Station group auto-answer option

Added GRP AUTOANS option at MMC 601 in order to allow or not auto-answer

fewture for station group. This option works for Sequential and Distribute mode.

- [STUK_iDCS080] Using a 3rd party voice mail voicemail connected to SLI ccts. The

trunk information is not sent when an external call forwards to voicemail. This

causes the call to route to the AA and not the mailbox.

: Fixed. DDI number will be sent to voice mail as trunk information.

- [STUK_050] BRI link fail

When either a BRI link fails, the associated trunks are not taken out of service,

as is the case with Analogue trunks and PRI trunks. This causes the problem that

when trying to dial out you keep seizing the dead line.

: Fixed. When BRI channel is on link fail, system will check the link status every

2 minutes.

- [STUK_PCR083] Enhanced UCD

- Calls into a UCD group should be able to be given a priority. That is several

DDI numbers can be terminated at a UCD group, but the system will assign a priority to

the DDI number so that calls from a high priority DDI number will be placed at the front

of the Queue. To make sure that calls on low priority lines are not always at the back of

the queue, there must be a timer option that allows the priority of a call to be increased

if it is not answered in a specified time. 3 calls come into the UCD group, the agent 203

is busy, the first call is on DDI priority 3, the second on DDI priority 2, the third on DDI

priority 1. Agent 203 becomes available; the call with priority 1 is put through to Agent.

After this call is finished the call with priority 2 is put through to the Agent.

2 more calls arrive at the UCD group all on DDI priority 2. However the timer t has

expired for the DDI priority 3 call, this has changed the priority of the call to 1, so this

call is put through to the free agent. If the UCD group overflows (final destination) to

another UCD group, the overflow calls should have their priority raised to 1.

- The Agent Log In ID should be unique to a user, which is once the agent ID has been used to log an agent into a group that ID should not be able to be used again by any other agents trying to log into the group, until the original user logs out of the group.

- There should be an option to allow an Agent to log into ALL UCD groups from 1 button. If an agent is a member of several groups, using the ALL UCD button, they should be able to log in or out of all UCD groups that they are a member of.

: Applied. New MMC option was added at MMC 714. That is ‘P(priority’.

- [STUK_PCR070] To change the following defaults when the country is set to

Belgium in MMC812

MMC506

Ring Back Tone     1000 3000   1000 3000

DID Ringback        200   200    200  100

: Changed.

- [LSP,CIS_All] I am having a problem with the QSIG networking using 2 iDCS100.

From the second node (file Okuma 2#2 below), it cannot dial more than 17 digits on

ISDN outgoing calls. The ISDN trunks are in the other node. This means that

extensions from one of the nodes cannot dial international numbers, since

such numbers always require more than 17 digits.

: Modified software for this. We can send 32 digits across network but can send up to

22 digits to CO including trunk access code or LCR code.

For this function you have to use new TEPRI software.(later than V1.05)

- [LSP_AUS] Added ISDN NUM PLAN & TYPE at MMC 419 & 420.

You can select ISDN type and PLAN in several types and plans.(AUS only)

- [SEI_Italy] Changed display procedure for outgoing call. When NOT CONT.CID is set,

dialed digits will be not cleared on LCD after SMDR start time.

- [SEI_Italy] When busy station that set OFFHOOK RING receives ring signal, that

station will be heard beep tone instead of tone. (keyset only)

Caller is station : once beep tone

Caller is trunk : twice beep tone

- [SEI_All] Added RP at MMC 724 for all countries.

- [SEI_Italy] When DISA call is incoming caller heard 1st dial tone, MOH and 2nd dial

tone if DISA MOH is set at MMC 400. On new software caller will hear MOH

instead of 2nd dial tone.

- [SEI_All] System speed dial block is increased up to 95(950bins).

This is applied for iDCS 500 L only.

SYS SPEED : 500~999  050~999

STN SPEED : 00~49   000~049

- [STA/CIS_All] When caller made an outgoing call across network, trunk number of

main system was sent to CO as Caller ID. So add PASS R-CID option at MMC

400 to bypass original Caller ID to CO. When PASS R-CID of main system trunk

is set original caller ID will be sent.(assigned at MMC 323 SEND CLIP NO)

- [CIS_All] On E1-PRI tandem, though called party was not answer ANS_BACK was

sent to caller(E1). Fixed this problem. ANS_BACK will be sent to E1 when called

party answers.

- [CIS_All] When call is incoming with pre-assigned Caller ID, system will reject the call.

For this function MMC 765 CID REJECT was added.

- [NIMAVA_Belgium] When country is Belgium, LNR did not worked.

: Fixed.

- [CIS_All] On PRI-VoIP tandem, call was sometimes released.

: Fixed.

- [CTI PC console] Fixed message missing when VMS Call Record is OFF.

- [CTI ] Added Call clear feature to release the blocking extension.

- [CTI] When conference master hooked on at ring-back state, hook-on message for

called party was not sent to CTI server.

: Fixed.

- [CTI] Phone connect and disconnect message was not sent to CTI server for DECT

terminal.

: Fixed.

- Added ITM3 options.

MMC 834  H.245 TUNNELING (ENABLE/DISABLE)

MMC 835  TYPE OF SERVICE (0~255)

T38. REDUNDANCY (0~4)

[ L1.34 / M1.34 ]

- [New DGP_All] New types of keyset were implemented.

New Keysets are DS-5038D, DS-5014S, DS-5007S and DS-5000S.

MMC 723 new options : 38 BTN SETS, 14 BTN SETS, DS-07S SETS

MMC 807 ‘38D’ was added.

MMC 501 ‘LONG KEY DETECT’,’LONG KEY REPEAT’

Have to use SCP/LCP V1.12 (2004.1.29)

- [UKPCR083_else of USA] Cleared problem of DDI priority.

- Each DDI number can have priority number.(MMC 714)

- When all UCD members are busy, incoming calls will be waited.

- When idle member occurs, system check the priority of waiting calls.

Top priority call will be allocated on idle UCD member.

- If priority is same, system checks the incoming order.

- If one call is waiting and next calls are higher than first waiting call, priority of first call will be upgrade as one step every PRIORITY INCREASE TIME.(MMC 607)

161. [UK_All] When upload the DECT AUTH & PAIR by PCMMC, data was broken.

Fixed.

162. [CTI_All] PC Console was implemented for CTI.

163. [CTI_All] CTI orbit Park was added.

164. [PCMMC_All] When download database from system by PCMMC, sometimes Fail was

occurred. For clearing this problem, PC RESPONSE OK TIME was changed from 15

seconds to 60 seconds. This was applied from PCMMC V2.08.

165. [LSP032_All] When ISDN call was transferd to outside, the call was recalled directly.

Fixed.

166. [All] After the system power restart, the BGM port assigned in MMC309 STN MOH

SRC clear. It is fixed.

167. [All] When E&M call follows the DID Translation table and incoming number is invalid, the

call will be transferred to MMC 406 destination.

168. [All] When caller transferred the call to outside by VoIP and called party was busy, caller

could not return to privious state. Fixed.

169. [All] If DECT try to make a outgoing call using remote trunk via Q-sig as ENBLOCK mode,

caller couldnot make a call and heard error tone. Fixed.

170. [All] All type of trunk can be attached CO-CO DISCONNECT TM. On privious version, this

timer was attached for analog trunk only. For this new MMC was added at MMC 210

(CO-CO TM ALL).

171. [All] UCD Group Auto Answer feature is added.

172. [STUK_PCR] Emergency call

When caller dials the emgerency number but all trunks are busy. This time system

releases by force one of trunks on busy and make a outgoing call.

For this emgerency numbers have to assign MMC 709 TOLL OVERRIDE and

Trunk group has to assign MMC 709 OVRD USE TRK GRP.

System will check OVRD USE TRK GRP and it’s trunk members.

173. [All] AME for ISDN incoming call

When ISDN incoming call was transfer to VMS port by AME, original call destination

could not hear the greet ment. Fixed.

174. [STUK_BUG084] iDCS 500 M slow processing

If check the UCD status and UCD data was empty, slow processing was occurred.

This problem was duplicated at iDCS 500 M. Fixed.

175. [Russia] Changed R2MFC default data

MMC 501 CID RECEIVE TIME 1  6sec

MMC 418 CLG EXT RESP : OFF

Else           : ON

176. [All] Smart Media

Files in smart media card were sometimes out of control as file attribute.

Fixed.

177. [All] System does not return to Ring Plan based mode from Holiday mode. It is fixed.

178. [STA_SF052_USA]  System Speed Dial Block is increased up to 95 (950bins)

This is applied for iDCS 500 L only.

SYS SPEED : 500~999  050~999

STN SPEED : 00~49   000~049

179. [STA_SF058_USA] Canadian PRI CID Name display is added. (Using TEPRI V1.06)

180. [STA_SF059_All] Trunk, SLI, AA card fails diagnostics test. It is fixed.

181. [STA_SF060_All] E&M T1 hunting sequence problem is fixed.

182. [STA_SF061_All] When PASS R-CID option is set to ON in MMC400 and caller make an

outgoing call across network, the original caller ID is sent to CO.

But if CID number length is more 16 digits, this CID number that made up

[node ID + extension number + * + CID number (assigned MMC323)]

does not sent to CO correctly. So, if the CID number length is more

16 digits, the CID number assigned at MMC323 is only sent to CO except

to [node ID + extension number + * ]

183. [STA_SF062_All] Hotel Rate Calculation table input method is changed.

184. [STA_SF063_All] Hold Recall problem is fixed.

185. [STA_SF065_All] Unstable conference calls. It is fixed.

186. [STA_SF066_All] Network systems will not pass 19+ digits. It is fixed.

187. [STA_SF067_All] Alarm Reporting problem is fixed.

188. [STUK_iDCS098_All] Retry fails to dial on BRI/PRI if LCR is programmed and LCR

ALLOW is ON in MMC 400. It is fixed.

189. [STUK_iDCS095_Spain] Day & Date not displayed correctly when systems/phones set to

a lanuage other than English. It is fixed.

190. [STUK_iDCS096_Spain] When Keyphone set to Spanish there is corruption on the

display during ringing. It is fixed.

191. [All] PCMMC 100. DND mode and PCMMC 433. ERROR status added.

The user allow to set or reset DND status by using PCMMC 100 DND mode.

192. [All] Add Economic Keyset for MMC 103,111 and 114.

[ L1.36 / M1.36 ]

193. [iDCS/100]	If the system is put into a specific Ring Plan with the RP Key, it does not

remain in the plan but defaults back to RP1. It is fixed.

194. VT key transfer adding # to integration digits. For example, Incoming call rings in to station

201. Station 201 presses the transfer key, and dials 204 (monitored transfer). After several

rings, the call is forward to station 204 personal mailbox greeting. Station 201 presses the

transfer key to return to the outside caller. Station 201 announces to the caller that “ station

204 is away from the desk”.  The outside party then request to be sent to station 204’s

mailbox.  Now station 201 presses the VT key, then dials 204 and the ans/rls key to

complete the transfer.  The outside caller is transferred to the general mailbox (not to the

personal mailbox). It is fixed.

195. Analog DID card cannot receive DP digits. It is fixed.

196. [ITALY] Hotel SMDR problem is fixed.

* PROPOSAL FOR SMDR FUNCTION KEY

When you press the SMDR key the LCD should display as follows.

[1]

Extension Number

[2]

TOTAL PHONE BILL

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [ | E | X | T | ] |  |  |  |  |  |  | E | 0 | , | 0 | 0 |
| P | R | I | N | T |  | E | X | I | T |  | C | L | E | A | R |

[3]

PRINT & SAVE

PHONE BILL

[4]

CLEAR

PHONE BILL

[5]

EXIT THE PROCEDURE

[1]: EXTENSION NUMBER FIELD

No changing on this field. (Please leave it like it is now)

[2]: TOTAL GUEST BILL FIELD

This field has to display the phone call cost only.

[3]: PRINT & SAVE SOFT KEY (STAMPA)

This soft key has to print and save the total of the phone bill only.

[4]: CLEAR SOFT KEY (CANC)

This soft key has to clear the total of the phone bill only.

[5]: EXIT SOFT KEY (ESCI) Please add this soft key to exit the procedure.

197. R/D Trunk Release Feature is added. R/D Trunk can release when the ring signal is

detected for a conversation by pressing dialpad.

198. TEPRI Card reset problem is fixed.

199. [iDCS/081] CLIP review information intermittently does not work. It is fixed.

200. UCD Call duration is incorrect. It is fixed.

201. [iDCS/101] If a UCD group has only one member and in MMC607 GROUP BUSY NEXT is

enabled for this group, when this group overflows to another normal group, a second call in the queue hears nothing but the call rings the group with internal ring tone cadence. It is fixed.

202. [iDCS/105] Set country to Denmark on system, when 1 new message is displayed on

anytype of display phone the message should read 1 NY Besked, at the moment display is

reading 1NY Medd. If there is multiple messages, display should show 2 NYE Beskeder at

the moment is reading 2 NYE Medd.

Display data is changed as below,

1 NY Medd -> 1NY Besked.

2 NYE Medd. -> 2 NYE Besked.  :  2 NYE Beskeder is too long. Display phone can

display only 16 characters.

203. Spain Default Data is changed as below.

204. [iDCS/064] When using a UCD group without an AA card, on the DCS we could hunt the

call on no answer around the members of the group using the ANS-NXT timer and the

RETRY counter. Then send the call to the final destination. This is not possible in the iDCS

software. It is fixed.

205. [iDCS/069] The Hunt nxt timer option in MMC601 does not work for a S0 bus group. It is

fixed.

206. [iDCS/073] Calls to an unallocated number (cause code 1) are not cleared by the system.

The call key is locked up for 35 seconds until the network clears the call with a release

message. It is fixed.

207. [iDCS/092] DDI call to remote site via the Central site (tandem system) using QSIG will not

return DTMF if keypad pressed. It is fixed.

[ L1.40 / M1.40 ]

208. [ITALY_HOTEL] When ‘NO COST PRINT’ option is set YES (MMC300) then both SMDR

and HTM RPT print out a call made from a guest room or a meeting room which has a real

cost unit charged as call cost 0. No problem for call made from normal station, fax and

ADMIN. It is fixed.

209. [ITALY_HOTEL] The SDMR output doesn’t report the unit cost when the call is made as

ISDN AOC. No problem if it is the call is made as Analog TRK MPD. It is fixed.

210. [ITALY_HOTEL] SMDR button doesn’t display on the LCD the call cost made from the FAX

paired with a room. However, if you press the SMDR PRINT button of the paired room,

HTM prints out the call cost properly although the field cost is 0. It is fixed.

211. [ITALY_HOTEL] Both room and telephone credit adding procedure and room bill charging

procedure clear the SMDR info displayed on the LCD. However, if you press the SMDR

PRINT button of the room, HTM prints out the call cost properly although the field cost is 0.

It is fixed.

212. [iDCS/069] The Hunt nxt timer option in MMC601 does not work for a S0 bus group. It is

fixed.

213. [iDCS/106] When an agent uses the ABW key (Agent Busy), callers in the queue do not

hear AA queue messages. It is fixed.

206. [STA_SF074_All] Call button light goes out during incoming PRI calls. It is fixed.

( Example:	Make an incoming PRI call that rings on call button 1 of station 201.  Station

201 answers the call.  Make a second incoming PRI call to station 201. While station

201 is on call 1, call button 2 flashes on station 201.  Station 201 places call 1 on hold

and answers call button 2.  Now station 201 places call button 2 on hold (call 2 light

flashing) and return to call on call button 1 (call 1 light solid).  Now the calling party

holding on the call button 2 hangs up the call.  The problem is the call button lights for

both call buttons 1 and 2 goes out.

Correction:	Only call button 2 should go out and call button 1 should  stay on (solid)

because station 201 is still connected and talking on the PRI call on call button 1.  This

problem should be fixed.)

207. [STA_SF077_All] Inaccurate UCD report problem is fixed.

208. [STA_SF078_All] Cadence voicemail startup problem is fixed.

209. 2 IDCS500 systems are connected via Q-sig with common numbering plan.

If the user(node A) dial the networking extension number(node B) using CTI link(node A),

the wrong extension number is sent to the node B system. This problem is fixed.

210. System clear all bearer capability to 64 Speech for ISDN transit calls. It is fixed.

211. Calls made from a FAX station don’t decrease the phone credit of the paired room.

It’s fixed (ITALY_IT037)

212. [STA_SF042] MMC 503 F-DGT DELY timer: This timer’s minimum value is changed to 000

msec. (Timer range 000 ~ 2500 msec. Can be adjusted to 000, 100, 200msec.)

213. When the user make an ISDN outgoing call, the user can transfer this call to the other

extension or outgoing line before the called party answer.

214. [STA_HOT054] Call costing error is fixed.

215. The default baud rate of Modem port (SIO Port3 without LAN or SIO Port 5 with LAN) is

changed to 9600bps.

216. When the toll deny for tenant 1 is set, the toll deny for tenant2 is automatically set to be all

‘1’ in MMC 702. This problem is fixed.