| Change Note |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CITEL Technologies Ltd. Unit 4 Wheatcroft Business Park Landmere Lane Edwalton Nottingham NG12 4DG |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number |  |  | 50450 |  |  |  |  |  |  |  |  |  |  |  |  |  | Date |  |  | 05/10/04 |  |  |  |
| Originator |  |  | Allen Poyzer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class |  |  | Major |  |  |  |  |  |  |  |  |  | POI |  |  | Immediate |  |  |  |  |  |  |  |
| Product Part No: |  |  |  | ### 1547: 3Com Analog Handset Gateway Software |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for Change Note |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ### Release of 1547: 3Com Analog Handset Gateway Software 2.0.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Documents / Part Affected / Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Number/Version:; 1574. Description:; ### 3Com Analog Handset Gateway Software. Media:; N/A. No: Of Docs\DRWs; 1. Version:; 2.0.0.1. Release Date:; 04/10/2004. Location of CN:; \\engineering\QA\1547_3Com_Analog\Software(SW)\2.0.0.1. Name of file/s:; 1574_2_0_0_1_AHG.zip. Location of CN:; \\engineering\QA\1547_3Com_Analog\Software(SW)\2.0.0.1. Printed Copy:; 3Com Analog 1547. |
| Summary Of Changes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Changes/Fixes In 2.0.0.1 (since 2.0.0.0) Released 04/10/2004 - GM1601 External Paging (6200 - Ext. only & 6202 - Int. and Ext.), from local HGW cards and local NBSets, stops working (announce tone still played) after several External Pages from local HGW cards - GM1775 CLID information not displayed on Memorex MPH615 and Bell Phones Easy Touch 77519. - GM1776 Incorrect Month and Day displayed on Binatone Cliptel 100 phone. - GM2127 Caller ID fails to Memorex phones - Clock loss recovery / Long beep Changes/Fixes In 2.0.0.0 (since 1.0.2.7) Released 30/01/2004 - GM2214 Caller id problem with certain phones - GM2123 We do not set the IP TOS bits from the call state message in outgoing IP packets Changes/Fixes In 1.0.2.7 (since 1.0.2.6) Released 27/11/03 - GM2218 Flash config not sent to NLP when card is booting up Changes/Fixes In 1.0.2.6 (since 1.0.2.5) Released 25/11/03 (CITEL Test Dept only) - GM1691 – Caller ID does not work across a VTL Changes/Fixes In 1.0.2.5 (since 1.0.2.4) Released 22/11/03 - GM2021 - Setting the Ring Frequency does not work. - GM1960 - Caller ID message is not sent on AHG when Caller name exceeds 15 characters - GM1808 - No beep played when Feature 431 (Conference drop) is used. - GM1696 - When bringing a 4th party into a Conference, Conference error. These issues have been re-tested and classed as fixed in 1.0.2.5. - GM1733 - DTMF tones sent between Citel analog phones are longer in duration compared to - the 3Com ATA. - GM1964 – Analog version number now displayed correctly in NetSet - GM1931 - 100.9 % Max. CPU load reported by the DCP System Stats on HGW cards. Note that card functionality appears to be unaffected. - GM1778 - IP/On/Fly only: Contention fixes stop paging working - GM1661 - 'H3: unrecognised command: 0x12' printed on the DCP console when the Redial key is pressed or Feature 401 is dialled - GM1534 - When changing from flash to serial boot mode and vice-versa the following error message is displayed on the DCP console:'ERROR:RCPM_GET_NEXT_BOOT_FAILED'. - GM1522 - SSC / TDM drift causes minor audio disturbance - GM1495 - Need to discover multiple MAC addresses apparently. - NLPconsole. - GM1478 - Transferring Analog Conferences over VTL's results in Conferences being unable to be set-up from any type of phone. - GM1300 - On a remote HGW card there is a 30 second delay after the Auto-attendant - GM1172 - NCP MAC address not cleared when link is lost. - GM1166 - Default should be - don't use DHCP - when not configured - GM1165 - IPOTF will not be re-established after loss and re-establishment of the link to the NBX - GM1162 - HG card, remote from NCP over a network that does not forward broadcasts. - GM1039 - When a Gateway card has booted up 'SSC Resync' / 'SSC Failed' messages are displayed - GM1005 - When using Voice Mail from a remote HGW phone there is a 30 second delay Changes/Fixes In 1.0.2.4 (since 1.0.2.3) Released 22/09/03 (CITEL Test Dept only) - New upgrade method implemented Changes/Fixes In 1.0.2.3 (since 1.0.2.2) Released 21/07/03 - GM1753. When a cross-NBX (VTL) call over a WAN / INTERNET rolls over to Voice Mail no Auto-attendant prompts are heard and VM messages cannot be left. - GM1516/GM1576: initialisation to the switch takes several times before completion - GM1762: CPU max load > 100 % reported - GM1763: locks-up when running a 6 (12 port) Fax load test after approx. 3 hours. Changes/Fixes In 1.0.2.2 (since 1.0.2.1) Released 10/07/03 (CITEL Test Dept only) - GM1692 - Watchdog poll failures. - GM1691 - Caller ID does not work across a VTL Changes/Fixes In 1.0.2.1 (since 1.0.2.0) Released 26/06/03 - GM1689 - No audio from the local Analog to the local NBset when in a three party IP Conference involving a local Nbset - GM 1615 – Loosing audio on a conference call with personal speed dials (PRB03701) Changes/Fixes In 1.0.2.0 (since 1.0.1.9) Released 26/06/03 - GM1686 - No audio an cross-NBX Transfers. - GM1620 - Reopened - No audio on a conference call Changes/Fixes In 1.0.1.9 (since 1.0.1.8) Released 26/06/03 (CITEL Test Dept only) - GM1677 - pbufDataPrint can loop indefintiely - GM1676 - Fix SSC startup errors, remove annoying texts and 'double free' print on NLP. - GM1675 - No Auto-attendant prompt heard when a cross-NBX (VTL) call rolls over to Voice Mail. - GM1671 - sup discs are too short and occasional spurious ones seen (PRB03752) - GM1663 - The first message to the NLP gets lost in an SSC resync - GM1660 - blade mac address printed from h3 not eth struct - GM1659 - contention algorithm times out if addresses configured - GM1657 - text output confusing if NLP fails to resolve contention and timeout occurs - Text string implemented correctly to show SW revision in NetSet: (PRB03764) Changes/Fixes In 1.0.1.8 (since 1.0.1.7) Released 24/06/03 - GM1663 - The first message to the NLP gets lost in an SSC resync - GM1626 - A panic on the NLP does not leave a PM trace - GM1621 - fax tones detected during conference transfer leads to one way audio to 3COM phone Changes/Fixes In 1.0.1.7 (since 1.0.1.6) Released 23/06/03 (CITEL Test Dept only) - GM1626 - A panic on the NLP does not leave a PM trace - GM1621 - fax tones detected during conference transfer leads to one way audio to 3COM phone - GM1620 - No audio on a conference call (PRB03727 ) - GM1619 - Attempting to Transfer an IP-level Conference from the local card to the cross-card causes both the remote and local cards to lock-up / crash - GM1618 - If the citel board has contention then it should not continually reboot (PRB03713) - GM1617 - Not receiving dial tone after entering in feature code 890 (PRB03726) - GM1613 - Citel board is looking for a layer 3 system state msg instead of layer 2 (PRB03712) - GM1610 - Hammer load test only runs about 1800 calls before the board goes offline (PRB03698) - GM1609 - Citel board sees multiple NCPs when there is only one on the system. (PRB03676) - GM1594 - remote citel board will never have channels in the online state, always unknown (PRB03667) Changes/Fixes In 1.0.1.6 (since 1.0.1.7) Released 13/06/03 - GM1604 - Ports Malfunction under CITEL Load Test. - GM1596 - Success tone on direct mail transfer is not correct. (PRB03646) - GM1590 - Ring back in park does not work. (PRB03650) - GM1588 - Volume in Conference is too low and echos. (PRB03661) - GM1583 - Direct mail transfer of an internal call to voice mail box not working (PRB03645) - GM1581 - Fax working – local fax working- new GM 1582 for off card fax problems. - GM1580 - DTMF generation problems – recirculation DTMF - GM1578 - Busy tone does not get included in a conference – Non Issue - GM1577 - Hear beep on all phones when blind conference is cancelled (PRB03625) - GM1567 - Electronic/ conference beep too loud - GM1528 - Disconnect tone selection is not working (PRB03608) - GM1526 - Improve Boot Time (shaved 10 Seconds off time) - GM1488 - Report status information. (PRB03657) - GM1485 - Status fields need zeroing (PRB03657) - GM1445 - Pbuf Starvation for Faxes. (test on Fax load test) Bug Fixes inherited from other products with Common Modules: - GM1584 Buffer over page boundary. non issue - GM1518 Voice Mail broken/crackling audio - GM1598 FunTest crash Changes/Fixes In 1.0.1.5 (since 1.0.1.3) Released 11/06/03 (CITEL Test Dept only) - GM1596 - Success tone on direct mail transfer is not correct. (PRB03646) - GM1590 - Ring back in park does not work. (PRB03650) - GM1588 - Volume in Conference is too low and echos. (PRB03661) - GM1583 - Direct mail transfer of an internal page to voice mail box not working (PRB03645) - GM1581 - Fax working – local fax working- new GM 1582 for off card fax problems. - GM1580 - DTMF generation problems – recirculating DTMF - GM1578 - Busy tone does not get included in a conference – Non Issue (PRB03623) - GM1577 - Hear beep on all phones when blind conference is cancelled (PRB03625) - GM1567 - Electronic/ conference beep too loud - GM1528 - Disconnect tone selection is not working (PRB03523) - GM1526 - Improve Boot Time (shaved 10 Seconds off time) (PRB3558, PRB3532, PRB3525) - GM1488 - Report status information. - GM1485 - Status fields need zeroing (PRB03457) - GM1445 - Pbuf Starvation for Faxes. (test on Fax load test) Bug Fixes inherited from other products with Common Modules: - GM1584 Buffer over page boundary. non issue - GM1518 Voice Mail broken/crackling audio - GM1598 FunTest crash Changes/Fixes In 1.0.1.4 (since 1.0.1.3) Not Released (CITEL Test Dept only) Changes/Fixes In 1.0.1.3 (since 1.0.1.2) Released 30/05/03 (CITEL Test Dept only) - GM1528 - dial tone is lost after remote party hangs up only if sup disc period set to 0when the remote party hags up and the sup disc period is set to 0 dial tone does not reappear. Before fix 1515 tone was always lost. - GM1519 - ring back tone disappears after one ring - GM1517 - difficulty in configuration of IPOTF remote - GM1516 - initialisation to the switch takes several times before completion - GM1515 - on hook in supervisory disconnect is lost and subsequent calls go to voicemail - GM1501 - IGMP fix from Analog - GM1495 – Need to discover multiple MAC addresses - and learn IP address of system state message broadcasts - GM1494 - Power down of phones required to re-start s/w. Hardware Problem. - GM1490 - Issue Id: PRB03503. Summary: Citel analog board will not auto discover with a large database. - GM1489 - Issue Id: PRB03507: Citel analog feature codes. Lift handset, hit # key. Should be different tone - GM1486 - Issue Id: PRB03458 Summary: Dial Tone is raspy in comparison with 3Com's Description: - GM1483 - NBX100. Dial tone not present on certain Analogue phones on certain ports. For example port 0 (Autophon) and port 15 (BT Viscount) analogue phones have no dial tone. - GM1482 - NBX100. Side-tone too loud on some analog phones (e.g. BT Duet 500 and Cortelco 8150). - GM1476 - (A1.0.0.8) 1.32. 28/04/03. (NBX 100, ST-IP). The higher pitched tone of the 'Feature Active Tone' has phasing volume on analogue phones. - GM1475 - (A1.0.0.8) 1.31. 28/04/03. (NBX 100, ST-IP). No 'Feature Entry Tone' on Analogue phones. - GM1461 - ICMP response to broadcast - GM1460 - NBX100, Ethernet + ST-IP. Intermittent fault. Phones sometimes do not initialise (do not 'chirp') when card the HGW card (MAC: 00:06:e4:dd:05:00) boots-up and hence, have no functionality. When this occurs disconnect the phone power supply for a couple of minutes then reconnect power supply (may also have to reboot the card) to initialise the phones. - GM1456 - Ethernet + ST-IP. Intermittent fault (seen 3 times): Wrong analog phone rings - GM1442 - Large configuration/large H3 message handling – variable length DCP buffers. - GM1435 - (A1.0.0.8) On Ethernet only configuration, (and only on this configuration) when the caller initiates a page then the caller hears a faint road drill noise. - GM1412 - fifo full errors from ssc receive seen in analogue load test. This means messages from the NBX are being lost. - GM1411 - The voicemail crash is back! - GM1382 - Timeslot slippage on TDM/DSP interface.Jitter on DSP clock. C.f. 1.8 - GM1355 - Problem with IPOTF - put a conference on hold. Problem bringing it back. - GM1346 - crash when fax from us to an ATA - GM1345 - copyright statements are now printed in full but require better formatting for hyper-terminal - GM1333 - SLIC interrupt line seizes ON - GM1311 - crash in isr_read_audio_from_net at line 577 happens infrequently - GM1291 - analog card freezes then CR at DCP console causes flash boot - GM1203 - Dataworks/CEI CT10027 - Feature 100, tone is missing which should announce incoming call - GM1162 - HG card, remote from NCP over a network that does not forward broadcasts will not boot - GM993 - (1.2.3.0 Analogue) Port zero fails with errors 1,2 & 4. Overheat, sync error etc. - GM958 - Voicemail over VTL Changes/Fixes In 1.0.1.2 (since 1.0.1.1) Released 29/05/03 - GM1435 - (A1.0.0.8) On Ethernet only configuration, (and only on this configuration) when the caller initiates a page then the caller hears a faint road drill noise. - GM1412 - fifo full errors from ssc receive seen in analogue load test. This means messages from the NBX are being lost. Changes/Fixes In 1.0.1.1 (since 1.0.1.0) Released 30/04/03 (CITEL Test Dept only) - Same fixes as 1.0.1.0 Changes/Fixes In 1.0.1.0 (since 1.0.0.9) Released 29/04/03 (CITEL Test Dept only) - GM1489 - Issue Id: PRB03507: Citel analog feature codes. Lift handset, hit # key. Should be different tone - GM1490 - Issue Id: PRB03503. Summary: Citel analog board will not auto discover with a large database. - GM01486 - Issue Id: PRB03458 Summary: Dial Tone is raspy in comparison with 3Com's Description: - GM1506/1524 - The remote Norstar phone in a 4-party Conference with a local card loses audio when a second IP conference is created - GM1501 - IGMP fix from Analog - GM1495 – Need to discover multiple MAC addresses - and learn IP address of system state message broadcasts - GM1461 – ICMP response to broadcast - GM1516 - initialisation to the switch takes several times before completion - GM1517 – difficulty in configuration of IPOTF remote - GM958 - Voicemail over VTL - GM1162 - HG card, remote from NCP over a network that does not forward broadcasts will not boot - GM1442 - Large configuration/large H3 message handling – variable length DCP buffers. - GM1456 - Ethernet + ST-IP. Intermittent fault (seen 3 times): Wrong analog phone rings - GM01333 - SLIC interrupt line seizes ON - GM01346 - crash when fax from us to an ATA - GM01382 - Timeslot slippage on TDM/DSP interface.Jitter on DSP clock. C.f. 1.8 voltsupply - GM01411 - the voicemail crash is back! - GM01311 - crash in isr_read_audio_from_net at line 577 happens infrequently - GM01519 - ring back tone disappears after one ring - GM00993 - (1.2.3.0 Analog) Port zero fails with errors 1,2 & 4. Overheat, sync error etc. - GM01345 - copyright statements are now printed in full but require better formatting for hyperterm - GM01355 - Problem with IPOTF - put a conference on hold. Problem bringing it back. - GM01450 - the stack sizes are too small in certain crash situations. - GM01516 - to the switch takes several times before completion - GM01515 - on hook in supervisory disconnect is lost and subsequent calls go to voicemail - GM01291 - analog card freezes then CR at DCP console causes flash boot - GM01412 - fifo full errors from ssc receive seen in analog load test. This means messages from the NBX are being lost - GM1483 - NBX100. Dial tone not present on certain Analog phones on certain ports. For example port 0 (Autophon) and port 15 (BT Viscount) analog phones have no dial tone. - GM01476 - (A1.0.0.8) 1.32. 28/04/03. (NBX 100, ST-IP). The higher pitched tone of the 'Feature Active Tone' has phasing volume on analog phones. - GM01482 - NBX100. Side-tone too loud on some analog phones (e.g. BT Duet 500 and Cortelco 8150). - GM01528 - dial tone is lost after remote party hagns up only if sup disc period set to 0when the remote party hags up and the sup disc period is set to 0 dial tone does not reappear. Before fix 1515 tone was always lost. Changes/Fixes In 1.0.0.9 (since 1.0.0.8) Released 10/04/03 - This release is the same as 1.0.0.8 except it is built to inter-work with 3COM software version 4.2, using the CITEL analog device type. Changes/Fixes In 1.0.0.8 (since 1.0.0.7) Released 09/04/03 - GM1382 - Timeslot slippage on TDM/DSP interface.Jitter on DSP clock. C.f. 1.8 volt supply. - GM1354 - DHCP problems. - GM1005 - IPST+IPOTF. 1 minute delay when retrieving Voice mail messages from Remote Norstars. - GM1382 - Timeslot slippage on TDM/DSP interface. Jitter on DSP clock. C.f. 1.8 volt supply - GM1333 - SLIC interrupt - GM1335 - SUPERVISORY disconnect handling - Phone off hook before boot is complete - horrible growling, and card may not recover. - This build comes with manufacturing support - US & UK settings. Country code has to set using hyperterminal – Config Tool updated to support this. - Post mortem crash capability and python debugger compatibility. Changes/Fixes In 1.0.0.7 (since 1.0.0.6) Released 01/04/03 (CITEL Test Dept only) - GM1382 - Timeslot slippage on TDM/DSP interface.Jitter on DSP clock. C.f. 1.8 volt supply. Changes/Fixes In 1.0.0.6 (since 1.0.0.5) Released 25/03/03 (CITEL Test Dept only) - An issue with audio between local ports has been resolved; this was caused by changes to the fax audio driver. - GM1354 - DHCP problems in new repository. Also simplified DHCP flags. - GM1355 - Problem with IPOTF – put a conference on hold. Problem bringing it back. Changes/Fixes In 1.0.0.5 (since 1.0.0.4) Not Released Changes/Fixes In 1.0.0.4 (since 1.0.0.3) Released 17/03/03 (CITEL Test Dept only) - GM1344: Copyright text at boot now correct - check - Conference and voice mail beep is too loud. - Country codes available. - LED problem in 1.0.0.2. LEDs now flash when handset lifted. - Functional test software adapted to include the Slicofi's initialisation sequence. - Recall button on BT phones now tested. - Click in continuous tones. - Phone off hook before boot is complete - horrible growling, and card may not recover. - V1.0.0.4 to comes with manufacturing support - GM1005 - IPST+IPOTF. 1 minute delay when retrieving Voice mail messages from Remote Norstars. - GM1063 : Strobing Window on Console. & no icon available. - GM1220 - Conference teardown, IPOTF - GM1321 - Ethernet chip trace and fix from Steve L. - GM1209 Duplicate IP addresses. See h3_protocol.c non_conf - GM1165 - IPOTF will not be re-establish after loss and re-establish of link to NBX. - GM1346. crash when fax from us to an ATA Changes/Fixes In 1.0.0.3 (since 1.0.0.2) Released 06/03/03 (CITEL Test Dept only) - DHCP testing not complete. Changes/Fixes In 1.0.0.2 (since 1.0.0.1) Released 06/02/03 (CITEL Test Dept only) - All fixes implemented in 3COM Norstar 2.0.2.5 reviewed and tested in this build, as it is at the same base level of software. Changes/Fixes In 1.0.0.1 (since 1.0.0.0) Not Released Changes/Fixes In 1.0.0.0 Released 01/09/02 - Initial release of Analog software for conformance testing (part build) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distribution /Affected Depts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distribute To Following Areas: <place an “X” in relevant sections> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Customer |  |  |  | X | Quality |  |  |  | X |  | Manufacturing |  |  |  |  |  |  | X |  | Software Eng |  |  |
|  | Hardware Eng |  |  |  |  | Sub-contractor |  |  |  | X |  | Sales |  |  |  |  |  |  | X |  | Support |  |  |
|  | Despatch |  |  |  |  | Design / Print |  |  |  | X |  | Test |  |  |  |  |  |  |  |  | Partner |  |  |
| Approvals |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Originator |  | Manager |  | Manufacture |  |  |  | Approvals |  |  | VP Engineering Director |  |  |  |  |  |  | Partner |  |  |  | QA |  |
| Date: 04/10/04 |  | Date: 04/10/04 |  | Date: 04/10/04 |  |  |  | Date: 04/10/04 |  |  | Date: 04/10/04 |  |  |  |  |  |  | Date: 04/10/04 |  |  |  | Date: 04/10/04 |  |

Part Number: 1274_5.0