Avaya

- Instructions for Upgrading  –

- Integral Voicemail for 3.0 Software

Draft 1

The Master electronic file is held in the Product Document Management System

The company reserves the right to make alterations or amendments to this specification at its discretion.  The information in this document does not imply freedom from patent or other protective rights of Avaya or others.

Template:- Product Document Management System – Product Definition Document

- Table of Contents

1.	Table of Contents	2

2.	Introduction	3

3.	Upgrading to Version 3.0	4

4.	History	5

- Introduction

This document contains the installation instructions for upgrading IP Office systems with Integral Voicemail to use the 3.0 release of the IP Office core switch software. This procedure is necessary when upgrading Small Office systems with “Old Style” memory cards containing sound clips for a single locale.

# Upgrading to Version 3.0

This should only affect existing Small Office systems that are upgraded to version 3.0 software. The following procedure should be followed  :-

- First, the 3.0 software should be installed. When the system starts up, the new directories will be created to contain the country-specific sound-clips. Wait for the “Voicemail Activated”  or “NO INTEGRATED VOICEMAIL due to previous errors” message to be displayed on the SysMonitor utility.

- Next, transfer the new sound-clips files to the system as follows :-

  - Ensure that the system has a TFTP Server IP Address configured on the System tab, and also that the File Writer IP Address is configured to be the IP address of the users PC.

  - Load the Installation CD into the PC’s CD drive, and run a DOS command tool on the PC. Then change the working directory in the command tool to the CD drive:LVMSound\G723 Files.

  - Run the batch file send723. A separate “Transfer Succeeded” message should be displayed for each sound clip transferred. The whole process should take around 30 minutes.

- When all of the sound-clips have been transferred, restart the system. The system will assimilate the sound-clips during the first system start-up and the integral voicemail will be activated in the normal manner. This may take up to 10 minutes. Subsequent start-ups will take substantially less time.

- History

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Version | Author | Date | Changes |
| Issue |  |  |  |