Avaya

- Instructions for Upgrading  –

- Integral Voicemail for 4.0 Software

Draft 1

The Master electronic file is held in the Product Document Management System

The company reserves the right to make alterations or amendments to this specification at its discretion.  The information in this document does not imply freedom from patent or other protective rights of Avaya or others.

Template:- Product Document Management System – Product Definition Document

- Table of Contents

1.	Table of Contents	2

2.	Introduction	3

3.	Upgrading to Version 4.0	4

4.	History	5

- Introduction

This document contains the installation instructions for upgrading IP Office systems with Integral Voicemail to use the 4.0 release of the IP Office core switch software. This procedure is necessary when upgrading Small Office or IP406v2 systems with 3.0 or 3.1 software installed. For earlier versions of the software, refer first to “Instructions for Upgrading  Integral Voicemail for 3.0 Software”.

# Upgrading to Version 4.0

This should only be performed when upgrading Small Office or IP406v2 systems from  version 3.0 or 3.1 software to version 4.0 software. For systems with earlier versions of the software installed, refer first to “Instructions for Upgrading  Integral Voicemail for 3.0 Software”. The following procedure should be followed  :-

- First, the 4.0 software should be installed. Wait for the “Voicemail Activated”  message to be displayed on the SysMonitor utility.

- Next, transfer the new sound-clips files to the system as follows :-

  - Ensure that the system has a TFTP Server IP Address configured on the System tab, and also that the File Writer IP Address is configured to be the IP address of the users PC.

  - Load the Installation CD into the PC’s CD drive, and run a DOS command tool on the PC. Then change the working directory in the command tool to the appropriate CD drive, ie.  LVMSound\G723 Files ( Small Office Edition ) or LVMSound\G711 Files ( IP406v2 ).

  - Run the appropriate batch file, ie. Upgrade4p0_723 ( Small Office Edition ) or Upgrade4p0_711 ( IP406v2 ). A separate “Transfer Succeeded” message should be displayed for each sound clip transferred. The whole process should take around 5 minutes.

- When all of the sound-clips have been transferred, wait for the next housekeeping cycle to be performed ( maximum of 5 minutes ). The system will assimilate the sound-clips during the next the next housekeeping cycle.

- History

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Version | Author | Date | Changes |
| Issue |  |  |  |