1. (Single Choice) Which of the following is not a common virtualization platform system?
A.  FusionSphere
B.  VMware vSphere
C.  Hadoop
D.  Hyper-V

2. (Multiple Choice) Which of the following statements about the security solution supported by the active-active data center are correct?
A. Supporting Internet Access Zone Security Solution
B. Supporting Internet Access Security Solution
C. Supporting the Data Center Interconnection Area Security Solution
D. Supporting the Security Solution for the Data Center Service Area

3. (True or False) In the Disaster Recovery Data Center Solution (Active-Active Mode), two data centers are running and can bear production services at the same time. Currently, two active-active data centers are available: AP active-active and AA active-active, but the AP active-active is the real one.

4. (Multiple Choice) Which of the following statements about the Huawei SDN data center solution are correct?
A. The server layer supports mainstream virtualization platforms and does not support mainstream virtualization platforms.
B. Fabric network layer: Supports the Network/Hybrid/Host Overlay VXLAN-based Fabric network solution.
C. The network control layer controls the physical and virtual networks in a unified manner and interconnects with the cloud platform through the northbound interface.
D. Service provision/orchestration layer: Customers can select their own application portals, Huawei ManageOne products, or use the AC network orchestration page to provision services.

5. (Multiple Choice) Which of the following are VXLAN solutions in the cloud data center SDN solution?
A.  Network Overlay
B.  Host Overlay
C.  Hybrid Overlay
D.  Tenant Overlay

6. (Single Choice) During the operation of enterprise B, some services are planned to be placed on the public cloud and some services are placed on the private cloud based on the cost. The following describes the planning and design of the VDC and VPC, which of the following statements is incorrect?
A. When services are deployed in the private cloud, the IP addresses of the entire network are planned in a unified manner, and the public IP addresses do not overlap.
B. When services are deployed in the private cloud, multiple VDCs can be deployed in one POD, and a single VDC can be deployed only in the same POD.
C. During the deployment of services on the public cloud, tenants' internal networks are planned and IP addresses of tenants can overlap.
D. During the deployment of services on the public cloud, the VPC resources created by tenants can only be a subset of VDCs. Therefore, VPCs can be deployed only in one POD in multi-tenant scenarios.

7. (True or False) The NIP is a network intrusion prevention device, and the virus does not belong to the intrusion scope. Therefore, the NIP cannot defend against viruses and needs to be deployed with antivirus measures.

8. (Single choice) In the Huawei SDN solution, which of the following statements about the vDC active/standby DR scenario is incorrect?
A. Single Domain Multiple Fabric Unified Resource Pools
B. VPC cross-fabric
C. Intra-VPC FWs Cross-Fabric Active/Standby
D. VPC Associating with a Single External Network

9. (Single choice) In the Huawei SDN solution, which of the following statements about the AC DR switchover in the vDC active/standby DR scenario is incorrect?
A. Two clusters work in active/standby mode and back up each other.
B. Active/standby switchover takes about 20 minutes.
C. The link between two DCs is Down. The cluster splits. The arbitration service is used to determine whether a split-brain occurs. The master is determined based on the pre-configured priority to avoid dual-active.
D. Single-DC Power Failure and Standby Switchover to the Active State

10. (Single choice) In the Huawei SDN solution, which of the following statements about the SLA requirements for the DC interconnection network of the vDC active/standby DR site is incorrect?
A. Delay RTT<2ms
B. Distance <200km
C. The management network bandwidth is >1GE. 10GE links are recommended.
D. The storage network bandwidth must be greater than the peak I/O of the DR disk array, and at least 20% bandwidth margin must be reserved.

11. (Multiple Choice) Which of the following statements about load balancing devices in Huawei cloud data center are correct?
A. SLB is responsible for server load balancing.
B. GSLB is responsible for server load balancing.
C. SLB is responsible for global service load balancing.
D. GSLB is responsible for global service load balancing.

12. (Multiple Choice) Which of the following are the service capabilities provided by the load balancing device in Huawei cloud data center?
A.  VIP（Virtual IP）
B.  IPSec VPN
C. SSL Certificate
D. Session Holding Policy

13. (Multiple Choice) Which of the following can be deployed in CloudFabric?
A. Spine and Border Leaf
B. Spine and Server Leaf
C. Spine and Service Leaf
D. Border Leaf and Service Leaf

14. (Single Choice) In the cloud network integration scenario, which of the following statements about creating a routed network on Service Center is true?
A. The cloud platform automatically creates routers and subnets.
B. The AC-DCN automatically creates a logical router.
C. The AC-DCN automatically creates a routed network.
D. The cloud platform automatically creates a subnet.

15. (Single Choice) A company uses 80 RH servers to deploy a cloud platform and provides 1550 VMs for employees. How many VRM nodes are recommended?
A. 6 vCPU, 16 GB memory, 80 GB hard disk
B. 8 vCPU, 32 GB memory, 80 GB hard disk
C. 10 vCPU, 32 GB memory, 80 GB hard disk
D. 14 vCPU, 64 GB memory, 80 GB hard disk

16. (Single Choice) There are two records on the KVM storage resource pool page of the CPS. How many hosts need to be added to the host to add the block storage-driver role?
A.  3
B.  4
C.  5
D.  6

17. (Single choice) When a company purchases a cloud data center device, the hardware must be installed in one cabinet. Which of the following is the installation sequence from bottom to top?
A. Storage -Rack Server -Switch
B. Rack Server -Storage -Switch
C. Rack Server -Switch -Storage
D. Storage -Switch -Rack Server

18. (Single Choice) In a cloud data center scenario (the virtualization software is FusionCompute (VRM+CNA, Xen)), which of the following storage partitions is not required on FusionSphere OpenStack controller nodes?
A.  image
B.  image-cache
C.  swift
D.  ceilometer

19. (Single Choice) Which of the following methods is used to purchase some storage devices to ensure that the storage usage is greater than 2 TB?
A. 6*900G disk, among which one is a hot spare disk and the other is RAID 10.
B. 6*600G disks, among which one is a hot spare disk and the other is a RAID 6 group.
C. 6*600G disk, among which one is a hot spare disk and the other is a RAID5 group.
D. 6*600G disk, among which one is a hot spare disk and the other is RAID 1.

20. (Multiple Choice) Which of the following configuration modes are supported by FusionCompute?
A. Common Disk
B. Thin Provisioning
C. Common lazy zeroed disk
D. Differential Disk



Answer:
/ 1  C    / 2  ABCD    / 3  F    / 4  BCD    / 5  ABC    / 6  B    / 7  F    / 8  D    / 9  B    / 10  B    / 11  AD    / 12  ACD    / 13  ACD    / 14  D    / 15  C    / 16  D    / 17  A    / 18  B    / 19  C    / 20  ABC