- (TorF) IS-IS fast convergence is an extended feature of IS-IS to speed up route convergence. Incremental SPF (I-SPF) calculates routes only for the affected nodes if the network topology changes.

- (TorF) On a BGP network, you can configure prefix-based ORF to enable a BGP device to receive only the required routes.

- (TorF) OSPFv3 and OSPFv2 support the same types of messages. OSPFv3 uses the link-local address as the source address of packets to be sent, and messages can be forwarded beyond the originating link.

- (Single) IGP route convergence is faster than BGP route convergence, which may cause network traffic loss. On an OSPF network, OSPF-BGP synchronization can be enabled to allow a device to remain in a specified state within a specified synchronization period, preventing traffic loss. Which of the following statements about the specific state is false?

  - A device in this state is called a stub router.

  - A device in this state speeds up BGP convergence.

  - A device in this state advertises LSAs with the maximum link metric.

  - Other devices do not preferentially select this device to forward data.

- (Single) When a network is stable, which of the following statements is false about how to speed up IS-IS network convergence?

  - Reduce the interval at which LSPs are sent on the device interface to speed up convergence.

  - Configure LSP fast flooding on the device to speed up LSDB synchronization and convergence.

  - Enable the intelligent timer on the device to speed up LSP generation, which speeds up route convergence.

  - Enable the intelligent timer on the device to prolong the maximum delay for SPF route calculation, speeding up route convergence.

- (Single) As shown in the figure, AS101, AS102, and AS103 communicate with each other through BGP. R1 establishes an EBGP peer relationship with R2, and R2 establishes an EBGP peer relationship with R3. R1 adds the community <101:1> to the route destined for 10.1.1.1, and R3 adds the community <no-export> to the route destined for 10.1.1.1,After network convergence is complete, an engineer performs Step 1 to check the routing table of R3's route destined for 10.1.1.1. Later, the engineer performs configurations in Step 2 on R2. Which of the following statements about the preceding scenario is true?

  - The command output in step1 shows that R1 imports the route destined for 10.1.1.1 into the BGP routing table using the "import-route" command.

  - The command output in step1 shows that R3 will advertise the route to other ASs after receiving the route.

  - According to the configuration in Step 2, after the network is converged again, check the entry of the route 10.1.1.1 on R3. The route carries only the No-Export attribute.

  - According to the configuration in Step 2, after network convergence is performed again, R3 cannot receive the route to 10.1.1.1.

- (Single) As shown in the figure, R1 transmits EBGP routes to R2, and R2 transmits EBGP routes to R3. An engineer checks the BGP routing tables of R2 and R3 and finds that no entry can be displayed on R3. In addition, the engineer configures the AS_Path filter only on R3. Which of the following statements about the preceding scenario is false?

  - At the end of the AS_Path filter list created on R3, the following AS_Path filter may not be created: ip as-path-filter 1 permit .*

  - The following AS_Path filter may be created on R3: ip as-path-filter 1 deny ^101_

  - The following AS_Path filter may be created on R3: ip as-path-filter 1 deny _101$

  - The following AS_Path filter may be created on R3: ip as-path-filter 1 deny ^10.

- (Single) A BGP peer group is a set of peers with the same policy. Which of the following statements about a peer group is true?

  - When a route-policy is applied to a peer group, routing entries of all members in the peer group may be affected.

  - A BGP peer group is valid only in an AS. Therefore, only IBGP peer groups can be created.

  - When a route-policy is applied to a peer group, the routing tables of members in the peer group are not affected.

  - After a peer group is created on a BGP device, all peers of the BGP device are added to the peer group by default.

- (Single) As shown in the figure, an engineer uses two routers to test IPv6 services. The two routers are directly connected through a network cable, and R1 is reachable to the Loopback0 address of R2 through a static route. Which of the following configuration modes is true for the engineer to meet the requirement?

  - [R1] ipv6 route-static 2001:DB8:2345:2::2 128 GigabitEthernet 0/0/1

  - [R1] ipv6 route-static :: 0 GigabitEthernet 0/0/2

  - [R1] ipv6 route-static 2001:DB8:2345:2::2 128 2001:DB8:2345:12::2

  - [R1] ipv6 route-static 2001:DB8:2345:2::2 128 FE80::2E0:FCFF:FE15:3512

- (Single) As shown in the figure, both IPv4 and IPv6 are deployed on a campus network for service tests. IS-IS is used to implement network interconnection. All devices are Level-2 devices. R1, R4, and R5 support IPv4, not IPv6. By default, which of the following devices is the next hop for R3 to access R6 in the IPv6 SPT?

  - R1

  - R2||

  - |R4

  - R5

- (Multiple) MP-BGP is an extension of BGP-4 and uses address families to differentiate network layer protocols. Which of the following address families are supported by MP-BGP?

  - IPv4 multicast address family

  - VPNv4 address family

  - IPv6 unicast address family

  - VPNv6 address family

- (Multiple) OSPF supports multi-process, and the typical application of OSPF multi-process is in the VPN scenario. Which of the following statements about OSPF multi-process are true?

  - One OSPF process can be associated with only one VPN instance.

  - One OSPF process can be associated with multiple VPN instances.

  - One VPN instance can be associated with multiple OSPF processes.

  - If multiple OSPF processes are created on an OSPF router, the OSPF processes are independent of each other.

- (Multiple) As shown in the figure, OSPF is deployed on a campus network to implement interworking. An engineer wants to reduce the number of LSAs received by R4. Which of the following configurations can meet this requirement?

  - A

  - B

  - C

  - D

- (Multiple) If the "ip as-path-filter 1 permit ^10[^3-7]$" command is run on a BGP router, which of the following AS_Path attributes can match the filter in this command?

  - AS_Path 102

  - AS_Path 104

  - AS_Path 106

  - AS_Path 108

- (Multiple) OSPFv3 and OSPFv2 have slightly different LSA functions. Which of the following statements about OSPFv3 LSAs are true?

  - Intra-Area-Prefix-LSA describes network segment information.

  - Inter-Area-Router-LSA describes the route to an ASBR.

  - Link-LSA describes the link-local address and IPv6 prefix of a local interface.

  - Inter-Area-Prefix-LSA describes the route to a certain network segment in an area.

- (Multiple) On a campus network, IPv6 is deployed for service tests. Four routers are deployed on the network, and OSPFv3 is used for IPv6 network connectivity. As shown in the figure, an engineer obtains an Intra-Area-Prefix-LSA from the LSDB of R2. Which of the following statements about the LSA are true?

  - The router that generates the LSA is R4.

  - R4 is the DR on the network segment where GE0/0/2 of R4 resides.

  - The interface ID of GE0/0/2 of R4 is 0.0.0.5.

  - The LSA describes the IPv6 prefix address associated with the Route-LSA.

- (Single) Which of the following statements about VLAN aggregation are true?

  - When a VLAN is configured as a super VLAN, the VLANIF interface corresponding to the VLAN is always up after being assigned an IP address.

  - When VLAN aggregation is configured, a super VLAN and a separate VLAN must be defined.

  - Different from a common VLAN, a super VLAN can only be configured on a Layer 3 VLANIF interface, not a physical interface.

  - VLAN aggregation can conserve VLAN resources.

- (Single) MUX VLAN needs to be deployed on an enterprise office network. Which type of VLAN is used to isolate customers,but allow them to access enterprise resources?

  - Separate VLAN

  - Super VLAN

  - Client VLAN

  - Subordinate VLAN

- (Multiple) Which of the following technologies can be used to ensure the security of an Ethernet switching network?

- MACSec

- IP Source Guard

- port security

- DHCP Snooping

- (Single) DHCP snooping is a security feature of DHCP. Which of the following statements about DHCP snooping is false?

  - DHCP snooping uses the trusted interface function and the binding table to ensure DHCP network security.

  - An untrusted interface discards all DHCP messages.

  - A trusted interface receives DHCP ACK, NAK, and Offer messages from a DHCP server.

  - DHCP snooping can be used together with IPSG.

- (Multiple) When a static LSP is configured, which of the following labels can be used as the outgoing label of a transit LSR?

  - 0

  - 3

  - 1000

  - 6343

- (TorF) An MPLS header is added between the data link layer and the network layer to implement fast data forwarding. MPLS supports multi-layer MPLS label nesting.

- (Single) When LDP is deployed on an MPLS network to distribute labels, to establish LSPs on demand and save label space as much as possible, which of the following combination of the label distribution control mode and label retention mode should be used?

  - DoD+Conservative

  - DoD+Liberal

  - Du+Conservative

  - Du+Liberal

- (Single) Which of the following statements about BGP/MPLS IP VPN is false?

  - MPLS VPN uses MP-BGP to advertise VPN routes on carriers' backbone networks (IP networks) and uses MPLS to forward VPN packets on carriers' backbone networks.

  - BGP/MPLS IP VPN, also called MPLS VPN, is a common L3VPN technology.

  - VRFs are created on PEs to store routing entries of different VPNs.

  - All labels used during MPLS VPN traffic forwarding are allocated by LDP.

- (Single) The figure shows the MPLS VPN topology in Hub&Spoke networking. Site 1 and site 3 are spoke sites, and site 2 is a hub site. Spoke sites communicate with each other through the hub site. Which of the following are the correct settings for X and Y in the preceding figure?

  - X=100:1,200:1;Y=100:1

  - X=200:1;Y=100:1,200:1

  - X=100:1;Y=100:1

  - X=100:1;Y=200:1，100:1

- (Multiple) When MPLS VPN is deployed, PEs and CEs can use static route, an IGP, or BGP to exchange routing information. Which of the following operations can be used to prevent routing loops when PEs and CEs use OSPF to exchange routing information?

  - A sham link is deployed between the PE and CE.

  - The PE sets the VPN route tag when importing routes to the OSPF process of the VPN instance.

  - The PE sets the DN when importing routes to the OSPF process of the VPN instance.

  - A routing policy is used to filter out the routes that may cause loops.

- (Single) Which of the following routes can trigger the establishment of an LDP LSP by default?

  - 192.168.1.1/32

  - 192.168.1.0/30

  - 192.168.1.0/24

  - 192.168.0.0/16

- (TorF) Network maintenance is classified into routine maintenance and troubleshooting.

- (Multiple) Which of the following fault factors need to be confirmed in the fault confirmation phase during network fault troubleshooting?

  - Fault entity

  - Fault symptom

  - Fault occurrence time

  - Fault location

- (TorF) In the network migration process, the rollback solution is required only when the service test fails after a network migration is performed.

Answer:

1-5：T、T、F、B、D

6-10：D、B、A、C、C

11-15：ABCD、ACD、AC、AD、ABCD

16-20：C、A、ABCD、B、CD

21-:25：CD、T、A、D、A

26-30：BCD、A、T、ABCD、F