Huawei Certification Training

HCIP-Datacom-Campus Network Planning and Deployment

Data Communication Senior Engineer

Lab Guide

ISSUE:1.0

HUAWEI TECHNOLOGIES CO., LTD.

Copyright © Huawei Technologies Co., Ltd. 2021. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | http://e.huawei.com |

Huawei Certificate System

Huawei Certification is an integral part of the company's "Platform + Ecosystem" strategy, and it supports the ICT infrastructure featuring "Cloud-Pipe-Device". It evolves to reflect the latest trends of ICT development. Huawei Certification consists of two categories: ICT Infrastructure Certification, and Cloud Service & Platform Certification, making it the most extensive technical certification program in the industry.

Huawei offers three levels of certification: Huawei Certified ICT Associate (HCIA), Huawei Certified ICT Professional (HCIP), and Huawei Certified ICT Expert (HCIE).

With its leading talent development system and certification standards, Huawei is committed to developing ICT professionals in the digital era, building a healthy ICT talent ecosystem.

Huawei offers three levels of certification: Huawei Certified ICT Associate (HCIA), Huawei Certified ICT Professional (HCIP), and Huawei Certified ICT Expert (HCIE). Huawei Certification covers all ICT fields and adapts to the industry trend of ICT convergence. With its leading talent development system and certification standards, it is committed to fostering new ICT talent in the digital era, and building a sound ICT talent ecosystem.

HCIP-Datacom-Campus Network Planning and Deployment (HCIP-Datacom-Campus) is designed for Huawei's frontline engineers and anyone who want to understand Huawei's datacom products and technologies. HCIP-Datacom-Campus Network Planning and Deployment certification covers traditional campus network technologies, Huawei CloudCampus solution, network access control, free mobility, VXLAN, BGP EVPN, campus network virtualization, campus WLAN delivery and acceptance, and CampusInsight intelligent O&M. such as knowledge. You can be qualified as a network engineer (account manager, project manager, pre-sales engineer, post-sales engineer, and O&M engineer) in campus scenarios in the datacom field. You can use Huawei datacom products to perform campus network planning, automatic network deployment, and O&M.

The Huawei certification system introduces the industry, fosters innovation, and imparts cutting-edge datacom knowledge.

About This Document

Overview

This document is an HCIP-Datacom-Campus Network Planning and Deployment certification training course.

This document is intended for trainees who are going to take the HCIP-Datacom-Campus Network Planning and Deployment exams. It is also targeting learners who want to study traditional campus network technologies, Huawei's CloudCampus Solution, network admission control, free mobility, VXLAN, BGP EVPN, campus network virtualization technologies, and campus WLAN delivery and acceptance, as well as CampusInsight-based intelligent O&M.

Description

This document guides you through five labs, namely, three technology labs and two solution labs as follows:

Network admission control technology

VXLAN technology

Campus multi-branch interconnection technology

VXLAN-based virtualized campus network deployment solution

Small- and medium-sized campus network deployment solution

Background Knowledge Required

This course is for Huawei's advanced certification. To better understand this course, familiarize yourself with the following requirements:

Have basic computer skills.

Be familiar with the principles of the TCP/IP protocol stack.

Be familiar with the basic working principles of Ethernet switches and routers.

Knowledge and skills described in the HCIP-Datacom-Core Technology course

Symbol Conventions

Lab Environment

Networking

This lab environment is intended for datacom network engineers preparing for the HCIP-Datacom-Campus Network Planning and Deployment exam. Each lab environment consists of three switches (not supporting PoE), two PoE switches, two APs, two routers, and several servers.

Device Introduction

To meet the requirements of the HCIP-Datacom-Campus Network Planning and Deployment experiment, you are advised to use the following configurations for each lab environment:

The mapping between device names, models, and versions is as follows:

| Device | Model | Software version |
| --- | --- | --- |
| Switch | CloudEngine S5731-H24T4XC | V200R019C00 or later |
| Switch（PoE） | CloudEngine S5731-H24P4XC | V200R019C00 or later |
| AP | AirEngine 5760-51 | V200R019C00 or later |
| AR | NetEngine AR6120 | V300R019 or later |

Note: The port information, display information, and configuration information of all devices in this document are provided based on the devices in the recommended topology. The information may vary according to lab environments.

Pre-configuration

For the overall topology of this lab, refer the 《HCIP-Datacom-Campus Network Planning and Deployment V1.0 Lab Setup Guide》.

In this experiment, SW1 functions as the gateway of the iMaster NCE server and the gateway of AR1 and AR2. SW1 provides the following functions:

- Enable AR1 and AR2 to communicate with the iMaster NCE server.

- Delivers the IP address of the iMaster NCE controller to the device through DHCP Option 148.

Perform the following pre-configuration on SW1:

- Create VLAN 67, assign the interfaces connected to AR1 and AR2 to VLAN 67, and create VLANIF 67.

<Huawei>system-view

[Huawei]sysname SW1

[SW1]vlan 67

[SW1]interface GigabitEthernet 0/0/17

[SW1-GigabitEthernet0/0/17]port link-type access

[SW1-GigabitEthernet0/0/17]port default vlan 67

[SW1-GigabitEthernet0/0/17]description to_AR1

[SW1-GigabitEthernet0/0/17]quit

[SW1]interface GigabitEthernet 0/0/18

[SW1-GigabitEthernet0/0/18]port link-type access

[SW1-GigabitEthernet0/0/18]port default vlan 67

[SW1-GigabitEthernet0/0/18]description to_AR2

[SW1-GigabitEthernet0/0/18]quit

[SW1]interface Vlanif 67

[SW1-Vlanif67]ip address 67.0.0.254 24

[SW1-Vlanif67]description AR_Gateway

- Create VLAN 40, assign the interface connected to the iMaster NCE server to VLAN 40, and create VLANIF 40.

[SW1]vlan 40

[SW1]interface GigabitEthernet 0/0/20

[SW1-GigabitEthernet0/0/20]port link-type access

[SW1-GigabitEthernet0/0/20]port default vlan 40

[SW1-GigabitEthernet0/0/20]description to_iMaster NCE

[SW1-GigabitEthernet0/0/20]quit

[SW1]interface Vlanif 40

[SW1-Vlanif40]ip address 192.168.4.254 24

[SW1-Vlanif40]description iMaster_NCE_Gateway

Note that the number of servers used varies depending on the iMaster NCE deployment mode. Ensure that the server is connected to SW1 and the gateway is set to the IP address of VLANIF40 on SW1.

- Enable the DHCP service, enable interface-based address allocation, assign fixed IP addresses to AR1 and AR2 interfaces, and configure DHCP Option 148.

[SW1]dhcp enable

[SW1]interface Vlanif 67

[SW1-Vlanif67] dhcp select interface

[SW1-Vlanif67] dhcp server static-bind ip-address 67.0.0.2 mac-address c8a7-7600-3f63

[SW1-Vlanif67] dhcp server static-bind ip-address 67.0.0.1 mac-address cc64-a651-7f13

[SW1-Vlanif67] dhcp server option 148 ascii agilemode=agile-cloud;agilemanage-

mode=ip;agilemanage-domain=192.168.4.104;agilemanage-port=10020;

To ensure that the IP addresses of AR1 and AR2 interfaces are fixed, statically bound addresses are used. The actual MAC addresses of AR interfaces vary.

Configure DHCP Option 148 to notify iMaster NCE of the IP address and port number. The actual IP address and port number may vary according to the actual situation.

Content

About This Document	3

Overview	3

Description	3

Background Knowledge Required	3

Symbol Conventions	4

Lab Environment	4

1 Network Admission Control	10

1.1 Introduction	10

1.1.1 About This Lab	10

1.1.2 Objectives	10

1.1.3 Networking Topology	10

1.2 Lab Configuration	11

1.2.1 Configuration Roadmap	11

1.2.2 Configuration Procedure	11

1.3 Configuration Reference	29

1.3.1 Configurations of AR2	29

1.4 Quiz	30

2 VXLAN and EVPN Configuration	31

2.1 Layer 2 Communication Through a Static VXLAN Tunnel	31

2.1.1 Introduction	31

2.1.2 Lab Configuration	33

2.1.3 Configuration Reference	40

2.1.4 Quiz	43

2.2 Centralized VXLAN Gateway	44

2.2.1 Introduction	44

2.2.2 Lab Configuration	45

2.2.3 Configuration Reference	54

2.2.4 Quiz	57

2.3 Distributed VXLAN Gateway	58

2.3.1 Introduction	58

2.3.2 Lab Configuration	59

2.3.3 Configuration Reference	79

2.3.4 Quiz	83

3 Campus Multi-Branch Interconnection Technology	84

3.1 Introduction	84

3.1.1 About This Lab	84

3.1.2 Objectives	84

3.1.3 Networking Topology	84

3.2 Lab Configuration	85

3.2.1 Configuration Roadmap	85

3.2.2 Configuration Procedure	85

3.3 Configuration Reference	90

3.3.1 Configurations of AR1	90

3.3.2 Configuration of AR2	91

3.4 Quiz	92

4 VXLAN-based Campus Network Deployment	1

4.1 Introduction	1

4.1.1 About This Lab	1

4.1.2 Objectives	1

4.1.3 Networking Topology	2

4.1.4 Data Plan	2

4.2 Lab Configuration	16

4.2.1 Configuration Roadmap	16

4.2.2 Preconfiguring the Environment	16

4.2.3 (Optional) Configuration Deletion	20

4.2.4 Creating a Site and Onboarding Devices	20

4.2.5 Fabric and Underlay Network Automation	24

4.2.6 Overlay Network Automation	49

4.2.7 Free Mobility and Access Authentication	60

4.2.8 WLAN Services	75

4.3 Verification	81

4.3.1 Verifying Access Authentication	81

4.3.2 Verifying Network Connectivity	88

4.4 Configuration Deletion	89

4.4.1 Delete Authentication Rules, Authorization Rules, and Authorization Results	90

4.4.2 Forcibly Log Out Users and Delete the Users	90

4.4.3 Delete the Policy Control Matrices and Security Groups	90

4.4.4 Delete VN Interworking Policies and VNs	91

4.4.5 Delete Access Management Configurations	91

4.4.6 Delete Nerwork Service Resources and External Netwrok	91

4.4.7 Delete the devices from the Fabric and Delete the Fabric	91

4.4.8 Delete Fabric Global Resource Pools and Underlay Automation Resource Pools	92

4.4.9 Delete Devices Managed by iMaster NCE-Campus	92

4.4.10 Delete Sites	92

4.4.11 Delete Authentication Templates and Server Templates	92

4.4.12 Delete Configurations on Switches	93

4.4.13 Delete Configurations on AR1	93

4.5 Samples of Configurations Automatically Delivered by iMaster NCE-Campus	94

4.5.1 Configurations of CORE1	94

4.5.2 Configurations of AGG1	98

4.5.3 Configurations of AGG2	99

4.5.4 Configurations of ACC1	101

4.5.5 Configurations of ACC2	106

4.6 Quiz	112

5 Small- and Medium-Sized Campus Network Deployment	113

5.1 Introduction	113

5.1.1 About This Lab	113

5.1.2 Objectives	113

5.1.3 Networking Topology	114

5.2 Lab Configuration	114

5.2.1 Configuration Roadmap	114

5.2.2 Configuration Procedure	115

5.3 Quiz	148

Reference Answers	149

Network Admission Control

Introduction

About This Lab

This lab guides you through how to configure MAC address authentication and 802.1X authentication to implement network admission control.

Objectives

Upon completion of this lab, you will be able to:

Configure 802.1X authentication through the CLI

Configure MAC address authentication through the CLI

Configure hybrid authentication through the CLI

Understand the RADIUS server authorization process by running the debugging commands.

Networking Topology

Network admission control topology

As shown in the preceding figure, the network consists of branch (Site2) and cloud.

This lab uses a part of the overall topology, where SW1 and the controller are configured as planned.

On SW1, configure the interface connected to AR2 as an access interface and communicate with AR2 through VLANIF 67.0.0.2. On SW1, the IP address of VLANIF 67 is 67.0.0.254.

The branch consists of an AR2 (AR6120) and a PC. AR2 functions as the access device and the PC functions as the authentication client.

AR2 of the branch network functions as an authentication access device. MAC address authentication, 802.1X authentication, and hybrid authentication are enabled on GE0/0/2 of the branch network.

In this experiment, the iMaster NCE-Campus controller functions as the authentication and authorization server to deliver attributes to authenticated users, including VLAN, ACL, and user group attributes. You can observe how the RADIUS server delivers these attributes through debugging.

Lab Configuration

Configuration Roadmap

- Complete connectivity configuration.

- Create a RADIUS server template, an AAA authentication scheme, and an authentication domain.

- Create a RADIUS authentication template and an authentication account on iMaster NCE-Campus.

- Configure 802.1X authentication.

- Observe the 802.1X authentication and authorization process.

- Configure MAC address authentication

- Configure hybrid authentication.

Configuration Procedure

Complete connectivity configuration.

In the pre-configuration,SW1 has configured VLANs, created VLANIF interfaces, and allocated DHCP addresses. In this case, you only need to enable DHCP client function on AR2 interfaces to obtain IP addresses.

#Device name

omit

# GE0/0/8 of AR2 automatically obtains an IP address through DHCP.

[AR2]interface GigabitEthernet0/0/8

[AR2-GigabitEthernet0/0/8]ip address dhcp-alloc

Create a RADIUS server template, an AAA authentication scheme, and an authentication domain.

Create a RADIUS server template named t1, and set the RADIUS server IP address to 192.168.4.104 and the pre-shared key (PSK) to Huawei@123.

Create an authentication scheme named site2, an accounting scheme named site2, and then an authentication domain named auth. After that, invoke the authentication scheme site2, accounting scheme site2, and RADIUS server template t1.

# Create and configure a RADIUS server template named t1.

[AR2]radius-server template t1

[AR2-radius-t1]radius-server shared-key cipher Huawei@123

[AR2-radius-t1]radius-server authentication 192.168.4.104 1812 source ip-address 67.0.0.2

[AR2-radius-t1]radius-server accounting 192.168.4.104 1813 source ip-address 67.0.0.2

[AR2-radius-t1]quit

Set the source IP address to 67.0.0.2, which is used to communicate with the RADIUS server. By default, the source IP address is the IP address of the outbound interface of packets.

# Create an AAA authentication scheme named site2, and set the authentication mode to RADIUS.

[AR2]aaa

[AR2-aaa]authentication-scheme site2

[AR2-aaa-authen-site2]authentication-mode radius

[AR2-aaa-authen-site2]quit

# Create an AAA accounting scheme named site2, and set the accounting mode to RADIUS.

[AR2-aaa]accounting-scheme site2

[AR2-aaa-accounting-site2]accounting-mode radius

[AR2-aaa-accounting-site2]accounting realtime 5

[AR2-aaa-accounting-site2]quit

Sets the accounting interval to 5 minutes.

# Create an authentication domain named auth.

[AR2-aaa]domain auth

[AR2-aaa-domain-auth]authentication-scheme site2

[AR2-aaa-domain-auth]accounting-scheme site2

[AR2-aaa-domain-auth]radius-server t1

[AR2-aaa-domain-auth]quit

Complete configuration on iMaster NCE-Campus

On iMaster NCE-Campus, add the authentication access device AR2 and create an authentication account, an authorization rule, and an authorization result.

# Add an authentication access device.

Choose Admission > Admission Resources > Admission Device to add an authentication access device (AR2 in this case). Set the parameters as follows:

- Set IP address to 67.0.0.2, which is the source IP address configured on AR2.

- Set Device Series to Huawei NAC.

- Enable RADIUS authentication parameter and set other parameters as follows:

  - Set Authentication and accounting key to Huawei@123.

  - Set Authorization key to Huawei@123.

  - Set Accounting interval (min) to 5.

The preceding parameter settings must be the same as those in the RADIUS server template on AR2.

# Create an 802.1X authentication user group.

Choose Admission > Admission Resources > User Management, click  on the left, and create a user group named 802.1x_User.

# Create an 802.1X authentication user.

Click the created user group 802.1x_User on the left, and then click Create on the right.

In the dialog box that is displayed, create a user named dot1x and set the password to Huawei@123. Then, disable Change password upon next login. Otherwise, the user needs to manually change the password after the first login.

# Test the AAA account on AR2.

<AR2>test-aaa dot1x Huawei@123 radius-template t1 chap

Info: Account test succeeded.

The created account can normally perform AAA authentication.

# Create an authorization result.

Choose Admission > Admission Policy > Authentication and Authorization. On the Authorization Result tab page, create an authorization result. Specifically, set Name to Site2_dot1x, and mainly authorize ACLs and VLANs, which can be achieved by setting Custom Authorization Parameters.

- ACL: The Filter-ID field is used for authorizing and delivering the ACL information. On the AR, the Filter-ID field represents an ACL or a user group. If an ACL number does not exist on the AR, the Filter-ID field will be parsed as a user group. If the user group does not exist on the AR, authorization fails.

- VLAN: On the AR, the following three attributes must be used to deliver VLAN information:

- Tunnel-Type: This attribute must be set to VLAN or 13.

- Tunnel-Medium-Type: This attribute must be set to 802 or 6.

- Tunnel-Private-Group-ID: This attribute can be the VLAN ID or VLAN description.

For the LAN switch, you can directly set VLAN on the page for authorizing and delivering it, without the need to set Custom Authorization Parameters. In this lab, VLAN 300 is authorized and delivered.

The authorized VLAN and ACL take effect only when they exist on the local device. Therefore, you need to manually create ACL 3005（at least containing a rule） and VLAN 300 on the device.

# Create an authorization rule.

Create an authorization rule, enable account matching, match the user account named dot1x, and set the authorization result to Site2_dot1x.

Notice：Set the access mode to wired.

Configure 802.1X authentication.

Create an 802.1X access profile named d1, configure an authentication profile named p1, bind the access profile d1 to the authentication profile p1, and configure a forcible authentication domain named auth for users using this authentication profile.

# Create an 802.1X access profile named d1.

[AR2]dot1x-access-profile name d1

[AR2-dot1x-access-profile-d1]quit

# Create an authentication profile named p1.

[AR2]authentication-profile name p1

Authentication profile p1 : done.

[AR2-authen-profile-p1]dot1x-access-profile d1

[AR2-authen-profile-p1]access-domain auth force

[AR2-authen-profile-p1]quit

# Bind the authentication profile p1 to GE0/0/2.

[AR2]interface GigabitEthernet 0/0/2

[AR2-GigabitEthernet0/0/2]authentication-profile p1

Verify 802.1X authentication.

Enable 802.1X authentication on the network adapter of the terminal and enter the 802.1X authentication account and password.

# Choose Control Panel > Network and Internet > Network Connections, find the wired network adapter, and modify its properties.

# On the Authentication tab page, click Additional Settings. In the dialog box that is displayed, select User or computer authentication. In the smaller dialog box that is displayed, click Save credentials.

In the dialog box that is displayed, enter the user name and password, and click OK.

# On AR2, check information about online users.

[AR2]display access-user username dot1x detail

------------------------------------------------------------------------------

Basic:

User ID                           : 1136

User name                        : dot1x

Domain-name                     : auth

User MAC                         : 286e-d489-d3d8

User IP address                     : 169.254.122.57

User vpn-instance                  : -

User IPv6 address                   : -

User access Interface               : GigabitEthernet0/0/2

User vlan event                     : Success

QinQVlan/UserVlan                 : 0/300

User vlan source                    : server vlan

User access time                   : 2020/12/03 12:10:43

User accounting session ID          : AR200002000000001d7****0000470

Option82 information               : -

User access type                    : 802.1x

Terminal Device Type               : Data Terminal

Dynamic VLAN ID                  : 300

Dynamic ACL ID(Effective)           : 3005

AAA:

User authentication type         : 802.1x authentication

Current authentication method   : RADIUS

Current authorization method    : -

Current accounting method       : RADIUS

------------------------------------------------------------------------------

Total: 1, printed: 1

The command output shows that the user has gone online successfully, and that the configured VLAN and ACL have been successfully authorized to the user.

# On iMaster NCE-Campus, check authentication and authorization logs.

Choose Monitoring > Event Logs > Terminal Authentication Logs and then filter logs. Specifically, set Access Site to Not in Any Sites (in this lab, we manually add AR2 as the authentication access device, which is not managed by iMaster NCE-Campus), and specify Time. After such settings, you can filter logs.

# Check log details.

Click For Details. In the dialog box that is displayed, you can see the user's authentication domain (auth) information is carried by the RADIUS attribute Huawei: HW-Domain-Name of the authentication access device (AR2).

In Authorization Result, you can also see the details of authorized contents.

RadiusPacketType=AccessAccept

Tunnel-Private-Group-ID=300;

Tunnel-Medium-Type=6;

Tunnel-Type=13;

Filter-Id=3005;

State=0x01762708b44e000001762708b44e

The details show that iMaster NCE-Campus has authorized the manually configured RADIUS attributes, and these attributes are identified and accepted by AR2.

In this lab, AR2 is used as an example for access authentication. In actual scenarios, if a switch is used as an authentication access device, we will use a different way to authorize VLAN, ACL, and UCL group information.

Return to the Authorization Result page.

You can configure authorization information (including ACL, security group, and VLAN information) for switches managed by iMaster NCE-Campus on the web page.

User login logs on a device managed by iMaster NCE-Campus are used as an example. If you check the authorization modes of devices, you will find that devices vary with the authorization modes.

# Display user login logs at a site managed by iMaster NCE-Campus.

RadiusPacketType=AccessAccept Huawei:HW-User-Policy=Guest_Result; Tunnel-Type=13; Tunnel-Medium-Type=6; Tunnel-Private-Group-ID=101; Huawei:HW-UCL-Group=1

The log shows that the VLAN is authorized in the same way as manual VLAN authorization, both of which authorize three RADIUS attributes Tunnel-Type, Tunnel-Medium-Type, and Tunnel-Private-Group-ID. Besides, another two attributes, namely, Huawei:HW-UCL-Group=1 and Huawei:HW-User-Policy=Guest_Result, are also authorized. The two additional attributes correspond to the UCL group and service scheme on the device.

# Run the debug radius packet command to check detailed information about the packets sent from the RADIUS server to AR2.

<AR2>debugging radius packet receive detail

<AR2>terminal debugging

Info: Current terminal debugging is on.

<AR2>terminal monitor

Info: Current terminal monitor is on.

# Disable and then enable the network adapter of the PC to trigger 802.1X authentication again. Then, check the debugging command output.

Server Template: 1

Server IP   : 192.168.4.104

Server Port : 1812

Protocol: Standard

Code    : 2

Len     : 83

ID      : 16

[Tunnel-Private-Group-ID    									 ] [5 ] [300]

[Tunnel-Medium-Type         							 ] [6 ] [6]

[Tunnel-Type            													  ] [6 ] [13]

[Filter-Id                     		     ] [6 ] [3005]

[EAP-Message                      	] [6 ] [03 d8 00 04 ]

[State                              ] [16] [\001v'\202\315$]

[Message-Authenticator            	 	] [18] [00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 ]

The command output shows that the RADIUS server (namely, iMaster NCE-Campus at 192.168.4.104) sends a RADIUS packet carrying Tunnel-Private-Group-ID, Tunnel-Medium-Type, Tunnel-Type, and Filter-Id attributes to AR2. Based on such attributes, AR2 obtains authorization information.

Configure MAC address authentication.

On AR2, create a MAC access profile named m1 and an authentication profile named p2, bind the access profile m1 to the authentication profile p2, and bind the authentication profile p2 to an interface.

# Create a MAC access profile named m1.

[AR2]mac-access-profile name m1

[AR2-mac-access-profile-m1]mac-authen username macaddress format with-hyphen

[AR2-mac-access-profile-m1]quit

Change the format of the user name for MAC address authentication. By default, both the user name and password for MAC address authentication are MAC addresses without hyphens (-). Here, you should manually change them to MAC addresses with hyphens (-).

# Create an authentication profile named p2.

[AR2]authentication-profile name p2

[AR2-authen-profile-p2] mac-access-profile m1

[AR2-authen-profile-p2] access-domain auth force

[AR2-authen-profile-p2] quit

Bind the access profile m1 to it and specify a forcible authentication domain named auth.

# On GE0/0/2, change the bound authentication profile p2.

[AR2]interface GigabitEthernet 0/0/2

[AR2-GigabitEthernet0/0/2]authentication-profile p2

Warning: Changing the authentication profile will cause online users to go offline. Continue? [Y/N]y

In this case, the configuration on the interface is overwritten, and 802.1X authentication is then disabled on the terminal.

# Check the MAC address of the terminal.

[C:\~]$ ipconfig /all

Windows IP Configuration

Host Name . . . . . . . . . . . . : DESKTOP-R5G9137

Primary Dns Suffix  . . . . . . . :

Node Type . . . . . . . . . . . . : Hybrid

IP Routing Enabled. . . . . . . . : No

WINS Proxy Enabled. . . . . . . . : No

Ethernet adapter Ethernet:

Connection-specific DNS Suffix  . :

Description . . . . . . . . . . . : Red Hat VirtIO Ethernet Adapter

Physical Address. . . . . . . . . : 28-6E-D4-89-D3-D8

DHCP Enabled. . . . . . . . . . . : No

Autoconfiguration Enabled . . . . : Yes

Link-local IPv6 Address . . . . . : fe80::8d29:6c4a:310d:7062%8(Preferred)

IPv4 Address. . . . . . . . . . . : 169.254.122.57(Preferred)

Subnet Mask . . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . . :

DHCPv6 IAID . . . . . . . . . . . : 606629588

DHCPv6 Client DUID. . . . . . . . : 00-01-00-01-27-51-41-CD-50-2B-73-D9-6A-9A

DNS Servers . . . . . . . . . . . : fec0:0:0:ffff::1%1

fec0:0:0:ffff::2%1

fec0:0:0:ffff::3%1

NetBIOS over Tcpip. . . . . . . . : Enabled

In the cmd window, run the ipconfig /all command to check the MAC address of the network adapter. The obtained MAC address is 28-6E-D4-89-D3-D8.

# On iMaster NCE-Campus, create a MAC authentication account.

Choose Admission > Admission Resources > User Management. Click MAC Account, click Create, set MAC Account Name to Site2, and add the MAC address (28-6E-D4-89-D3-D8) of the PC to MAC address list.

# Configure an authorization rule.

Choose Admission > Admission Policy > Authentication and Authorization and click Authorization Rules to create an authorization rule. Set Name to Site2_MAC and configure the rule as follows:

- Authentication Mode: Select MAC address authentication.

- Access Mode: Select Wired.

- Enable Account matching and select the created MAC account (Site2).

- In Authentication result, use the configured authorization result Site2_dot1x.

# On AR2, check the authentication result.

[AR2]display access-user domain auth

------------------------------------------------------------------------------------------------------

UserID  Username               IP address                  MAC            Status

------------------------------------------------------------------------------------------------------

1160    286e-d489-d3d8         169.254.122.57            286e-d489-d3d8   Success

------------------------------------------------------------------------------------------------------

Total: 1, printed: 1

The command output shows that the user named 286e-d489-d3d8 has been successfully authenticated.

# Check detailed user information.

[AR2]display access-user mac-address 286e-d489-d3d8

Basic:

User ID                           : 1160

User name                        : 286e-d489-d3d8

Domain-name                     : auth

User MAC                         : 286e-d489-d3d8

User IP address                    : 169.254.122.57

User vpn-instance                  : -

User IPv6 address                  : -

User access Interface               : GigabitEthernet0/0/2

User vlan event                    : Success

QinQVlan/UserVlan                : 0/1

User vlan source                   : user request

User access time                   : 2020/12/03 14:40:14

User accounting session ID          : AR20000200000000105****0000488

Option82 information              : -

User access type                   : MAC

Terminal Device Type               : Data Terminal

AAA:

User authentication type           : MAC authentication

Current authentication method     : RADIUS

Current authorization method      : -

Current accounting method        : RADIUS

The command output shows that user authentication is successful, but the authorized VLAN is not found.

# Check RADIUS logs.

The logs show that MAC address authentication matches the default authentication rule but does not match the created authorization rule Site2-MAC.

# On AR2, disconnect the user.

[AR2]aaa

[AR2-aaa]cut access-user user-id 1160

Info: Total 1  user(s) being cut.

Forcibly log out the MAC address authentication account based on the queried user ID.

# View detailed information about the access user again.

<AR2>display access-user mac-address 286e-d489-d3d8

Basic:

User ID                           : 1161

User name                        : 286e-d489-d3d8

Domain-name                     : auth

User MAC                         : 286e-d489-d3d8

User IP address                    : 169.254.122.57

User vpn-instance                  : -

User IPv6 address                  : -

User access Interface               : GigabitEthernet0/0/2

User vlan event                    : Success

QinQVlan/UserVlan                : 0/300

User vlan source                   : server vlan

User access time                   : 2020/12/03 15:01:31

User accounting session ID          : AR2000020000000015d****0000489

Option82 information              : -

User access type                   : MAC

Terminal Device Type               : Data Terminal

Dynamic VLAN ID                  : 300

Dynamic ACL ID(Effective)          : 3005

AAA:

User authentication type           : MAC authentication

Current authentication method     : RADIUS

Current authorization method      : -

Current accounting method        : RADIUS

MAC address authentication is performed again, and you can see that the authorization information delivered by the RADIUS server is successfully obtained.

Configure hybrid authentication.

Create an authentication profile named p3, bind the MAC access profile named m1 and 802.1X access profile named d1 to the authentication profile, and bind the authentication profile p3 to GE0/0/2 on AR2.

# Create an authentication profile named p3.

[AR2]authentication-profile name p3

[AR2-authen-profile-p3] dot1x-access-profile d1

[AR2-authen-profile-p3] mac-access-profile m1

[AR2-authen-profile-p3] access-domain auth force

[AR2-authen-profile-p3] authentication dot1x-mac-bypass

[AR2-authen-profile-p3] quit

Bind the MAC access profile and 802.1X access profile to the authentication profile, specify a forcible authentication domain named auth, and enable MAC address bypass authentication.

By default, access profiles can be bound to the authentication profile in any sequence. The device triggers the corresponding authentication based on received authentication packets.

You can configure MAC address bypass authentication to authenticate terminals such as printers that cannot have the 802.1X client installed. After that, the device preferentially performs 802.1X authentication for users. If the user name request fails, the device initiates MAC address authentication for users.

# Bind the authentication profile named p3 to GE0/0/2.

[AR2]interface GigabitEthernet 0/0/2

[AR2-GigabitEthernet0/0/2]authentication-profile p3

# View access user information.

[AR2]display  access-user

------------------------------------------------------------------------------------------------------

UserID  Username               IP address   MAC            Status

------------------------------------------------------------------------------------------------------

4       admin                   -           -                Success

1162                            -         286e-d489-d3d8   Pre-authen

------------------------------------------------------------------------------------------------------

Total: 2, printed: 2

[AR2]display  access-user

Dec  3 2020 15:16:22+08:00 AR2 MSTP/4/PFWD:OID 1.3.6.1.4.1.2011.5.25.42.4.2.1 The port has been set to forwarding state. (InstanceID=0, PortInstanceID=0, PortID=3, IfIndex=5, PortName=GigabitEthernet0/0/2)

[AR2]display access-user

Dec  3 2020 15:16:22+08:00 AR2 %%01MSTP/4/SET_PORT_FORWARDING(l)[255]:In MSTP process 0 instance 0, MSTP set port GigabitEthernet0/0/2 state as  forwarding.

[AR2]display  access-user

------------------------------------------------------------------------------------------------------

UserID  Username               IP address   MAC            Status

------------------------------------------------------------------------------------------------------

4       admin                   -            -                Success

1162    286e-d489-d3d8         -          286e-d489-d3d8   Success

------------------------------------------------------------------------------------------------------

The command output shows that the user with the MAC address being 286e-d489-d3d8 is still in Pre-authen state. In this case, the device preferentially performs 802.1X authentication. However, 802.1X authentication is not enabled on the PC. Therefore, after the device sends EAP packets multiple times and receives no response, the device then performs MAC address authentication.

Then, check access user information again. It is found that the user is in the Success state.

# Enable 802.1X authentication on the PC and disconnect the user on AR2.

[AR2]aaa

[AR2-aaa]cut access-user user-id 1162

Info: Total 1  user(s) being cut.

# Check user access information again.

[AR2]display access-user

------------------------------------------------------------------------------------------------------

UserID  Username               IP address    MAC               Status

------------------------------------------------------------------------------------------------------

4       admin                    -             -                 Success

1164    dot1x                     -            286e-d489-d3d8    Success

------------------------------------------------------------------------------------------------------

Total: 2, printed: 2

You can see that the PC successfully goes online using the 802.1X authentication account dot1x.

----End

Configuration Reference

Configurations of AR2

<AR2>display current-configuration

[V300R019C10SPC300]

#

sysname AR2

#

vlan 300

#

authentication-profile name default_authen_profile

authentication-profile name dot1x_authen_profile

authentication-profile name dot1xmac_authen_profile

authentication-profile name mac_authen_profile

authentication-profile name multi_authen_profile

authentication-profile name p1

dot1x-access-profile d1

access-domain auth force

authentication-profile name p2

mac-access-profile m1

access-domain auth force

authentication-profile name p3

dot1x-access-profile d1

mac-access-profile m1

access-domain auth force

authentication dot1x-mac-bypass

authentication-profile name portal_authen_profile

#

domain default admin

#

radius-server template default

radius-serrvertemplate t1

radius-server shared-key cipher %^%#|3K=6E=T#5HY)9%0b\iW@HjwHH"SNA3`8-/-e=GY%^%#

radius-server authentication 192.168.4.104 1812 source ip-address 67.0.0.2 weight 80

radius-server accounting 192.168.4.104 1813 source ip-address 67.0.0.2 weight 80

#

acl number 3005

rule 1 permit ip

#

aaa

authentication-scheme default

authentication-mode local

authentication-scheme radius

authentication-mode radius

authentication-scheme site2

authentication-mode radius

authorization-scheme default

authorization-mode local

accounting-scheme default

accounting-mode none

accounting-scheme site2

accounting-mode radius

accounting realtime 5

local-aaa-user password policy administrator

domain auth

authentication-scheme site2

accounting-scheme site2

radius-server t1

domain default

authentication-scheme default

accounting-scheme default

domain default_admin

authentication-scheme default

accounting-scheme default

local-user admin password irreversible-cipher $1a$=H4v-:mGk=$TYo!'m<U]NB7TN4/'o1F256:9H;v~!s0z!*V`D#2$

local-user admin privilege level 15

local-user admin service-type terminal ssh http

#

interface GigabitEthernet0/0/2

authentication-profile p3

#

interface GigabitEthernet0/0/8

ip address 67.0.0.2 255.255.255.0

#

ip route-static 0.0.0.0 0.0.0.0 67.0.0.254

#

dot1x-access-profile name d1

dot1x-access-profile name dot1x_access_profile

#

mac-access-profile name m1

mac-authen username macaddress format with-hyphen

mac-access-profile name mac_access_profile

#

Return

Quiz

Which terminals are recommended to use MAC address authentication for authentication and authorization?

VXLAN and EVPN Configuration

Layer 2 Communication Through a Static VXLAN Tunnel

Introduction

About This Lab

This lab guides you through how to configure a static VXLAN tunnel and enable terminals on the same network segment to communicate with each other over the VXLAN network.

Objectives

Upon completion of this lab, you will be able to:

Create a static VXLAN tunnel through the CLI.

Implement Layer 2 communication through a VXLAN tunnel.

Implement VXLAN service access through VLAN binding.

Networking Topology

Topology of Layer 2 communication through a static VXLAN tunnel

Loopback0 interfaces are created on SW1, SW2, and SW3, and their IP addresses are in the format of 10.0.x.x, where x indicates the device ID.

A trunk link is established between SW1 and SW2, which allows packets from VLAN 12 to pass through. VLANIF 12 is created on SW1 and SW2 for Layer 3 communication, and its IP address is set to 10.0.12.x/24, where x indicates the device ID.

A trunk link is established between SW2 and SW3, which allows packets from VLAN 23 to pass through. VLANIF 23 is created on SW2 and SW3 for Layer 3 communication, and its IP address is set to 10.0.23.x/24, where x indicates the device ID.

OSPF runs on SW1, SW2, and SW3, the IP address of Loopback0 is used as the router ID, and OSPF is enabled on loopback and interconnection interfaces.

SW1 and SW3 use the IP address of Loopback0 as the source IP address of the VXLAN NVE interface to establish a static VXLAN tunnel.

ACC-1 and ACC-2 function as access switches and connect to PC1 and PC2 through access interfaces. SW1 and SW3 connect to ACC-1 and ACC-2 through Layer 2 sub-interfaces, allow packets from VLAN 100 to pass through, and are bound with BD 20. Doing so allows packets from terminals to access a VXLAN network through Layer 2 sub-interfaces.

Notice

The three experiments in VXLAN and EVPN use some devices in the HCIP-Datacom-Campus lab environment. To facilitate device identification, the names of the three switches in the VXLAN domain are changed to SW1, SW2, and SW3 instead of CORE1, AGG1, and AGG2, be careful not to be confused with SW1 in other labs,they are not the same SW1.

The details are not described in the following documents: Centralized VXLAN Gateway and Distributed VXLAN Gateway.

Lab Configuration

Configuration Roadmap

- Complete connectivity configuration.

- Configure a service access point.

- Configure a static VXLAN tunnel.

Configuration Procedure

Complete connectivity configuration.

Configure interconnection interfaces and OSPF on SW1, SW2, and SW3.

# Name the devices.

The configuration details are not provided.

# On SW1, configure VLANs and interfaces.

[SW1]vlan batch 12 100

[SW1]interface GigabitEthernet0/0/1

[SW1-GigabitEthernet0/0/1] port link-type trunk

[SW1-GigabitEthernet0/0/1] port trunk allow-pass vlan 12

[SW1-GigabitEthernet0/0/1] quit

[SW1]interface Vlanif12

[SW1-Vlanif12] ip address 10.0.12.1 255.255.255.0

[SW1-Vlanif12] quit

[SW1]interface LoopBack 0

[SW1-LoopBack0] ip address 10.0.1.1 32

[SW1-LoopBack0] quit

# On SW2, configure VLANs and interfaces.

[SW2]vlan batch 12 23

[SW2]interface GigabitEthernet0/0/2

[SW2-GigabitEthernet0/0/2] port link-type trunk

[SW2-GigabitEthernet0/0/2] port trunk allow-pass vlan 12

[SW2-GigabitEthernet0/0/2] quit

[SW2]interface GigabitEthernet0/0/3

[SW2-GigabitEthernet0/0/3] port link-type trunk

[SW2-GigabitEthernet0/0/3] port trunk allow-pass vlan 23

[SW2-GigabitEthernet0/0/3] quit

[SW2]interface Vlanif12

[SW2-Vlanif12] ip address 10.0.12.2 255.255.255.0

[SW2-Vlanif12] quit

[SW2]interface Vlanif23

[SW2-Vlanif23] ip address 10.0.23.2 255.255.255.0

[SW2-Vlanif23] quit

[SW2]interface LoopBack 0

[SW2-LoopBack0] ip address 10.0.2.2 32

[SW2-LoopBack0] quit

# On SW3, configure VLANs and interfaces.

[SW3]vlan batch 23 100

[SW3]interface GigabitEthernet0/0/1

[SW3-GigabitEthernet0/0/1] port link-type trunk

[SW3-GigabitEthernet0/0/1] port trunk allow-pass vlan 23

[SW3-GigabitEthernet0/0/1] quit

[SW3]interface Vlanif 23

[SW3-Vlanif23] ip address 10.0.23.3 255.255.255.0

[SW3-Vlanif23] quit

[SW3]interface LoopBack 0

[SW3-LoopBack0] ip address 10.0.3.3 32

[SW3-LoopBack0] quit

# Test the connectivity of interconnection interfaces.

<SW2>ping 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=50 ms

Reply from 10.0.12.1: bytes=56 Sequence=2 ttl=255 time=40 ms

Reply from 10.0.12.1: bytes=56 Sequence=3 ttl=255 time=50 ms

Reply from 10.0.12.1: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.12.1: bytes=56 Sequence=5 ttl=255 time=30 ms

--- 10.0.12.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/42/50 ms

<SW2>ping 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=60 ms

Reply from 10.0.23.3: bytes=56 Sequence=2 ttl=255 time=30 ms

Reply from 10.0.23.3: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.23.3: bytes=56 Sequence=4 ttl=255 time=30 ms

Reply from 10.0.23.3: bytes=56 Sequence=5 ttl=255 time=50 ms

--- 10.0.23.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/42/60 ms

On SW2, test the connectivity with VLANIF interfaces on SW1 and SW3.

# On SW1, configure OSPF.

[SW1]ospf 1 router-id 10.0.1.1

[SW1-ospf-1] area 0.0.0.0

[SW1-ospf-1-area-0.0.0.0] network 10.0.1.1 0.0.0.0

[SW1-ospf-1-area-0.0.0.0] network 10.0.12.0 0.0.0.255

[SW1-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0 and VLANIF 12.

# On SW2, configure OSPF.

[SW2]ospf 1 router-id 10.0.2.2

[SW2-ospf-1] area 0.0.0.0

[SW2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[SW2-ospf-1-area-0.0.0.0] network 10.0.12.0 0.0.0.255

[SW2-ospf-1-area-0.0.0.0] network 10.0.23.0 0.0.0.255

[SW2-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0, VLANIF 12, and VLANIF 23.

# On SW3, configure OSPF.

[SW3]ospf 1 router-id 10.0.3.3

[SW3-ospf-1] area 0.0.0.0

[SW3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[SW3-ospf-1-area-0.0.0.0] network 10.0.23.0 0.0.0.255

[SW3-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0 and VLANIF 23.

# On SW2, check the OSPF neighbor relationship and OSPF routing table.

<SW2>display ospf peer

OSPF Process 1 with Router ID 10.0.2.2

Neighbors

Area 0.0.0.0 interface 10.0.23.2(Vlanif23)'s neighbors

Router ID: 10.0.3.3         Address: 10.0.23.3

State: Full  Mode:Nbr is  Master  Priority: 1

DR: 10.0.23.3  BDR: 10.0.23.2  MTU: 0

Dead timer due in 32  sec

Retrans timer interval: 4

Neighbor is up for 00:04:52

Authentication Sequence: [ 0 ]

Neighbors

Area 0.0.0.0 interface 10.0.12.2(Vlanif12)'s neighbors

Router ID: 10.0.1.1         Address: 10.0.12.1

State: Full  Mode:Nbr is  Slave  Priority: 1

DR: 10.0.12.1  BDR: 10.0.12.2  MTU: 0

Dead timer due in 39  sec

Retrans timer interval: 6

Neighbor is up for 00:01:39

Authentication Sequence: [ 0 ]

OSPF neighbor relationships have been established between SW1 and SW2 as well as between SW3 and SW2.

<SW2>display ospf routing

OSPF Process 1 with Router ID 10.0.2.2

Routing Tables

Routing for Network

Destination        Cost   Type      NextHop       AdvRouter       Area

10.0.2.2/32         0     Stub       10.0.2.2        10.0.2.2        0.0.0.0

10.0.12.0/24       	1     Transit     10.0.12.2       10.0.2.2        0.0.0.0

10.0.23.0/24       	1     Transit     10.0.23.2       10.0.2.2        0.0.0.0

10.0.1.1/32         1     Stub       10.0.12.1       10.0.1.1        0.0.0.0

10.0.3.3/32        	1     Stub       10.0.23.3       10.0.3.3        0.0.0.0

Total Nets: 5

Intra Area: 5  Inter Area: 0  ASE: 0  NSSA: 0

SW2 has learned the routes generated by Loopback0 interfaces on SW1 and SW3.

# Test the connectivity of Loopback0 interfaces between SW1 and SW3.

<SW1>ping -a 10.0.1.1 10.0.3.3

PING 10.0.3.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.3.3: bytes=56 Sequence=1 ttl=254 time=36 ms

Reply from 10.0.3.3: bytes=56 Sequence=2 ttl=254 time=34 ms

Reply from 10.0.3.3: bytes=56 Sequence=3 ttl=254 time=37 ms

Reply from 10.0.3.3: bytes=56 Sequence=4 ttl=254 time=38 ms

Reply from 10.0.3.3: bytes=56 Sequence=5 ttl=254 time=31 ms

--- 10.0.3.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 31/35/38 ms

Loopback0 interfaces on SW1 and SW3 can communicate with each other.

Configure a service access point.

Configure a service access point on SW1 and SW3, create sub-interfaces to communicate with ACC-1 and ACC-2 and terminate packets from VLAN 100, and bind the sub-interfaces to BD 20.

Configure the interfaces connecting ACC-1 and ACC-2 to PCs as access interfaces and set PVID to 100. Configure the interfaces connecting ACC-1 and ACC-2 to SW1 and SW3 as trunk interfaces, and allow packets from VLAN 100 to pass through.

# Configure SW1.

[SW3]bridge-domain 20

[SW3-bd20] vxlan vni 1000

[SW3-bd20] quit

[SW3]interface GigabitEthernet0/0/2.100 mode l2

[SW3-GigabitEthernet0/0/2.100] encapsulation dot1q vid 100

[SW3-GigabitEthernet0/0/2.100] bridge-domain 20

[SW3-GigabitEthernet0/0/2.100] quit

# Configure SW3.

[SW3]bridge-domain 20

[SW3-bd20] vxlan vni 1000

[SW3-bd20] quit

[SW3]interface GigabitEthernet0/0/2.100 mode l2

[SW3-GigabitEthernet0/0/2.100] encapsulation dot1q vid 100

[SW3-GigabitEthernet0/0/2.100] bridge-domain 20

[SW3-GigabitEthernet0/0/2.100] quit

# Configure ACC-1.

[ACC-1]vlan 100

[ACC-1]interface GigabitEthernet0/0/24

[ACC-1-GigabitEthernet0/0/24] port link-type access

[ACC-1-GigabitEthernet0/0/24] port default vlan 100

[ACC-1-GigabitEthernet0/0/24] quit

[ACC-1]interface GigabitEthernet0/0/1

[ACC-1-GigabitEthernet0/0/1 port link-type trunk

[ACC-1-GigabitEthernet0/0/1] port trunk allow-pass vlan 100

[ACC-1-GigabitEthernet0/0/1] quit

# Configure ACC-2.

[ACC-2]vlan 100

[ACC-2]interface GigabitEthernet0/0/24

[ACC-2-GigabitEthernet0/0/24] port link-type access

[ACC-2-GigabitEthernet0/0/24] port default vlan 100

[ACC-2-GigabitEthernet0/0/24] quit

[ACC-2]interface GigabitEthernet0/0/1

[ACC-2-GigabitEthernet0/0/1] port link-type trunk

[ACC-2-GigabitEthernet0/0/1] port trunk allow-pass vlan 100

[ACC-2-GigabitEthernet0/0/1] quit

Configure a static VXLAN tunnel.

On SW1 and SW3, use the IP address of Loopback0 as the source address of the NVE interface to establish a static VXLAN tunnel, achieving communication between PC1 and PC2.

# Create an NVE interface on SW1.

[SW1]interface Nve1

[SW1-Nve1] source 10.0.1.1

[SW1-Nve1] vni 1000 head-end peer-list 10.0.3.3

[SW1-Nve1] quit

# Create an NVE interface on SW3.

[SW3]interface Nve1

[SW3-Nve1] source 10.0.3.3

[SW3-Nve1] vni 1000 head-end peer-list 10.0.1.1

[SW3-Nve1] quit

# Check the VXLAN tunnel information of SW1 and SW3.

[SW1]display vxlan tunnel

Number of vxlan tunnel : 1

Tunnel ID   Source               Destination           State    Type

----------------------------------------------------------------------------------------------------------------------

4026531841  10.0.1.1              10.0.3.3               up      static

[SW3]display vxlan tunnel

Number of vxlan tunnel : 1

Tunnel ID   Source                Destination          State    Type

----------------------------------------------------------------------------------------------------------------------

4026531841  10.0.3.3               10.0.1.1              up      static

The command output shows that a static VXLAN tunnel has been established on SW1 and SW3.

Verify the configuration.

Ping PC2 from PC1 and check VXLAN-related forwarding entries.

# Ping PC2 from PC1.

PC1>ping 172.16.1.2

Ping 172.16.1.2: 32 data bytes, Press Ctrl_C to break

From 172.16.1.2: bytes=32 seq=1 ttl=128 time=78 ms

From 172.16.1.2: bytes=32 seq=2 ttl=128 time=94 ms

From 172.16.1.2: bytes=32 seq=3 ttl=128 time=94 ms

From 172.16.1.2: bytes=32 seq=4 ttl=128 time=125 ms

From 172.16.1.2: bytes=32 seq=5 ttl=128 time=93 ms

--- 172.16.1.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 78/96/125 ms

The command output shows that PC1 can communicate with PC2.

# Check the MAC address entries on SW1 and SW3.

<SW1>display	mac-address

-------------------------------------------------------------------------------

MAC	Address							VLAN/VSI/BD			Learned-From								Type

-------------------------------------------------------------------------------

4c1f-cc3d-01c6						12/-/-										GE0/0/1															dynamic

5489-98d4-80d7					-/-/20										10.0.3.3															dynamic

5489-985e-4597					-/-/20										GE0/0/2.100									dynamic

-------------------------------------------------------------------------------

On SW1, you can find that one MAC address is learned from both the local sub-interface GE1/0/2.100 and the remote VTEP (10.0.3.3). The learned MAC addresses correspond to the MAC addresses of PC1 and PC2 respectively. One MAC address is learned when the local sub-interface is receiving data frames, and one MAC address is learned when the remote VTEP is sending a terminal response packet.

<SW3>display	 mac-address

-------------------------------------------------------------------------------

MAC	Address						VLAN/VSI/BD				Learned-From									Type

-------------------------------------------------------------------------------

4c1f-cc3d-01c6	23			/-/-												GE0/0/1															dynamic

5489-98d4-80d7				-/-/20											GE0/0/2.100									dynamic

5489-985e-4597				-/-/20											10.0.1.1															dynamic

----------------------------------------------------------------------------------------

Similarly, on SW3, you can find the MAC addresses learned from the local sub-interface and remote VTEP (10.0.1.1).

# Obtain and view VXLAN packets forwarded between SW1 and SW3.

Ethernet II, Src: HuaweiTe_3d:01:c6 (4c:1f:cc:3d:01:c6), Dst: HuaweiTe_dc:79:4a (70:7b:e8:dc:79:4a)

802.1Q Virtual LAN, PRI: 0, DEI: 0, ID: 23

000. .... .... .... = Priority: Best Effort (default) (0)

...0 .... .... .... = DEI: Ineligible

.... 0000 0001 0111 = ID: 23

Type: IPv4 (0x0800)

Internet Protocol Version 4, Src: 10.0.1.1, Dst: 10.0.3.3

User Datagram Protocol, Src Port: 4789, Dst Port: 4789

Virtual eXtensible Local Area Network

Flags: 0x0800, VXLAN Network ID (VNI)

Group Policy ID: 0

VXLAN Network Identifier (VNI): 1000

Reserved: 0

Ethernet II, Src: HuaweiTe_5e:45:97 (54:89:98:5e:45:97), Dst: Broadcast (ff:ff:ff:ff:ff:ff)

Address Resolution Protocol (request)

Hardware type: Ethernet (1)

Protocol type: IPv4 (0x0800)

Hardware size: 6

Protocol size: 4

Opcode: request (1)

Sender MAC address: HuaweiTe_5e:45:97 (54:89:98:5e:45:97)

Sender IP address: 172.16.1.1

Target MAC address: Broadcast (ff:ff:ff:ff:ff:ff)

Target IP address: 172.16.1.2

On SW2, perform mirroring to obtain the packets transmitted between SW2 and SW3. Check the transmitted VXLAN packets.

The preceding command output shows the ARP Request packet sent by PC1 to request the MAC address of PC2. You can find that the VLAN ID in the outer data frame is 23 and, in the outer IP header, the source IP address is 10.0.1.1 and the destination IP address is 10.0.3.3. This packet is sent from SW1 to SW3.

In the inner data frame, you can see that the VNI in the VXLAN header is 1000, the target IP address is 172.16.1.2, and the sender IP address is 172.16.1.1.

Ethernet II, Src: HuaweiTe_dc:79:4a (70:7b:e8:dc:79:4a), Dst: HuaweiTe_3d:01:c6 (4c:1f:cc:3d:01:c6)

802.1Q Virtual LAN, PRI: 0, DEI: 0, ID: 23

000. .... .... .... = Priority: Best Effort (default) (0)

...0 .... .... .... = DEI: Ineligible

.... 0000 0001 0111 = ID: 23

Type: IPv4 (0x0800)

Internet Protocol Version 4, Src: 10.0.3.3, Dst: 10.0.1.1

User Datagram Protocol, Src Port: 4789, Dst Port: 4789

Virtual eXtensible Local Area Network

Flags: 0x0800, VXLAN Network ID (VNI)

Group Policy ID: 0

VXLAN Network Identifier (VNI): 1000

Reserved: 0

Ethernet II, Src: HuaweiTe_d4:80:d7 (54:89:98:d4:80:d7), Dst: HuaweiTe_5e:45:97 (54:89:98:5e:45:97)

Internet Protocol Version 4, Src: 172.16.1.2, Dst: 172.16.1.1

Internet Control Message Protocol

Type: 0 (Echo (ping) reply)

Code: 0

Checksum: 0xb827 [correct]

[Checksum Status: Good]

Identifier (BE): 54869 (0xd655)

Identifier (LE): 21974 (0x55d6)

Sequence number (BE): 1 (0x0001)

Sequence number (LE): 256 (0x0100)

The preceding command output shows the ICMP Reply packet sent by PC2 to PC1. In the outer IP header, the source IP address is 10.0.3.3 and the destination IP address is 10.0.1.1. In the inner VXLAN header, the VNI is 1000.

----End

Configuration Reference

Configurations of ACC-1

#

sysname ACC-1

#

vlan batch 100

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/24

port link-type access

port default vlan 100

#

Return

Configurations of ACC-2

#

sysname ACC-2

#

vlan batch 100

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/24

port link-type access

port default vlan 100

#

Return

Configurations of SW1

#

sysname SW1

#

vlan batch 12 100

#

bridge-domain 20

vxlan vni 1000

#

interface Vlanif12

ip address 10.0.12.1 255.255.255.0

#

interface GigabitEthernet0/0/2.100 mode l2

encapsulation dot1q vid 100

bridge-domain 20

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 12

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

interface Nve1

source 10.0.1.1

vni 1000 head-end peer-list 10.0.3.3

#

ospf 1 router-id 10.0.1.1

area 0.0.0.0

network 10.0.1.1 0.0.0.0

network 10.0.12.0 0.0.0.255

#

Return

Configurations of SW2

#

sysname SW2

#

vlan batch 12 23

#

interface Vlanif12

ip address 10.0.12.2 255.255.255.0

#

interface Vlanif23

ip address 10.0.23.2 255.255.255.0

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 12

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 23

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.12.0 0.0.0.255

network 10.0.23.0 0.0.0.255

#

Return

Configurations of SW3

#

sysname SW3

#

vlan batch 23 100

#

bridge-domain 20

vxlan vni 1000

#

interface Vlanif23

ip address 10.0.23.3 255.255.255.0

#

interface GigabitEthernet0/0/2.100 mode l2

encapsulation dot1q vid 100

bridge-domain 20

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 23

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

interface Nve1

source 10.0.3.3

vni 1000 head-end peer-list 10.0.1.1

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.23.0 0.0.0.255

#

Return

Quiz

How does a VTEP process broadcast packets received from hosts in a local BD?

Centralized VXLAN Gateway

Introduction

About This Lab

This lab guides you how to configure a switch to achieve a centralized VXLAN gateway, so that users on different network segments can communicate at Layer 3 through VBDIF interfaces on the centralized VXLAN gateway.

Objectives

Upon completion of this lab, you will be able to:

Configure a centralized VXLAN gateway to implement communication across Layer 3.

Describe the packet forwarding process in the centralized VXLAN gateway scenario.

Networking Topology

Centralized VXLAN gateway topology

Loopback0 interfaces are created on SW1, SW2, and SW3, and their IP addresses are in the format of 10.0.x.x, where x indicates the device ID.

A trunk link is established between SW1 and SW2, which allows packets from VLAN 12 to pass through. VLANIF 12 is created on SW1 and SW2 for Layer 3 communication, and its IP address is set to 10.0.12.x/24, where x indicates the device ID.

A trunk link is established between SW2 and SW3, which allows packets from VLAN 23 to pass through. VLANIF 23 is created on SW2 and SW3 for Layer 3 communication, and its IP address is set to 10.0.23.x/24, where x indicates the device ID.

OSPF runs on SW1, SW2, and SW3, the IP address of Loopback0 is used as the router ID, and OSPF is enabled on loopback and interconnection interfaces.

SW1, SW2, and SW3 use the IP address of Loopback0 as the source address of the VXLAN NVE interface. A VXLAN tunnel is established between SW1 and SW2 to transmit traffic of BD 10. Similarly, a VXLAN tunnel is established between SW2 and SW3 to transmit traffic of BD 20.

ACC-1 and ACC-2 function as access switches and connect to PC1 and PC2 through access interfaces. PC1 and PC2 are added to VLAN 100 and VLAN 200 respectively. SW1 and SW2 connect to ACC-1 and ACC-2 through Layer 2 sub-interfaces to terminate packets from VLAN 100 and VLAN 200 respectively, and these sub-interfaces are associated with BD 10 and BD 20 respectively.

On SW3, create VBDIF 10 and VBDIF 20 as gateways for terminals in BD 10 and BD 20 to implement Layer 3 communication between terminals in BD 10 and BD 20.

Lab Configuration

Configuration Roadmap

- Complete connectivity configuration.

- Configure a service access point.

- Configure static VXLAN tunnels between SW1 and SW2 as well as between SW2 and SW3.

- Create a VBDIF interface.

Configuration Procedure

          - Complete connectivity configuration.

Configure interconnection interfaces and OSPF on SW1, SW2, and SW3.

# Name the devices.

The configuration details are not provided.

# On SW1, configure VLANs and interfaces.

[SW1]vlan batch 12 100

[SW1]interface GigabitEthernet0/0/1

[SW1-GigabitEthernet0/0/1] port link-type trunk

[SW1-GigabitEthernet0/0/1] port trunk allow-pass vlan 12

[SW1-GigabitEthernet0/0/1] quit

[SW1]interface Vlanif12

[SW1-Vlanif12] ip address 10.0.12.1 255.255.255.0

[SW1-Vlanif12] quit

[SW1]interface LoopBack 0

[SW1-LoopBack0] ip address 10.0.1.1 32

[SW1-LoopBack0] quit

# On SW2, configure VLANs and interfaces.

[SW2]vlan batch 12 23

[SW2]interface GigabitEthernet0/0/2

[SW2-GigabitEthernet0/0/2] port link-type trunk

[SW2-GigabitEthernet0/0/2] port trunk allow-pass vlan 12

[SW2-GigabitEthernet0/0/2 quit

[SW2]interface GigabitEthernet0/0/3

[SW2-GigabitEthernet0/0/3] port link-type trunk

[SW2-GigabitEthernet0/0/3] port trunk allow-pass vlan 23

[SW2-GigabitEthernet0/0/3] quit

[SW2]interface Vlanif12

[SW2-Vlanif12] ip address 10.0.12.2 255.255.255.0

[SW2-Vlanif12] quit

[SW2]interface Vlanif23

[SW2-Vlanif23] ip address 10.0.23.2 255.255.255.0

[SW2-Vlanif23] quit

[SW2]interface LoopBack 0

[SW2-LoopBack0] ip address 10.0.2.2 32

[SW2-LoopBack0] quit

# On SW3, configure VLANs and interfaces.

[SW3]vlan batch 23 200

[SW3]interface GigabitEthernet0/0/1

[SW3-GigabitEthernet0/0/1] port link-type trunk

[SW3-GigabitEthernet0/0/1] port trunk allow-pass vlan 23

[SW3-GigabitEthernet0/0/1] quit

[SW3]interface Vlanif 23

[SW3-Vlanif23] ip address 10.0.23.3 255.255.255.0

[SW3-Vlanif23] quit

[SW3]interface LoopBack 0

[SW3-LoopBack0] ip address 10.0.3.3 32

[SW3-LoopBack0] quit

# Test the connectivity of interconnection interfaces.

<SW2>ping 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=50 ms

Reply from 10.0.12.1: bytes=56 Sequence=2 ttl=255 time=40 ms

Reply from 10.0.12.1: bytes=56 Sequence=3 ttl=255 time=50 ms

Reply from 10.0.12.1: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.12.1: bytes=56 Sequence=5 ttl=255 time=30 ms

--- 10.0.12.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/42/50 ms

<SW2>ping 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=60 ms

Reply from 10.0.23.3: bytes=56 Sequence=2 ttl=255 time=30 ms

Reply from 10.0.23.3: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.23.3: bytes=56 Sequence=4 ttl=255 time=30 ms

Reply from 10.0.23.3: bytes=56 Sequence=5 ttl=255 time=50 ms

--- 10.0.23.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/42/60 ms

On SW2, test the connectivity with VLANIF interfaces on SW1 and SW3.

# On SW1, configure OSPF.

[SW1]ospf 1 router-id 10.0.1.1

[SW1-ospf-1] area 0.0.0.0

[SW1-ospf-1-area-0.0.0.0] network 10.0.1.1 0.0.0.0

[SW1-ospf-1-area-0.0.0.0] network 10.0.12.0 0.0.0.255

[SW1-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0 and VLANIF 12.

# On SW2, configure OSPF.

[SW2]ospf 1 router-id 10.0.2.2

[SW2-ospf-1] area 0.0.0.0

[SW2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[SW2-ospf-1-area-0.0.0.0] network 10.0.12.0 0.0.0.255

[SW2-ospf-1-area-0.0.0.0] network 10.0.23.0 0.0.0.255

[SW2-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0, VLANIF 12, and VLANIF 23.

# On SW3, configure OSPF.

[SW3]ospf 1 router-id 10.0.3.3

[SW3-ospf-1] area 0.0.0.0

[SW3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[SW3-ospf-1-area-0.0.0.0] network 10.0.23.0 0.0.0.255

[SW3-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0 and VLANIF 23.

# On SW2, check the OSPF neighbor relationship and OSPF routing table.

<SW2>display ospf peer

OSPF Process 1 with Router ID 10.0.2.2

Neighbors

Area 0.0.0.0 interface 10.0.23.2(Vlanif23)'s neighbors

Router ID: 10.0.3.3         Address: 10.0.23.3

State: Full  Mode:Nbr is  Master  Priority: 1

DR: 10.0.23.3  BDR: 10.0.23.2  MTU: 0

Dead timer due in 32  sec

Retrans timer interval: 4

Neighbor is up for 00:04:52

Authentication Sequence: [ 0 ]

Neighbors

Area 0.0.0.0 interface 10.0.12.2(Vlanif12)'s neighbors

Router ID: 10.0.1.1         Address: 10.0.12.1

State: Full  Mode:Nbr is  Slave  Priority: 1

DR: 10.0.12.1  BDR: 10.0.12.2  MTU: 0

Dead timer due in 39  sec

Retrans timer interval: 6

Neighbor is up for 00:01:39

Authentication Sequence: [ 0 ]

OSPF neighbor relationships have been established between SW1 and SW2 as well as between SW3 and SW2.

<SW2>display	ospf	routing

OSPF	Process	1	with	Router	ID	10.0.2.2

Routing	Tables

Routing	for	Network

Destination									Cost			Type								NextHop							AdvRouter						Area

10.0.2.2/32											0					Stub								10.0.2.2								10.0.2.2								0.0.0.0

10.0.12.0/24										1					Transit					10.0.12.2							10.0.2.2								0.0.0.0

10.0.23.0/24										1					Transit					10.0.23.2							10.0.2.2								0.0.0.0

10.0.1.1/32											1					Stub								10.0.12.1							10.0.1.1								0.0.0.0

10.0.3.3/32											1					Stub								10.0.23.3							10.0.3.3								0.0.0.0

Total	Nets:	5

Intra	Area:	5		Inter	Area:	0		ASE:	0		NSSA:	0

SW2 has learned the routes generated by Loopback0 interfaces on SW1 and SW3.

# Test the connectivity of Loopback0 interfaces between SW1 and SW3.

<SW1>ping -a 10.0.1.1 10.0.3.3

PING 10.0.3.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.3.3: bytes=56 Sequence=1 ttl=254 time=36 ms

Reply from 10.0.3.3: bytes=56 Sequence=2 ttl=254 time=34 ms

Reply from 10.0.3.3: bytes=56 Sequence=3 ttl=254 time=37 ms

Reply from 10.0.3.3: bytes=56 Sequence=4 ttl=254 time=38 ms

Reply from 10.0.3.3: bytes=56 Sequence=5 ttl=254 time=31 ms

--- 10.0.3.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 31/35/38 ms

Loopback0 interfaces on SW1 and SW3 can communicate with each other.

Configure a service access point.

Configure a service access point on SW1 and SW3, create sub-interfaces to communicate with ACC-1 and ACC-2 and terminate packets from VLAN 100 and VLAN 200, and bind the sub-interfaces to BD 10 and BD 20.

Configure the interfaces connecting ACC-1 and ACC-2 to PCs as access interfaces. Set PVIDs to 100 and 200. Configure the interfaces connecting ACC-1 and ACC-2 to SW1 and SW3 as trunk interfaces, and allow packets from VLAN 100 and VLAN 200 to pass through.

# Configure SW1.

[SW1]bridge-domain 10

[SW1-bd20] vxlan vni 1000

[SW1-bd20] quit

[SW1]interface GigabitEthernet0/0/2.100 mode l2

[SW1-GigabitEthernet0/0/2.100] encapsulation dot1q vid 100

[SW1-GigabitEthernet0/0/2.100] bridge-domain 10

[SW1-GigabitEthernet0/0/2.100] quit

# Configure SW3.

[SW3]bridge-domain 20

[SW3-bd20] vxlan vni 2000

[SW3-bd20] quit

[SW3]interface GigabitEthernet0/0/2.200 mode l2

[SW3-GigabitEthernet0/0/2.200] encapsulation dot1q vid 200

[SW3-GigabitEthernet0/0/2.200] bridge-domain 20

[SW3-GigabitEthernet0/0/2.200] quit

# Configure ACC-1.

[ACC-1]vlan 100

[ACC-1]interface GigabitEthernet0/0/24

[ACC-1-GigabitEthernet0/0/24] port link-type access

[ACC-1-GigabitEthernet0/0/24] port default vlan 100

[ACC-1-GigabitEthernet0/0/24] quit

[ACC-1]interface GigabitEthernet0/0/1

[ACC-1-GigabitEthernet0/0/1] port link-type trunk

[ACC-1-GigabitEthernet0/0/1] port trunk allow-pass vlan 100

[ACC-1-GigabitEthernet0/0/1] quit

# Configure ACC-2.

[ACC-2]vlan 200

[ACC-2]interface GigabitEthernet0/0/24

[ACC-2-GigabitEthernet0/0/24] port link-type access

[ACC-2-GigabitEthernet0/0/24] port default vlan 200

[ACC-2-GigabitEthernet0/0/24] quit

[ACC-2]interface GigabitEthernet0/0/1

[ACC-2-GigabitEthernet0/0/1] port link-type trunk

[ACC-2-GigabitEthernet0/0/1] port trunk allow-pass vlan 200

[ACC-2-GigabitEthernet0/0/1] quit

Configure a static VXLAN tunnel and a VBDIF interface.

SW1, SW2, and SW3 use the IP address of Loopback0 as the source address of the NVE interface to establish VXLAN tunnels. Specifically, a VXLAN tunnel is established between SW1 and SW2 to transmit traffic of BD 10. Similarly, a VXLAN tunnel is established between SW2 and SW3 to transmit traffic of BD 20.

On SW2, create VBDIF 10 and VBDIF 20 as the gateways for PC1 and PC2.

# On SW1 and SW2, configure NVE interfaces to establish a static VXLAN tunnel.

[SW1]interface Nve1

[SW1-Nve1] source 10.0.1.1

[SW1-Nve1] vni 1000 head-end peer-list 10.0.2.2

[SW1-Nve1] quit

[SW2]interface Nve1

[SW2-Nve1] source 10.0.2.2

[SW2-Nve1] vni 1000 head-end peer-list 10.0.1.1

[SW2-Nve1] quit

# On SW2 and SW3, configure NVE interfaces to establish a static VXLAN tunnel.

[SW2]interface Nve1

[SW2-Nve1] vni 2000 head-end peer-list 10.0.3.3

[SW2-Nve1] quit

[SW3]interface Nve1

[SW3-Nve1] source 10.0.3.3

[SW3-Nve1] vni 2000 head-end peer-list 10.0.2.2

[SW3-Nve1] quit

# On SW2, create BD 10 and BD 20, and configure VXLAN VNIs for the BDs.

[SW2]bridge-domain 10

[SW2-bd10] vxlan vni 1000

[SW2-bd10] quit

[SW2]bridge-domain 20

[SW2-bd20] vxlan vni 2000

[SW2-bd20] quit

# On SW2, create VBDIF 10 and VBDIF 20.

[SW2]interface Vbdif10

[SW2-Vbdif10] ip address 172.16.1.254 255.255.255.0

[SW2-Vbdif10] quit

[SW2]interface Vbdif20

[SW2-Vbdif20] ip address 172.16.2.254 255.255.255.0

[SW2-Vbdif20]quit

Verify the configuration.

Try to ping PC2 from PC1 and check the entire forwarding process.

# Ping PC2 from PC1.

PC>ping 172.16.2.2

Ping 172.16.2.2: 32 data bytes, Press Ctrl_C to break

From 172.16.2.2: bytes=32 seq=1 ttl=127 time=78 ms

From 172.16.2.2: bytes=32 seq=2 ttl=127 time=78 ms

From 172.16.2.2: bytes=32 seq=3 ttl=127 time=78 ms

From 172.16.2.2: bytes=32 seq=4 ttl=127 time=63 ms

From 172.16.2.2: bytes=32 seq=5 ttl=127 time=62 ms

--- 172.16.2.2 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 62/71/78 ms

The command output shows that PC1 can communicate with PC2 across network segments.

# On SW2, check the MAC addresses of VBDIF 10 and VBDIF 20.

[SW2]display interface Vbdif 10

Vbdif10 current state : UP (ifindex: 21)

Line protocol current state : UP

Last line protocol up time : 2020-11-20 18:45:07

Description:

Route Port,The Maximum Transmit Unit is 1500

Internet Address is 172.16.1.254/24

IP Sending Frames' Format is PKTFMT_ETHNT_2, Hardware address is 707b-e843-277d

….

….

….

[~SW2]display interface Vbdif 20

Vbdif20 current state : UP (ifindex: 22)

Line protocol current state : UP

Last line protocol up time : 2020-11-20 18:46:06

Description:

Route Port,The Maximum Transmit Unit is 1500

Internet Address is 172.16.2.254/24

IP Sending Frames' Format is PKTFMT_ETHNT_2, Hardware address is 707b-e843-277d

….

….

….

Similar to VLANIF interfaces, VBDIF 10 and VBDIF 20 have the same MAC address, which is 707b-e843-277d.

# On SW1, check MAC address entries in BD 10.

<SW1>display	 mac-address 	bridge-domain	10

-------------------------------------------------------------------------------

MAC	Address								VLAN/VSI/BD			Learned-From									Type								Age

-------------------------------------------------------------------------------

707b-e843-277d								-/-/10								10.0.2.2																dynamic							-

5489-988d-3e41								-/-/10								GE0/0/2.100										dynamic							-

-------------------------------------------------------------------------------

Total	items:	2

As shown in the command output, there are two MAC address entries in BD 10, which are learned respectively from the local sub-interface GE0/0/2.100 and the remote VTEP (10.0.2.2). One of the learned two MAC addresses is the MAC address of PC1, and the other is the MAC address of VBDIF 10 on SW2.

# On SW2, perform mirroring to obtain the packets sent by SW1. Check the ICMP packets sent from PC1 to PC2.

Ethernet II, Src: HuaweiTe_f7:64:fe (70:7b:e8:f7:64:fe), Dst: HuaweiTe_43:27:7d (70:7b:e8:43:27:7d)

802.1Q Virtual LAN, PRI: 0, DEI: 0, ID: 12

Internet Protocol Version 4, Src: 10.0.1.1, Dst: 10.0.2.2

User Datagram Protocol, Src Port: 4789, Dst Port: 4789

Virtual eXtensible Local Area Network

Flags: 0x0800, VXLAN Network ID (VNI)

Group Policy ID: 0

VXLAN Network Identifier (VNI): 1000

Reserved: 0

Ethernet II, Src: HuaweiTe_8d:3e:41 (54:89:98:8d:3e:41), Dst: HuaweiTe_43:27:7d (70:7b:e8:43:27:7d)

Internet Protocol Version 4, Src: 172.16.1.1, Dst: 172.16.2.2

Internet Control Message Protocol

The ICMP packets sent from PC1 to PC2 are encapsulated by SW1 (10.0.1.1) into VXLAN packets (VNI 1000) and then sent to SW2 (10.0.2.2). The destination MAC address carried in the inner data frame is 707b-e843-277d, that is, the MAC address of VBDIF 10 on SW2.

After receiving the data frame, SW2 checks the MAC address (707b-e843-277d) in the outer data frame. Because the MAC address is the MAC address of VLANIF 12, SW2 determines that the data frame needs to be processed locally. Then, SW2 decapsulates the data frame and checks the destination IP address, finding that the destination IP address is 10.0.2.2, which is the IP address of the Loopback0. Then, SW2 further decapsulates the data frame and checks the upper-layer data, finding that it is encapsulated in VXLAN mode. Then, SW2 checks the VNI in the VXLAN header and determines that the inner data frame needs to be processed in BD 10 based on VNI 1000.

Subsequently, SW2 checks the destination MAC address (707b-e843-277d) of the inner data frame in BD 10. Because the destination MAC address is the MAC address of VBDIF 10, SW2 determines that the data frame needs to be processed by VBDIF 10. Then, SW2 further decapsulates the data frame to check the upper-layer data, finding that the network-layer destination IP address of the inner data frame is 172.16.2.2, which is not the IP address of the local interface. In this case, SW2 performs the Layer 3 table lookup before forwarding and checks the local IP routing table.

# Check the IP routing table on SW2.

<SW2>display ip routing-table 172.16.2.2

Proto: Protocol        Pre: Preference

Route Flags: R - relay, D - download to fib, T - to vpn-instance, B - black hole

route

------------------------------------------------------------------------------

Routing Table : _public_

Summary Count : 1

Destination/Mask    Proto   Pre  Cost        Flags  NextHop      Interface

172.16.2.0/24  	Direct   0    0         				D    172.16.2.254    Vbdif20

Run the display ip routing-table 172.16.2.2 command to check the result of Layer 3 table lookup for the packets destined for 172.16.2.2. In the result, there is a direct route generated by VBDIF 20. In this case, SW2 determines the forwarding interface based on the ARP entry.

# Check ARP entries on SW2.

<SW2>display arp | include 172.16.2.2

ARP Entry Types: D - Dynamic, S - Static, I - Interface, O - OpenFlow, RD - Redirect

EXP: Expire-time VLAN:VLAN or Bridge Domain

IP ADDRESS      MAC ADDRESS    EXP(M)   TYPE/VLAN   INTERFACE  VPN-INSTANCE

----------------------------------------------------------------------------------------------------------

172.16.2.254        707b-e843-277d              I              Vbdif20

172.16.2.2          5489-9896-2e95      19      D/BD20       	10.0.3.3

----------------------------------------------------------------------------------------------------------

Check the ARP entry whose IP address is 172.16.2.2. The result shows that the entry is in BD 20, the interface is the remote VTEP (10.0.3.3) on SW3, and the MAC address is 5489-9896-2e95. In this case, SW2 re-encapsulates the inner data frame (by replacing the source and destination MAC addresses), and then sends the data frame to SW3 through the VXLAN tunnel (VNI 2000) corresponding to BD 20.

# On SW2, perform mirroring to obtain the packets sent by SW2. Check the ICMP packets sent from PC1 to PC2.

Ethernet II, Src: HuaweiTe_43:27:7d (70:7b:e8:43:27:7d), Dst: HuaweiTe_2a:0b:83 (70:7b:e8:2a:0b:83)

802.1Q Virtual LAN, PRI: 0, DEI: 0, ID: 23

Internet Protocol Version 4, Src: 10.0.2.2, Dst: 10.0.3.3

User Datagram Protocol, Src Port: 4789, Dst Port: 4789

Virtual eXtensible Local Area Network

Flags: 0x0800, VXLAN Network ID (VNI)

Group Policy ID: 0

VXLAN Network Identifier (VNI): 2000

Reserved: 0

Ethernet II, Src: HuaweiTe_43:27:7d (70:7b:e8:43:27:7d), Dst: HuaweiTe_96:2e:95 (54:89:98:96:2e:95)

Internet Protocol Version 4, Src: 172.16.1.1, Dst: 172.16.2.2

Internet Control Message Protocol

The ICMP packets sent from PC1 to PC2 are forwarded to SW3 through SW2. The source and destination IP addresses of the outer data frame are 10.0.2.2 and 10.0.3.3, and the VXLAN VNI is 2000. The source MAC address of the inner data frame is the MAC address of VBDIF 20 (70:7b:e8:43:27:7d), and the destination MAC address is the MAC address of PC2 (54:89:98:96:2e:95).

After receiving the data frame, SW3 checks the MAC address (70:7b:e8:2a:0b:83) in the outer data frame. Because the MAC address is the MAC address of VLANIF 23, SW3 determines that the data frame needs to be processed locally. Then, SW3 decapsulates the data frame and checks the destination IP address, finding that the destination IP address is 10.0.3.3, which is the IP address of the local Loopback0. Then, SW3 further decapsulates the data frame and checks the upper-layer data, finding that it is encapsulated in VXLAN mode. Then, SW3 checks the VNI in the VXLAN header and determines that the inner data frame needs to be processed in BD 20 based on VNI 2000.

Subsequently, SW3 checks the destination MAC address (54:89:98:96:2e:95) of the inner data frame in BD 20. This destination MAC address exists in the MAC address table, so SW3 performs Layer 2 forwarding based on the entry in the MAC address table.

# Check the MAC address table in BD 20 on SW3.

<SW3>display	mac-address 	bridge-domain	20

---------------------------------------------------------------------------------------------------------

MAC	Address						VLAN/VSI/BD					Learned-From								Type										Age

--------------------------------------------------------------------------------------------------------

707b-e843-277d				-/-/20											10.0.2.2													dynamic								-

5489-9896-2e95				-/-/20											GE0/0/2.200								dynamic								-

--------------------------------------------------------------------------------------------------------

Total	items:	2

In the result, there is an entry with the outbound interface being GE0/0/2.200. In this case, SW3 forwards the inner data frame carrying VLAN tag 200 through GE0/0/2, which is then forwarded to PC2 through ACC-2.

By now, the ICMP packets sent by PC1 to PC2 arrive at PC2.

----End

Configuration Reference

Configurations of ACC-1

#

sysname ACC-1

#

vlan batch 100

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/24

port link-type access

port default vlan 100

#

Return

Configurations of ACC-2

#

sysname ACC-2

#

vlan batch 200

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 200

#

interface GigabitEthernet0/0/24

port link-type access

port default vlan 200

#

Return

Configurations of SW1

#

sysname SW1

#

vlan batch 12 100

#

bridge-domain 10

vxlan vni 1000

#

interface Vlanif12

ip address 10.0.12.1 255.255.255.0

#

interface GigabitEthernet0/0/2.100 mode l2

encapsulation dot1q vid 100

bridge-domain 10

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 12

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

interface Nve1

source 10.0.1.1

vni 1000 head-end peer-list 10.0.2.2

#

ospf 1 router-id 10.0.1.1

area 0.0.0.0

network 10.0.1.1 0.0.0.0

network 10.0.12.0 0.0.0.255

#

Return

Configurations of SW2

#

sysname SW2

#

vlan batch 12 23

#

bridge-domain 10

vxlan vni 1000

#

bridge-domain 20

vxlan vni 2000

#

interface Vbdif10

ip address 172.16.1.254 255.255.255.0

#

interface Vbdif20

ip address 172.16.2.254 255.255.255.0

#

interface Vlanif12

ip address 10.0.12.2 255.255.255.0

#

interface Vlanif23

ip address 10.0.23.2 255.255.255.0

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 12

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 23

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

interface Nve1

source 10.0.2.2

vni 1000 head-end peer-list 10.0.1.1

vni 2000 head-end peer-list 10.0.3.3

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.12.0 0.0.0.255

network 10.0.23.0 0.0.0.255

#

Return

Configurations of SW3

#

sysname SW3

#

vlan batch 23 200

#

bridge-domain 20

vxlan vni 2000

#

interface Vlanif23

ip address 10.0.23.3 255.255.255.0

#

interface GigabitEthernet0/0/2.200 mode l2

encapsulation dot1q vid 200

bridge-domain 20

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 23

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

interface Nve1

source 10.0.3.3

vni 2000 head-end peer-list 10.0.2.2

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.23.0 0.0.0.255

#

Return

Quiz

What is the advantage of centralized gateways compared with the distributed gateways？

Distributed VXLAN Gateway

Introduction

About This Lab

This lab guides you through how to configure distributed VXLAN gateways and BGP EVPN, so that terminals on different network segments can communicate at Layer 3 across the VXLAN network.

Objectives

Upon completion of this lab, you will be able to:

Configure distributed VXLAN gateways to implement Layer 3 communication.

Describe the packet forwarding process on the same network segment or across different network segments in the distributed VXLAN gateway scenario.

Networking Topology

Distributed VXLAN gateway topology

Loopback0 interfaces are created on SW1, SW2, and SW3, and their IP addresses are in the format of 10.0.x.x, where x indicates the device ID.

A trunk link is established between SW1 and SW2, which allows packets from VLAN 12 to pass through. VLANIF 12 is created on SW1 and SW2 for Layer 3 communication, and its IP address is set to 10.0.12.x/24, where x indicates the device ID.

A trunk link is established between SW2 and SW3, which allows packets from VLAN 23 to pass through. VLANIF 23 is created on SW2 and SW3 for Layer 3 communication, and its IP address is set to 10.0.23.x/24, where x indicates the device ID.

OSPF runs on SW1, SW2, and SW3, the IP address of Loopback0 is used as the router ID, and OSPF is enabled on loopback and interconnection interfaces.

SW1 and SW3 use the IP address of Loopback0 as the source address of the VXLAN NVE interface to transmit traffic of BD 10 and BD 20.

ACC-1 and ACC-2 function as access switches and connect to PC1, PC2, and PC3 through access interfaces. On ACC-1, PC1 is added to VLAN 100. On ACC-2, PC2 and PC3 are added to VLAN 100 and VLAN 200, respectively. SW1 and SW2 are connected to ACC-1 and ACC-2 through Layer 2 sub-interfaces. SW1 terminates packets from VLAN 100, and SW3 terminates packets from VLAN 100 and VLAN 200. In addition, the sub-interfaces are associated with BD 10 and BD 20.

VBDIF 10 and VBDIF 20 are created on SW1 and SW3 to function as the gateways (distributed gateways) for terminals in BD 10 and BD 20. A BGP EVPN peer relationship is established between SW1 and SW3 to transmit Type 2 routes, thereby transmitting host routes and MAC addresses.

Lab Configuration

Configuration Roadmap

- Complete connectivity configuration.

- Configure a service access point.

- Create a VBDIF interface and configure an NVE interface.

- Configure an EVPN instance and an IP VPN instance, and bind these instances to BDs and VBDIF interfaces.

- Establish a BGP EVPN peer relationship.

Configuration Procedure

          - Complete connectivity configuration.

# Name the devices.

The configuration details are not provided.

# On SW1, configure VLANs and interfaces.

[SW1]vlan batch 12 100

[SW1]interface GigabitEthernet0/0/1

[SW1-GigabitEthernet0/0/1] port link-type trunk

[SW1-GigabitEthernet0/0/1] port trunk allow-pass vlan 12

[SW1-GigabitEthernet0/0/1] quit

[SW1]interface Vlanif12

[SW1-Vlanif12] ip address 10.0.12.1 255.255.255.0

[SW1-Vlanif12] quit

[SW1]interface LoopBack 0

[SW1-LoopBack0] ip address 10.0.1.1 32

[SW1-LoopBack0] quit

# On SW2, configure VLANs and interfaces.

[SW2]vlan batch 12 23

[SW2]interface GigabitEthernet0/0/2

[SW2-GigabitEthernet0/0/2] port link-type trunk

[SW2-GigabitEthernet0/0/2] port trunk allow-pass vlan 12

[SW2-GigabitEthernet0/0/2] quit

[SW2]interface GigabitEthernet0/0/3

[SW2-GigabitEthernet0/0/3] port link-type trunk

[SW2-GigabitEthernet0/0/3] port trunk allow-pass vlan 23

[SW2-GigabitEthernet0/0/3] quit

[SW2]interface Vlanif12

[SW2-Vlanif12] ip address 10.0.12.2 255.255.255.0

[SW2-Vlanif12] quit

[SW2]interface Vlanif23

[SW2-Vlanif23] ip address 10.0.23.2 255.255.255.0

[SW2-Vlanif23] quit

[SW2]interface LoopBack 0

[SW2-LoopBack0] ip address 10.0.2.2 32

[SW2-LoopBack0] quit

# On SW3, configure VLANs and interfaces.

[SW3]vlan batch 23 100 200

[SW3]interface GigabitEthernet0/0/1

[SW3-GigabitEthernet0/0/1] port link-type trunk

[SW3-GigabitEthernet0/0/1] port trunk allow-pass vlan 23

[SW3-GigabitEthernet0/0/1] quit

[SW3]interface Vlanif 23

[SW3-Vlanif23] ip address 10.0.23.3 255.255.255.0

[SW3-Vlanif23] quit

[SW3]interface LoopBack 0

[SW3-LoopBack0] ip address 10.0.3.3 32

[SW3-LoopBack0] quit

# Test the connectivity of interconnection interfaces.

<SW2>ping 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=50 ms

Reply from 10.0.12.1: bytes=56 Sequence=2 ttl=255 time=40 ms

Reply from 10.0.12.1: bytes=56 Sequence=3 ttl=255 time=50 ms

Reply from 10.0.12.1: bytes=56 Sequence=4 ttl=255 time=40 ms

Reply from 10.0.12.1: bytes=56 Sequence=5 ttl=255 time=30 ms

--- 10.0.12.1 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/42/50 ms

<SW2>ping 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=60 ms

Reply from 10.0.23.3: bytes=56 Sequence=2 ttl=255 time=30 ms

Reply from 10.0.23.3: bytes=56 Sequence=3 ttl=255 time=40 ms

Reply from 10.0.23.3: bytes=56 Sequence=4 ttl=255 time=30 ms

Reply from 10.0.23.3: bytes=56 Sequence=5 ttl=255 time=50 ms

--- 10.0.23.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/42/60 ms

On SW2, test the connectivity with VLANIF interfaces on SW1 and SW3.

# On SW1, configure OSPF.

[SW1]ospf 1 router-id 10.0.1.1

[SW1-ospf-1] area 0.0.0.0

[SW1-ospf-1-area-0.0.0.0] network 10.0.1.1 0.0.0.0

[SW1-ospf-1-area-0.0.0.0] network 10.0.12.0 0.0.0.255

[SW1-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0 and VLANIF 12.

# On SW2, configure OSPF.

[SW2]ospf 1 router-id 10.0.2.2

[SW2-ospf-1] area 0.0.0.0

[SW2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[SW2-ospf-1-area-0.0.0.0] network 10.0.12.0 0.0.0.255

[SW2-ospf-1-area-0.0.0.0] network 10.0.23.0 0.0.0.255

[SW2-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0, VLANIF 12, and VLANIF 23.

# On SW3, configure OSPF.

[SW3]ospf 1 router-id 10.0.3.3

[SW3-ospf-1] area 0.0.0.0

[SW3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[SW3-ospf-1-area-0.0.0.0] network 10.0.23.0 0.0.0.255

[SW3-ospf-1-area-0.0.0.0] quit

Use the IP address of Loopback0 as the router ID, and enable OSPF on Loopback0 and VLANIF 23.

# On SW2, check the OSPF neighbor relationship and OSPF routing table.

<SW2>display ospf peer

OSPF Process 1 with Router ID 10.0.2.2

Neighbors

Area 0.0.0.0 interface 10.0.23.2(Vlanif23)'s neighbors

Router ID: 10.0.3.3         Address: 10.0.23.3

State: Full  Mode:Nbr is  Master  Priority: 1

DR: 10.0.23.3  BDR: 10.0.23.2  MTU: 0

Dead timer due in 32  sec

Retrans timer interval: 4

Neighbor is up for 00:04:52

Authentication Sequence: [ 0 ]

Neighbors

Area 0.0.0.0 interface 10.0.12.2(Vlanif12)'s neighbors

Router ID: 10.0.1.1         Address: 10.0.12.1

State: Full  Mode:Nbr is  Slave  Priority: 1

DR: 10.0.12.1  BDR: 10.0.12.2  MTU: 0

Dead timer due in 39  sec

Retrans timer interval: 6

Neighbor is up for 00:01:39

Authentication Sequence: [ 0 ]

OSPF neighbor relationships have been established between SW1 and SW2 as well as between SW3 and SW2.

<SW2>display	ospf	routing

OSPF	Process	1	with	Router	ID	10.0.2.2

Routing	Tables

Routing	for	Network

Destination								Cost			Type							NextHop						AdvRouter							Area

10.0.2.2/32									0					Stub								10.0.2.2								10.0.2.2								0.0.0.0

10.0.12.0/24								1					Transit					10.0.12.2							10.0.2.2								0.0.0.0

10.0.23.0/24								1					Transit					10.0.23.2							10.0.2.2								0.0.0.0

10.0.1.1/32									1					Stub								10.0.12.1							10.0.1.1								0.0.0.0

10.0.3.3/32									1					Stub								10.0.23.3							10.0.3.3								0.0.0.0

Total	Nets:	5

Intra	Area:	5		Inter	Area:	0		ASE:	0		NSSA:	0

SW2 has learned the routes generated by Loopback0 interfaces on SW1 and SW3.

# Test the connectivity of Loopback0 interfaces between SW1 and SW3.

<SW1>ping -a 10.0.1.1 10.0.3.3

PING 10.0.3.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.3.3: bytes=56 Sequence=1 ttl=254 time=36 ms

Reply from 10.0.3.3: bytes=56 Sequence=2 ttl=254 time=34 ms

Reply from 10.0.3.3: bytes=56 Sequence=3 ttl=254 time=37 ms

Reply from 10.0.3.3: bytes=56 Sequence=4 ttl=254 time=38 ms

Reply from 10.0.3.3: bytes=56 Sequence=5 ttl=254 time=31 ms

--- 10.0.3.3 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 31/35/38 ms

Loopback0 interfaces on SW1 and SW3 can communicate with each other.

Configure a service access point.

Configure service access points on SW1 and SW3, and create sub-interfaces to communicate with ACC-1 and ACC-2. Create a sub-interface GE0/0/2.100 on SW1 and SW3 to terminate packets from VLAN 100, and associate the sub-interface with BD 10. Create a sub-interface GE0/0/2.200 on SW3 to terminate packets from VLAN 200 and associate the sub-interface with BD 20.

Configure the interfaces connecting ACC-1 and ACC-2 to PCs as access interfaces. On ACC-1, add PC1 to VLAN 100. On ACC-2, add PC2 to VLAN 100 and add PC3 to VLAN 200. Configure the interfaces connecting ACC-1 and ACC-2 to SW1 and SW3 as trunk interfaces, and allow packets from the corresponding VLANs to pass through.

# Configure SW1.

[SW1]bridge-domain 10

[SW1-bd20] vxlan vni 1000

[SW1-bd20] quit

[SW1]bridge-domain 20

[SW1-bd20] vxlan vni 2000

[SW1-bd20] quit

[SW1]interface GigabitEthernet0/0/2.100 mode l2

[SW1-GigabitEthernet0/0/2.100] encapsulation dot1q vid 100

[SW1-GigabitEthernet0/0/2.100] bridge-domain 10

[SW1-GigabitEthernet0/0/2.100] quit

# Configure SW3.

[SW3]bridge-domain 10

[SW3-bd10] vxlan vni 1000

[SW3-bd10] quit

[SW3]bridge-domain 20

[SW3-bd20] vxlan vni 2000

[SW3-bd20] quit

[SW3]interface GigabitEthernet0/0/2.100 mode l2

[SW3-GigabitEthernet0/0/2.100] encapsulation dot1q vid 100

[SW3-GigabitEthernet0/0/2.100] bridge-domain 10

[SW3-GigabitEthernet0/0/2.100] quit

[SW3]interface GigabitEthernet0/0/2.200 mode l2

[SW3-GigabitEthernet0/0/2.200] encapsulation dot1q vid 200

[SW3-GigabitEthernet0/0/2.200] bridge-domain 20

[SW3-GigabitEthernet0/0/2.200] quit

# Configure ACC-1.

[ACC-1]vlan 100

[ACC-1]interface GigabitEthernet0/0/24

[ACC-1-GigabitEthernet0/0/24] port link-type access

[ACC-1-GigabitEthernet0/0/24] port default vlan 100

[ACC-1-GigabitEthernet0/0/24] quit

[ACC-1]interface GigabitEthernet0/0/1

[ACC-1-GigabitEthernet0/0/1] port link-type trunk

[ACC-1-GigabitEthernet0/0/1] port trunk allow-pass vlan 100

[ACC-1-GigabitEthernet0/0/1] quit

# Configure ACC-2.

[ACC-2]vlan batch 100 200

[ACC-2]interface GigabitEthernet0/0/24

[ACC-2-GigabitEthernet0/0/24] port link-type access

[ACC-2-GigabitEthernet0/0/24] port default vlan 100

[ACC-2-GigabitEthernet0/0/24] quit

[ACC-2]interface GigabitEthernet0/0/23

[ACC-2-GigabitEthernet0/0/23] port link-type access

[ACC-2-GigabitEthernet0/0/23] port default vlan 200

[ACC-2-GigabitEthernet0/0/23] quit

[ACC-2]interface GigabitEthernet0/0/1

[ACC-2-GigabitEthernet0/0/1] port link-type trunk

[ACC-2-GigabitEthernet0/0/1] port trunk allow-pass vlan 100 200

[ACC-2-GigabitEthernet0/0/1] quit

Create a VBDIF interface and configure an NVE interface.

Create NVE interfaces on SW1 and SW3, and set the ingress replication list protocol to BGP.

# On SW1, configure an NVE interface.

[SW1]interface Nve1

[SW1-Nve1] source 10.0.1.1

[SW1-Nve1] vni 1000 head-end peer-list protocol bgp

[SW1-Nve1] vni 2000 head-end peer-list protocol bgp

[SW1-Nve1] quit

# On SW3, configure an NVE interface.

[SW3]interface Nve1

[SW3-Nve1] source 10.0.3.3

[SW3-Nve1] vni 1000 head-end peer-list protocol bgp

[SW3-Nve1] vni 2000 head-end peer-list protocol bgp

[SW3-Nve1] quit

# On SW1, create VBDIF 10 and VBDIF 20.

[SW1]interface Vbdif10

[SW1-Vbdif10] ip address 172.16.1.254 255.255.255.0

[SW1-Vbdif10] mac-address 0000-0000-0010

[SW1-Vbdif10] arp collect host enable

[SW1-Vbdif10] quit

[SW1]interface Vbdif 20

[SW1-Vbdif20] ip address 172.16.2.254 255.255.255.0

[SW1-Vbdif20] mac-address 0000-0000-0020

[SW1-Vbdif20] arp collect host enable

[SW1-Vbdif20] quit

Create VBDIF10 and VBDIF20 on SW1 and SW3. To ensure that the MAC addresses of the gateway remain unchanged after the hosts and VMs are migrated and that the hosts and VMs do not need to learn ARP entries again, you need to manually change the MAC addresses of the interfaces to 0000-0000-0010 and 0000-0000-0020 respectively and run the arp collect host enable command on the VBDIF interface to generate Type 2 routes based on ARP information of hosts and transmit them through BGP EVPN.

# On SW3, create VBDIF 10 and VBDIF 20.

[SW3]interface Vbdif10

[SW3-Vbdif10] ip address 172.16.1.254 255.255.255.0

[SW3-Vbdif10] mac-address 0000-0000-0010

[SW3-Vbdif10] arp collect host enable

[SW3-Vbdif10] quit

[SW3]interface Vbdif20

[SW3-Vbdif20] ip address 172.16.2.254 255.255.255.0

[SW3-Vbdif20] mac-address 0000-0000-0020

[SW3-Vbdif20] arp collect host enable

[SW3-Vbdif20] quit

Configure an EVPN instance and an IP VPN instance, and bind these instances to BDs and VBDIF interfaces.

Create an EVPN instance and an IP VPN instance on SW1 and SW3, and bind the instances to BDs and VBDIF interfaces. To ensure that EVPN routes and IP routes can normally learn from each other, plan EVPN and IP VPN instances as follows.

EVPN instance plan on SW1

| EVPN Instance Name | BD | RD | Export RT | Import RT |
| --- | --- | --- | --- | --- |
| SW1_BD_10 | 10 | 21:10 | 213:10, 313:12 | 213:10 |
| SW1_BD_20 | 20 | 21:20 | 213:20, 313:12 | 213:20 |

EVPN instance plan on SW3

| EVPN Instance Name | BD | RD | Export RT | Import RT |
| --- | --- | --- | --- | --- |
| SW3_BD_10 | 10 | 23:10 | 213:10, 313:12 | 213:10 |
| SW3_BD_20 | 20 | 23:20 | 213:20, 313:12 | 213:20 |

EVPN instance plan on SW3

| VPN Instance name | VBDIF | L3 VNI | RD | Export RT | Import RT |
| --- | --- | --- | --- | --- | --- |
| SW1_VPN_10_20 | 10 | 1020 | 31:12 | 313:12 | 313:12 |
|  | 20 |  |  |  |  |

EVPN instance plan on SW3

| VPN Instance Name | VBDIF | L3 VNI | RD | Export RT | Import RT |
| --- | --- | --- | --- | --- | --- |
| SW3_VPN_10_20 | 10 | 1020 | 33:12 | 313:12 | 313:12 |
|  | 20 |  |  |  |  |

About EVPN instance planning:

- The RD value is 2Y:Z, where 2 indicates an EVPN instance, Y indicates the device ID, and Z indicates the BD value. Take BD 10 on SW1 as an example, where the RD value for the corresponding EVPN instance is 21:10.

- The Import RT value is 213:Z, where 2 indicates an EVPN instance, 13 indicates that the RT value is used to transmit EVPN routes between SW1 and SW3, and Z indicates the BD value. Take BD 10 on SW1 as an example, where the Import RT value for the corresponding EVPN instance is 213:10.

- There are two Export RT values. One is 213:Z, which is the same as that of the Import RT value, and the other is 313:12, which is used to import IP routes to the IP VPN instance routing table.

About IP VPN instance planning:

- The RD value is 3X:12, where 3 indicates an IP VPN instance, X indicates the device ID, and 12 indicates the IP VPN instance is used by VBDIF 10 and VBDIF 20.

- Both the Export RT and Import RT values are 313:12, where 3 indicates an IP VPN instance, 13 indicates that the IP VPN instance is used between SW1 and SW3, and 12 indicates VBDIF 10 and VBDIF 20.

- The L3 VNI value is 1020, indicating that a Layer 3 VNI is used for communication between BDs 10 and 20.

Pay attention to the following points when planning EVPN and IP VPN instances:

For network segments that need to communicate at Layer 3, their VBDIF interfaces must be bound to the same IP VPN instance, which is similar to MPLS VPN. In addition, the Export RT value of the EVPN instance must be the same as the Import RT value of the IP VPN instance. In this way, the routing entries carried in Type 2 routes can be added to the routing table of the corresponding IP VPN instance.

In this lab, terminals in BD 10 and BD 20 need to communicate with each other at Layer 3. To this end, VBDIF 10 and VBDIF 20 are bound to the same IP VPN instance on both SW1 and SW3.

The following figure helps you understand the precautions for planning EVPN and IP VPN instances.

# On SW1, create an EVPN instance and bind it to a BD.

[SW1]evpn vpn-instance SW1_BD_10 bd-mode

[SW1-evpn-instance-SW1_BD_10] route-distinguisher 21:10

[SW1-evpn-instance-SW1_BD_10] vpn-target 213:10 export-extcommunity

[SW1-evpn-instance-SW1_BD_10] vpn-target 313:12 export-extcommunity

[SW1-evpn-instance-SW1_BD_10] vpn-target 213:10 import-extcommunity

[SW1-evpn-instance-SW1_BD_10] quit

[SW1]evpn vpn-instance SW1_BD_20 bd-mode

[SW1-evpn-instance-SW1_BD_20] route-distinguisher 21:20

[SW1-evpn-instance-SW1_BD_20] vpn-target 213:20 export-extcommunity

[SW1-evpn-instance-SW1_BD_20] vpn-target 313:12 export-extcommunity

[SW1-evpn-instance-SW1_BD_20] vpn-target 213:20 import-extcommunity

[SW1-evpn-instance-SW1_BD_20] quit

[SW1]bridge-domain 10

[SW1-bd10]evpn binding vpn-instance SW1_BD_10

[SW1-bd10]quit

[SW1]bridge-domain 20

[SW1-bd20]evpn binding vpn-instance SW1_BD_20

[SW1-bd20]quit

# On SW3, create an EVPN instance and bind it to a BD.

[SW3]evpn vpn-instance SW3_BD_10 bd-mode

[SW3-evpn-instance-SW3_BD_10] route-distinguisher 23:10

[SW3-evpn-instance-SW3_BD_10] vpn-target 213:10 export-extcommunity

[SW3-evpn-instance-SW3_BD_10] vpn-target 313:12 export-extcommunity

[SW3-evpn-instance-SW3_BD_10] vpn-target 213:10 import-extcommunity

[SW3-evpn-instance-SW3_BD_10] quit

[SW3]evpn vpn-instance SW3_BD_20 bd-mode

[SW3-evpn-instance-SW3_BD_20] route-distinguisher 23:20

[SW3-evpn-instance-SW3_BD_20] vpn-target 213:20 export-extcommunity

[SW3-evpn-instance-SW3_BD_20] vpn-target 313:12 export-extcommunity

[SW3-evpn-instance-SW3_BD_20] vpn-target 213:20 import-extcommunity

[SW3-evpn-instance-SW3_BD_20] quit

[SW3]bridge-domain 10

[SW3-bd10]evpn binding vpn-instance SW3_BD_10

[SW3-bd10]quit

[SW3]bridge-domain 20

[SW3-bd20]evpn binding vpn-instance SW3_BD_20

[SW3-bd20]quit

# On SW1, create an IP VPN instance and bind it to a VBDIF.

[SW1]ip vpn-instance SW1_VPN_10_20

[SW1-vpn-instance-SW1_VPN_10_20] ipv4-family

[SW1-vpn-instance-SW1_VPN_10_20-af-ipv4] route-distinguisher 31:12

[SW1-vpn-instance-SW1_VPN_10_20-af-ipv4] vpn-target 313:12 export-extcommunity evpn

[SW1-vpn-instance-SW1_VPN_10_20-af-ipv4] vpn-target 313:12 import-extcommunity evpn

[SW1-vpn-instance-SW1_VPN_10_20-af-ipv4] quit

[SW1-vpn-instance-SW1_VPN_10_20] vxlan vni 1020

[SW1-vpn-instance-SW1_VPN_10_20] quit

[SW1]interface Vbdif 10

[SW1-Vbdif10]ip binding vpn-instance SW1_VPN_10_20

[SW1-Vbdif10]quit

[SW1]interface Vbdif 20

[SW1-Vbdif20]ip binding vpn-instance SW1_VPN_10_20

[SW1-Vbdif20]quit

After an IP VPN instance is bound to an interface, the IP address configuration of the interface will be cleared. In this case, you need to reconfigure an IP address for the interface.

[SW1]interface Vbdif10

[SW1-Vbdif10] ip address 172.16.1.254 255.255.255.0

[SW1-Vbdif10] quit

[SW1]interface Vbdif 20

[SW1-Vbdif20] ip address 172.16.2.254 255.255.255.0

[SW1-Vbdif20] quit

# On SW3, create an IP VPN instance and bind it to a VBDIF.

[SW3]ip vpn-instance SW3_VPN_10_20

[SW3-vpn-instance-SW3_VPN_10_20] ipv4-family

[SW3-vpn-instance-SW3_VPN_10_20-af-ipv4] route-distinguisher 33:12

[SW3-vpn-instance-SW3_VPN_10_20-af-ipv4] vpn-target 313:12 export-extcommunity

[SW3-vpn-instance-SW3_VPN_10_20-af-ipv4] vpn-target 313:12 export-extcommunity evpn

[SW3-vpn-instance-SW3_VPN_10_20-af-ipv4] vpn-target 313:12 import-extcommunity

[SW3-vpn-instance-SW3_VPN_10_20-af-ipv4] vpn-target 313:12 import-extcommunity evpn

[SW3-vpn-instance-SW3_VPN_10_20-af-ipv4] quit

[SW3-vpn-instance-SW3_VPN_10_20] vxlan vni 1020

[SW3-vpn-instance-SW3_VPN_10_20] quit

[SW3]interface Vbdif 10

[SW3-Vbdif10]ip binding vpn-instance SW3_VPN_10_20

[SW3-Vbdif10]quit

[SW3]interface Vbdif 20

[SW3-Vbdif20]ip binding vpn-instance SW3_VPN_10_20

[SW3-Vbdif20]quit

After an IP VPN instance is bound to an interface, the IP address configuration of the interface will be cleared. In this case, you need to reconfigure an IP address for the interface.

[SW3]interface Vbdif10

[SW3-Vbdif10] ip address 172.16.1.254 255.255.255.0

[SW3-Vbdif10] quit

[SW3]interface Vbdif20

[SW3-Vbdif20] ip address 172.16.2.254 255.255.255.0

[SW3-Vbdif20] quit

Establish a BGP EVPN peer relationship.

Establish an IBGP peer relationship between SW1 and SW3, use the IP address of loopback interface as the source IP address of the BGP session, enable IRB route advertisement in the EVPN address family, and enable L2VPN EVPN in the VPN instance.

# On SW1, configure BGP.

[SW1]bgp 100

[SW1-bgp] router-id 10.0.1.1

[SW1-bgp] peer 10.0.3.3 as-number 100

[SW1-bgp] peer 10.0.3.3 connect-interface LoopBack0

[SW1-bgp] ipv4-family vpn-instance SW1_VPN_10_20

[SW1-bgp-SW1_VPN_10_20] advertise l2vpn evpn

[SW1-bgp-SW1_VPN_10_20] quit

[SW1-bgp] l2vpn-family evpn

[SW1-bgp-af-evpn] policy vpn-target

[SW1-bgp-af-evpn] peer 10.0.3.3 enable

[SW1-bgp-af-evpn] peer 10.0.3.3 advertise irb

[SW1-bgp-af-evpn] quit

# On SW3, configure BGP.

[SW3]bgp 100

[SW3-bgp] router-id 10.0.3.3

[SW3-bgp] peer 10.0.1.1 as-number 100

[SW3-bgp] peer 10.0.1.1 connect-interface LoopBack0

[SW3-bgp] ipv4-family vpn-instance SW3_VPN_10_20

[SW3-bgp-SW3_VPN_10_20] advertise l2vpn evpn

[SW3-bgp-SW3_VPN_10_20] quit

[SW3-bgp] l2vpn-family evpn

[SW3-bgp-af-evpn] policy vpn-target

[SW3-bgp-af-evpn] peer 10.0.1.1 enable

[SW3-bgp-af-evpn] peer 10.0.1.1 advertise irb

[SW3-bgp-af-evpn] quit

# On SW1 and SW3, check the BGP EVPN peer status.

[SW1]display bgp evpn peer

BGP local router ID        : 10.0.1.1

Local AS number         : 100

Total number of peers     : 1

Peers in established state    : 1

Peer           V         AS    MsgRcvd  MsgSent  OutQ  Up/Down       State      PrefRcv

10.0.3.3        4         100       12       12     0     		00:02:25     Established    6

[SW3-bgp]display bgp evpn peer

BGP local router ID        : 10.0.3.3

Local AS number          : 100

Total number of peers      : 1

Peers in established state    : 1

Peer         V         AS    MsgRcvd  MsgSent  OutQ  Up/Down       State     PrefRcv

10.0.1.1       4         100       14       14     0    	00:03:55      Established     6

The command output shows that the BGP peer relationship has been established between SW1 and SW3.

Analyze the forwarding process.

On PC1, ping PC2 on the same network segment as PC1 and also ping PC3 on a different network segment from PC1 to check VXLAN tunnel establishment using BGP EVPN, the ingress replication list, the intra-subnet forwarding process, and the inter-subnet forwarding process.

# Use PC1, PC2, and PC3 to ping their gateways.

PC1>ping 172.16.1.254

Ping 172.16.1.254: 32 data bytes, Press Ctrl_C to break

From 172.16.1.254: bytes=32 seq=1 ttl=255 time=32 ms

From 172.16.1.254: bytes=32 seq=2 ttl=255 time=31 ms

From 172.16.1.254: bytes=32 seq=3 ttl=255 time=31 ms

From 172.16.1.254: bytes=32 seq=4 ttl=255 time=47 ms

From 172.16.1.254: bytes=32 seq=5 ttl=255 time=31 ms

--- 172.16.1.254 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 31/34/47 ms

PC2>ping 172.16.1.254

Ping 172.16.1.254: 32 data bytes, Press Ctrl_C to break

From 172.16.1.254: bytes=32 seq=1 ttl=255 time=47 ms

From 172.16.1.254: bytes=32 seq=2 ttl=255 time=32 ms

From 172.16.1.254: bytes=32 seq=3 ttl=255 time=31 ms

From 172.16.1.254: bytes=32 seq=4 ttl=255 time=31 ms

From 172.16.1.254: bytes=32 seq=5 ttl=255 time=31 ms

--- 172.16.1.254 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 31/34/47 ms

PC3>ping 172.16.2.254

Ping 172.16.2.254: 32 data bytes, Press Ctrl_C to break

From 172.16.2.254: bytes=32 seq=1 ttl=255 time=47 ms

From 172.16.2.254: bytes=32 seq=2 ttl=255 time=32 ms

From 172.16.2.254: bytes=32 seq=3 ttl=255 time=31 ms

From 172.16.2.254: bytes=32 seq=4 ttl=255 time=47 ms

From 172.16.2.254: bytes=32 seq=5 ttl=255 time=31 ms

--- 172.16.2.254 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 31/37/47 ms

In this case, PC1, PC2, and PC3 can communicate with the gateway address of the VBDIF interface.

# Check the ARP entries of PC1 and PC3.

PC1>arp -a

Internet Address      Physical Address    Type

172.16.1.254        00-00-00-00-00-10   dynamic

PC3>arp -a

Internet Address      Physical Address    Type

172.16.2.254        00-00-00-00-00-20   dynamic

The command output shows that the gateway MAC address is the MAC address manually configured on the VBDIF interface.

# Check VXLAN tunnel information on SW1.

<SW1>display vxlan tunnel

Number of vxlan tunnel : 1

Tunnel ID   Source           Destination          State     Type     Uptime

-----------------------------------------------------------------------------------------------------

4026531842  10.0.1.1          10.0.3.3              up     dynamic    00:18:27

The command output shows that the VXLAN tunnel type is dynamic. This means that the VXLAN tunnel is dynamically generated based on BGP EVPN Type 3 routes.

# Check Type 3 BGP EVPN routes on SW1.

<SW1>display bgp evpn all routing-table inclusive-route

Local AS number : 100

BGP  Local router  ID is 10.0.1.1

Status codes: * - valid, > - best, d - damped, x - best external, a - add path,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

EVPN address family:

Number of Inclusive Multicast Routes: 4

Route Distinguisher: 21:10

Network(EthTagId/IpAddrLen/OriginalIp)                 NextHop

*>     0:32:10.0.1.1                                           0.0.0.0

Route Distinguisher: 21:20

Network(EthTagId/IpAddrLen/OriginalIp)                 NextHop

*>     0:32:10.0.1.1                                           0.0.0.0

Route Distinguisher: 23:10

Network(EthTagId/IpAddrLen/OriginalIp)                 NextHop

*>i    0:32:10.0.3.3                                           10.0.3.3

Route Distinguisher: 23:20

Network(EthTagId/IpAddrLen/OriginalIp)                 NextHop

*>i    0:32:10.0.3.3                                           10.0.3.3

The command output shows that there are two Type 3 routes from 10.0.3.3. After receiving these Type 3 routes, the device creates a Layer 2 ingress replication list.

# Check the VXLAN ingress replication list on SW1.

<SW1>display vxlan peer

Number of peers : 2

Vni ID    Source                 Destination           Type

-------------------------------------------------------------------------------

1000      10.0.1.1                10.0.3.3               dynamic

2000      10.0.1.1                10.0.3.3               dynamic

The command output shows that two dynamic ingress replication lists are created, with VNIs being 1000 and 2000, respectively.

# Analyze the process of communication between PC1 and PC2 (which is intra-subnet communication, excluding the ARP learning process).

PC1>arp -a

Internet Address      Physical Address    Type

172.16.1.2          54-89-98-96-2E-95   dynamic

On PC1, check ARP information about PC2 (172.16.1.2). It is found that the MAC address of PC2 is 54-89-98-96-2E-95.

# On SW1, check the MAC address entries in BD 10.

<SW1>display	mac-address 	bridge-domain	10

-------------------------------------------------------------------------------

MAC	Address						VLAN/VSI/BD			Learned-From								Type																Age

-------------------------------------------------------------------------------

5489-988d-3e41					-/-/10								GE0/0/2.100									dynamic															-

5489-9896-2e95					-/-/10								10.0.3.3															dynamic															-

-------------------------------------------------------------------------------

Total 	items:	4

The command output shows that the data frame with the destination MAC address being 54-89-98-96-2E-95 is received by SW1. Then, SW1 checks the destination MAC address, finding that the destination MAC address is not the MAC address of the local interface (the MAC address of the VBDIF interface is 0000-0000-0010 or 0000-0000-0020). Subsequently, SW1 searches the MAC address table in the corresponding BD, finding that there is a matching MAC address entry. This MAC address entry is generated based on the routing entry learned through BGP EVPN, and its outbound interface is the remote VTEP.

# Check BGP EVPN Type 2 routes on SW1.

<SW1>display bgp evpn  all routing-table  mac-route

Local AS number : 100

BGP Local router ID is 10.0.1.1

Status codes: * - valid, > - best, d - damped, x - best external, a - add path,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

EVPN address family:

Number of Mac Routes: 9

Route Distinguisher: 21:10

Network(EthTagId/MacAddrLen/MacAddr/IpAddrLen/IpAddr)  NextHop

*>     0:48:0000-0000-0010:0:0.0.0.0                               	0.0.0.0

*>     0:48:0000-0000-0010:32:172.16.1.254                         0.0.0.0

Route Distinguisher: 21:20

Network(EthTagId/MacAddrLen/MacAddr/IpAddrLen/IpAddr)  NextHop

*>     0:48:0000-0000-0020:0:0.0.0.0                               	0.0.0.0

*>     0:48:0000-0000-0020:32:172.16.2.254                         0.0.0.0

Route Distinguisher: 23:10

Network(EthTagId/MacAddrLen/MacAddr/IpAddrLen/IpAddr)  NextHop

*>i    0:48:0000-0000-0010:0:0.0.0.0                              	10.0.3.3

*>i    0:48:0000-0000-0010:32:172.16.1.254                        10.0.3.3

*>i    0:48:5489-9896-2e95:32:172.16.1.2                          	10.0.3.3

Route Distinguisher: 23:20

Network(EthTagId/MacAddrLen/MacAddr/IpAddrLen/IpAddr)  NextHop

*>i    0:48:0000-0000-0020:0:0.0.0.0                              	10.0.3.3

*>i    0:48:0000-0000-0020:32:172.16.2.254                        10.0.3.3

The command output shows that there is a Type 2 route from 10.0.3.3, which contains the MAC address of PC2.

# Check detailed information about this Type 2 route.

<SW1>display bgp evpn all routing-table mac-route 0:48:5489-9896-2e95:32:172.16.1.2

BGP local router ID : 10.0.1.1

Local AS number : 100

Total routes of Route Distinguisher(23:10): 1

BGP routing table entry information of 0:48:5489-9896-2e95:32:172.16.1.2:

Label information (Received/Applied): 1000 1020/NULL

From: 10.0.3.3 (10.0.3.3)

Route Duration: 0d01h29m04s

Relay IP Nexthop: 10.0.12.2

Relay Tunnel Out-Interface: VXLAN

Original nexthop: 10.0.3.3

Qos information : 0x0

Ext-Community: RT <213 : 10>, RT <313 : 12>, Tunnel Type <VxLan>, Router's MAC

<707b-e82a-0b83>

AS-path Nil, origin incomplete, localpref 100, pref-val 0, valid, internal, best, select, pre 255, IGP cost 2

Route Type: 2 (MAC Advertisement Route)

Ethernet Tag ID: 0, MAC Address/Len: 5489-9896-2e95/48, IP Address/Len: 172.16.1.2/32, ESI:0000.0000.0000.0000.0000

Not advertised to any peer yet

In the label information of this Type 2 route, it is found that the first VXLAN VNI is L2 VNI (1000) and the second VXLAN VNI is L3 VNI (1020). The RT values are 213:10 and 313:12.

This route is generated by SW3 based on the host ARP information in BD 10. (After the arp collect host enable command is run on the VBDIF interface, SW3 converts the learned ARP information of a downstream terminal into an EVPN Type 2 route and sends the route out.) The route carries the host IP address, host MAC address, L2 VNI, and L3 VNI.

At this moment, SW1 performs L2 forwarding. That is, SW1 encapsulates the data frame into the VXLAN header based on the result of searching the MAC address table, and then forwards the VXLAN packet to SW3. The VNI carried in the VXLAN packet is L2 VNI, which is the VNI (1000) bound to BD 10.

# On SW3, perform mirroring to obtain the packets of ICMP request from PC1.

Ethernet II, Src: HuaweiTe_43:27:7d (70:7b:e8:43:27:7d), Dst: HuaweiTe_2a:0b:83 (70:7b:e8:2a:0b:83)

802.1Q Virtual LAN, PRI: 0, DEI: 0, ID: 23

Internet Protocol Version 4, Src: 10.0.1.1, Dst: 10.0.3.3

User Datagram Protocol, Src Port: 4789, Dst Port: 4789

Virtual eXtensible Local Area Network

Flags: 0x0800, VXLAN Network ID (VNI)

Group Policy ID: 0

VXLAN Network Identifier (VNI): 1000

Reserved: 0

Ethernet II, Src: HuaweiTe_8d:3e:41 (54:89:98:8d:3e:41), Dst: HuaweiTe_96:2e:95 (54:89:98:96:2e:95)

Internet Protocol Version 4, Src: 172.16.1.1, Dst: 172.16.1.2

Internet Control Message Protocol

The command output shows that the VNI in the VXLAN header is 1000 and the destination MAC address is 54:89:98:96:2e:95, which are the same as those in intra-subnet communication when a VXLAN tunnel is manually established.

After receiving the data frame, SW3 checks the destination MAC address in the outer data frame and determines that it needs to decapsulate the data frame locally. SW3 then finds that the network-layer destination IP address is the IP address of its Loopback0 interface. Subsequently, SW3 further parses the data frame to check the upper-layer data, and based on VXLAN VNI 1000, determines that it needs to process the packet in BD 10. After that, SW3 checks the destination MAC address of the inner data frame, finding that the destination MAC address is 54:89:98:96:2e:95, which is not the MAC address of any local Layer 3 interface. SW3 then searches the MAC address table in BD 10. According to the search result, SW3 forwards the packet to ACC-2, which then sends it to PC2.

# Analyze the process of communication between PC1 and PC3 (which is inter-subnet communication).

[SW1]display ip routing-table vpn-instance SW1_VPN_10_20 172.16.2.2

Proto: Protocol        Pre: Preference

Route Flags: R - relay, D - download to fib, T - to vpn-instance, B - black hole route

------------------------------------------------------------------------------

Routing Table : SW1_VPN_10_20

Summary Count : 1

Destination/Mask    Proto    Pre  Cost        Flags NextHop         Interface

172.16.2.2/32   IBGP    255  0         RD   10.0.3.3           VXLAN

PC1 and PC3 communicate across subnets. PC1 sends packets to the gateway (VBDIF 10). In this case, the destination MAC address is 0000-0000-0010, which is the MAC address of any local interface on SW1. Then, SW1 checks the network-layer destination IP address, finding that the IP address is not the IP address of a local interface, so SW1 proceeds to search the routing table.

Because the Layer 3 interface (VBDIF 10) that receives packets is bound to the IP VPN instance SW1_VPN_10_20, SW1 searches for a route in the IP VPN instance. The search result is an IBGP route with the next hop being 10.0.3.3 and outbound interface being the logical interface VXLAN. In this case, SW1 encapsulates the data frame in the VXLAN header and sends it to 10.0.3.3. The value of VNI in the VXLAN header is determined by checking the BGP EVPN routing entry.

# Check BGP VPNv4 routes on SW1.

[SW1]display bgp vpnv4 vpn-instance SW1_VPN_10_20 routing-table 172.16.2.2

BGP local router ID : 10.0.1.1

Local AS number : 100

VPN-Instance SW1_VPN_10_20, Router ID 10.0.1.1:

Paths:   1 available, 1 best, 1 select, 0 best-external, 0 add-path

BGP routing table entry information of 172.16.2.2/32:

Route Distinguisher: 31:12

Evpn route: Type 2, irb

Label information (Received/Applied): 1020/NULL

From: 10.0.3.3 (10.0.3.3)

Route Duration: 0d00h10m43s

Relay Tunnel Out-Interface: VXLAN

Original nexthop: 10.0.3.3

Qos information : 0x0

Ext-Community: RT <213 : 20>, RT <313 : 12>, Tunnel Type <VxLan>, Router's Mac

<707B-E82A-0B83>

AS-path Nil, origin incomplete, localpref 100, pref-val 0, valid, internal, best, select, pre 255

Not advertised to any peer yet

In the BGP VPNv4 routing entry, it is found that the type of the routing entry 172.16.2.2 is an EVPN Type IRB route. The label (L3 VNI) value is 1020, and the router's MAC address is 707B-E82A-0B83. In this case, SW1 needs to re-encapsulate the inner data frame, with the destination MAC address being the router's MAC address, indicating that SW3 needs to perform Layer 3 routing table lookup before forwarding.

The BGP VPNv4 route is generated based on the BGP EVPN Type 2 route. Next, check the BGP EVPN Type 2 route.

# Check the BGP EVPN Type 2 route on SW1.

[SW1]display bgp evpn all routing-table mac-route

Local AS number : 100

BGP Local router ID is 10.0.1.1

Status codes: * - valid, > - best, d - damped, x - best external, a - add path,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

EVPN address family:

Number of Mac Routes: 11

Route Distinguisher: 21:10

Network(EthTagId/MacAddrLen/MacAddr/IpAddrLen/IpAddr)  NextHop

*>     0:48:0000-0000-0010:0:0.0.0.0                              		0.0.0.0

*>     0:48:0000-0000-0010:32:172.16.1.254                        0.0.0.0

*>     0:48:5489-988d-3e41:32:172.16.1.1                          	0.0.0.0

Route Distinguisher: 21:20

Network(EthTagId/MacAddrLen/MacAddr/IpAddrLen/IpAddr)  NextHop

*>     0:48:0000-0000-0020:0:0.0.0.0                              		0.0.0.0

*>     0:48:0000-0000-0020:32:172.16.2.254                        0.0.0.0

Route Distinguisher: 23:10

Network(EthTagId/MacAddrLen/MacAddr/IpAddrLen/IpAddr)  NextHop

*>i    0:48:0000-0000-0010:0:0.0.0.0                             	 	10.0.3.3

*>i    0:48:0000-0000-0010:32:172.16.1.254                        10.0.3.3

*>i    0:48:5489-9896-2e95:32:172.16.1.2                          	10.0.3.3

Route Distinguisher: 23:20

Network(EthTagId/MacAddrLen/MacAddr/IpAddrLen/IpAddr)  NextHop

*>i    0:48:0000-0000-0020:0:0.0.0.0                              		10.0.3.3

*>i    0:48:0000-0000-0020:32:172.16.2.254                        	10.0.3.3

*>i    0:48:5489-98c2-107a:32:172.16.2.2                          		10.0.3.3

The command output shows that there is a Type 2 route from 10.0.3.3. The route contains the host MAC address (5489-98c2-107a), host IP address (172.16.2.2), and RD value (23:20). According to the RD value, we can determine that the route is generated based on the host ARP entry learned by VBDIF 20 after the arp collect host enable command is run on SW3.

# Check detailed information about the Type 2 route 0:48:5489-98c2-107a:32:172.16.2.2.

<SW1>display bgp evpn all routing-table mac-route 0:48:5489-98c2-107a:32:172.16.2.2

BGP  local  router  ID : 10.0.1.1

Local AS number : 100

Total routes of Route Distinguisher(23:20): 1

BGP routing table entry information of 0:48:5489-98c2-107a:32:172.16.2.2:

Label information (Received/Applied): 2000 1020/NULL

From: 10.0.3.3 (10.0.3.3)

Route Duration: 0d00h23m09s

Relay IP Nexthop: 10.0.12.2

Relay Tunnel Out-Interface: VXLAN

Original nexthop: 10.0.3.3

Qos information : 0x0

Ext-Community: RT <213 : 20>, RT <313 : 12>, Tunnel Type <VxLan>, Router's MAC <707b-e82a-0b83>

AS-path Nil, origin incomplete, localpref 100, pref-val 0, valid, internal, best, select, pre 255, IGP cost 2

Route Type: 2 (MAC Advertisement  Route)

Ethernet Tag ID: 0, MAC Address/Len: 5489-98c2-107a/48, IP Address/Len: 172.16.2.2/32, ESI:0000.0000.0000.0000.0000

Not advertised to any peer yet

The command output shows that the route carries the L2 and L3 VNIs, as well as the Export RT value bound to the EVPN instance on SW3. After receiving the route, SW1 compares the Export RT value with the Import RT values of the local EVPN instance and IP VPN instance. The comparison result shows that there is a matching EVPN instance and IP VPN instance SW1_VPN_10_20 bound to BD20. Because the route carries the host IP address and host MAC address, SW1 adds the host MAC address to the local MAC address table of BD 20 and adds the host IP address to the routing table of the corresponding IP VPN instance.

By controlling the RT values, SW1 learns MAC address entries and IP routes through Type 2 IRB routes.

# On SW3, perform mirroring to obtain the packets of ICMP request from PC1.

Ethernet II, Src: HuaweiTe_43:27:7d (70:7b:e8:43:27:7d), Dst: HuaweiTe_2a:0b:83 (70:7b:e8:2a:0b:83)

802.1Q Virtual LAN, PRI: 0, DEI: 0, ID: 23

Internet Protocol Version 4, Src: 10.0.1.1, Dst: 10.0.3.3

User Datagram Protocol, Src Port: 4789, Dst Port: 4789

Virtual eXtensible Local Area Network

Flags: 0x0800, VXLAN Network ID (VNI)

Group Policy ID: 0

VXLAN Network Identifier (VNI): 1020

Reserved: 0

Ethernet II, Src: 00:00:00_00:00:10 (00:00:00:00:00:10), Dst: HuaweiTe_2a:0b:83 (70:7b:e8:2a:0b:83)

Internet Protocol Version 4, Src: 172.16.1.1, Dst: 172.16.2.2

Internet Control Message Protocol

The packets sent from SW1 to SW3 are forwarded at Layer 3. The VNI carried in the packets is the VXLAN VNI configured in the IP VPN instance bound to VBDIF 20 on SW3. The source MAC address of the inner data frame is the MAC address (0000-0000-0010) of VBDIF 10 on SW1, and the destination MAC address is the router MAC address carried in the BGP EVPN route. Similar to the bridge MAC address of the device, this destination MAC address is used to identify the device itself and is also the MAC address of the NVE logical interface of the device.

# Check the MAC address of the interface NVE1 on SW3.

<SW3>display interface Nve 1

Nve1 current state : UP (ifindex: 19)

Line protocol current state : UP

Description:

IP Sending Frames' Format is PKTFMT_ETHNT_2, Hardware address is 707b-e82a-0b83

After receiving the data frame, SW3 checks the destination MAC address in the inner data frame and determines that it needs to process such data frame locally. Then, SW3 checks the network-layer IP address of the inner data frame and finds that it needs to perform Layer 3 table lookup before forwarding. Then, SW3 searches the routing table in the IP VPN instance corresponding to the VNI (1020) carried in the VXLAN packet.

# Check the routing table in the IP VPN instance SW3_VPN_10_20 on SW3.

<SW3>display ip routing-table vpn-instance SW3_VPN_10_20 172.16.2.2

Proto: Protocol        Pre: Preference

Route Flags: R - relay, D - download to fib, T - to vpn-instance, B - black hole

route

------------------------------------------------------------------------------

Routing Table : SW3_VPN_10_20

Summary Count : 1

Destination/Mask    Proto   Pre  Cost        Flags NextHop       Interface

172.16.2.0/24  	Direct  			0    0           D   172.16.2.254    Vbdif20

SW3 searches the Layer 3 routing table in the IP VPN instance SW3_VPN_10_20. The search result is a direct route generated by VBDIF 20.

# Search the ARP table and BD 20's MAC address table.

<SW3>display arp | in 172.16.2.2

ARP Entry Types: D - Dynamic, S - Static, I - Interface, O - OpenFlow, RD - Redi

rect

EXP: Expire-time VLAN:VLAN or Bridge Domain

IP ADDRESS   MAC ADDRESS      EXP(M)   TYPE/VLAN      INTERFACE       VPN-INSTANCE

----------------------------------------------------------------------------------------

172.16.2.254    0000-0000-0020              I                Vbdif20          SW3_VPN_10_20

172.16.2.2      5489-98c2-107a        5     D/BD20         GE0/0/2.200	      SW3_VPN_10_20

----------------------------------------------------------------------------------------

Total:6         Dynamic:3       Static:0    Interface:3    OpenFlow:0

Redirect:0

<SW3>display mac-address 5489-98c2-107a

Flags: * - Backup

BD   : bridge-domain   Age : dynamic MAC learned time in seconds

-------------------------------------------------------------------------------

MAC Address      VLAN/VSI/BD   Learned-From        Type                Age

-------------------------------------------------------------------------------

5489-98c2-107a    -/-/20         GE0/0/2.200	         dynamic               -

According to the search result, the data frame is re-encapsulated and sent out from GE0/0/2.200. Then, ACC-2 looks up the Layer 2 forwarding table and forwards the data frame accordingly.

In this case, the destination MAC address of the data frame is PC3's real MAC address, and the source MAC address is VBDIF 20's MAC address 0000-0000-0020.

# On SW3, perform mirroring to obtain the packets of pinging PC3 from PC1 (SW3 sends packets to PC3).

Ethernet II, Src: 00:00:00_00:00:20 (00:00:00:00:00:20), Dst: HuaweiTe_c2:10:7a (54:89:98:c2:10:7a)

802.1Q Virtual LAN, PRI: 0, DEI: 0, ID: 200

Internet Protocol Version 4, Src: 172.16.1.1, Dst: 172.16.2.2

Internet Control Message Protocol

The command output shows that the source MAC address is the MAC address (0000-0000-0020) of VBDIF 20.

By now, the packet forwarding process from PC1 to PC3 is complete. In the entire process, both the ingress VTEP (SW1) and egress VTEP (SW3) search the Layer 3 routing table and forward the packets based on the search result. When packets are forwarded between VTEPs, the VNI carried in the inner data frame is an L3 VNI.

The egress VTEP generates a Type 2 route based on the ARP information learned by VBDIF 20. When the route is transmitted to the ingress VTEP, it carries the VNI value configured for the IP VPN instance on the VBDIF interface. The VNI is an L3 VNI.

----End

Configuration Reference

Configurations of ACC-1

#

sysname ACC-1

#

vlan batch 100

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/2

port link-type access

port default vlan 100

#

Return

Configurations of ACC-2

#

sysname ACC-2

#

vlan batch 100 200

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100 200

#

interface GigabitEthernet0/0/24

port link-type access

port default vlan 200

#

interface GigabitEthernet0/0/23

port link-type access

port default vlan 100

#

Return

Configurations of SW1

#

sysname SW1

#

vlan batch 12 100

#

ip vpn-instance SW1_VPN_10_20

ipv4-family

route-distinguisher 31:12

vpn-target 313:12 export-extcommunity evpn

vpn-target 313:12 import-extcommunity evpn

vxlan vni 1020

#

evpn vpn-instance SW1_BD_10 bd-mode

route-distinguisher 21:10

vpn-target 213:10 export-extcommunity

vpn-target 313:12 export-extcommunity

vpn-target 213:10 import-extcommunity

quit

#

evpn vpn-instance SW1_BD_20 bd-mode

route-distinguisher 21:20

vpn-target 213:20 export-extcommunity

vpn-target 313:12 export-extcommunity

vpn-target 213:20 import-extcommunity

#

bridge-domain 10

vxlan vni 1000

evpn binding vpn-instance SW1_BD_10

#

bridge-domain 20

vxlan vni 2000

evpn binding vpn-instance SW1_BD_20

#

interface Vbdif10

ip binding vpn-instance SW1_VPN_10_20

ip address 172.16.1.254 255.255.255.0

mac-address 0000-0000-0010

arp collect host enable

#

interface Vbdif20

ip binding vpn-instance SW1_VPN_10_20

ip address 172.16.2.254 255.255.255.0

mac-address 0000-0000-0020

arp collect host enable

#

interface Vlanif12

ip address 10.0.12.1 255.255.255.0

#

interface GigabitEthernet0/0/2.100 mode l2

encapsulation dot1q vid 100

bridge-domain 10

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 12

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

interface Nve1

source 10.0.1.1

vni 1000 head-end peer-list protocol bgp

vni 2000 head-end peer-list protocol bgp

#

bgp 100

router-id 10.0.1.1

peer 10.0.3.3 as-number 100

peer 10.0.3.3 connect-interface LoopBack0

#

ipv4-family unicast

peer 10.0.3.3 enable

#

ipv4-family vpn-instance SW1_VPN_10_20

advertise l2vpn evpn

#

l2vpn-family evpn

policy vpn-target

peer 10.0.3.3 enable

peer 10.0.3.3 advertise irb

#

ospf 1 router-id 10.0.1.1

area 0.0.0.0

network 10.0.1.1 0.0.0.0

network 10.0.12.0 0.0.0.255

#

Return

Configurations of SW2

#

sysname SW2

#

vlan batch 12 23

#

interface Vlanif12

ip address 10.0.12.2 255.255.255.0

#

interface Vlanif23

ip address 10.0.23.2 255.255.255.0

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 12

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 23

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.12.0 0.0.0.255

network 10.0.23.0 0.0.0.255

#

Return

Configurations of SW3

#

sysname SW3

#

vlan batch 23 100

#

ip vpn-instance SW3_VPN_10_20

ipv4-family

route-distinguisher 33:12

vpn-target 313:12 export-extcommunity evpn

vpn-target 313:12 import-extcommunity evpn

vxlan vni 1020

#

evpn vpn-instance SW3_BD_10 bd-mode

route-distinguisher 23:10

vpn-target 213:10 export-extcommunity

vpn-target 313:12 export-extcommunity

vpn-target 213:10 import-extcommunity

quit

#

evpn vpn-instance SW3_BD_20 bd-mode

route-distinguisher 23:20

vpn-target 213:20 export-extcommunity

vpn-target 313:12 export-extcommunity

vpn-target 213:20 import-extcommunity

quit

#

bridge-domain 10

vxlan vni 1000

evpn binding vpn-instance SW3_BD_10

#

bridge-domain 20

vxlan vni 2000

evpn binding vpn-instance SW3_BD_20

#

interface Vbdif10

ip binding vpn-instance SW3_VPN_10_20

ip address 172.16.1.254 255.255.255.0

mac-address 0000-0000-0010

arp collect host enable

#

interface Vbdif20

ip binding vpn-instance SW3_VPN_10_20

ip address 172.16.2.254 255.255.255.0

mac-address 0000-0000-0020

arp collect host enable

#

interface GigabitEthernet0/0/2.100 mode l2

encapsulation dot1q vid 100

bridge-domain 10

#

interface GigabitEthernet0/0/2.200 mode l2

encapsulation dot1q vid 200

bridge-domain 20

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

interface Nve1

source 10.0.3.3

vni 1000 head-end peer-list protocol bgp

vni 2000 head-end peer-list protocol bgp

#

interface NULL0

#

bgp 100

router-id 10.0.3.3

peer 10.0.1.1 as-number 100

peer 10.0.1.1 connect-interface LoopBack0

#

ipv4-family unicast

peer 10.0.1.1 enable

#

ipv4-family vpn-instance SW3_VPN_10_20

advertise l2vpn evpn

#

l2vpn-family evpn

policy vpn-target

peer 10.0.1.1 enable

peer 10.0.1.1 advertise irb

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.23.0 0.0.0.255

#

return

Quiz

What is the advantage of distributed gateways compared with centralized gateways？

Campus Multi-Branch Interconnection Technology

Introduction

About This Lab

This lab guides you through how to configure IPsec VPN between two AR routers to implement communication between sites.

Objectives

Upon completion of this lab, you will be able to:

Understand the basic principles of IPsec VPN.

Configure IPsec VPN through the CLI.

Networking Topology

WAN interconnection topology

AR1 and AR2 function as the egresses of the headquarters and a branch respectively, and SW1 connects to the Internet.

The IP addresses of AR1 and AR2 are shown in the figure.

AR1 and AR2 have their own network segments: 172.16.1.0/24 and 172.16.2.0/24. The two network segments are used for intranet terminals.

Establish an IPsec VPN between AR1 and AR2 based on IKE to implement communication between intranet network segments.

Lab Configuration

Configuration Roadmap

- Complete connectivity configuration.

- Create IKE proposal 1 by setting the authentication mode to PSK, authentication algorithm to MD5, encryption algorithm to 3DES, and DH-Group to 1.

- Create an IKE peer by setting the IKE version to 1, exchange-mode to main, and PSK to Huawei@123, as well as referencing IKE proposal 1.

- Create IPsec proposal 1 by setting the encapsulation mode to tunnel, encapsulation protocol to ESP, encryption algorithm to 3DES, and authentication algorithm to SHA1.

- Create ACLs on AR1 and AR2 to match the traffic from the local network segment of intranet users to the remote network segment.

- Create an IPsec policy in ISAKMP mode, reference the created ACLs, IKE peer, and IPsec proposal, and apply the created IPsec policy to egress interfaces (AR1: GE0/0/0; AR2: GE0/0/8).

Configuration Procedure

          - Complete connectivity configuration.

Configure AR1 and AR2 to complete basic connectivity configuration.

# Name the devices.

The configuration details are not provided.

# Configure IP addresses for interfaces on AR1.

[AR1]interface LoopBack0

[AR1-LoopBack0] ip address 172.16.1.1 255.255.255.0

[AR1-LoopBack0] quit

[AR1]interface GigabitEthernet0/0/0

[AR1-GigabitEthernet0/0/0] ip address 67.0.0.1 255.255.255.0

[AR1-GigabitEthernet0/0/0] quit

# Configure IP addresses for interfaces on AR2.

[AR2]interface LoopBack0

[AR2-LoopBack0] ip address 172.16.2.1 255.255.255.0

[AR2-LoopBack0] quit

[AR2]interface GigabitEthernet0/0/8

[AR2-GigabitEthernet0/0/8] ip address 67.0.0.2 255.255.255.0

[AR2-GigabitEthernet0/0/8] quit

Static IP addresses have been bound to AR1 and AR2 during lab environment preconfiguration. You can also use dynamic IP address obtaining.

# Configure default routes on AR1 and AR2.

[AR1]ip route-static 172.16.2.0 24 67.0.0.2

[AR2]ip route-static 172.16.1.0 24 67.0.0.1

Configure a route to the peer private network segment.

Configure an IKE proposal.

Create IKE proposal 1 on AR1 and AR2. The following table lists the parameters.

IKE proposal parameters

| Parameter | Setting |
| --- | --- |
| Authentication mode | PSK |
| Authentication algorithm | MD5 |
| Encryption algorithm | 3DES |
| DH-Group | 1 |

# Configure IKE proposal 1 on AR1.

[AR1]ike proposal 1

[AR1-ike-proposal-1] authentication-method pre-share

[AR1-ike-proposal-1] authentication-algorithm md5

[AR1-ike-proposal-1] encryption-algorithm 3des-cbc

[AR1-ike-proposal-1] dh group1

# Configure IKE proposal 2 on AR2.

[AR2]ike proposal 1

[AR2-ike-proposal-1] authentication-method pre-share

[AR2-ike-proposal-1] authentication-algorithm md5

[AR2-ike-proposal-1] encryption-algorithm 3des-cbc

[AR2-ike-proposal-1] dh group1

Configure an IKE peer.

Create an IKE peer on AR1 and AR2. The following table lists the parameters.

IKE peer parameters

| Parameter | Setting |
| --- | --- |
| IKE version | Version 1 |
| Exchange-mode | Main |
| PSK | Huawei@123 |
| IKE proposal to be invoked | 1 |
| Remote address | IP address of the remote interface |

# Configure an IKE peer on AR1.

[AR1]ike peer 1

[AR1-ike-peer-1] version 1

[AR1-ike-peer-1] exchange-mode main

[AR1-ike-peer-1] pre-shared-key cipher Huawei@123

[AR1-ike-peer-1] ike-proposal 1

[AR1-ike-peer-1] remote-address 67.0.0.2

# Configure an IKE peer on AR2.

[AR2]ike peer 1

[AR2-ike-peer-1] version 1

[AR2-ike-peer-1] exchange-mode main

[AR2-ike-peer-1] pre-shared-key cipher Huawei@123

[AR2-ike-peer-1] ike-proposal 1

[AR2-ike-peer-1] remote-address 67.0.0.1

Configure an IPsec proposal and a security ACL.

Create a security ACL and IPsec proposal 1 on AR1 and AR2. The following table lists the parameters of the IPsec proposal.

IPsec proposal parameters

| Parameter | Setting |
| --- | --- |
| Encapsulation mode | Tunnel |
| Encapsulation protocol | ESP |
| Encryption algorithm | 3DES |
| Authentication algorithm | SHA1 |

# Configure IPsec proposal 1 on AR1.

[AR1]ipsec proposal 1

[AR1-ipsec-proposal-1] encapsulation-mode tunnel

[AR1-ipsec-proposal-1] esp authentication-algorithm sha1

[AR1-ipsec-proposal-1] esp encryption-algorithm 3des

[AR1-ipsec-proposal-1] quit

# Configure IPsec proposal 1 on AR2.

[AR2]ipsec proposal 1

[AR2-ipsec-proposal-1] encapsulation-mode tunnel

[AR2-ipsec-proposal-1] esp authentication-algorithm sha1

[AR2-ipsec-proposal-1] esp encryption-algorithm 3des

[AR2-ipsec-proposal-1] quit

#Create a security ACL.

[AR1]acl number 3001

[AR1-acl-adv-3001] rule 1 permit ip source 172.16.1.0 0.0.0.255 destination 172.16.2.0 0.0.0.255

[AR2]acl number 3001

[AR2-acl-adv-3001]rule 1 permit ip source 172.16.2.0 0.0.0.255 destination 172.16.1.0 0.0.0.255

The ACL matches traffic that has the following attributes: The source is the network segment where the local Loopback0 interface resides, and the destination is the network segment where the remote Loopback0 interface resides. This is used to simulate traffic exchanged between intranet users.

Create an IPsec policy and apply it to the egress interface for connecting to an external network.

Create an IPsec policy in ISAKMP mode, reference the created security ACL, IKE peer 1 and IPsec proposal 1, and then apply the IPsec policy to the interface connected to the Internet.

# Create an IPsec policy.

[AR1]ipsec policy 1 10 isakmp

[AR1-ipsec-policy-isakmp-1-10] security acl 3001

[AR1-ipsec-policy-isakmp-1-10] ike-peer 1

[AR1-ipsec-policy-isakmp-1-10] proposal 1

[AR1-ipsec-policy-isakmp-1-10] quit

[AR2]ipsec policy 1 10 isakmp

[AR2-ipsec-policy-isakmp-1-10] security acl 3001

[AR2-ipsec-policy-isakmp-1-10] ike-peer 1

[AR2-ipsec-policy-isakmp-1-10] proposal 1

[AR2-ipsec-policy-isakmp-1-10] quit

# Apply the IPsec policy.

[AR1]interface GigabitEthernet0/0/0

[AR1-GigabitEthernet0/0/0] ipsec policy 1

[AR1-GigabitEthernet0/0/0] quit

[AR2]interface GigabitEthernet0/0/8

[AR2-GigabitEthernet0/0/8]ipsec policy 1

Verify the configuration.

Ping AR2 from AR1 to generate traffic matching the security ACL and trigger IPsec negotiation. Then, check IPsec information.

# On AR1, trigger IPsec negotiation through traffic.

[AR1]ping -a 172.16.1.1 172.16.2.1

PING 172.16.2.1: 56  data bytes, press CTRL_C to break

Request time out

Reply from 172.16.2.1: bytes=56 Sequence=2 ttl=255 time=1 ms

Reply from 172.16.2.1: bytes=56 Sequence=3 ttl=255 time=1 ms

Reply from 172.16.2.1: bytes=56 Sequence=4 ttl=255 time=1 ms

Reply from 172.16.2.1: bytes=56 Sequence=5 ttl=255 time=1 ms

Generally, the first and second packets may fail to be forwarded, because negotiation is not complete and there is no IKE SA or IPsec SA. This is a normal phenomenon.

# On AR1, check IKE SA and IPsec SA information.

[AR1] display ike sa

IKE SA information :

Conn-ID   Peer           VPN   Flag(s)   Phase  RemoteType  RemoteID

--------------------------------------------------------------------------------

3        	67.0.0.2:500       					RD|ST     v1:2    		IP          67.0.0.2

2        	67.0.0.2:500       					RD|ST     v1:1   	 		IP          	67.0.0.2

Number of IKE SA : 2

--------------------------------------------------------------------------------

RD--READY   ST--STAYALIVE   RL--REPLACED   FD--FADING   TO--TIMEOUT

HRT--HEARTBEAT   LKG--LAST KNOWN GOOD SEQ NO.   BCK--BACKED UP

M--ACTIVE   S--STANDBY   A--ALONE  NEG--NEGOTIATING

The command output shows that both phase-1 and phase-2 IKE SAs exist.

# On AR1, check IPsec SA information.

<AR1>display	ipsec sa

===============================

Interface:	GigabitEthernet0/0/0

Path	MTU:	0

===============================

-----------------------------

IPSec	policy	name		:	"1"

Sequence	number		:	10

Acl	Group										:	3001

Acl	rule												:	1

Mode													:	ISAKMP

-----------------------------

Connection	ID								:	3

Encapsulation	mode			:	Tunnel

Tunnel	local										:	67.0.0.1

Tunnel	remote								:	67.0.0.2

Flow	 source											:	172.16.1.1/255.255.255.255	0/0

Flow	 destination						:	172.16.2.1/255.255.255.255	0/0

Qos	pre-classify							:	Disable

[Outbound	ESP	SAs]

SPI:	3373058988	(0xc90ccbac)

Proposal:	ESP-ENCRYPT-3DES-192	ESP-AUTH-SHA1

SA	remaining	key	duration	(bytes/sec):	0/1560

Max	sent	sequence-number:	0

UDP	encapsulation	used	for	NAT	traversal:	N

[Inbound	ESP	SAs]

SPI:	1399745699	(0x536e6ca3)

Proposal:	ESP-ENCRYPT-3DES-192	ESP-AUTH-SHA1

SA	remaining	key	duration	(bytes/sec):	0/1560

Max	received	sequence-number:	0

Anti-replay	window	size:

UDP	encapsulation	used	for	NAT	traversal:	N

The command output shows that the IPsec SA has been established. The outbound SPI is 3373058988 and the inbound SPI is 1399745699.

When AR1 transmits data to AR2 through the IPsec VPN, the ESP packet carries the outbound SPI (that is, 3373058988). When AR2 transmits data to AR1 through the IPsec VPN, the ESP packet carries the inbound SPI (that is, 1399745699).

Based on the SPI in a packet, IPsec can determine the SA to which the packet belongs and decapsulate the packet accordingly.

----End

Configuration Reference

Configurations of AR1

#

sysname AR1

#

acl number 3001

rule 1 permit ip source 172.16.1.0 0.0.0.255 destination 172.16.2.0 0.0.0.255

#

ipsec proposal 1

esp authentication-algorithm sha1

esp encryption-algorithm 3des

#

ike proposal 1

encryption-algorithm 3des-cbc

authentication-algorithm md5

#

ike peer 1

pre-shared-key cipher 1f$A=6KZy$7S230wKjSG~.2#

ike-proposal 1

remote-address 67.0.0.2

#

ipsec policy 1 10 isakmp

security acl 3001

ike-peer 1

proposal 1

#

interface GigabitEthernet0/0/0

ip address 67.0.0.1 255.255.255.0

ipsec policy 1

#

interface LoopBack0

ip address 172.16.1.1 255.255.255.255

#

ip route-static 172.16.2.0 24 67.0.0.2

#

return

Configuration of AR2

sysname AR2

#

acl number 3001

rule 1 permit ip source 172.16.2.0 0.0.0.255 destination 172.16.1.0 0.0.0.255

#

ipsec proposal 1

esp authentication-algorithm sha1

esp encryption-algorithm 3des

#

ike proposal 1

encryption-algorithm 3des-cbc

authentication-algorithm md5

#

ike peer 1

pre-shared-key cipher 1f$A=6KZy$7S230wKjSG~.2#

ike-proposal 1

remote-address 67.0.0.1

#

ipsec policy 1 10 isakmp

security acl 3001

ike-peer 1

proposal 1

#

interface GigabitEthernet0/0/8

ip address 67.0.0.2 255.255.255.0

ipsec policy 1

#

interface LoopBack0

ip address 172.16.2.1 255.255.255.255

#

ip route-static 172.16.1.0 24 67.0.0.1

#

Return

Quiz

Should ESP or AH be chosen for scenarios with high data transmission security?

VXLAN-based Campus Network Deployment

Introduction

About This Lab

This document is a demo of the CloudCampus Virtualized Campus Network Solution for the HCIP-Datacom-Campus Network Planning and Deployment. It provides guidance for trainees to independently complete operations in this scenario, including device onboarding, automatic underlay network deployment, fabric deployment, overlay network deployment, deployment of free mobility and access authentication, and WLAN service deployment.

Objectives

After completing this lab, you will be able to understand the process of deploying the CloudCampus VXLAN-based Virtualized Campus Network Solution, including:

The process of creating sites and onboarding devices.

The process of creating a fabric, deploying resources, and automatically deploying an underlay network.

The process of deploying an overlay network.

The process of deployment free mobility and access control.

The process of deploying WLAN services.

Networking Topology

Topology of the CloudCampus Virtualized Campus Network

As shown in the figure, the network consists of two parts: HQ and cloud.

HQ: ACC1 and ACC2 function as access devices that connect to wired terminals and provide network services for wired users. An AP is connected to ACC2 to provide network services for wireless users. AGG1 and AGG2 serve as aggregation devices, and CORE1 is the core device. AR1 functions as both the campus egress and the DHCP server that allocates IP addresses to other devices and user terminals at the HQ.

Cloud: SW1 is used to simulate the cloud. It connects the HQ and iMaster NCE-Campus, and also functions as the gateway of iMaster NCE-Campus.

VXLAN network: The network topology for this lab uses a distributed gateway model, in which VXLAN is deployed across core and access layers. ACC1 and ACC2 function as edge nodes of the VXLAN network, whereas CORE1 functions as the border node of the VXLAN network.

After completing this experiment, you can use iMaster NCE-Campus to centrally manage networks and services as well as deliver configurations to devices.

Data Plan

This section provides the plan for all the data required in subsequent steps. It is recommended that you get familiar with the data plan.

Preconfiguration Plan

AR1 obtains the IP address of iMaster NCE-Campus through static configuration. AR1 also functions as the DHCP server to provide IP addresses for downstream devices and user terminals, and notifies the downstream devices of the iMaster NCE-Campus IP address through DHCP Option 148.

VLAN port type and parameter plan

| Device | Port | Port Type | VLAN Parameters |
| --- | --- | --- | --- |
| AR1 | GE0/0/1 | Trunk | PVID: VLAN 100 Allow-Pass VLAN: 100, 120, 121, 122, 130 |

IP address plan

| Device | Port | IP Address |
| --- | --- | --- |
| iMaster NCE-Campus | / | 192.168.4.104 |
| AR1 | GE0/0/0 | 67.0.0.1/24 |
|  | VLANIF100 | 192.168.100.1/24 |

Device Onboarding Plan

On iMaster NCE-Campus, you can add devices to a site using their device models or ESNs.

Device ESN and role plan

| Device | Device ESN | Device Model | Site | Device Type | Role |
| --- | --- | --- | --- | --- | --- |
| CORE1 | 101990066905 | S5731-H24P4XC | HQ | LSW, WAC | Core |
| AGG1 | 101990066822 | S5731-H24P4XC |  | LSW, WAC | Aggregation |
| AGG2 | 101990066922 | S5731-H24P4XC |  | LSW, WAC | Aggregation |
| ACC1 | 101990066914 | S5731-H24P4XC |  | LSW, WAC | Access |
| ACC2 | 101990066909 | S5731-H24P4XC |  | LSW, WAC | Access |
| AP1 | 2102352UBR10L6001282 | AirEngine 5760-51 |  | AP | AP |

The ESNs and device models in the preceding table are for reference only. Change them to the actual ESNs and device models used in the lab.

Fabric and Underlay Network Plan

Before creating a fabric based on a physical network, you need to configure the resources used in the fabric, including the network resource pool (consisting of VLAN, IP address, BD, and VNI resources).

In addition, you need to plan external networks, network service resources, and templates (including server templates and authentication templates). This data will be used to create virtual networks and configure access management.

Network Resource Pool Plan

Plan for the global resource pool of the fabric

| Network Resource | Value Range | Description |
| --- | --- | --- |
| VLAN | 101-300 | Service VLAN ID pool, including the VLANs for connecting to external networks, VLANs for connecting to network service resources, policy association management VLAN, and terminal access VLAN. |
| Loopback Interface IP address | 1.1.1.0-1.1.1.0/24 | IP address of a loopback interface. Loopback interface IP addresses are used to establish BGP EVPN peer relationships when the underlay routing domain is automatically configured and the fabric is automatically connected to network service resources. |
| Bridge Domain (BD) | 1-1000 | Each BD is identified by a BD ID. iMaster NCE-Campus automatically selects a BD ID when delivering configurations to a device. |
| VXLAN Network Identifier (VNI) | 1-1000 | VNIs are used to distinguish isolation domains on a VXLAN network. |

Plan for the underlay automation resource pool

| Resource | Value Range | Description |
| --- | --- | --- |
| Interconnection VLAN | 10-20 | Used for the interconnection between the border and edge nodes in a fabric. |
| Interworking IP | 172.20.0.0-172.20.0.0/16 | Used for the interconnection between the border and edge nodes in a fabric. |

Fabric Network Plan

A fabric is a virtualized network on which VXLAN is deployed across core and access layers. It uses the distributed gateway networking mode.

Fabric network plan

| Parameter | Value |
| --- | --- |
| Name | HQ (customizable) |
| Networking Type | Distributed |
| Wireless authentication device | Edge as authentication control point for wireless users |
| Report terminal identification information | Off |
| Automatic routing domain configuration | On |
| Configuration Switch | On |
| Encryption | None (customizable) |
| Domain | Single area (customizable) |
| AS number | 100 (customizable) |
| RR cluster ID | 1 |

Role plan for fabric devices

| Device | Role |
| --- | --- |
| CORE1 | Border |
| AGG1 | / |
| AGG2 | / |
| ACC1 | Edge |
| ACC2 | Edge |

Plan for External Networks and Network Service Resources

Create three external networks for access of virtual networks OA_VN, RD_VN, and MKT_VN. Create DHCP network service resources, so access users on virtual networks OA_VN, RD_VN, and MKT_VN can obtain IP addresses.

Plan for the OA external network

| Parameter | Value |
| --- | --- |
| Connection between the fabric and external network | L3 exclusive egress |
| Name | OA |
| Internet connection | On |
| Border device | CORE1 |
| Interconnection port | GigabitEthernet0/0/1 |
| VLAN | 120 |
| IP address type | IPv4 |
| Local IPv4 address | 192.168.120.1 |
| Remote IPv4 address | 192.168.120.2 |
| IPv4 address mask: | 30 |

Plan for the RD external network

| Parameter | Value |
| --- | --- |
| Connection between the fabric and external network | L3 exclusive egress |
| Name | RD |
| Internet connection | Off |
| External service IP address | 172.16.12.0/24 |
| Border device | CORE1 |
| Interconnection port | GigabitEthernet0/0/1 |
| VLAN | 121 |
| IP address type | IPv4 |
| Local IPv4 address | 192.168.121.1 |
| Remote IPv4 address | 192.168.121.2 |
| IPv4 address mask: | 30 |

Plan for the MKT external network

| Parameter | Value |
| --- | --- |
| Connection between the fabric and external network | L3 exclusive egress |
| Name | MKT |
| Internet connection | Off |
| External service IP address | 172.16.13.0/24 |
| Border device | CORE1 |
| Interconnection port | GigabitEthernet0/0/1 |
| VLAN | 122 |
| IP address type | IPv4 |
| Local IPv4 address | 192.168.122.1 |
| Remote IPv4 address | 192.168.122.2 |
| IPv4 address mask: | 30 |

Plan for the network service resource - DHCP server

| Parameter | Value |
| --- | --- |
| Name | DHCP_Service |
| Server type | DHCP |
| DHCP Server | 192.168.130.2 |
| Server interconnection address pool | / |
| Scenario | Directly connect to a switch |
| Interconnection device | CORE1 |
| External Port | GigabitEthernet0/0/1 |
| External VLAN | 130 |
| Interconnection IPv4 address | 192.168.130.1 |
| Peer IPv4 address | 192.168.130.2 |
| Mask | 30 |

Access Management Plan

Configure authentication templates and authentication servers to authenticate access users. To be specific, configure 802.1X or MAC address authentication for wired users, and Portal authentication for wireless users.

ACC1 provides access only for wired users, whereas ACC2 provides access for both wired and wireless users.

Plan for the RADIUS server template

| Parameter | Value |
| --- | --- |
| Name | HQ_RADIUS_Server |
| Using Built-in Server | On |
| Key | Huawei@123 |

Plan for the Portal server template

| Parameter | Value |
| --- | --- |
| Name | HQ_Portal_Server |
| Using Built-in Server | On |
| Page push protocol | HTTPS |
| Key | Huawei@123 |

Plan for the 802.1X and MAC address authentication template

| Parameter | Value |
| --- | --- |
| Name | HQ_MAC_802.1X |
| Authentication mode | MAC, 802.1X |
| RADIUS server template | HQ_RADIUS_Server |

Plan for the Portal authentication template

| Parameter | Value |
| --- | --- |
| Name | HQ_Portal |
| Authentication mode | Portal |
| RADIUS server template | HQ_RADIUS_Server |
| Portal server template | HQ_Portal_Server |

Access management plan for ACC1

| Parameter | Value |
| --- | --- |
| Authentication Control Point | ACC1 |
| Authentication Control Point Management Parameter |  |
| Port Name | GigabitEthernet0/0/24 |
| Set Connected Device Type | Terminal (PC, Phone, dumb terminal, non-fabric extended switch/AP) |
| Set Authentication Template | HQ_MAC_802.1X |

Access management plan for ACC2

| Parameter | Value |
| --- | --- |
| Authentication Control Point | ACC2 |
| Authentication Control Point Management Parameter |  |
| Number of enforcement point devices | 126 (customizable) |
| Management VLAN of CAPWAP | 111 |
| Management IP address of CAPWAP | 172.111.1.1 |
| Port Name | GigabitEthernet0/0/23 |
| Set Connected Device Type | Extended AP |
| Set Authentication Template | -- |
| Port Name | GigabitEthernet0/0/24 |
| Set Connected Device Type | Terminal (PC, Phone, dumb terminal, non-fabric extended switch/AP) |
| Set Authentication Template | HQ_MAC_802.1X |
| Wireless Access Configuration |  |
| SSID Name | HQ_Guest |
| Authentication Template | HQ_Portal |

Plan for Overlay Network Automation

Create three virtual networks — OA_VN, RD_VN, and MKT_VN — and allow them to access external networks OA, RD, and MKT, respectively. Access users of the three virtual networks can obtain IP addresses from the external service resource (DHCP server). In addition, users of OA_VN can access only the OA external network; users of RD_VN and MKT_VN can not only access their respective external networks but also communicate with each other.

Virtual Network Plan

Data plan for the OA virtual network

| Parameter | Value |
| --- | --- |
| Create VN |  |
| Name | OA_VN (customizable) |
| User gateway location | Inside the fabric |
| User-defined VRF name | OA (customizable) |
| External network | OA |
| Network service resources | DHCP_Service |
| User gateway: Manually Specified |  |
| OA | OA (customizable) |
| VLAN Type | Dynamic VLAN |
| VLAN | 101 |
| IPv4 subnet | 172.16.1.0/24 |
| IPv4 gateway address | 172.16.1.1 |
| User access: Wired access |  |
| Service name | OA (customizable) |
| Device Name: ACC1 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |
| Device Name: ACC2 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |
| User access: Wireless access |  |
| Site | HQ |
| Device Name | ACC2 |

Data plan for the RD virtual network

| Parameter | Value |
| --- | --- |
| Create VN |  |
| Name | RD_VN (customizable) |
| User gateway location | Inside the fabric |
| User-defined VRF name | RD (customizable) |
| External network | RD |
| Network service resources | DHCP_Service |
| User gateway: Manually Specified |  |
| OA | RD (customizable) |
| VLAN Type | Dynamic VLAN |
| VLAN | 102 |
| IPv4 subnet | 172.16.2.0/24 |
| IPv4 gateway address | 172.16.2.1 |
| User access: Wired access |  |
| Service name | RD (customizable) |
| Device Name: ACC1 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |
| Device Name: ACC2 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |

Data plan for the MKT virtual network

| Parameter | Value |
| --- | --- |
| Create VN |  |
| Name | MKT_VN (customizable) |
| User gateway location | Inside the fabric |
| User-defined VRF name | MKT (customizable) |
| External network | MKT |
| Network service resources | DHCP_Service |
| User gateway: Manually Specified |  |
| OA | MKT (customizable) |
| VLAN Type | Dynamic VLAN |
| VLAN | 103 |
| IPv4 subnet | 172.16.3.0/24 |
| IPv4 gateway address | 172.16.3.1 |
| User access: Wired access |  |
| Service name | MKT (customizable) |
| Device Name: ACC1 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |
| Device Name: ACC2 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |

Plan for Communication Between Virtual Networks

Plan for communication between virtual networks

| Parameter | Value |
| --- | --- |
| Name | RD_to_MKT (customizable) |
| Interconnection device | CORE1 |
| Interworking Mode | Partial Inerwork |
| IP address type | IPv4 |
| Source VN | RD |
| Source IPv4 Prefix | 172.16.12.0/24 |
| Destination VN | MKT |
| Destination IPv4 Prefix | 172.16.13.0/24 |

Plan for Free Mobility and Access Authentication

Free Mobility Plan

Create three security groups: Guest_Group, Research_Group, and Sales_Group. They are access users of the virtual networks OA_VN, RD_VN, and MKT_VN, respectively.

Users of OA_VN can only access the OA external network via wireless access after passing Portal authentication. Users of RD_VN and MKT_VN can not only access their respective external networks but also communicate with each other, after passing 802.1X or MAC address authentication.

Policy control matrix plan

| Destination Security Group Source Security Group | Guest_Group | Research_Group | Sales_Group |
| --- | --- | --- | --- |
| Guest_Group |  | Status: Enable Default: Deny | Status: Enable Default: Deny |
| Research_Group | Status: Enable Default: Permit |  | Status: Enable Default: Permit |
| Sales_Group | Status: Enable Default: Permit | Status: Enable Default: Permit |  |
| unknown |  | Status: Enable Default: Deny | Status: Enable Default: Deny |

Access Authentication Plan

Plan for users' authentication accounts

| Parameter | Value |
| --- | --- |
| Guest_Group user (Portal authentication) |  |
| User name | Guest |
| Password & Comfirm password | Huawei@123 |
| Change password upon next login | Off |
| Research_Group user (802.1X authentication) |  |
| User name | Research |
| Password & Comfirm password | Huawei@123 |
| Change password upon next login | Off |
| Sales_Group user (802.1X authentication) |  |
| User name | Sales |
| Password & Comfirm password | Huawei@123 |
| Change password upon next login | Off |

Authorization result plan

| Parameter | Value |
| --- | --- |
| Guest_Group user (Portal authentication) |  |
| Name | Guest_Result |
| Security group | Guest_Group |
| VLAN | 101 |
| Research_Group user (802.1X authentication) |  |
| Name | Research_Result |
| Security group | Research_Group |
| VLAN | 102 |
| Sales_Group user (802.1X authentication) |  |
| Name | Sales_Result |
| Security group | Sales_Group |
| VLAN | 103 |

Authorization rule plan

| Parameter | Value |
| --- | --- |
| Guest_Group user (Portal authentication) |  |
| Name | Guest_Rule |
| Authentication Mode | User access authentication |
| Enabling the Portal-HACA Protocol | Off |
| Access Mode | Wireless |
| Account matching | On Add a user named Guest. |
| Authorization result | Guest_Result |
| Research_Group user (802.1X authentication) |  |
| Name | Research_Rule |
| Authentication Mode | User access authentication |
| Enabling the Portal-HACA Protocol | Off |
| Access Mode | Wired |
| Account matching | On Add a user named Research. |
| Authorization result | Research_Result |
| Sales_Group user (802.1X authentication) |  |
| Name | Sales_Rule |
| Authentication Mode | User access authentication |
| Enabling the Portal-HACA Protocol | Off |
| Access Mode | Wired |
| Account matching | On Add a user named Sales. |
| Authorization result | Sales_Result |

WLAN Service Plan

Configure WLAN services, so that APs and wireless users can go online and wireless users can access the OA external network.

WLAN service plan

| Parameter | Value |
| --- | --- |
| Management VLAN | 111 |
| Service VLAN | 101 |
| DHCP server | ACC2 functions as a DHCP server to assign IP addresses to APs. ACC2 functions as a DHCP relay agent to assign IP addresses to STAs. |
| IP address pool for APs | 172.111.1.0/23, with gateway 172.111.1.1 |
| IP address pool for STAs | 172.16.1.0/24, with gateway 172.16.1.1 |
| AC's source interface IP address | VLANIF 111: 172.111.1.1 |
| AP Group | Default Referenced profile: VAP profile HQ_VAP WLAN ID: 1 |
| Regulatory Domain Profile | Default |
| SSID Profile | Name: HQ_SSID SSID name: HQ_Guest |
| Security Profile | Default |
| VAP Profile | Name: HQ_VAP Service VLAN: VLAN 101 Forwarding mode: Tunnel Referenced profiles: SSID profile HQ_SSID and authentication profile HQ_Portal |

Configure the CAPWAP management VLAN and CAPWAP management IP address. That is, create the management VLAN and CAPWAP tunnel source interface in the WLAN service, and assign an IP address to the source interface. The DHCP server assigns IP addresses from the interface address pool to APs. After the preceding configurations are complete, APs can go online successfully.

Lab Configuration

Configuration Roadmap

- Complete pre-configuration, so that AR1 and iMaster NCE-Campus can communicate with each other. Configure AR1 as the DHCP server for devices and end users and configure the simulated external network. Ensure that there are reachable routes between AR1 and the external network.

- Create a site named HQ, and onboard all the devices (CORE1, AGG1, AGG2, ACC1, and ACC2) at the site (APs are currently not onboarded).

- Configure fabric and underlay resources, create a fabric, and automatically deploy the underlay network. In addition, configure external networks, network service resources, and access management, and onboard APs.

- Complete automatic deployment of the overlay network, including creating virtual networks and enabling communication between virtual networks.

- Complete free mobility deployment, including creation of security groups and the policy control matrix.

- Complete access control deployment, including user account creation, authentication and authorization configuration, and page management. Authentication and authorization configuration includes configuration of authentication rules, authorization results, and authorization rules.

- Deploy WLAN services.

Preconfiguring the Environment

Summary of Pre-configurations

- AR1: Complete pre-configurations such as VLAN assignment, VLANIF interface creation, IP routing, LLDP enable, DHCP server, and simulated external networks.

- There are no pre-configurations for other devices, and iMaster NCE-Campus is also not configured.

Pre-configuring the DHCP Server (for Device Plug-and-Play)

AR1 functions as a DHCP server to allocate management IP addresses to the devices at the HQ site. In addition, AR1 needs to notify these devices of the iMaster NCE-Campus address.

          - Configure VLANs.

# Create VLAN 100 on AR1.

<Huawei>sys

[Huawei]sysname AR1

[AR1]vlan 100

# Configure the type of AR1's downlink interface and the VLAN to which this interface belongs.

[AR1]interface GigabitEthernet 0/0/1

[AR1-GigabitEthernet0/0/1]portswitch

[AR1-GigabitEthernet0/0/1]port link-type trunk

[AR1-GigabitEthernet0/0/1]port trunk pvid vlan 100

[AR1-GigabitEthernet0/0/1]port trunk allow-pass vlan 100

[AR1-GigabitEthernet0/0/1]quit

Configure IP addresses.

# Create VLANIF 100 on AR1 and configure an IP address for VLANIF 100.

[AR1]interface Vlanif 100

[AR1-Vlanif100]ip address 192.168.100.1 24

#Configure an IP address for AR1's uplink interface.

[AR1]interface GigabitEthernet 0/0/0

[AR1-GigabitEthernet0/0/0]ip address 67.0.0.1 24

Configure the DHCP server.

# Enable the DHCP service on AR1, and configure it to assign management IP addresses from the interface address pool to the HQ devices.

[AR1]dhcp enable

[AR1]interface Vlanif 100

[AR1-Vlanif100]dhcp select interface

[AR1-Vlanif100]description HQ_Gateway

Specify the controller.

# Configure AR1 to notify HQ devices of the iMaster NCE-Campus IP address through DHCP Option 148.

[AR1]interface Vlanif 100

[AR1-Vlanif100]dhcp server option 148 ascii agilemode=agile-cloud;ap-agilemode=tradition-fit;agilemanage-mode=ip;agilemanage-domain=192.168.4.104;agilemanage-port=10020

Configure routing.

# Create a static route on AR1 so that AR1 can access iMaster NCE-Campus.

[AR1]ip route-static 0.0.0.0 0 67.0.0.254

# Create a static route on SW1 so that iMaster NCE-Campus can access the HQ internal network.

[SW1]ip route-static 192.168.100.0 24 67.0.0.1

----End

Pre-configuring the DHCP Server (for End Users)

AR1 functions as a DHCP server to assign IP addresses to users at the HQ site. In addition, AR1 also implements communication between access users and iMaster NCE-Campus.

          - Configure the DHCP service for access users of the OA, RD, and MKT networks.

# Configure an address pool for the OA network.

[AR1]ip pool OA

[AR1-ip-pool-OA]gateway-list 172.16.1.1

[AR1-ip-pool-OA]network 172.16.1.0 mask 255.255.255.0

[AR1-ip-pool-OA]dns-list 114.114.114.114

# Configure an address pool for the RD network.

[AR1]ip pool RD

[AR1-ip-pool-RD]gateway-list 172.16.2.1

[AR1-ip-pool-RD]network 172.16.2.0 mask 255.255.255.0

[AR1-ip-pool-RD]dns-list 114.114.114.114

# Configure an address pool for the MKT network.

[AR1]ip pool MKT

[AR1-ip-pool-MKT]gateway-list 172.16.3.1

[AR1-ip-pool-MKT]network 172.16.3.0 mask 255.255.255.0

[AR1-ip-pool-MKT]dns-list 114.114.114.114

# Create VLANIF 130, enable DHCP on the interface, and configure the DHCP server to assign IP addresses from the global address pool.

[AR1]vlan 130

[AR1-vlan130]quit

[AR1]interface Vlanif 130

[AR1-Vlanif130]ip address 192.168.130.2 30

[AR1-Vlanif130]dhcp select global

Configure VLANs.

# Configure GE0/0/1 on AR1 to allow packets from VLAN 130 to communicate with the fabric network.

[AR1]interface GigabitEthernet 0/0/1

[AR1-GigabitEthernet0/0/1]port trunk allow-pass vlan 130

[AR1-GigabitEthernet0/0/1]quit

Configure routing.

# Create a static route on AR1 so that AR1 can communicate with the access user network.

[AR1]ip route-static 172.16.0.0 16 192.168.130.1

# Create static routes on SW1 so that SW1 (iMaster NCE-Campus) can communicate with the access user network.

[SW1]ip route-static 172.16.1.0 24 67.0.0.1

[SW1]ip route-static 172.16.2.0 24 67.0.0.1

[SW1]ip route-static 172.16.3.0 24 67.0.0.1

----End

Pre-configuring Simulated External Networks

AR1 needs to simulate external networks for the HQ site, including the Internet, RD external network, and MKT external network.

          - Configure the simulated Internet.

# Create the interface LoopBack1 to simulate the Internet.

[AR1]interface LoopBack 1

[AR1-LoopBack1]ip address 30.1.1.1 32

# Create VLAN 120 and configure an IP address for the interconnection interface VLANIF 120.

[AR1]vlan 120

[AR1-vlan120]quit

[AR1]interface Vlanif 120

[AR1-Vlanif120]ip address 192.168.120.2 30

Configure the RD simulated network.

# Create the interface LoopBack2 to simulate the RD network.

[AR1]interface LoopBack 2

[AR1-LoopBack2]ip address 172.16.12.1 24

# Create VLAN 121 and configure an IP address for the interconnection interface VLANIF 121.

[AR1]vlan 121

[AR1-vlan121]quit

[AR1]interface Vlanif 121

[AR1-Vlanif121]ip address 192.168.121.2 30

Configure the MKT simulated network.

# Create the interface LoopBack3 to simulate the MKT network.

[AR1]interface LoopBack 3

[AR1-LoopBack3]ip address 172.16.13.1 24

# Create VLAN 122 and configure an IP address for the interconnection interface VLANIF 122.

[AR1]vlan 122

[AR1-vlan122]quit

[AR1]interface Vlanif 122

[AR1-Vlanif122]ip address 192.168.122.2 30

Configure VLANs.

# Configure GE0/0/1 on AR1 to allow packets from VLAN 120, VLAN 121, VLAN 122 to communicate with the fabric network.

[AR1]interface GigabitEthernet 0/0/1

[AR1-GigabitEthernet0/0/1]port trunk allow-pass vlan 120 to 122

[AR1-GigabitEthernet0/0/1]quit

----End

Enable LLDP on AR1

Enable LLDP on AR1 so that the controller can discover links between the fabric and external networks.

[AR1]lldp enable

----End

(Optional) Configuration Deletion

Before using the iMaster NCE-Campus controller to manage devices, you need to clear the configurations that have been performed in the environment so that the environment can remain in the initial state before subsequent experiments.

For details, see section 1.4.

Note: Skip this step if the lab environment is in the initialization state.

Creating a Site and Onboarding Devices

In an actual project, there are a large number of switches on a network. After being powered on, the switches need to be initialized and configured with basic functions such as IP connectivity. This initialization process is called deployment. Traditional deployment requires massive amounts of manual operation, which is inefficient. The CloudCampus Solution supports Zero Touch Provisioning (ZTP) of network devices — also known as device plug-and-play. With this function, the devices will be automatically deployed after they are powered on and connected to the network with factory settings, greatly reducing O&M and management costs.

Currently, two deployment scenarios are supported: onboarding before configuration and configuration before onboarding. This document describes the first one.

          - # Check the device ESN.

# Check the ESN of CORE1. (Repeat this step to check the ESNs of the other devices.)

<CORE1>display esn

ESN of device: 101990066905

An ESN is used to add a device on iMaster NCE-Campus. You can obtain an ESN from the device label or obtain an ESN by running the corresponding command.

Plan for device ESNs and roles

| Name | ESN | Device Model | Site | Device Type | Role |
| --- | --- | --- | --- | --- | --- |
| CORE1 | 101990066905 | S5731-H24P4XC | HQ | LSW, WAC | Core |
| AGG1 | 101990066822 | S5731-H24P4XC |  | LSW, WAC | Aggregation |
| AGG2 | 101990066922 | S5731-H24P4XC |  | LSW, WAC | Aggregation |
| ACC1 | 101990066914 | S5731-H24P4XC |  | LSW, WAC | Access |
| ACC2 | 101990066909 | S5731-H24P4XC |  | LSW, WAC | Access |
| AP1 | 2102352UBR10L6001282 | AirEngine 5760-51 |  | AP | AP |

Log in to iMaster NCE-Campus and choose Deployment Scenario.

# Log in to iMaster NCE-Campus and click Fabric Network. The Fabric network configuration page is displayed.

Create a site and add devices.

Log in to the O&M page of iMaster NCE-Campus using a tenant account, create the HQ site, and add devices to the site.

# On the Site Design page, click Create.

# Create a site named HQ and add all devices using their models or ESNs.

Set Site Name to HQ, and set Device type to LSW and WAC (if there are other devices on the network, select the corresponding device types). Click By Model, enter the device type, model, quantity, and role, and then click OK.

# Click By ESN, add the AP, change the device names and ESNs of other devices, and then click Apply and Deploy.

Check device registration status.

# Choose Fabric Network > Device Management. On the device management page that is displayed, the device status is normal (or indicates that an alarm exists), because the devices have been initialized. The devices can successfully register with iMaster NCE-Campus after a site is created and they are added to iMaster NCE-Campus.

The Fit AP named AP1 in the topology cannot get onboarded for the time being, because access management is not configured. The following sections will describe how to configure access management and WLAN services.

# Check the physical network topology.

Choose Design > Physical Topology. On the page that is displayed, view the physical network topology.

----End

Fabric and Underlay Network Automation

Configuring Network Resources

          - Configure a global resource pool for the fabric.

Before creating a fabric on top of the physical network, you need to configure required network resources (such as VLANs, IP addresses, BDs, and VNIs) as well as templates (such as server templates and authentication templates).

Plan for the global resource pool of the fabric

| Network Resource | Value Range | Description |
| --- | --- | --- |
| VLAN | 101-300 | Service VLAN ID pool, including the VLANs for connecting to external networks, VLANs for connecting to network service resources, CAPWAP management VLAN, and access VLANs of terminals. |
| Loopback Interface IP address | 1.1.1.0-1.1.1.0/24 | IP address of a loopback interface. Loopback interface IP addresses are used to establish BGP EVPN peer relationships when the underlay routing domain is automatically configured and the fabric is automatically connected to network service resources. |
| Bridge Domain (BD) | 1-1000 | Each BD is identified by a BD ID. iMaster NCE-Campus automatically selects a BD ID when delivering configurations to a device. |
| VXLAN Network Identifier (VNI) | 1-1000 | VNIs are used to distinguish isolation domains on a VXLAN network. |

# Choose Fabric Network > Network Plan. On the resource pool configuration page that is displayed, click Fabric Global Resource Pool.

# On the Fabric Global Resource Pool page, set the parameters and click + to confirm.

Configure an underlay automation resource pool.

The underlay automation resource pool contains VLAN and IP address resources, which are used for implementing connectivity on the underlay network.

Plan for the underlay automation resource pool

| Resource | Value Range | Description |
| --- | --- | --- |
| Interconnection VLAN | 10-20 | Used for the interconnection between the border and edge nodes in a fabric. |
| Interworking IP | 172.20.0.0-172.20.0.0/16 | Used for the interconnection between the border and edge nodes in a fabric. |

# Choose Fabric Network > Network Plan. On the resource pool configuration page that is displayed, click the Underlay Automation Resource Pool tab, set related parameters, and click + to confirm.

----End

Configuring a Fabric Network

          - Create a fabric and complete automatic deployment of an underlay network.

Fabric network plan

| Parameter | Value |
| --- | --- |
| Name | HQ (customizable) |
| Networking Type | Distributed |
| Wireless authentication device | Edge as authentication control point for wireless users |
| Report terminal identification information | Off |
| Automatic routing domain configuration | On |
| Configuration Switch | On |
| Encryption | None (customizable) |
| Domain | Single area (customizable) |
| AS number | 100 (customizable) |
| RR cluster ID | 1 |

Role plan for fabric devices

| Device | Role |
| --- | --- |
| CORE1 | Border |
| AGG1 | / |
| AGG2 | / |
| ACC1 | Edge |
| ACC2 | Edge |

# Choose Fabric Network > Fabric Management. The fabric management page is displayed. Click Create to create a fabric.

# Click Create to create a fabric.

Name indicates the fabric name, which can be customized. In this example, the fabric uses distributed gateways. Therefore, select Distributed in Networking Type. Scroll down to configure the device role, and then click Next.

Currently, the fabric network is deployed at only one site. Therefore, the site is selected by default. All switches at the site are displayed on the right of the page. In this step, you need to configure a role for each switch in the fabric. In this example, configure access switches ACC1 and ACC2 as edge nodes (edge of the VXLAN network), and configure the core switch CORE1 as a border node (edge device that connects the fabric to external networks).

# Configure physical network deployment.

Enable the Automatic routing domain configuration function to implement automatic deployment of the underlay routing domain. Select the site you created and enable Configuration Switch. Then, configure the underlay routing domain on the right of the page.

Currently, the CloudCampus Solution uses OSPF to implement automatic deployment of the routing domain on the underlay network. iMaster NCE-Campus can detect the range of an OSPF routing domain (in this example, from the edge nodes to the border node) once the range of the fabric is specified in the previous step.

After the deployment is complete, iMaster NCE-Campus automatically delivers underlay configurations to fabric devices (edge and border switches), including those for underlay connectivity and OSPF adjacency establishment.

In the BGP-EVPN Basic Configuration area, AS number 100 is automatically assigned to the fabric, and CORE1 is selected as the route reflector (RR) by default. Click Apply.

iMaster NCE-Campus will automatically complete the underlay network configuration because Automatic routing domain configuration is enabled. In addition, iMaster NCE-Campus will automatically create loopback interfaces on CORE1, ACC1, and ACC2, and specify these loopback interfaces as source interfaces for sending BGP packets. If Automatic routing domain configuration is disabled, you need to configure the underlay network manually, including creating loopback interfaces on devices.

# Verify the configuration.

After the preceding steps are complete, iMaster NCE-Campus will deliver fabric configurations to network devices. To verify that the fabric is correctly configured, perform the following operations to check the configuration delivery result. Choose Fabric Network > Fabric Management. On the fabric management page that is displayed, click the fabric that you just created. The fabric topology page is displayed.

Click Configuration Status. The Configuration Status values for all devices are Succeeded.

Notes on the configurations to be delivered:

1. Create VLANIF interfaces and LoopBack1 interfaces on CORE1, AGG1, AGG2, ACC1, and ACC2, and configure IP addresses for the interfaces.

2. Configure OSPF on CORE1, AGG1, AGG2, ACC1, and ACC2 to implement underlay connectivity.

3. Establish BGP EVPN peer relationships between CORE1 and ACC1 as well as between CORE1 and ACC2 using their LoopBack1 addresses, and complete BGP EVPN configurations. Create an NVE interface.

Configure external networks.

Configuring external networks for a fabric means configuring the connectivity between the fabric and external networks. In this example, three external networks will be defined for the three virtual networks that will be created later.

Configure one external network for the OA virtual network to access the Internet.

Configure two external networks for users on the RD and MKT virtual networks.

When configuring an external network, you need to configure the interface, VLAN, and IP address used by the border device to connect to a device outside the fabric (such as the egress router AR1), as well as the peer device's IP address and a route with the peer device being the next hop.

Plan for the OA external network

| Parameter | Value |
| --- | --- |
| Connection between the fabric and external network | L3 exclusive egress |
| Name | OA |
| Internet connection | On |
| Border device | CORE1 |
| Interconnection port | GigabitEthernet0/0/1 |
| VLAN | 120 |
| IP address type | IPv4 |
| Local IPv4 address | 192.168.120.1 |
| Remote IPv4 address | 192.168.120.2 |
| IPv4 address mask: | 30 |

Plan for the RD external network

| Parameter | Value |
| --- | --- |
| Connection between the fabric and external network | L3 exclusive egress |
| Name | RD |
| Internet connection | Off |
| External service IP address | 172.16.12.0/24 |
| Border device | CORE1 |
| Interconnection port | GigabitEthernet0/0/1 |
| VLAN | 121 |
| IP address type | IPv4 |
| Local IPv4 address | 192.168.121.1 |
| Remote IPv4 address | 192.168.121.2 |
| IPv4 address mask: | 30 |

Plan for the MKT external network

| Parameter | Value |
| --- | --- |
| Connection between the fabric and external network | L3 exclusive egress |
| Name | MKT |
| Internet connection | Off |
| External service IP address | 172.16.13.0/24 |
| Border device | CORE1 |
| Interconnection port | GigabitEthernet0/0/1 |
| VLAN | 122 |
| IP address type | IPv4 |
| Local IPv4 address | 192.168.122.1 |
| Remote IPv4 address | 192.168.122.2 |
| IPv4 address mask: | 30 |

# Click the fabric you just created. The fabric page is displayed. Click External Network in the fabric topology. The Configure External Network pane is displayed on the right of the page.

In this example, three external networks are configured, all of which use Layer 3 exclusive egresses, meaning that an egress is exclusively occupied by a virtual network. Each of the three external networks establishes a logical communication channel with AR1 connected to the border device.

The following describes the procedure for creating the OA virtual network. The procedure for configuring the RD and MKT virtual networks is similar.

# Click Create to create an external network. Select L3 exclusive egress and click OK.

# Configure the basic information.

Name: indicates the name of the external network, which can be customized. Enter OA here.

Internet connection: In this example, you need to enable Internet connection. Then, iMaster NCE-Campus will direct traffic to the destination network using a default route. If this function is disabled, route prefixes need to be configured. After basic information is configured, click Next.

# Configure the connection information.

Select CORE1 from the Boarder device drop-down list box, meaning that CORE1 is used as the border device to connect to the external network. Click Add next to Interconnection Port to add an interface for connecting the switch to the external network, and then configure the connection information based on the data plan. Click OK and then Next.

Name: Self-define a name.

Port: port used by CORE1 to connect to the peer device on the external network.

VLAN: VLAN used by CORE1 to connect to the peer device on the external network.

IP address type: whether local and peer devices use IPv4 and/or IPv6 addresses.

Local IPv4/IPv6 address: IP address of the local interface that connects CORE1 to the peer device on the external network (in this example, it is the IP address of the VLANIF interface corresponding to the VLAN).

Remote IPv4/IPv6 address: IP address of the peer device on the external network.

IPv4/IPv6 address mask: IPv4 or IPv6 address mask for the local and peer devices.

# Configure routing information.

On the page shown below, enable Deliver static routes. iMaster NCE-Campus will deliver the static route destined for the external network to CORE1 (only when the external network is invoked by the virtual network). After the route is configured, click Apply.

# Create the RD and MKT external networks.

Repeat the preceding steps to create the RD and MKT external networks based on the corresponding external network data plans. Then, there will be a total of three external networks in this example.

Notes on the configurations to be delivered:

1. After the preceding operations are complete, iMaster NCE-Campus does not deliver commands for configuring the external networks for the time being.

Configure network service resources.

Configuring network service resources means configuring the connectivity between the fabric and network service resources (DHCP server, RADIUS server, Portal server, and other servers). The connectivity configuration includes the device IP address, interconnection VLAN, interconnection IP address, peer IP address, and interconnection port.

Plan for the network service resource - DHCP server

| Parameter | Value |
| --- | --- |
| Name | DHCP_Service |
| Server type | DHCP |
| DHCP Server | 192.168.130.2 |
| Server interconnection address pool | / |
| Scenario | Directly connect to a switch |
| Interconnection device | CORE1 |
| External Port | GigabitEthernet0/0/1 |
| External VLAN | 130 |
| Interconnection IPv4 address | 192.168.130.1 |
| Peer IPv4 address | 192.168.130.2 |
| Mask | 30 |

# Click the fabric you just created. The fabric page is displayed. In the fabric topology, click Network Service Resources. The Configure Network Service Resources pane is displayed on the right.

# Click Create to create a network service resource, and configure the DHCP server based on the network service resource plan. Then click Next.

Name: Self-define a name.

Server type: server resource specified by the network service resource. In this example, the defined network service resource is a DHCP server.

DHCP server: IPv4 and/or IPv6 address of a DHCP server.

Server interconnection address pool: If an IPv6 address pool is configured for the DHCP server, this parameter is mandatory. If only an IPv4 address pool is configured for the DHCP server, this parameter is optional.

# Configure devices.

Set Scenario to Directly connect to a switch. In this example, the border device (CORE1) is connected to the network service resource server through a switch (simulated by AR1). Set the other parameters based on the network service resource plan.

If Directly connected to a switch is selected, the border device adds the interconnection port to the interconnection VLAN in tagged mode, so as to connect to network resources. If Directly connected to a server is selected, the switch adds the interconnection port to the interconnection VLAN in untagged mode.

Interconnection device: Select the border device (CORE1). The device functions as a border gateway that connects the fabric to the external network service resource.

External port: port used by the border device to connect to the network service resource.

External VLAN: VLAN used by the border device to connect to the network service resource.

Interconnection IPv4/IPv6 address: IP address of the port used by the border device to connect to the network service resource.

Peer IPv4/IPv6 address: IP address of the peer device.

Mask: network mask.

# View the configuration status.

Click Configuration Status to check whether the network service resource configuration is successfully delivered.

Notes on the configurations to be delivered:

1. Configure the interface, VLAN, and IP address for connecting the border device (CORE1) and the network service resource, based on the parameters configured by the user.

2. Create a VRF (named netresurf_xxx) for the communication between the device and network service resource.

3. Add the user-configured static route destined for the network service resource to the VRF routing table.

4. Import the static route configured in the VRF to BGP.

Configure access management.

Access control needs to be deployed on the campus network to authenticate access users, including 802.1X authentication, MAC address authentication, and Portal authentication. The implementation of 802.1X authentication and MAC address authentication requires the RADIUS server, whereas the implementation of Portal authentication requires the RADIUS server and Portal server.

To implement access control on the campus network, you need to configure 802.1X authentication, MAC address authentication, and Portal authentication templates on iMaster NCE-Campus. In this example, one authentication template is configured for both 802.1X authentication and MAC address authentication, because multiple authentication modes can be specified in a single authentication template. In addition, the configured authentication template needs to be referenced on the interface of the access device. The RADIUS server and Portal server functions are provided by built-in servers of iMaster NCE-Campus.

Plan for the RADIUS server template

| Parameter | Value |
| --- | --- |
| Name | HQ_RADIUS_Server |
| Using Built-in Server | On |
| Key | Huawei@123 |

Plan for the Portal server template

| Parameter | Value |
| --- | --- |
| Name | HQ_Portal_Server |
| Using Built-in Server | On |
| Page push protocol | HTTPS |
| Key | Huawei@123 |

Plan for the 802.1X and MAC address authentication template

| Parameter | Value |
| --- | --- |
| Name | HQ_MAC_802.1X |
| Authentication mode | MAC, 802.1X |
| RADIUS server template | HQ_RADIUS_Server |

Plan for the Portal authentication template

| Parameter | Value |
| --- | --- |
| Name | HQ_Portal |
| Authentication mode | Portal |
| RADIUS server template | HQ_RADIUS_Server |
| Portal server template | HQ_Portal_Server |

Configure a policy template.

# Choose Fabric Network > Fabric Management. The fabric management page is displayed. Click the fabric that has been created to display the fabric topology page. Click Access Management. The Design > Basic Network Design > Template Management > Policy Template page is displayed.

# Configure a RADIUS server template.

In the navigation area, click RADIUS Server. Click Create to create a RADIUS server template. Set the parameters based on the data plan, and retain the default values for parameters that are not included in the data plan. Then click OK.

This template will be referenced in the 802.1X and MAC address authentication template as well as the Portal authentication template.

# Create a Portal server template.

In the navigation area, click Portal Server. Click Create to create a Portal server template. Set the parameters based on the data plan, and retain the default values for parameters that are not included in the plan. Then click OK.

This template will be referenced by the Portal authentication template.

# Configure an 802.1X and MAC address authentication template.

In the navigation area, click Authentication Template. Click Create to create an 802.1X and MAC address authentication template. Set the parameters based on the data plan, and retain the default values for parameters that are not included in the plan. Then click OK.

Select I have read the bypass policy selection notice, and I know the contents and risks of both policies and retain the default values for other parameters.

# Create a Portal authentication template.

In the navigation area, click Authentication Template. Click Create to create a Portal authentication template. Set the parameters based on the data plan, and retain the default values for parameters that are not included in the plan. Then click OK.

Select I have read the bypass policy selection notice, and I know the contents and risks of both policies and retain the default values for other parameters.

Configure access management for access devices (ACC1 and ACC2).

Access management plan for ACC1

| Parameter | Value |
| --- | --- |
| Authentication Control Point | ACC1 |
| Authentication Control Point Management Parameter |  |
| Port Name | GigabitEthernet0/0/24 |
| Set Connected Device Type | Terminal (PC, Phone, dumb terminal, non-fabric extended switch/AP) |
| Set Authentication Template | HQ_MAC_802.1X |

Access management plan for ACC2

| Parameter | Value |
| --- | --- |
| Authentication Control Point | ACC2 |
| Authentication Control Point Management Parameter |  |
| Number of enforcement point devices | 126 (customizable) |
| Management VLAN of CAPWAP | 111 |
| Management IP address of CAPWAP | 172.111.1.1 |
| Port Name | GigabitEthernet0/0/23 |
| Set Connected Device Type | Extended AP |
| Set Authentication Template | -- |
| Port Name | GigabitEthernet0/0/24 |
| Set Connected Device Type | Terminal (PC, Phone, dumb terminal, non-fabric extended switch/AP) |
| Set Authentication Template | HQ_MAC_802.1X |
| Wireless Access Configuration |  |
| SSID Name | HQ_Guest |
| Authentication Template | HQ_Portal |

# Choose Fabric Network > Fabric Management. On the fabric management page that is displayed, click the Access Management tab.

# Configure an authentication control point named ACC1.

Configure ACC1 based on the ACC1 access management plan, retain the default values for parameters that are not included in the plan, and then click Apply.

# Configure an authentication control point named ACC2.

Configure ACC2 based on the ACC2 access management plan, retain the default values for parameters that are not included in the plan, and then click Apply.

Configure the CAPWAP management VLAN and CAPWAP management IP address. That is, create the management VLAN and CAPWAP tunnel source interface in the WLAN service, and assign an IP address to the source interface. The DHCP server assigns IP addresses from the interface address pool to APs. After this step is performed, the APs can get onboarded successfully.

Check the fabric configuration delivery result.

# To verify that the preceding configuration is correct, check the configuration delivery result.

Choose Fabric Network > Fabric Management. On the fabric management page that is displayed, click the fabric that you just created. On the fabric topology page, click Configuration Status.

Currently, the Configuration Status values for all access management configuration items are Succeeded.

# Check whether AP1 is online.

Choose Fabric Network > Device Management. On the device management page that is displayed, click the site HQ. AP1 is in Normal state, indicating that it is online.

Notes on the configurations to be delivered:

1. Deliver authentication server configurations, including those of the RADIUS authentication server and Portal authentication server.

2. Deliver access authentication configurations, including the configurations of the 802.1X and MAC address authentication template as well as the Portal authentication template, and the configuration for referencing authentication templates on interfaces.

3. Complete basic configurations for onboarding the AP on ACC2, including creating an interface address pool (VLANIF 111) for assigning an IP address to the AP, using VLANIF 111 as the source interface to establish a CAPWAP tunnel, authenticating the AP in SN mode, and changing the AP name to AP1. In this way, the AP can get onboarded successfully.

----End

Overlay Network Automation

Creating Virtual Networks

In this lab, three virtual networks — OA_VN, RD_VN, and MKT_VN — will be created. The following three tables provide data plans for the three virtual networks.

Data plan for the OA virtual network

| Parameter | Value |
| --- | --- |
| Create VN |  |
| Name | OA_VN (customizable) |
| User gateway location | Inside the fabric |
| User-defined VRF name | OA (customizable) |
| External network | OA |
| Network service resources | DHCP_Service |
| User gateway: Manually Specified |  |
| OA | OA (customizable) |
| VLAN Type | Dynamic VLAN |
| VLAN | 101 |
| IPv4 subnet | 172.16.1.0/24 |
| IPv4 gateway address | 172.16.1.1 |
| User access: Wired access |  |
| Service name | OA (customizable) |
| Device Name: ACC1 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |
| Device Name: ACC2 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |
| User access: Wireless access |  |
| Site | HQ |
| Device Name | ACC2 |

Data plan for the RD virtual network

| Parameter | Value |
| --- | --- |
| Create VN |  |
| Name | RD_VN (customizable) |
| User gateway location | Inside the fabric |
| User-defined VRF name | RD (customizable) |
| External network | RD |
| Network service resources | DHCP_Service |
| User gateway: Manually Specified |  |
| OA | RD (customizable) |
| VLAN Type | Dynamic VLAN |
| VLAN | 102 |
| IPv4 subnet | 172.16.2.0/24 |
| IPv4 gateway address | 172.16.2.1 |
| User access: Wired access |  |
| Service name | RD (customizable) |
| Device Name: ACC1 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |
| Device Name: ACC2 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |

Data plan for the MKT virtual network

| Parameter | Value |
| --- | --- |
| Create VN |  |
| Name | MKT_VN (customizable) |
| User gateway location | Inside the fabric |
| User-defined VRF name | MKT (customizable) |
| External network | MKT |
| Network service resources | DHCP_Service |
| User gateway: Manually Specified |  |
| OA | MKT (customizable) |
| VLAN Type | Dynamic VLAN |
| VLAN | 103 |
| IPv4 subnet | 172.16.3.0/24 |
| IPv4 gateway address | 172.16.3.1 |
| User access: Wired access |  |
| Service name | MKT (customizable) |
| Device Name: ACC1 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |
| Device Name: ACC2 | Port: GigabitEthernet0/0/24 Service VLAN: Dynamic VLAN |

          - Create the OA virtual network.

# Choose Fabric Network > VN Management. The logical network configuration page is displayed.

# Click Create VN to create a virtual network named OA_VN.

Set the parameters for creating a virtual network based on the data plan for the OA virtual network. Retain the default values for the parameters that are not included in the plan table.

The table under User gateway provides the IP address segment of access users in the virtual network. iMaster NCE-Campus provides two ways of obtaining IP address segments: Manually Specified (manually add network segments one by one) and Automatic Allocation. In Automatic Allocation mode, you only need to specify the number of subnets and a start network segment, iMaster NCE-Campus can then automatically calculate all address segments based on the start network segment.

# Configure wired access.

Click Create, select devices ACC1 and ACC2, find the corresponding GigabitEthernet0/0/24 interfaces, and set Service VLAN to Dynamic VLAN. Then click OK.

User access is configured on the access points of the current virtual network. We want users to access OA_VN through ACC1 and ACC2 after being authenticated, so we need to select the switches' interfaces on which authentication is enabled.

# Configure wireless access.

Click Create, select HQ from the Site drop-down list box, select the device ACC2, and click OK.

Select ACC2 for wireless access. In this example, ACC2 is a switch that provides the native AC function. It manages APs through native AC, thereby providing wireless access for end users.

After the preceding steps are complete, the OA_VN virtual network has been configured. Click OK to deliver the configuration.

Create the RD virtual network.

# Create the RD_VN virtual network by referring to the procedure for creating the OA_VN virtual network.

Create the MKT virtual network.

# Create the MKT_VN virtual network by referring to the procedure for creating the OA_VN virtual network.

View the configuration status.

# Choose Fabric Network > VN Management. The logical network configuration page is displayed. Click  next to the site name to view the configuration result. The Configuration Status values of all virtual networks (logical networks) are Succeeded.

Notes on the configuration delivered to the border device (CORE1):

1. Create three VPN instances (VRFs) — OA, RD, and MKT, each corresponding to a virtual network.

2. Since an external network is selected when a virtual network is created, iMaster NCE-Campus parses the external network information to obtain configurations such as the interconnection port, VLAN, and IP address, and then delivers the configurations to the border node. In addition, iMaster NCE-Campus obtains the route to the external network, imports the route to the routing table of the VRF for this virtual network (create a static route for the VPN instance), and advertises the route to the two edge devices (ACC1 and ACC2) using BGP EVPN.

3. Create IPv4 address families for the VRFs in BGP, and import static routes and direct routes to BGP.

Notes on the configuration delivered to the edge nodes (ACC1 and ACC2):

1. Create three VPN instances (VRFs) — OA, RD, and MKT, each corresponding to a virtual network.

2. Create three EVPN instances.

3. Create three BDs, and bind them to the corresponding access VLANs and EVPN instances.

4. Create a VBDIF interface in each BD and configure the VBDIF interfaces as Layer 3 user gateways. Enable the DHCP relay function on the VBDIF interfaces.

5. Create IPv4 address families for the VRFs in BGP, and import static routes and direct routes to BGP.

----End

Configuring Communication Between Virtual Networks

By default, virtual networks are isolated from each other. However, in some scenarios, they may need to communicate with each other, so reachable routes must be configured between them.

This example assumes that the Guest user accesses the OA virtual network, the Research user accesses the RD virtual network, and the Sales user accesses the MKT virtual network. We want the Research and Sales users to be able to communicate with each other.

Plan for communication between virtual networks

| Parameter | Value |
| --- | --- |
| Name | RD_to_MKT (customizable) |
| Interconnection device | CORE1 |
| Interworking Mode | Partial Inerwork |
| IP address type | IPv4 |
| Source VN | RD |
| Source IPv4 Prefix | 172.16.12.0/24 |
| Destination VN | MKT |
| Destination IPv4 Prefix | 172.16.13.0/24 |

          - Configure communication between the virtual networks.

# Choose Fabric Network > VN Management. On the logical network configuration page that is displayed, click  to access the VN Interwork page.

# Click Add. Configure inter-VN communication based on the data plan for communication between the virtual networks, and click Apply.

View the configuration status.

# Choose Fabric Network > VN Management. The logical network configuration page is displayed. Click  next to the site name to view the configuration result. Currently, the Configuration Status value for the inter-VN communication configuration is Succeeded.

Notes on the configuration delivered to the border device (CORE1):

1. Create static routes for the VPN instances so that access users in RD_VN and MKT_VN can communicate at Layer 3.

2. Import the static routes for the VPN instances to BGP.

----End

Free Mobility and Access Authentication

On large-sized campus networks, employees are usually allowed access from any location, any VLAN, and any IP network segment with controlled network access rights.

In this example, three security groups — Guest_Group, Research_Group, and Sales_Group — will be created, and the security groups can access OA_VN, RD_VN, and MKT_VN, respectively.

Users in the Research_Group and Sales_Group can communicate with each other and access the Internet through the OA virtual network. In contrast, users in the Guest_Group can only access the Internet through the OA virtual network.

Policy control matrix plan

| Destination Security Group Source Security Group | Guest_Group | Research_Group | Sales_Group |
| --- | --- | --- | --- |
| Guest_Group |  | Status: Enable Default: Deny | Status: Enable Default: Deny |
| Research_Group | Status: Enable Default: Permit |  | Status: Enable Default: Permit |
| Sales_Group | Status: Enable Default: Permit | Status: Enable Default: Permit |  |
| unknown |  | Status: Enable Default: Deny | Status: Enable Default: Deny |

Free Mobility

          - Create security groups.

# Choose Fabric Network > Service Deployment. The service deployment page is displayed. Select Security Group and click Create.

# Create three security groups. Set Name to Guest_Group, Research_Group, and Sales_Group, and retain the default values for other parameters. Then click OK.

Create a policy control matrix.

# Click the Policy Control tab. To create a policy control matrix and deliver it to the policy enforcement point, click .

# Configure a policy control matrix.

Set required parameters. You can customize the matrix name (for example, HQ_Policy). Set Scene to Site scenario, and click Add next to Select Device to add edge devices (ACC1 and ACC2). Then click OK.

# Click the name of the created policy control matrix and set policy control parameters.

This example provides the settings for enabling the communication between the Research_Group and Sales_Group. The policy control parameters for communication between other groups are similar.

Because the Research_Group and Sales_Group need to mutually communicate with each other, enable Policy reversion. (If mutual communication is not required, disable Policy reversion.) Then click OK.

# Create other policies based on the policy control matrix plan.

Question: What policies are deployed in the blank areas of the policy control matrix?

# Click Deploy to deliver the policy control matrix configuration to the corresponding devices.

View the configuration status.

# Choose Maintenance > Configuration Maintenance > Configuration Result. The logical network configuration page is displayed.

# Select the HQ site, select device ACC1 or ACC2, and click the arrow on the front of the device. The configuration status of Policy Control is Success.

Notes on the configuration delivered to the edge nodes (ACC1 and ACC2):

1. Create security groups.

2. Create ACLs to match interested traffic, including the traffic of source security groups Guest_Group, Research_Group, Sales_Group, and unknown.

3. Create a traffic filter to filter traffic, thus implementing policy control for inter-security group access.

----End

Access Authentication

To configure user access authentication, you need to configure user accounts for access authentication on the user management page of iMaster NCE-Campus.

After configuring user accounts, you need to configure authentication and authorization, including authentication rules, authorization results, and authorization rules.

          - Create user authentication accounts.

Plan for user authentication accounts

| Parameter | Value |
| --- | --- |
| Guest_Group user (Portal authentication) |  |
| User name | Guest |
| Password & Comfirm password | Huawei@123 |
| Change password upon next login | Off |
| Research_Group user (802.1X authentication) |  |
| User name | Research |
| Password & Comfirm password | Huawei@123 |
| Change password upon next login | Off |
| Sales_Group user (802.1X authentication) |  |
| User name | Sales |
| Password & Comfirm password | Huawei@123 |
| Change password upon next login | Off |

# Choose Admission > Admission Resources > User Management. The User Management page is displayed.

# On the User Management page, click the corresponding tab for creating users based on the authentication user type. Multiple users can be created if needed.

# Create three user accounts based on the user authentication account plan, and retain the default values for other parameters that are not included in the plan table.

Create authentication rules.

# Choose Admission > Admission Policy > Authentication and Authorization. The Authentication and Authorization page is displayed.

# Authentication rules do not need to be created manually. The default authentication rule Default can be directly used as the authentication rule for users Guest, Sales, and Research.

The following figure shows the configuration of the authentication rule Default.

Create authorization results.

Authorization result plan

| Parameter | Value |
| --- | --- |
| Guest_Group user (Portal authentication) |  |
| Name | Guest_Result |
| Security group | Guest_Group |
| VLAN | 101 |
| Research_Group user (802.1X authentication) |  |
| Name | Research_Result |
| Security group | Research_Group |
| VLAN | 102 |
| Sales_Group user (802.1X authentication) |  |
| Name | Sales_Result |
| Security group | Sales_Group |
| VLAN | 103 |

# Go to the Authorization Result page, and click Create to create three authorization results — Guest_Result, Research_Result, and Sales_Result — based on the authorization result plan. Retain the default values for parameters that are not included in the plan table. Then click OK. (This example describes how to create the Guest_Result authorization result. The procedure for creating the other two authorization results is the same as that for Guest_Result.)

# Click Yes to bind the authorization result to a site.

# Click Add, select the site HQ, and click OK.

# The Site binding status value is Success, indicating that the authorization result is successfully bound to the site.

# Repeat the preceding steps to create the authorization results Research_Result and Sales_Result and bind them to the site HQ.

# Choose Maintenance > Configuration Maintenance > Configuration Result. The logical network configuration page is displayed.

# Select the HQ site, select device ACC1 or ACC2, and click the arrow on the front of the device. The configuration status of Authorization Result is Success.

Notes on the configuration delivered to the edge nodes (ACC1 and ACC2):

1. Create service schemes Guest_Result, Research_Result, and Sales_Result in the AAA view, and plan authorization resources including user VLANs.

Create authorization rules.

Authorization rule plan

| Parameter | Value |
| --- | --- |
| Guest_Group user (Portal authentication) |  |
| Name | Guest_Rule |
| Authentication Mode | User access authentication |
| Enabling the Portal-HACA Protocol | Off |
| Access Mode | Wireless |
| Account matching | On Add a user named Guest. |
| Authorization result | Guest_Result |
| Research_Group user (802.1X authentication) |  |
| Name | Research_Rule |
| Authentication Mode | User access authentication |
| Enabling the Portal-HACA Protocol | Off |
| Access Mode | Wired |
| Account matching | On Add a user named Research. |
| Authorization result | Research_Result |
| Sales_Group user (802.1X authentication) |  |
| Name | Sales_Rule |
| Authentication Mode | User access authentication |
| Enabling the Portal-HACA Protocol | Off |
| Access Mode | Wired |
| Account matching | On Add a user named Sales. |
| Authorization result | Sales_Result |

# Go to the Authorization Rules page, and click Create to create three authorization rules — Guest_Rule, Research_Rule, and Sales_Rule — based on the authorization rule plan. Retain the default values for parameters that are not included in the plan table. Then click OK. (This example provides the procedure for creating the Guest_Rule authorization rule. The procedure for creating the other two authorization rules is the same as that for Guest_Rule.)

# Configure user information. Enable Account matching, click Add, and select the corresponding user account Guest.

# Select Guest_Result from the Authorization result drop-down list box. Then click OK.

# Repeat the preceding steps to create authorization rules Research_Rule and Sales_Rule.

----End

WLAN Services

In this example, ACC2 is a switch that provides the native AC function, through which it manages APs. To manage APs on ACC2, you need to configure the WLAN service on it. You need to configure an SSID on the AC for APs to provide Wi-Fi signals for guest access, and configure Portal authentication for guests who attempt to connect to the SSID.

You need to log in to the web management system of ACC2 to configure the WLAN service.

WLAN service plan

| Parameter | Value |
| --- | --- |
| Management VLAN | 111 |
| Service VLAN | 101 |
| DHCP server | ACC2 functions as a DHCP server to assign IP addresses to APs. ACC2 functions as a DHCP relay agent to assign IP addresses to STAs. |
| IP address pool for APs | 172.111.1.0/23, with gateway 172.111.1.1 |
| IP address pool for STAs | 172.16.1.0/24, with gateway 172.16.1.1 |
| AC's source interface IP address | VLANIF 111: 172.111.1.1 |
| AP Group | Default Referenced profile: VAP profile HQ_VAP WLAN ID: 1 |
| Regulatory Domain Profile | Default |
| SSID Profile | Name: HQ_SSID SSID name: HQ_Guest |
| Security Profile | Default |
| VAP Profile | Name: HQ_VAP Service VLAN: VLAN 101 Forwarding mode: Tunnel Referenced profiles: SSID profile HQ_SSID and authentication profile HQ_Portal |

          - Log in to the web management system of ACC2.

# Choose Fabric Network > Device Management. On the device management page that is displayed, select ACC2.

# Click Device Configuration to access the web management system of ACC2.

# Enter the user name admin and password Huawei@123 to log in to the web management system.

Create and configure an SSID profile.

# Choose Configuration > Wireless Services > Profile > Wireless Service > SSID Profile. The SSID profile configuration page is displayed.

# Click Create. Set the profile name, such as HQ_SSID and click OK.

# On the page that is displayed, set SSID to HQ_Guest, and click Apply.

Create and configure a VAP profile.

# On the Profile Management tab page, choose VAP Profile.

# Click Create. Set the profile name, such as HQ_VAP and click OK.

# On the page that is displayed, set VAP profile parameters based on the WLAN service plan, and retain the default values for the parameters that are not included in the plan table. Click Apply.

# Bind the SSID profile to the VAP profile.

On the Profile Management tab page, select the VAP profile you just created, expand the profile, and click SSID Profile. On the page displayed on the right, select HQ_SSID from the SSID Profile drop-down list box and click Apply.

# Bind the Portal authentication profile to the VAP profile.

On the Profile Management tab page, select the VAP profile you just created, expand the profile, and click Authentication Profile. On the page displayed on the right, click ... next to Authentication Profile. In the dialog box that is displayed, select the Portal authentication profile HQ_Portal that have been created on iMaster NCE-Campus and click Apply.

Bind the VAP profile to an AP group.

Bind the created VAP profile to an AP group to be onboarded to make the WLAN configuration take effect. The APs in the group can then work based on the configurations in the profile after they get onboarded.

# Click AP Group, and select default on the AP Group tab page. By default, APs belong to the default AP group after they get onboarded.

# Click default, select VAP Configuration, and click Add to bind the created VAP profile to the default AP group.

# Select HQ_VAP from the VAP profile name drop-down list box, and enter the WLAN ID. Then click OK.

After the preceding operations are complete, ACC2 will deliver the WLAN service configuration to APs.

Notes on the configuration delivered to the edge node ACC2:

1. Create an SSID profile.

2. Create a VAP profile, reference the SSID profile and authentication profile in the VAP profile, and configure the service VLAN and forwarding mode.

3. Bind the VAP profile to the AP group.

----End

Verification

Verifying Access Authentication

802.1X Authentication

This example verifies 802.1X authentication using a wired terminal.

          - Modify network adapter attributes.

# On a wired terminal, choose Control Panel > Network and Internet > Network Connections, find the network adapter that connects to ACC1 (or ACC2), and modify the attributes of the network adapter.

Authenticate the identity.

# Click the Authentication tab and then click Additional Settings. In the dialog box that is displayed, select User or Computer Authentication and click Replace credentials.

In the dialog box that is displayed, enter the user name and password. (The user name and password of an R&D employee are Research and Huawei@123, and those of a sales employee are Sales and Huawei@123.) Click OK in each window that is displayed.

View online users.

# After the users are successfully authenticated, they are displayed in the online user list of ACC1 and ACC2.

[ACC1]display access-user

------------------------------------------------------------------------------------------------------

UserID		Username  IP address						MAC																Status

------------------------------------------------------------------------------------------------------

6									admin								-																	- 																			Success

16404			Sales								172.16.3.191  000e-c6bf-8d13 Success

------------------------------------------------------------------------------------------------------

[ACC2]display access-user

------------------------------------------------------------------------------------------------------

UserID		Username  IP address							MAC																Status

------------------------------------------------------------------------------------------------------

3									admin								-																			- 																			Success

16406			Research			 172.16.2.242  000c-2935-3f19 	Success

------------------------------------------------------------------------------------------------------

# You can view details about a user using the user ID.

[ACC1]display access-user user-id 16404

Basic:

User ID      : 16404

User name     : Sales

Domain-name    : default

User MAC     : 000e-c6bf-8d13

User IP address    : 172.16.3.191

User vpn-instance    : MKT

User IPv6 address    : FE80::208D:6A96:EE48:8F64

User IPv6 link local address  : FE80::208D:6A96:EE48:8F64

User access Interface   : GigabitEthernet0/0/24

User vlan event    : Success

QinQVlan/UserVlan   : 0/103

User vlan source    : server vlan

User access time    : 2020/11/25 11:01:09

User accounting session ID  : ACC10002400000010326****0100014

User access type    : 802.1x

Terminal Device Type   : Data Terminal

Dynamic VLAN ID    : 103

Dynamic group index(Effective) : 6

Dynamic group name(Effective) : Sales_Group

Dynamic service scheme  : Sales_Result

Service Scheme Priority   : 0

AAA:

User authentication type  : 802.1x authentication

Current authentication method : RADIUS

Current authorization method : -

Current accounting method  : RADIUS

[ACC2]display access-user user-id 16406

Basic:

User ID      : 16406

User name     : Research

Domain-name    : default

User MAC     : 000c-2935-3f19

User IP address    : 172.16.2.242

User vpn-instance    : RD

User IPv6 address    : FE80::AC73:9028:7732:FA4B

User IPv6 link local address  : FE80::AC73:9028:7732:FA4B

User access Interface   : GigabitEthernet0/0/24

User vlan event    : Success

QinQVlan/UserVlan   : 0/102

User vlan source    : server vlan

User access time    : 2020/11/25 11:05:46

User accounting session ID  : ACC20002400000010211****0100016

User access type    : 802.1x

Terminal Device Type   : Data Terminal

Dynamic VLAN ID    : 102

Dynamic group index(Effective) : 5

Dynamic group name(Effective) : Research_Group

Dynamic service scheme  : Research_Result

Service Scheme Priority   : 0

AAA:

User authentication type  : 802.1x authentication

Current authentication method : RADIUS

Current authorization method : -

Current accounting method  : RADIUS

----End

Portal Authentication

This example verifies Portal authentication using a wireless terminal.

          - Connect to an SSID.

# Expand the Wi-Fi list, find the previously defined SSID HQ_Guest, and connect to the SSID.

Check the IP address.

After connecting to the SSID, run the ipconfig command on the command-line interface. The command output shows that the wireless network adapter has obtained an IP address from the 172.16.1.0/24 network segment.

Log in using Portal authentication.

# Open a browser and access any website. The authentication point (switch) redirects the user to the Portal authentication page, because the user has not yet been authenticated.

# On the authentication page, enter the guest account Guest and password Huawei@123, and click Log In to complete Portal authentication.

View online users.

# After the user is successfully authenticated, it is displayed in the online user list of ACC2.

[ACC2]display access-user

------------------------------------------------------------------------------------------------------

UserID  Username  IP address								MAC																			Status

------------------------------------------------------------------------------------------------------

2											admin								-																			-																							Success

16406					Researc					172.16.2.242						000c-2935-3f19   	Success

32786					Guest							172.16.1.173					b0d5-9dc7-3c41				Success

------------------------------------------------------------------------------------------------------

# You can view details about the user using the user ID.

[ACC2]display access-user user-id 32786

Basic:

User ID       : 32786

User name      : Guest

Domain-name     : default

User MAC      : b0d5-9dc7-3c41

User IP address     : 172.16.1.173

User vpn-instance     : OA

User IPv6 address     : -

User access Interface    : Wlan-Dbss2277

User vlan event     : Success

QinQVlan/UserVlan    : 0/101

User vlan source     : user request

User access time     : 2020/11/21 11:29:06

User accounting session ID   : ACC20002300000010183****0200012

User access type     : WEB

AP name      : AP1

Radio ID       : 0

AP MAC       : f4de-af36-b160

SSID       : HQ_Guest

Online time      : 278(s)

Web-server IP address    : 192.168.4.104

Dynamic group index(Effective)  : 1

Dynamic group name(Effective)  : Guest_Group

Dynamic service scheme   : Guest_Result

Service Scheme Priority    : 0

AAA:

User authentication type   : WEB authentication

Current authentication method  : RADIUS

Current authorization method  : -

Current accounting method   : RADIUS

----End

Viewing RADIUS Authentication Logs

# Choose Monitoring > Terminal Authentication Logs. The Terminal Authentication Logs page is displayed.

# Click the RADIUS Login and Logout logs tab, select the site HQ, and set the authentication result to Success. Modify the authentication time, and view the user status.

Verifying Network Connectivity

          - Verify that hosts can successfully access the Internet.

# After all configurations are complete, ensure that all wireless terminals have passed access authentication. Then ping the Internet (IP address of the simulated Internet, that is, 30.1.1.1) on the wireless terminal to test the IP connectivity from the terminal to the Internet.

The wireless terminal can access the Internet normally.

Verify that users in different VNs can communicate with each other.

# In this example, the users in the RD_VN virtual network and users in the MKT_VN virtual network must be able to communicate with each other, and the traffic between them must be forwarded through the border device.

To test the connectivity between the two virtual networks, ping the IP address (172.16.2.242) of an RD_VN user from the terminal (IP address 172.16.3.191) of a MKT_VN user.

----End

Configuration Deletion

Before the experiment, you need to clear the configurations that have been performed in the environment to keep the environment in the initial state.

Note: Delete the configurations on the iMaster NCE-Campus Controller and then the configurations on the devices. (Only the configurations of devices managed by the iMaster NCE-Campus Controller, including CORE1, AGG1, AGG2, ACC1, and ACC2, are deleted.)

Delete Authentication Rules, Authorization Rules, and Authorization Results

          - Choose Admission > Admission Policy > Authentication And Authorization from the main menu. The authentication and authorization page is displayed.

Delete the Authentication Rules.

If other authentication rules are manually created on the controller except the default authentication rule (Default), select all the created authentication rules and click Delete.

Delete the Authorization Rules.

Click the Authorization Rules tab, select all manually created authorization rules, and click Delete.

Delete the Authorization Results.

Click the Authorization Results tab, click ”” to bind a site, select HQ, and click Delete.

After unbinding, select all manually created authorization results, and click Delete.

----End

Forcibly Log Out Users and Delete the Users

Note: To ensure that the users can be deleted, you need to forcibly log out the online users first.

          - Choose Admission > Admission Policy > Online User Control from the main menu. On the Online User tab, select all users and click Force Offline.

Choose Admission > Admission Resources > User Management from the main menu. The user management page is displayed.

Click the User tab, select all users, and click Delete.

----End

Delete the Policy Control Matrices and Security Groups

          - Choose Fabric Network > Service Deployment. The service deployment page is displayed.

Click the Policy Control tab, click “” to select all policy control matrices, and click Delete.

Click the Security Group tab, select all security groups, and click Delete.

----End

Delete VN Interworking Policies and VNs

          - Choose Fabric Network > VN Management > VN Interwork, and delete the VN interworking policies.

Hover the pointer over each VN, and click the delete button “” in the upper right corner of the icon to delete the VN. All VNs except the default VN need to be deleted.

----End

Delete Access Management Configurations

          - Choose Fabric Network > Fabric Management, the fabric management page is displayed. Click the created fabric, the fabric page is displayed.

Click the Access Management tab. On the displayed page, scroll to the bottom of the page and click Reset to reset the access management configurations.

Repeat the preceding steps to delete access management configurations of other access devices.

----End

Delete Nerwork Service Resources and External Netwrok

          - Choose Fabric Network > Fabric Management, the fabric management page is displayed. Click the created fabric, the fabric page is displayed.

After you click Network Service Resources in the fabric topology, the window for configuring network service resources is displayed on the right. Click delete icon “” to delete network service resources.

After you click External Network in the fabric topology, the window for configuring external network is displayed on the right. Click delete icon “” to delete the external network.

----End

Delete the devices from the Fabric and Delete the Fabric

          - Choose Fabric Network > Fabric Management, the fabric management page is displayed. Click the created fabric, the fabric page is displayed.

In the fabric topology, click the List Mode icon “” in the upper right corner.

Select all devices and click Delete Device.

Delete the Fabric.

Method 1: Choose Fabric Network > Fabric Management, the fabric management page is displayed. On the created fabric, click “” to delete the fabric.

Method 2: Choose Fabric Network > Fabric Management, the fabric management page is displayed. Click the created fabric, the fabric page is displayed. Click the Drop-down Arrow icon next to the fabric name and click Delete icon “” to delete the fabric.

----End

Delete Fabric Global Resource Pools and Underlay Automation Resource Pools

          - Choose Fabric Network > Network Plan > Fabric Global Resource Pool, the fabric global resource pool page is displayed. Select the resources to be deleted and click the Delete icon “” to delete the fabric global resources in sequence.

          - Choose Fabric Network > Network Plan > Underlay Automation Resource Pool, the underlay automation resource pool page is displayed. Select the resources to be deleted and click the Delete icon “” to delete the underlay automation resources in sequence.

----End

Delete Devices Managed by iMaster NCE-Campus

          - Choose Fabric Network > Device Management, the device management page is displayed. On the Device Management tab, select all devices and click Delete Device.

----End

Delete Sites

          - Choose Fabric Network > Site Design, click the Delete icon “” of a site to delete the site.

----End

Delete Authentication Templates and Server Templates

          - Delete Authentication Templates.

Choose Design > Template Management from the main menu. On the template management page, click the Policy Template tab. Choose Authentication Template from the navigation pane, click Delete of a template to delete it, or select the templates to be deleted and click the Delete button.

Delete RADIUS Server Templates.

Choose Design > Template Management from the main menu. On the template management page, click the Policy Template tab. Choose RADIUS Server from the navigation pane, click Delete of a template to delete it, or select the templates to be deleted and click the Delete button.

Delete Portal Server Templates.

Choose Design > Template Management from the main menu. On the template management page, click the Policy Template tab. Choose Portal Server from the navigation pane, click Delete of a template to delete it, or select the templates to be deleted and click the Delete button.

----End

Delete Configurations on Switches

          - Log in to the ACC2 switch, run the following commands to delete the saved configurations.

<ACC2>reset saved-configuration

Warning: This command may cause a configuration conflict in NETCONF mode. Continue? [Y/N]:y

Warning: The action will delete the saved configuration in the device.

The configuration will be erased to reconfigure. Continue? [Y/N]:y

Note: The WLAN service configuration on ACC2 is not directly delivered by iMaster NCE-Campus through NETCONF, but is manually configured by the administrator on the Web page. Therefore, you need to run the preceding command to clear the existing configuration.

Log in to the CORE1, AGG1, AGG2, ACC1, and ACC2 switches, run the following commands to delete the configurations, and then restart the switches.

[Huawei]reset netconf db-configuration

Warning: This operation will clear the database configuration and saved configuration file and restart the device. Continue? [Y/N]:y

----End

Delete Configurations on AR1

          - Log in to the AR1 router, run the following commands to delete the configurations, and then restart the router.

<AR1>reset saved-configuration

This will delete the configuration in the flash memory.

The device configurations will be erased to reconfigure.

Are you sure? (y/n)[n]:y

Clear the configuration in the device successfully.

<AR1>reboot fast

System will reboot! Continue ? [y/n]:y

----End

Samples of Configurations Automatically Delivered by iMaster NCE-Campus

Configurations of CORE1

[CORE1]display current-configuration

#

sysname CORE1

#

vlan batch 11 to 12 120 to 122 130

#

lnp disable

#

ucl-group 1 name Guest_Group

ucl-group 5 name Research_Group

ucl-group 6 name Sales_Group

#

dhcp enable

#

ip vpn-instance MKT

ipv4-family

route-distinguisher 3:6

vpn-target 0:6 export-extcommunity

vpn-target 0:6 export-extcommunity evpn

vpn-target 0:6 import-extcommunity

vpn-target 0:6 import-extcommunity evpn

ipv6-family

vxlan vni 6

#

ip vpn-instance OA

ipv4-family

route-distinguisher 3:2

vpn-target 0:2 export-extcommunity

vpn-target 0:2 export-extcommunity evpn

vpn-target 0:2 import-extcommunity

vpn-target 0:2 import-extcommunity evpn

ipv6-family

vxlan vni 2

#

ip vpn-instance RD

ipv4-family

route-distinguisher 3:4

vpn-target 0:4 export-extcommunity

vpn-target 0:4 export-extcommunity evpn

vpn-target 0:4 import-extcommunity

vpn-target 0:4 import-extcommunity evpn

ipv6-family

vxlan vni 4

#

ip vpn-instance netresvrf_f8a775d11

ipv4-family

route-distinguisher 1002:1

vpn-target 0:1 export-extcommunity

vpn-target 0:1 export-extcommunity evpn

vpn-target 0:1 1:0 import-extcommunity

vpn-target 0:1 1:0 import-extcommunity evpn

ipv6-family

vxlan vni 1

#

qos-profile name Guest_Result

qos-profile name Research_Result

qos-profile name Sales_Result

#

vlan 1

description vlan1

name vlan1

#

aaa

service-scheme Guest_Result

user-vlan 101

qos-profile Guest_Result

service-scheme Research_Result

user-vlan 102

qos-profile Research_Result

service-scheme Sales_Result

user-vlan 103

qos-profile Sales_Result

domain default

authentication-scheme radius

accounting-scheme default

radius-server default

domain default_admin

authentication-scheme default

accounting-scheme default

domain domain_accampus

authentication-scheme default

accounting-scheme default

radius-server default

local-user admin password irreversible-cipher $1c$~-c=I5N83I$8@EsJ;nU7WILdEI)ovdDfTEB!GQn3IL|5L$S0G04$

local-user admin privilege level 15

local-user admin service-type terminal ssh http

local-user accampus@domain_accampus password irreversible-cipher $1c$^T(2&ni*1;$`%;TW^AgOT}UGt02.~WA6WQ+DLZb.%qDT[4.DPr&$

local-user accampus@domain_accampus privilege level 15

local-user accampus@domain_accampus service-type ssh http

#

interface Vlanif1

ip address dhcp-alloc

#

interface Vlanif11

ip address 172.20.0.10 255.255.255.252

ospf enable 1 area 0.0.0.0

#

interface Vlanif12

ip address 172.20.0.14 255.255.255.252

ospf enable 1 area 0.0.0.0

#

interface Vlanif120

ip binding vpn-instance OA

ip address 192.168.120.1 255.255.255.252

#

interface Vlanif121

ip binding vpn-instance RD

ip address 192.168.121.1 255.255.255.252

#

interface Vlanif122

ip binding vpn-instance MKT

ip address 192.168.122.1 255.255.255.252

#

interface Vlanif130

ip binding vpn-instance netresvrf_f8a775d11

ip address 192.168.130.1 255.255.255.252

#

interface MEth0/0/1

ip address 192.168.1.253 255.255.255.0

#

interface MEth0/0/2

ip address 169.254.2.1 255.255.255.252

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 120 to 122 130

trust dscp

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 11

trust dscp

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 12

trust dscp

#

interface LoopBack1

ip address 1.1.1.2 255.255.255.255

ospf enable 1 area 0.0.0.0

#

interface Nve1

source 1.1.1.2

#

bgp 100

router-id 1.1.1.2

peer 1.1.1.0 as-number 100

peer 1.1.1.0 connect-interface LoopBack1

peer 1.1.1.1 as-number 100

peer 1.1.1.1 connect-interface LoopBack1

#

ipv4-family unicast

undo synchronization

peer 1.1.1.0 enable

peer 1.1.1.1 enable

#

l2vpn-family evpn

reflector cluster-id 1

undo policy vpn-target

peer 1.1.1.0 enable

peer 1.1.1.0 advertise irb

peer 1.1.1.0 advertise irbv6

peer 1.1.1.0 reflect-client

peer 1.1.1.1 enable

peer 1.1.1.1 advertise irb

peer 1.1.1.1 advertise irbv6

peer 1.1.1.1 reflect-client

#

ipv4-family vpn-instance MKT

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ipv4-family vpn-instance OA

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ipv4-family vpn-instance RD

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ipv4-family vpn-instance netresvrf_f8a775d11

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ospf 1 router-id 1.1.1.2

import-route direct cost 1 type 2 tag 1

area 0.0.0.0

#

ip route-static vpn-instance netresvrf_f8a775d11 192.168.130.2 255.255.255.255 192.168.130.2

ip route-static vpn-instance OA 0.0.0.0 0.0.0.0 192.168.120.2

ip route-static vpn-instance RD 172.16.3.0 255.255.255.0 vpn-instance MKT

ip route-static vpn-instance RD 172.16.12.0 255.255.255.0 192.168.121.2

ip route-static vpn-instance MKT 172.16.2.0 255.255.255.0 vpn-instance RD

ip route-static vpn-instance MKT 172.16.13.0 255.255.255.0 192.168.122.2

#

netconf

management-vlan 1

smi-server

#

return

Configurations of AGG1

[AGG1]display current-configuration

#

sysname AGG1

#

vlan batch 10 to 11

#

lnp disable

#

ucl-group 1 name Guest_Group

ucl-group 5 name Research_Group

ucl-group 6 name Sales_Group

#

dhcp enable

#

qos-profile name Guest_Result

qos-profile name Research_Result

qos-profile name Sales_Result

#

vlan 1

description vlan1

name vlan1

#

aaa

service-scheme Guest_Result

user-vlan 101

qos-profile Guest_Result

service-scheme Research_Result

user-vlan 102

qos-profile Research_Result

service-scheme Sales_Result

user-vlan 103

qos-profile Sales_Result

domain default

authentication-scheme radius

accounting-scheme default

radius-server default

domain default_admin

authentication-scheme default

accounting-scheme default

domain domain_accampus

authentication-scheme default

accounting-scheme default

radius-server default

local-user admin password irreversible-cipher $1c$.S.N>aBW~A$"Iyl4PgqH>G>QqJQKDIGo)lJDx~6+>BXk_NmsUwL$

local-user admin privilege level 15

local-user admin service-type terminal ssh http

local-user accampus@domain_accampus password irreversible-cipher $1c$pQiM%2u|R%$"q*5@^#UFDQ"iv7PEr_5Kw*+&c`+wY{gA&KabB')$

local-user accampus@domain_accampus privilege level 15

local-user accampus@domain_accampus service-type ssh http

#

interface Vlanif1

ip address dhcp-alloc

#

interface Vlanif10

ip address 172.20.0.2 255.255.255.252

ospf enable 1 area 0.0.0.0

#

interface Vlanif11

ip address 172.20.0.9 255.255.255.252

ospf enable 1 area 0.0.0.0

#

interface MEth0/0/1

ip address 192.168.1.253 255.255.255.0

#

interface MEth0/0/2

ip address 169.254.2.1 255.255.255.252

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 11

trust dscp

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 10

trust dscp

#

ospf 1 router-id 172.20.0.9

import-route direct cost 1 type 2 tag 1

area 0.0.0.0

#

netconf

management-vlan 1

smi-server

#

return

Configurations of AGG2

[AGG2]display current-configuration

#

sysname AGG2

#

vlan batch 10 12

#

lnp disable

#

ucl-group 1 name Guest_Group

ucl-group 5 name Research_Group

ucl-group 6 name Sales_Group

#

dhcp enable

#

qos-profile name Guest_Result

qos-profile name Research_Result

qos-profile name Sales_Result

#

vlan 1

description vlan1

name vlan1

#

aaa

service-scheme Guest_Result

user-vlan 101

qos-profile Guest_Result

service-scheme Research_Result

user-vlan 102

qos-profile Research_Result

service-scheme Sales_Result

user-vlan 103

qos-profile Sales_Result

domain default

authentication-scheme radius

accounting-scheme default

radius-server default

domain default_admin

authentication-scheme default

accounting-scheme default

domain domain_accampus

authentication-scheme default

accounting-scheme default

radius-server default

local-user admin password irreversible-cipher $1c$`3Y|@!U-b8$c,$j83(n-9oeB%,d7J:A_-CW.(ej@,'4j,*IA.x'$

local-user admin privilege level 15

local-user admin service-type terminal ssh http

local-user accampus@domain_accampus password irreversible-cipher $1c$#hVk;2ry5U$&}MLXwwg9<d:hI5oUVa"eeQ6$+daH9}`}s#;Sb;N$

local-user accampus@domain_accampus privilege level 15

local-user accampus@domain_accampus service-type ssh http

#

interface Vlanif1

ip address dhcp-alloc

#

interface Vlanif10

ip address 172.20.0.6 255.255.255.252

ospf enable 1 area 0.0.0.0

#

interface Vlanif12

ip address 172.20.0.13 255.255.255.252

ospf enable 1 area 0.0.0.0

#

interface MEth0/0/1

ip address 192.168.1.253 255.255.255.0

#

interface MEth0/0/2

ip address 169.254.2.1 255.255.255.252

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 12

trust dscp

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 10

trust dscp

#

ospf 1 router-id 172.20.0.13

import-route direct cost 1 type 2 tag 1

area 0.0.0.0

#

netconf

management-vlan 1

smi-server

#

return

Configurations of ACC1

[ACC1]display current-configuration

#

sysname ACC1

#

vlan batch 10 101 to 103

#

lnp disable

#

authentication-profile name HQ_MAC_802.1X

dot1x-access-profile dot1x_access_profile

mac-access-profile mac_access_profile

access-domain default force

ucl-group 1 name Guest_Group

ucl-group 5 name Research_Group

ucl-group 6 name Sales_Group

#

portal https-redirect wired enable

#

dhcp enable

#

dhcp option82 vendor-specific format vendor-sub-option 2 ip-address 1.1.1.0

#

ip vpn-instance MKT

ipv4-family

route-distinguisher 2:6

vpn-target 0:6 1:0 export-extcommunity

vpn-target 0:6 1:0 export-extcommunity evpn

vpn-target 0:6 0:1 import-extcommunity

vpn-target 0:6 0:1 import-extcommunity evpn

ipv6-family

vxlan vni 6

#

ip vpn-instance OA

ipv4-family

route-distinguisher 2:2

vpn-target 0:2 1:0 export-extcommunity

vpn-target 0:2 1:0 export-extcommunity evpn

vpn-target 0:2 0:1 import-extcommunity

vpn-target 0:2 0:1 import-extcommunity evpn

ipv6-family

vxlan vni 2

#

ip vpn-instance RD

ipv4-family

route-distinguisher 2:4

vpn-target 0:4 1:0 export-extcommunity

vpn-target 0:4 1:0 export-extcommunity evpn

vpn-target 0:4 0:1 import-extcommunity

vpn-target 0:4 0:1 import-extcommunity evpn

ipv6-family

vxlan vni 4

#

evpn vpn-instance evpn_1 bd-mode

route-distinguisher 2:3

vpn-target 0:2 0:3 export-extcommunity

vpn-target 0:3 import-extcommunity

#

evpn vpn-instance evpn_2 bd-mode

route-distinguisher 2:5

vpn-target 0:4 0:5 export-extcommunity

vpn-target 0:5 import-extcommunity

#

evpn vpn-instance evpn_3 bd-mode

route-distinguisher 2:7

vpn-target 0:6 0:7 export-extcommunity

vpn-target 0:7 import-extcommunity

#

radius-server template HQ_RADIUS_Server

radius-server shared-key cipher %^%#X-|CL9Kla@$/IZ9KhP,-/OeTM]FsBZO#t'CHv$l0%^%#

radius-server authentication 192.168.4.104 1812 weight 80

radius-server accounting 192.168.4.104 1813 weight 80

radius-server template default

radius-server authorization 192.168.4.104 shared-key cipher %^%#}uxT*s~/5OEyauEx&x!"BjOMV=#:hNh"SB/MgNgQ%^%#

#

acl number 9996

rule 1 permit ip source ucl-group name Research_Group destination ucl-group name Guest_Group

rule 2 permit ip source ucl-group name Research_Group destination ucl-group name Sales_Group

acl number 9997

rule 1 deny ip source ucl-group 0 destination ucl-group name Research_Group

rule 2 deny ip source ucl-group 0 destination ucl-group name Sales_Group

acl number 9998

rule 1 permit ip source ucl-group name Sales_Group destination ucl-group name Research_Group

rule 2 permit ip source ucl-group name Sales_Group destination ucl-group name Guest_Group

acl number 9999

rule 1 deny ip source ucl-group name Guest_Group destination ucl-group name Research_Group

rule 2 deny ip source ucl-group name Guest_Group destination ucl-group name Sales_Group

#

qos-profile name Guest_Result

qos-profile name Research_Result

qos-profile name Sales_Result

#

vlan 1

description vlan1

name vlan1

#

aaa

authentication-scheme HQ_RADIUS_Server

authentication-mode radius

accounting-scheme HQ_RADIUS_Server

accounting-mode radius

accounting realtime 5

service-scheme Guest_Result

user-vlan 101

qos-profile Guest_Result

service-scheme Research_Result

user-vlan 102

qos-profile Research_Result

service-scheme Sales_Result

user-vlan 103

qos-profile Sales_Result

domain default

authentication-scheme HQ_RADIUS_Server

accounting-scheme HQ_RADIUS_Server

radius-server HQ_RADIUS_Server

domain default_admin

authentication-scheme default

accounting-scheme default

domain domain_accampus

authentication-scheme default

accounting-scheme default

radius-server default

local-user admin password irreversible-cipher $1c$w6<r(*sKE($9+y)UyvXv-q*'X!Le`k#=9}j3<Cb<MWfDI(1P<PJ$

local-user admin privilege level 15

local-user admin service-type terminal ssh http

local-user accampus@domain_accampus password irreversible-cipher $1c$,{'zR^Q5Y2$D5u:8$~"|;Zf2xC=Pet;caV>>rMO(G1;mYGDVm9)$

local-user accampus@domain_accampus privilege level 15

local-user accampus@domain_accampus service-type ssh http

#

bridge-domain 1

l2 binding vlan 101

vxlan vni 3

evpn binding vpn-instance evpn_1

dhcp option82 rebuild enable

dhcp option82 encapsulation vendor-specific-id

bridge-domain 2

l2 binding vlan 102

vxlan vni 5

evpn binding vpn-instance evpn_2

dhcp option82 rebuild enable

dhcp option82 encapsulation vendor-specific-id

bridge-domain 3

l2 binding vlan 103

vxlan vni 7

evpn binding vpn-instance evpn_3

dhcp option82 rebuild enable

dhcp option82 encapsulation vendor-specific-id

#

interface Vlanif1

ip address dhcp-alloc

#

interface Vlanif10

ip address 172.20.0.1 255.255.255.252

ospf enable 1 area 0.0.0.0

#

interface MEth0/0/1

ip address 192.168.1.253 255.255.255.0

#

interface MEth0/0/2

ip address 169.254.2.1 255.255.255.252

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 10

trust dscp

#

interface GigabitEthernet0/0/24

port link-type hybrid

authentication-profile HQ_MAC_802.1X

port-isolate enable group 1

trust dscp

#

interface LoopBack1

ip address 1.1.1.0 255.255.255.255

ospf enable 1 area 0.0.0.0

#

interface Vbdif1

mac-address 0000-5e00-0102

ip binding vpn-instance OA

arp collect host enable

arp distribute-gateway enable

ip address 172.16.1.1 255.255.255.0

arp-proxy enable

arp-proxy local enable

dhcp select relay

dhcp relay server-ip 192.168.130.2

dhcp relay anycast gateway re-route enable

#

interface Vbdif2

mac-address 0000-5e00-0102

ip binding vpn-instance RD

arp collect host enable

arp distribute-gateway enable

ip address 172.16.2.1 255.255.255.0

arp-proxy enable

arp-proxy local enable

dhcp select relay

dhcp relay server-ip 192.168.130.2

dhcp relay anycast gateway re-route enable

#

interface Vbdif3

mac-address 0000-5e00-0102

ip binding vpn-instance MKT

arp collect host enable

arp distribute-gateway enable

ip address 172.16.3.1 255.255.255.0

arp-proxy enable

arp-proxy local enable

dhcp select relay

dhcp relay server-ip 192.168.130.2

dhcp relay anycast gateway re-route enable

#

interface Nve1

source 1.1.1.0

vni 3 head-end peer-list protocol bgp

vni 5 head-end peer-list protocol bgp

vni 7 head-end peer-list protocol bgp

#

bgp 100

router-id 1.1.1.0

peer 1.1.1.2 as-number 100

peer 1.1.1.2 connect-interface LoopBack1

#

ipv4-family unicast

undo synchronization

peer 1.1.1.2 enable

#

l2vpn-family evpn

policy vpn-target

peer 1.1.1.2 enable

peer 1.1.1.2 advertise irb

peer 1.1.1.2 advertise irbv6

#

ipv4-family vpn-instance MKT

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ipv4-family vpn-instance OA

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ipv4-family vpn-instance RD

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ospf 1 router-id 1.1.1.0

import-route direct cost 1 type 2 tag 1

area 0.0.0.0

#

traffic-filter inbound acl 9999

traffic-filter inbound acl 9998

traffic-filter inbound acl 9997

traffic-filter inbound acl 9996

#

netconf

management-vlan 1

smi-server

#

return

Configurations of ACC2

[ACC2]display current-configuration

#

sysname ACC2

#

vlan batch 10 101 to 103 111

#

lnp disable

#

authentication-profile name HQ_MAC_802.1X

dot1x-access-profile dot1x_access_profile

mac-access-profile mac_access_profile

access-domain default force

authentication-profile name HQ_Portal

portal-access-profile HQ_Portal_Server

access-domain default force

ucl-group 1 name Guest_Group

ucl-group 5 name Research_Group

ucl-group 6 name Sales_Group

#

lldp auto-vlan 111 sensor ap

#

portal https-redirect wired enable

#

dhcp enable

#

dhcp option82 vendor-specific format vendor-sub-option 2 ip-address 1.1.1.1

#

ip vpn-instance MKT

ipv4-family

route-distinguisher 1:6

vpn-target 0:6 1:0 export-extcommunity

vpn-target 0:6 1:0 export-extcommunity evpn

vpn-target 0:6 0:1 import-extcommunity

vpn-target 0:6 0:1 import-extcommunity evpn

ipv6-family

vxlan vni 6

#

ip vpn-instance OA

ipv4-family

route-distinguisher 1:2

vpn-target 0:2 1:0 export-extcommunity

vpn-target 0:2 1:0 export-extcommunity evpn

vpn-target 0:2 0:1 import-extcommunity

vpn-target 0:2 0:1 import-extcommunity evpn

ipv6-family

vxlan vni 2

#

ip vpn-instance RD

ipv4-family

route-distinguisher 1:4

vpn-target 0:4 1:0 export-extcommunity

vpn-target 0:4 1:0 export-extcommunity evpn

vpn-target 0:4 0:1 import-extcommunity

vpn-target 0:4 0:1 import-extcommunity evpn

ipv6-family

vxlan vni 4

#

evpn vpn-instance evpn_1 bd-mode

route-distinguisher 1:3

vpn-target 0:2 0:3 export-extcommunity

vpn-target 0:3 import-extcommunity

#

evpn vpn-instance evpn_2 bd-mode

route-distinguisher 1:5

vpn-target 0:4 0:5 export-extcommunity

vpn-target 0:5 import-extcommunity

#

evpn vpn-instance evpn_3 bd-mode

route-distinguisher 1:7

vpn-target 0:6 0:7 export-extcommunity

vpn-target 0:7 import-extcommunity

#

radius-server template HQ_RADIUS_Server

radius-server shared-key cipher %^%#!}">;Jeb]+ECA+"]50:5$JBNR=21KJVC1R6Ce*s0%^%#

radius-server authentication 192.168.4.104 1812 weight 80

radius-server accounting 192.168.4.104 1813 weight 80

called-station-id wlan-user-format ac-mac include-ssid

radius-server template default

radius-server authorization 192.168.4.104 shared-key cipher %^%#[S~^#\a!vT!l|G<n0p%(/.#l)wiyh2",n~31]d~W%^%#

#

acl number 9996

rule 1 permit ip source ucl-group name Research_Group destination ucl-group name Guest_Group

rule 2 permit ip source ucl-group name Research_Group destination ucl-group name Sales_Group

acl number 9997

rule 1 deny ip source ucl-group 0 destination ucl-group name Research_Group

rule 2 deny ip source ucl-group 0 destination ucl-group name Sales_Group

acl number 9998

rule 1 permit ip source ucl-group name Sales_Group destination ucl-group name Research_Group

rule 2 permit ip source ucl-group name Sales_Group destination ucl-group name Guest_Group

acl number 9999

rule 1 deny ip source ucl-group name Guest_Group destination ucl-group name Research_Group

rule 2 deny ip source ucl-group name Guest_Group destination ucl-group name Sales_Group

#

qos-profile name Guest_Result

qos-profile name Research_Result

qos-profile name Sales_Result

#

url-template name HQ_Portal_Server

url https://192.168.4.104:19008/portal

url-parameter device-mac lsw-mac redirect-url redirect-url ssid ssid user-ipaddress uaddress user-mac umac

#

web-auth-server HQ_Portal_Server

server-ip 192.168.4.104

port 50100

shared-key cipher %^%#YF'GGsWOp)0~F5P*xk26y_bU%*|>]Ual(W%Z}WG8%^%#

url https://192.168.4.104:19008/portal

url-template HQ_Portal_Server

#

portal-access-profile name HQ_Portal_Server

web-auth-server HQ_Portal_Server direct

#

vlan 1

description vlan1

name vlan1

#

aaa

authentication-scheme HQ_RADIUS_Server

authentication-mode radius

accounting-scheme HQ_RADIUS_Server

accounting-mode radius

accounting realtime 5

service-scheme Guest_Result

user-vlan 101

qos-profile Guest_Result

service-scheme Research_Result

user-vlan 102

qos-profile Research_Result

service-scheme Sales_Result

user-vlan 103

qos-profile Sales_Result

domain default

authentication-scheme HQ_RADIUS_Server

accounting-scheme HQ_RADIUS_Server

radius-server HQ_RADIUS_Server

domain default_admin

authentication-scheme default

accounting-scheme default

domain domain_accampus

authentication-scheme default

accounting-scheme default

radius-server default

local-user admin password irreversible-cipher $1c$T{N6Es0g>8$;>]!S}aWsOKXD^1iMeZIXwq8<|j[USSD-:&Qk]99$

local-user admin privilege level 15

local-user admin service-type terminal ssh http

local-user accampus@domain_accampus password irreversible-cipher $1c$WpN(YlG4l7$WKjCJONZ0Vi6Fg~|\AQHmFcV*4qi%QTIlr$(w6oX$

local-user accampus@domain_accampus privilege level 15

local-user accampus@domain_accampus service-type ssh http

#

bridge-domain 1

l2 binding vlan 101

vxlan vni 3

evpn binding vpn-instance evpn_1

dhcp option82 rebuild enable

dhcp option82 encapsulation vendor-specific-id

bridge-domain 2

l2 binding vlan 102

vxlan vni 5

evpn binding vpn-instance evpn_2

dhcp option82 rebuild enable

dhcp option82 encapsulation vendor-specific-id

bridge-domain 3

l2 binding vlan 103

vxlan vni 7

evpn binding vpn-instance evpn_3

dhcp option82 rebuild enable

dhcp option82 encapsulation vendor-specific-id

#

interface Vlanif1

ip address dhcp-alloc

#

interface Vlanif10

ip address 172.20.0.5 255.255.255.252

ospf enable 1 area 0.0.0.0

#

interface Vlanif111

ip address 172.111.1.1 255.255.254.0

dhcp select interface

dhcp server option 43 ip-address 172.111.1.1

#

interface MEth0/0/1

ip address 192.168.1.253 255.255.255.0

#

interface MEth0/0/2

ip address 169.254.2.1 255.255.255.252

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 10

trust dscp

#

interface GigabitEthernet0/0/24

port link-type hybrid

authentication-profile HQ_MAC_802.1X

port-isolate enable group 1

trust dscp

#

interface LoopBack1

ip address 1.1.1.1 255.255.255.255

ospf enable 1 area 0.0.0.0

#

interface Vbdif1

mac-address 0000-5e00-0102

ip binding vpn-instance OA

arp collect host enable

arp distribute-gateway enable

ip address 172.16.1.1 255.255.255.0

arp-proxy enable

arp-proxy local enable

dhcp select relay

dhcp relay server-ip 192.168.130.2

dhcp relay anycast gateway re-route enable

#

interface Vbdif2

mac-address 0000-5e00-0102

ip binding vpn-instance RD

arp collect host enable

arp distribute-gateway enable

ip address 172.16.2.1 255.255.255.0

arp-proxy enable

arp-proxy local enable

dhcp select relay

dhcp relay server-ip 192.168.130.2

dhcp relay anycast gateway re-route enable

#

interface Vbdif3

mac-address 0000-5e00-0102

ip binding vpn-instance MKT

arp collect host enable

arp distribute-gateway enable

ip address 172.16.3.1 255.255.255.0

arp-proxy enable

arp-proxy local enable

dhcp select relay

dhcp relay server-ip 192.168.130.2

dhcp relay anycast gateway re-route enable

#

interface Nve1

source 1.1.1.1

vni 3 head-end peer-list protocol bgp

vni 5 head-end peer-list protocol bgp

vni 7 head-end peer-list protocol bgp

#

bgp 100

router-id 1.1.1.1

peer 1.1.1.2 as-number 100

peer 1.1.1.2 connect-interface LoopBack1

#

ipv4-family unicast

undo synchronization

peer 1.1.1.2 enable

#

l2vpn-family evpn

policy vpn-target

peer 1.1.1.2 enable

peer 1.1.1.2 advertise irb

peer 1.1.1.2 advertise irbv6

#

ipv4-family vpn-instance MKT

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ipv4-family vpn-instance OA

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ipv4-family vpn-instance RD

default-route imported

import-route direct

import-route static

maximum load-balancing 8

advertise l2vpn evpn

#

ospf 1 router-id 1.1.1.1

import-route direct cost 1 type 2 tag 1

area 0.0.0.0

#

traffic-filter inbound acl 9999

traffic-filter inbound acl 9998

traffic-filter inbound acl 9997

traffic-filter inbound acl 9996

#

capwap source interface vlanif111

#

wlan

security-profile name default

security-profile name default-wds

security-profile name default-mesh

ssid-profile name HQ_SSID

ssid HQ_Guest

vap-profile name HQ_VAP

forward-mode tunnel

service-vlan vlan-id 101

ssid-profile HQ_SSID

authentication-profile HQ_Portal

regulatory-domain-profile name default

ap auth-mode sn-auth

ap-group name default

radio 0

vap-profile HQ_VAP wlan 1

radio 1

vap-profile HQ_VAP wlan 1

radio 2

vap-profile HQ_VAP wlan 1

ap-id 0 type-id 115 ap-mac f4de-af36-b160 ap-sn 2102352UBR10L6001282

ap-name AP1

#

as-auth

auth-mode none

#

netconf

management-vlan 1

smi-server

#

Return

Quiz

For the VN configured on the iMaster NCE controller, the specific commands on the device are?

Small- and Medium-Sized Campus Network Deployment

Introduction

About This Lab

With technology advances and industry digitalization, chain stores and small enterprise branches alike require IT systems featuring high levels of information integration. However, they generally lack professional IT maintenance personnel, so HQ personnel have to travel to maintain IT systems onsite, resulting in high costs. For this reason, cloud management has become a trend for small- and medium-sized campus networks. By deploying cloud-managed APs, routers, switches, and firewalls, enterprises can quickly deploy and provision services while reducing O&M costs and greatly improving management efficiency.

This lab guides you through how to deploy Huawei's iMaster NCE-Campus-based small- and medium-sized campus network solution.

Objectives

Upon completion of this lab, you will be able to:

Understand the process of creating sites and onboarding devices.

Master how to deliver the IP address of iMaster NCE-Campus through DHCP Option 148.

Know how to configure AR services on the web pages of iMaster NCE-Campus.

Grasp how to configure AP services on the web pages of iMaster NCE-Campus.

Networking Topology

Small- and medium-sized campus network deployment topology in Huawei's CloudCampus Solution

The entire network topology consists of two parts: branch (Site2) and cloud.

The branch consists of one AR2 (AR6120) and one AP2 (AirEngine 5760-51). AR2 functions as the campus egress and also provides wireless access for AP2. AP2 is directly connected to the Layer 2 interface of AR2. AR2 allocates IP addresses through DHCP and also implements NAT for access to the external network. AP2 works in cloud management mode. All devices at the branch register with iMaster NCE-Campus for unified management.

The cloud is simulated by a switch and is responsible for connecting the branch to the access switch where iMaster NCE-Campus resides.

In this lab, AR2 in the branch obtains the IP address of iMaster NCE-Campus through DHCP; it also functions as the DHCP gateway to allocate an IP address to AP2, and notifies AP2 of the IP address of iMaster NCE-Campus through DHCP Option 148.

Lab Configuration

Configuration Roadmap

- On iMaster NCE-Campus, create a site named Branch and add AR2 to the site. Then, restart AR2 so that AR2 can obtain an IP address through DHCP and go online.

- On AR2, configure the DHCP address pool, VLAN on the LAN interface, and NAT on the WAN interface. In this way, the downstream AP2 can obtain its IP address and the IP address of iMaster NCE-Campus as well as communicate with iMaster NCE-Campus through AR2.

- On iMaster NCE-Campus, add AP2. After AP2 goes online, configure wireless services, including creating an SSID (iMaster_NCE_Demo) with Portal authentication for guests, an SSID (Employee) with 802.1X authentication for employees, and accounts used for authentication.

- Connect a wireless terminal to the configured SSIDs and verify the authentication results.

Configuration Procedure

Onboarding the Egress Device at the Branch

Precautions:

In this lab, AR2 functions as the egress device at the branch and goes online in DHCP mode. In general, when DHCP mode is used, routers do not need to be configured, and they must be unconfigured, have no console port input, or have no user login. In this way, after such routers are connected to the network, they automatically obtain the cloud management platform's IP address information using DHCP.

The static IP address and DHCP Option 148 have been assigned to AR2 during lab environment pre-configuration. Keep AR2 running without any configuration.

          - # Check the ESN of AR2.

<AR2>display esn

ESN of device: 1002352RLG1980065092

An ESN is used to add a device on iMaster NCE-Campus. You can obtain an ESN from the device label or obtain an ESN by running the corresponding command.

On iMaster NCE-Campus, create a site and add a device to the site.

Log in to the O&M plane of iMaster NCE-Campus using a tenant account, create a site named Branch, and add AR2 to the site.

# Choose Design > Site Management and click Create.

# Create a site named Branch.

Set Site Name to Branch. In Device Type, select AR and AP. Then, click By ESN, enter the device name and ESN, and click OK.

Restore AR2 to factory settings.

To meet the requirements of unconfigured routers, ensure that AR2 uses factory settings. If AR2 is a new device, skip this step.

# Restore AR2 to factory settings.

<AR2>set factory-configuration from default

Warning: The current factory configuration will be replaced, and it's irreversible. Are you sure to set the factory configuration?[Y/N]:y

Info: Successfully set factory config!

# Restart AR2. Note: Do not save the configuration.

<AR2>reboot fast

System will reboot! Continue? [y/n]:y

Restart AR2 quickly without saving the configuration.

Check the online status of AR2.

After restart, AR2 attempts to obtain an IP address from the WAN interface (GE0/0/8 of AR6120 in this lab) through DHCP. During the DHCP process, AR2 also attempts to obtain the IP address of iMaster NCE-Campus through DHCP Option 148. Do not perform any input operations on AR2 until AR2 goes online.

# Wait until AR2 completes restart.

Begin to start NGE process. Time(s):1

OK

Press any key to get started

If the Press any key to get started message is displayed, AR2 has been restarted.

# On SW1, check whether AR2 obtains an IP address.

<SW1>display arp all  | in 67.0.0.2

IP ADDRESS   MAC ADDRESS    EXPIRE(M)    TYPE       INTERFACE           VPN-INSTANCE

VLAN/CEVLAN(SIP/DIP)          PVC

------------------------------------------------------------------------------

67.0.0.254      ac75-1db1-5148                  I -          Vlanif67

67.0.0.2        c8a7-7600-3f63     19           D-0       		 GE0/0/18

------------------------------------------------------------------------------

Total:14        Dynamic:11      Static:0     Interface:3

The ARP information shows that AR2 has obtained an IP address.

An AR must obtain an IP address through the WAN interface. Otherwise, the AR cannot automatically register with iMaster NCE-Campus.

# On iMaster NCE-Campus, check whether AR2 goes online.

Choose Design > Site Management > Branch. You can see that AR2 has gone online (that is, not in the unregistered status) at the branch.

# Log in to AR2 to check its online status.

<AR2>display agile-controller status

********************************************************************************

*                       Register to AC status information                      *

********************************************************************************

AC Host                       192.168.4.104

AC Port                        10020

Register Result                  Success

The command output shows that AR2 has been successfully registered with iMaster NCE-Campus. You can view the IP address and port number (10020) of iMaster NCE-Campus.

(Optional) Manually specify the IP address of iMaster NCE-Campus.

In addition to DHCP mode, you can also manually specify the IP address of iMaster NCE-Campus on the AR. Compared with the DHCP mode, the manual mode does not require restoring the AR's factory settings and only needs to ensure the IP reachability between the AR and iMaster NCE-Campus.

If the manual mode is used, skip Step 3 and manually set DHCP as the mode to obtain an IP address from the WAN interface on AR2.

# Specify the IP address and port number of iMaster NCE-Campus.

[AR2]agile controller host 192.168.4.104 port 10020

By default, the port number of iMaster NCE-Campus is 10020.

----End

Onboarding the AP at the Branch

Configure the DHCP function on AR2 to assign an IP address to the downstream AP2 and notify AP2 of the IP address of iMaster NCE-Campus through DHCP Option 148. To ensure the IP connectivity between AP2 and iMaster NCE-Campus, configure source NAT on AR2 so that AP2 can access iMaster NCE-Campus through the IP address of the WAN interface on AR2.

          - Create an IP address pool.

# Choose Site Configuration > Site, switch to the Branch site, and click AR.

# Click LAN, select Local Internet access, and click Create.

# Enter the subnet name, VLAN ID, IP address, and mask, click Create in DHCP option, and select cloud platform address(148) in Option.

# Click Value. In the dialog box that is displayed, enter the IP address and port number of iMaster NCE-Campus.

Specifically, change the information highlighted in the red frame to the IP address of iMaster NCE-Campus. In this lab, enter 192.168.4.104.

The following figure shows the final result.

The complete value of DHCP Option 148 is as follows, which is for your reference:

agilemode=agile-cloud;agilemanage-mode=domain;agilemanage-domain=192.168.4.104;agilemanage-port=10020;

# Click Submit and then OK.

# Log in to AR2 to check the configuration.

<AR2>display current-configuration interface vlanif10

[V300R019C10SPC300]

#

interface Vlanif10

ip address 172.16.10.254 255.255.255.0

zone trust

dhcp select interface

dhcp server dns-list 172.16.10.254

dhcp server option 148 ascii agilemode=agile-cloud;agilemanage-mode=domain;agilemanage-domain=192.168.4.104;agilemanage-port=10020;

On AR2, check the configuration of VLANIF 10. The command output shows that VLANIF 10 has been generated, its IP address is 172.16.10.254, interface-based DHCP has been enabled, and Option 148 has been configured.

Configure a LAN interface on AR2.

Configure the interface connecting AR2 to AP2, allow packets from VLAN 10 to pass through, and change the PVID to 10, so that AP2 can obtain its IP address and the IP address of iMaster NCE-Campus through DHCP.

# On the Site Configuration tab page, choose AR > Interface > Customized.

# Click the interface connecting to the AP. In this lab, the interface is GE0/0/1.

# Set both Default VLAN and Allowed VLAN to 10, so that the AP can obtain the IP address of VLANIF 10.

# Check the configuration of GE0/0/1 on AR2.

<AR2>display current-configuration interface GigabitEthernet 0/0/1

[V300R019C10SPC300]

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 10

undo port trunk allow-pass vlan 1

port trunk allow-pass vlan 10

The command output shows that GE0/0/1 allows packets from VLAN 10 to pass through and that its PVID is VLAN 10.

# Check whether AP2 has obtained an IP address.

<AR2>display arp all | in 172.16.10

IP ADDRESS  MAC ADDRESS      EXPIRE(M)  TYPE        INTERFACE        VPN-INSTANCE

VLAN/CEVLAN(SIP/DIP)        PVC

--------------------------------------------------------------------------------------------------------------------

172.16.10.254   c8a7-7600-3f62                 I -              Vlanif10

172.16.10.236   f4de-af36-b580     20          D-0            	GE0/0/1

--------------------------------------------------------------------------------------------------------------------

Total:7         Dynamic:4       Static:0     Interface:3

The command output shows that apart from a gateway IP entry, there is an ARP entry with the interface being GE0/0/1, indicating that AP2 has obtained an IP address.

Configure NAT on AR2.

AP2 has obtained its IP address and the IP address of iMaster NCE-Campus. However, SW1 does not have a route destined for 172.16.10.0/24. To ensure the IP reachability between AP2 and iMaster NCE-Campus, you need to configure source NAT on AR2.

# Choose Design > Template Management > Policy Template, click ACL, and click Create to create an ACL for AR2 to perform source NAT.

# Set ACL parameters.

Specifically, click IPv4, enter the name, set ACL type to Advanced, click Add, and set Source IP Address to 172.16.1.0/24 (which is the network segment where AP2 resides). Click √ and then click OK.

Note that the ACL number must be greater than 3100.

# Choose Provision > Site Management, choose AR > Network, and click Create in the NAT configuration area to create a NAT rule.

# Configure the NAT rule parameters.

Select GE0/0/8 (WAN interface on AR2 in this lab) from the drop-down list box marked 1 and click the area marked 2. In the dialog box that is displayed, select the created ACL template and click OK.

# Check the NAT configuration on AR2.

<AR2>display current-configuration interface GigabitEthernet0/0/8

[V300R019C10SPC300]

#

interface GigabitEthernet0/0/8

nat outbound 3101

zone untrust

ip address dhcp-alloc

The command output shows that the source NAT configuration has been delivered to GE0/0/8.

# Check the IP connectivity between 172.16.10.0/24 and iMaster NCE-Campus.

<AR2>ping -a 172.16.10.254 192.168.4.104

PING 192.168.4.104: 56  data bytes, press CTRL_C to break

Reply from 192.168.4.104: bytes=56 Sequence=1 ttl=62 time=1 ms

Reply from 192.168.4.104: bytes=56 Sequence=2 ttl=62 time=1 ms

Reply from 192.168.4.104: bytes=56 Sequence=3 ttl=62 time=1 ms

Reply from 192.168.4.104: bytes=56 Sequence=4 ttl=62 time=1 ms

Reply from 192.168.4.104: bytes=56 Sequence=5 ttl=62 time=1 ms

--- 192.168.4.104 ping statistics ---

5 packet(s) transmitted

5 packet(s) received

0.00% packet loss

round-trip min/avg/max = 1/1/1 ms

The command output shows that AP2's network segment can communicate with iMaster NCE-Campus.

Add AP2 to the Branch site on iMaster NCE-Campus.

Add AP2 to the Branch site using the ESN. The ESN can be obtained from the AP label or can be obtained by logging in to the AP through SSH and running commands on the AP after it is connected using the default wireless signal. For details, see the AP product documentation.

The following figure shows AP product documentation where the Fat AP and cloud AP are used as an example:

In this product documentation, choose Installation > Appendix > AP Login > AP First-Time Login to see how to log in to an AP.

# Switch the AP mode.

<Huawei> system-view

[Huawei] ap-mode-switch cloud

If the AP works in non-cloud mode, you need to switch the AP to the cloud mode first. After such switching, the AP restarts.

In this lab, AirEngine 5760-51 works in cloud mode by default. You do not need to switch the AP working mode.

# On iMaster NCE-Campus, add AP2 to the Branch site.

Choose Design > Device Management. On the page that is displayed, choose Branch > Add Device > Add.

# Add AP2.

Specifically, set Protocol type to NETCONF protocol. Then, add AP2 using the ESN by setting Mode to ESN, clicking Add in Device information, entering the ESN, and setting Name to AP2.

Note: For Huawei devices that adapt to the CloudCampus Solution, set Protocol type to NETCONF protocol. For other devices (or third-party devices), set Protocol type to SNMP protocol.

# Check whether AP2 is online.

On the device management page, you can see that AP2 has gone online.

----End

Configuring Cloud AP Services

Create an ACL invoked by the portal free rule to permit DNS resolution traffic of users.

Configure wireless services for guests. Specifically, set the SSID to iMaster_NCE_Demo, forwarding mode to Layer 2 forwarding, and user VLAN to VLAN 100. Also, enable Portal authentication, configure user-based traffic limiting, and create a to-be-authenticated user named huawei and set the password to Huawei@123.

Configure wireless services for employees. Specifically, set the SSID to Employee, forwarding mode to Layer 2 forwarding, and user VLAN to VLAN 200. Also, enable 802.1X authentication, configure user-based traffic limiting, and create a to-be-authenticated user named dot1x and set the password to Huawei@123.

          - Create a user ACL invoked by the portal free rule.

Create a user-defined ACL numbered 6001 to permit DNS resolution traffic of users. This ensures that users can use the DNS protocol to resolve the IP address of the web server and initiate HTTP access before authentication, so that the AP can perform TCP hijacking and push the Portal authentication page.

# Create a user-defined ACL.

Choose Design > Basic Network Design > Template Management. On the Policy Template tab page, click ACL, and create an ACL.

- Set Name to For_DNS_Resolve.

- Set ACL Number to 6001.

- In the Rule List area, click Add, set Protocol to UDP, set Port to 53, and click √ to submit the configuration.

- Ultimately, click OK.

Create an SSID for guests.

Create an SSID named iMaster_NCE_Demo, enable Portal authentication, set the forwarding mode to Layer 2 forwarding, and set the user VLAN ID to 100.

# Create an SSID.

Choose Provision > Site Configuration, select the Branch site, click AP, and click Create to create an SSID.

# Configure basic SSID parameters.

Configure as follows:

- Set SSID Name to iMaster_NCE_Demo.

- Retain the default setting for Effective radio. That is, all options are selected.

- Retain the default setting for Network connection mode. That is, Layer 2 forwarding is selected.

- Set VLAN ID to 100.

- Click Submit and then Next.

The VLAN ID configured here is the ID of the VLAN to which wireless users belong, that is, the VLAN specified by the service-vlan vlan-id command in the VAP profile on the AC.

In this lab, the VLAN ID is set to 100. When wireless users connect to AP2, AP2 adds a VLAN 100 tag to the data frames of wireless users and sends the tagged data frames through the uplink interface. Therefore, you need to configure the LAN interface on AR2 (that is, GE0/0/1 on AR6120 in this lab) to allow packets from VLAN 100 to pass through. In addition, a DHCP address pool is configured on AR2 to assign an IP address to users in VLAN 100.

# Configure SSID security authentication.

Configure as follows:

- Set Authentication mode to Open network. That is, use the Portal authentication mode.

- Enable Push pages (Portal authentication). Then, new configuration items are displayed.

- Retain the default setting for Page pushing mode. That is, Built-in authentication by cloud platform is selected.

- Set Push Page to Default User Name and Password Authentication Page.

- Disable Self-registration. This means that users are not allowed to register new accounts for authentication. Only existing accounts can be used for authentication.

- Enable Portal authentication-free. That is, retain the default authentication-free validity period.

- In Default permit rule, select the created user ACL, that is, For_DNS_Resolve.

- Select Bypass policy and retain the default value User access is allowed, without authentication.

After the preceding configuration is complete, click Next.

# Configure SSID policy control.

Configure as follows:

- Enable Static terminal rate limiting, and set both the uplink and downlink traffic to 5 Mbit/s.

- Click OK and wait until the configuration is delivered to AP2.

# Verify the configuration.

The web page shows that the created SSID has been enabled on the radios of AP2.

# Verify using a wireless terminal.

On a wireless terminal, you can find the wireless signal with the SSID iMaster_NCE_Demo sent by AP2.

Create an SSID for employees.

Create an SSID named Employee, enable 802.1X authentication, set the forwarding mode to Layer 2 forwarding, and set the user VLAN ID to 200.

# Create an SSID.

Choose Provision > Site Configuration, select the Branch site, click AP, and click Create to create an SSID.

# Configure basic SSID parameters.

Configure as follows:

- Set SSID Name to Employee.

- Retain the default setting for Effective radio. That is, all options are selected.

- Retain the default setting for Network connection mode. That is, Layer 2 forwarding is selected.

- Set VLAN ID to 200.

- Click Submit and then Next.

# Configure SSID security authentication.

Configure as follows:

            - Set Authentication mode to Secure network.

- Set Encryption mode to WPA2.

- Set Encryption algorithm to AES.

- Set RADIUS server to RADIUS.

Then, click Next.

# Configure SSID policy control.

Configure as follows:

- Enable Static terminal rate limiting, and set both the uplink and downlink traffic to 5 Mbit/s.

- Click OK and wait until the configuration is delivered to AP2.

# Verify the configuration.

An SSID named Employee has been generated.

# Verify using a wireless terminal.

On a wireless terminal, you can find the wireless signal with the SSID Employee sent by AP2.

Configure AR2.

To ensure that wireless terminals can obtain IP addresses and access external networks, perform the following configurations on AR2:

- Configure the LAN interface on AR2 to allow packets from VLANs 100 and 200 to pass through.

- Create a DHCP address pool on AR2 and assign IP addresses to wireless terminal users from the address pools on VLANIF 100 and VLANIF 200.

- Add the network segments of wireless terminal users to the ACL used for source NAT.

# Configure the LAN interface on AR2 to allow packets from VLANs 100 and 200 to pass through.

Choose Provision > Site Configuration, select the Branch site, choose AR > Interface > Customized, select LAN interface 1, and add VLAN 100 and VLAN 200 to Allowed VLAN.

# Create a DHCP address pool named For_Wireless_User to assign IP addresses to wireless guests. Also, create a DHCP address pool named For_Employee_User to assign IP addresses to wireless employees.

Choose AR > Network, click LAN, and create a DHCP address pool. Set the parameters according to the preceding figure. The VLAN IDs must be 100 and 200, respectively. The IP addresses and masks can be customized. Ultimately, click OK.

# Modify the ACL used for source NAT on AR2.

Specifically, choose Design > Template Management > Policy Template, click the created ACL For_AR2_Source_NAT, click the modification icon, and add an ACL rule. To add the ACL rule, you only need to set the source IP address range to the address segment assigned to users. In this lab, the source IP address range is 172.16.100.0/24 and 172.16.200.0/24. After that, click OK.

# Verify the configuration on AR2.

[AR2]display current-configuration interface

[V300R019C10SPC300]

#

interface Vlanif10

ip address 172.16.10.254 255.255.255.0

zone trust

dhcp select interface

dhcp server dns-list 172.16.10.254

dhcp server option 148 ascii agilemode=agile-cloud;agilemanage-mode=domain;agilemanage-domain=192.168.4.104;agilemanage-port=10020;

#

interface Vlanif100

ip address 172.16.100.254 255.255.255.0

zone trust

dhcp select interface

dhcp server dns-list 172.16.100.254

#

interface Vlanif200

ip address 172.16.200.254 255.255.255.0

zone trust

dhcp select interface

dhcp server dns-list 172.16.200.254

#

interface GigabitEthernet0/0/0

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk pvid vlan 10

undo port trunk allow-pass vlan 1

port trunk allow-pass vlan 10 100 200

The command output shows that VLANIF 100 and VLANIF 200 are created, the interface-based DHCP address pools are enabled, and LAN interface GE0/0/1 allows packets from VLAN 100 and VLAN 200 to pass through.

<AR2>display acl 3101

Advanced ACL 3101, 2 rules

Acl's step is 5

rule 5 permit ip source 172.16.10.0 0.0.0.255

rule 10 permit ip source 172.16.100.0 0.0.0.255

The command output shows that the IP address segments of wireless users are added to ACL 3101 used for source NAT.

Configure authentication.

In this lab, local authentication is used. That is, iMaster NCE-Campus functions as the Portal server and RADIUS server. You need to create to-be-authenticated users and specify authentication and authorization results on iMaster NCE-Campus.

# Create wireless user groups for authentication.

Choose Admission > User Management, click the icon marked 1 in the left pane, and create two user groups named Portal_User and 802.1X_User.

# Create a wireless user for Portal authentication.

Click the newly created user group Portal_User on the left, and then click Create on the right.

Create a user named huawei and set the password to Huawei@123. Then, disable Change password upon next login; otherwise, the user needs to manually change the password after the first login.

In this lab, the user uses a public account, which is only used to demonstrate the Portal authentication effect. Therefore, you do not need to enable Change password upon next login. In practice, if each user has an independent account, you are advised to enable Change password upon next login.

# Create a wireless user for 802.1X authentication.

Click the newly created user group 802.1x_User on the left, and then click Create on the right.

Create a user named dot1x and set the password to Huawei@123. Then, disable Change password upon next login; otherwise, the user needs to manually change the password after the first login.

In this lab, the user uses a public account, which is only used to demonstrate the Portal authentication effect. Therefore, you do not need to enable Change password upon next login. In practice, if each user has an independent account, you are advised to enable Change password upon next login.

# Modify an authorization rule.

Choose Admission > Authentication and Authorization, click the Authorization Rules tab, modify the authorization rule Default, and change the authorization result of this authorization rule to Permit Access.

Verify Portal authentication.

Connect a wireless terminal to the SSID (iMaster_NCE_Demo) of AP2. Open the browser and enter the IP address 1.1.1.1. On the Portal authentication page that is displayed, enter the user name and password to verify the success of Portal authentication.

# Connect a wireless terminal to the SSID (iMaster_NCE_Demo).

# Verify that the Portal authentication page is displayed normally.

Enter any IP address in the address box of the browser and verify that the Portal authentication page can be displayed normally.

If a page similar to the following is displayed, the Portal authentication page is displayed normally.

The URL in the browser is as follows (in this lab):

https://192.168.4.104:19008/portalpage/00000000-0000-0000-0000-000000000000/username01/pc/auth.html?apmac=f4deaf36b580&uaddress=172.16.100.8&umac=b0d59dc73c41&authType=1&lang=zh_CN&ssid=aU1hc3Rlcl9OQ0VfRGVtbw==&pushPageId=0470709e-b6d4-4ff8-bcd6-63c7dcef9644

This URL shows that iMaster NCE-Campus provides services for the Portal authentication page. This URL also carries the apmac and uaddress parameters, which indicate the MAC address of the AP that the user accesses and the IP address of the user. These two parameters are provided by the AP, and are carried in the URL and notified to the Portal server for subsequent user identification and management.

In this lab, the DNS server address for wireless terminals is set to the IP address of the VLANIF interface on AR2. Although the DNS proxy function is enabled on AR2, no external DNS server is configured on AR2 and no DNS recording is performed on AR2. Therefore, AR2 cannot resolve or respond to the DNS request from a wireless terminal. To simulate the scenario where the Portal authentication page is displayed when a user opens the browser and attempts to access the network through the URL, you need to configure a static DNS resolution record on AR2.

# Configure static DNS resolution on AR2.

[AR2]ip host www.portal.com 67.0.0.1

Configure a static DNS resolution record to resolve the domain name www.portal.com to the IP address 67.0.0.1. This domain name is used only for testing Portal authentication through the domain name in the browser.

# Test DNS resolution on a wireless terminal.

[C:\~]$ nslookup www.portal.com

DNS request timed out.

timeout was 2 seconds.

Server:  UnKnown

Address:  172.16.100.254

DNS request timed out.

timeout was 2 seconds.

Name:    www.portal.com

Address:  67.0.0.1

In the cmd window of the wireless terminal, run the nslookup command to resolve the DNS address. www.portal.com can be resolved successfully.

# Verify the network connectivity of the wireless terminal before authentication.

[C:\~]$ ping 172.16.100.254

Pinging 172.16.100.254 with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 172.16.100.254:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss),

[C:\~]$ ping 67.0.0.1

Pinging 67.0.0.1 with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 67.0.0.1:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss),

In this command output, the wireless terminal cannot communicate with IP address 67.0.0.1 or its own gateway. That is, the wireless terminal does not have the right to access the network before the authentication succeeds.

# Verify that the Portal authentication page can be displayed through the domain name mode.

Enter www.portal.com in the address box of the browser and press Enter.

On the page that is displayed, enter the user name (huawei) and password (Huawei@123).

Portal authentication succeeds.

# Verify the network connectivity of the wireless terminal again.

[C:\~]$ ping 172.16.100.254

Pinging 172.16.100.254 with 32 bytes of data:

Reply from 172.16.100.254: bytes=32 time=3ms TTL=255

Reply from 172.16.100.254: bytes=32 time=3ms TTL=255

Reply from 172.16.100.254: bytes=32 time=3ms TTL=255

Reply from 172.16.100.254: bytes=32 time=3ms TTL=255

Ping statistics for 172.16.100.254:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 3ms, Maximum = 3ms, Average = 3ms

[C:\~]$ ping 67.0.0.1

Pinging 67.0.0.1 with 32 bytes of data:

Reply from 67.0.0.1: bytes=32 time=3ms TTL=254

Reply from 67.0.0.1: bytes=32 time=3ms TTL=254

Reply from 67.0.0.1: bytes=32 time=3ms TTL=254

Reply from 67.0.0.1: bytes=32 time=3ms TTL=254

Ping statistics for 67.0.0.1:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 3ms, Maximum = 3ms, Average = 3ms

After Portal authentication succeeds, the wireless terminal can access the network, including communicating with its own gateway and the IP address 67.0.0.1 (which is used to simulate an external network).

Verify 802.1X authentication.

Use a wireless terminal to connect to the SSID (Employee) of AP2. Enter the user name dot1x and password Huawei@123 to connect to the SSID. Verify that 802.1X authentication of the wireless terminal succeeds.

# Connect a wireless terminal to the SSID (Employee).

# Enter the user name and password.

Enter the user name dot1x and password Huawei@123 for 802.1X authentication.

# Check the IP address of the wireless terminal after successful connection.

[C:\~]$ ipconfig

Windows IP Configuration

Wireless LAN adapter Wi-Fi:

Connection-specific DNS Suffix  . :

Link-local IPv6 Address . . . . . : fe80::fd05:6635:11e6:26%9

IPv4 Address. . . . . . . . . . . : 172.16.200.179

Subnet Mask . . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . . : 172.16.200.254

The command output shows that the wireless terminal has obtained the IP address 172.16.200.179. This IP address is assigned from the address pool on VLANIF 200.

# Test the connectivity of the wireless terminal with the external network.

[C:\~]$ ping 67.0.0.1

Pinging 67.0.0.1 with 32 bytes of data:

Reply from 67.0.0.1: bytes=32 time=3ms TTL=254

Reply from 67.0.0.1: bytes=32 time=3ms TTL=254

Reply from 67.0.0.1: bytes=32 time=3ms TTL=254

Reply from 67.0.0.1: bytes=32 time=3ms TTL=254

Ping statistics for 67.0.0.1:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 3ms, Maximum = 3ms, Average = 3ms

The wireless terminal can successfully access the external network.

----End

Quiz

To use the iMaster NCE controller to manage APs, which of the following working modes should APs be?

Reference Answers

Answers to Network Admission Control Technology:

- MAC address authentication is recommended for dumb terminals, such as cameras and printers that cannot perform active authentication.

Answers to the VXLAN and EVPN Technology:

- Packets are flooded in the local DB domain and copied to the remote VTEP in the ingress replication list.

- The configuration and implementation logic are simple, and EVPN instance and IP VPN instance parameters do not need to be planned or configured.

- Compared with the centralized gateway, each distributed gateway maintains only some forwarding entries, and does not need to maintain all terminal ARP entries.

Answers to the Campus Multi-Branch Interconnection Technology:

- ESP is preferred. AH does not support data encryption.

Answers to the CloudCampus Virtualized Campus Network Deployment:

- An IP VPN instance is created on the CLI. One VN corresponds to one IP VPN instance.

Answers to CloudCampus Small- and Medium-Sized Campus Network Deployment:

- Cloud mode.