|  | HCIP-Datacom-Campus Network Planning and Deployment Lab Setup Guide |  |  |  |
| --- | --- | --- | --- | --- |
|  | Applicable to version 1.0 |  |  |  |
|  | Huawei Technologies Co., Ltd. |  |  |  |

Copyright Huawei Technologies Co., Ltd. 2020. All rights reserved. All contents of this book are protected by copyright laws. Huawei owns all copyrights, except the contents quoted from other parties. Without prior written permission from Huawei Technologies Co., Ltd., no person or organization shall copy, distribute, reproduce, store, or use any content in the information retrieval system in any manner, or use it for any other commercial purpose. All rights reserved. Trademark Declaration and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks or registered trademarks mentioned in this document are the property of their respective owners.

| Enterprise Business Training Certification Dept, Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Headquarters, Bantian, Longgang District, Shenzhen Postcode: 518129 |
| Website: | http://learning.huawei.com |
| email: | learning@huawei.com |

Update History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 2020-12-12 | New version |

Document Information

| Prepared by | Applicable Time Range | Applicable to |
| --- | --- | --- |
| Talent Ecosystem Development Dept | 2020-12-12~... | All those who want to learn Huawei Datacom certification |

目录

1.	About this Document	4

2.	Physical topology	4

3.	Equipment procurement	4

4.	Cable connection	5

5.	Device Configuration	6

6.	Environment Pre-configuration	6

7.	Other Precautions	7

8.	Attachment	8

About this Document

The HCIP-Datacom-Campus Network Planning and Deployment V1.0 Lab Setup Guide defines the lab equipment and generates the list of lab equipment and physical connection diagrams.

This document provides guidance for regional offices, suppliers, HALPs, and network academies to purchase devices, install modules, connect cables, configure control devices, and construct a topology view. Then, perform the corresponding experiments based on the HCIP-Datacom-Campus Network Planning and Deployment V1.0 lab manual.

Physical topology

Physical Topology

: This topology is a physical topology that contains all devices (excluding hosts) in the device purchase list.

Equipment Purchasing

For details about the equipment purchase list, see the attachment.

File name: HCIP-Datacom-Campus Network Planning and Deployment V1.0 Equipment List.xlsx

Pay attention to the purchase of equipment according to the experiment requirements. If you have any questions, contact email: learning@huawei.com.

Cable connection

The equipment must be connected based on the topology shown in Figure1.

Ensure that the correct models are used for the different types of equipment based on the mapping provided in Table 1.

Mapping between device names and models

| Device Name | Device Model |
| --- | --- |
| AR1 | AR6121 |
| AR2 | AR6121 |
| SW1 | S5731-H24T4XC |
| CORE1 | S5731-H24T4XC |
| AGG1 | S5731-H24T4XC |
| AGG2 | S5731-H24T4XC |
| ACC1 | S5731-H24P4XC |
| ACC2 | S5731-H24P4XC |
| AP1 | AirEngine5760-51 |
| AP2 | AirEngine5760-51 |

Check whether the ports are connected correctly based on Table 2.

Interface connection table

| AR1 | GE0/0/0 | SW1 |
| --- | --- | --- |
|  | GE0/0/1 | CORE1 |
| AR2 | GE0/0/8 | SW1 |
|  | GE0/0/1 | AP2 |
| SW1 | GE0/0/17 | AR1 |
|  | GE0/0/18 | AR2 |
|  | - (subject to the actual situation) | iMaster NCE-Campus server |
| CORE1 | GE0/0/1 | AR1 |
|  | GE0/0/2 | AGG1 |
|  | GE0/0/3 | AGG2 |
| AGG1 | GE0/0/1 | ACC1 |
|  | GE0/0/2 | CORE1 |
| AGG2 | GE0/0/1 | CORE1 |
|  | GE0/0/2 | ACC2 |
| ACC1 | GE0/0/1 | AGG1 |
|  | GE0/0/24 | Host1 |
| ACC2 | GE0/0/1 | AGG2 |
|  | GE0/0/23 | AP1 |
|  | GE0/0/24 | Host2 |
| AP1 | GE0/0/0 | ACC2 |
| AP2 | GE0/0/0 | AR2 |

Note: AP2 is powered by AC/DC power cables.

Device Configuration

GE0/0/0 to GE0/0/7 on the AR6121 are Layer 2 interfaces by default. You can run the undo portswitch command to switch the interfaces to Layer 3 interfaces.

Environment Pre-configuration

In this experiment, SW1 functions as the gateway of the iMaster NCE-Campus server and the gateway of AR1 and AR2. SW1 provides the following functions:

- Enables AR1 and AR2 to communicate with the iMaster NCE server.

- The iMaster NCE-Campus controller IP address can be delivered to devices through DHCP Option 148.

To solve this problem, perform the following configuration on SW1:

- Create VLAN 67, assign the interfaces connected to AR1 and AR2 to VLAN 67, and create VLANIF 67.

<Huawei>system-view

[Huawei]sysname SW1

[SW1]vlan 67

[SW1]interface GigabitEthernet 0/0/17

[SW1-GigabitEthernet0/0/17]port link-type access

[SW1-GigabitEthernet0/0/17]port default vlan 67

[SW1-GigabitEthernet0/0/17]description to_AR1

[SW1-GigabitEthernet0/0/17]quit

[SW1]interface GigabitEthernet 0/0/18

[SW1-GigabitEthernet0/0/18]port link-type access

[SW1-GigabitEthernet0/0/18]port default vlan 67

[SW1-GigabitEthernet0/0/18]description to_AR2

[SW1-GigabitEthernet0/0/18]quit

[SW1]interface Vlanif 67

[SW1-Vlanif67]ip address 67.0.0.254 24

[SW1-Vlanif67]description AR_Gateway

- Create VLAN 40, assign the interface connected to the iMaster NCE server to VLAN 40, and create VLANIF 40.

[SW1]vlan 40

[SW1]interface GigabitEthernet 0/0/20

[SW1-GigabitEthernet0/0/20]port link-type access

[SW1-GigabitEthernet0/0/20]port default vlan 40

[SW1-GigabitEthernet0/0/20]description to_iMaster NCE

[SW1-GigabitEthernet0/0/20]quit

[SW1]interface Vlanif 40

[SW1-Vlanif40]ip address 192.168.4.254 24

[SW1-Vlanif40]description iMaster_NCE_Gateway

Note that the number of servers used varies depending on the iMaster NCE-Campus deployment mode. Ensure that the server is connected to SW1 and the gateway is set to the IP address of VLANIF40 on SW1.

- Enable the DHCP service, enable interface-based address allocation, assign fixed IP addresses to AR1 and AR2 interfaces, and configure DHCP Option 148.

[SW1]dhcp enable

[SW1]interface Vlanif 67

[SW1-Vlanif67] dhcp select interface

[SW1-Vlanif67] dhcp server static-bind ip-address 67.0.0.2 mac-address c8a7-7600-3f63

[SW1-Vlanif67] dhcp server static-bind ip-address 67.0.0.1 mac-address cc64-a651-7f13

[SW1-Vlanif67] dhcp server option 148 ascii agilemode=agile-cloud;agilemanage-

mode=ip;agilemanage-domain=192.168.4.104;agilemanage-port=10020;

To ensure that the IP addresses of AR1 and AR2 interfaces are fixed, statically bound addresses are used. The actual MAC addresses of AR interfaces vary.

Configure DHCP Option 148 to notify iMaster NCE of the IP address and port number. The actual IP address and port number may vary according to the actual situation.

Other Precautions

It is recommended that the trainer explain the experiment materials and the students complete the experiment materials separately. If troubleshooting is required, the trainer can participate in the experiment materials.

To prevent residual configurations from affecting the experiment, you are advised to clear the configuration information saved on the device before shutting down the device after the experiment is complete. At the beginning of the experiment, ensure that the device starts from the empty configuration. Otherwise, clear the configuration and restart the device.

In this experiment, the physical server of iMaster NCE-Campus is one device, connected to SW1 through GE0/0/20, and the gateway is set on SW1. In actual environments, iMaster NCE can be installed in various modes, and the number of physical servers is not fixed, the connection mode between the physical server and SW1 depends on the purchased environment.

Attachment