Huawei Certification Training

HCIP-Datacom-Core Technology

Data Communication Senior Engineer

Lab Guide

Issue: 1.0

Huawei Technologies Co., Ltd.

Copyright © Huawei Technologies Co., Ltd. 2020. All rights reserved.

No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd.

Trademarks and Permissions

and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd.

All other trademarks and trade names mentioned in this document are the property of their respective holders.

Notice

The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied.

The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://e.huawei.com/ |

Huawei Certification System

Huawei Certification follows the "platform + ecosystem" development strategy, which is a new collaborative architecture of ICT infrastructure based on "Cloud-Pipe-Terminal". Huawei has set up a complete certification system consisting of three categories: ICT infrastructure certification, platform and service certification, and ICT vertical certification. It is the only certification system that covers all ICT technical fields in the industry. Huawei offers three levels of certification: Huawei Certified ICT Associate (HCIA), Huawei Certified ICT Professional (HCIP), and Huawei Certified ICT Expert (HCIE). Huawei Certification covers all ICT fields and adapts to the industry trend of ICT convergence. With its leading talent development system and certification standards, it is committed to fostering new ICT talent in the digital era, and building a sound ICT talent ecosystem.

Huawei Certified ICT Professional-Datacom-Core Technology (HCIP-Datacom-Core Technology) is designed for Huawei's frontline engineers and anyone who want to understand Huawei's datacom products and technologies. HCIP-Datacom-Core Technology certification covers advanced routing, advanced Ethernet switching, large-scale WLAN networking, multicast technology, IPv6 technology, network security, network reliability, network service and management, and enterprise network solutions.

The Huawei certification system introduces the industry, fosters innovation, and imparts cutting-edge datacom knowledge.

About This Document

Overview

This document is an HCIP-Datacom-Core Technology certification training course. It is intended for trainees who are going to take the HCIP-Datacom-Core Technology exams or readers who want to learn advanced technologies, such as routing, Ethernet switching, large-scale WLAN networking, multicast, IPv6, network security, network reliability, and network service and management.

Background Knowledge Required

This course is for Huawei's advanced certification. To better understand this course, familiarize yourself with the following requirements:

Have basic computer skills.

Have participated in HCIA-Datacom training.

Have passed the HCIA-Datacom exams.

Be familiar with the principles of the TCP/IP protocol stack.

Be familiar with the basic working principles of Ethernet switches and routers.

Symbol Conventions

Lab Environment

Networking

This lab environment is intended for datacom network engineers who are preparing for the HCIP-Datacom-Core Technology exam. Each lab environment includes three switches (not supporting PoE), two PoE switches, two APs, five routers, and one firewall.

Device Introduction

To meet exercise requirements, the recommended configurations of the environment are as follows:

The following table describes the mapping among devices, models, and versions.

| Device Name | Device Model | Software version |
| --- | --- | --- |
| Switch | CloudEngine S5731-H24T4XC | V200R019C00 or later |
| PoE switch | CloudEngine S5731-H24P4XC | V200R019C00 or later |
| AP | AirEngine 5760-10 | V200R019C00 or later |
| Router | NetEngine AR6120 | V300R019 or later |
| Firewall | USG6307E | V500R001C50 or later |

The port, output, and configuration information of devices in this document is provided based on the recommended topology. The actual information may vary according to the lab environment.

Contents

Overview	3

Background Knowledge Required	3

Symbol Conventions	3

Lab Environment	4

1 Basic OSPF Configurations	8

1.1 Lab 1: Single-Area OSPF	8

1.1.1 Introduction	8

1.1.2 Lab Configuration	9

1.1.3 Quiz	22

1.1.4 Configuration Reference	22

1.2 Lab 2: Multi-Area OSPF	23

1.2.1 Introduction	23

1.2.2 Lab Configuration	25

1.2.3 Quiz	37

1.2.4 Configuration Reference	37

1.3 Lab 3: OSPF Adjacencies and LSAs	40

1.3.1 Introduction	40

1.3.2 Lab Configuration	41

1.3.3 Quiz	57

1.3.4 Configuration Reference	57

1.4 Lab 4: OSPF Stub Area and NSSA	59

1.4.1 Introduction	59

1.4.2 Lab Configuration	60

1.4.3 Quiz	73

1.4.4 Configuration Reference	73

2 IS-IS Basics Experiment	76

2.1 IS-IS Configuration Experiment	76

2.1.1 Introduction	76

2.1.2 Lab Configuration	77

2.1.3 Quiz	87

2.1.4 Configuration Reference	87

3 BGP Configurations	90

3.1 Lab 1: Basic BGP Configurations	90

3.1.1 Introduction	90

3.1.2 Lab Configuration	91

3.1.3 Quiz	99

3.1.4 Configuration Reference	99

3.2 Lab 2: BGP Route Summarization	103

3.2.1 Introduction	103

3.2.2 Lab Configuration	103

3.2.3 Quiz	110

3.2.4 Configuration Reference	111

3.3 Lab 3: BGP RR	112

3.3.1 Introduction	112

3.3.2 Lab Configuration	113

3.3.3 Quiz	126

3.3.4 Configuration Reference	126

3.4 Lab 4: BGP Route Selection	129

3.4.1 Introduction	129

3.4.2 Lab Configuration	130

3.4.3 Quiz	149

3.4.4 Configuration Reference	149

4 Routing Policy and Routing Control	155

4.1 Route Import and Control	155

4.1.1 Introduction	155

4.1.2 Lab Configuration	156

4.1.3 Quiz	165

4.1.4 Configuration Reference	165

5 RSTP and MSTP	168

5.1 Basic RSTP and MSTP Configurations	168

5.1.1 Introduction	168

5.1.2 Lab Configuration	169

5.1.3 Quiz	176

5.1.4 Configuration Reference	176

6 Multicast	180

6.1 IGMP, IGMP Snooping, and PIM-DM	180

6.1.1 Introduction	180

6.1.2 Lab Configuration	181

6.1.3 Quiz	192

6.1.4 Configuration Reference	192

6.2 PIM-SM, BSR, and PIM-SSM	194

6.2.1 Introduction	194

6.2.2 Lab Configuration	195

6.2.3 Quiz	205

6.2.4 Configuration Reference	205

7 Firewall Technology	209

7.1 Firewall Security Policy	209

7.1.1 Introduction	209

7.1.2 Lab Configuration	210

7.1.3 Quiz	216

7.1.4 Configuration Reference	216

8 VRRP	220

8.1 Basic VRRP Configurations	220

8.1.1 Introduction	220

8.1.2 Lab Configuration	221

8.1.3 Quiz	227

8.1.4 Configuration Reference	227

9 DHCP	231

9.1 DHCP Relay Configuration	231

9.1.1 Introduction	231

9.1.2 Lab Configuration	232

9.1.3 Quiz	239

9.1.4 Configuration Reference	239

10 WLAN	242

10.1 Inter-AC Roaming on a Large-Scale WLAN	242

10.1.1 Introduction	242

10.1.2 Lab Configuration	244

10.1.3 Quiz	252

10.1.4 Configuration Reference	252

10.2 VRRP HSB Configuration	256

10.2.1 Introduction	256

10.2.2 Lab Configuration	258

10.2.3 Quiz	270

10.2.4 Configuration Reference	270

10.3 Dual-Link Cold Backup Configuration	274

10.3.1 Introduction	274

10.3.2 Lab Configuration	276

10.3.3 Quiz	284

10.3.4 Configuration Reference	284

Reference Answers	288

- Basic OSPF Configurations

  - Lab 1: Single-Area OSPF

    - Introduction

      - Objectives

Upon completion of this task, you will be able to:

Configure OSPF in a single area.

Configure OSPF area authentication.

Describe the process of establishing OSPF neighbor relationships on a multi-access network.

Change the cost of an OSPF interface.

Illustrate how to configure silent interfaces in OSPF.

Run the display commands to check various OSPF states.

      - Networking Topology

              - Single-area OSPF

R1, R2, and R3 are connected through S1, and their interfaces and IP addresses are shown in the figure. Loopback0 is created on each of R1, R2, and R3, and its IP address is in the format of 10.0.x.x/24, where x indicates the device number.

All interfaces of R1, R2, and R3 belong to area 0, and OSPF is activated on the interconnection and Loopback0 interfaces.

      - Background

You are a network administrator of a company. Currently, there are three AR routers on the company's network, which communicate with each other through an Ethernet. On broadcast multi-access networks such as Ethernet, there may be security risks. To prevent malicious routing attacks, you choose to use OSPF area authentication.

    - Lab Configuration

      - Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF on R1, R2, and R3, manually specify router IDs, and activate OSPF on the interconnection and Loopback0 interfaces.

- After the configuration is complete, check the OSPF neighbor relationship status and OSPF routing table on each of R1, R2, and R3, and check the connectivity between the loopback interfaces of R1, R2, and R3.

- Manually shut down the interconnection interfaces of R1, R2, and R3 and enable the debugging function to check the establishment of OSPF neighbor relationships. Then, enable the interconnection interfaces again and observe the debugging information on the devices.

- Manually change the network type of Loopback0 on R2 and observe the change in the mask length of the OSPF route.

- Manually change the costs of OSPF interfaces.

- Configure the interconnection and Loopback0 interfaces as silent interfaces.

      - Configuration Procedure

Configure IP addresses for the interconnection and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/3 and the loopback interface on R1.

<R1>system-view

Enter system view, return user view with Ctrl+Z.

[R1]interface GigabitEthernet 0/0/3

[R1-GigabitEthernet0/0/3] ip address 10.0.123.1 24

[R1-GigabitEthernet0/0/3] quit

[R1]interface LoopBack 0

[R1-LoopBack0] ip address 10.0.1.1 24

[R1-LoopBack0] quit

# Configure IP addresses for GE0/0/4 and the loopback interface on R2.

<R2>system-view

Enter system view, return user view with Ctrl+Z.

[R2]interface GigabitEthernet 0/0/4

[R2-GigabitEthernet0/0/4] ip address 10.0.123.2 24

[R2-GigabitEthernet0/0/4] quit

[R2]interface LoopBack 0

[R2-LoopBack0] ip address 10.0.2.2 24

[R2-LoopBack0] quit

# Configure IP addresses for GE0/0/4 and the loopback interface on R3.

<R3>system-view

Enter system view, return user view with Ctrl+Z.

[R3]interface GigabitEthernet 0/0/4

[R3-GigabitEthernet0/0/4] ip address 10.0.123.3 24

[R3-GigabitEthernet0/0/4] quit

[R3]interface LoopBack 0

[R3-LoopBack0] ip address 10.0.3.3 24

[R3-LoopBack0] quit

# Verify the connectivity on R1.

<R1>ping -c 1 10.0.123.2

PING 10.0.123.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.2: bytes=56 Sequence=1 ttl=255 time=2 ms

--- 10.0.123.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 2/2/2 ms

<R1>ping -c 1 10.0.123.3

PING 10.0.123.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.3: bytes=56 Sequence=1 ttl=255 time=2 ms

--- 10.0.123.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 2/2/2 ms

Configure OSPF in a single area.

# Configure the Loopback0 IP addresses of R1, R2, and R3 as their OSPF router IDs, and set the OSPF process ID to 1.

[R1]ospf 1 router-id 10.0.1.1

[R2]ospf 1 router-id 10.0.2.2

[R3]ospf 1 router-id 10.0.3.3

# Activate OSPF on the interconnection and Loopback0 interfaces of R1, R2, and R3.

[R1]ospf 1

[R1-ospf-1]area 0

[R1-ospf-1-area-0.0.0.0] network 10.0.123.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] network 10.0.1.1 0.0.0.0

[R2]ospf 1

[R2-ospf-1]area 0

[R2-ospf-1-area-0.0.0.0] network 10.0.123.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R3]ospf 1

[R3-ospf-1]area 0

[R3-ospf-1-area-0.0.0.0] network 10.0.123.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

# To ensure security, configure OSPF area authentication, use the cleartext mode, and set the password to huawei.

[R1]ospf 1

[R1-ospf-1]area 0

[R1-ospf-1-area-0.0.0.0] authentication-mode simple plain huawei

[R2]ospf 1

[R2-ospf-1]area 0

[R2-ospf-1-area-0.0.0.0] authentication-mode simple plain huawei

[R3]ospf 1

[R3-ospf-1]area 0

[R3-ospf-1-area-0.0.0.0] authentication-mode simple plain huawei

Verify the OSPF configuration.

# Check the OSPF neighbor information on R1, R2, and R3.

<R1>display ospf peer

OSPF Process 1 with Router ID 10.0.1.1

Neighbors

Area 0.0.0.0 interface 10.0.123.1(GigabitEthernet0/0/3)'s neighbors

Router ID	: 10.0.2.2         Address: 10.0.123.2

State: Full  Mode	:Nbr is  Master  Priority: 1

DR: 10.0.123.1  BDR: 10.0.123.2  MTU: 0

Dead timer due in 39  sec

Retrans timer interval: 5

Neighbor is up for 00:24:56

Authentication Sequence: [ 0 ]

Router ID: 10.0.3.3          Address: 10.0.123.3

State: Full  Mode:Nbr is  Slave  Priority: 1

DR: 10.0.123.1  BDR: 10.0.123.2  MTU: 0

Dead timer due in 38  sec

Retrans timer interval: 5

Neighbor is up for 00:24:32

Authentication Sequence: [ 0 ]

The command output shows that R1 has established OSPF neighbor relationships with R2 and R3.

<R2>display ospf peer

OSPF Process 1 with Router ID 10.0.2.2

Neighbors

Area 0.0.0.0 interface 10.0.123.2(GigabitEthernet0/0/4)'s neighbors

Router ID: 10.0.1.1         Address: 10.0.123.1

State: Full  Mode:Nbr is  Slave  Priority: 1

DR: 10.0.123.1  BDR: 10.0.123.2  MTU: 0

Dead timer due in 34  sec

Retrans timer interval: 0

Neighbor is up for 00:27:10

Authentication Sequence: [ 0 ]

Router ID: 10.0.3.3          Address: 10.0.123.3

State: Full  Mode:Nbr is  Slave  Priority: 1

DR: 10.0.123.1  BDR: 10.0.123.2  MTU: 0

Dead timer due in 36  sec

Retrans timer interval: 5

Neighbor is up for 00:26:50

Authentication Sequence: [ 0 ]

The command output shows that R2 has established OSPF neighbor relationships with R1 and R3.

<R3>display ospf peer

OSPF Process 1 with Router ID 10.0.3.3

Neighbors

Area 0.0.0.0 interface 10.0.123.3(GigabitEthernet0/0/4)'s neighbors

Router ID: 10.0.1.1         Address: 10.0.123.1

State: Full  Mode:Nbr is  Master  Priority: 1

DR: 10.0.123.1  BDR: 10.0.123.2  MTU: 0

Dead timer due in 31  sec

Retrans timer interval: 0

Neighbor is up for 00:28:06

Authentication Sequence: [ 0 ]

Router ID: 10.0.2.2         Address: 10.0.123.2

State: Full  Mode:Nbr is  Master  Priority: 1

DR: 10.0.123.1  BDR: 10.0.123.2  MTU: 0

Dead timer due in 34  sec

Retrans timer interval: 5

Neighbor is up for 00:28:09

Authentication Sequence: [ 0 ]

The command output shows that R3 has established OSPF neighbor relationships with R1 and R2.

# Check the OSPF routing tables of R1, R2, and R3.

[R1]display ospf routing

OSPF Process 1 with Router ID 10.0.1.1

Routing Tables

Routing	for	Network

Destination						Cost			Type							NextHop						AdvRouter							Area

10.0.1.1/32								0					Stub							10.0.1.1								10.0.1.1								0.0.0.0

10.0.123.0/24						1					Transit					10.0.123.1						10.0.1.1								0.0.0.0

10.0.2.2/32								1					Stub							10.0.123.2						10.0.2.2								0.0.0.0

10.0.3.3/32								1					Stub							10.0.123.3						10.0.3.3								0.0.0.0

Total Nets: 4

Intra Area: 4  Inter Area: 0  ASE: 0  NSSA: 0

The command output shows that R1 has learned the Loopback0 routes from R2 and R3.

[R2]display ospf routing

OSPF Process 1 with Router ID 10.0.2.2

Routing Tables

Routing	for	Network

Destination						Cost			Type							NextHop							AdvRouter						Area

10.0.2.2/32								0					Stub							10.0.2.2								10.0.2.2								0.0.0.0

10.0.123.0/24						1					Transit					10.0.123.2						10.0.2.2								0.0.0.0

10.0.1.1/32								1					Stub							10.0.123.1						10.0.1.1								0.0.0.0

10.0.3.3/32								1					Stub							10.0.123.3						10.0.3.3								0.0.0.0

Total Nets: 4

Intra Area: 4  Inter Area: 0  ASE: 0  NSSA: 0

The command output shows that R2 has learned the Loopback0 routes from R1 and R3.

[R3]display ospf routing

OSPF Process 1 with Router ID 3.3.3.3

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter							Area

10.0.3.3/32								0					Stub							10.0.3.3								10.0.3.3									0.0.0.0

10.0.123.0/24						1					Transit					10.0.123.3						10.0.3.3									0.0.0.0

10.0.1.1/32								1					Stub							10.0.123.1						10.0.1.1									0.0.0.0

10.0.2.2/32								1					Stub							10.0.123.2						10.0.2.2									0.0.0.0

Total Nets: 4

Intra Area: 4  Inter Area: 0  ASE: 0  NSSA: 0

The command output shows that R3 has learned the Loopback0 routes from R1 and R2.

# Check the connectivity between the loopback interfaces.

<R1>ping -c 1 -a 10.0.1.1 10.0.2.2

PING 10.0.2.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.2.2: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.2.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

<R1>ping -c 1 -a 10.0.1.1 10.0.3.3

PING 10.0.3.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.3.3: bytes=56 Sequence=1 ttl=255 time=60 ms

--- 10.0.3.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 60/60/60 ms

On R1, use the IP address of Loopback0 as the source address to ping the Loopback0 interfaces of R2 and R3.

# Check the OSPF LSDB on R1.

<R1>display	ospf	lsdb

OSPF	Process	1	with	Router	ID	10.0.1.1

Link	State	Database

Area:	0.0.0.0

Type					LinkState	ID				AdvRouter							Age	Len					Sequence					Metric

Router				10.0.3.3							10.0.3.3								468		48						80000005						0

Router				10.0.2.2							10.0.2.2								472		48						8000000B					0

Router				10.0.1.1							10.0.1.1								467		48						8000000D					0

Network			10.0.123.1					10.0.1.1								467		36						80000008						0

The command output shows four LSAs, among which the first three are Type 1 LSAs generated by R1, R2, and R3 separately. The AdvRouter field for each LSA indicates the router that generates the LSA. The fourth LSA is a Type 2 LSA, which is generated by the DR on a network segment. In this scenario, R1 is the DR on the network segment 10.0.123.0/24. Therefore, the AdvRouter field value of the Type 2 LSA is 10.0.1.1.

# Check the Type 1 LSA generated by R1.

[R1]display ospf lsdb router self-originate

OSPF Process 1 with Router ID 10.0.1.1

Area: 0.0.0.0

Link State Database

Type								:	Router

Ls	id								:	10.0.1.1

Adv	rtr						:	10.0.1.1

Ls	age							:	430

Len									:	48

Options					:	E

seq#								:	80000009

chksum					:	0x8188

Link	count		:	2

*	Link	ID			:	10.0.1.1

Data					:	255.255.255.255

Link	Type:	StubNet

Metric			:	0

Priority			:	Medium

*	Link	ID			:	10.0.123.1

Data					: 10.0.123.1

Link	Type:	TransNet

Metric			:	1

The command output shows that this LSA describes two Links. The first Link indicates the network segment where the loopback interface resides. The value of Link Type is StubNet, and the values of Link ID and Data are the IP address and mask of the stub network segment, respectively. The second Link describes the network segment on which the three routers are interconnected. The value of Link Type is TransNet. The value of Link ID is the DR's interface address 10.0.123.1, and the value of Data is the IP address of the local interface on the network segment, 10.0.123.1.

# Check the Type 2 LSA generated by R1.

[R1]display ospf lsdb network self-originate

OSPF Process 1 with Router ID 10.0.1.1

Area: 0.0.0.0

Link State Database

Type					:	Network

Ls	id					:	10.0.123.1

Adv	rtr			:	10.0.1.1

Ls	age				:	1662

Len						:	36

Options		:	E

seq#					:	80000005

chksum		:	0x3d58

Net	mask	:	255.255.255.0

Priority			:	Low

Attached Router    10.0.1.1

Attached Router    10.0.2.2

Attached Router    10.0.3.3

The Attached Router field in the Type 2 LSA describes the neighbor information of the network segment where the DR resides.

Check the process of establishing OSPF neighbor relationships.

Based on the preceding OSPF neighbor information, the interface IP address of the DR is 10.0.123.1, which is different from the expected result obtained based on DR election rules. In OSPF, the non-preemption mode is used for DR election. When a DR or BDR exists on a network, a newly deployed router on the network cannot preempt the role of the DR or BDR. However, if OSPF is configured at different time on the network devices, the elected DR may be the device that starts first.

To prevent this issue, you can shut down the interconnection interfaces of R1, R2, and R3, and run the debugging ospf 1 event command to observe the process of establishing the OSPF neighbor relationships. Then, try to re-enable the interfaces of R1, R2, and R3 at the same time, and check the DR and BDR election process based on the debugging information.

# Shut down the interconnection interfaces of R1, R2, and R3.

[R1] interface GigabitEthernet0/0/3

[R1-GigabitEthernet0/0/3] shutdown

[R2] interface GigabitEthernet0/0/4

[R2-GigabitEthernet0/0/4] shutdown

[R3] interface GigabitEthernet0/0/4

[R3-GigabitEthernet0/0/4] shutdown

# Enable the debugging function on R1, R2, and R3, and enable OSPF event debugging.

<R1>terminal debugging

Info: Current terminal debugging is on.

<R1>terminal monitor

<R1>debugging ospf 1 event

The operations on R2 and R3 are the same as that on R1, and are not provided here.

# Re-enable the interconnection interfaces of R1, R2, and R3.

[R1] interface GigabitEthernet0/0/3

[R1-GigabitEthernet0/0/3] undo shutdown

[R2] interface GigabitEthernet0/0/4

[R2-GigabitEthernet0/0/4] undo shutdown

[R3] interface GigabitEthernet0/0/4

[R3-GigabitEthernet0/0/4] undo shutdown

# Observe the debugging information on R3.

May 22 2020 14:32:25-08:00 R3 %%01PHY/1/PHY(l)[20]:    GigabitEthernet0/0/4: change status to up

May 22 2020 14:32:25-08:00 R3 %%01IFNET/4/LINK_STATE(l)[21]:The line protocol IP on the interface GigabitEthernet0/0/4 has entered the UP state.

May 22 2020 14:32:25.650.5-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 1281 Level: 0x20

OSPF 1: Intf 10.0.123.3 Rcv InterfaceUp State Down -> Waiting.

May 22 2020 14:32:25.650.6-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 1395 Level: 0x20

OSPF 1 Send Hello Interface Up on 10.0.123.3

May 22 2020 14:32:29-08:00 R3 DS/4/DATASYNC_CFGCHANGE:OID 1.3.6.1.4.1.2011.5.25.191.3.1 configurations have been changed. The current change number is 20, the change loop count is 0, and the maximum number of records is 4095.

May 22 2020 14:33:06-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[22]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.2, NeighborEvent=HelloReceived, NeighborPreviousState=Down, NeighborCurrentState=Init)

May 22 2020 14:33:06.320.2-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1119 Level: 0x20

OSPF 1: Nbr 10.0.123.2 Rcv HelloReceived State Down -> Init.

May 22 2020 14:33:08.390.1-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 2061 Level: 0x20

OSPF 1 Send Hello Interface State Changed on 10.0.123.3

May 22 2020 14:33:08.390.2-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 2072 Level: 0x20

OSPF 1: Intf 10.0.123.3 Rcv WaitTimer State Waiting -> DR.

May 22 2020 14:33:08-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[23]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.2, NeighborEvent=2WayReceived, NeighborPreviousState=Init, NeighborCurrentState=ExStart)

May 22 2020 14:33:08-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[24]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.2, NeighborEvent=NegotiationDone, NeighborPreviousState=ExStart, NeighborCurrentState=Exchange)

May 22 2020 14:33:08.480.1-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1715 Level: 0x20

OSPF 1: Nbr 10.0.123.2 Rcv 2WayReceived State Init -> ExStart.

May 22 2020 14:33:08.530.1-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1828 Level: 0x20

OSPF 1: Nbr 10.0.123.2 Rcv NegotiationDone State ExStart -> Exchange.

May 22 2020 14:33:08-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[25]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.2, NeighborEvent=ExchangeDone, NeighborPreviousState=Exchange, NeighborCurrentState=Loading)

May 22 2020 14:33:08-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[26]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.2, NeighborEvent=LoadingDone, NeighborPreviousState=Loading, NeighborCurrentState=Full)

May 22 2020 14:33:08.590.3-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1940 Level: 0x20

OSPF 1: Nbr 10.0.123.2 Rcv ExchangeDone State Exchange -> Loading.

May 22 2020 14:33:08.590.4-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 2339 Level: 0x20

OSPF 1: Nbr 10.0.123.2 Rcv LoadingDone State Loading -> Full.

May 22 2020 14:33:10-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[27]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.1, NeighborEvent=HelloReceived, NeighborPreviousState=Down, NeighborCurrentState=Init)

May 22 2020 14:33:10-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[28]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.1, NeighborEvent=2WayReceived, NeighborPreviousState=Init, NeighborCurrentState=ExStart)

May 22 2020 14:33:10-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[29]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.1, NeighborEvent=NegotiationDone, NeighborPreviousState=ExStart, NeighborCurrentState=Exchange)

May 22 2020 14:33:10.340.1-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1119 Level: 0x20

OSPF 1: Nbr 10.0.123.1 Rcv HelloReceived State Down -> Init.

May 22 2020 14:33:10.340.2-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1715 Level: 0x20

OSPF 1: Nbr 10.0.123.1 Rcv 2WayReceived State Init -> ExStart.

May 22 2020 14:33:10.420.1-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1828 Level: 0x20

OSPF 1: Nbr 10.0.123.1 Rcv NegotiationDone State ExStart -> Exchange.

May 22 2020 14:33:10-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[30]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.1, NeighborEvent=ExchangeDone, NeighborPreviousState=Exchange, NeighborCurrentState=Loading)

May 22 2020 14:33:10-08:00 R3 %%01OSPF/4/NBR_CHANGE_E(l)[31]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.1, NeighborEvent=LoadingDone, NeighborPreviousState=Loading, NeighborCurrentState=Full)

May 22 2020 14:33:10.460.3-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1940 Level: 0x20

OSPF 1: Nbr 10.0.123.1 Rcv ExchangeDone State Exchange -> Loading.

May 22 2020 14:33:10.460.4-08:00 R3 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 2339 Level: 0x20

OSPF 1: Nbr 10.0.123.1 Rcv LoadingDone State Loading -> Full.

When OSPF is enabled almost at the same time, the debugging information shows that R3 has become the DR.

# Observe the debugging information on R2.

May 22 2020 14:32:29-08:00 R2 DS/4/DATASYNC_CFGCHANGE:OID 1.3.6.1.4.1.2011.5.25.191.3.1 configurations have been changed. The current change number is 15, the change loop count is 0, and the maximum number of records is 4095.

May 22 2020 14:32:29-08:00 R2 %%01PHY/1/PHY(l)[18]:    GigabitEthernet0/0/4: change status to up

May 22 2020 14:32:29-08:00 R2 %%01IFNET/4/LINK_STATE(l)[19]:The line protocol IP on the interface GigabitEthernet0/0/4 has entered the UP state.

May 22 2020 14:32:29.760.5-08:00 R2 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 1281 Level: 0x20

OSPF 1: Intf 10.0.123.2 Rcv InterfaceUp State Down -> Waiting.

May 22 2020 14:32:29.760.6-08:00 R2 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 1395 Level: 0x20

OSPF 1 Send Hello Interface Up on 10.0.123.2

May 22 2020 14:33:06.310.1-08:00 R2 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 2061 Level: 0x20

OSPF 1 Send Hello Interface State Changed on 10.0.123.2

May 22 2020 14:33:06.310.2-08:00 R2 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 2072 Level: 0x20

OSPF 1: Intf 10.0.123.2 Rcv WaitTimer State Waiting -> DR.

May 22 2020 14:33:08-08:00 R2 %%01OSPF/4/NBR_CHANGE_E(l)[20]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.3, NeighborEvent=HelloReceived, NeighborPreviousState=Down, NeighborCurrentState=Init)

May 22 2020 14:33:08-08:00 R2 %%01OSPF/4/NBR_CHANGE_E(l)[21]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.3, NeighborEvent=2WayReceived, NeighborPreviousState=Init, NeighborCurrentState=ExStart)

May 22 2020 14:33:08-08:00 R2 %%01OSPF/4/NBR_CHANGE_E(l)[22]:Neighbor changes event: neighbor status changed. (ProcessId=1, NeighborAddress=10.0.123.3, NeighborEvent=NegotiationDone, NeighborPreviousState=ExStart, NeighborCurrentState=Exchange)

May 22 2020 14:33:08.420.1-08:00 R2 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1119 Level: 0x20

OSPF 1: Nbr 10.0.123.3 Rcv HelloReceived State Down -> Init.

May 22 2020 14:33:08.420.2-08:00 R2 RM/6/RMDEBUG:

FileID: 0x7017802d Line: 1715 Level: 0x20

OSPF 1: Nbr 10.0.123.3 Rcv 2WayReceived State Init -> ExStart.

May 22 2020 14:33:08.420.3-08:00 R2 RM/6/RMDEBUG:

FileID: 0x7017802c Line: 2501 Level: 0x20

OSPF 1: Intf 10.0.123.2 Rcv NeighborChange State DR -> BackupDR.

The debugging information shows that R2 has become the BDR.

Configure the network type of an OSPF interface.

# In the OSPF routing table on R1, check the Loopback0 routes learned from R2 and R3.

<R1>display ospf routing 10.0.2.2

OSPF Process 1 with Router ID 10.0.1.1

Destination	:	10.0.2.2/32

AdverRouter	:	10.0.2.2																	Area						:	0.0.0.0

Cost								:	1																							Type						:	Stub

NextHop				:	10.0.123.2															Interface		:	GigabitEthernet0/0/3

Priority						:	Medium																	Age						:	00h09m02s

<R1>display ospf routing 10.0.3.3

OSPF Process 1 with Router ID 10.0.1.1

Destination : 10.0.3.3/32

AdverRouter	:	10.0.3.3																	Area						:	0.0.0.0

Cost								:	1																							Type						:	Stub

NextHop				:	10.0.123.3															Interface		:	GigabitEthernet0/0/3

Priority						:	Medium																	Age						:	00h09m13s

The command output shows that the masks of the Loopback0 routes are both 32 bits, not 24 bits.

# Take R2 as an example, and check the Type 1 LSA on R2.

<R2>display ospf lsdb router 10.0.2.2

OSPF Process 1 with Router ID 10.0.2.2

Area: 0.0.0.0

Link State Database

Type								:	Router

Ls	id								:	10.0.2.2

Adv	rtr						:	10.0.2.2

Ls	age							:	1528

Len									:	48

Options					:	E

seq#								:	80000020

chksum					:	0x9653

Link	count			:	2

*	Link	ID				:	10.0.2.2

Data					:	255.255.255.255

Link	Type	:	StubNet

Metric			:	0

Priority			:	Medium

*	Link	ID			:	10.0.123.3

Data					:	10.0.123.2

Link	Type	:	TransNet

Metric			:	1

The command output shows that the mask of the LSA describing Loopback0 on R2 has been set to 32 bits. OSPF considers a loopback interface as a stub network, which is connected to only one node. Therefore, no matter how many bits are actually set for the network mask of the loopback interface, OSPF uses a 32-bit network mask (host mask) when advertising the Type 1 LSA to describe this interface.

For details about OSPF loopback interfaces, see section 9.1 in RFC 2328.

# Change the network type of Loopback0 on R2.

[R2]interface LoopBack 0

[R2-LoopBack0] ospf network-type broadcast

After the network type of the loopback interface is changed to broadcast, OSPF uses the actual mask of the interface to advertise its network information. In this example, OSPF uses Loopback0's actual mask length of 24 bits.

# In the OSPF routing table on R1, check the Loopback0 route learned from R2 again.

<R1>display ospf routing 10.0.2.2

OSPF Process 1 with Router ID 10.0.1.1

Destination		:	10.0.2.0/24

AdverRouter	:	10.0.2.2																	Area						:	0.0.0.0

Cost								:	1																							Type						:	Stub

NextHop				:	10.0.123.2															Interface		:	GigabitEthernet0/0/3

Priority						:	Low																				Age						:	00h04m10s

The command output shows that the mask length of the route has changed to 24 bits.

Change the costs of OSPF interfaces.

# In the OSPF routing table on R1, check the Loopback0 route learned from R3.

<R1>display ospf routing 10.0.3.3

OSPF Process 1 with Router ID 10.0.1.1

Destination		:	10.0.3.3/32

AdverRouter	:	10.0.3.3																	Area						:	0.0.0.0

Cost								:	1																							Type						:	Stub

NextHop				:	10.0.123.3															Interface		:	GigabitEthernet0/0/3

Priority						:	Medium																	Age						:	00h46m56s

The command output shows that the route cost is 1.

# Change the OSPF cost of GE0/0/3 on R1 to 20 and that of GE0/0/4 on R3 to 10.

[R1]interface GigabitEthernet0/0/3

[R1-GigabitEthernet0/0/3] ospf cost 20

[R3]interface GigabitEthernet0/0/4

[R3-GigabitEthernet0/0/4] ospf cost 10

# In the OSPF routing table on R1, check the Loopback0 route learned from R2 again.

<R1>display ospf routing 10.0.2.2

OSPF Process 1 with Router ID 10.0.1.1

Destination		:	10.0.2.0/24

AdverRouter	:	10.0.2.2																	Area					:	0.0.0.0

Cost								:	20																						Type					:	Stub

NextHop				:	10.0.123.2															Interface	:	GigabitEthernet0/0/3

Priority						:	Low																				Age					:	00h04m19s

The command output shows that the cost is 20.

# In the OSPF routing table on R3, check the Loopback0 route learned from R1.

<R3>display ospf routing 10.0.1.1

OSPF Process 1 with Router ID 10.0.3.3

Destination		:	10.0.1.1/32

AdverRouter	:	10.0.1.1																	Area					:	0.0.0.0

Cost								:	10																						Type					:	Stub

NextHop				:	10.0.123.1															Interface	:	GigabitEthernet0/0/4

Priority						:	Medium																	Age					:	00h06m07s

The command output shows that the cost is 10.

Configure an OSPF silent interface.

# Configure GE0/0/3 of R1 as a silent interface.

[R1]ospf 1

[R1-ospf-1] silent-interface GigabitEthernet 0/0/3

# Check the OSPF neighbor information on R1.

<R1>display ospf peer

OSPF Process 1 with Router ID 10.0.1.1

After the configuration is completed, the interconnection interface no longer sends or receives Hello packets. The neighbor relationship that has been established using this interface disappears.

# Check information about the OSPF interface GE0/0/3 on R1.

<R1>display ospf interface GigabitEthernet 0/0/3

OSPF Process 1 with Router ID 10.0.1.1

Interfaces

Interface: 10.0.123.1 (GigabitEthernet0/0/3)

Cost: 20      State: DR        Type: Broadcast    MTU: 1500

Priority: 1

Designated Router: 10.0.123.1

Backup Designated Router: 0.0.0.0

Timers: Hello 10 , Dead 40 , Poll  120 , Retransmit 5 , Transmit Delay 1

Silent interface, No hellos

The command output shows that the interface is configured as a silent interface and no Hello packet exists on the interface.

# Delete the silent interface configuration on R1.

The configuration details are not provided.

# Configure the Loopback0 interfaces on R2 and R3 as silent interfaces.

[R2]ospf 1

[R2-ospf-1] silent-interface LoopBack 0

[R3]ospf 1

[R3-ospf-1] silent-interface LoopBack 0

# Check the OSPF routing table on R1.

<R1>display ospf routing

OSPF Process 1 with Router ID 10.0.1.1

Routing Tables

Routing for Network

Destination						Cost			Type							NextHop							AdvRouter						Area

10.0.1.1/32								0					Stub							10.0.1.1								10.0.1.1								0.0.0.0

10.0.123.0/24						20				Transit					10.0.123.1						10.0.1.1								0.0.0.0

10.0.2.0/24								20				Stub							10.0.123.2						10.0.2.2								0.0.0.0

10.0.3.3/32								20				Stub							10.0.123.3						10.0.3.3								0.0.0.0

Total Nets: 4

Intra Area: 4  Inter Area: 0  ASE: 0  NSSA: 0

The command output shows that routes the Loopback0 routes learned from R2 and R3 still exist.

----End

    - Quiz

Analyze which interfaces can be configured as silent interfaces in actual networking scenarios.

    - Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/3

ip address 10.0.123.1 255.255.255.0

ospf cost 20

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.0

#

ospf 1 router-id 10.0.1.1

area 0.0.0.0

authentication-mode simple plain huawei

network 10.0.123.1 0.0.0.0

network 10.0.1.1 0.0.0.0

#

return

Configuration on R2

#

sysname R2

#

interface GigabitEthernet0/0/4

ip address 10.0.123.2 255.255.255.0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.2.2

silent-interface LoopBack0

area 0.0.0.0

authentication-mode simple plain huawei

network 10.0.123.2 0.0.0.0

network 10.0.2.2 0.0.0.0

#

return

Configuration on R3

#

sysname R3

#

interface GigabitEthernet0/0/4

ip address 10.0.123.3 255.255.255.0

ospf cost 10

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.0

#

ospf 1 router-id 10.0.3.3

silent-interface LoopBack0

area 0.0.0.0

authentication-mode simple plain huawei

network 10.0.3.3 0.0.0.0

network 10.0.123.3 0.0.0.0

user-interface vty 16 20

#

Return

  - Lab 2: Multi-Area OSPF

Introduction

      - Objectives

Upon completion of this task, you will be able to:

Manually specify OSPF router IDs.

Configure OSPF in multiple areas.

Illustrate how to configure route summarization between OSPF areas.

Illustrate how to configure an OSPF bandwidth reference value.

Illustrate how to configure OSPF to import external routes.

Illustrate how to perform route summarization when OSPF imports external routes.

Illustrate how to import default routes to the OSPF routing table.

Illustrate how to change the preferences of different types of OSPF routes.

      - Networking Topology

              - OSPF multi-area

The preceding figure shows the interconnection interfaces and their IP addresses. Loopback0 is created on each device, and its IP address is in the format of 10.0.x.x/24, where x indicates the device number.

All interfaces of R1 and R3 and GE0/0/4 of R2 belong to OSPF area 2. The Loopback0 and interconnection interfaces between R2 and R4 belong to OSPF area 0. The interconnection interfaces between R4 and R5 and the loopback interfaces 0, 1, and 2 of R5 belong to OSPF area 1.

Loopback1 and Loopback2 are created on R2 to simulate external network segments.

      - Background

You are a network administrator of a company. The company's network now has five AR routers, among which R2 and R4 reside in the headquarters, and R1, R3, and R5 reside in branches. To control LSA flooding on the large-scale network, multi-area OSPF is designed.

To specify the router ID of each device, the devices are configured to use fixed IP addresses as their router IDs.

To improve the efficiency of forwarding routes on the devices, automatic route summarization is configured on the ABR.

R1 is connected to the Internet. You need to configure a default route and import it to the OSPF areas so that all routers in the OSPF areas know how to access the Internet.

In addition, the OSPF routing information is classifies as internal routes or external routes. The preferences of these routes are changed to prevent potential risks.

In OSPF, the cost of a specific route is the sum of the costs of all the links that the route passes through before reaching the destination network. The cost of a link is obtained by dividing the bandwidth reference value by the interface bandwidth. The default bandwidth reference value is 100 Mbit/s. The actual interface bandwidth may be 1000 Mbit/s, and cost values are integers. As a result, the OSPF costs of an FE interface and a GE interface are both 1. To differentiate these links, you can set the bandwidth reference value to 10 Gbit/s.

    - Lab Configuration

      - Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF areas as planned.

- Verify the OSPF configuration by checking the OSPF neighbor relationship status and the OSPF LSDBs on the ABR.

- Configure route summarization on ABRs and ASBRs to reduce the number of inter-area and AS external routes.

- Change the bandwidth reference value of OSPF.

- Import a default route to the OSPF routing table.

- Change the default preferences of intra-area, inter-area, and AS external OSPF routes.

      - Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/3 and Loopback0 of R1.

[R1]interface GigabitEthernet0/0/3

[R1-GigabitEthernet0/0/3] ip address 10.0.123.1 24

[R1-GigabitEthernet0/0/3] quit

[R1]interface LoopBack 0

[R1-LoopBack0] ip address 10.0.1.1 24

[R1-LoopBack0] quit

# Configure IP addresses for GE0/0/4 and GE0/0/2 as well as the loopback interfaces on R2.

[R2]interface GigabitEthernet0/0/4

[R2-GigabitEthernet0/0/4] ip address 10.0.123.2 24

[R2-GigabitEthernet0/0/4] quit

[R2] interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] ip address 10.0.24.2 24

[R2-GigabitEthernet0/0/2] quit

[R2]interface LoopBack 0

[R2-LoopBack0] ip address 10.0.2.2 24

[R2-LoopBack0] quit

[R2]interface LoopBack1

[R2-LoopBack1] ip address 10.2.0.1 255.255.255.0

[R2-LoopBack1] quit

[R2]interface LoopBack2

[R2-LoopBack2] ip address 10.2.1.1 255.255.255.0

[R2-LoopBack2] quit

# Configure IP addresses for GE0/0/4 and the loopback interfaces on R3.

[R3]interface GigabitEthernet0/0/4

[R3-GigabitEthernet0/0/4] ip address 10.0.123.3 24

[R3-GigabitEthernet0/0/4] quit

[R3]interface LoopBack 0

[R3-LoopBack0] ip address 10.0.3.3 24

[R3-LoopBack0] quit

[R3]interface LoopBack 1

[R3-LoopBack1] ip address 10.3.0.1 24

[R3-LoopBack1] quit

[R3]interface LoopBack 2

[R3-LoopBack2] ip address 10.3.1.1 24

[R3-LoopBack2] quit

# Configure IP addresses for GE0/0/3, GE0/0/2, and Loopback0 of R4.

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] ip address 10.0.24.4 24

[R4-GigabitEthernet0/0/3] quit

[R4]interface GigabitEthernet0/0/2

[R4-GigabitEthernet0/0/2] ip address 10.0.45.4 24

[R4-GigabitEthernet0/0/2] quit

[R4]interface LoopBack 0

[R4-LoopBack0] ip address 10.0.4.4 24

[R4-LoopBack0] quit

# Configure IP addresses for GE0/0/3 and the loopback interfaces on R5.

[R5]interface GigabitEthernet0/0/3

[R5-GigabitEthernet0/0/3] ip address 10.0.45.5 24

[R5-GigabitEthernet0/0/3] quit

[R5]interface LoopBack 0

[R5-LoopBack0] ip address 10.0.5.5 24

[R5-LoopBack0] quit

[R5]interface LoopBack 1

[R5-LoopBack1] ip address 10.5.0.1 24

[R5-LoopBack1] quit

[R5]int LoopBack 2

[R5-LoopBack2] ip address 10.5.1.1 24

[R5-LoopBack2] quit

# On R2, ping the IP addresses of R1, R3, and R4 to test the connectivity.

<R2>ping -c 1 10.0.123.1

PING 10.0.123.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.1: bytes=56 Sequence=1 ttl=255 time=70 ms

--- 10.0.123.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 70/70/70 ms

<R2>ping -c 1 10.0.123.3

PING 10.0.123.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.3: bytes=56 Sequence=1 ttl=255 time=110 ms

--- 10.0.123.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 110/110/110 ms

<R2>ping -c 1 10.0.24.4

PING 10.0.24.4: 56  data bytes, press CTRL_C to break

Reply from 10.0.24.4: bytes=56 Sequence=1 ttl=255 time=40 ms

--- 10.0.24.4 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/40/40 ms

# On R4, ping the IP address of R5 to test the connectivity.

<R4>ping -c 1 10.0.45.5

PING 10.0.45.5: 56  data bytes, press CTRL_C to break

Reply from 10.0.45.5: bytes=56 Sequence=1 ttl=255 time=80 ms

--- 10.0.45.5 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 80/80/80 ms

Configure multi-area OSPF.

# Configure OSPF on R1, activate OSPF on GE0/0/3 and Loopback0, and change the network type of Loopback0 to broadcast.

[R1]ospf 1 router-id 10.0.1.1

[R1-ospf-1]area 2

[R1-ospf-1-area-0.0.0.2] network 10.0.123.1 0.0.0.0

[R1-ospf-1-area-0.0.0.2] network 10.0.1.1 0.0.0.0

[R1-ospf-1-area-0.0.0.2] quit

[R1-ospf-1]quit

[R1]interface LoopBack 0

[R1-LoopBack0] ospf network-type broadcast

[R1-LoopBack0] quit

# Configure OSPF on R2, activate OSPF on GE0/0/2, GE0/0/4, and Loopback0, and change the network type of Loopback0 to broadcast.

[R2]ospf 1 router-id 10.0.2.2

[R2-ospf-1]area 0

[R2-ospf-1-area-0.0.0.0] network 10.0.24.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] quit

[R2-ospf-1]area 2

[R2-ospf-1-area-0.0.0.2] network 10.0.123.2 0.0.0.0

[R2-ospf-1-area-0.0.0.2] quit

[R2-ospf-1]quit

[R2]interface LoopBack 0

[R2-LoopBack0] ospf network-type broadcast

[R2-LoopBack0] quit

# Configure OSPF on R3, activate OSPF on GE0/0/4, Loopback0, Loopback1, and Loopback2, and change the network types of Loopback0, Loopback1, and Loopback2 to broadcast.

[R3]ospf 1 router-id 10.0.3.3

[R3-ospf-1]area 2

[R3-ospf-1-area-0.0.0.2] network 10.0.123.3 0.0.0.0

[R3-ospf-1-area-0.0.0.2] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.2] network 10.3.0.1 0.0.0.0

[R3-ospf-1-area-0.0.0.2] network 10.3.1.1 0.0.0.0

[R3-ospf-1-area-0.0.0.2] quit

[R3-ospf-1]quit

[R3]interface LoopBack 0

[R3-LoopBack0] ospf network-type broadcast

[R3-LoopBack0] quit

[R3]interface LoopBack 1

[R3-LoopBack1] ospf network-type broadcast

[R3-LoopBack1] quit

[R3]interface LoopBack 2

[R3-LoopBack2] ospf network-type broadcast

[R3-LoopBack2] quit

# Configure OSPF on R4, activate OSPF on GE0/0/2, GE0/0/3, and Loopback0, and change the network type of Loopback0 to broadcast.

[R4]ospf 1 router-id 10.0.4.4

[R4-ospf-1]area 0

[R4-ospf-1-area-0.0.0.0] network 10.0.24.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] quit

[R4-ospf-1]area 1

[R4-ospf-1-area-0.0.0.1] network 10.0.45.4 0.0.0.0

[R4-ospf-1-area-0.0.0.1] quit

[R4-ospf-1]quit

[R4]interface LoopBack 0

[R4-LoopBack0] ospf network-type broadcast

[R4-LoopBack0] quit

# Configure OSPF on R5, activate OSPF on GE0/0/3, Loopback0, Loopback1, and Loopback2, and change the network types of Loopback0, Loopback1, and Loopback2 to broadcast.

[R5]ospf 1 router-id 10.0.5.5

[R5-ospf-1]area 1

[R5-ospf-1-area-0.0.0.1] network 10.0.5.5 0.0.0.0

[R5-ospf-1-area-0.0.0.1] network 10.5.0.1 0.0.0.0

[R5-ospf-1-area-0.0.0.1] network 10.5.1.1 0.0.0.0

[R5-ospf-1-area-0.0.0.1] network 10.0.45.5 0.0.0.0

[R5-ospf-1-area-0.0.0.1] quit

[R5-ospf-1]quit

[R5]interface LoopBack 0

[R5-LoopBack0] ospf network-type broadcast

[R5-LoopBack0] quit

[R5]interface LoopBack 1

[R5-LoopBack1] ospf network-type broadcast

[R5-LoopBack1] quit

[R5]interface LoopBack 2

[R5-LoopBack2] ospf network-type broadcast

[R5-LoopBack2] quit

Verify the OSPF configuration.

# Check the brief information about OSPF neighbor relationships on R2.

<R2>display ospf peer brief

OSPF Process 1 with Router ID 10.0.2.2

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id     State

0.0.0.0          GigabitEthernet0/0/2             10.0.4.4        Full

0.0.0.2          GigabitEthernet0/0/4             10.0.1.1        Full

0.0.0.2          GigabitEthernet0/0/4             10.0.3.3        Full

----------------------------------------------------------------------------

# Check the brief information about neighbor relationships and the OSPF routing table on R5.

<R5>display ospf peer brief

OSPF Process 1 with Router ID 10.0.5.5

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.1          GigabitEthernet0/0/3             10.0.4.4         Full

----------------------------------------------------------------------------

<R5>display ospf routing

OSPF Process 1 with Router ID 10.0.5.5

Routing Tables

Routing for Network

Destination							Cost		Type								NextHop						AdvRouter							Area

10.0.5.0/24								0					Stub								10.0.5.5								10.0.5.5								0.0.0.1

10.0.45.0/24							1					Transit						10.0.45.5							10.0.5.5								0.0.0.1

10.5.0.0/24								0					Stub								10.5.0.1								10.0.5.5								0.0.0.1

10.5.1.0/24								0					Stub								10.5.1.1								10.0.5.5								0.0.0.1

10.0.1.0/24								3					Inter-area			10.0.45.4							10.0.4.4								0.0.0.1

10.0.2.0/24								2					Inter-area			10.0.45.4							10.0.4.4								0.0.0.1

10.0.3.0/24								3					Inter-area			10.0.45.4							10.0.4.4								0.0.0.1

10.0.4.0/24								1					Inter-area			10.0.45.4							10.0.4.4								0.0.0.1

10.0.24.0/24							2					Inter-area			10.0.45.4							10.0.4.4								0.0.0.1

10.0.123.0/24						3					Inter-area			10.0.45.4							10.0.4.4								0.0.0.1

10.3.0.0/24								3					Inter-area			10.0.45.4							10.0.4.4								0.0.0.1

10.3.1.0/24								3					Inter-area			10.0.45.4							10.0.4.4								0.0.0.1

Total Nets: 12

Intra Area: 4  Inter Area: 8  ASE: 0  NSSA: 0

# Check the OSPF LSDBs on R2.

<R2>display	ospf	lsdb

OSPF	Process	1	with	Router	ID	10.0.2.2

Link	State	Database

Area:	0.0.0.0

Type						LinkState	ID					AdvRouter									Age		Len			Sequence				Metric

Router					10.0.4.4								10.0.4.4											54			48				8000000B						0

Router					10.0.2.2								10.0.2.2											54			48				80000008							0

Network				10.0.24.4							10.0.4.4											54			32				80000003							0

Sum-Net			10.3.1.0								10.0.2.2									1332			28				80000001							1

Sum-Net			10.3.0.0								10.0.2.2									1332			28				80000001							1

Sum-Net			10.5.1.0								10.0.4.4										259			28				80000002							1

Sum-Net			10.0.3.0								10.0.2.2									1332			28				80000001							1

Sum-Net			10.5.0.0								10.0.4.4										268			28				80000002							1

Sum-Net			10.0.1.0								10.0.2.2										244			28				80000001							1

Sum-Net			10.0.5.0								10.0.4.4										278			28				80000002							1

Sum-Net			10.0.45.0							10.0.4.4										500			28				80000002							1

Sum-Net			10.0.123.0						10.0.2.2											45			28				80000002							1

Area:	0.0.0.2

Type						LinkState	ID				AdvRouter										Age		Len			Sequence				Metric

Router					10.0.3.3								10.0.3.3										247			72				80000017							0

Router					10.0.2.2								10.0.2.2										247			36				80000008							1

Router					10.0.1.1								10.0.1.1										246			48				80000008							1

Network				10.0.123.3						10.0.3.3										247			36				80000006							0

Sum-Net			10.0.24.0							10.0.2.2											45			28				80000002							1

Sum-Net			10.5.1.0								10.0.2.2											45			28				80000002							2

Sum-Net			10.5.0.0								10.0.2.2											45			28				80000002							2

Sum-Net			10.0.2.0								10.0.2.2											45			28				80000002							0

Sum-Net			10.0.5.0								10.0.2.2											45			28				80000002							2

Sum-Net			10.0.4.0								10.0.2.2											45			28				80000002							1

Sum-Net			10.0.45.0							10.0.2.2											45			28				80000002							2

R2 functions as an ABR to maintain the LSDBs of area 0 and area 2. The LSAs in the LSDBs are used to describe routes in these two areas.

Configure route summarization for OSPF inter-area routes and AS external routes.

# Check the OSPF routing tables on R2 and R4.

<R2>display ospf routing

OSPF Process 1 with Router ID 10.0.2.2

Routing Tables

Routing for Network

Destination						Cost			Type							NextHop							AdvRouter						Area

10.0.2.0/24								0					Stub							10.0.2.2								10.0.2.2								0.0.0.0

10.0.24.0/24							1					Transit					10.0.24.2							10.0.2.2								0.0.0.0

10.0.123.0/24						1					Transit					10.0.123.2						10.0.2.2								0.0.0.2

10.0.1.0/24								1					Stub							10.0.123.1						10.0.1.1								0.0.0.2

10.0.3.0/24								1					Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.0.4.0/24								1					Stub							10.0.24.4							10.0.4.4								0.0.0.0

10.0.5.0/24								2					Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

10.0.45.0/24							2					Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

10.3.0.0/24								1					Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.3.1.0/24								1					Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.5.0.0/24								2					Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

10.5.1.0/24								2					Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

Total Nets: 12

<R4>display ospf routing

OSPF Process 1 with Router ID 10.0.4.4

Routing Tables

Routing for Network

Destination						Cost			Type							NextHop							AdvRouter						Area

10.0.4.0/24								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.24.0/24							1					Transit					10.0.24.4							10.0.4.4								0.0.0.0

10.0.45.0/24							1					Transit					10.0.45.4							10.0.4.4								0.0.0.1

10.0.1.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.0.2.0/24								1					Stub							10.0.24.2							10.0.2.2								0.0.0.0

10.0.3.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.0.5.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

10.0.123.0/24						2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.3.0.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.3.1.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.5.0.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

10.5.1.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

Total Nets: 12

Intra Area: 7  Inter Area: 5  ASE: 0  NSSA: 0

The OSPF inter-area routes of Loopback1 and Loopback2 on R5 in the OSPF routing table of R2 as well as those of Loopback1 and Loopback2 on R3 in the OSPF routing table of R4 can be summarized before being advertised to other areas. This reduces the number of routing entries in other areas and the possibility of route flapping.

# On R4, summarize the Loopback1 and Loopback2 routes learned from R5.

[R4]ospf 1

[R4-ospf-1]area 1

[R4-ospf-1-area-0.0.0.1] abr-summary 10.5.0.0 255.255.254.0

[R4-ospf-1-area-0.0.0.1] quit

# Check the OSPF routing table on R2.

<R2>display ospf routing

OSPF Process 1 with Router ID 10.0.2.2

Routing Tables

Routing for Network

Destination							Cost		Type								NextHop							AdvRouter						Area

10.0.2.0/24								0					Stub								10.0.2.2								10.0.2.2								0.0.0.0

10.0.24.0/24							1					Transit						10.0.24.2							10.0.2.2								0.0.0.0

10.0.123.0/24						1					Transit						10.0.123.2						10.0.2.2								0.0.0.2

10.0.1.0/24								1					Stub								10.0.123.1						10.0.1.1								0.0.0.2

10.0.3.0/24								1					Stub								10.0.123.3						10.0.3.3								0.0.0.2

10.0.4.0/24								1					Stub								10.0.24.4							10.0.4.4								0.0.0.0

10.0.5.0/24								2					Inter-area			10.0.24.4							10.0.4.4								0.0.0.0

10.0.45.0/24							2					Inter-area			10.0.24.4							10.0.4.4								0.0.0.0

10.3.0.0/24								1					Stub								10.0.123.3						10.0.3.3								0.0.0.2

10.3.1.0/24								1					Stub								10.0.123.3						10.0.3.3								0.0.0.2

10.5.0.0/23								2					Inter-area			10.0.24.4							10.0.4.4								0.0.0.0

Total Nets: 11

Intra Area: 8  Inter Area: 3  ASE: 0  NSSA: 0

The command output shows that the Loopback1 and Loopback2 routes of R5 are summarized into an inter-area summary route.

# On R2, summarize the Loopback1 and Loopback2 routes learned from R3.

[R2]ospf 1

[R2-ospf-1]area 2

[R2-ospf-1-area-0.0.0.2] abr-summary 10.3.0.0 255.255.254.0

# Check the OSPF routing table on R4.

<R4>display ospf routing

OSPF Process 1 with Router ID 10.0.4.4

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter						Area

10.0.4.0/24								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.24.0/24							1					Transit					10.0.24.4							10.0.4.4								0.0.0.0

10.0.45.0/24							1					Transit					10.0.45.4							10.0.4.4								0.0.0.1

10.0.1.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.0.2.0/24								1					Stub							10.0.24.2							10.0.2.2								0.0.0.0

10.0.3.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.0.5.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

10.0.123.0/24						2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.3.0.0/23								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.5.0.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

10.5.1.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

Total Nets: 11

Intra Area: 7  Inter Area: 4  ASE: 0  NSSA: 0

The command output shows that the Loopback1 and Loopback2 routes of R3 are summarized into an inter-area summary route.

# Import the Loopback1 and Loopback2 routes to the OSPF routing table on R2.

[R2]ospf 1

[R2-ospf-1] import-route direct

# Check the OSPF routing table on R4.

<R4>display	ospf	routing

OSPF	Process	1	with	Router	ID	10.0.4.4

Routing	Tables

Routing	for	Network

Destination							Cost		Type							NextHop							AdvRouter						Area

10.0.4.0/24								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.24.0/24							1					Transit					10.0.24.4							10.0.4.4								0.0.0.0

10.0.45.0/24							1					Transit					10.0.45.4							10.0.4.4								0.0.0.1

10.0.1.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.0.2.0/24								1					Stub							10.0.24.2							10.0.2.2								0.0.0.0

10.0.3.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.0.5.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

10.0.123.0/24						2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.3.0.0/23								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.5.0.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

10.5.1.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

Routing	for	ASEs

Destination							Cost					Type								Tag								NextHop							AdvRouter

10.2.0.0/24									1							Type2							1											10.0.24.2							10.0.2.2

10.2.1.0/24									1							Type2							1											10.0.24.2							10.0.2.2

Total	Nets:	13

Intra	Area:	7		Inter	Area:	4		ASE:	2		NSSA:	0

The OSPF routing table of R4 contains the Loopback1 and Loopback2 routes of R2.

# Configure AS external route summarization on R2.

[R2]ospf 1

[R2-ospf-1] asbr-summary 10.2.0.0 255.255.254.0

# Check the OSPF routing table on R4 again.

<R4>display	ospf	routing

OSPF	Process	1	with	Router	ID	10.0.4.4

Routing	Tables

Routing	for	Network

Destination							Cost		Type							NextHop							AdvRouter							Area

10.0.4.0/24								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.24.0/24							1					Transit					10.0.24.4							10.0.4.4								0.0.0.0

10.0.45.0/24							1					Transit					10.0.45.4							10.0.4.4								0.0.0.1

10.0.1.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.0.2.0/24								1					Stub							10.0.24.2							10.0.2.2								0.0.0.0

10.0.3.0/24								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.0.5.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

10.0.123.0/24						2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.3.0.0/23								2					Inter-area		10.0.24.2							10.0.2.2								0.0.0.0

10.5.0.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

10.5.1.0/24								1					Stub							10.0.45.5							10.0.5.5								0.0.0.1

Routing	for	ASEs

Destination						Cost						Type							Tag								NextHop							AdvRouter

10.2.0.0/23								2								Type2						1											10.0.24.2							10.0.2.2

Total	Nets:	12

Intra	Area:	7		Inter	Area:	4		ASE:	1		NSSA:	0

The command output shows that the Loopback1 and Loopback2 routes of R2 are summarized into an AS external summary route.

Change the bandwidth reference value of OSPF.

Gigabit or even 10-Gigabit Ethernet may be used in actual networking scenarios. The default bandwidth reference value of OSPF is 100 Mbit/s, and an interface cost is an integer. As a result, OSPF cannot distinguish an FE interface from a GE interface in terms of bandwidth.

Multiple OSPF areas must use the same bandwidth reference value. Otherwise, OSPF cannot work properly.

# Change the OSPF bandwidth reference value of each router to 10 Gbit/s.

[R1]ospf 1

[R1-ospf-1] bandwidth-reference 10000

[R1-ospf-1] quit

[R2]ospf 1

[R2-ospf-1] bandwidth-reference 10000

[R2-ospf-1] quit

[R3]ospf 1

[R3-ospf-1] bandwidth-reference 10000

[R3-ospf-1] quit

[R4]ospf 1

[R4-ospf-1] bandwidth-reference 10000

[R4-ospf-1] quit

[R5]ospf 1

[R5-ospf-1] bandwidth-reference 10000

[R5-ospf-1] quit

# Take R2 as an example to check its OSPF routing table.

[R2]display	ospf	routing

OSPF	Process	1	with	Router	ID	10.0.2.2

Routing	Tables

Routing	for	Network

Destination							Cost		Type							NextHop							AdvRouter							Area

10.0.2.0/24								0					Stub							10.0.2.2								10.0.2.2								0.0.0.0

10.0.24.0/24							10				Transit					10.0.24.2							10.0.2.2								0.0.0.0

10.0.123.0/24						10				Transit					10.0.123.2						10.0.2.2								0.0.0.2

10.0.1.0/24								10				Stub							10.0.123.1						10.0.1.1								0.0.0.2

10.0.3.0/24								10				Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.0.4.0/24								10				Stub							10.0.24.4							10.0.4.4								0.0.0.0

10.0.5.0/24								20				Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

10.0.45.0/24							20				Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

10.3.0.0/24								10				Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.3.1.0/24								10				Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.5.0.0/23								20				Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

Total	Nets:	11

Intra	Area:	8		Inter	Area:	3		ASE:	0		NSSA:	0

The command output shows that the costs of the routes have changed.

Configure OSPF to import a default route.

# Use Loopback0 of R1 to simulate an interface accessing the Internet, and configure a default route on R1, with Loopback0 specified as the outbound interface.

[R1]ip route-static 0.0.0.0 0.0.0.0 LoopBack 0

# Import the default route to the OSPF routing table and set the type of the AS external route to Type 1.

[R1]ospf 1

[R1-ospf-1] default-route-advertise always type 1

[R1-ospf-1] quit

# Check the OSPF routing table on R2.

[R2]display ospf routing

OSPF Process 1 with Router ID 10.0.2.2

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter						Area

10.0.2.0/24								0					Stub							10.0.2.2								10.0.2.2								0.0.0.0

10.0.24.0/24							10				Transit					10.0.24.2							10.0.2.2								0.0.0.0

10.0.123.0/24						10				Transit					10.0.123.2						10.0.2.2								0.0.0.2

10.0.1.0/24								10				Stub							10.0.123.1						10.0.1.1								0.0.0.2

10.0.3.0/24								10				Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.0.4.0/24								10				Stub							10.0.24.4							10.0.4.4								0.0.0.0

10.0.5.0/24								20				Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

10.0.45.0/24							20				Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

10.3.0.0/24								10				Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.3.1.0/24								10				Stub							10.0.123.3						10.0.3.3								0.0.0.2

10.5.0.0/23								20				Inter-area		10.0.24.4							10.0.4.4								0.0.0.0

Routing for ASEs

Destination       Cost      Type       Tag         NextHop       AdvRouter

0.0.0.0/0          11        Type1      1           10.0.123.1      10.0.1.1

Total Nets: 12

Intra Area: 8  Inter Area: 3  ASE: 1  NSSA: 0

The command output shows that R2 has learned the default route with R1 as the next hop through a Type 5 LSA.

Change the preferences of the two types of OSPF routes.

By default, the preference of intra-area and inter-area OSPF routes is 10; the preference of AS external routes is 150.

# On R1 and R3, change the preference of intra-area and inter-area routes to 20, and change the preference of AS external routes to 50.

[R1]ospf 1

[R1-ospf-1] preference 20

[R1-ospf-1] preference ase 50

[R1-ospf-1] quit

[R3]ospf 1

[R3-ospf-1] preference 20

[R3-ospf-1] preference ase 50

[R3-ospf-1] quit

The operation in this step only shows how to change the preferences of internal and external routes, and does not have actual meaning in this experiment.

# Check the OSPF routes in the IP routing table on R3.

<R3>display	ip	routing-table	protocol	ospf

Route	Flags:	R	-	relay,	D	-	download	to	fib

------------------------------------------------------------------------------

Public	routing	table	:	OSPF

Destinations	:	9								Routes	:	9

OSPF	routing	table	status	:	<Active>

Destinations	:	9								Routes	:	9

Destination/Mask			Proto					Pre		Cost							Flags	NextHop							Interface

0.0.0.0/0			O_ASE				50			11										D			10.0.123.1						GigabitEthernet0/0/3

10.0.1.0/24		OSPF					20			10										D			10.0.123.1						GigabitEthernet0/0/3

10.0.2.0/24		OSPF					20			10										D			10.0.123.2						GigabitEthernet0/0/3

10.0.4.0/24		OSPF					20			20										D			10.0.123.2						GigabitEthernet0/0/3

10.0.5.0/24		OSPF					20			30										D			10.0.123.2						GigabitEthernet0/0/3

10.0.24.0/24		OSPF					20			20										D			10.0.123.2						GigabitEthernet0/0/3

10.0.45.0/24		OSPF					20			30										D			10.0.123.2						GigabitEthernet0/0/3

10.2.0.0/23		O_ASE				50				2										D			10.0.123.2						GigabitEthernet0/0/3

10.5.0.0/23		OSPF					20			30										D			10.0.123.2						GigabitEthernet0/0/3

OSPF	routing	table	status	:	<Inactive>

Destinations	:	0								Routes	:	0

The command output shows that the preferences of the OSPF routes have changed.

----End

    - Quiz

OSPF can import two types of AS external routes: Type 1 and Type 2. What are their differences?

    - Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/3

ip address 10.0.123.1 255.255.255.0

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.1.1

default-route-advertise always type 1

preference 20

preference ase 50

bandwidth-reference 10000

area 0.0.0.2

network 10.0.1.1 0.0.0.0

network 10.0.123.1 0.0.0.0

#

ip route-static 0.0.0.0 0.0.0.0 LoopBack0

#

Configuration on R2

#

sysname R2

#

interface GigabitEthernet0/0/2

ip address 10.0.24.2 255.255.255.0

#

interface GigabitEthernet0/0/4

ip address 10.0.123.2 255.255.255.0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.0

ospf network-type broadcast

#

interface LoopBack1

ip address 10.2.0.1 255.255.255.0

#

interface LoopBack2

ip address 10.2.1.1 255.255.255.0

#

ospf 1 router-id 10.0.2.2

asbr-summary 10.2.0.0 255.255.254.0

import-route direct

bandwidth-reference 10000

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.24.2 0.0.0.0

area 0.0.0.2

abr-summary 10.3.0.0 255.255.254.0

network 10.0.123.2 0.0.0.0

#

Configuration on R3

#

sysname R3

#

interface GigabitEthernet0/0/4

ip address 10.0.123.3 255.255.255.0

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.0

ospf network-type broadcast

#

interface LoopBack1

ip address 10.3.0.1 255.255.255.0

ospf network-type broadcast

#

interface LoopBack2

ip address 10.3.1.1 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.3.3

preference 20

preference ase 50

bandwidth-reference 10000

area 0.0.0.2

network 10.0.123.3 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.3.0.1 0.0.0.0

network 10.3.1.1 0.0.0.0

#

Configuration on R4

#

sysname R4

#

interface GigabitEthernet0/0/2

ip address 10.0.45.4 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.24.4 255.255.255.0

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.4.4

bandwidth-reference 10000

area 0.0.0.0

network 10.0.24.4 0.0.0.0

network 10.0.4.4 0.0.0.0

area 0.0.0.1

abr-summary 10.5.0.0 255.255.254.0

network 10.0.45.4 0.0.0.0

#

Configuration on R5

#

sysname R5

#

interface GigabitEthernet0/0/3

ip address 10.0.45.5 255.255.255.0

#

interface LoopBack0

ip address 10.0.5.5 255.255.255.0

ospf network-type broadcast

#

interface LoopBack1

ip address 10.5.0.1 255.255.255.0

ospf network-type broadcast

#

interface LoopBack2

ip address 10.5.1.1 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.5.5

bandwidth-reference 10000

area 0.0.0.0

area 0.0.0.1

network 10.0.5.5 0.0.0.0

network 10.5.0.1 0.0.0.0

network 10.5.1.1 0.0.0.0

network 10.0.45.5 0.0.0.0

#

  - Lab 3: OSPF Adjacencies and LSAs

Introduction

      - Objectives

Upon completion of this task, you will be able to:

Illustrate the process of establishing neighbor relationships when multiple routers are connected to a multi-access network.

Control OSPF DR election.

Describe the contents and functions of the five types of LSAs.

      - Networking Topology

              - OSPF adjacencies and LSAs

The preceding figure shows the device interconnection mode and IP address plan. Loopback0 of R1 belongs to OSPF area 2, GE0/0/2 of R4 belongs to OSPF area 1, and the other interfaces of R1, R2, R3, and R4 belong to OSPF area 0.

GE0/0/3 on R5 belongs to OSPF area 1, and Loopback0 on R5 does not belong to any OSPF area.

      - Background

You are a network administrator of a company. The company's network has five AR routers, among which R1, R2, R3, and R4 reside in the headquarters and are connected through an Ethernet. R5 resides in a branch and is connected to R4 in the headquarters. To control LSA flooding on the large-scale network, multi-area OSPF is designed.

To specify the router ID of each device, the devices are configured to use fixed IP addresses as their router IDs.

On the network where R1, R2, R3, and R4 are interconnected, you need to intervene in the election of the DR and BDR. In practice, R3 is defined as the DR, R2 as the BDR, and R1 and R4 as DR others.

    - Lab Configuration

      - Configuration Roadmap

- Configure IP addresses for the devices.

- Configure multiple OSPF areas as planned.

- Verify the OSPF configuration by checking the OSPF neighbor relationship status, OSPF routing tables, and OSPF LSDBs.

- Manually change the DR priorities of the OSPF interfaces to affect the DR and BDR election results.

- Configure R5 to import the direct route to the OSPF routing table, and observe Type 5 LSAs on R1.

- Observe the Type 1, Type 2, Type 3, and Type 4 LSAs separately.

- Run the debugging commands on R1 to observe the OSPF LSU, LSAck, and LSR packets.

      - Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/1 and Loopback0 of R1.

[R1]interface GigabitEthernet0/0/1

[R1-GigabitEthernet0/0/1] ip address 10.0.123.1 24

[R1-GigabitEthernet0/0/1] quit

[R1]interface LoopBack 0

[R1-LoopBack0] ip address 10.0.1.1 24

[R1-LoopBack0] quit

# Configure IP addresses for GE0/0/1 and Loopback0 of R2.

[R2]interface GigabitEthernet0/0/1

[R2-GigabitEthernet0/0/1] ip address 10.0.123.2 24

[R2-GigabitEthernet0/0/1] quit

[R2]interface LoopBack 0

[R2-LoopBack0] ip address 10.0.2.2 24

[R2-LoopBack0] quit

# Configure IP addresses for GE0/0/1 and Loopback0 of R3.

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] ip address 10.0.123.3 24

[R3-GigabitEthernet0/0/1] quit

[R3]interface LoopBack 0

[R3-LoopBack0] ip address 10.0.3.3 24

[R3-LoopBack0] quit

# Configure IP addresses for GE0/0/1, GE0/0/2, and Loopback0 of R4.

[R4]interface GigabitEthernet0/0/1

[R4-GigabitEthernet0/0/1] ip address 10.0.123.4 24

[R4-GigabitEthernet0/0/1] quit

[R4] interface GigabitEthernet0/0/2

[R4-GigabitEthernet0/0/2] ip address 10.0.45.4 24

[R4-GigabitEthernet0/0/2] quit

[R4]interface LoopBack 0

[R4-LoopBack0] ip address 10.0.4.4 24

[R4-LoopBack0] quit

# Configure IP addresses for GE0/0/3 and Loopback0 of R5.

[R5]interface GigabitEthernet0/0/3

[R5-GigabitEthernet0/0/3] ip address 10.0.45.5 24

[R5-GigabitEthernet0/0/3] quit

[R5]interface LoopBack 0

[R5-LoopBack0] ip address 10.0.5.5 24

[R5-LoopBack0] quit

# On R4, ping the IP addresses of the interconnected devices to test the connectivity.

<R4>ping -c 1 10.0.123.1

PING 10.0.123.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.1: bytes=56 Sequence=1 ttl=255 time=100 ms

--- 10.0.123.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 100/100/100 ms

<R4>ping -c 1 10.0.123.2

PING 10.0.123.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.2: bytes=56 Sequence=1 ttl=255 time=110 ms

--- 10.0.123.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 110/110/110 ms

<R4>ping -c 1 10.0.123.3

PING 10.0.123.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.3: bytes=56 Sequence=1 ttl=255 time=110 ms

--- 10.0.123.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 110/110/110 ms

<R4>ping -c 1 10.0.45.5

PING 10.0.45.5: 56  data bytes, press CTRL_C to break

Reply from 10.0.45.5: bytes=56 Sequence=1 ttl=255 time=60 ms

--- 10.0.45.5 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 60/60/60 ms

Configure multi-area OSPF.

Configure multi-area OSPF as planned and change the network type of Loopback0 to broadcast.

# Configure R1.

[R1]ospf 1 router-id 10.0.1.1

[R1-ospf-1]area 0

[R1-ospf-1-area-0.0.0.0] network 10.0.123.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] quit

[R1-ospf-1]area 2

[R1-ospf-1-area-0.0.0.2] network 10.0.1.1 0.0.0.0

[R1-ospf-1-area-0.0.0.2] quit

[R1-ospf-1]quit

[R1]interface LoopBack 0

[R1-LoopBack0] ospf network-type broadcast

[R1-LoopBack0] quit

# Configure R2.

[R2]ospf 1 router-id 10.0.2.2

[R2-ospf-1]area 0

[R2-ospf-1-area-0.0.0.0] network 10.0.123.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] quit

[R2-ospf-1]quit

[R2]interface LoopBack 0

[R2-LoopBack0] ospf network-type broadcast

[R2-LoopBack0] quit

# Configure R3.

[R3]ospf 1 router-id 10.0.3.3

[R3-ospf-1]area 0

[R3-ospf-1-area-0.0.0.0] network 10.0.123.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] quit

[R3-ospf-1]quit

[R3]interface LoopBack 0

[R3-LoopBack0] ospf network-type broadcast

[R3-LoopBack0] quit

# Configure R4.

[R4]ospf 1 router-id 10.0.4.4

[R4-ospf-1]area 0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.123.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] quit

[R4-ospf-1]area 1

[R4-ospf-1-area-0.0.0.1] network 10.0.45.4 0.0.0.0

[R4-ospf-1-area-0.0.0.1] quit

[R4-ospf-1]quit

[R4]interface LoopBack 0

[R4-LoopBack0] ospf network-type broadcast

[R4-LoopBack0] quit

# Configure R5.

[R5]ospf 1 router-id 10.0.5.5

[R5-ospf-1]area 1

[R5-ospf-1-area-0.0.0.1] network 10.0.45.5 0.0.0.0

[R5-ospf-1-area-0.0.0.1] quit

[R5-ospf-1]quit

Verify the OSPF configuration.

# Check the brief information about OSPF neighbor relationships on R4.

[R4]display ospf peer brief

OSPF Process 1 with Router ID 10.0.4.4

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/1             10.0.1.1         Full

0.0.0.0          GigabitEthernet0/0/1             10.0.2.2         Full

0.0.0.0          GigabitEthernet0/0/1             10.0.3.3         2-Way

0.0.0.1          GigabitEthernet0/0/2             10.0.5.5         Full

----------------------------------------------------------------------------

The command output shows that R3 and R4 have established only a neighbor relationship instead of an adjacency.

# Check the OSPF routing table on R4.

[R4]display ospf routing

OSPF Process 1 with Router ID 10.0.4.4

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter							Area

10.0.4.0/24								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.45.0/24							1					Transit					10.0.45.4							10.0.4.4								0.0.0.1

10.0.123.0/24						1					Transit					10.0.123.4						10.0.4.4								0.0.0.0

10.0.1.0/24								1					Inter-area		10.0.123.1						10.0.1.1								0.0.0.0

10.0.2.0/24								1					Stub							10.0.123.2						10.0.2.2								0.0.0.0

10.0.3.0/24								1					Stub							10.0.123.3						10.0.3.3								0.0.0.0

Total Nets: 6

Intra Area: 5  Inter Area: 1  ASE: 0  NSSA: 0

# Check the OSPF LSDB on R5.

[R5]display	ospf	lsdb

OSPF	Process	1	with	Router	ID	10.0.5.5

Link	State	Database

Area:	0.0.0.1

Type						LinkState	ID						AdvRouter									Age		Len				Sequence					Metric

Router						10.0.5.5								10.0.5.5										470				36				80000008							1

Router						10.0.4.4								10.0.4.4										1660			36				80000005							1

Network					10.0.45.4							10.0.4.4										1660			32				80000002							0

Sum-Net				10.0.3.0								10.0.4.4										1710			28				80000001							1

Sum-Net				10.0.2.0								10.0.4.4										1710			28				80000001							1

Sum-Net				10.0.1.0								10.0.4.4										1710			28				80000001							1

Sum-Net				10.0.4.0								10.0.4.4										1700			28				80000001							0

Sum-Net				10.0.123.0						10.0.4.4										1710			28				80000001							1

As only two routers exist in area 1, only two Type 1 LSAs exist in the LSDB of R5, and the five Type-3 LSAs are inter-area routes advertised by R4 to R5.

# Check the OSPF LSDB on R2.

[R2]display	ospf	lsdb

OSPF	Process	1	with	Router	ID	10.0.2.2

Link	State	Database

Area:	0.0.0.0

Type						LinkState	ID						AdvRouter									Age	Len			Sequence					Metric

Router					10.0.3.3								10.0.3.3											256		48				8000000B							1

Router					10.0.4.4								10.0.4.4											211		48				8000000A							1

Router					10.0.2.2								10.0.2.2											268		48				8000000C							1

Router					10.0.1.1								10.0.1.1											270		36				8000000B							1

Network		10.0.123.1								10.0.1.1											270		40				80000007								0

Sum-Net			10.0.1.0								10.0.1.1											399		28				80000002								0

Sum-Net		10.0.45.0								10.0.4.4											265		28				80000002								1

The LSDB on R2 contains not only four Type 1 LSAs, but also one Type 2 LSA. GE0/0/1 of R2 connects to a broadcast network, on which the DR generates a Type 2 LSA to describe all neighbors. Based on the AdvRouter field, the router that generates the LSA is R1, which matches the result that R1 is the DR on this network segment.

Change the DR priorities of the device interfaces to affect DR election.

# Change the DR priority of GE0/0/1 on R4 to 255 to ensure that R4 becomes the DR on the network segment 10.0.123.0/24.

[R4]interface GigabitEthernet0/0/1

[R4-GigabitEthernet0/0/1] ospf dr-priority 255

[R4-GigabitEthernet0/0/1] quit

# Change the DR priority of GE0/0/1 on R3 to 254 to ensure that R3 becomes the BDR on the network segment 10.0.123.0/24.

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] ospf dr-priority 254

[R3-GigabitEthernet0/0/1] quit

# Change the DR priority of GE0/0/1 on R2 to 0 to ensure that R2 does not participate in DR election.

[R2]interface GigabitEthernet0/0/1

[R2-GigabitEthernet0/0/1] ospf dr-priority 0

[R2-GigabitEthernet0/0/1] quit

# Shut down and then re-enable GE0/0/1 of R1, R2, R3, and R4 to trigger DR and BDR re-election.

[R1]interface GigabitEthernet 0/0/1

[R1-GigabitEthernet0/0/1] shutdown

[R2]interface GigabitEthernet 0/0/1

[R2-GigabitEthernet0/0/1] shutdown

[R3]interface GigabitEthernet 0/0/1

[R3-GigabitEthernet0/0/1] shutdown

[R4]interface GigabitEthernet 0/0/1

[R4-GigabitEthernet0/0/1] shutdown

[R1]interface GigabitEthernet 0/0/1

[R1-GigabitEthernet0/0/1] undo shutdown

[R1-GigabitEthernet0/0/1] quit

[R2]interface GigabitEthernet 0/0/1

[R2-GigabitEthernet0/0/1] undo shutdown

[R2-GigabitEthernet0/0/1] quit

[R3]interface GigabitEthernet 0/0/1

[R3-GigabitEthernet0/0/1] undo shutdown

[R3-GigabitEthernet0/0/1] quit

[R4]interface GigabitEthernet 0/0/1

[R4-GigabitEthernet0/0/1] undo shutdown

[R4-GigabitEthernet0/0/1] quit

To ensure that the election result is determined based on the configured priorities, you are advised to enable the interfaces at the same time. Otherwise, the router whose interface is enabled first may become the DR or BDR.

# Check the DR and BDR election results on R3.

<R3>display ospf peer

OSPF Process 1 with Router ID 10.0.3.3

Neighbors

Area 0.0.0.0 interface 10.0.123.3(GigabitEthernet0/0/1)'s neighbors

Router ID: 10.0.1.1         Address: 10.0.123.1

State: Full  Mode:Nbr is  Slave  Priority: 1

DR: 10.0.123.4  BDR:  10.0.123.3  MTU: 0

Dead timer due in 40  sec

Retrans timer interval: 5

Neighbor is up for 00:59:26

Authentication Sequence: [ 0 ]

Router ID: 10.0.2.2         Address: 10.0.123.2

State: Full  Mode:Nbr is  Slave  Priority: 0

DR: 10.0.123.4  BDR:  10.0.123.3  MTU: 0

Dead timer due in 36  sec

Retrans timer interval: 4

Neighbor is up for 00:59:36

Authentication Sequence: [ 0 ]

Router ID: 10.0.4.4         Address: 10.0.123.4

State: Full  Mode:Nbr is  Master  Priority: 255

DR: 10.0.123.4  BDR:  10.0.123.3  MTU: 0

Dead timer due in 34  sec

Retrans timer interval: 0

Neighbor is up for 00:59:53

Authentication Sequence: [ 0 ]

The command output shows that R4 is the DR and R3 is the BDR.

# Check the neighbor relationship between R1 and R2 on R1.

<R1>display ospf peer brief

OSPF Process 1 with Router ID 10.0.1.1

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/1             10.0.2.2         2-Way

0.0.0.0          GigabitEthernet0/0/1             10.0.3.3         Full

0.0.0.0          GigabitEthernet0/0/1             10.0.4.4         Full

----------------------------------------------------------------------------

R1 and R2 are both DR others, and their neighbor relationship remains in the 2-way state. This means that R1 and R2 has established only a neighbor relationship and no adjacency.

Import direct routes to OSPF.

# Configure R5 to import the Loopback0 route to its OSPF routing table. As previously described, R5's Loopback0 does not belong to any OSPF area.

[R5]ospf 1

[R5-ospf-1] import-route direct

# Check the imported external route on R1.

<R1>display ospf routing

OSPF Process 1 with Router ID 10.0.1.1

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter						Area

10.0.1.0/24								0					Stub							10.0.1.1								10.0.1.1								0.0.0.2

10.0.123.0/24						1					Transit					10.0.123.1						10.0.1.1								0.0.0.0

10.0.2.0/24								1					Stub							10.0.123.2						10.0.2.2								0.0.0.0

10.0.3.0/24								1					Stub							10.0.123.3						10.0.3.3								0.0.0.0

10.0.4.0/24								1					Stub							10.0.123.4						10.0.4.4								0.0.0.0

10.0.45.0/24							2					Inter-area		10.0.123.4						10.0.4.4								0.0.0.0

Routing for ASEs

Destination       Cost      Type       Tag         NextHop       AdvRouter

10.0.5.0/24        1         Type2      1           10.0.123.4      10.0.5.5

Total Nets: 7

Intra Area: 5  Inter Area: 1  ASE: 1  NSSA: 0

The Loopback0 route has been successfully imported to the OSPF routing table as an external route.

# Check Type 5 LSAs on R1.

<R1>display ospf lsdb ase

OSPF Process 1 with Router ID 10.0.1.1

Link State Database

Type														:	External

Ls	id														:	10.0.5.0

Adv	rtr												:	10.0.5.5

Ls	age													:	429

Len															:	36

Options											:	E

seq#														:	80000001

chksum											:	0xa904

Net	mask										:	255.255.255.0

TOS	0		Metric						:	1

E	type													:	2

Forwarding	Address	:	0.0.0.0

Tag														:	1

Priority												:	Low

Type														:	External

Ls	id																:	10.0.45.0

Adv	rtr												:	10.0.5.5

Ls	age													:	429

Len															:	36

Options											:	E

seq#														:	80000001

chksum											:	0xef95

Net	mask										:	255.255.255.0

TOS	0		Metric						:	1

E	type													:	2

Forwarding	Address	:	0.0.0.0

Tag														:	1

Priority												:	Low

The command output shows two Type 5 LSAs, but there is only one external route 10.0.5.0/24 in the OSPF routing table of R1. This is because an inter-area route to the destination 10.0.45.0/24 exists in addition to the AS external route, and the preference of the inter-area route is higher than that of the AS external route.

# Check Type 3 LSAs on R1. (The following command output shows Type 3 LSAs only in area 0.)

<R1>display ospf lsdb summary

OSPF Process 1 with Router ID 10.0.1.1

Area: 0.0.0.0

Link State Database

Type							:	Sum-Net

Ls	id							:	10.0.1.0

Adv	rtr					:	10.0.1.1

Ls	age						:	1487

Len								:	28

Options				:	E

seq#							:	80000003

chksum				:	0x72d1

Net	mask			:	255.255.255.0

Tos	0		metric:	0

Priority					:	Low

Type							:	Sum-Net

Ls	id							:	10.0.45.0

Adv	rtr					:	10.0.4.4

Ls	age						:	1506

Len								:	28

Options				:	E

seq#							:	80000003

chksum				:	0x6fa1

Net	mask			:	255.255.255.0

Tos	0		metric:	1

Priority					:	Low

The command output shows that a Type 3 LSA also describes a route to the destination 10.0.45.0/24. If the route prefixes and masks described in a Type 3 LSA and a Type 5 LSA are the same, OSPF preferentially selects the route calculated using the Type 3 LSA and installs the route into its routing table.

Observe the various types of LSAs.

# Check Type 1 LSAs 10.0.1.1 on R1.

<R1>display ospf lsdb router 10.0.1.1

OSPF Process 1 with Router ID 10.0.1.1

Area: 0.0.0.0

Link State Database

Type										:	Router

Ls	id										:	10.0.1.1

Adv	rtr								:	10.0.1.1

Ls	age									:	202

Len											:	36

Options							:	ABR		E

seq#										:	80000015

chksum							:	0x31e4

Link	count					:	1

*	Link	ID						:	10.0.123.4

Data							:	10.0.123.1

Link	Type			:	TransNet

Metric					:	1

Area	:	0.0.0.2

Link State Database

Type								:	Router

Ls	id								:	10.0.1.1

Adv	rtr						:	10.0.1.1

Ls	age							:	180

Len									:	36

Options					:		ABR		E

seq#								:	80000005

chksum					:	0x1615

Link	count			:	1

*	Link	ID				:	10.0.1.0

Data					:	255.255.255.0

Link	Type	:	StubNet

Metric			:	0

Priority			:	Low

In a Type 1 LSA, the Ls id field indicates the router ID of the router that generates the LSA.

The command output shows that R1 has generated two Type 1 LSAs: one flooded in area 0, and the other flooded in area 2.

In area 0, R1 is connected to a network segment of the transit type. The value of the Link ID field in the LSA is the interface IP address of the DR on the network segment, and the value of the Data field is the IP address of the local interface connected to the DR.

In area 2, R1's Loopback0 belongs to this area. The value of the Link Type field in the LSA is StubNet, the value of the Link ID field is the IP address of the stub network segment, and the value of the Data field is the network mask of the stub network segment.

# Check the Type 2 LSA on R2.

<R2>display ospf lsdb network

OSPF Process 1 with Router ID 10.0.2.2

Area	: 0.0.0.0

Link State Database

Type						:	Network

Ls	id						:	10.0.123.4

Adv	rtr				:	10.0.4.4

Ls	age					:	817

Len							:	40

Options			:	E

seq#						:	80000007

chksum			:	0x373d

Net	mask		:	255.255.255.0

Priority				:	Low

Attached Router    10.0.4.4

Attached Router    10.0.1.1

Attached Router    10.0.2.2

Attached Router    10.0.3.3

The Type 2 LSA is generated by the DR. This can be proved by the Adv rtr field, whose value is 10.0.4.4 (that is, the DR). For a Type 2 LSA, the value of the Ls id field is the interface IP address of the DR on the network segment, and the values of the Attached Router fields are the router IDs of all routers on the network segment.

# Check Type 3 LSAs 10.0.45.0 on R1.

<R1>display ospf lsdb summary 10.0.45.0

OSPF Process 1 with Router ID 10.0.1.1

Area: 0.0.0.0

Link State Database

Type							:	Sum-Net

Ls	id							:	10.0.45.0

Adv	rtr					:	10.0.4.4

Ls	age						:	1290

Len								:	28

Options				:	E

seq#							:	80000004

chksum				:	0x6da2

Net	mask			:	255.255.255.0

Tos	0		metric:	1

Priority					:	Low

Area	:	0.0.0.2

Link State Database

Type							:	Sum-Net

Ls	id							:	10.0.45.0

Adv	rtr					:	10.0.1.1

Ls	age						:	1250

Len								:	28

Options				:	E

seq#							:	80000004

chksum				:	0x9e76

Net	mask			:	255.255.255.0

Tos	0		metric:	2

Priority					:	Low

The Ls id field in a Type 3 LSA indicates a network prefix, and the Net mask field carries the network mask. Two Type 3 LSAs are displayed on R1. One is in the LSDB of area 0. Based on the Adv rtr field, this LSA is generated by R4, which advertises it from area 1 to area 0. The other is in the LSDB of area 2. Based on the Adv rtr field, this LSA is generated by R1 itself. R1 functions as the ABR connecting area 0 and area 2, and generates the Type 3 LSA to advertise it to area 2.

# Check the Type 4 LSAs on R1.

<R1>display ospf lsdb asbr 10.0.5.5

OSPF Process 1 with Router ID 10.0.1.1

Area: 0.0.0.0

Link State Database

Type										:	Sum-Asbr

Ls	id										:	10.0.5.5

Adv	rtr								:	10.0.4.4

Ls	age									:	1257

Len											:	28

Options							:	E

seq#										:	80000002

chksum							:	0xea49

Tos	0		metric			:	1

Area:	0.0.0.2

Link State Database

Type							:	Sum-Asbr

Ls	id							:	10.0.5.5

Adv	rtr					:	10.0.1.1

Ls	age						:	1256

Len								:	28

Options				:	E

seq#							:	80000002

chksum				:	0x1c1d

Tos	0		metric:	2

Type 4 LSAs are used to describe routes to ASBRs. The command output shows that R1 has two Type 4 LSAs. One is in the LSDB of area 0, and is generated by R4 based on the Adv rtr field. The other is generated by R1 itself as the value of the Adv rtr field is R1's own router ID. R1 functions as the ABR connecting area 0 and area 2.

Observe the LSR, LSU, and LSAck packets.

By default, an OSPF router sends LSU packets at the interval of 30 minutes when the network runs stably. To trigger OSPF to send LSR and LSU packets, cancel the OSPF activation on Loopback0 of R4. Then, observe the OSPF packets on R1.

# Run the debugging ospf packet update and debugging ospf packet ack commands on R1.

<R1>terminal debugging

Info: Current terminal debugging is on.

<R1>terminal monitor

Info: Current terminal monitor is on.

<R1>debugging ospf packet update

<R1>debugging ospf packet ack

# Cancel the OSPF activation on Loopback0 of R4.

[R4]ospf 1

[R4-ospf-1]area 0

[R4-ospf-1-area-0.0.0.0] undo network 10.0.4.4 0.0.0.0

# Observe the debugging information on R1.

May 25 2020 20:27:47.210.1-08:00 R1 RM/6/RMDEBUG:

FileID: 0x70178024 Line: 2218 Level: 0x20

OSPF 1: RECV Packet. Interface: GigabitEthernet0/0/1

May 25 2020 20:27:47.210.2-08:00 R1 RM/6/RMDEBUG:  Source Address: 10.0.123.4

May 25 2020 20:27:47.210.3-08:00 R1 RM/6/RMDEBUG:  Destination Address: 224.0.0.5

May 25 2020 20:27:47.210.4-08:00 R1 RM/6/RMDEBUG:  Ver# 2, Type: 4 (Link-State Update)

May 25 2020 20:27:47.210.5-08:00 R1 RM/6/RMDEBUG:  Length: 64, Router: 10.0.4.4

May 25 2020 20:27:47.210.6-08:00 R1 RM/6/RMDEBUG:  Area: 0.0.0.0, Chksum: 5451

May 25 2020 20:27:47.210.7-08:00 R1 RM/6/RMDEBUG:  AuType: 00

May 25 2020 20:27:47.210.8-08:00 R1 RM/6/RMDEBUG:  Key(ascii): 0 0 0 0 0 0 0 0

May 25 2020 20:27:47.210.9-08:00 R1 RM/6/RMDEBUG:  # LSAS: 1

May 25 2020 20:27:47.210.10-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 20:27:47.210.11-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.4.4

May 25 2020 20:27:47.210.12-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 20:27:47.210.13-08:00 R1 RM/6/RMDEBUG:    LSA Age: 1

May 25 2020 20:27:47.210.14-08:00 R1 RM/6/RMDEBUG:    Options: ExRouting:ON

May 25 2020 20:27:47.210.15-08:00 R1 RM/6/RMDEBUG:    Length: 36, Seq# 80000017

May 25 2020 20:27:47.210.16-08:00 R1 RM/6/RMDEBUG:    CheckSum: f014

May 25 2020 20:27:47.210.17-08:00 R1 RM/6/RMDEBUG:    NtBit: 0  VBit: 0 EBit: 0 BBit: 1

May 25 2020 20:27:47.210.18-08:00 R1 RM/6/RMDEBUG:    # Links: 1

May 25 2020 20:27:47.210.19-08:00 R1 RM/6/RMDEBUG:    LinkID: 10.0.123.4

May 25 2020 20:27:47.210.20-08:00 R1 RM/6/RMDEBUG:      LinkData: 10.0.123.4

May 25 2020 20:27:47.210.21-08:00 R1 RM/6/RMDEBUG:      LinkType: 2

May 25 2020 20:27:47.210.22-08:00 R1 RM/6/RMDEBUG:      TOS# 0 Metric 1

May 25 2020 20:27:47.210.23-08:00 R1 RM/6/RMDEBUG:

May 25 2020 20:27:47.570.1-08:00 R1 RM/6/RMDEBUG:

FileID: 0x70178024 Line: 2218 Level: 0x20

OSPF 1: RECV Packet. Interface: GigabitEthernet0/0/1

May 25 2020 20:27:47.570.2-08:00 R1 RM/6/RMDEBUG:  Source Address: 10.0.123.3

May 25 2020 20:27:47.570.3-08:00 R1 RM/6/RMDEBUG:  Destination Address: 224.0.0.5

May 25 2020 20:27:47.570.4-08:00 R1 RM/6/RMDEBUG:  Ver# 2, Type: 5 (Link-State Ack)

May 25 2020 20:27:47.570.5-08:00 R1 RM/6/RMDEBUG:  Length: 44, Router: 10.0.3.3

May 25 2020 20:27:47.570.6-08:00 R1 RM/6/RMDEBUG:  Area: 0.0.0.0, Chksum: 6271

May 25 2020 20:27:47.570.7-08:00 R1 RM/6/RMDEBUG:  AuType: 00

May 25 2020 20:27:47.570.8-08:00 R1 RM/6/RMDEBUG:  Key(ascii): 0 0 0 0 0 0 0 0

May 25 2020 20:27:47.570.9-08:00 R1 RM/6/RMDEBUG:  # LSA Headers: 1

May 25 2020 20:27:47.570.10-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 20:27:47.570.11-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.4.4

May 25 2020 20:27:47.570.12-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 20:27:47.570.13-08:00 R1 RM/6/RMDEBUG:    LSA Age: 1

May 25 2020 20:27:47.570.14-08:00 R1 RM/6/RMDEBUG:    Options: ExRouting:ON

May 25 2020 20:27:47.570.15-08:00 R1 RM/6/RMDEBUG:    Length: 36, Seq# 80000017

May 25 2020 20:27:47.570.16-08:00 R1 RM/6/RMDEBUG:    CheckSum: f014

May 25 2020 20:27:47.570.17-08:00 R1 RM/6/RMDEBUG:

May 25 2020 20:27:47.990.1-08:00 R1 RM/6/RMDEBUG:

FileID: 0x70178025 Line: 4427 Level: 0x20

OSPF 1: SEND Packet. Interface: GigabitEthernet0/0/1

May 25 2020 20:27:47.990.2-08:00 R1 RM/6/RMDEBUG:  Source Address: 10.0.123.1

May 25 2020 20:27:47.990.3-08:00 R1 RM/6/RMDEBUG:  Destination Address: 224.0.0.6

May 25 2020 20:27:47.990.4-08:00 R1 RM/6/RMDEBUG:  Ver# 2, Type: 5 (Link-State Ack)

May 25 2020 20:27:47.990.5-08:00 R1 RM/6/RMDEBUG:  Length: 44, Router: 10.0.1.1

May 25 2020 20:27:47.990.6-08:00 R1 RM/6/RMDEBUG:  Area: 0.0.0.0, Chksum: 6472

May 25 2020 20:27:47.990.7-08:00 R1 RM/6/RMDEBUG:  AuType: 00

May 25 2020 20:27:47.990.8-08:00 R1 RM/6/RMDEBUG:  Key(ascii): 0 0 0 0 0 0 0 0

May 25 2020 20:27:47.990.9-08:00 R1 RM/6/RMDEBUG:  # LSA Headers: 1

May 25 2020 20:27:47.990.10-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 20:27:47.990.11-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.4.4

May 25 2020 20:27:47.990.12-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 20:27:47.990.13-08:00 R1 RM/6/RMDEBUG:    LSA Age: 2

May 25 2020 20:27:47.990.14-08:00 R1 RM/6/RMDEBUG:    Options: ExRouting:ON

May 25 2020 20:27:47.990.15-08:00 R1 RM/6/RMDEBUG:    Length: 36, Seq# 80000017

May 25 2020 20:27:47.990.16-08:00 R1 RM/6/RMDEBUG:    CheckSum: f014

The debugging information shows three packets. The first is an LSU packet, which is sent by R4 (the DR). The destination address of the packet is 224.0.0.5, and the packet contains only one network segment. Therefore, the value of the Links field is 1.

The second is an LSAck packet, which is sent by R3 (the BDR). The destination address of the packet is 224.0.0.5. The third is also an LSAck packet, which is sent by R1 to the DR and BDR. The destination address of the packet is 224.0.0.6.

# Re-activate OSPF on the Loopback0 interface.

[R4]ospf 1

[R4-ospf-1]area 0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

# Observe the debugging information on R1.

May 25 2020 20:39:26.150.1-08:00 R1 RM/6/RMDEBUG:

FileID: 0x70178024 Line: 2218 Level: 0x20

OSPF 1: RECV Packet. Interface: GigabitEthernet0/0/1

May 25 2020 20:39:26.150.2-08:00 R1 RM/6/RMDEBUG:  Source Address: 10.0.123.4

May 25 2020 20:39:26.150.3-08:00 R1 RM/6/RMDEBUG:  Destination Address: 224.0.0.5

May 25 2020 20:39:26.150.4-08:00 R1 RM/6/RMDEBUG:  Ver# 2, Type: 4 (Link-State Update)

May 25 2020 20:39:26.150.5-08:00 R1 RM/6/RMDEBUG:  Length: 76, Router: 10.0.4.4

May 25 2020 20:39:26.150.6-08:00 R1 RM/6/RMDEBUG:  Area: 0.0.0.0, Chksum: c8cf

May 25 2020 20:39:26.150.7-08:00 R1 RM/6/RMDEBUG:  AuType: 00

May 25 2020 20:39:26.150.8-08:00 R1 RM/6/RMDEBUG:  Key(ascii): 0 0 0 0 0 0 0 0

May 25 2020 20:39:26.150.9-08:00 R1 RM/6/RMDEBUG:  # LSAS: 1

May 25 2020 20:39:26.150.10-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 20:39:26.150.11-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.4.4

May 25 2020 20:39:26.150.12-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 20:39:26.150.13-08:00 R1 RM/6/RMDEBUG:    LSA Age: 2

May 25 2020 20:39:26.150.14-08:00 R1 RM/6/RMDEBUG:    Options: ExRouting:ON

May 25 2020 20:39:26.150.15-08:00 R1 RM/6/RMDEBUG:    Length: 48, Seq# 8000001b

May 25 2020 20:39:26.150.16-08:00 R1 RM/6/RMDEBUG:    CheckSum: 6b77

May 25 2020 20:39:26.150.17-08:00 R1 RM/6/RMDEBUG:    NtBit: 0  VBit: 0 EBit: 0 BBit: 1

May 25 2020 20:39:26.150.18-08:00 R1 RM/6/RMDEBUG:    # Links: 2

May 25 2020 20:39:26.150.19-08:00 R1 RM/6/RMDEBUG:    LinkID: 10.0.123.4

May 25 2020 20:39:26.150.20-08:00 R1 RM/6/RMDEBUG:      LinkData: 10.0.123.4

May 25 2020 20:39:26.150.21-08:00 R1 RM/6/RMDEBUG:      LinkType: 2

May 25 2020 20:39:26.150.22-08:00 R1 RM/6/RMDEBUG:      TOS# 0 Metric 1

May 25 2020 20:39:26.150.23-08:00 R1 RM/6/RMDEBUG:    LinkID: 10.0.4.0

May 25 2020 20:39:26.150.24-08:00 R1 RM/6/RMDEBUG:      LinkData: 255.255.255.0

May 25 2020 20:39:26.150.25-08:00 R1 RM/6/RMDEBUG:      LinkType: 3

May 25 2020 20:39:26.150.26-08:00 R1 RM/6/RMDEBUG:      TOS# 0 Metric 0

May 25 2020 20:39:26.150.27-08:00 R1 RM/6/RMDEBUG:

May 25 2020 20:39:26.580.1-08:00 R1 RM/6/RMDEBUG:

FileID: 0x70178024 Line: 2218 Level: 0x20

OSPF 1: RECV Packet. Interface: GigabitEthernet0/0/1

May 25 2020 20:39:26.580.2-08:00 R1 RM/6/RMDEBUG:  Source Address: 10.0.123.3

May 25 2020 20:39:26.580.3-08:00 R1 RM/6/RMDEBUG:  Destination Address: 224.0.0.5

May 25 2020 20:39:26.580.4-08:00 R1 RM/6/RMDEBUG:  Ver# 2, Type: 5 (Link-State Ack)

May 25 2020 20:39:26.580.5-08:00 R1 RM/6/RMDEBUG:  Length: 44, Router: 10.0.3.3

May 25 2020 20:39:26.580.6-08:00 R1 RM/6/RMDEBUG:  Area: 0.0.0.0, Chksum: e6fd

May 25 2020 20:39:26.580.7-08:00 R1 RM/6/RMDEBUG:  AuType: 00

May 25 2020 20:39:26.580.8-08:00 R1 RM/6/RMDEBUG:  Key(ascii): 0 0 0 0 0 0 0 0

May 25 2020 20:39:26.580.9-08:00 R1 RM/6/RMDEBUG:  # LSA Headers: 1

May 25 2020 20:39:26.580.10-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 20:39:26.580.11-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.4.4

May 25 2020 20:39:26.580.12-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 20:39:26.580.13-08:00 R1 RM/6/RMDEBUG:    LSA Age: 2

May 25 2020 20:39:26.580.14-08:00 R1 RM/6/RMDEBUG:    Options: ExRouting:ON

May 25 2020 20:39:26.580.15-08:00 R1 RM/6/RMDEBUG:    Length: 48, Seq# 8000001b

May 25 2020 20:39:26.580.16-08:00 R1 RM/6/RMDEBUG:    CheckSum: 6b77

May 25 2020 20:39:26.580.17-08:00 R1 RM/6/RMDEBUG:

May 25 2020 20:39:26.910.1-08:00 R1 RM/6/RMDEBUG:

FileID: 0x70178025 Line: 4427 Level: 0x20

OSPF 1: SEND Packet. Interface: GigabitEthernet0/0/1

May 25 2020 20:39:26.910.2-08:00 R1 RM/6/RMDEBUG:  Source Address: 10.0.123.1

May 25 2020 20:39:26.910.3-08:00 R1 RM/6/RMDEBUG:  Destination Address: 224.0.0.6

May 25 2020 20:39:26.910.4-08:00 R1 RM/6/RMDEBUG:  Ver# 2, Type: 5 (Link-State Ack)

May 25 2020 20:39:26.910.5-08:00 R1 RM/6/RMDEBUG:  Length: 44, Router: 10.0.1.1

May 25 2020 20:39:26.910.6-08:00 R1 RM/6/RMDEBUG:  Area: 0.0.0.0, Chksum: e8fe

May 25 2020 20:39:26.910.7-08:00 R1 RM/6/RMDEBUG:  AuType: 00

May 25 2020 20:39:26.910.8-08:00 R1 RM/6/RMDEBUG:  Key(ascii): 0 0 0 0 0 0 0 0

May 25 2020 20:39:26.910.9-08:00 R1 RM/6/RMDEBUG:  # LSA Headers: 1

May 25 2020 20:39:26.910.10-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 20:39:26.910.11-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.4.4

May 25 2020 20:39:26.910.12-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 20:39:26.910.13-08:00 R1 RM/6/RMDEBUG:    LSA Age: 3

May 25 2020 20:39:26.910.14-08:00 R1 RM/6/RMDEBUG:    Options: ExRouting:ON

May 25 2020 20:39:26.910.15-08:00 R1 RM/6/RMDEBUG:    Length: 48, Seq# 8000001b

May 25 2020 20:39:26.910.16-08:00 R1 RM/6/RMDEBUG:    CheckSum: 6b77

May 25 2020 20:39:26.910.17-08:00 R1 RM/6/RMDEBUG:

The first remains an LSU packet, which is generated by R4 (the DR). The value of the Links field is 2, indicating that a Loopback0 route is added. The second and third packets remain the same as before: the LSAck packet replied by the BDR and that replied by R1 itself.

# Run the debugging ospf packet request command on R1, and then reset the OSPF process.

<R1>debugging ospf packet request

<R1>reset ospf process 1

# Observe the debugging information on R1.

FileID: 0x70178025 Line: 2886 Level: 0x20

OSPF 1: SEND Packet. Interface: GigabitEthernet0/0/1

May 25 2020 21:18:01.400.2-08:00 R1 RM/6/RMDEBUG:  Source Address: 10.0.123.1

May 25 2020 21:18:01.400.3-08:00 R1 RM/6/RMDEBUG:  Destination Address: 10.0.123

.3

May 25 2020 21:18:01.400.4-08:00 R1 RM/6/RMDEBUG:  Ver# 2, Type: 3 (Link-State Req)

May 25 2020 21:18:01.400.5-08:00 R1 RM/6/RMDEBUG:  Length: 108, Router: 10.0.1.1

May 25 2020 21:18:01.400.6-08:00 R1 RM/6/RMDEBUG:  Area: 0.0.0.0, Chksum: e85a

May 25 2020 21:18:01.400.7-08:00 R1 RM/6/RMDEBUG:  AuType: 00

May 25 2020 21:18:01.400.8-08:00 R1 RM/6/RMDEBUG:  Key(ascii): 0 0 0 0 0 0 0 0

May 25 2020 21:18:01.400.9-08:00 R1 RM/6/RMDEBUG:  # Requesting LSAs: 7

May 25 2020 21:18:01.400.10-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 21:18:01.400.11-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.1.1

May 25 2020 21:18:01.400.12-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.1.1

May 25 2020 21:18:01.400.13-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 21:18:01.400.14-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.2.2

May 25 2020 21:18:01.400.15-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.2.2

May 25 2020 21:18:01.400.16-08:00 R1 RM/6/RMDEBUG:  LSA Type 3

May 25 2020 21:18:01.400.17-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.45.0

May 25 2020 21:18:01.400.18-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 21:18:01.400.19-08:00 R1 RM/6/RMDEBUG:  LSA Type 3

May 25 2020 21:18:01.400.20-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.1.0

May 25 2020 21:18:01.400.21-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.1.1

May 25 2020 21:18:01.400.22-08:00 R1 RM/6/RMDEBUG:  LSA Type 4

May 25 2020 21:18:01.400.23-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.5.5

May 25 2020 21:18:01.400.24-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 21:18:01.400.25-08:00 R1 RM/6/RMDEBUG:  LSA Type 5

May 25 2020 21:18:01.400.26-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.5.0

May 25 2020 21:18:01.400.27-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.5.5

May 25 2020 21:18:01.400.28-08:00 R1 RM/6/RMDEBUG:  LSA Type 5

May 25 2020 21:18:01.400.29-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.45.0

May 25 2020 21:18:01.400.30-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.5.5

May 25 2020 21:18:01.400.31-08:00 R1 RM/6/RMDEBUG:

May 25 2020 21:18:01.430.1-08:00 R1 RM/6/RMDEBUG:

FileID: 0x70178025 Line: 2886 Level: 0x20

OSPF 1: SEND Packet. Interface: GigabitEthernet0/0/1

May 25 2020 21:18:01.430.2-08:00 R1 RM/6/RMDEBUG:  Source Address: 10.0.123.1

May 25 2020 21:18:01.430.3-08:00 R1 RM/6/RMDEBUG:

May 25 2020 21:18:01.430.4-08:00 R1 RM/6/RMDEBUG:  Ver# 2, Type: 3 (Link-State R

eq)

May 25 2020 21:18:01.430.5-08:00 R1 RM/6/RMDEBUG:  Length: 108, Router: 10.0.1.1

May 25 2020 21:18:01.430.6-08:00 R1 RM/6/RMDEBUG:  Area: 0.0.0.0, Chksum: e85a

May 25 2020 21:18:01.430.7-08:00 R1 RM/6/RMDEBUG:  AuType: 00

May 25 2020 21:18:01.430.8-08:00 R1 RM/6/RMDEBUG:  Key(ascii): 0 0 0 0 0 0 0 0

May 25 2020 21:18:01.430.9-08:00 R1 RM/6/RMDEBUG:  # Requesting LSAs: 7

May 25 2020 21:18:01.430.10-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 21:18:01.430.11-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.1.1

May 25 2020 21:18:01.430.12-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.1.1

May 25 2020 21:18:01.430.13-08:00 R1 RM/6/RMDEBUG:  LSA Type 1

May 25 2020 21:18:01.430.14-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.2.2

May 25 2020 21:18:01.430.15-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.2.2

May 25 2020 21:18:01.430.16-08:00 R1 RM/6/RMDEBUG:  LSA Type 3

May 25 2020 21:18:01.430.17-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.1.0

May 25 2020 21:18:01.430.18-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.1.1

May 25 2020 21:18:01.430.19-08:00 R1 RM/6/RMDEBUG:  LSA Type 3

May 25 2020 21:18:01.430.20-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.45.0

May 25 2020 21:18:01.430.21-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 21:18:01.430.22-08:00 R1 RM/6/RMDEBUG:  LSA Type 4

May 25 2020 21:18:01.430.23-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.5.5

May 25 2020 21:18:01.430.24-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.4.4

May 25 2020 21:18:01.430.25-08:00 R1 RM/6/RMDEBUG:  LSA Type 5

May 25 2020 21:18:01.430.26-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.5.0

May 25 2020 21:18:01.430.27-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.5.5

May 25 2020 21:18:01.430.28-08:00 R1 RM/6/RMDEBUG:  LSA Type 5

May 25 2020 21:18:01.430.29-08:00 R1 RM/6/RMDEBUG:    LS ID: 10.0.45.0

May 25 2020 21:18:01.430.30-08:00 R1 RM/6/RMDEBUG:    Adv Rtr: 10.0.5.5

The debugging information shows that R1 has sent LSR packets to R3 (the BDR) and R4 (the DR).

----End

    - Quiz

When does a Type 4 LSA exist, and what is its function?

    - Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/1

ip address 10.0.123.1 255.255.255.0

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.1.1

area 0.0.0.0

network 10.0.123.1 0.0.0.0

area 0.0.0.2

network 10.0.1.1 0.0.0.0

#

Configuration on R2

#

sysname R2

#

interface GigabitEthernet0/0/1

ip address 10.0.123.2 255.255.255.0

ospf dr-priority 0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.123.2 0.0.0.0

network 10.0.2.2 0.0.0.0

#

Configuration on R3

#

sysname R3

#

interface GigabitEthernet0/0/1

ip address 10.0.123.3 255.255.255.0

ospf dr-priority 254

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.123.3 0.0.0.0

network 10.0.3.3 0.0.0.0

#

Configuration on R4

#

sysname R4

#

interface GigabitEthernet0/0/1

ip address 10.0.123.4 255.255.255.0

ospf dr-priority 255

#

interface GigabitEthernet0/0/2

ip address 10.0.45.4 255.255.255.0

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.4.4

area 0.0.0.0

network 10.0.123.4 0.0.0.0

network 10.0.4.4 0.0.0.0

area 0.0.0.1

network 10.0.45.4 0.0.0.0

#

Configuration on R5

#

sysname R5

#

interface GigabitEthernet0/0/3

ip address 10.0.45.5 255.255.255.0

#

interface LoopBack0

ip address 10.0.5.5 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.5.5

import-route direct

area 0.0.0.1

network 10.0.45.5 0.0.0.0

#

  - Lab 4: OSPF Stub Area and NSSA

Introduction

      - Objectives

Upon completion of this task, you will be able to:

Configure an OSPF stub area.

Configure an OSPF NSSA.

Describe the content in a Type 7 LSA.

Describe the process of translating Type 7 LSAs into Type 5 LSAs.

      - Networking Topology

              - OSPF stub area and NSSA

The preceding figure shows the device interconnection mode and IP address plan. The OSPF areas are planned as follows:

- The interconnection interfaces between R1 and R3 and R1's Loopback0 belong to OSPF area 2.

- The interconnection interfaces between R3 and R4 and their Loopback0 interfaces belong to OSPF area 0.

- The interconnection interfaces between R4 and R5 belong to OSPF area 1, and R5's Loopback0 does not belong to any area.

- The interconnection interfaces between R2 and R3 belong to OSPF area 3, and R2's Loopback0 does not belong to any area.

      - Background

You are a network administrator of a company. The company's network has five AR routers, among which R2, R3, and R4 reside in the headquarters. R5 and R1 reside in different branches of the company.

To reduce the pressure on the devices in branches, area 1 is configured as an NSSA and area 2 as a stub area.

To specify the router ID of each device, the devices are configured to use fixed IP addresses as their router IDs.

    - Lab Configuration

      - Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF areas as planned.

- Verify the OSPF configuration by checking the OSPF neighbor relationship status and OSPF routing tables.

- Configure R2 and R5 to import AS external routes to their OSPF routing tables.

- Configure area 2 as a stub area, and observe the changes of the OSPF routing table and LSDB in area 2.

- Configure area 1 as an NSSA, and observe the changes of the OSPF routing table and LSDB in area 1.

- Check the OSPF role of R4, and observe the translation from Type 7 LSAs into Type 5 LSAs on R4.

      - Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/1 and Loopback0 of R1.

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.0

[R1-LoopBack0] quit

[R1]interface GigabitEthernet0/0/1

[R1-GigabitEthernet0/0/1] ip address 10.0.13.1 255.255.255.0

[R1-GigabitEthernet0/0/1] quit

# Configure IP addresses for GE0/0/2 and Loopback0 of R2.

[R2]interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] ip address 10.0.23.2 255.255.255.0

[R2-GigabitEthernet0/0/2] quit

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.0

[R2-LoopBack0] quit

# Configure IP addresses for GE0/0/1, GE0/0/2, GE0/0/3, and Loopback0 of R3.

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.0

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] ip address 10.0.13.3 255.255.255.0

[R3-GigabitEthernet0/0/1] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] ip address 10.0.34.3 255.255.255.0

[R3-GigabitEthernet0/0/2] quit

[R3]interface GigabitEthernet0/0/3

[R3-GigabitEthernet0/0/3] ip address 10.0.23.3 255.255.255.0

[R3-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R4.

[R4]interface LoopBack0

[R4-LoopBack0] ip address 10.0.4.4 255.255.255.0

[R4-LoopBack0] quit

[R4]interface GigabitEthernet0/0/2

[R4-GigabitEthernet0/0/2] ip address 10.0.45.4 255.255.255.0

[R4-GigabitEthernet0/0/2] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] ip address 10.0.34.4 255.255.255.0

[R4-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/3 and Loopback0 of R5.

[R5]interface LoopBack0

[R5-LoopBack0] ip address 10.0.5.5 255.255.255.0

[R5-LoopBack0] quit

[R5]interface GigabitEthernet0/0/3

[R5-GigabitEthernet0/0/3] ip address 10.0.45.5 255.255.255.0

[R5-GigabitEthernet0/0/3] quit

# On R3 and R5, ping the IP addresses of the interconnected devices to test the connectivity.

<R3>ping -c 1 10.0.13.1

PING 10.0.13.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.13.1: bytes=56 Sequence=1 ttl=255 time=40 ms

--- 10.0.13.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/40/40 ms

<R3>ping -c 1 10.0.23.2

PING 10.0.23.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.2: bytes=56 Sequence=1 ttl=255 time=60 ms

--- 10.0.23.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 60/60/60 ms

<R3>ping -c 1 10.0.34.4

PING 10.0.34.4: 56  data bytes, press CTRL_C to break

Reply from 10.0.34.4: bytes=56 Sequence=1 ttl=255 time=60 ms

--- 10.0.34.4 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 60/60/60 ms

<R5>ping -c 1 10.0.45.4

PING 10.0.45.4: 56  data bytes, press CTRL_C to break

Reply from 10.0.45.4: bytes=56 Sequence=1 ttl=255 time=70 ms

--- 10.0.45.4 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 70/70/70 ms

Configure multi-area OSPF.

Configure OSPF as planned. Manually specify the IP address of Loopback0 as the OSPF router ID on each device, and change the network type of Loopback0 to broadcast.

# Configure R1.

[R1] ospf 1 router-id 10.0.1.1

[R1-ospf-1] area 0.0.0.2

[R1-ospf-1-area-0.0.0.2] network 10.0.1.1 0.0.0.0

[R1-ospf-1-area-0.0.0.2] network 10.0.13.1 0.0.0.0

[R1-ospf-1-area-0.0.0.2] quit

[R1-ospf-1] quit

[R1] interface LoopBack0

[R1-LoopBack0] ospf network-type broadcast

# Configure R2.

[R2] ospf 1 router-id 10.0.2.2

[R2-ospf-1] area 0.0.0.3

[R2-ospf-1-area-0.0.0.3] network 10.0.23.2 0.0.0.0

[R2-ospf-1-area-0.0.0.3] quit

[R2-ospf-1] quit

[R2] interface LoopBack0

[R2-LoopBack0] ospf network-type broadcast

# Configure R3.

[R3] ospf 1 router-id 10.0.3.3

[R3-ospf-1] area 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.34.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] area 0.0.0.2

[R3-ospf-1-area-0.0.0.2] network 10.0.13.3 0.0.0.0

[R3-ospf-1-area-0.0.0.2] area 0.0.0.3

[R3-ospf-1-area-0.0.0.3] network 10.0.23.3 0.0.0.0

[R3-ospf-1-area-0.0.0.3] quit

[R3-ospf-1] quit

[R3] interface LoopBack0

[R3-LoopBack0] ospf network-type broadcast

# Configure R4.

[R4] ospf 1 router-id 10.0.4.4

[R4-ospf-1] area 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.34.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] area 0.0.0.1

[R4-ospf-1-area-0.0.0.1] network 10.0.45.4 0.0.0.0

[R4-ospf-1-area-0.0.0.1] quit

[R4-ospf-1] quit

[R4] interface LoopBack0

[R4-LoopBack0] ospf network-type broadcast

# Configure R5.

[R5] ospf 1 router-id 10.0.5.5

[R5-ospf-1] area 1

[R5-ospf-1-area-0.0.0.1] network 10.0.45.5 0.0.0.0

[R5-ospf-1-area-0.0.0.1] quit

[R5-ospf-1] quit

[R5] interface LoopBack0

[R5-LoopBack0] ospf network-type broadcast

Verify the multi-area OSPF configuration.

# Check the brief information about OSPF neighbor relationships on R3.

<R3>display ospf peer brief

OSPF Process 1 with Router ID 10.0.3.3

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/2             10.0.4.4         Full

0.0.0.2          GigabitEthernet0/0/1             10.0.1.1         Full

0.0.0.3          GigabitEthernet0/0/3             10.0.2.2         Full

----------------------------------------------------------------------------

# Check the brief information about OSPF neighbor relationships on R5.

<R5>display ospf peer brief

OSPF Process 1 with Router ID 10.0.5.5

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.1          GigabitEthernet0/0/3             10.0.4.4         Full

----------------------------------------------------------------------------

The command outputs show that the OSPF neighbor relationships between all devices are normal.

# Check the OSPF routing table on R3.

<R3>display ospf routing

OSPF Process 1 with Router ID 10.0.3.3

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter						Area

10.0.3.0/24								0					Stub							10.0.3.3								10.0.3.3								0.0.0.0

10.0.13.0/24							1					Transit					10.0.13.3							10.0.3.3								0.0.0.2

10.0.23.0/24							1					Transit					10.0.23.3							10.0.3.3								0.0.0.3

10.0.34.0/24							1					Transit					10.0.34.3							10.0.3.3								0.0.0.0

10.0.1.0/24								1					Stub							10.0.13.1							10.0.1.1								0.0.0.2

10.0.4.0/24								1					Stub							10.0.34.4							10.0.4.4								0.0.0.0

10.0.45.0/24							2					Inter-area		10.0.34.4							10.0.4.4								0.0.0.0

Total Nets: 7

Intra Area: 6  Inter Area: 1  ASE: 0  NSSA: 0

R3 has learned the routes to all interfaces except R2's Loopback0 and R5's Loopback0 because the two Loopback0 interfaces do not have OSPF activated.

Import AS external routes into the OSPF routing tables.

# Configure R5 to import the Loopback0 route to its OSPF routing table.

[R5] ospf 1

[R5-ospf-1] import-route direct

# Configure a default route on R2, with Loopback0 specified as the outbound interface. Configure R2 to import the default route to its OSPF routing table, with the type of the external route being set to type 1, cost being set to 20, and the always parameter not specified.

[R2] ip route-static 0.0.0.0 0.0.0.0 LoopBack 0

[R2] ospf 1

[R2-ospf-1] default-route-advertise type 1 cost 20

# Check the imported external routes on R3 and test their connectivity.

<R3>display ospf routing 0.0.0.0

OSPF Process 1 with Router ID 10.0.3.3

Destination		: 0.0.0.0/0

AdverRouter	:	10.0.2.2																	Tag						:	1

Cost								:	21																						Type						:	Type1

NextHop				:	10.0.23.2																Interface		:	GigabitEthernet0/0/3

Priority						:	Low																				Age						:	00h01m15s

<R3>display ospf routing 10.0.5.5

OSPF Process 1 with Router ID 10.0.3.3

Destination		:	10.0.5.0/24

AdverRouter	:	10.0.5.5																	Tag						:	1

Cost								:	1																							Type						:	Type2

NextHop				:	10.0.34.4																Interface		:	GigabitEthernet0/0/2

Priority						:	Low																				Age						:	00h05m20s

<R3>ping -c 1 10.0.5.5

PING 10.0.5.5: 56  data bytes, press CTRL_C to break

Reply from 10.0.5.5: bytes=56 Sequence=1 ttl=254 time=50 ms

--- 10.0.5.5 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

<R3>ping -c 1 10.0.2.2

PING 10.0.2.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.2.2: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.2.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

Configure area 2 as a stub area.

# Check the OSPF routing table on R1.

<R1>display ospf routing

OSPF Process 1 with Router ID 10.0.1.1

Routing Tables

Routing for Network

Destination						Cost			Type								NextHop							AdvRouter							Area

10.0.1.0/24							0					Stub								10.0.1.1								10.0.1.1								0.0.0.2

10.0.13.0/24							1					Transit						10.0.13.1							10.0.1.1								0.0.0.2

10.0.3.0/24							1					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.4.0/24							2					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.23.0/24							2					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.34.0/24							2					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.45.0/24							3					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

Routing for ASEs

Destination					Cost		Type								Tag								NextHop							AdvRouter

0.0.0.0/0							22			Type1							1											10.0.13.3							10.0.2.2

10.0.5.0/24							1				Type2							1											10.0.13.3							10.0.5.5

Total Nets: 9

Intra Area: 2  Inter Area: 5  ASE: 2  NSSA: 0

The command output shows that the default route is an OSPF external route.

# Configure area 2 as a stub area on R1 and R3.

[R1] ospf 1

[R1-ospf-1] area 0.0.0.2

[R1-ospf-1-area-0.0.0.2] stub

[R3] ospf 1

[R3-ospf-1] area 0.0.0.2

[R3-ospf-1-area-0.0.0.2] stub

# Check the OSPF routing table on R1 again.

<R1>display ospf routing

OSPF Process 1 with Router ID 10.0.1.1

Routing Tables

Routing for Network

Destination						Cost			Type								NextHop						AdvRouter							Area

10.0.1.0/24							0					Stub								10.0.1.1								10.0.1.1								0.0.0.2

10.0.13.0/24							1					Transit						10.0.13.1							10.0.1.1								0.0.0.2

0.0.0.0/0							2					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.3.0/24							1					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.4.0/24							2					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.23.0/24							2					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.34.0/24							2					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

10.0.45.0/24							3					Inter-area			10.0.13.3							10.0.3.3								0.0.0.2

Total Nets: 8

Intra Area: 2  Inter Area: 6  ASE: 0  NSSA: 0

In this case, R1 does not have any OSPF external route. The original OSPF external routes 0.0.0.0/0 and 10.0.5.0/24 have been replaced by a default OSPF inter-area route.

# Check the OSPF LSDB on R1.

<R1>display	ospf	lsdb

OSPF	Process	1	with	Router	ID	10.0.1.1

Link	State	Database

Area:	0.0.0.2

Type						LinkState	ID					AdvRouter										Age		Len			Sequence					Metric

Router					10.0.3.3								10.0.3.3											628			36				80000004							1

Router					10.0.1.1								10.0.1.1											619			48				80000007							0

Network				10.0.13.1							10.0.1.1											619			32				80000002							0

Sum-Net			0.0.0.0									10.0.3.3											631			28				80000001							1

Sum-Net			10.0.34.0							10.0.3.3											631			28				80000001							1

Sum-Net			10.0.3.0								10.0.3.3											631			28				80000001							0

Sum-Net			10.0.4.0								10.0.3.3											631			28				80000001							1

Sum-Net			10.0.45.0							10.0.3.3											631			28				80000001							2

Sum-Net			10.0.23.0							10.0.3.3											631			28				80000001							1

R1 does not have Type 4 or Type 5 LSAs. The default route carried in the Type 3 LSA generated by the ABR is used to reach a destination outside the OSPF domain. In addition, Type 3 LSAs destined for other areas still exist.

This proves that an ABR blocks the transmission of Type 4 and Type 5 LSAs to the area that has been configured as a stub area and instead floods a default route destined for the ABR itself in this area through a Type 3 LSA.

# Configure area 2 as a totally stubby area on R3.

[R3] ospf 1

[R3-ospf-1] area 0.0.0.2

[R3-ospf-1-area-0.0.0.2] stub no-summary

# Check the OSPF routing table and LSDB on R1 again.

<R1>display ospf routing

OSPF Process 1 with Router ID 10.0.1.1

Routing Tables

Routing for Network

Destination						Cost		Type							NextHop						AdvRouter							Area

10.0.1.0/24							0					Stub							10.0.1.1								10.0.1.1								0.0.0.2

10.0.13.0/24							1					Transit					10.0.13.1							10.0.1.1								0.0.0.2

0.0.0.0/0							2					Inter-area		10.0.13.3							10.0.3.3								0.0.0.2

Total Nets: 3

Intra Area: 2  Inter Area: 1  ASE: 0  NSSA: 0

<R1>display	ospf	lsdb

OSPF	Process	1	with	Router	ID	10.0.1.1

Link	State	Database

Area:	0.0.0.2

Type						LinkState	ID					AdvRouter									Age		Len			Sequence			Metric

Router					10.0.3.3								10.0.3.3										125			36				80000005				1

Router					10.0.1.1								10.0.1.1										121			48				8000000C			0

Network				10.0.13.1							10.0.1.1										121			32				80000002				0

Sum-Net			0.0.0.0									10.0.3.3										961			28				80000001				1

The originally multiple OSPF inter-area routes have been replaced with only one default route 0.0.0.0/0, and the LSDB contains only one Type 3 LSA 0.0.0.0.

This proves that the ABR in a totally stubby area blocks Type 3, Type 4, and Type 5 LSAs and instead generates a Type 3 LSA to advertise a default route destined for the ABR itself.

Configure area 1 as an NSSA.

# Check the OSPF routing table on R4.

<R4>display ospf routing

OSPF Process 1 with Router ID 10.0.4.4

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop						AdvRouter							Area

10.0.4.0/24								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.34.0/24							1					Transit					10.0.34.4							10.0.4.4								0.0.0.0

10.0.45.0/24							1					Transit					10.0.45.4							10.0.4.4								0.0.0.1

10.0.1.0/24								2					Inter-area		10.0.34.3							10.0.3.3								0.0.0.0

10.0.3.0/24								1					Stub							10.0.34.3							10.0.3.3								0.0.0.0

10.0.13.0/24							2					Inter-area		10.0.34.3							10.0.3.3								0.0.0.0

10.0.23.0/24							2					Inter-area		10.0.34.3							10.0.3.3								0.0.0.0

Routing for ASEs

Destination							Cost						Type							Tag								NextHop							AdvRouter

0.0.0.0/0										22								Type1						1											10.0.34.3							10.0.2.2

10.0.5.0/24								1									Type2						1											10.0.45.5							10.0.5.5

Total Nets: 9

Intra Area: 4  Inter Area: 3  ASE: 2  NSSA: 0

The command output shows that R5 has an external route 10.0.5.0/24 described by a Type 5 LSA.

# Check the OSPF routing table on R5.

<R5>display ospf routing

OSPF Process 1 with Router ID 10.0.5.5

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop						AdvRouter							Area

10.0.45.0/24							1					Transit					10.0.45.5							10.0.5.5								0.0.0.1

10.0.1.0/24								3					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.3.0/24								2					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.4.0/24								1					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.13.0/24							3					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.23.0/24							3					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.34.0/24							2					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

Routing for ASEs

Destination							Cost						Type							Tag								NextHop							AdvRouter

0.0.0.0/0										23								Type1						1											10.0.45.4							10.0.2.2

Total Nets: 8

Intra Area: 1  Inter Area: 6  ASE: 1  NSSA: 0

The default route in the OSPF routing table of R5 is described by a Type 5 LSA, which is generated by R2.

# Configure area 1 as an NSSA on R4 and R5.

[R4]ospf 1

[R4-ospf-1] area 0.0.0.1

[R4-ospf-1-area-0.0.0.1] nssa

[R5]ospf 1

[R5-ospf-1] area 0.0.0.1

[R5-ospf-1-area-0.0.0.1] nssa

# Check the OSPF routing table on R5 again.

<R5>display ospf routing

OSPF Process 1 with Router ID 10.0.5.5

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop						AdvRouter							Area

10.0.45.0/24							1					Transit					10.0.45.5							10.0.5.5								0.0.0.1

10.0.1.0/24								3					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.3.0/24								2					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.4.0/24								1					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.13.0/24							3					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.23.0/24							3					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

10.0.34.0/24							2					Inter-area		10.0.45.4							10.0.4.4								0.0.0.1

Routing for NSSAs

Destination        Cost      Type       Tag         NextHop       AdvRouter

0.0.0.0/0           1         Type2      1           10.0.45.4       10.0.4.4

Total Nets: 8

Intra Area: 1  Inter Area: 6  ASE: 0  NSSA: 1

The command output shows that there is no default route advertised by R2. Instead, there is an OSPF default route described by a Type 7 LSA, which is advertised by R4.

# Check the LSDB on R5.

<R5>display	ospf	lsdb

OSPF	Process	1	with	Router	ID	10.0.5.5

Link	State	Database

Area:	0.0.0.1

Type						LinkState	ID						AdvRouter										Age	Len			Sequence				Metric

Router						10.0.5.5								10.0.5.5											100		36				80000005							1

Router						10.0.4.4								10.0.4.4											105		36				80000005							1

Network					10.0.45.5							10.0.5.5											100		32				80000002							0

Sum-Net				10.0.34.0							10.0.4.4											151		28				80000001							1

Sum-Net				10.0.13.0							10.0.4.4											151		28				80000001							2

Sum-Net				10.0.3.0								10.0.4.4											151		28				80000001							1

Sum-Net				10.0.1.0								10.0.4.4											151		28				80000001							2

Sum-Net				10.0.4.0								10.0.4.4											151		28				80000001							0

Sum-Net				10.0.23.0							10.0.4.4											151		28				80000001							2

NSSA							10.0.5.0								10.0.5.5											143		36				80000001							1

NSSA							10.0.45.0							10.0.5.5											143		36				80000002							1

NSSA							0.0.0.0									10.0.4.4											151		36				80000001							1

The command output shows no Type 4 or Type 5 LSAs. Instead, external routes exist in the form of Type 7 LSAs (NSSA-LSAs).

# Check the OSPF routing table on R4.

[R4]display ospf routing

OSPF Process 1 with Router ID 10.0.4.4

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter							Area

10.0.4.0/24								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.34.0/24							1					Transit					10.0.34.4							10.0.4.4								0.0.0.0

10.0.45.0/24							1					Transit					10.0.45.4							10.0.4.4								0.0.0.1

10.0.1.0/24								2					Inter-area		10.0.34.3							10.0.3.3								0.0.0.0

10.0.3.0/24								1					Stub							10.0.34.3							10.0.3.3								0.0.0.0

10.0.13.0/24							2					Inter-area		10.0.34.3							10.0.3.3								0.0.0.0

10.0.23.0/24							2					Inter-area		10.0.34.3							10.0.3.3								0.0.0.0

Routing for ASEs

Destination							Cost						Type							Tag								NextHop							AdvRouter

0.0.0.0/0										22								Type1						1											10.0.34.3							10.0.2.2

Routing for NSSAs

Destination							Cost						Type							Tag								NextHop							AdvRouter

10.0.5.0/24								1									Type2						1											10.0.45.5							10.0.5.5

Total Nets: 9

Intra Area: 4  Inter Area: 3  ASE: 1  NSSA: 1

The external route 10.0.5.0/24 imported by R5 is described in a Type 7 LSA.

This proves that the ABR in the NSSA blocks external Type 4 and Type 5 LSAs from being transmitted to this area and the ABR delivers a default route described by a Type 7 LSA to the NSSA. The ASBR delivers Type 7 LSAs to the NSSA to describe the AS external routes imported to this area.

Observe the impact of the NSSA on OSPF.

# Check the brief OSPF information on R4.

<R4>display ospf brief

OSPF Process 1 with Router ID 10.0.4.4

OSPF Protocol Information

RouterID: 10.0.4.4         Border Router:  AREA  AS  NSSA

Multi-VPN-Instance	is	not	enabled

Global	DS-TE	Mode										:	Non-Standard	IETF	Mode

Spf-schedule-interval								:	max	10000ms,	start	500ms,	hold	1000ms

Default	ASE	parameters					:	Metric:	1	Tag:	1	Type:	2

Route	Preference												:	10

ASE	Route	Preference								:	150

SPF	Computation	Count							:	22

RFC	1583	Compatible

Retransmission	limitation	is	disabled

Area	Count:	2			Nssa	Area	Count	:	1

ExChange/Loading	Neighbors		:	0

Area: 0.0.0.0          (MPLS TE not enabled)

Authtype:	None			Area	flag				:	Normal

SPF	scheduled	Count										:	22

ExChange/Loading	Neighbors		:	0

Router	ID	conflict	state									:	Normal

Interface: 10.0.4.4 (LoopBack0)

Cost: 0       State: DR        Type: Broadcast    MTU: 1500

Priority																				: 1

Designated Router											: 10.0.4.4

Backup Designated Router					: 0.0.0.0

Timers: Hello 10 , Dead 40 , Poll  120 , Retransmit 5 , Transmit Delay 1

Interface: 10.0.34.4 (GigabitEthernet0/0/3)

Cost:	1				State:	BDR		Type	:	Broadcast				MTU:	1500

Priority																					:	1

Designated	Router											:	10.0.34.3

Backup	Designated	Router					:	10.0.34.4

Timers: Hello 10 , Dead 40 , Poll  120 , Retransmit 5 , Transmit Delay 1

Area: 0.0.0.1          (MPLS TE not enabled)

Authtype: None   Area flag			:   NSSA

SPF scheduled Count								: 6

ExChange/Loading Neighbors		: 0

NSSA Translator State								: Elected

Router ID conflict state								: Normal

Interface: 10.0.45.4 (GigabitEthernet0/0/2)

Cost: 1       State: BDR       Type: Broadcast    MTU: 1500

Priority: 1

Designated Router											: 10.0.45.5

Backup Designated Router					: 10.0.45.4

Timers: Hello 10 , Dead 40, Poll 120 , Retransmit 5 , Transmit Delay 1

The Border Router field is displayed as AREA  AS  NSSA, indicating that R4 is both an ABR and an ASBR and has one or more interfaces belonging to the NSSA.

# On R4, observe the process of translating Type 7 LSAs into Type 5 LSAs. The following uses the LSA 10.0.5.0/24 as an example to describe how routing information is transmitted.

<R4>display ospf lsdb nssa 10.0.5.0

OSPF Process 1 with Router ID 10.0.4.4

Area:	0.0.0.0

Link	State	Database

Area:	0.0.0.1

Link	State	Database

Type								:	NSSA

Ls	id								:	10.0.5.0

Adv	rtr						:	10.0.5.5

Ls	age							:	587

Len									:	36

Options					:	NP

seq#								:	80000001

chksum					:	0x3336

Net	mask				:	255.255.255.0

TOS	0		Metric:	1

E	type							:	2

Forwarding	Address	:	10.0.45.5

Tag								:	1

Priority						:	Low

In the Type 7 LSA that describes the route 10.0.5.0/24, the value of the Options field is NP, indicating that the LSA can be translated into a Type 5 LSA by the ABR.

# Check the Type 5 LSA generated on R4 to describe the route 10.0.5.0/24.

<R4>display ospf lsdb ase 10.0.5.0

OSPF Process 1 with Router ID 10.0.4.4

Link State Database

Type								:	External

Ls	id								:	10.0.5.0

Adv	rtr						:	10.0.4.4

Ls	age							:	753

Len									:	36

Options					:	E

seq#								:	80000001

chksum					:	0xb6bc

Net	mask				:	255.255.255.0

TOS	0		Metric:	1

E	type							:	2

Forwarding	Address	:	10.0.45.5

Tag								:	1

Priority						:	Low

The Type 5 LSA carries the same Ls id, Net mask, and Forwarding Address fields as those in the Type 7 LSA. However, the value of the Adv rtr field is changed from 10.0.5.5 to 10.0.4.4, indicating that the Type 5 LSA is generated by R4.

----End

    - Quiz

In which scenarios is an NSSA applicable?

    - Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/1

ip address 10.0.13.1 255.255.255.0

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.1.1

area 0.0.0.2

network 10.0.1.1 0.0.0.0

network 10.0.13.1 0.0.0.0

stub

#

Configuration on R2

#

sysname R2

#

interface GigabitEthernet0/0/2

ip address 10.0.23.2 255.255.255.0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.2.2

default-route-advertise cost 20 type 1

area 0.0.0.3

network 10.0.23.2 0.0.0.0

#

ip route-static 0.0.0.0 0.0.0.0 LoopBack0

#

Configuration on R3

#

sysname R3

#

interface GigabitEthernet0/0/1

ip address 10.0.13.3 255.255.255.0

#

interface GigabitEthernet0/0/2

ip address 10.0.34.3 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.23.3 255.255.255.0

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.34.3 0.0.0.0

area 0.0.0.2

network 10.0.13.3 0.0.0.0

stub no-summary

area 0.0.0.3

network 10.0.23.3 0.0.0.0

#

Configuration on R4

#

sysname R4

#

interface GigabitEthernet0/0/2

ip address 10.0.45.4 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.34.4 255.255.255.0

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.4.4

area 0.0.0.0

network 10.0.4.4 0.0.0.0

network 10.0.34.4 0.0.0.0

area 0.0.0.1

network 10.0.45.4 0.0.0.0

nssa

#

Configuration on R5

#

sysname R5

#

interface GigabitEthernet0/0/3

ip address 10.0.45.5 255.255.255.0

#

interface LoopBack0

ip address 10.0.5.5 255.255.255.0

ospf network-type broadcast

#

ospf 1 router-id 10.0.5.5

import-route direct

area 0.0.0.1

network 10.0.45.5 0.0.0.0

nssa

#

IS-IS Basics Experiment

IS-IS Configuration Experiment

Introduction

Objectives

Upon completion of this task, you will be able to:

Perform basic IS-IS configurations.

Change the IS-IS DIS priority.

Change the IS-IS network type.

Import external routes to IS-IS.

Change the IS-IS interface cost.

Configure IS-IS route leaking.

Networking Topology

              - IS-IS topology

The preceding figure shows the IP addresses, IS-IS areas, and IS-IS router levels. R1, R2, and R3 belong to area 49.0001, and R4 and R5 belong to area 49.0002. Loopback0 interfaces are created on all routers, and their IP addresses are in the format of 10.0.x.x/32, where x indicates the device ID.

Background

A customer's network uses IS-IS as an IGP. R4 and R5 are Level-2 routers and run in area 49.0002. R1, R2, and R3 belong to area 49.0001. R1 is a Level-1 router, whereas R2 and R3 are Level-1-2 routers. R5 imports an external route 192.168.1.0/24.

Requirements: R1 can access the destination of the external route imported by R5. GE0/0/1 of R1 functions as the DIS. Bidirectional traffic between R1 and R5 is forwarded along the path between R3 and R4. You can control the route selection result by changing the cost or configuring route leaking as required.

Lab Configuration

Configuration Roadmap

- Configure IP addresses for the devices.

- Configure IS-IS as planned.

- Check IS-IS configurations and IS-IS neighbor information on R1 and R4.

- Manually change the DIS priority of R1's GE0/0/1 so that R1 becomes the DIS.

- Create Loopback1 on R5 and import Loopback1's route as an external route to IS-IS. Check the IS-IS routing tables on R4 and R1, and test the connectivity between R1 and the destination address of the external route.

- Manually change the IS-IS cost of GE0/0/3 on R4 so that R4 preferentially selects the route with R2 as the next hop to R1.

- Configure IS-IS route leaking on R3 so that R1 learns specific routes in the Level-2 area from R3. Based on the longest match rule, R1 preferentially selects the specific route with the next hop being R3 to the Level-2 area.

Configuration Procedure

          - Configure IP addresses for the interconnection and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/1 and Loopback0 of R1.

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.255

[R1-LoopBack0] quit

[R1]interface GigabitEthernet0/0/1

[R1-GigabitEthernet0/0/1] ip address 10.0.123.1 255.255.255.0

[R1-GigabitEthernet0/0/1] quit

# Configure IP addresses for GE0/0/1, GE0/0/5, and Loopback0 of R2.

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.255

[R2-LoopBack0] quit

[R2]interface GigabitEthernet0/0/1

[R2-GigabitEthernet0/0/1] ip address 10.0.123.2 255.255.255.0

[R2-GigabitEthernet0/0/1] quit

[R2]interface GigabitEthernet0/0/5

[R2-GigabitEthernet0/0/5] ip address 10.0.24.2 255.255.255.0

[R2-GigabitEthernet0/0/5] quit

# Configure IP addresses for GE0/0/1, GE0/0/2, and Loopback0 of R3.

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.255

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] ip address 10.0.123.3 255.255.255.0

[R3-GigabitEthernet0/0/1] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] ip address 10.0.34.3 255.255.255.0

[R3-GigabitEthernet0/0/2] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, GE0/0/5, and Loopback0 of R4.

[R4]interface LoopBack0

[R4-LoopBack0] ip address 10.0.4.4 255.255.255.255

[R4-LoopBack0] quit

[R4]interface GigabitEthernet0/0/2

[R4-GigabitEthernet0/0/2] ip address 10.0.45.4 255.255.255.0

[R4-GigabitEthernet0/0/2] quit

[R4]interface GigabitEthernet0/0/5

[R4-GigabitEthernet0/0/5] ip address 10.0.24.4 255.255.255.0

[R4-GigabitEthernet0/0/5] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] ip address 10.0.34.4 255.255.255.0

[R4-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/3 and Loopback0 of R5.

[R5]interface LoopBack0

[R5-LoopBack0] ip address 10.0.5.5 255.255.255.255

[R5-LoopBack0] quit

[R5]interface GigabitEthernet0/0/3

[R5-GigabitEthernet0/0/3] ip address 10.0.45.5 255.255.255.0

[R5-GigabitEthernet0/0/3] quit

# On R1 and R4, ping the IP addresses of the interconnected devices to test the connectivity.

<R1>ping -c 1 10.0.123.2

PING 10.0.123.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.2: bytes=56 Sequence=1 ttl=255 time=90 ms

--- 10.0.123.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 90/90/90 ms

<R1>ping -c 1 10.0.123.3

PING 10.0.123.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.123.3: bytes=56 Sequence=1 ttl=255 time=140 ms

--- 10.0.123.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 140/140/140 ms

<R4>ping -c 1 10.0.24.2

PING 10.0.24.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.24.2: bytes=56 Sequence=1 ttl=255 time=70 ms

--- 10.0.24.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 70/70/70 ms

<R4>ping -c 1 10.0.34.3

PING 10.0.34.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.34.3: bytes=56 Sequence=1 ttl=255 time=60 ms

--- 10.0.34.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 60/60/60 ms

<R4>ping -c 1 10.0.45.5

PING 10.0.45.5: 56  data bytes, press CTRL_C to break

Reply from 10.0.45.5: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.45.5 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

          - Configure IS-IS.

Configure IS-IS process 1 on each router, and use the device ID of each device when setting NETs. For example, set the NET of R1 to 49.0001.0000.0000.0001.00.

# Configure R1.

[R1]isis 1

[R1-isis-1] is-level level-1

[R1-isis-1] network-entity 49.0001.0000.0000.0001.00

[R1-isis-1] quit

[R1]interface LoopBack0

[R1-LoopBack0] isis enable 1

[R1-LoopBack0] quit

[R1]interface GigabitEthernet0/0/1

[R1-GigabitEthernet0/0/1] isis enable 1

[R1-GigabitEthernet0/0/1] quit

# Configure R2.

[R2]isis 1

[R2-isis-1] network-entity 49.0001.0000.0000.0002.00

[R2-isis-1] quit

[R2]interface LoopBack0

[R2-LoopBack0] isis enable 1

[R2-LoopBack0] quit

[R2]interface GigabitEthernet0/0/1

[R2-GigabitEthernet0/0/1] isis enable 1

[R2-GigabitEthernet0/0/1] quit

[R2]interface GigabitEthernet0/0/5

[R2-GigabitEthernet0/0/5] isis enable 1

[R2-GigabitEthernet0/0/5] quit

# Configure R3.

[R3]isis

[R3-isis-1] network-entity 49.0001.0000.0000.0003.00

[R3-isis-1] quit

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.255

[R3-LoopBack0] isis enable 1

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] isis enable 1

[R3-GigabitEthernet0/0/1] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] isis enable 1

[R3-GigabitEthernet0/0/2] quit

# Configure R4.

[R4]isis 1

[R4-isis-1] is-level level-2

[R4-isis-1] network-entity 49.0002.0000.0000.0004.00

[R4-isis-1] quit

[R4]interface LoopBack0

[R4-LoopBack0] isis enable 1

[R4-LoopBack0] quit

[R4]interface GigabitEthernet0/0/2

[R4-GigabitEthernet0/0/2] isis enable 1

[R4-GigabitEthernet0/0/2] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] isis enable 1

[R4-GigabitEthernet0/0/3] quit

[R4]interface GigabitEthernet0/0/5

[R4-GigabitEthernet0/0/5] isis enable 1

[R4-GigabitEthernet0/0/5] quit

# Configure R5.

[R5]isis 1

[R5-isis-1] is-level level-2

[R5-isis-1] network-entity 49.0002.0000.0000.0005.00

[R5-isis-1] quit

[R5]interface LoopBack0

[R5-LoopBack0] isis enable 1

[R5-LoopBack0] quit

[R5]interface GigabitEthernet0/0/3

[R5-GigabitEthernet0/0/3] isis enable 1

[R5-GigabitEthernet0/0/3] quit

# To ensure security, configure IS-IS interface authentication, with the authentication mode being MD5, and the password being huawei.

[R1]interface GigabitEthernet0/0/1

[R1-GigabitEthernet0/0/1] isis authentication-mode md5 huawei

[R1-GigabitEthernet0/0/1] quit

[R2]interface GigabitEthernet0/0/1

[R2-GigabitEthernet0/0/1] isis authentication-mode md5 huawei

[R2-GigabitEthernet0/0/1] quit

[R2]interface GigabitEthernet0/0/5

[R2-GigabitEthernet0/0/5] isis authentication-mode md5 huawei

[R2-GigabitEthernet0/0/5] quit

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] isis authentication-mode md5 huawei

[R3-GigabitEthernet0/0/1] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] isis authentication-mode md5 huawei

[R3-GigabitEthernet0/0/2] quit

[R4]interface GigabitEthernet0/0/2

[R4-GigabitEthernet0/0/2] isis authentication-mode md5 huawei

[R4-GigabitEthernet0/0/2] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] isis authentication-mode md5 huawei

[R4-GigabitEthernet0/0/3] quit

[R4]interface GigabitEthernet0/0/5

[R4-GigabitEthernet0/0/5] isis authentication-mode md5 huawei

[R4-GigabitEthernet0/0/5] quit

[R5]interface GigabitEthernet0/0/3

[R5-GigabitEthernet0/0/3] isis authentication-mode md5 huawei

[R5-GigabitEthernet0/0/3] quit

          - Check IS-IS configurations.

# Check the IS-IS neighbor relationships on R1 and R4.

<R1>display	isis	peer

Peer	information	for	ISIS(1)

System	Id						Interface											Circuit	Id								State		HoldTime		Type					PRI

-------------------------------------------------------------------------------

0000.0000.0002		GE0/0/1												0000.0000.0002.01	Up			8s									L1							64

0000.0000.0003		GE0/0/1												0000.0000.0002.01	Up			29s								L1							64

Total	Peer(s):	2

According to the preceding command output, R1 has established Level-1 IS-IS neighbor relationships with R2 and R3.

<R4>display	isis	peer

Peer	information	for	ISIS(1)

System	Id						Interface											Circuit	Id								State		HoldTime		Type					PRI

-------------------------------------------------------------------------------

0000.0000.0005		GE0/0/2												0000.0000.0004.01	Up			24s								L2							64

0000.0000.0003		GE0/0/3												0000.0000.0004.02	Up			27s								L2							64

0000.0000.0002		GE0/0/5												0000.0000.0004.03	Up			23s								L2							64

Total	Peer(s):	3

According to the preceding command output, R4 has established Level-2 IS-IS neighbor relationships with R2, R3, and R5.

# Check the IS-IS routing table on R4.

<R4>display	isis	route

Route	information	for	ISIS(1)

-----------------------------

ISIS(1)	Level-2	Forwarding	Table

--------------------------------

IPV4	Destination					IntCost						ExtCost			ExitInterface					NextHop							Flags

-------------------------------------------------------------------------------

10.0.24.0/24									10									NULL				GE0/0/5									Direct										D/-/L/-

10.0.3.3/32										10									NULL				GE0/0/3									10.0.34.3							A/-/-/-

10.0.2.2/32										10									NULL				GE0/0/5									10.0.24.2							A/-/-/-

10.0.5.5/32										10									NULL				GE0/0/2									10.0.45.5							A/-/-/-

10.0.123.0/24								20									NULL				GE0/0/3									10.0.34.3							A/-/-/-

GE0/0/5									10.0.24.2

10.0.45.0/24									10									NULL				GE0/0/2									Direct										D/-/L/-

10.0.1.1/32										20									NULL				GE0/0/3									10.0.34.3							A/-/-/-

GE0/0/5									10.0.24.2

10.0.4.4/32											0									NULL				Loop0											Direct										D/-/L/-

10.0.34.0/24									10									NULL				GE0/0/3									Direct										D/-/L/-

Flags: 	D-Direct,	A-Added	to	URT,	L-Advertised	in	LSPs,	S-IGP	Shortcut,U-Up/Down	Bit	Set

According to the preceding command output, R4 has learned the routes on the entire network, the routes to 10.0.123.0/24 and 10.0.1.1/32 are in the load balancing state.

          - Change the DIS priority of GE0/0/1 on R1.

Change the DIS priority of GE0/0/1 on R1 so that R1 is elected as the DIS among R1, R2, and R3 that are on the same broadcast network.

# Check the IS-IS interface status on R1.

<R1>display	isis	interface

Interface	information	for	ISIS(1)

---------------------------------

Interface		Id						IPV4.State														IPV6.State					MTU		Type					DIS

Loop0				001								Up																			Down								1500			L1/L2			--

GE0/0/1		001								Up																			Down								1497			L1/L2			No/No

According to the preceding command output, GE0/0/1 on R1 is not the DIS.

# Change the DIS priority of GE0/0/1 on R1.

[R1]interface GigabitEthernet0/0/1

[R1-GigabitEthernet0/0/1] isis dis-priority 127

# Check the IS-IS interface status on R1.

<R1>display	isis	interface

Interface	information	for	ISIS(1)

---------------------------------

Interface		Id								IPV4.State										IPV6.State					MTU		Type				DIS

Loop0					001								Up															Down									1500		L1/L2			--

GE0/0/1			001								Up															Down									1497		L1/L2			Yes/No

According to the preceding command output, GE0/0/1 on R1 becomes the DIS.

          - Import the external route.

# Create Loopback1 on R5, set the IP address to 192.168.1.1, and import the route 192.168.1.1 as an external route to IS-IS.

[R5]interface LoopBack 1

[R5-LoopBack1] ip address 192.168.1.1 32

[R5-LoopBack1] quit

[R5]isis 1

[R5-isis-1] import-route direct

[R5-isis-1] quit

# Check the IS-IS routing table on R5.

<R5>display	isis	route

Route	information	for	ISIS(1)

-----------------------------

ISIS(1)	Level-2	Forwarding	Table

--------------------------------

IPV4	Destination					IntCost						ExtCost			ExitInterface					NextHop								Flags

-------------------------------------------------------------------------------

10.0.24.0/24									20									NULL				GE0/0/3									10.0.45.4							A/-/-/-

10.0.3.3/32										20									NULL				GE0/0/3									10.0.45.4							A/-/-/-

10.0.2.2/32										20									NULL				GE0/0/3									10.0.45.4							A/-/-/-

10.0.5.5/32											0									NULL				Loop0											Direct										D/-/L/-

10.0.123.0/24								30									NULL				GE0/0/3									10.0.45.4							A/-/-/-

10.0.45.0/24									10									NULL				GE0/0/3									Direct										D/-/L/-

10.0.1.1/32										30									NULL				GE0/0/3									10.0.45.4							A/-/-/-

10.0.4.4/32										10									NULL				GE0/0/3									10.0.45.4							A/-/-/-

10.0.34.0/24									20									NULL				GE0/0/3									10.0.45.4							A/-/-/-

Flags: 	D-Direct,	A-Added	to	URT,	L-Advertised	in	LSPs,	S-IGP	Shortcut,U-Up/Down	Bit	Set

ISIS(1) Level-2 Redistribute Table

----------------------------------

Type IPV4 Destination     IntCost    ExtCost Tag

-------------------------------------------------------------------------------

D    192.168.1.1/32       0          0

Type: D-Direct, I-ISIS, S-Static, O-OSPF, B-BGP, R-RIP, U-UNR

According to the preceding command output, the imported external route is displayed in the routing table.

# Check the IS-IS route 192.168.1.1 on R4.

<R4>display	isis	route	192.168.1.1

Route	information	for	ISIS(1)

-----------------------------

ISIS(1)	Level-2	Forwarding	Table

--------------------------------

IPV4	Destination					IntCost						ExtCost		ExitInterface					NextHop							Flags

-------------------------------------------------------------------------------

192.168.1.1/32							10									0							GE0/0/2									10.0.45.5							A/-/-/-

Flags: D-Direct, A-Added to URT, L-Advertised in LSPs, S-IGP Shortcut,U-Up/Down Bit Set

According to the preceding command output, R4 has learned the IS-IS route 192.168.1.1/32.

# Check the IS-IS routing table on R1.

<R1>display	isis	route

Route	information	for	ISIS(1)

-----------------------------

ISIS(1)	Level-1	Forwarding	Table

--------------------------------

IPV4	Destination					IntCost						ExtCost			ExitInterface					NextHop							Flags

-------------------------------------------------------------------------------

0.0.0.0/0												10									NULL				GE0/0/1									10.0.123.3						A/-/-/-

GE0/0/1									10.0.123.2

10.0.24.0/24									20									NULL				GE0/0/1									10.0.123.2						A/-/-/-

10.0.3.3/32										10									NULL				GE0/0/1									10.0.123.3						A/-/-/-

10.0.2.2/32										10									NULL				GE0/0/1									10.0.123.2						A/-/-/-

10.0.123.0/24								10									NULL				GE0/0/1									Direct										D/-/L/-

10.0.1.1/32											0									NULL				Loop0											Direct										D/-/L/-

10.0.34.0/24									20									NULL				GE0/0/1									10.0.123.3						A/-/-/-

Flags: 	D-Direct,	A-Added	to	URT,	L-Advertised	in	LSPs,	S-IGP	Shortcut,U-Up/Down	Bit	Set

According to the preceding command output, the IS-IS routing table on R1 does not contain the route 192.168.1.1/32 because Level-1-2 routers do not leak Level-2 routes to Level-1 routers by default. Therefore, R1 does not have the imported external route to 192.168.1.1/32. However, R1 has two default routes to the backbone area, and the two routes are in the load balancing state.

# On R1, ping R5's Loopback1.

<R1>ping -c 1 192.168.1.1

PING 192.168.1.1: 56  data bytes, press CTRL_C to break

Reply from 192.168.1.1: bytes=56 Sequence=1 ttl=253 time=90 ms

--- 192.168.1.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 90/90/90 ms

According to the preceding command output, R1 can communicate with Loopback1 of R5.

          - Change the cost of an IS-IS interface.

The traffic from R4 to R1 is balanced by R2 and R3 (the conclusion can be drawn according to the routing table). To enable the traffic from R4 to R1 to pass through R2, change the interface cost on R4.

# Check the IS-IS route 10.0.1.1/32 on R4.

<R4>display	isis	route	10.0.1.1

Route	information	for	ISIS(1)

-----------------------------

ISIS(1)	Level-2	Forwarding	Table

--------------------------------

IPV4	Destination					IntCost						ExtCost			ExitInterface					NextHop							Flags

-------------------------------------------------------------------------------

10.0.1.1/32										20									NULL				GE0/0/5									10.0.24.2							A/-/-/-

GE0/0/3									10.0.34.3

Flags: 	D-Direct,	A-Added	to	URT,	L-Advertised	in	LSPs,	S-IGP	Shortcut,U-Up/Down	Bit	Set

The routes from R4 to Loopback0 of R1 work in load balancing mode, and the next hops are 10.0.24.2 and 10.0.34.3.

# Change the IS-IS cost of GE0/0/3 on R4.

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] isis cost 15

# Check the IS-IS route 10.0.1.1/32 on R4 again.

<R4>display isis route 10.0.1.1 32

Route information for ISIS(1)

-----------------------------

ISIS(1) Level-2 Forwarding Table

--------------------------------

IPV4 Destination     IntCost    	ExtCost			 ExitInterface   	NextHop         Flags

-------------------------------------------------------------------------------

10.0.1.1/32          20         NULL    GE0/0/5        10.0.24.2       A/-/-/-

Flags: D-Direct, A-Added to URT, L-Advertised in LSPs, S-IGP Shortcut,

U-Up/Down Bit Set

According to the preceding command output, there is only one next hop (10.0.24.2) from R4 to Loopback0 of R1.

          - Configure IS-IS route leaking.

By default, R1 does not have specific routes to the Level-2 area and forwards packets to the Level-2 area only through the default routes advertised by Level-1-2 routers. In this example, R1 uses R2 and R3 as equal-cost next hops to reach the Level-2 area. To divert the traffic sent from R1 to R5 to R3, you can configure route leaking on R3 so that R3 can leak the routes destined for the Level-2 area to the Level-1 area. In this way, R1 can learn desired routes through IS-IS.

# Check the route to Loopback0 on R5 in the IP routing table of R1.

<R1>display ip routing-table 10.0.5.5

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing	Table			:	Public

Summary	Count	:	2

Destination/Mask			Proto		Pre		Cost						Flags		NextHop									Interface

0.0.0.0/0							ISIS-L1	15			10										D			10.0.123.3						GigabitEthernet0/0/1

ISIS-L1	15			10										D			10.0.123.2						GigabitEthernet0/0/1

According to the preceding command output, traffic from R1 to 10.0.5.5 is balanced between R2 and R3.

# Configure IS-IS route leaking on R3.

[R3]isis 1

[R3-isis-1] import-route isis level-2 into level-1

[R3-isis-1] quit

# Check the route to Loopback0 on R5 in the IP routing table of R1 again.

<R1>display ip routing-table 10.0.5.5

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing	Table		:	Public

Summary	Count	:	1

Destination/Mask				Proto			Pre		Cost						Flags			NextHop									Interface

10.0.5.5/32							ISIS-L1		15			30									D					10.0.123.3						GigabitEthernet0/0/1

According to the preceding command output, the next hop of the route from R1 to 10.0.5.5 is 10.0.123.3, that is, R3. In addition, this route is a specific route rather than a default route.

----End

Quiz

What are the conditions for establishing an IS-IS neighbor relationship between Ethernet interfaces?

Configuration Reference

Configuration on R1

#

sysname R1

#

isis 1

is-level level-1

network-entity 49.0001.0000.0000.0001.00

#

interface GigabitEthernet0/0/1

ip address 10.0.123.1 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

isis dis-priority 127

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

isis enable 1

#

return

Configuration on R2

sysname R2

#

isis 1

network-entity 49.0001.0000.0000.0002.00

#

interface GigabitEthernet0/0/1

ip address 10.0.123.2 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

#

interface GigabitEthernet0/0/5

ip address 10.0.24.2 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

isis enable 1

#

Configuration on R3

#

sysname R3

#

isis 1

network-entity 49.0001.0000.0000.0003.00

import-route isis level-2 into level-1

#

interface GigabitEthernet0/0/1

ip address 10.0.123.3 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

#

interface GigabitEthernet0/0/2

ip address 10.0.34.3 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

isis enable 1

#

Configuration on R4

#

sysname R4

#

isis 1

is-level level-2

network-entity 49.0002.0000.0000.0004.00

#

interface GigabitEthernet0/0/2

ip address 10.0.45.4 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

#

interface GigabitEthernet0/0/3

ip address 10.0.34.4 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

isis cost 15

#

interface GigabitEthernet0/0/5

ip address 10.0.24.4 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.255

isis enable 1

#

Configuration on R5

#

sysname R5

#

isis 1

is-level level-2

network-entity 49.0002.0000.0000.0005.00

import-route direct

#

interface GigabitEthernet0/0/3

ip address 10.0.45.5 255.255.255.0

isis enable 1

isis authentication-mode md5 huawei

#

interface LoopBack0

ip address 10.0.5.5 255.255.255.255

isis enable 1

isis authentication-mode md5 huawei

#

interface LoopBack1

ip address 192.168.1.1 255.255.255.255

#

- BGP Configurations

  - Lab 1: Basic BGP Configurations

Introduction

Upon completion of this task, you will be able to:

Configure IBGP.

Configure EBGP.

Observe the BGP peer table.

Specify the source interface for sending BGP messages.

Configure EBGP multi-hop.

Observe the changes in the next hops of IBGP and EBGP routes.

      - Networking Topology

              - Basic BGP configurations

The preceding figure shows the device interconnection mode, IP address plan, and BGP AS numbers. Loopback0 is created on each device, and its IP address is in the format of 10.0.x.x/32, where x indicates the device number. The IP address of Loopback0 on each device is used as the BGP router ID of the device. Loopback1 is configured on R1 and R5 to simulate a user network segment.

OSPF runs on R2, R3, and R4, and is activated on the interconnection and Loopback0 interfaces of R2, R3, and R4.

      - Background

You are a network administrator of a company. The company's network uses BGP as the routing protocol. The network consists of multiple ASs, with different branches using different AS numbers. Now, you need to complete the establishment of the company's network. OSPF is used as the IGP in the headquarters, and private BGP AS numbers are used in different branches. After the network is set up, you need to observe the transmission of BGP routing information.

    - Lab Configuration

      - Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF in AS 64512.

- Configure full-mesh IBGP peer relationships in AS 64512.

- Establish EBGP peer relationships between AS 64512, AS 64513, and AS 64514.

- Configure R1 and R5 to advertise their Loopback1 routes to their BGP routing tables. Configure R2 and R4 to change the next-hop addresses of BGP routes to the IP addresses of their source interfaces when advertising the routes to specified peers.

      - Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/2, Loopback0, and Loopback1 of R1.

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] ip address 10.0.12.1 255.255.255.0

[R1-GigabitEthernet0/0/2] quit

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.255

[R1-LoopBack0] quit

[R1]interface LoopBack1

[R1-LoopBack1] ip address 10.1.1.1 255.255.255.0

[R1-LoopBack1] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R2.

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.255

[R2-LoopBack0] quit

[R2]interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] ip address 10.0.23.2 255.255.255.0

[R2-GigabitEthernet0/0/2] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] ip address 10.0.12.2 255.255.255.0

[R2-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R3.

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.255

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] ip address 10.0.34.3 255.255.255.0

[R3-GigabitEthernet0/0/2] quit

[R3]interface GigabitEthernet0/0/3

[R3-GigabitEthernet0/0/3] ip address 10.0.23.3 255.255.255.0

[R3-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R4.

[R4]interface GigabitEthernet0/0/2

[R4-GigabitEthernet0/0/2] ip address 10.0.45.4 255.255.255.0

[R4-GigabitEthernet0/0/2] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] ip address 10.0.34.4 255.255.255.0

[R4-GigabitEthernet0/0/3] quit

[R4]interface LoopBack0

[R4-LoopBack0] ip address 10.0.4.4 255.255.255.255

[R4-LoopBack0] quit

# Configure IP addresses for GE0/0/3, Loopback0, and Loopback1 of R5.

[R5]interface LoopBack0

[R5-LoopBack0] ip address 10.0.5.5 255.255.255.255

[R5-LoopBack0] quit

[R5]interface LoopBack1

[R5-LoopBack1] ip address 10.1.5.5 255.255.255.0

[R5-LoopBack1] quit

[R5]interface GigabitEthernet0/0/3

[R5-GigabitEthernet0/0/3] ip address 10.0.45.5 255.255.255.0

[R5-GigabitEthernet0/0/3] quit

# On R2 and R4, ping the IP addresses of the interconnected devices to test the connectivity.

<R2>ping -c 1 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=80 ms

--- 10.0.12.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 80/80/80 ms

<R2>ping -c 1 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=20 ms

--- 10.0.23.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 20/20/20 ms

<R4>ping -c 1 10.0.34.3

PING 10.0.34.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.34.3: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.34.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

<R4>ping -c 1 10.0.45.5

PING 10.0.45.5: 56  data bytes, press CTRL_C to break

Reply from 10.0.45.5: bytes=56 Sequence=1 ttl=255 time=30 ms

--- 10.0.45.5 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/30/30 ms

Configure OSPF in AS 64512.

Configure the IP address of Loopback0 as the router ID on each of R2, R3, and R4.

# Configure R2, and activate OSPF on Loopback0 and GE0/0/2.

[R2]ospf 1 router-id 10.0.2.2

[R2-ospf-1] area 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.23.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] quit

[R2-ospf-1] quit

# Configure R3, and activate OSPF on Loopback0, GE0/0/2, and GE0/0/3.

[R3]ospf 1 router-id 10.0.3.3

[R3-ospf-1] area 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.23.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.34.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] quit

[R3-ospf-1] quit

# Configure R4, and activate OSPF on Loopback0 and GE0/0/3.

[R4]ospf 1 router-id 10.0.4.4

[R4-ospf-1] area 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.34.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] quit

[R4-ospf-1]quit

# Check the brief information about OSPF neighbor relationships on R3.

<R3>display ospf peer brief

OSPF Process 1 with Router ID 10.0.3.3

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/2             10.0.4.4         Full

0.0.0.0          GigabitEthernet0/0/3             10.0.2.2         Full

----------------------------------------------------------------------------

The command output shows that R3 has established OSPF neighbor relationships with R2 and R4.

# Check the OSPF routing table on R3.

<R3>display ospf routing

OSPF Process 1 with Router ID 10.0.3.3

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop						AdvRouter							Area

10.0.3.3/32								0					Stub							10.0.3.3								10.0.3.3								0.0.0.0

10.0.23.0/24							1					Transit					10.0.23.3							10.0.3.3								0.0.0.0

10.0.34.0/24							1					Transit					10.0.34.3							10.0.3.3								0.0.0.0

10.0.2.2/32								1					Stub							10.0.23.2							10.0.2.2								0.0.0.0

10.0.4.4/32								1					Stub							10.0.34.4							10.0.4.4								0.0.0.0

Total Nets: 5

Intra Area: 5  Inter Area: 0  ASE: 0  NSSA: 0

The command output shows that R3 has learned the Loopback0 routes from R2 and R4.

Configure IBGP peers.

Establish full-mesh IBGP peer relationships between Loopback0 interfaces of R2, R3, and R4.

# Configure BGP on R2.

[R2]bgp 64512

[R2-bgp] router-id 10.0.2.2

[R2-bgp] peer 10.0.3.3 as-number 64512

[R2-bgp] peer 10.0.3.3 connect-interface LoopBack0

[R2-bgp] peer 10.0.4.4 as-number 64512

[R2-bgp] peer 10.0.4.4 connect-interface LoopBack0

# Configure BGP on R3.

[R3]bgp 64512

[R3-bgp] router-id 10.0.3.3

[R3-bgp] peer 10.0.2.2 as-number 64512

[R3-bgp] peer 10.0.2.2 connect-interface LoopBack0

[R3-bgp] peer 10.0.4.4 as-number 64512

[R3-bgp] peer 10.0.4.4 connect-interface LoopBack0

# Configure BGP on R4.

[R4]bgp 64512

[R4-bgp] peer 10.0.2.2 as-number 64512

[R4-bgp] peer 10.0.2.2 connect-interface LoopBack0

[R4-bgp] peer 10.0.3.3 as-number 64512

[R4-bgp] peer 10.0.3.3 connect-interface LoopBack0

# Check the status of BGP peer relationships on R2, R3, and R4.

<R2>display bgp peer

BGP local router ID				: 10.0.2.2

Local AS number						: 64512

Total number of peers	: 2		  Peers in established state : 2

Peer											V						AS						MsgRcvd		MsgSent	OutQ		Up/Down			State						PrefRcv

10.0.3.3								4							64512							3								3					0						00:01:57				Established			0

10.0.4.4								4							64512							3								4					0						00:01:56				Established			0

<R3>display bgp peer

BGP local router ID 			: 10.0.3.3

Local AS number						: 64512

Total number of peers	: 2		  Peers in established state : 2

Peer											V						AS						MsgRcvd		MsgSent	OutQ		Up/Down				State					PrefRcv

10.0.2.2								4							64512							3								3					0						00:02:23				Established			0

10.0.4.4								4							64512							3								4					0						00:02:25				Established			0

<R4>display bgp peer

BGP local router ID				: 10.0.4.4

Local AS number						: 64512

Total number of peers	: 2		  Peers in established state : 2

Peer											V						AS						MsgRcvd		MsgSent	OutQ		Up/Down				State						PrefRcv

10.0.2.2								4							64512							3								3					0						00:06:33				Established			0

10.0.3.3								4							64512							3								4					0						00:06:38				Established			0

The command outputs show that R2, R3, and R4 have established full-mesh IBGP peer relationships with each other.

Configure EBGP peers.

Establish EBGP peer relationships between Loopback0 interfaces of R1 and R2 and between Loopback0 interfaces of R4 and R5. To ensure proper establishment, configure static routes on R1 and R2 to ensure routing reachability between Loopback0 interfaces. Perform the same operation on R4 and R5.

# Configure static routes on R1 and R2.

[R1]ip route-static 10.0.2.2 32 10.0.12.2

[R2]ip route-static 10.0.1.1 32 10.0.12.1

# Configure static routes on R4 and R5.

[R4]ip route-static 10.0.5.5 32 10.0.45.5

[R5]ip route-static 10.0.4.4 32 10.0.45.4

# Test the connectivity between the loopback interfaces.

<R1>ping -c 1 -a 10.0.1.1 10.0.2.2

PING 10.0.2.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.2.2: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.2.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

<R5>ping -c 1 -a 10.0.5.5 10.0.4.4

PING 10.0.4.4: 56  data bytes, press CTRL_C to break

Reply from 10.0.4.4: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.4.4 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

# Configure an EBGP peer relationship between R1 and R2.

[R1]bgp 64513

[R1-bgp] router-id 10.0.1.1

[R1-bgp] peer 10.0.2.2 as-number 64512

[R1-bgp] peer 10.0.2.2 ebgp-max-hop 2

[R1-bgp] peer 10.0.2.2 connect-interface LoopBack0

[R2]bgp 64512

[R2-bgp] peer 10.0.1.1 as-number 64513

[R2-bgp] peer 10.0.1.1 ebgp-max-hop 2

[R2-bgp] peer 10.0.1.1 connect-interface LoopBack0

By default, the maximum number of hops allowed for an EBGP connection is 1. In this case, EBGP peers can establish a peer relationship only through a direct link. To use a loopback interface as the source interface to send BGP messages, you need to manually change the maximum number of hops allowed for an EBGP connection.

# Configure an EBGP peer relationship between R4 and R5.

[R4]bgp 64512

[R4-bgp] peer 10.0.5.5 as-number 64514

[R4-bgp] peer 10.0.5.5 ebgp-max-hop 2

[R4-bgp] peer 10.0.5.5 connect-interface LoopBack0

[R5]bgp 64514

[R5-bgp] router-id 10.0.5.5

[R5-bgp] peer 10.0.4.4 as-number 64512

[R5-bgp] peer 10.0.4.4 ebgp-max-hop 2

[R5-bgp] peer 10.0.4.4 connect-interface LoopBack0

# Check the EBGP peer relationship status on R1 and R5.

<R1>display bgp peer

BGP local router ID : 10.0.1.1

Local AS number : 64513

Total number of peers : 1		  Peers in established state : 1

Peer           V        AS     MsgRcvd  MsgSent  OutQ  Up/Down      State     PrefRcv

10.0.2.2        4       64512        7       10     0      00:05:47    			Established    0

<R5>display bgp peer

BGP local router ID : 10.0.5.5

Local AS number : 64514

Total number of peers : 1		  Peers in established state : 1

Peer           V        AS     MsgRcvd  MsgSent  OutQ  Up/Down      State     PrefRcv

10.0.4.4        4       64512        7       10     0      00:03:25    			Established    0

The preceding command outputs show that R1 and R2 as well as R4 and R5 have successfully established EBGP peer relationships.

Configure devices to advertise routes to their BGP routing tables.

Configure R1 and R5 to advertise their Loopback1 routes to their BGP routing tables.

# Run the network command on R1 and R5 to advertise the routes.

[R1]bgp 64513

[R1-bgp] network 10.1.1.1 24

[R5]bgp 64514

[R5-bgp] network 10.1.5.5 24

# Check the BGP routing table on R3.

<R3>display bgp routing-table

BGP Local router ID is 10.0.3.3

Status codes: * - valid, > - best, d - damped,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

Total Number of Routes: 2

Network          NextHop      MED        LocPrf    PrefVal Path/Ogn

i  10.1.1.0/24        10.0.1.1        0          100        0      64513i

i  10.1.5.0/24        10.0.5.5        0          100        0      64514i

The command output shows that R3 has learned the BGP routes advertised by R1 and R5, but the routes are invalid because their next hops are unreachable to R3. To resolve this issue, run the peer next-hop-local command on R2 and R4 to configure the devices to change the next-hop addresses of BGP routes to the IP addresses of their source interfaces when advertising these routes.

# Configure R2 and R4 to change the next-hop addresses of BGP routes to their own IP addresses when advertising these routes.

[R2]bgp 64512

[R2-bgp] peer 10.0.3.3 next-hop-local

[R2-bgp] peer 10.0.4.4 next-hop-local

[R4]bgp 64512

[R4-bgp] peer 10.0.2.2 next-hop-local

[R4-bgp] peer 10.0.3.3 next-hop-local

# Check the BGP routing table on R3 again.

<R3>display bgp routing-table

BGP Local router ID is 10.0.3.3

Status codes: * - valid, > - best, d - damped,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

Total Number of Routes: 2

Network         NextHop      MED        LocPrf    PrefVal Path/Ogn

*>i  10.1.1.0/24        10.0.2.2        0          100        0      64513i

*>i  10.1.5.0/24        10.0.4.4        0          100        0      64514i

The command output shows that the two BGP routes have become valid and the optimal.

# Check the BGP routing tables on R1 and R5.

<R1>display bgp routing-table

BGP Local router ID is 10.0.1.1

Status codes: * - valid, > - best, d - damped,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

Total Number of Routes: 2

Network          NextHop       MED        LocPrf    PrefVal Path/Ogn

*>   10.1.1.0/24        0.0.0.0         	0                   		  0     			i

*>   10.1.5.0/24        10.0.2.2                             				0   	64512 64514i

<R5>display bgp  routing-table

BGP Local router ID is 10.0.5.5

Status codes: * - valid, > - best, d - damped,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

Total Number of Routes: 2

Network         NextHop       MED        LocPrf    PrefVal Path/Ogn

*>   10.1.1.0/24        10.0.4.4                              0      64512 64513i

*>   10.1.5.0/24        0.0.0.0         0                     0      i

The command outputs show that R1 and R5 have learned the Loopback1 routes from each other.

# Test the connectivity between Loopback 1 interfaces on R1 and R5.

<R1>ping -c 1 -a 10.1.1.1 10.1.5.5

PING 10.1.5.5: 56  data bytes, press CTRL_C to break

Reply from 10.1.5.5: bytes=56 Sequence=1 ttl=252 time=130 ms

--- 10.1.5.5 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 130/130/130 ms

----End

    - Quiz

What are the advantages of using loopback interface addresses to establish EBGP peer relationships compared with using physical interface addresses?

    - Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/2

ip address 10.0.12.1 255.255.255.0

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

interface LoopBack1

ip address 10.1.1.1 255.255.255.0

#

bgp 64513

router-id 10.0.1.1

peer 10.0.2.2 as-number 64512

peer 10.0.2.2 ebgp-max-hop 2

peer 10.0.2.2 connect-interface LoopBack0

#

ipv4-family unicast

undo synchronization

network 10.0.1.0 255.255.255.0

network 10.1.1.0 255.255.255.0

peer 10.0.2.2 enable

#

ip route-static 10.0.2.2 255.255.255.255 10.0.12.2

#

Configuration on R2

#

sysname R2

#

interface GigabitEthernet0/0/2

ip address 10.0.23.2 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.12.2 255.255.255.0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

bgp 64512

router-id 10.0.2.2

peer 10.0.1.1 as-number 64513

peer 10.0.1.1 ebgp-max-hop 2

peer 10.0.1.1 connect-interface LoopBack0

peer 10.0.3.3 as-number 64512

peer 10.0.3.3 connect-interface LoopBack0

peer 10.0.4.4 as-number 64512

peer 10.0.4.4 connect-interface LoopBack0

#

ipv4-family unicast

undo synchronization

peer 10.0.1.1 enable

peer 10.0.3.3 enable

peer 10.0.3.3 next-hop-local

peer 10.0.4.4 enable

peer 10.0.4.4 next-hop-local

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.23.2 0.0.0.0

#

ip route-static 10.0.1.1 255.255.255.255 10.0.12.1

#

return

Configuration on R3

#

sysname R3

#

interface GigabitEthernet0/0/2

ip address 10.0.34.3 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.23.3 255.255.255.0

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

bgp 64512

router-id 10.0.3.3

peer 10.0.2.2 as-number 64512

peer 10.0.2.2 connect-interface LoopBack0

peer 10.0.4.4 as-number 64512

peer 10.0.4.4 connect-interface LoopBack0

#

ipv4-family unicast

undo synchronization

peer 10.0.2.2 enable

peer 10.0.4.4 enable

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.23.3 0.0.0.0

network 10.0.34.3 0.0.0.0

#

return

Configuration on R4

#

sysname R4

#

interface GigabitEthernet0/0/2

ip address 10.0.45.4 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.34.4 255.255.255.0

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.255

#

bgp 64512

router-id 10.0.4.4

peer 10.0.2.2 as-number 64512

peer 10.0.2.2 connect-interface LoopBack0

peer 10.0.3.3 as-number 64512

peer 10.0.3.3 connect-interface LoopBack0

peer 10.0.5.5 as-number 64514

peer 10.0.5.5 ebgp-max-hop 2

peer 10.0.5.5 connect-interface LoopBack0

#

ipv4-family unicast

undo synchronization

peer 10.0.2.2 enable

peer 10.0.2.2 next-hop-local

peer 10.0.3.3 enable

peer 10.0.3.3 next-hop-local

peer 10.0.5.5 enable

#

ospf 1 router-id 10.0.4.4

area 0.0.0.0

network 10.0.4.4 0.0.0.0

network 10.0.34.4 0.0.0.0

#

ip route-static 10.0.5.5 255.255.255.255 10.0.45.5

#

return

Configuration on R5

#

sysname R5

#

interface GigabitEthernet0/0/3

ip address 10.0.45.5 255.255.255.0

#

interface LoopBack0

ip address 10.0.5.5 255.255.255.255

#

interface LoopBack1

ip address 10.1.5.5 255.255.255.0

#

bgp 64514

router-id 10.0.5.5

peer 10.0.4.4 as-number 64512

peer 10.0.4.4 ebgp-max-hop 2

peer 10.0.4.4 connect-interface LoopBack0

#

ipv4-family unicast

undo synchronization

network 10.1.5.0 255.255.255.0

peer 10.0.4.4 enable

#

ip route-static 10.0.4.4 255.255.255.255 10.0.45.4

#

Return

  - Lab 2: BGP Route Summarization

    - Introduction

      - Objectives

Upon completion of this task, you will be able to:

Implement automatic summarization for routes imported using the import-route command.

Implement manual route summarization using the aggregate command.

Use the as-set parameter for manual route summarization to prevent routing loops.

      - Networking Topology

              - BGP route summarization

The preceding figure shows the BGP AS numbers and IP addresses of interconnection interfaces. Loopback0 is created on each device, and its IP address is in the format of 10.0.x.x/32, where x indicates the device number.

R1, R2, and R3 use the IP addresses of Loopback0 as their BGP router IDs and establish EBGP peer relationships through directly connected interfaces.

Loopback1 and Loopback2 are created on each of R1 and R3 to simulate user network segments.

      - Background

You are a network administrator of a company. The company's network uses BGP as the routing protocol. The network consists of multiple ASs, with different branches using different AS numbers. As the network scale expands, more and more routing entries are stored in the routing tables on the routers, making it urgent to summarize BGP routes. After testing several methods of route summarization, you have finally selected a proper method to implement route summarization.

    - Lab Configuration

      - Configuration Roadmap

- Configure IP addresses for the devices.

- Configure EBGP peer relationships between R1, R2, and R3 as planned.

- Configure R1 to advertise its Loopback1 and Loopback2 routes to the BGP routing table and implement automatic route summarization. Check detailed information about the summary route on R2.

- Configure R3 to advertise its Loopback1 and Loopback2 routes to the BGP routing table, and manually summarize the routes on R2. Check detailed information about the summary route on R2 and R3. Perform manual summarization on R2 again, and this time configure the as-set parameter. Then, check detailed information about the summary route on R2.

      - Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/2, Loopback0, Loopback1, and Loopback2 of R1.

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] ip address 10.0.12.1 255.255.255.0

[R1-GigabitEthernet0/0/2] quit

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.255

[R1-LoopBack0] quit

[R1]interface LoopBack1

[R1-LoopBack1] ip address 172.16.1.1 255.255.255.0

[R1-LoopBack1] quit

[R1]interface LoopBack2

[R1-LoopBack1] ip address 172.16.2.1 255.255.255.0

[R1-LoopBack1] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R2.

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.255

[R2-LoopBack0] quit

[R2]interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] ip address 10.0.23.2 255.255.255.0

[R2-GigabitEthernet0/0/2] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] ip address 10.0.12.2 255.255.255.0

[R2-GigaitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/3, Loopback0, Loopback1, and Loopback2 of R3.

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.255

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/3

[R3-GigabitEthernet0/0/3] ip address 10.0.23.3 255.255.255.0

[R3-GigabitEthernet0/0/3] quit

[R3]interface LoopBack1

[R3-LoopBack1] ip address 172.17.1.1 255.255.255.0

[R3-LoopBack1] quit

[R3]interface LoopBack2

[R3-LoopBack1] ip address 172.17.2.1 255.255.255.0

[R3-LoopBack1] quit

# On R2, ping the IP addresses of the interconnected devices to test the connectivity.

<R2>ping -c 1 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=80 ms

--- 10.0.12.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 80/80/80 ms

<R2>ping -c 1 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=20 ms

--- 10.0.23.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 20/20/20 ms

Configure EBGP peer relationships.

Configure EBGP peer relationships between R1 and R2, and between R2 and R3 through directly connected interfaces.

# Configure R1.

[R1]bgp 64511

[R1-bgp] router-id 10.0.1.1

[R1-bgp] peer 10.0.12.2 as-number 64512

# Configure R2.

[R2]bgp 64512

[R2-bgp] router-id 10.0.2.2

[R2-bgp] peer 10.0.12.1 as-number 64511

[R2-bgp] peer 10.0.23.3 as-number 64513

# Configure R3.

[R3]bgp 64513

[R3-bgp] router-id 10.0.3.3

[R3-bgp] peer 10.0.23.2 as-number 64512

# Check the BGP peer relationship status on R2.

<R2>display bgp peer

BGP local router ID : 10.0.2.2

Local AS number : 64512

Total number of peers : 2		  Peers in established state : 2

Peer            V        AS      MsgRcvd  MsgSent OutQ  Up/Down      State    PrefRcv

10.0.12.1        4       64511        3        3     0      00:02:41 		   Established   0

10.0.23.3        4       64513        3        4     0      00:01:20  		  Established   0

The command output shows that EBGP peer relationships have been successfully established between R1 and R2, and between R2 and R3.

Configure automatic BGP route summarization.

Enable automatic BGP route summarization on R1, and configure R1 to advertise its Loopback1 and Loopback2 routes to the BGP routing table so that R1 automatically summarizes these routes.

# Create IP prefix list 1 to match the Loopback1 and Loopback2 routes.

[R1]ip ip-prefix 1 permit 172.16.0.0 16 greater-equal 24 less-equal 24

# Create a route-policy named hcip, create node 10, and configure an if-match clause with IP prefix list 1 specified.

[R1]route-policy hcip permit node 10

[R1-route-policy] if-match ip-prefix 1

[R1-route-policy] quit

# Configure R1 to advertise the Loopback1 and Loopback2 routes to the BGP routing table, and enable automatic BGP route summarization on R1.

[R1]bgp 64511

[R1-bgp] import-route direct route-policy hcip

[R1-bgp] summary automatic

Info: Automatic summarization is valid only for the routes imported through the import-route command.

Automatic summarization takes effect only on the routes imported using the import-route command.

# Check the BGP routing table on R1.

<R1>display	bgp	routing-table

BGP	Local	router	ID	is	10.0.1.1

Status	codes:	*	-	valid,	>	-	best,	d	-	damped,

h	-	history,		i	-	internal,	s	-	suppressed,	S	-	Stale

Origin	:	i	-	IGP,	e	-	EGP,	?	-	incomplete

Total	Number	of	Routes	:	3

Network									NextHop						MED								LocPrf				PrefVal		Path/Ogn

*>				172.16.0.0							127.0.0.1																												0							?

s>				172.16.1.0/24					0.0.0.0									0																					0							?

s>				172.16.2.0/24					0.0.0.0									0																					0							?

The Loopback1 and Loopback2 routes have been advertised to the BGP routing table. As automatic BGP route summarization is enabled on R1, R1 summarizes these routes into the summary route 172.16.0.0/16 and suppresses all the specific routes. In the routing table, the s flag displayed before each specific route indicates that the route is suppressed. As a result, R1 advertises only the summary route 172.16.0.0/16.

# Check the BGP routing table on R2.

<R2>display bgp routing-table

BGP Local router ID is 10.0.2.2

Status codes: * - valid, > - best, d - damped,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

Total Number of Routes: 1

Network          NextHop        MED     LocPrf    PrefVal  Path/Ogn

*>   172.16.0.0         10.0.12.1                             0      64511?

The command output shows only the summary route 172.16.0.0/16 on R2.

# Check detailed information about the BGP summary route 172.16.0.0 on R2.

<R2>display bgp routing-table 172.16.0.0

BGP local router ID : 10.0.2.2

Local AS number : 64512

Paths:   1 available, 1 best, 1 select

BGP routing table entry information of 172.16.0.0/16:

From: 10.0.12.1 (10.0.1.1)

Route Duration: 01h09m27s

Direct Out-interface: GigabitEthernet0/0/3

Original nexthop: 10.0.12.1

Qos information : 0x0

AS-path 64511, origin incomplete, pref-val 0, valid, external, best, select, active, pre 255

Aggregator: AS 64511, Aggregator ID 10.0.1.1

Advertised to such 2 peers:

10.0.12.1

10.0.23.3

The path attributes of this route include the Aggregator attribute, which carries the AS number and router ID of the device that generates the summary route.

Configure manual BGP route summarization.

Configure R3 to advertise its Loopback1 and Loopback2 routes to the BGP routing table. Run the aggregate command on R2 to manually summarize these routes and suppress the advertisement of the specific routes.

# Create IP prefix list 1 to match the Loopback1 and Loopback2 routes.

[R3]ip ip-prefix 1 permit 172.17.0.0 16 greater-equal 24 less-equal 24

# Create a route-policy named hcip, create node 10, and configure an if-match clause with IP prefix list 1 specified.

[R3]route-policy hcip permit node 10

[R3-route-policy] if-match ip-prefix 1

[R3-route-policy] quit

# Configure R3 to advertise its Loopback1 and Loopback2 routes to the BGP routing table.

[R3]bgp 64513

[R3-bgp] import-route direct route-policy hcip

# Check the BGP routing table on R2.

<R2>display	bgp	routing-table

BGP	Local	router	ID	is	10.0.2.2

Status	codes:	*	-	valid,	>	-	best,	d	-	damped,

h	-	history,		i	-	internal,	s	-	suppressed,	S	-	Stale

Origin	:	i	-	IGP,	e	-	EGP,	?	-	incomplete

Total	Number	of	Routes:	3

Network									NextHop					MED						LocPrf				PrefVal			Path/Ogn

*>			172.16.0.0								10.0.12.1																											0						64511?

*>			172.17.1.0/24						10.0.23.3							0																				0						64513?

*>			172.17.2.0/24						10.0.23.3							0																				0						64513?

The BGP routing table of R2 contains the BGP routes 172.17.1.0/24 and 172.17.2.0/24 advertised by R3.

# On R2, manually summarize the routes 172.17.1.0/24 and 172.17.2.0/24 into the summary route 172.17.0.0/22, and suppress the advertisement of the specific routes.

[R2]bgp 64512

[R2-bgp] aggregate 172.17.0.0 22 detail-suppressed

# Check the BGP routing table on R2.

<R2>display	bgp	routing-table

BGP	Local	router	ID	is	10.0.2.2

Status	codes:	*	-	valid,	>	-	best,	d	-	damped,

h	-	history,		i	-	internal,	s	-	suppressed,	S	-	Stale

Origin	:	i	-	IGP,	e	-	EGP,	?	-	incomplete

Total	Number	of	Routes:	4

Network									NextHop						MED						LocPrf						PrefVal		Path/Ogn

*>			172.16.0.0								10.0.12.1																													0						64511?

*>			172.17.0.0/22						127.0.0.1																													0						?

s>			172.17.1.0/24						10.0.23.3							0																						0						64513?

s>			172.17.2.0/24						10.0.23.3							0																						0						64513?

The summary route is displayed in the BGP routing table of R2.

# Check detailed information about the BGP summary route 172.16.0.0/22 on R2.

<R2>display bgp routing-table 172.17.0.0 22

BGP local router ID : 10.0.2.2

Local AS number : 64512

Paths:   1 available, 1 best, 1 select

BGP routing table entry information of 172.17.0.0/22:

Aggregated route.

Route Duration: 00h02m44s

Direct Out-interface: NULL0

Original nexthop: 127.0.0.1

Qos information : 0x0

AS-path Nil, origin incomplete, pref-val 0, valid, local, best, select, active, pre 255

Aggregator: AS 64512, Aggregator ID 10.0.2.2, Atomic-aggregate

Advertised to such 2 peers:

10.0.12.1

10.0.23.3

The command output shows that the AS-path field value is Nil, indicating that the AS_Path attribute is empty. This means that the AS_Path attribute values of the specific routes are lost. BGP depends on the AS_Path attribute to prevent routing loops. Therefore, the loss of the AS_Path attribute value may cause a routing loop. The command output also shows the BGP peers to which the summary route is advertised, and these peers include the peer 10.0.23.3 (R3).

# Check the BGP routing table on R3.

<R3>display	bgp	routing-table

BGP	Local	router	ID	is	10.0.3.3

Status	codes:	*	-	valid,	>	-	best,	d	-	damped,

h	-	history,		i	-	internal,	s	-	suppressed,	S	-	Stale

Origin	:	i	-	IGP,	e	-	EGP,	?	-	incomplete

Total	Number	of	Routes	:	4

Network								NextHop						MED						LocPrf					PrefVal				Path/Ogn

*>			172.16.0.0							10.0.23.2																												0						64512	64511?

*>			172.17.0.0/22					10.0.23.2																												0						64512?

*>			172.17.1.0/24					0.0.0.0										0																				0						?

*>			172.17.2.0/24					0.0.0.0										0																				0						?

The BGP routing table of R3 contains the summary route 172.17.0.0/22.

# To prevent routing loops, specify the as-set parameter when performing manual route summarization on R2.

[R2]bgp 64512

[R2-bgp] aggregate 172.17.0.0 255.255.252.0 detail-suppressed as-set

# Check detailed information about the BGP summary route 172.17.0.0/22 on R2 again.

[R2]display bgp routing-table 172.17.0.0 22

BGP local router ID : 10.0.2.2

Local AS number : 64512

Paths:   1 available, 1 best, 1 select

BGP routing table entry information of 172.17.0.0/22:

Aggregated route.

Route Duration: 00h09m31s

Direct Out-interface: NULL0

Original nexthop: 127.0.0.1

Qos information : 0x0

AS-path 64513, origin incomplete, pref-val 0, valid, local, best, select, active, pre 255

Aggregator: AS 64512, Aggregator ID 10.0.2.2, Atomic-aggregate

Advertised to such 2 peers:

10.0.12.1

10.0.23.3

The command output shows that the value of the AS_Path attribute in the summary route is 64513, and the route is still advertised to the peer 10.0.23.3 (R3).

# Check the BGP routing table on R3 again.

<R3>display	bgp	routing-table

BGP	Local	router	ID	is	10.0.3.3

Status	codes:	*	-	valid,	>	-	best,	d	-	damped,

h	-	history,		i	-	internal,	s	-	suppressed,	S	-	Stale

Origin	:	i	-	IGP,	e	-	EGP,	?	-	incomplete

Total	Number	of	Routes:	4

Network									NextHop						MED						LocPrf				PrefVal				Path/Ogn

*>			172.16.0.0								10.0.23.2																												0						64512	64511?

*>			172.17.1.0/24						0.0.0.0									0																					0						?

*>			172.17.2.0/24						0.0.0.0									0																					0						?

After R3 receives the summary route 172.17.0.0/22, it finds its own AS number (64153) in the AS_Path attribute of the route and ignores this route. In this case, the summary route 172.17.0.0/22 does not exist in the BGP routing table of R3. Therefore, using the as-set parameter for manual route summarization effectively prevents a routing loop.

----End

    - Quiz

What are the differences between the path attributes carried in a summary route generated using the aggregate command and that generated using the summary automatic command?

    - Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/2

ip address 10.0.12.1 255.255.255.0

#

interface NULL0

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

interface LoopBack1

ip address 172.16.1.1 255.255.255.0

#

interface LoopBack2

ip address 172.16.2.1 255.255.255.0

#

bgp 64511

router-id 10.0.1.1

peer 10.0.12.2 as-number 64512

#

ipv4-family unicast

undo synchronization

summary automatic

import-route direct route-policy hcip

peer 10.0.12.2 enable

#

route-policy hcip permit node 10

if-match ip-prefix 1

#

ip ip-prefix 1 index 10 permit 172.16.0.0 16 greater-equal 24 less-equal 24

#

return

Configuration on R2

#

sysname R2

#

interface GigabitEthernet0/0/2

ip address 10.0.23.2 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.12.2 255.255.255.0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

bgp 64512

router-id 10.0.2.2

peer 10.0.12.1 as-number 64511

peer 10.0.23.3 as-number 64513

#

ipv4-family unicast

undo synchronization

aggregate 172.17.0.0 255.255.252.0 as-set detail-suppressed

peer 10.0.12.1 enable

peer 10.0.23.3 enable

#

return

Configuration on R3

#

sysname R3

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

interface LoopBack1

ip address 172.17.1.1 255.255.255.0

#

interface LoopBack2

ip address 172.17.2.1 255.255.255.0

#

bgp 64513

router-id 10.0.3.3

peer 10.0.23.2 as-number 64512

#

ipv4-family unicast

undo synchronization

import-route direct route-policy hcip

peer 10.0.23.2 enable

#

route-policy hcip permit node 10

if-match ip-prefix 1

#

ip ip-prefix 1 index 10 permit 172.17.0.0 16 greater-equal 24 less-equal 24

#

return

  - Lab 3: BGP RR

Introduction

      - Objectives

Upon completion of this task, you will be able to:

Deploy RRs in an AS.

Analyze how the BGP path attribute Originator_ID implements routing loop prevention in an RR environment.

Analyze how the BGP path attribute Cluster_List implements routing loop prevention in an RR environment.

      - Networking Topology

              - BGP RR

R1, R2, R3, and R4 belong to AS 64511. The preceding figure shows the device interconnection mode and IP addresses of interconnection interfaces. Loopback0 is created on each device, and its IP address is in the format of 10.0.x.x/32, where x indicates the device number. The Loopback1 addresses of R1 and R2 are 10.1.1.1/24 and 10.2.2.2/24, respectively. The loopback interfaces are used to simulate user network segments.

All devices use the IP addresses of Loopback0 as their BGP router IDs. IBGP peer relationships are established between R1 and R2, R2 and R3, R3 and R4, and R4 and R2 through directly connected interfaces. R1 is the RR client of R2, R2 is the RR client of R3, and R3 is the RR client of R4.

      - Background

The headquarters network of a company uses BGP as the routing protocol. The four routers in the headquarters establish IBGP peer relationships (not fully meshed). To enable the four routers to learn complete BGP routes, BGP RRs need to be deployed on the network.

    - Lab Configuration

      - Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF in the AS, and activate OSPF on the interconnection and Loopback0 interfaces.

- Establish IBGP peer relationships through directly connected interfaces in the AS.

- Configure RRs, and specify R1 as the RR client of R2, R2 as the RR client of R3, and R3 as the RR client of R4.

- Configure R2 to advertise its Loopback1 route to the BGP routing table, and observe how the Originator_ID attribute is used to prevent a routing loop.

- Configure R1 to advertise its Loopback1 route to the BGP routing table, and observe how the Cluster_List attribute is used to prevent a routing loop.

      - Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/2, Loopback0, and Loopback1 of R1.

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] ip address 10.0.12.1 255.255.255.0

[R1-GigabitEthernet0/0/2] quit

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.255

[R1-LoopBack0] quit

[R1]interface LoopBack1

[R1-LoopBack1] ip address 10.1.1.1 255.255.255.0

[R1-LoopBack1] quit

# Configure IP addresses for GE0/0/1, GE0/0/2, GE0/0/3, and Loopback0 of R2.

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.255

[R2-LoopBack0] quit

[R2]interface GigabitEthernet0/0/1

[R2-GigabitEthernet0/0/1] ip address 10.0.24.2 255.255.255.0

[R2-GigabitEthernet0/0/1] quit

[R2]interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] ip address 10.0.23.2 255.255.255.0

[R2-GigabitEthernet0/0/2] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] ip address 10.0.12.2 255.255.255.0

[R2-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R3.

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.255

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] ip address 10.0.34.3 255.255.255.0

[R3-GigabitEthernet0/0/2] quit

[R3]interface GigabitEthernet0/0/3

[R3-GigabitEthernet0/0/3] ip address 10.0.23.3 255.255.255.0

[R3-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/1, GE0/0/3, and Loopback0 of R4.

[R4]interface LoopBack0

[R4-LoopBack0] ip address 10.0.4.4 255.255.255.255

[R4-LoopBack0] quit

[R4]interface GigabitEthernet0/0/1

[R4-GigabitEthernet0/0/1] ip address 10.0.24.4 255.255.255.0

[R4-GigabitEthernet0/0/1] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] ip address 10.0.34.4 255.255.255.0

[R4-GigabitEthernet0/0/3] quit

# On R2 and R3, ping the IP addresses of the interconnected devices to test the connectivity.

<R2>ping -c 1 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=40 ms

--- 10.0.12.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/40/40 ms

<R2>ping -c 1 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=10 ms

--- 10.0.23.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 10/10/10 ms

<R2>ping -c 1 10.0.24.4

PING 10.0.24.4: 56  data bytes, press CTRL_C to break

Reply from 10.0.24.4: bytes=56 Sequence=1 ttl=255 time=80 ms

--- 10.0.24.4 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 80/80/80 ms

<R3>ping -c 1 10.0.34.4

PING 10.0.34.4: 56  data bytes, press CTRL_C to break

Reply from 10.0.34.4: bytes=56 Sequence=1 ttl=255 time=10 ms

--- 10.0.34.4 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 10/10/10 ms

Configure OSPF in AS 64511.

Configure R1, R2, R3, and R4 to use the IP addresses of Loopback0 as their router IDs, and activate OSPF on the interconnection and Loopback0 interfaces.

# Configure R1.

[R1]ospf 1 router-id 10.0.1.1

[R1-ospf-1] area 0.0.0.0

[R1-ospf-1-area-0.0.0.0] network 10.0.1.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] network 10.0.12.1 0.0.0.0

# Configure R2.

[R2]ospf 1 router-id 10.0.2.2

[R2-ospf-1]area 0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.12.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.23.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.24.2 0.0.0.0

# Configure R3.

[R3]ospf 1 router-id 10.0.3.3

[R3-ospf-1]area 0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.23.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.34.3 0.0.0.0

# Configure R4.

[R4]ospf 1 router-id 10.0.4.4

[R4-ospf-1]area 0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.24.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.34.4 0.0.0.0

# Check the brief information about OSPF neighbor relationships on R2 and R3.

<R2>display ospf peer brief

OSPF Process 1 with Router ID 10.0.2.2

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/2             10.0.3.3         Full

0.0.0.0          GigabitEthernet0/0/3             10.0.1.1         Full

0.0.0.0          GigabitEthernet0/0/1             10.0.4.4         Full

----------------------------------------------------------------------------

<R3>display ospf peer brief

OSPF Process 1 with Router ID 10.0.3.3

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/3             10.0.2.2         Full

0.0.0.0          GigabitEthernet0/0/2             10.0.4.4         Full

----------------------------------------------------------------------------

The command outputs show that all the OSPF neighbor relationships have been established properly.

# Check the OSPF routing table on R4.

<R4>display ospf routing

OSPF Process 1 with Router ID 10.0.4.4

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter						Area

10.0.4.4/32								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.24.0/24							1					Transit					10.0.24.4							10.0.4.4								0.0.0.0

10.0.34.0/24							1					Transit					10.0.34.4							10.0.4.4								0.0.0.0

10.0.1.1/32								2					Stub							10.0.24.2							10.0.1.1								0.0.0.0

10.0.2.2/32								1					Stub							10.0.24.2							10.0.2.2								0.0.0.0

10.0.3.3/32								1					Stub							10.0.34.3							10.0.3.3								0.0.0.0

10.0.12.0/24							2					Transit					10.0.24.2							10.0.1.1								0.0.0.0

10.0.23.0/24							2					Transit					10.0.24.2							10.0.2.2								0.0.0.0

10.0.23.0/24							2					Transit					10.0.34.3							10.0.2.2								0.0.0.0

Total Nets: 9

Intra Area: 9  Inter Area: 0  ASE: 0  NSSA: 0

The command output shows that R4 has learned the routes on the entire network.

Configure IBGP peer relationships.

Establish IBGP peer relationships between Loopback0 interfaces in the AS.

# Configure R1.

[R1]bgp 64511

[R1-bgp] router-id 10.0.1.1

[R1-bgp] peer 10.0.12.2 as-number 64511

# Configure R2.

[R2-bgp] router-id 10.0.2.2

[R2-bgp] peer 10.0.12.1 as-number 64511

[R2-bgp] peer 10.0.23.3 as-number 64511

[R2-bgp] peer 10.0.24.4 as-number 64511

# Configure R3.

[R3]bgp 64511

[R3-bgp] router-id 10.0.3.3

[R3-bgp] peer 10.0.23.2 as-number 64511

[R3-bgp] peer 10.0.34.4 as-number 64511

# Configure R4.

[R4]bgp 64511

[R4-bgp] router-id 10.0.4.4

[R4-bgp] peer 10.0.24.2 as-number 64511

[R4-bgp] peer 10.0.34.3 as-number 64511

# Check the IBGP peer relationship status on R2 and R3.

<R2>display bgp peer

BGP local router ID : 10.0.2.2

Local AS number : 64511

Total number of peers : 3		  Peers in established state : 3

Peer												V							AS						MsgRcvd		MsgSent	OutQ		Up/Down						State				PrefRcv

10.0.12.1								4							64511								3								3					0						00:05:39							Established			0

10.0.23.3								4							64511								3								4					0						00:05:23							Established			0

10.0.24.4								4							64511								3								4					0						00:05:16							Established			0

<R3>display bgp peer

BGP local router ID : 10.0.3.3

Local AS number : 64511

Total number of peers : 2	  Peers in established state : 2

Peer            V        AS      MsgRcvd  MsgSent OutQ  Up/Down      State      PrefRcv

10.0.23.2        4       64511        7        8     0      00:04:33   			 Established   0

10.0.34.4        4       64511        8        9     0      00:04:32  			  Established   0

The command outputs show that the IBGP peer relationships have been successfully established in the AS.

Configure RRs.

# Configure R1 as an RR client on R2.

[R2]bgp 64511

[R2-bgp] peer 10.0.12.1 reflect-client

# Configure R2 as an RR client on R3.

[R3]bgp 64511

[R3-bgp] peer 10.0.23.2 reflect-client

# Configure R3 as an RR client on R4.

[R4]bgp 64511

[R4-bgp] peer 10.0.34.3 reflect-client

Verify that the Originator_ID attribute can prevent routing loops.

In this step, configure R2 to advertise the BGP route 10.2.2.0/24. Observe whether the route is advertised back to R2 after being reflected by R3 and R4 in sequence. If so, a routing loop may occur.

By default, after R2 advertises a BGP route, the route is directly advertised by R2 to R4. In addition, the route is reflected by R3 to R4. In this case, R4 preferentially selects the route directly advertised by R2 and does not reflect the route reflected by R3 back to R2. For the purpose of this experiment, a route-policy needs to be configured on R2 to prevent R2 from directly advertising the route 10.2.2.0/24 to R4.

# Configure a route-policy.

[R2]acl number 2000

[R2-acl-basic-2000] rule 5 permit

[R2-acl-basic-2000] quit

[R2]route-policy bgp deny node 10

[R2-route-policy] if-match acl 2000

# Apply the route-policy to filter routes to be advertised to the specified BGP peer.

[R2]bgp 64511

[R2-bgp] peer 10.0.24.4 route-policy bgp export

# Configure R2 to advertise the route 10.2.2.0/24.

[R2]bgp 64511

[R2-bgp] network 10.2.2.0 24

# Check detailed information about the route 10.2.2.0/24 on R2.

<R2>display bgp routing-table 10.2.2.0 24

BGP local router ID		: 10.0.2.2

Local AS number				: 64511

Paths													: 1 available, 1 best, 1 select

BGP routing table entry information of 10.2.2.0/24:

Network route.

From													: 0.0.0.0 (0.0.0.0)

Route Duration					: 00h00m36s

Direct Out-interface	: LoopBack1

Original nexthop				: 10.2.2.2

Qos information				: 0x0

AS-path Nil, origin igp, MED 0, pref-val 0, valid, local, best, select, pre 0

Advertised to such 2 peers:

10.0.23.3    #R3

10.0.12.1    #R1

The command output shows that R2 has advertised this route to R3 and R1, but not to R4.

# Check detailed information about the BGP route 10.2.2.0/24 on R3.

<R3>display bgp routing-table 10.2.2.0 24

BGP local router ID			: 10.0.3.3

Local AS number					: 64511

Paths														: 1 available, 1 best, 1 select

BGP routing table entry information of 10.2.2.0/24:

RR-client route.

From														: 10.0.23.2 (10.0.2.2)

Route Duration						: 00h31m14s

Relay IP Nexthop				: 0.0.0.0

Relay IP Out-Interface: GigabitEthernet0/0/3

Original nexthop					: 10.0.23.2

Qos information					: 0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255

Advertised to such 1 peers:

10.0.34.4

R3 has reflected the BGP route 10.2.2.0/24 from its RR client to the peer 10.0.34.4 (R4). In addition, the next-hop address of the BGP route is 10.0.23.2.

# Check detailed information about the BGP route 10.2.2.0/24 on R4.

<R4>display bgp routing-table 10.2.2.0 24

BGP	local	router	ID					:	10.0.4.4

Local	AS	number					:	64511

Paths															:	1	available,	1	best,	1	select

BGP	routing	table	entry	information	of	10.2.2.0/24:

RR-client	route.

From															:	10.0.34.3	(10.0.3.3)

Route	Duration							:	00h23m59s

Relay	IP	Nexthop						:	10.0.24.2

Relay	IP	Out-Interface		:	GigabitEthernet0/0/1

Original	nexthop						:	10.0.23.2

Qos	information						:	0x0

AS-path	Nil,	origin	igp,	MED	0,	localpref	100,	pref-val	0,	valid,	internal,	best,	select,	active,	pre	255,	IGP	cost	2

Originator											:		10.0.2.2

Cluster	list											:	10.0.3.3

Advertised to such 1 peers:

10.0.24.2

The route is received from the RR client R3. When R3 reflects the original route, the next-hop address of the route remains unchanged, and R3 adds the Originator_ID attribute with the value of 10.0.2.2 to the route. After receiving this route from R3, R4 reflects it to R2.

# Check detailed information about the BGP route 10.2.2.0/24 on R2 again.

<R2>display bgp routing-table 10.2.2.0 24

BGP	local	router	ID			:	10.0.2.2

Local	AS	number			:	64511

Paths													:	1	available,	1	best,	1	select

BGP	routing	table	entry	information	of	10.2.2.0/24:

Network	route.

From:	0.0.0.0	(0.0.0.0)

Route	Duration					:	00h57m17s

Direct	Out-interface		:	LoopBack1

Original	nexthop				:	10.2.2.2

Qos	information				:	0x0

AS-path Nil, origin igp, MED 0, pref-val 0, valid, local, best, select, pre 0

Advertised to such 2 peers:

10.0.23.3

10.0.12.1

Only the locally advertised BGP route exists in the routing table, and the BGP route advertised by R4 does not exist.

# Check detailed information about the BGP peer 10.0.24.4 on R2.

<R2>display bgp peer 10.0.24.4 verbose

BGP Peer is 10.0.24.4,  remote AS 64511

Type: IBGP link

BGP version 4, Remote router ID 10.0.4.4

Update-group ID								: 2

BGP current state								: Established, Up for 00h27m44s

BGP current event								: RecvKeepalive

BGP last state											: OpenConfirm

BGP Peer Up count							: 2

Received total routes					: 0

Received active routes total: 0

Advertised total routes				: 0

Port:  Local - 179	Remote - 64495

Configured: Connect-retry Time	: 32 sec

Configured: Active Hold Time			: 180 sec	Keepalive Time:60 sec

Received  : Active Hold Time			: 180 sec

Negotiated: Active Hold Time			: 180 sec	Keepalive Time:60 sec

Peer optional capabilities:

Peer supports bgp multi-protocol extension

Peer supports bgp route refresh capability

Peer supports bgp 4-byte-as capability

Address family IPv4 Unicast: advertised and received

Received: Total 30 messages

Update messages 		1

Open messages 			1

KeepAlive messages 		28

Notification messages  	0

Refresh messages 		0

Sent: Total 30 messages

Update messages 		0

Open messages 			2

KeepAlive messages 		28

Notification messages  	0

Refresh messages 		0

Authentication type configured: None

Last keepalive received: 2020-06-02 14:12:02-08:00

Minimum route advertisement interval is 15 seconds

Optional capabilities:

Route refresh capability has been enabled

4-byte-as capability has been enabled

Peer Preferred Value: 0

Routing policy configured:

No import update filter list

No export update filter list

No import prefix list

No export prefix list

No import route policy

Export route policy is: bgp

No import distribute policy

No export distribute policy

The command output shows that R2 receives an Update message from R4 and does not send an Update message to R4 (due to routing filtering by the route-policy). However, the BGP route 10.2.2.0/24 advertised by R4 does not exist in R2's BGP routing table.

# Trigger an import soft reset on R2 to allow R4 to re-send Update messages.

<R2>refresh bgp 10.0.24.4 import

# Check the number of Update messages sent and received on R2 again.

<R2>display bgp peer 10.0.24.4 verbose | in Update

Update-group ID		: 2

BGP current event	: RecvUpdate

Update messages 		2

Update messages 		0

The number of received Update messages increases. R2 receives the BGP route 10.2.2.0/24 from R4.

# Check detailed information about the BGP route 10.2.2.0/24 on R2 again.

<R2>display bgp routing-table 10.2.2.0 24

BGP local router ID		: 10.0.2.2

Local AS number				: 64511

Paths													: 1 available, 1 best, 1 select

BGP routing table entry information of 10.2.2.0/24:

Network route.

From													: 0.0.0.0 (0.0.0.0)

Route Duration					: 01h07m12s

Direct Out-interface	: LoopBack1

Original nexthop				: 10.2.2.2

Qos information				: 0x0

AS-path Nil, origin igp, MED 0, pref-val 0, valid, local, best, select, pre 0

Advertised to such 2 peers:

10.0.23.3

10.0.12.1

Still only the locally advertised BGP route exists in the routing table. The value of the Originator_ID attribute of the BGP route advertised by R4 is the same as the local router ID. As a result, R2 ignores the route advertised by R4.

Verify that the Cluster_List attribute can prevent routing loops.

To facilitate observation, cancel BGP route advertisement on R2. Configure R1 to advertise its Loopback1 route to the BGP routing table. Observe how the Cluster_List attribute prevents routing loops.

# Delete the BGP route advertised by R2.

[R2]bgp 64511

[R2-bgp] undo network 10.2.2.0 255.255.255.0

# Configure R1 to advertise its Loopback1 route to the BGP routing table.

[R1]bgp 64511

[R1-bgp] network 10.1.1.0 24

# Check detailed information about the BGP route 10.1.1.0 /24 on R1, R2, R3, and R4 in sequence.

[R1]display bgp routing-table 10.1.1.0 24

BGP local router ID		: 10.0.1.1

Local AS number				: 64511

Paths													: 1 available, 1 best, 1 select

BGP routing table entry information of 10.1.1.0/24:

Network route.

From													: 0.0.0.0 (0.0.0.0)

Route Duration					: 00h01m41s

Direct Out-interface	: LoopBack1

Original nexthop				: 10.1.1.1

Qos information				: 0x0

AS-path Nil, origin igp, MED 0, pref-val 0, valid, local, best, select, pre 0

Advertised to such 1 peers:

10.0.12.2

R1 is the originator of the BGP route 10.1.1.0/24 and advertises it to R2 (10.0.12.2).

<R2>display bgp routing-table 10.1.1.0 24

BGP local router ID	: 10.0.2.2

Local AS number			: 64511

Paths												: 1 available, 1 best, 1 select

BGP routing table entry information of 10.1.1.0/24:

RR-client route.

From												: 10.0.12.1 (10.0.1.1)

Route Duration				: 00h02m03s

Relay IP Nexthop		: 0.0.0.0

Relay IP Out-Interface: GigabitEthernet0/0/3

Original nexthop			: 10.0.12.1

Qos information			: 0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255

Advertised to such 1 peers:

10.0.23.3

R2 receives the BGP route 10.1.1.0/24 from its RR client R1 and reflects it to R3 (10.0.23.3).

<R3>display bgp routing-table 10.1.1.0 24

BGP local router ID	: 10.0.3.3

Local AS number			: 64511

Paths												: 1 available, 1 best, 1 select

BGP routing table entry information of 10.1.1.0/24:

RR-client route.

From												: 10.0.23.2 (10.0.2.2)

Route Duration				: 00h02m21s

Relay IP Nexthop		: 10.0.23.2

Relay IP Out-Interface: GigabitEthernet0/0/3

Original nexthop			: 10.0.12.1

Qos information			: 0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255, IGP cost 2

Originator								: 10.0.1.1

Cluster list								: 10.0.2.2

Advertised to such 1 peers:

10.0.34.4

R3 receives the BGP route 10.1.1.0/24 from its RR client R2, which added the Cluster_List attribute with the value of 10.0.2.2 to the route when reflecting it. R3 then reflects the received route to R4 (10.0.34.4).

<R4>display bgp routing-table 10.1.1.0 24

BGP local router ID	: 10.0.4.4

Local AS number			: 64511

Paths												: 1 available, 1 best, 1 select

BGP routing table entry information of 10.1.1.0/24:

RR-client route.

From												: 10.0.34.3 (10.0.3.3)

Route Duration				: 00h02m44s

Relay IP Nexthop		: 10.0.24.2

Relay IP Out-Interface: GigabitEthernet0/0/1

Original nexthop			: 10.0.12.1

Qos information			: 0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255, IGP cost 2

Originator								: 10.0.1.1

Cluster list								: 10.0.3.3, 10.0.2.2

Advertised to such 1 peers:

10.0.24.2

R4 receives the BGP route 10.1.1.0/24 from its RR client R3, which added the Cluster_List attribute with the values of 10.0.3.3 and 10.0.2.2 to the route when reflecting it. R4 then reflects the received route to R2 (10.0.24.2).

# Check the BGP routing table on R2 again.

<R2>display bgp routing-table

BGP Local router ID is 10.0.2.2

Status codes: * - valid, > - best, d - damped,

h - history,  i - internal, s - suppressed, S - Stale

Origin : i - IGP, e - EGP, ? - incomplete

Total Number of Routes: 1

Network           NextHop     MED      LocPrf    PrefVal   Path/Ogn

*>i  10.1.1.0/24         10.0.12.1       0         100        0         i

The BGP routing table of R2 still contains only the BGP route 10.1.1.0/24 received from the peer 10.0.12.1.

# Check detailed information about the BGP peer 10.0.24.4 on R2.

<R2>display bgp peer 10.0.24.4 verbose

BGP Peer is 10.0.24.4,  remote AS 64511

Type: IBGP link

BGP version 4, Remote router ID 10.0.4.4

Update-group	ID														:	2

BGP	current	state														:	Established,	Up	for	00h29m13s

BGP	current	event													:	RecvKeepalive

BGP	last	state																	:	OpenConfirm

BGP	Peer	Up	count												:	2

Received	total	routes										:	0

Received	active	routes	total					:	0

Advertised	total	routes									:	0

Port:		Local	-	179	Remote						-		64495

Configured:	Connect-retry	Time	:	32	sec

Configured:	Active	Hold	Time					180	sec	Keepalive	Time:60	sec

Received		:	Active	Hold	Time				:	180	sec

Negotiated:	Active	Hold	Time				:	180	sec	Keepalive	Time:60	sec

Peer	optional	capabilities:

Peer supports bgp multi-protocol extension

Peer supports bgp route refresh capability

Peer supports bgp 4-byte-as capability

Address family IPv4 Unicast: advertised and received

Received: Total 32 messages

Update messages 		1

Open messages 			1

KeepAlive messages 		30

Notification messages  	0

Refresh messages 		0

Sent: Total 32 messages

Update messages 		0

Open messages 			2

KeepAlive messages 		30

Notification messages  	0

Refresh messages 		0

Authentication type configured: None

Last keepalive received: 2020-06-02 14:14:03-08:00

Minimum route advertisement interval is 15 seconds

Optional capabilities:

Route refresh capability has been enabled

4-byte-as capability has been enabled

Peer Preferred Value: 0

Routing policy configured:

No import update filter list

No export update filter list

No import prefix list

No export prefix list

No import route policy

Export route policy is: bgp

No import distribute policy

No export distribute policy

R2 receives an Update message from R4 and does not send an Update message to R4 (due to route filtering by the route-policy). However, the local BGP routing table does not contain the BGP route 10.1.1.0/24 advertised by R4.

# Trigger an import soft reset on R2 to allow R4 to re-send Update messages.

<R2>refresh bgp 10.0.24.4 import

# Check the number of Update messages sent and received on R2 again.

<R2>display bgp peer 10.0.24.4 verbose | in Update

Update-group ID: 2

BGP current event: RecvUpdate

Update messages 		2

Update messages 		0

The number of received Update messages increases. R2 receives the BGP route 10.1.1.0/24 from R4.

# Check detailed information about the BGP route 10.1.1.0 24 on R2 again.

<R2>display bgp routing-table 10.1.1.0 24

BGP	local	router	ID				:	10.0.2.2

Local	AS	number				:	64511

Paths														:	1	available,	1	best,	1	select

BGP	routing	table	entry	information	of	10.1.1.0/24:

RR-client	route.

From														:	10.0.12.1	(10.0.1.1)

Route	Duration						:	00h31m20s

Relay	IP	Nexthop					:	0.0.0.0

Relay	IP	Out-Interface	:	GigabitEthernet0/0/3

Original	nexthop					:	10.0.12.1

Qos	information					:	0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255

Advertised to such 1 peers:

10.0.23.3

Still only the BGP route advertised by R1 exists in the routing table. The Cluster_List attribute of the BGP route advertised by R4 contains the cluster ID of R2. As a result, R2 ignores the route advertised by R4.

    - Quiz

Do the routes advertised by BGP to EBGP peers carry the Originator_ID and Cluster_List attributes?

    - Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/2

ip address 10.0.12.1 255.255.255.0

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

interface LoopBack1

ip address 10.1.1.1 255.255.255.0

#

bgp 64511

router-id 10.0.1.1

peer 10.0.12.2 as-number 64511

#

ipv4-family unicast

undo synchronization

summary automatic

network 10.1.1.0 255.255.255.0

peer 10.0.12.2 enable

#

ospf 1 router-id 10.0.1.1

area 0.0.0.0

network 10.0.1.1 0.0.0.0

network 10.0.12.1 0.0.0.0

#

#

return

Configuration on R2

<R2>display current-configuration

#

sysname R2

#

acl number 2000

rule 5 permit

#

interface GigabitEthernet0/0/1

ip address 10.0.24.2 255.255.255.0

#

interface GigabitEthernet0/0/2

ip address 10.0.23.2 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.12.2 255.255.255.0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

interface LoopBack1

ip address 10.2.2.2 255.255.255.0

#

bgp 64511

router-id 10.0.2.2

peer 10.0.12.1 as-number 64511

peer 10.0.23.3 as-number 64511

peer 10.0.24.4 as-number 64511

#

ipv4-family unicast

undo synchronization

peer 10.0.12.1 enable

peer 10.0.12.1 reflect-client

peer 10.0.23.3 enable

peer 10.0.24.4 enable

peer 10.0.24.4 route-policy bgp export

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.12.2 0.0.0.0

network 10.0.23.2 0.0.0.0

network 10.0.24.2 0.0.0.0

#

route-policy bgp deny node 10

if-match acl 2000

#

return

Configuration on R3

<R3>display current-configuration

#

sysname R3

#

interface GigabitEthernet0/0/2

ip address 10.0.34.3 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.23.3 255.255.255.0

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

bgp 64511

router-id 10.0.3.3

peer 10.0.23.2 as-number 64511

peer 10.0.34.4 as-number 64511

#

ipv4-family unicast

undo synchronization

peer 10.0.23.2 enable

peer 10.0.23.2 reflect-client

peer 10.0.34.4 enable

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.23.3 0.0.0.0

network 10.0.34.3 0.0.0.0

#

return

Configuration on R4

<R4>display current-configuration

#

sysname R4

#

interface GigabitEthernet0/0/1

ip address 10.0.24.4 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.34.4 255.255.255.0

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.255

#

bgp 64511

router-id 10.0.4.4

peer 10.0.24.2 as-number 64511

peer 10.0.34.3 as-number 64511

#

ipv4-family unicast

undo synchronization

peer 10.0.24.2 enable

peer 10.0.34.3 enable

peer 10.0.34.3 reflect-client

#

ospf 1 router-id 10.0.4.4

area 0.0.0.0

network 10.0.4.4 0.0.0.0

network 10.0.24.4 0.0.0.0

network 10.0.34.4 0.0.0.0

#

return

  - Lab 4: BGP Route Selection

Introduction

      - Objectives

Upon completion of this task, you will be able to:

Change the AS_Path attribute to affect route selection.

Change the Local_Pref attribute to affect route selection.

Change the MED attribute to affect route selection.

Change the PrefVal attribute to affect route selection.

      - Networking Topology

              - BGP route selection

The preceding figure shows the device interconnection mode and IP addresses of interconnection interfaces. Loopback0 is created on each device, and its IP address is in the format of 10.0.x.x/32, where x indicates the device number. All devices use the IP addresses of Loopback0 as their BGP router IDs.

R1 resides in AS 100; R5 resides in AS 200; R2, R3, and R4 reside in AS 64512. OSPF runs in AS 64512, and OSPF is activated on the interconnection interfaces (excluding the interfaces connected to external ASs) and Loopback0 interfaces.

EBGP peer relationships are established through directly connected interfaces, and IBGP peer relationships are established through Loopback0 interfaces.

R1 and R5 share the following network segments: 172.16.1.0/24, 172.16.2.0/24, 172.16.3.0/24, and 172.16.4.0/24. R1 and R5 are configured to advertise routes destined for these network segments to their BGP routing tables for route selection.

      - Background

You are a network administrator of a company. The company's network uses BGP to access two service providers, ISP1 and ISP2. The company uses the private AS number 64512. The AS number of ISP1 is 100, and that of ISP2 is 200. The same network can be reached through AS 100 and AS 200. You can adjust the route direction by changing various BGP attributes.

    - Lab Configuration

      - Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF in AS 64512, and activate OSPF on the interconnection interfaces (excluding the interfaces connected to external ASs) and Loopback0 interfaces.

- Configure BGP peer relationships as planned, and configure R1 and R5 to advertise network segment routes to their BGP routing tables.

- On R1, configure a route-policy to change the AS_Path attribute of the BGP route 172.16.1.0/24 so that R3 preferentially selects the BGP route 172.16.1.0/24 advertised by R5.

- On R4, configure a route-policy to change the Local_Pref attribute of the BGP route 172.16.2.0/24 so that R3 preferentially selects the BGP route 172.16.2.0/24 advertised by R4.

- On R2, configure a route-policy to change the MED attribute of the BGP route 172.16.3.0/24 so that R3 preferentially selects the BGP route 172.16.3.0/24 advertised by R5.

- On R3, configure a route-policy to change the PrefVal attribute of the BGP route 172.16.4.0/24 so that R3 preferentially selects the BGP route 172.16.4.0/24 advertised by R4.

      - Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/2 and Loopback0 of R1.

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] ip address 10.0.12.1 255.255.255.0

[R1-GigabitEthernet0/0/2] quit

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.255

[R1-LoopBack0] quit

# Create multiple loopback interfaces on R1 so that R1 can advertise the loopback routes to the BGP routing table.

[R1]interface LoopBack1

[R1-LoopBack1] ip address 172.16.1.1 255.255.255.0

[R1-LoopBack1] quit

[R1]interface LoopBack2

[R1-LoopBack2] ip address 172.16.2.1 255.255.255.0

[R1-LoopBack2] quit

[R1]interface LoopBack3

[R1-LoopBack3] ip address 172.16.3.1 255.255.255.0

[R1-LoopBack3] quit

[R1]interface LoopBack4

[R1-LoopBack4] ip address 172.16.4.1 255.255.255.0

[R1-LoopBack4] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R2.

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.255

[R2-LoopBack0] quit

[R2]interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] ip address 10.0.23.2 255.255.255.0

[R2-GigabitEthernet0/0/2] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] ip address 10.0.12.2 255.255.255.0

[R2-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R3.

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.255

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] ip address 10.0.34.3 255.255.255.0

[R3-GigabitEthernet0/0/2] quit

[R3]interface GigabitEthernet0/0/3

[R3-GigabitEthernet0/0/3] ip address 10.0.23.3 255.255.255.0

[R3-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 of R4.

[R4]interface GigabitEthernet0/0/2

[R4-GigabitEthernet0/0/2] ip address 10.0.45.4 255.255.255.0

[R4-GigabitEthernet0/0/2] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] ip address 10.0.34.4 255.255.255.0

[R4-GigabitEthernet0/0/3] quit

[R4]interface LoopBack0

[R4-LoopBack0] ip address 10.0.4.4 255.255.255.255

[R4-LoopBack0] quit

# Configure IP addresses for GE0/0/3 and Loopback0 of R5.

[R5]interface LoopBack0

[R5-LoopBack0] ip address 10.0.5.5 255.255.255.255

[R5-LoopBack0] quit

[R5]interface GigabitEthernet0/0/3

[R5-GigabitEthernet0/0/3] ip address 10.0.45.5 255.255.255.0

[R5-GigabitEthernet0/0/3] quit

# Create multiple loopback interfaces on R5 so that R5 can advertise the loopback routes to the BGP routing table.

[R5]interface LoopBack1

[R5-LoopBack1] ip address 172.16.1.1 255.255.255.0

[R5-LoopBack1] quit

[R5]interface LoopBack2

[R5-LoopBack2] ip address 172.16.2.1 255.255.255.0

[R5-LoopBack2] quit

[R5]interface LoopBack3

[R5-LoopBack3] ip address 172.16.3.1 255.255.255.0

[R5-LoopBack3] quit

[R5]interface LoopBack4

[R5-LoopBack4] ip address 172.16.4.1 255.255.255.0

[R5-LoopBack4] quit

# On R2 and R4, ping the IP addresses of the interconnected devices to test the connectivity.

<R2>ping -c 1 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=80 ms

--- 10.0.12.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 80/80/80 ms

<R2>ping -c 1 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=20 ms

--- 10.0.23.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 20/20/20 ms

<R4>ping -c 1 10.0.34.3

PING 10.0.34.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.34.3: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.34.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

<R4>ping -c 1 10.0.45.5

PING 10.0.45.5: 56  data bytes, press CTRL_C to break

Reply from 10.0.45.5: bytes=56 Sequence=1 ttl=255 time=30 ms

--- 10.0.45.5 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/30/30 ms

Configure OSPF in AS 64512.

Configure R2, R3, and R4 use the IP addresses of Loopback0 as their router IDs, and activate OSPF on the interconnection interfaces (excluding the interfaces connected to external ASs) and Loopback0 interfaces.

# Configure R2, and activate OSPF on Loopback0 and GE0/0/2.

[R2]ospf 1 router-id 10.0.2.2

[R2-ospf-1] area 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.23.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] quit

[R2-ospf-1] quit

# Configure R3, and activate OSPF on Loopback0, GE0/0/2, and GE0/0/3.

[R3]ospf 1 router-id 10.0.3.3

[R3-ospf-1] area 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.23.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.34.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] quit

[R3-ospf-1] quit

# Configure R4, and activate OSPF on Loopback0 and GE0/0/3.

[R4]ospf 1 router-id 10.0.4.4

[R4-ospf-1] area 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.34.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] quit

[R4-ospf-1] quit

# Check the brief information about OSPF neighbor relationships on R3.

<R3>display ospf peer brief

OSPF Process 1 with Router ID 10.0.3.3

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/2             10.0.4.4         Full

0.0.0.0          GigabitEthernet0/0/3             10.0.2.2         Full

----------------------------------------------------------------------------

The command output shows that R3 has established neighbor relationships with R2 and R4.

# Check the OSPF routing table on R3.

<R3>display ospf routing

OSPF Process 1 with Router ID 10.0.3.3

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop						AdvRouter							Area

10.0.3.3/32								0					Stub							10.0.3.3								10.0.3.3								0.0.0.0

10.0.23.0/24							1					Transit					10.0.23.3							10.0.3.3								0.0.0.0

10.0.34.0/24							1					Transit					10.0.34.3							10.0.3.3								0.0.0.0

10.0.2.2/32								1					Stub							10.0.23.2							10.0.2.2								0.0.0.0

10.0.4.4/32								1					Stub							10.0.34.4							10.0.4.4								0.0.0.0

Total Nets: 5

Intra Area: 5  Inter Area: 0  ASE: 0  NSSA: 0

The command output shows that R3 has learned the Loopback0 routes from R2 and R4.

Configure BGP peer relationships.

Establish IBGP peer relationships through loopback interfaces, and establish EBGP peer relationships through interconnection interfaces.

# Configure R1.

[R1]bgp 100

[R1-bgp] router-id 10.0.1.1

[R1-bgp] peer 10.0.12.2 as 64512

# Configure R2.

[R2]bgp 64512

[R2-bgp] router-id 10.0.2.2

[R2-bgp] peer 10.0.3.3 as-number 64512

[R2-bgp] peer 10.0.3.3 connect-interface LoopBack0

[R2-bgp] peer 10.0.3.3 next-hop-local

[R2-bgp] peer 10.0.12.1 as-number 100

As OSPF is not activated on interconnection interfaces between ASs, R2 needs to be configured to change the next-hop address of routes to the IP address of its source interface when advertising them to the IBGP peer R3.

# Configure R3.

[R3]bgp 64512

[R3-bgp] router-id 10.0.3.3

[R3-bgp] peer 10.0.2.2 as-number 64512

[R3-bgp] peer 10.0.2.2 connect-interface LoopBack0

[R3-bgp] peer 10.0.4.4 as-number 64512

[R3-bgp] peer 10.0.4.4 connect-interface LoopBack0

# Configure R4.

[R4]bgp 64512

[R4-bgp] router-id 10.0.4.4

[R4-bgp] peer 10.0.3.3 as-number 64512

[R4-bgp] peer 10.0.3.3 connect-interface LoopBack0

[R4-bgp] peer 10.0.3.3 next-hop-local

[R4-bgp] peer 10.0.45.5 as-number 200

As OSPF is not activated on interconnection interfaces between ASs, R4 needs to be configured to change the next-hop address of routes to the IP address of its source interface when advertising them to the IBGP peer R3.

# Configure R5.

[R5]bgp 200

[R5-bgp] router-id 10.0.5.5

[R5-bgp] peer 10.0.45.4 as 64512

# Check the BGP peer relationship status on R2 and R4.

<R2>display bgp peer

BGP local router ID : 10.0.2.2

Local AS number : 64512

Total number of peers : 2	    Peers in established state : 2

Peer											V								AS					MsgRcvd		MsgSent		OutQ		Up/Down						State					PrefRcv

10.0.3.3								4							64512								27							30					0						00:03:49					Established				0

10.0.12.1							4							100										11							11					0						00:03:54					Established				0

<R4>display bgp peer

BGP local router ID : 10.0.4.4

Local AS number : 64512

Total number of peers : 2	    Peers in established state : 2

Peer											V						AS					MsgRcvd		MsgSent		OutQ		Up/Down						State					PrefRcv

10.0.3.3								4							64512							39							33					0						00:03:39						Established				0

10.0.45.5							4							200										4								6					0						00:02:54						Established				0

All the BGP peer relationships have been successfully established.

# Configure each of R1 and R5 to advertise the Loopback1, Loopback2, Loopback3, and Loopback4 routes to the BGP routing table.

[R1]bgp 100

[R1-bgp] network 172.16.1.0 24

[R1-bgp] network 172.16.2.0 24

[R1-bgp] network 172.16.3.0 24

[R1-bgp] network 172.16.4.0 24

[R5]bgp 200

[R5-bgp] network 172.16.1.0 24

[R5-bgp] network 172.16.2.0 24

[R5-bgp] network 172.16.3.0 24

[R5-bgp] network 172.16.4.0 24

# Check the BGP routing table on R3 to check whether R3 has learned these BGP routes successfully.

<R3>display	bgp	routing-table

BGP	Local	router	ID	is	10.0.3.3

Status	codes:	*	-	valid,	>	-	best,	d	-	damped,

h	-	history,		i	-	internal,	s	-	suppressed,	S	-	Stale

Origin	:	i	-	IGP,	e	-	EGP,	?	-	incomplete

Total	Number	of	Routes:	8

Network										NextHop					MED								LocPrf				PrefVal			Path/Ogn

*>i		172.16.1.0/24								10.0.2.2								0										100								0						100i

*	i																					10.0.4.4								0										100								0						200i

*>i		172.16.2.0/24								10.0.2.2								0										100								0						100i

*	i																					10.0.4.4								0										100								0						200i

*>i		172.16.3.0/24								10.0.2.2								0										100								0						100i

*	i																					10.0.4.4								0										100								0						200i

*>i		172.16.4.0/24								10.0.2.2								0										100								0						100i

*	i																					10.0.4.4								0										100								0						200i

R3 has learned the routes advertised by R1 and R5, and preferentially selects the route advertised by R2.

Change the AS_Path attribute.

On R1, configure a route-policy to change the AS_Path attribute of the BGP route 172.16.1.0/24 so that R3 preferentially selects the BGP route 172.16.1.0/24 advertised by R5.

# Create IP prefix list 1 to match the Loopback1 route.

[R1]ip ip-prefix 1 permit 172.16.1.0 24 greater-equal 24 less-equal 24

# Create a route-policy named hcip, create node 10, configure an if-match clause with IP prefix list 1 specified, and configure an apply clause to modify the AS_Path attribute of the matched route.

[R1]route-policy hcip permit node 10

[R1-route-policy] if-match ip-prefix 1

[R1-route-policy] apply as-path 300 400 additive

[R1-route-policy] quit

[R1]route-policy hcip permit node 20

Create an empty node in the route-policy to ensure that no operation is performed on the other three BGP routes.

# Apply the route-policy to the BGP routes to be advertised to the BGP peer R2.

[R1]bgp 100

[R1-bgp] peer 10.0.12.2 route-policy hcip export

# Trigger an export soft reset on R1 to update the advertised BGP routes.

<R1>refresh bgp all export

# Check detailed information about the BGP routes 172.16.1.0/24 on R3.

<R3>display bgp routing-table 172.16.1.0 24

BGP	local	router	ID				:	10.0.3.3

Local	AS	number				:	64512

Paths														:	2	available,	1	best,	1	select

BGP	routing	table	entry	information	of	172.16.1.0/24:

From:	10.0.4.4	(10.0.4.4)

Route	Duration						:	00h46m54s

Relay	IP	Nexthop					:	10.0.34.4

Relay	IP	Out-Interface:	GigabitEthernet0/0/2

Original	nexthop					:	10.0.4.4

Qos	information					:	0x0

AS-path 200, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255, IGP cost 1

Not advertised to any peer yet

BGP routing table entry information of 172.16.1.0/24:

From: 10.0.2.2 (10.0.2.2)

Route Duration						: 00h04m54s

Relay IP Nexthop				: 10.0.23.2

Relay IP Out-Interface: GigabitEthernet0/0/3

Original nexthop					: 10.0.2.2

Qos information					: 0x0

AS-path 100 300 400, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, pre 255, IGP cost 1, not preferred for AS-Path

Not advertised to any peer yet

R3 preferentially selects the BGP route 172.16.1.0/24 advertised by R4. This is because the AS_Path length in the route advertised by R2 is longer than that of the route advertised by R4.

Change the Local_Pref attribute.

On R4, configure a route-policy to change the Local_Pref attribute of the BGP route 172.16.2.0/24 so that R3 preferentially selects the BGP route 172.16.2.0/24 advertised by R4.

# Create IP prefix list 1 to match the BGP route 172.16.2.0/24.

[R4]ip ip-prefix 1 permit 172.16.2.0 24 greater-equal 24 less-equal 24

# Create a route-policy named hcip, create node 10, configure an if-match clause with IP prefix list 1 specified, and configure an apply clause to modify the Local_Pref attribute of the matched route.

[R4]route-policy hcip permit node 10

[R4-route-policy] if-match ip-prefix 1

[R4-route-policy] apply local-preference 200

[R4-route-policy] quit

[R4]route-policy hcip permit node 20

Create an empty node in the route-policy to ensure that no operation is performed on the other three BGP routes.

# Apply the route-policy to the BGP routes to be advertised to the BGP peer R3.

[R4]bgp 64512

[R4-bgp] peer 10.0.3.3 route-policy hcip export

# Trigger an export soft reset on R4 to update the advertised BGP routes.

<R4>refresh bgp all export

# Check detailed information about the BGP routes 172.16.2.0/24 on R3.

<R3>display bgp routing-table 172.16.2.0 24

BGP local router ID				: 10.0.3.3

Local AS number						: 64512

Paths															: 2 available, 1 best, 1 select

BGP routing table entry information of 172.16.2.0/24:

From															: 10.0.4.4 (10.0.4.4)

Route Duration							: 00h01m00s

Relay IP Nexthop					: 10.0.34.4

Relay IP Out-Interface	: GigabitEthernet0/0/2

Original nexthop						: 10.0.4.4

Qos information						: 0x0

AS-path 200, origin igp, MED 0, localpref 200, pref-val 0, valid, internal, best, select, active, pre 255, IGP cost 1

Not advertised to any peer yet

BGP routing table entry information of 172.16.2.0/24:

From: 10.0.2.2 (10.0.2.2)

Route Duration							: 00h07m09s

Relay IP Nexthop					: 10.0.23.2

Relay IP Out-Interface	: GigabitEthernet0/0/3

Original nexthop						: 10.0.2.2

Qos information						: 0x0

AS-path 100, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, pre 255, IGP cost 1, not preferred for Local_Pref

Not advertised to any peer yet

R3 preferentially selects the BGP route 172.16.2.0/24 advertised by R4. The Local_Pref value of the BGP route advertised by R2 is 100, which is smaller than the Local_Pref value 200 of the BGP route advertised by R3. As a result, the BGP route advertised by R2 is not preferentially selected.

Change the MED attribute.

On R2, configure a route-policy to change the MED attribute of the BGP route 172.16.3.0/24 so that R3 preferentially selects the BGP route 172.16.3.0/24 advertised by R5.

# Create IP prefix list 1 to match the BGP route 172.16.3.0/24.

[R2]ip ip-prefix 1 permit 172.16.3.0 24 greater-equal 24 less-equal 24

# Create a route-policy named hcip, create node 10, configure an if-match clause with IP prefix list 1 specified, and configure an apply clause to modify the MED attribute of the matched route.

[R2]route-policy hcip permit node 10

[R2-route-policy] if-match ip-prefix 1

[R2-route-policy] apply cost 200

[R2-route-policy] quit

[R2]route-policy hcip permit node 20

Create an empty node in the route-policy to ensure that no operation is performed on the other three BGP routes.

# Apply the route-policy to the BGP routes received from the BGP peer R1.

[R2]bgp 64512

[R2-bgp] peer 10.0.12.1 route-policy hcip import

# Trigger an import soft reset on R2 to update the received BGP routes.

<R2>refresh bgp all import

# Configure R3 to compare the MED values of the BGP routes received from peers in different ASs.

[R3]bgp 64512

[R3-bgp] compare-different-as-med

By default, BGP does not compare the MED values of routes received from peers in different ASs.

# Check detailed information about the BGP routes 172.16.3.0/24 on R3.

<R3>display bgp routing-table 172.16.3.0 24

BGP local router ID				: 10.0.3.3

Local AS number						: 64512

Paths															: 2 available, 1 best, 1 select

BGP routing table entry information of 172.16.3.0/24:

From															: 10.0.4.4 (10.0.4.4)

Route Duration							: 00h14m27s

Relay IP Nexthop					: 10.0.34.4

Relay IP Out-Interface	: GigabitEthernet0/0/2

Original nexthop						: 10.0.4.4

Qos information						: 0x0

AS-path 200, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255, IGP cost 1

Not advertised to any peer yet

BGP routing table entry information of 172.16.3.0/24:

From: 10.0.2.2 (10.0.2.2)

Route Duration							: 00h03m00s

Relay IP Nexthop					: 10.0.23.2

Relay IP Out-Interface	: GigabitEthernet0/0/3

Original nexthop						: 10.0.2.2

Qos information						: 0x0

AS-path 100, origin igp, MED 200, localpref 100, pref-val 0, valid, internal, pre 255, IGP cost 1, not preferred for MED

Not advertised to any peer yet

The MED value of the BGP route 172.16.3.0/24 advertised by R2 is 200, and that of the BGP route advertised by R4 is 0. In this case, R3 preferentially selects the BGP route with a smaller MED value. As a result, the BGP route advertised by R2 is not preferentially selected.

Change the PrefVal attribute.

On R3, configure a route-policy to change the PrefVal attribute of the BGP route 172.16.4.0/24 so that R3 preferentially selects the BGP route 172.16.4.0/24 advertised by R4.

# Create IP prefix list 1 to match the BGP route 172.16.4.0/24.

[R3]ip ip-prefix 1 permit 172.16.4.0 24 greater-equal 24 less-equal 24

# Create a route-policy named hcip, create node 10, configure an if-match clause with IP prefix list 1 specified, and configure an apply clause to modify the PrefVal attribute of the matched route.

[R3]route-policy hcip permit node 10

[R3-route-policy] if-match ip-prefix 1

[R3-route-policy] apply preferred-value 300

[R3-route-policy] quit

[R3]route-policy hcip permit node 20

Create an empty node in the route-policy to ensure that no operation is performed on the other three BGP routes.

# Apply the route-policy to the BGP routes received from the BGP peer R4.

[R3]bgp 64512

[R3-bgp] peer 10.0.4.4 route-policy hcip import

# Trigger an import soft reset on R3 to update the received BGP routes.

<R3>refresh bgp all import

# Check detailed information about the BGP routes 172.16.4.0/24 on R3.

<R3>display bgp routing-table 172.16.4.0 24

BGP local router ID				: 10.0.3.3

Local AS number						: 64512

Paths															: 2 available, 1 best, 1 select

BGP routing table entry information of 172.16.4.0/24:

From															: 10.0.4.4 (10.0.4.4)

Route Duration							: 00h01m22s

Relay IP Nexthop					: 10.0.34.4

Relay IP Out-Interface	: GigabitEthernet0/0/2

Original nexthop						: 10.0.4.4

Qos information						: 0x0

AS-path 200, origin igp, MED 0, localpref 100, pref-val 300, valid, internal, best, select, active, pre 255, IGP cost 1

Not advertised to any peer yet

BGP routing table entry information of 172.16.4.0/24:

From: 10.0.2.2 (10.0.2.2)

Route Duration							: 00h00m04s

Relay IP Nexthop					: 10.0.23.2

Relay IP Out-Interface	: GigabitEthernet0/0/3

Original nexthop						: 10.0.2.2

Qos information						: 0x0

AS-path 100, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, pre255, IGP cost 1, not preferred for PreVal

Not advertised to any peer yet

The PrefVal value of the BGP route 172.16.3.0/24 advertised by R4 is 300, and that of the route advertised by R2 is 0. In this case, R3 preferentially selects the BGP route with a larger PrefVal value. As a result, R3 preferentially selects the BGP route advertised by R4.

# Check the BGP routing table on R3.

<R3>display	bgp	routing-table

BGP	Local	router	ID	is	10.0.3.3

Status	codes:	*	-	valid,	>	-	best,	d	-	damped,

h	-	history,		i	-	internal,	s	-	suppressed,	S	-	Stale

Origin	:	i	-	IGP,	e	-	EGP,	?	-	incomplete

Total	Number	of	Routes:	8

Network								NextHop						MED								LocPrf			PrefVal			Path/Ogn

*>i		172.16.1.0/24						10.0.4.4								0										100								0						200i

*	i																			10.0.2.2								0										100								0						100	300	400i

*>i		172.16.2.0/24						10.0.4.4								0										200								0						200i

*	i																			10.0.2.2								0										100								0						100i

*>i		172.16.3.0/24						10.0.4.4								0										100								0						200i

*	i																			10.0.2.2								200								100								0						100i

*>i		172.16.4.0/24						10.0.4.4								0										100								300				200i

*	i																			10.0.2.2								0										100								0						100i

The command output show that all the routes advertised by R4 are preferentially selected.

(Optional) Verify that a locally originated BGP route takes precedence over a BGP route learned from a peer.

Create Loopback1 on R2 and configure R2 to advertise the Loopback1 route to the OSPF routing table. Then, configure R2 and R3 to advertise the Loopback1 route to their BGP routing tables. In this case, the BGP routing table of R3 will contain two BGP routes to Loopback1 on R2.

# Create Loopback1 on R2 and set its IP address to 10.2.2.2/32.

[R2]interface LoopBack1

[R2-LoopBack1] ip address 10.2.2.2 255.255.255.255

[R2-LoopBack1] quit

# Activate OSPF on Loopback1.

[R2]ospf 1

[R2-ospf-1]area 0

[R2-ospf-1-area-0.0.0.0] network 10.2.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] quit

[R2-ospf-1] quit

# Check the OSPF route 10.2.2.2/32 on R3.

<R3>display ospf routing 10.2.2.2

OSPF Process 1 with Router ID 10.0.3.3

Destination		: 10.2.2.2/32

AdverRouter	:	10.0.2.2																	Area						:	0.0.0.0

Cost								:	1																							Type						:	Stub

NextHop				:	10.0.23.2																Interface		:	GigabitEthernet0/0/3

Priority						:	Medium																	Age						:	00h01m19s

R3 has learned the Loopback1 route from R2.

# Configure R2 and R3 to advertise their Loopback1 routes to their BGP routing tables.

[R2]bgp 64512

[R2-bgp] network 10.2.2.2 32

[R3]bgp 64512

[R3-bgp] network 10.2.2.2 32

# Check detailed information about the BGP routes 10.2.2.2/32 on R3.

<R3>display bgp routing-table 10.2.2.2 32

BGP local router ID				: 10.0.3.3

Local AS number						: 64512

Paths															: 2 available, 1 best, 1 select

BGP routing table entry information of 10.2.2.2/32:

Network route.

From															: 0.0.0.0 (0.0.0.0)

Route Duration							: 00h00m21s

Direct Out-interface			: GigabitEthernet0/0/3

Original nexthop						: 10.0.23.2

Qos information						: 0x0

AS-path Nil, origin igp, MED 1, pref-val 0, valid, local, best, select, pre 10

Advertised to such 2 peers:

10.0.2.2

10.0.4.4

BGP routing table entry information of 10.2.2.2/32:

From															: 10.0.2.2 (10.0.2.2)

Route Duration							: 00h00m50s

Relay IP Nexthop					: 10.0.23.2

Relay IP Out-Interface	: GigabitEthernet0/0/3

Original nexthop						: 10.0.2.2

Qos information						: 0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, pre 255, IGP cost 1, not preferred for route type

Not advertised to any peer yet

R3 preferentially selects the locally advertised BGP route 10.2.2.2/32. The BGP route 10.2.2.2/32 advertised by R1 is not preferentially selected because its route type is not preferred. A locally originated route takes precedence over a route learned from a BGP peer.

(Optional) Change the Origin attribute.

Create Loopback5 on R1 and R5, and configure the devices to advertise the Loopback5 routes to their BGP routing tables. Verify that the BGP route whose Origin attribute is IGP takes precedence over the BGP route whose Origin attribute is Incomplete.

# Create Loopback5 on R1 and R5 and set its IP address to 172.16.5.1/24.

[R1]interface LoopBack 5

[R1-LoopBack5] ip address 172.16.5.1 24

[R1-LoopBack5] quit

[R5]interface LoopBack 5

[R5-LoopBack5] ip address 172.16.5.1 24

[R5-LoopBack5] quit

# Configure R1 and R5 to advertise Loopback5 routes to their BGP routing tables using the network command.

[R1]bgp 100

[R1-bgp] network 172.16.5.0 24

[R5]bgp 200

[R5-bgp] network 172.16.5.0 24

# Check the BGP routing table on R3.

<R3>display	bgp	routing-table

BGP	Local	router	ID	is	10.0.3.3

Status	codes:	*	-	valid,	>	-	best,	d	-	damped,

h	-	history,		i	-	internal,	s	-	suppressed,	S	-	Stale

Origin	:	i	-	IGP,	e	-	EGP,	?	-	incomplete

Total	Number	of	Routes:	12

Network										NextHop								MED					LocPrf				PrefVal		Path/Ogn

*>			10.2.2.2/32									0.0.0.0									1																				0						i

*	i																				10.0.2.2								0										100								0						i

*>i		172.16.1.0/24							10.0.4.4								0										100								0						200i

*	i																				10.0.2.2								0										100								0						100	300	40

0i

*>i		172.16.2.0/24							10.0.4.4								0										200								0						200i

*	i																				10.0.2.2								0										100								0						100i

*>i		172.16.3.0/24							10.0.4.4								0										100								0						200i

*	i																				10.0.2.2								200								100								0						100i

*>i		172.16.4.0/24							10.0.4.4								0										100								300				200i

*	i																				10.0.2.2								0										100								0						100i

*>i		172.16.5.0/24							10.0.2.2								0										100								0						100i

*	i																				10.0.4.4								0										100								0						200i

R3 preferentially selects the BGP route 172.16.5.0/24 received from R2 (and originally advertised by R1). In this case, the Origin attribute of the BGP routes advertised by R2 and R4 is IGP.

# Delete the Loopback5 route advertised by R1 to the BGP routing table.

[R1]bgp 100

[R1-bgp] undo network 172.16.5.0 24

# Create IP prefix list 2 to match the Loopback5 route 172.16.5.0/24 of R1.

[R1]ip ip-prefix 2 permit 172.16.5.0 24 greater-equal 24 less-equal 24

# Create a route-policy named origin, create node 10, and configure an if-match clause with IP prefix list 2 specified.

[R1]route-policy origin permit node 10

[R1-route-policy] if-match ip-prefix 2

[R1-route-policy] quit

# Run the import-route direct command on R1 to advertise direct routes to the BGP routing table, and specify route-policy origin to import only the Loopback5 route.

[R1]bgp 100

[R1-bgp] import-route direct route-policy origin

# Check detailed information about the BGP routes 172.16.5.0/24 on R3.

<R3>display bgp routing-table 172.16.5.0 24

BGP	local	router	ID				:	10.0.3.3

Local	AS	number				:	64512

Paths														:	2	available,	1	best,	1	select

BGP	routing	table	entry	information	of	172.16.5.0/24:

From														:	10.0.4.4	(10.0.4.4)

Route	Duration						:	00h03m53s

Relay	IP	Nexthop					:	10.0.34.4

Relay	IP	Out-Interface:	GigabitEthernet0/0/2

Original	nexthop					:	10.0.4.4

Qos	information					:	0x0

AS-path	200,	origin	igp,	MED	0,	localpref	100,	pref-val	0,	valid,	internal,	bes

t,	select,	active,	pre	255,	IGP	cost	1

Not	advertised	to	any	peer	yet

BGP	routing	table	entry	information	of	172.16.5.0/24:

From														:	10.0.2.2	(10.0.2.2)

Route	Duration						:	00h01m27s

Relay	IP	Nexthop					:	10.0.23.2

Relay	IP	Out-Interface	:	GigabitEthernet0/0/3

Original	nexthop					:	10.0.2.2

Qos	information					:	0x0

AS-path 100, origin incomplete, MED 0, localpref 100, pref-val 0, valid, internal, pre 255, IGP cost 1, not preferred for Origin

Not advertised to any peer yet

R3 preferentially selects the BGP route 172.16.5.0/24 advertised by R4.

The Origin attribute of the BGP route 172.16.5.0/24 received from R2 (and originally advertised by R1) is incomplete (advertised to the BGP routing table using the import-route command). Due to the Origin attribute value, this route is not preferentially selected.

(Optional) Verify that an EBGP route takes precedence over an IBGP route.

Create Loopback6 on R1 and R3, and configure the devices to advertise the Loopback6 routes to their BGP routing tables. Observe the route selection result on R2.

# Create Loopback6 on R1 and R3.

[R1]interface LoopBack 6

[R1-LoopBack6] ip address 172.16.6.1 24

[R1-LoopBack6] quit

[R3]interface LoopBack 6

[R3-LoopBack6] ip address 172.16.6.1 24

[R3-LoopBack6] quit

# Configure R1 and R3 to advertise their Loopback6 routes to the BGP routing tables.

[R1]bgp 100

[R1-bgp] network 172.16.6.0 24

[R3]bgp 64512

[R3-bgp]network 172.16.6.0 24

# Check detailed information about the BGP routes 172.16.6.0/24 on R2.

<R2>display bgp routing-table 172.16.6.0 24

BGP local router ID			: 10.0.2.2

Local AS number					: 64512

Paths														: 2 available, 1 best, 1 select

BGP routing table entry information of 172.16.6.0/24:

From														: 10.0.3.3 (10.0.3.3)

Route Duration						: 00h03m13s

Relay IP Nexthop				: 10.0.23.3

Relay IP Out-Interface: GigabitEthernet0/0/2

Original nexthop					: 10.0.3.3

Qos information					: 0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255, IGP cost 1

Advertised to such 1 peers:

10.0.12.1

BGP routing table entry information of 172.16.6.0/24:

From													: 10.0.12.1 (10.0.1.1)

Route Duration					: 00h03m13s

Direct Out-interface	: GigabitEthernet0/0/3

Original nexthop				: 10.0.12.1

Qos information				: 0x0

AS-path 100, origin igp, MED 0, pref-val 0, valid, external, pre 255, not preferred for AS-Path

Not advertised to any peer yet

R2 preferentially selects the BGP route 172.16.6.0/24 advertised by R3 due to the AS_Path value.

# On R3, configure a route-policy to add an AS_Path value to the BGP route 172.16.6.0/24.

[R3]ip ip-prefix 2 permit 172.16.6.0 24 greater-equal 24 less-equal 24

[R3]route-policy as_path permit node 10

[R3-route-policy] if-match ip-prefix 2

[R3-route-policy] apply as-path 300 additive

[R3-route-policy] quit

[R3]route-policy as_path permit node 20

[R3]bgp 64512

[R3-bgp] peer 10.0.2.2 route-policy as_path export

# Trigger an export soft reset on R3 to update the advertised BGP routes.

<R3>refresh bgp all export

# Check detailed information about the BGP routes 172.16.6.0/24 on R2 again.

<R2>display bgp routing-table 172.16.6.0 24

BGP local router ID		: 10.0.2.2

Local AS number				: 64512

Paths													: 2 available, 1 best, 1 select

BGP routing table entry information of 172.16.6.0/24:

From													: 10.0.12.1 (10.0.1.1)

Route Duration					: 00h23m46s

Direct Out-interface	: GigabitEthernet0/0/3

Original nexthop				: 10.0.12.1

Qos information				: 0x0

AS-path 100, origin igp, MED 0, pref-val 0, valid, external, best, select, active, pre 255

Advertised to such 1 peers:

10.0.3.3

BGP routing table entry information of 172.16.6.0/24:

From														: 10.0.3.3 (10.0.3.3)

Route Duration						: 00h00m29s

Relay IP Nexthop				: 10.0.23.3

Relay IP Out-Interface: GigabitEthernet0/0/2

Original nexthop					: 10.0.3.3

Qos information					: 0x0

AS-path 300, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, pre 255, IGP cost 1, not preferred for peer type

Not advertised to any peer yet

The BGP route from R3 is not preferentially selected because BGP preferentially selects the route from an EBGP peer when the other route attributes are the same.

(Optional) Verify that BGP preferentially selects the route with the smallest IGP cost to the next hop.

Establish an IBGP peer relationship between R2 and R4 through loopback interfaces. Create Loopback7 on R2 and R3, and configure the devices to advertise the Loopback7 routes to their BGP routing tables. Then, observe the BGP route selection result on R4.

# Establish an IBGP peer relationship between R2 and R4.

[R2]bgp 64512

[R2-bgp] peer 10.0.4.4 as-number 64512

[R2-bgp] peer 10.0.4.4 connect-interface LoopBack 0

[R4]bgp 64512

[R4-bgp] peer 10.0.2.2 as-number 64512

[R4-bgp] peer 10.0.2.2 connect-interface LoopBack0

# Check the IBGP peer relationship status.

[R4]display	bgp	peer

BGP	local	router	ID	:	10.0.4.4

Local	AS	number	:	64512

Total	number	of	peers	:	3				Peers	in	established	state	:	3

Peer											V							AS				MsgRcvd		MsgSent				OutQ		Up/Down			State							PrefRcv

10.0.2.2								4							64512								7								3							0						00:00:01			Established				7

10.0.3.3								4							64512							37							36							0						00:31:57			Established				2

10.0.45.5							4									200							38							36							0						00:31:58			Established				5

The command output shows that the IBGP peer relationship has been successfully established.

# Create Loopback7 on R2 and R4, and configure the devices to advertise the Loopback7 routes to their BGP routing tables.

[R2]interface LoopBack 7

[R2-LoopBack7] ip address 172.16.7.1 24

[R2-LoopBack7] quit

[R2]bgp 64512

[R2-bgp] network 172.16.7.0 24

[R3]interface LoopBack 7

[R3-LoopBack7] ip address 172.16.7.1 24

[R3-LoopBack7] quit

[R3]bgp 64512

[R3-bgp] network 172.16.7.0 24

# Check detailed information about the BGP routes 172.16.7.0/24 on R4.

[R4]dis bgp routing-table 172.16.7.0 24

BGP local router ID			: 10.0.4.4

Local AS number					: 64512

Paths														: 2 available, 1 best, 1 select

BGP routing table entry information of 172.16.7.0/24:

From														: 10.0.3.3 (10.0.3.3)

Route Duration						: 00h10m48s

Relay IP Nexthop				: 10.0.34.3

Relay IP Out-Interface: GigabitEthernet0/0/3

Original nexthop					: 10.0.3.3

Qos information					: 0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, best, select, active, pre 255, IGP cost 1

Advertised to such 1 peers:

10.0.45.5

BGP routing table entry information of 172.16.7.0/24:

From														: 10.0.2.2 (10.0.2.2)

Route Duration						: 00h11m00s

Relay IP Nexthop				: 10.0.34.3

Relay IP Out-Interface: GigabitEthernet0/0/3

Original nexthop					: 10.0.2.2

Qos information					: 0x0

AS-path Nil, origin igp, MED 0, localpref 100, pref-val 0, valid, internal, pre 255, IGP cost 2, not preferred for IGP cost

Not advertised to any peer yet

R4 preferentially selects the BGP route advertised by R3 because its IGP cost is 1, which is lower than the IGP cost 2 of the BGP route advertised by R2.

The BGP route advertised by R2 is not preferentially selected due to the IGP cost.

----End

    - Quiz

Consider whether a routing policy can be used to delete an AS number from the AS_Path attribute.

    - Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/2

ip address 10.0.12.1 255.255.255.0

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

interface LoopBack1

ip address 172.16.1.1 255.255.255.0

#

interface LoopBack2

ip address 172.16.2.1 255.255.255.0

#

interface LoopBack3

ip address 172.16.3.1 255.255.255.0

#

interface LoopBack4

ip address 172.16.4.1 255.255.255.0

#

interface LoopBack5

ip address 172.16.5.1 255.255.255.0

#

interface LoopBack6

ip address 172.16.6.1 255.255.255.0

#

bgp 100

router-id 10.0.1.1

peer 10.0.12.2 as-number 64512

#

ipv4-family unicast

undo synchronization

network 172.16.1.0 255.255.255.0

network 172.16.2.0 255.255.255.0

network 172.16.3.0 255.255.255.0

network 172.16.4.0 255.255.255.0

network 172.16.6.0 255.255.255.0

import-route direct route-policy origin

peer 10.0.12.2 enable

peer 10.0.12.2 route-policy hcip export

#

route-policy hcip permit node 10

if-match ip-prefix 1

apply as-path 300 400 additive

#

route-policy hcip permit node 20

#

route-policy origin permit node 10

if-match ip-prefix 2

#

ip ip-prefix 1 index 10 permit 172.16.1.0 24 greater-equal 24 less-equal 24

ip ip-prefix 2 index 10 permit 172.16.5.0 24 greater-equal 24 less-equal 24

#

Return

Configuration on R2

#

sysname R2

#

interface GigabitEthernet0/0/2

ip address 10.0.23.2 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.12.2 255.255.255.0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

interface LoopBack1

ip address 10.2.2.2 255.255.255.255

#

interface LoopBack7

ip address 172.16.7.1 255.255.255.0

#

bgp 64512

router-id 10.0.2.2

peer 10.0.3.3 as-number 64512

peer 10.0.3.3 connect-interface LoopBack0

peer 10.0.4.4 as-number 64512

peer 10.0.4.4 connect-interface LoopBack0

peer 10.0.12.1 as-number 100

#

ipv4-family unicast

undo synchronization

network 10.2.2.2 255.255.255.255

network 172.16.7.0 255.255.255.0

peer 10.0.3.3 enable

peer 10.0.3.3 next-hop-local

peer 10.0.4.4 enable

peer 10.0.12.1 enable

peer 10.0.12.1 route-policy hcip import

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.23.2 0.0.0.0

network 10.2.2.2 0.0.0.0

#

route-policy hcip permit node 10

if-match ip-prefix 1

apply cost 200

#

route-policy hcip permit node 20

#

ip ip-prefix 1 index 10 permit 172.16.3.0 24 greater-equal 24 less-equal 24

#

ip route-static 10.0.1.1 255.255.255.255 10.0.12.1

#

return

Configuration on R3

#

sysname R3

#

interface GigabitEthernet0/0/2

ip address 10.0.34.3 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.23.3 255.255.255.0

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

interface LoopBack6

ip address 172.16.6.1 255.255.255.0

#

interface LoopBack7

ip address 172.16.7.1 255.255.255.0

#

bgp 64512

router-id 10.0.3.3

peer 10.0.2.2 as-number 64512

peer 10.0.2.2 connect-interface LoopBack0

peer 10.0.4.4 as-number 64512

peer 10.0.4.4 connect-interface LoopBack0

#

ipv4-family unicast

undo synchronization

compare-different-as-med

network 10.2.2.2 255.255.255.255

network 172.16.6.0 255.255.255.0

network 172.16.7.0 255.255.255.0

peer 10.0.2.2 enable

peer 10.0.2.2 route-policy as_path export

peer 10.0.4.4 enable

peer 10.0.4.4 route-policy hcip import

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.23.3 0.0.0.0

network 10.0.34.3 0.0.0.0

#

route-policy hcip permit node 10

if-match ip-prefix 1

apply preferred-value 300

#

route-policy hcip permit node 20

#

route-policy as_path permit node 10

if-match ip-prefix 2

apply as-path 300 additive

#

route-policy as_path permit node 20

#

ip ip-prefix 1 index 10 permit 172.16.4.0 24 greater-equal 24 less-equal 24

ip ip-prefix 2 index 10 permit 172.16.6.0 24 greater-equal 24 less-equal 24

#

return

Configuration on R4

#

sysname R4

#

interface GigabitEthernet0/0/2

ip address 10.0.45.4 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.34.4 255.255.255.0

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.255

#

bgp 64512

router-id 10.0.4.4

peer 10.0.2.2 as-number 64512

peer 10.0.2.2 connect-interface LoopBack0

peer 10.0.3.3 as-number 64512

peer 10.0.3.3 connect-interface LoopBack0

peer 10.0.45.5 as-number 200

#

ipv4-family unicast

undo synchronization

peer 10.0.2.2 enable

peer 10.0.3.3 enable

peer 10.0.3.3 route-policy hcip export

peer 10.0.3.3 next-hop-local

peer 10.0.45.5 enable

#

ospf 1 router-id 10.0.4.4

area 0.0.0.0

network 10.0.4.4 0.0.0.0

network 10.0.34.4 0.0.0.0

#

route-policy hcip permit node 10

if-match ip-prefix 1

apply local-preference 200

#

route-policy hcip permit node 20

#

ip ip-prefix 1 index 10 permit 172.16.2.0 24 greater-equal 24 less-equal 24

#

ip route-static 10.0.5.5 255.255.255.255 10.0.45.5

#

return

Configuration on R5

#

sysname R5

#

interface GigabitEthernet0/0/3

ip address 10.0.45.5 255.255.255.0

#

interface LoopBack0

ip address 10.0.5.5 255.255.255.255

#

interface LoopBack1

ip address 172.16.1.1 255.255.255.0

#

interface LoopBack2

ip address 172.16.2.1 255.255.255.0

#

interface LoopBack3

ip address 172.16.3.1 255.255.255.0

#

interface LoopBack4

ip address 172.16.4.1 255.255.255.0

#

interface LoopBack5

ip address 172.16.5.1 255.255.255.0

#

bgp 200

router-id 10.0.5.5

peer 10.0.45.4 as-number 64512

#

ipv4-family unicast

undo synchronization

network 172.16.1.0 255.255.255.0

network 172.16.2.0 255.255.255.0

network 172.16.3.0 255.255.255.0

network 172.16.4.0 255.255.255.0

network 172.16.5.0 255.255.255.0

peer 10.0.45.4 enable

#

ip route-static 10.0.4.4 255.255.255.255 10.0.45.4

#

return

Routing Policy and Routing Control

Route Import and Control

Introduction

Objectives

Upon completion of this task, you will be able to:

Use a route-policy to filter routes to be imported.

Use a route-policy to set route flags and filter labeled routes.

Use a filter-policy to filter routes to be imported into the OSPF routing table.

Networking Topology

              - Route import and control

The preceding figure shows the device interconnection mode and interconnection addresses. Loopback0 is created for each device. The IP address of Loopback0 is 10.0.x.x/32, where x indicates a device ID. OSPF is configured on interconnection interfaces and Loopback0 interfaces of R1, R2, and R3.

R3 and R4 belong to IS-IS area 49.0001 and both are Level-1 routers. The system IDs of R3 and R4 are in the format of 0000.0000.000x, where x indicates a device ID.

On R1, there are three network segments of services A, B, and C (simulated using routes destined for Loopback1, Loopback2, and Loopback3, respectively). On R1, direct routes are imported to the OSPF routing table. Routers within an OSPF area, however, do not need to import the route destined for the network segment of service C. Configure a route-policy on R1 to filter direct routes to be imported.

R2 does not need the route destined for service A's network segment; R3 needs the routes destined for network segments of services A and B. A filter-policy needs to be configured on R2 to filter the routes to be accepted by OSPF.

Routers in the IS-IS domain need to access service A. Therefore, route re-distribution needs to be performed on R3 to import OSPF routes to IS-IS. Routers in the IS-IS domain do not need to access service B. Therefore, when direct routes are imported on R1, the routes of the network segment of service B are marked with different route tags. When re-distribution is performed on R3, the route destined for the network segment of service B are filtered according to the route tags.

Background

The local enterprise network has two routing areas. One area runs OSPF, and the other area runs IS-IS. The border router in the OSPF area is connected to some service network segments of other enterprises. To allow the local enterprise network device to access these service network segments of other enterprises, import routes destined for the service network segments into the OSPF area. To enable devices in the IS-IS area to access those service network segments, import OSPF external routes to the IS-IS routing table. Different departments on the local enterprise network have different requirements for service network segment access. Therefore, route-policies and filter-policies need to be deployed to filter routes to be accepted and advertised.

Lab Configuration

Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF on each Loopback0 interface and the interfaces that connect R1, R2, R3, and R4. Configure an IS-IS neighbor relationship between R3 and R4.

- On R1, import direct routes to the OSPF routing table; configure a route-policy not to import routes destined for the service C network segment; add route tags 10 and 20 to the routes destined for network segments of services A and B, respectively.

- Configure a filter-policy on R2 to filter OSPF routes to be accepted. Only the routes destined for the service B network segment can be accepted.

- Import OSPF routes to the IS-IS routing table on R3. Use a route-policy to match route flags and import only OSPF external route destined for the service A network segment.

Procedure

          - Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure IP addresses for GE0/0/2 and Loopback0 on R1.

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] ip address 10.0.12.1 255.255.255.0

[R1-GigabitEthernet0/0/2] quit

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.255

[R1-LoopBack0] quit

# Create multiple loopback interfaces on R1 to simulate network segments of services A, B, and C.

[R1]interface LoopBack1

[R1-LoopBack1] ip address 172.16.1.1 255.255.255.0

[R1-LoopBack1] quit

[R1]interface LoopBack2

[R1-LoopBack2] ip address 172.16.2.1 255.255.255.0

[R1-LoopBack2] quit

[R1]interface LoopBack3

[R1-LoopBack3] ip address 172.16.3.1 255.255.255.0

[R1-LoopBack3] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 on R2.

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.255

[R2-LoopBack0] quit

[R2]interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] ip address 10.0.23.2 255.255.255.0

[R2-GigabitEthernet0/0/2] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] ip address 10.0.12.2 255.255.255.0

[R2-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/2, GE0/0/3, and Loopback0 on R3.

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.255

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] ip address 10.0.34.3 255.255.255.0

[R3-GigabitEthernet0/0/2] quit

[R3]interface GigabitEthernet0/0/3

[R3-GigabitEthernet0/0/3] ip address 10.0.23.3 255.255.255.0

[R3-GigabitEthernet0/0/3] quit

# Configure IP addresses for GE0/0/3 and Loopback0 on R4.

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] ip address 10.0.34.4 255.255.255.0

[R4-GigabitEthernet0/0/3] quit

[R4]interface LoopBack0

[R4-LoopBack0] ip address 10.0.4.4 255.255.255.255

[R4-LoopBack0] quit

# Check IP address connectivity on R2 and R4.

<R2>ping -c 1 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=80 ms

--- 10.0.12.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 80/80/80 ms

<R2>ping -c 1 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=20 ms

--- 10.0.23.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 20/20/20 ms

<R4>ping -c 1 10.0.34.3

PING 10.0.34.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.34.3: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.34.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

          - Configure OSPF and IS-IS.

On R1, R2, and R3, use the IP address of Loopback0 as a router ID, and activate OSPF on the interconnected interfaces and Loopback0 interfaces.

# Configure R1.

[R1]ospf 1 router-id 10.0.1.1

[R1-ospf-1] area 0

[R1-ospf-1-area-0.0.0.0] network 10.0.1.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] network 10.0.12.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] quit

[R1-ospf-1] quit

# Configure R2.

[R2]ospf 1 router-id 10.0.2.2

[R2-ospf-1] area 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.12.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.23.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] quit

[R2-ospf-1] quit

# Configure R3.

[R3]ospf 1 router-id 10.0.3.3

[R3-ospf-1] area 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.23.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] quit

[R3-ospf-1] quit

# Check brief information about OSPF neighbors on R2.

<R2>display ospf peer brief

OSPF Process 1 with Router ID 10.0.2.2

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/2             10.0.3.3         Full

0.0.0.0          GigabitEthernet0/0/3             10.0.1.1         Full

----------------------------------------------------------------------------

OSPF neighbor relationships have been established between R1 and R2, and between R2 and R3.

Configure IS-IS on R3 and R4. Set the area ID to 49.0001. Set a system ID in the format of 0000.0000.000x (x indicates a device ID). Configure R3 and R4 as Level-1 routers. Activate IS-IS on the interconnected interfaces and R4's Loopback0 interface.

# Configure R3.

[R3]isis 1

[R3-isis-1] is-level level-1

[R3-isis-1] network-entity 49.0001.0000.0000.0003.00

[R3-isis-1] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] isis enable 1

[R3-GigabitEthernet0/0/2] quit

# Configure R4.

[R4]isis 1

[R4-isis-1] is-level level-1

[R4-isis-1] network-entity 49.0001.0000.0000.0004.00

[R4-isis-1] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] isis enable 1

[R4-GigabitEthernet0/0/3] quit

[R4]interface LoopBack 0

[R4-LoopBack0] isis enable 1

[R4-LoopBack0] quit

# Check the IS-IS neighbor status on R3.

<R3>display isis peer

Peer information for ISIS(1)

System Id     Interface           Circuit Id       State   HoldTime  Type     PRI

-------------------------------------------------------------------------------

0000.0000.0004  GE0/0/2           0000.0000.0001.01 Up     22s       L1       64

Total Peer(s): 1

          - Import direct routes on R1.

On R1, import direct routes to the OSPF routing table, configure a route-policy to filter out the routes destined for the network segment of service C, and add route flags 10 and 20 to the routes to the network segments of services A and B, respectively.

# Create IP prefix list 1 to match the route destined for Loopback1 (network segment of service A).

[R1]ip ip-prefix 1 index 10 permit 172.16.1.0 24 greater-equal 24 less-equal 24

# Create IP prefix list 2 to match the route destined for Loopback2 (network segment of service B).

[R1]ip ip-prefix 2 index 10 permit 172.16.2.0 24 greater-equal 24 less-equal 24

# Create a route-policy named hcip, create nodes 10 and 20, apply IP prefix lists 1 and 2 to the two nodes, respectively, and add route flags.

[R1]route-policy hcip permit node 10

[R1-route-policy] if-match ip-prefix 1

[R1-route-policy] apply tag 10

[R1-route-policy] quit

[R1]route-policy hcip permit node 20

[R1-route-policy] if-match ip-prefix 2

[R1-route-policy] apply tag 20

[R1-route-policy] quit

# Import direct routes to the OSPF routing table on R1 and apply the route-policy named hcip.

[R1]ospf 1

[R1-ospf-1] import-route direct route-policy hcip

# Check the OSPF LSDB on R1.

[R1]display ospf lsdb

OSPF Process 1 with Router ID 10.0.1.1

Link State Database

Area:	0.0.0.0

Type						LinkState	ID					AdvRouter								Age			Len			Sequence					Metric

Router					10.0.3.3								10.0.3.3									1333			48				8000000C						1

Router					10.0.4.4								10.0.4.4									1639			48				80000006							1

Router					10.0.2.2								10.0.2.2									777				60				8000000D						1

Router					10.0.12.1							10.0.12.1								1373			48				80000006							1

Router					10.0.1.1								10.0.1.1									24					48				80000008							1

Network				10.0.23.3							10.0.3.3									1643			32				80000001							0

Network				10.0.12.2							10.0.2.2									777				32				80000002							0

Network				10.0.34.4							10.0.4.4									1639			32				80000002							0

AS External Database

Type     LinkState ID    AdvRouter         Age  Len   Sequence     Metric

External   172.16.2.0      10.0.1.1            24  36    80000001       1

External   172.16.1.0      10.0.1.1            24  36    80000001       1

Routes to Loopback1 and Loopback2 have been imported to the OSPF routing table.

# Check the AS-external LSA 172.16.1.0 in the OSPF LSDB on R1.

[R1]display ospf lsdb ase 172.16.1.0

OSPF Process 1 with Router ID 10.0.1.1

Link State Database

Type									:	External

Ls	id									:	172.16.1.0

Adv	rtr							:	10.0.1.1

Ls	age								:	165

Len										:	36

Options						:	E

seq#									:	80000001

chksum						:	0xa954

Net	mask					:	255.255.255.0

TOS	0		Metric	:	1

E	type								:	2

Forwarding	Address	:	0.0.0.0

Tag									:	10

Priority							:	Low

The external route destined for 172.16.1.0/24 has been tagged 10.

# Check AS-external LSA 172.16.2.0 in the OSPF LSDB on R1.

[R1]display ospf lsdb ase 172.16.2.0

OSPF Process 1 with Router ID 10.0.1.1

Link State Database

Type								:	External

Ls	id								:	172.16.2.0

Adv	rtr						:	10.0.1.1

Ls	age							:	355

Len									:	36

Options					:	E

seq#								:	80000001

chksum					:	0x539f

Net	mask				:	255.255.255.0

TOS	0		Metric:	1

E	type							:	2

Forwarding	Address	:	0.0.0.0

Tag								:	20

Priority						:	Low

The external route destined for 172.16.2.0/24 has been tagged 20.

          - Configure a filter-policy on R2.

Configure a filter-policy on R2 to filter OSPF routes to be accepted so that only the route destined for network segment of service B can be accepted.

# Check the OSPF routing table before the filter-policy is configured.

<R2>display ospf routing

OSPF Process 1 with Router ID 10.0.2.2

Routing Tables

Routing for Network

Destination							Cost		Type						NextHop							AdvRouter							Area

10.0.2.2/32								0					Stub							10.0.2.2								10.0.2.2								0.0.0.0

10.0.12.0/24							1					Transit					10.0.12.2							10.0.2.2								0.0.0.0

10.0.23.0/24							1					Transit					10.0.23.2							10.0.2.2								0.0.0.0

10.0.1.1/32								1					Stub							10.0.12.1							10.0.1.1								0.0.0.0

10.0.3.3/32								1					Stub							10.0.23.3							10.0.3.3								0.0.0.0

Routing for ASEs

Destination        Cost      Type       Tag         NextHop       AdvRouter

172.16.1.0/24      1         Type2      10          10.0.12.1       10.0.1.1

172.16.2.0/24      1         Type2      20          10.0.12.1       10.0.1.1

# Check the OSPF routes in the IP routing table before the filter-policy is configured.

<R2>display ip routing-table protocol ospf

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Public routing table						: OSPF

Destinations				: 4        Routes : 4

OSPF routing table status	: <Active>

Destinations				: 4        Routes : 4

Destination/Mask			Proto					Pre		Cost					Flags	NextHop									Interface

10.0.1.1/32		OSPF					10				1									D		10.0.12.1							GigabitEthernet0/0/3

10.0.3.3/32		OSPF					10				1									D		10.0.23.3							GigabitEthernet0/0/2

172.16.1.0/24		O_ASE				150			1									D		10.0.12.1							GigabitEthernet0/0/3

172.16.2.0/24		O_ASE				150			1									D		10.0.12.1							GigabitEthernet0/0/3

OSPF external routes destined for 172.16.1.0/24 and 172.16.2.0/24 are displayed in the OSPF routing table and IP routing table.

# Configure a basic ACL.

[R2]acl number 2000

[R2-acl-basic-2000] rule 5 deny source 172.16.1.0 0.0.0.255

[R2-acl-basic-2000] rule 10 permit

# Configure an import filter-policy for OSPF and apply ACL 2000.

[R2]ospf 1

[R2-ospf-1] filter-policy 2000 import

# Check the OSPF routing table after the filter-policy is configured.

<R2>display ospf routing

OSPF Process 1 with Router ID 10.0.2.2

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter						Area

10.0.2.2/32								0					Stub							10.0.2.2								10.0.2.2								0.0.0.0

10.0.12.0/24							1					Transit					10.0.12.2							10.0.2.2								0.0.0.0

10.0.23.0/24							1					Transit					10.0.23.2							10.0.2.2								0.0.0.0

10.0.1.1/32								1					Stub							10.0.12.1							10.0.1.1								0.0.0.0

10.0.3.3/32								1					Stub							10.0.23.3							10.0.3.3								0.0.0.0

Routing for ASEs

Destination        Cost      Type       Tag         NextHop       AdvRouter

172.16.1.0/24      1         Type2      10          10.0.12.1       10.0.1.1

172.16.2.0/24      1         Type2      20          10.0.12.1       10.0.1.1

# Check the OSPF routes in the IP routing table after the filter-policy is configured.

<R2>display ip routing-table protocol ospf

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Public routing table						: OSPF

Destination					: 4        Routes : 4

OSPF routing table status	: <Active>

Destinations				: 4        Routes : 4

Destination/Mask			Proto				Pre			Cost					Flags	NextHop							Interface

10.0.1.1/32		OSPF					10				1									D		10.0.12.1							GigabitEthernet0/0/3

10.0.3.3/32		OSPF					10				1									D		10.0.23.3							GigabitEthernet0/0/2

172.16.2.0/24		O_ASE				150			1									D		10.0.12.1							GigabitEthernet0/0/3

The route destined for 172.16.2.0/24 does not exist in the IP routing table but exists in the OSPF routing table. This proves that for OSPF, the filter-policy only restricts routes to be added to the IP routing table, but does not affect the local LSDB and LSA transmission.

# Check the OSPF routes in the IP routing table on R3.

<R3>display ip routing-table protocol ospf

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Public routing table						: OSPF

Destinations				: 5        Routes : 5

OSPF routing table status	: <Active>

Destinations				: 5        Routes : 5

Destination/Mask			Proto				Pre			Cost							Flags	NextHop									Interface

10.0.1.1/32		OSPF					10			2											D			10.0.23.2							GigabitEthernet0/0/3

10.0.2.2/32		OSPF					10			1											D			10.0.23.2							GigabitEthernet0/0/3

10.0.12.0/24		OSPF					10			2											D			10.0.23.2							GigabitEthernet0/0/3

172.16.1.0/24		O_ASE				150			1										D			10.0.23.2							GigabitEthernet0/0/3

172.16.2.0/24		O_ASE				150			1										D			10.0.23.2							GigabitEthernet0/0/3

The OSPF external routes destined for 172.16.1.0/24 and 172.16.2.0/24 still exist in the IP routing table of R3.

          - Import OSPF routes to the IS-IS routing table on R3.

Import OSPF routes to the IS-IS routing table on R3. Use a route-policy to match route flags and import only OSPF external route destined for network segment of service A.

# Create a route-policy named hcip.

[R3]route-policy hcip permit node 10

[R3-route-policy] if-match tag 10

[R3-route-policy] quit

# Import OSPF routes to the IS-IS routing table and apply the route-policy named hcip to import only OSPF external routes of the network segment of service A.

[R3]isis 1

[R3-isis-1] import-route ospf 1 level-1 route-policy hcip

# Check the IS-IS routing table of R3.

<R3>display isis route

Route information for ISIS(1)

-----------------------------

ISIS(1) Level-1 Forwarding Table

--------------------------------

IPV4	Destination					IntCost				ExtCost				ExitInterface						NextHop							Flags

-------------------------------------------------------------------------------

10.0.4.4/32										10									NULL				GE0/0/2									10.0.34.4							A/-/-/-

10.0.34.0/24									10									NULL				GE0/0/2									Direct										D/-/L/-

Flags: D-Direct, A-Added to URT, L-Advertised in LSPs, S-IGP Shortcut,

U-Up/Down Bit Set

ISIS(1) Level-1 Redistribute Table

----------------------------------

Type IPV4 Destination     IntCost    ExtCost Tag

-------------------------------------------------------------------------------

O    172.16.1.0/24        0          0

Type: D-Direct, I-ISIS, S-Static, O-OSPF, B-BGP, R-RIP, U-UNR

The Level-1 route redistribution table contains only a route destined for 172.16.1.0/24.

----End

Quiz

What are the differences when the filter-policy is used in distance-vector and link-state routing protocols?

Configuration Reference

Configuration on R1

#

sysname R1

#

interface GigabitEthernet0/0/2

ip address 10.0.12.1 255.255.255.0

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

interface LoopBack1

ip address 172.16.1.1 255.255.255.0

#

interface LoopBack2

ip address 172.16.2.1 255.255.255.0

#

interface LoopBack3

ip address 172.16.3.1 255.255.255.0

#

ospf 1 router-id 10.0.1.1

import-route direct route-policy hcip

area 0.0.0.0

network 10.0.1.1 0.0.0.0

network 10.0.12.1 0.0.0.0

#

route-policy hcip permit node 10

if-match ip-prefix 1

apply tag 10

#

route-policy hcip permit node 20

if-match ip-prefix 2

apply tag 20

#

ip ip-prefix 1 index 10 permit 172.16.1.0 24 greater-equal 24 less-equal 24

ip ip-prefix 2 index 10 permit 172.16.2.0 24 greater-equal 24 less-equal 24

#

return

Configuration on R2

#

sysname R2

#

acl number 2000

rule 5 deny source 172.16.1.0 0.0.0.255

rule 10 permit

#

interface GigabitEthernet0/0/2

ip address 10.0.23.2 255.255.255.0

#

interface GigabitEthernet0/0/3

ip address 10.0.12.2 255.255.255.0

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

ospf 1 router-id 10.0.2.2

filter-policy 2000 import

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.23.2 0.0.0.0

network 10.0.12.2 0.0.0.0

#

return

Configuration on R3

#

sysname R3

#

isis 1

is-level level-1

network-entity 49.0001.0000.0000.0003.00

import-route ospf 1 level-1 route-policy hcip

#

interface GigabitEthernet0/0/2

ip address 10.0.34.3 255.255.255.0

isis enable 1

#

interface GigabitEthernet0/0/3

ip address 10.0.23.3 255.255.255.0

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.23.3 0.0.0.0

#

route-policy hcip permit node 10

if-match tag 10

#

return

Configuration on R4

#

sysname R4

#

isis 1

is-level level-1

network-entity 49.0001.0000.0000.0004.00

#

interface GigabitEthernet0/0/3

ip address 10.0.34.4 255.255.255.0

isis enable 1

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.255

isis enable 1

#

return

RSTP and MSTP

Basic RSTP and MSTP Configurations

Introduction

Objectives

Upon completion of this task, you will be able to:

Manually change a bridge priority to affect root bridge election.

Manually change a port cost value to control root port election.

Manually change a port priority value to control root port election.

Configure MSTP to implement load balancing among VLANs.

Networking Topology

              - Basic RSTP and MSTP configurations

The preceding figure shows connections between switches. Configure RSTP and MSTP to break Layer 2 loops, and manually specify the primary root bridge and secondary root bridge.

Background

You are a network administrator of a company. The enterprise network uses a backup network. To prevent loops, RSTP is deployed. All VLANs share the same STP spanning tree. To load balancing data traffic between VLANs, MSTP needs to be deployed on the network.

Lab Configuration

Configuration Roadmap

- Enable STP and change the STP mode to RSTP.

- Manually specify S1 as the primary root bridge and S2 as the secondary root bridge.

- Change the interface cost so that S4's GE0/0/1 becomes the root port.

- Change the priority value of S1's GE0/0/11 so that S2's GE0/0/11 becomes the root port.

- Change the STP mode to MSTP, create MSTI1 and MSTI2. Specify SW1 as the root bridge of MSTI1 and secondary root bridge of MSTI2, and specify SW2 as the root bridge of MSTI2 and secondary root bridge of MSTI1.

Procedure

          - Perform basic RSTP configurations.

Enable STP on S1, S2, S3, and S4, and switch the STP mode to RSTP.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure S1.

[S1]stp enable

[S1]stp mode rstp

# Configure S2.

[S2]stp enable

[S2]stp mode rstp

# Configure S3.

[S3]stp enable

[S3]stp mode rstp

# Configure S4.

[S4]stp enable

[S4]stp mode rstp

# Check the STP status and statistics.

<S1>display stp instance 0 brief

MSTID		Port																				Role					STP	State									Protection

0					GigabitEthernet0/0/10					DESI					FORWARDING					NONE

0					GigabitEthernet0/0/11					DESI					FORWARDING					NONE

0					GigabitEthernet0/0/12					DESI					FORWARDING					NONE

0					GigabitEthernet0/0/13					DESI					FORWARDING					NONE

[S2]display stp brief

MSTID		Port																				Role					STP	State									Protection

0					GigabitEthernet0/0/10					ROOT				FORWARDING					NONE

0					GigabitEthernet0/0/11					ALTE				DISCARDING						NONE

0					GigabitEthernet0/0/12					ALTE				DISCARDING						NONE

0					GigabitEthernet0/0/13					DESI					FORWARDING					NONE

[S3]display stp brief

MSTID		Port																				Role					STP	State								Protection

0					GigabitEthernet0/0/1						ROOT				FORWARDING				NONE

0					GigabitEthernet0/0/2						ALTE				DISCARDING					NONE

0					GigabitEthernet0/0/3						ALTE				DISCARDING					NONE

[S4]display stp brief

MSTID		Port																				Role					STP	State								Protection

0				GigabitEthernet0/0/1							DESI					FORWARDING				NONE

0				GigabitEthernet0/0/2							ROOT				FORWARDING				NONE

0				GigabitEthernet0/0/3							DESI					FORWARDING				NONE

All ports on S1 are designated ports, and S1 is the root bridge. In practice, the actual test result may be different from the preceding result because the MAC address of a switch is uncertain.

# Check the STP status and statistics on S1. The following information is displayed:

<S1>display stp

-------[CIST Global Info][Mode RSTP]-------

CIST	Bridge									:32768.4c1f-cc1d-61a8

Config	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

Active	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

CIST	Root/ERPC						:32768.4c1f-cc1d-61a8	/	0

CIST	RegRoot/IRPC			:32768.4c1f-cc1d-61a8	/	0

CIST	RootPortId						:0.0

BPDU-Protection					:Disabled

TC	or	TCN	received			:15

TC	count	per	hello				:0

STP	Converge	Mode		:Normal

Time	since	last	TC				:0	days	0h:11m:14s

Number	of	TC							:17

Last	TC	occurred					:GigabitEthernet0/0/13

S1 is the root bridge.

          - Control root bridge election.

Configure S1 as the primary root bridge and S2 as the secondary root bridge.

# Manually adjust the STP priority and specify S1 as the primary root bridge and S2 as the secondary root bridge.

[S1]stp priority 4096

[S2]stp priority 8192

When the other two switches retain the default bridge priority (32768), S1 has the lowest bridge priority, followed by S2.

# Check the STP status and statistics on S1. The following information is displayed:

[S1]display stp

-------[CIST Global Info][Mode RSTP]-------

CIST	Bridge									:4096	.4c1f-cc1d-61a8

Config	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

Active	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

CIST	Root/ERPC						:4096	.4c1f-cc1d-61a8	/	0

CIST	RegRoot/IRPC			:4096	.4c1f-cc1d-61a8	/	0

CIST	RootPortId						:0.0

BPDU-Protection					:Disabled

TC	or	TCN	received			:75

TC	count	per	hello				:0

STP	Converge	Mode		:Normal

Time	since	last	TC				:0	days	0h:1m:16s

Number	of	TC							:45

Last	TC	occurred					:GigabitEthernet0/0/10

The bridge priority of S1 is 4096 and S1 is still the root bridge.

# Delete the configuration of manually adjusting the bridge priority on S1 and S2, and run the stp root command to specify the primary root bridge and secondary root bridge.

[S1]undo stp priority

[S1]stp root primary

[S2]undo stp priority

[S2]stp root secondary

# Check the STP status and statistics on S1 and S2. The following information is displayed:

[S1]display stp

-------[CIST Global Info][Mode RSTP]-------

CIST	Bridge									:0				.4c1f-cc1d-61a8

Config	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

Active	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

CIST	Root/ERPC						:0				.4c1f-cc1d-61a8	/	0

CIST	RegRoot/IRPC			:0				.4c1f-cc1d-61a8	/	0

CIST	RootPortId						:0.0

BPDU-Protection					:Disabled

CIST	Root	Type							:Primary	root

TC	or	TCN	received			:85

TC	count	per	hello				:0

STP	Converge	Mode		:Normal

Time	since	last	TC				:0	days	0h:0m:9s

Number	of	TC							:51

Last	TC	occurred					:GigabitEthernet0/0/10

[S2]display stp

-------[CIST Global Info][Mode RSTP]-------

CIST	Bridge									:4096	.4c1f-cc69-5bf7

Config	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

Active	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

CIST	Root/ERPC						:0				.4c1f-cc1d-61a8	/	20000

CIST	RegRoot/IRPC			:4096	.4c1f-cc69-5bf7	/	0

CIST	RootPortId						:128.10

BPDU-Protection					:Disabled

CIST	Root	Type							:Secondary	root

TC	or	TCN	received			:213

TC	count	per	hello				:0

STP	Converge	Mode		:Normal

Time	since	last	TC				:0	days	0h:0m:35s

Number	of	TC							:44

Last	TC	occurred					:GigabitEthernet0/0/12

The bridge priority of S1 is 0 and that of S2 is 4096. S1 is the primary root bridge and S2 is the secondary root bridge.

          - Change the interface cost to control root port election.

# Check the STP status and statistics on S4.

[S4]display  stp brief

MSTID	Port																						Role			STP	State								Protection

0				GigabitEthernet0/0/1								ALTE		DISCARDING						NONE

0				GigabitEthernet0/0/2								ROOT		FORWARDING					NONE

0				GigabitEthernet0/0/3								DESI			FORWARDING					NONE

GE0/0/2 on S4 has a smaller root path cost (RPC) and becomes the root port.

# Check the STP status and statistics on GE0/0/2 of S4.

[S4]display stp interface GigabitEthernet 0/0/2

-------[CIST Global Info][Mode RSTP]-------

CIST	Bridge									:32768.4c1f-cc49-4c7c

Config	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

Active	Times								:Hello	2s	MaxAge	20s	FwDly	15s	MaxHop	20

CIST	Root/ERPC						:0				.4c1f-cc1d-61a8	/	20000

CIST	RegRoot/IRPC			:32768.4c1f-cc49-4c7c	/	0

CIST	RootPortId						:128.2

BPDU-Protection					:Disabled

TC	or	TCN	received			:98

TC	count	per	hello				:0

STP	Converge	Mode		:Normal

Time	since	last	TC				:0	days	0h:8m:35s

Number	of	TC							:47

Last	TC	occurred					:GigabitEthernet0/0/2

----[Port2(GigabitEthernet0/0/2)][FORWARDING]----

Port	Protocol										:Enabled

Port	Role													:Root	Port

Port	Priority											:128

Port	Cost(Dot1T	)						:Config=auto	/	Active=20000

Designated	Bridge/Port	:0.4c1f-cc1d-61a8	/	128.13

Port	Edged												:Config=default	/	Active=disabled

Point-to-point									:Config=auto	/	Active=true

Transit	Limit										:147	packets/hello-time

Protection	Type								:None

Port	STP	Mode									:RSTP

Port	Protocol	Type						:Config=auto	/	Active=dot1s

BPDU	Encapsulation				:Config=stp	/	Active=stp

PortTimes												:Hello	2s	MaxAge	20s	FwDly	15s	RemHop	0

TC	or	TCN	send								:26

TC	or	TCN	received					:40

BPDU	Sent												:1747

TCN:	0,	Config:	0,	RST:	1747,	MST:	0

BPDU	Received								:1048

TCN:	0,	Config:	0,	RST:	1048,	MST:	0

In this case, the RPC calculation method is dot1t, and the STP cost of the interface is 20000.

# Change the STP cost of GE0/0/2 on S4 to 40001.

[S4]interface GigabitEthernet 0/0/2

[S4-GigabitEthernet0/0/2] stp cost 40001

# Check the STP status and statistics on S4 again.

<S4>display stp brief

MSTID	Port																				Role				STP	State									Protection

0				GigabitEthernet0/0/1						ROOT			FORWARDING					NONE

0				GigabitEthernet0/0/2						ALTE			DISCARDING						NONE

0				GigabitEthernet0/0/3						ALTE			DISCARDING						NONE

The RPC of GE0/0/1 is 40000, smaller than RPC 40001 of GE0/0/2. GE0/0/1 of S4 becomes the root port.

          - Change the interface priority to control root port election.

# Check the STP status and statistics on S2.

[S2]display stp brief

MSTID		Port																						Role			STP	State										Protection

0					GigabitEthernet0/0/10							ROOT		FORWARDING						NONE

0					GigabitEthernet0/0/11							ALTE		DISCARDING							NONE

0					GigabitEthernet0/0/12							DESI			FORWARDING						NONE

0					GigabitEthernet0/0/13							DESI			FORWARDING						NONE

The BPDUs received on GE0/0/10 and GE0/0/11 of S2 have the same RPC, bridge ID, and interface priority. Therefore, S2 compares interface numbers in the received BPDU interface IDs.

# Enable LLDP on S1 and S2 and check interface connections.

[S1]lldp enable

[S2]lldp enable

[S2]display lldp neighbor brief

Local	Intf			Neighbor	Dev													Neighbor	Intf															Exptime

GE0/0/10					S1																					GE0/0/10																		102

GE0/0/11					S1																					GE0/0/11																		102

GE0/0/12					S4																					GE0/0/1																			108

GE0/0/13					S3																					GE0/0/2																			103

The peer end of S2's GE0/0/10 is S1's GE0/0/10, and the peer end of S2's GE0/0/11 is S1's GE0/0/11. The BPDU received by GE0/0/10 on S2 has a smaller interface number, which is why GE0/0/10 becomes the root port.

# Change the STP priority of S1's GE0/0/11 so that the priority of BPDUs sent by GE0/0/11 becomes higher than that of BPDUs sent by GE0/0/10.

[S1]interface GigabitEthernet 0/0/11

[S1-GigabitEthernet0/0/11] stp port priority 64

The priority value of the STP interface is 128. The smaller the value, the higher the priority.

# Check the STP status and statistics on S2 again.

[S2]display stp brief

MSTID	Port																				Role					STP	State										Protection

0				GigabitEthernet0/0/10					ROOT				FORWARDING						NONE

0				GigabitEthernet0/0/11					ALTE				DISCARDING							NONE

0				GigabitEthernet0/0/12					DESI					FORWARDING						NONE

0				GigabitEthernet0/0/13					DESI					FORWARDING						NONE

S2's GE0/0/1 becomes the root port.

          - Basic MSTP Configurations

Create VLANs 10, 20, 30, 40, 50, 60, 70 and 80 on all switches. Configure an MSTP domain named hcip. Create two instances named Instance 1 and Instance 2. Map VLANs 10, 30, 50, and 70 to Instance 1. Map VLANs 20, 40, 60, and 80 to Instance 2. In addition, SW1 is configured as the primary root bridge of MSTI1 and the secondary root bridge of MSTI2, and SW2 is configured as the primary root bridge of MSTI2 and the secondary root bridge of MSTI1.

# Create VLANs.

[S1]vlan batch 10 20 30 40 50 60 70 80

[S2]vlan batch 10 20 30 40 50 60 70 80

[S3]vlan batch 10 20 30 40 50 60 70 80

[S4]vlan batch 10 20 30 40 50 60 70 80

# Configure all interconnection interfaces as trunk interfaces and allow packets from all VLANs to pass.

The configuration details are not provided.

# Change the STP mode to MSTP.

[S1]stp mode mstp

[S2]stp mode mstp

[S3]stp mode mstp

[S4]stp mode mstp

# Configure MSTP.

[S1]stp region-configuration

[S1-mst-region] region-name hcip

[S1-mst-region] revision-level 1

[S1-mst-region] instance 1 vlan 10 30 50 70

[S1-mst-region] instance 2 vlan 20 40 60 80

[S1-mst-region] active region-configuration

Info: This operation may take a few seconds. Please wait for a moment...done.

[S1-mst-region] quit

[S2]stp region-configuration

[S2-mst-region] region-name hcip

[S2-mst-region] revision-level 1

[S2-mst-region] instance 1 vlan 10 30 50 70

[S2-mst-region] instance 2 vlan 20 40 60 80

[S2-mst-region] active region-configuration

Info: This operation may take a few seconds. Please wait for a moment...done.

[S2-mst-region] quit

[S3]stp region-configuration

[S3-mst-region] region-name hcip

[S3-mst-region] revision-level 1

[S3-mst-region] instance 1 vlan 10 30 50 70

[S3-mst-region] instance 2 vlan 20 40 60 80

[S3-mst-region] active region-configuration

Info: This operation may take a few seconds. Please wait for a moment...done.

[S3-mst-region] quit

[S4]stp region-configuration

[S4-mst-region] region-name hcip

[S4-mst-region] revision-level 1

[S4-mst-region] instance 1 vlan 10 30 50 70

[S4-mst-region] instance 2 vlan 20 40 60 80

[S4-mst-region] active region-configuration

Info: This operation may take a few seconds. Please wait for a moment...done.

[S4-mst-region] quit

# Check mappings between MSTIs and VLANs on S1.

[S1]display stp region-configuration

Oper configuration

Format selector			:0

Region name 				:hcip

Revision level					:1

Instance   VLANs Mapped

0       1 to 9, 11 to 19, 21 to 29, 31 to 39, 41 to 49, 51 to 59, 61 to

69, 71 to 79, 81 to 4094

1       10, 30, 50, 70

2       20, 40, 60, 80

# Configure SW1 as the root bridge of MSTI1 and the secondary root bridge of MSTI2.

[S1]stp instance 1 root primary

[S1]stp instance 2 root secondary

# Configure SW2 as the primary root bridge of MSTI2 and the secondary root bridge of MSTI1.

[S2]stp instance 1 root secondary

[S2]stp instance 2 root primary

# Check the status and statistics of MSTI1 on S1.

[S1]display stp instance 1 brief

MSTID Port                        Role  STP State          Protection

1    GigabitEthernet0/0/10       DESI  FORWARDING      NONE

1    GigabitEthernet0/0/11       DESI  FORWARDING      NONE

1    GigabitEthernet0/0/12       DESI  FORWARDING      NONE

1    GigabitEthernet0/0/13       DESI  FORWARDING      NONE

All ports on S1 are designated ports, and S1 is the root bridge of MSTI1.

# Check the status and statistics of MSTI2 on S2.

[S2]display stp instance 2 brief

MSTID Port                        Role  STP State           Protection

2    GigabitEthernet0/0/10       DESI  FORWARDING      NONE

2    GigabitEthernet0/0/11       DESI  FORWARDING      NONE

2    GigabitEthernet0/0/12       DESI  FORWARDING      NONE

2    GigabitEthernet0/0/13       DESI  FORWARDING      NONE

All ports on S2 are designated ports, and S2 is the root bridge of MSTI2.

----End

Quiz

Compared with STP, which improvements are made in RTSP?

Configuration Reference

Configuration on S1

sysname S1

#

vlan batch 10 20 30 40 50 60 70 80

#

lldp enable

#

stp instance 0 root primary

stp instance 1 root primary

stp instance 2 root secondary

#

stp region-configuration

region-name hcip

revision-level 1

instance 1 vlan 10 30 50 70

instance 2 vlan 20 40 60 80

active region-configuration

#

interface GigabitEthernet0/0/10

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

interface GigabitEthernet0/0/11

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

stp instance 0 port priority 64

#

interface GigabitEthernet0/0/12

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

interface GigabitEthernet0/0/13

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

return

Configuration on S2

sysname S2

#

vlan batch 10 20 30 40 50 60 70 80

#

lldp enable

#

stp instance 0 root secondary

stp instance 1 root secondary

stp instance 2 root primary

#

stp region-configuration

region-name hcip

revision-level 1

instance 1 vlan 10 30 50 70

instance 2 vlan 20 40 60 80

active region-configuration

#

interface GigabitEthernet0/0/10

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

interface GigabitEthernet0/0/11

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

stp instance 0 port priority 64

#

interface GigabitEthernet0/0/12

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

interface GigabitEthernet0/0/13

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

return

Configuration on S3

#

sysname S3

#

vlan batch 10 20 30 40 50 60 70 80

#

lldp enable

#

stp region-configuration

region-name hcip

revision-level 1

instance 1 vlan 10 30 50 70

instance 2 vlan 20 40 60 80

active region-configuration

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

return

Configuration on S4

#

sysname S4

#

vlan batch 10 20 30 40 50 60 70 80

#

lldp enable

#

stp region-configuration

region-name hcip

revision-level 1

instance 1 vlan 10 30 50 70

instance 2 vlan 20 40 60 80

active region-configuration

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

stp instance 0 cost 40001

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60 70 80

#

return

Multicast

IGMP, IGMP Snooping, and PIM-DM

Introduction

Objectives

Upon completion of this task, you will be able to:

Know how to enable multicast routing for multicast traffic forwarding.

Know how to enable IGMP snooping on a switch and manually configure a static router port and member port.

Know how to use PIM-DM to forward multicast traffic.

Know how to control the election result of the Assert mechanism by changing the IGP cost.

Networking Topology

              - Lab topology for IGMP, IGMP snooping, and PIM-DM

In the preceding figure, OSPF runs on four routers. Loopback0 is created on each router. The IP address of Loopback0 is 10.0.x.x/32, where x is the device number.

The four routers form a multicast network. R1 is the first-hop router and is connected to multicast source 239.0.0.12. R4 is the last-hop router and is connected to receivers of multicast group 239.0.0.12. To ensure that the traffic from the multicast source can be received by multicast group members connected to R4, deploy PIM-DM on each router and activate IGMPv2 on GE0/0/5 of R4.

To optimize multicast traffic forwarding on S2, configure IGMP snooping on S2 and manually specify a static router port and member port.

Background

You are a network administrator of a company. Multicast needs to be configured to forward some services. The network size is small, so you can configure PIM-DM to implement multicast route learning. To improve network efficiency and security, you can manually control the election result of the PIM-DM Assert mechanism. To optimize multicast traffic forwarding on the switch connected to multicast receivers, you can enable IGMP snooping on the switch.

Lab Configuration

Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF on each Loopback0 interface and the interfaces that connect R1, R2, R3, and R4.

- Enable the multicast routing function on the routers, and enable PIM-DM on involved interfaces.

- On R1, simulate traffic of the multicast source, and then check the PIM routing table of each router.

- Change the OSPF cost of GE0/0/1 on R3 to control the election result of the Assert mechanism. Then, check the PIM routing tables of R2 and R3 again.

- Configure IGMP snooping on S2 and manually configure a static router port and member port.

Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure R1.

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.255

[R1-LoopBack0] quit

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] ip address 10.0.12.1 255.255.255.0

[R1-GigabitEthernet0/0/2] quit

[R1]interface GigabitEthernet0/0/1

[R1-GigabitEthernet0/0/1] ip address 10.0.13.1 255.255.255.0

[R1-GigabitEthernet0/0/1] quit

# Configure R2.

[R2]interface GigabitEthernet0/0/4

[R2-GigabitEthernet0/0/4] ip address 10.0.234.2 255.255.255.0

[R2-GigabitEthernet0/0/4] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] ip address 10.0.12.2 255.255.255.0

[R2-GigabitEthernet0/0/3] quit

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.255

[R2-LoopBack0] quit

# Configure R3.

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] ip address 10.0.13.3 24

[R3-GigabitEthernet0/0/1] quit

[R3]interface GigabitEthernet0/0/4

[R3-GigabitEthernet0/0/4] ip address 10.0.234.3 255.255.255.0

[R3-GigabitEthernet0/0/4] quit

[R3]interface LoopBack 0

[R3-LoopBack0] ip address 10.0.3.3 32

[R3-LoopBack0] quit

# Configure R4.

[R4]interface GigabitEthernet0/0/4

[R4-GigabitEthernet0/0/4] ip address 10.0.234.4 255.255.255.0

[R4-GigabitEthernet0/0/4] quit

[R4]interface GigabitEthernet0/0/5

[R4-GigabitEthernet0/0/5] ip address 192.168.1.1 255.255.255.0

[R4-GigabitEthernet0/0/5] quit

[R4]interface LoopBack0

[R4-LoopBack0] ip address 10.0.4.4 255.255.255.255

[R4-LoopBack0] quit

# Check IP connectivity on R1 and R4.

<R1>ping -c 1 10.0.12.2

PING 10.0.12.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.2: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.12.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

<R1>ping -c 1 10.0.13.3

PING 10.0.13.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.13.3: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.13.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

<R4>ping -c 1 10.0.234.2

PING 10.0.234.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.234.2: bytes=56 Sequence=1 ttl=255 time=70 ms

--- 10.0.234.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 70/70/70 ms

<R4>ping -c 1 10.0.234.3

PING 10.0.234.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.234.3: bytes=56 Sequence=1 ttl=255 time=80 ms

--- 10.0.234.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 80/80/80 ms

Configure OSPF.

Use the IP address of the Loopback0 interface as the router ID of each router and activate OSPF on the interconnection interfaces and each Loopback0 interface.

# Configure R1.

[R1]ospf 1 router-id 10.0.1.1

[R1-ospf-1] area 0

[R1-ospf-1-area-0.0.0.0] network 10.0.1.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] network 10.0.12.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] network 10.0.13.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] quit

[R1-ospf-1] quit

# Configure R2.

[R2]ospf 1 router-id 10.0.2.2

[R2-ospf-1] area 0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.12.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.234.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] quit

[R2-ospf-1] quit

# Configure R3.

[R3]ospf 1 router-id 10.0.3.3

[R3-ospf-1] area 0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.13.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.234.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] quit

[R3-ospf-1] quit

# Configure R4.

[R4]ospf 1 router-id 10.0.4.4

[R4-ospf-1]area 0

[R4-ospf-1-area-0.0.0.0] network 10.0.234.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 192.168.1.1 0.0.0.0

[R4-ospf-1-area-0.0.0.0] quit

[R4-ospf-1] quit

# Check the OSPF neighbor status on R1 and R4.

<R1>display ospf peer brief

OSPF Process 1 with Router ID 10.0.1.1

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/2             10.0.2.2         Full

0.0.0.0          GigabitEthernet0/0/1             10.0.3.3         Full

----------------------------------------------------------------------------

<R4>display ospf peer brief

OSPF Process 1 with Router ID 10.0.4.4

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/4             10.0.2.2         Full

0.0.0.0          GigabitEthernet0/0/4             10.0.3.3         Full

----------------------------------------------------------------------------

OSPF neighbor relationships have been established between routers.

# Check the OSPF routing table on R4.

[R4]display ospf routing

OSPF Process 1 with Router ID 10.0.4.4

Routing Tables

Routing for Network

Destination							Cost		Type							NextHop							AdvRouter						Area

10.0.4.4/32								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.234.0/24						1					Transit					10.0.234.4						10.0.4.4								0.0.0.0

192.168.1.0/24					1					Stub							192.168.1.1					10.0.4.4								0.0.0.0

10.0.1.1/32								2					Stub							10.0.234.3						10.0.1.1								0.0.0.0

10.0.1.1/32								2					Stub							10.0.234.2						10.0.1.1								0.0.0.0

10.0.2.2/32								1					Stub							10.0.234.2						10.0.2.2								0.0.0.0

10.0.3.3/32								1					Stub							10.0.234.3						10.0.3.3								0.0.0.0

10.0.12.0/24							2					Transit					10.0.234.2						10.0.1.1								0.0.0.0

10.0.13.0/24							2					Transit					10.0.234.3						10.0.1.1								0.0.0.0

Total Nets: 9

Intra Area: 9  Inter Area: 0  ASE: 0  NSSA: 0

R4 has learned OSPF routes on the entire network.

Configure PIM-DM.

Enable multicast routing on all routers and enable PIM-DM on involved interfaces.

# Enable multicast routing.

[R1]multicast routing-enable

[R2]multicast routing-enable

[R3]multicast routing-enable

[R4]multicast routing-enable

# Enable PIM-DM on involved interfaces of R1.

[R1]interface GigabitEthernet0/0/1

[R1-GigabitEthernet0/0/1] pim dm

[R1-GigabitEthernet0/0/1] quit

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] pim dm

[R1-GigabitEthernet0/0/2] quit

# Enable PIM-DM on involved interfaces of R2.

[R2]interface GigabitEthernet0/0/4

[R2-GigabitEthernet0/0/4] pim dm

[R2-GigabitEthernet0/0/4] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] pim dm

[R2-GigabitEthernet0/0/3] quit

# Enable PIM-DM on involved interfaces of R3.

[R3]interface GigabitEthernet0/0/4

[R3-GigabitEthernet0/0/4] pim dm

[R3-GigabitEthernet0/0/4] quit

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] pim dm

[R3-GigabitEthernet0/0/1] quit

# Enable PIM-DM on involved interfaces of R4.

[R4]interface GigabitEthernet0/0/4

[R4-GigabitEthernet0/0/4] pim dm

[R4-GigabitEthernet0/0/4] quit

[R4]interface GigabitEthernet0/0/5

[R4-GigabitEthernet0/0/5] pim dm

[R4-GigabitEthernet0/0/5] quit

# Check the PIM neighbor relationships on R1 and R4.

[R1]display pim neighbor

VPN-Instance: public net

Total Number of Neighbors = 2

Neighbor        Interface            Uptime    Expires   Dr-Priority  BFD-Session

10.0.13.3        GE0/0/1             00:04:14   00:01:31     1            N

10.0.12.2        GE0/0/2             00:04:50   00:01:26     1            N

[R4]display pim neighbor

VPN-Instance: public net

Total Number of Neighbors = 2

Neighbor        Interface            Uptime    Expires   Dr-Priority  BFD-Session

10.0.234.2       GE0/0/4             00:03:09   00:01:41     1            N

10.0.234.3       GE0/0/4             00:03:08   00:01:19     1            N

PIM neighbor relationships have been established between R1 and R2, between R1 and R3, between R4 and R2, and between R4 and R3.

# Enable IGMP on GE0/0/5 of R4 and configure GE0/0/5 to join the multicast group in static mode.

[R4]interface GigabitEthernet0/0/5

[R4-GigabitEthernet0/0/5] igmp enable

[R4-GigabitEthernet0/0/5] igmp static-group 239.0.0.12

# Check IGMP interface information on R4.

[R4]display igmp interface GigabitEthernet 0/0/5

Interface information of VPN-Instance: public net

GigabitEthernet0/0/5(192.168.1.1):

IGMP is enabled

Current IGMP version is 2

IGMP state: up

IGMP group policy: none

IGMP limit: -

Value of query interval for IGMP (negotiated): -

Value of query interval for IGMP (configured): 60 s

Value of other querier timeout for IGMP: 0 s

Value of maximum query response time for IGMP: 10 s

Querier for IGMP: 192.168.1.1 (this router)

The default IGMP version (IGMPv2) is used, and R4 is the IGMP querier.

Check the PIM routing table.

On R1, use the address of Loopback0 as the source address to send ICMP packets to 239.0.0.12 to simulate traffic of the multicast source. Then, check the PIM routing table on each router.

# Use R1 to send packets to simulate traffic of the multicast source.

ping -a 10.0.1.1 -c 10 239.0.0.12

After this command is run, R1 does not send multicast traffic, but it triggers PIM-DM State-Refresh messages.

# Query the content of the PIM-DM State-Refresh messages.

Frame 45: 70 bytes on wire (560 bits), 70 bytes captured (560 bits) on interface 0

Ethernet II, Src: HuaweiTe_0c:16:0a (54:89:98:0c:16:0a), Dst: IPv4mcast_0d (01:00:5e:00:00:0d)

Internet Protocol Version 4, Src: 10.0.12.1, Dst: 224.0.0.13

Protocol Independent Multicast

0010 .... = Version: 2

.... 1001 = Type: State-Refresh (9)

Reserved byte(s): 00

Checksum: 0x8295 [correct]

[Checksum Status: Good]

PIM Options

Group: 239.0.0.12/32

Source: 10.0.1.1

Originator: 10.0.12.1

0... .... = RP Tree: False

.000 0000 0000 0000 0000 0000 0000 0000 = Metric Preference: 0

Metric: 0

Masklen: 32

TTL: 255

0... .... = Prune indicator: Not set

.0.. .... = Prune now: Not set

..1. .... = Assert override: Set

Interval: 60

The State-Refresh messages carry the multicast source address (10.0.1.1) and multicast group address (239.0.0.12). After receiving the messages, the downstream device creates an (S, G) entry and forwards the State-Refresh messages downstream.

# Check statistics about the State-Refresh messages sent by R1.

<R1>display pim control-message counters message-type state-refresh interface GigabitEthernet 0/0/2

VPN-Instance: public net

PIM control-message counters for interface: GigabitEthernet0/0/2

Message Type     Received         Sent             Invalid          Filtered

State-Refresh       0                8                0                0

If the value of Sent is not 0, check the (S, G) entry on the downstream device. If the value of Sent is 0, you will find no (S, G) entry on the downstream device.

PIM-SM does not have State-Refresh messages. Therefore, this method cannot be used in PIM-SM scenarios.

# Check the PIM routing tables of the four routers.

<R1>display pim routing-table

VPN-Instance: public net

Total 0 (*, G) entry; 1 (S, G) entry

(10.0.1.1, 239.0.0.12)

Protocol	: pim-dm, Flag: LOC ACT

UpTime	: 00:04:19

Upstream interface				: LoopBack0

Upstream neighbor: NULL

RPF prime neighbor: NULL

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/1

Protocol: pim-dm, UpTime: 00:04:19, Expires: never

On R1, the inbound interface of the (S, G) entry is Loopback0. Because the multicast source is directly connected to R1, PRF prime neighbor is Null. The downstream interface is GE0/0/1, and R1 forwards the multicast traffic to R3.

<R2>display pim routing-table

VPN-Instance: public net

Total 0 (*, G) entry; 1 (S, G) entry

(10.0.1.1, 239.0.0.12)

Protocol	: pim-dm, Flag:

UpTime	: 00:01:25

Upstream interface				: GigabitEthernet0/0/3

Upstream neighbor: 10.0.12.1

RPF prime neighbor: 10.0.12.1

Downstream interface(s) information: None

On R2, the (S, G) entry does not have any downstream interface.

<R3>display pim routing-table

VPN-Instance: public net

Total 0 (*, G) entry; 1 (S, G) entry

(10.0.1.1, 239.0.0.12)

Protocol	: pim-dm, Flag:

UpTime	: 00:02:55

Upstream interface				: GigabitEthernet0/0/1

Upstream neighbor: 10.0.13.1

RPF prime neighbor: 10.0.13.1

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/4

Protocol: pim-dm, UpTime: 00:02:55, Expires: never

On R3, the downstream interface of the (S, G) entry is GE0/0/4.

The downstream interfaces of R2 and R3 and the upstream interface of R4 are on the same network segment. Therefore, the Assert mechanism is triggered. R2 and R3 send Assert messages through their respective GE0/0/4 for election. The unicast routes from R2 and R3 to the multicast source have the same preference and cost. However, GE0/0/4 of R3 has a higher IP address (10.0.234.3) than that (10.0.234.2) of R2. Therefore, R3 wins the Assert election and continues to forward multicast traffic to R4. R2 no longer forwards multicast traffic downstream through its GE0/0/4. This is why there is no downstream interface in the (S, G) entry in the PIM routing table of R2.

[R4]display pim routing-table

VPN-Instance: public net

Total 1 (*, G) entry; 1 (S, G) entry

(*, 239.0.0.12)

Protocol	: pim-dm, Flag: WC

UpTime	: 00:05:41

Upstream interface				: NULL

Upstream neighbor: NULL

RPF prime neighbor: NULL

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/5

Protocol: static, UpTime: 00:05:41, Expires: never

(10.0.1.1, 239.0.0.12)

Protocol	: pim-dm, Flag:

UpTime	: 00:01:52

Upstream interface				: GigabitEthernet0/0/4

Upstream neighbor: 10.0.234.2

RPF prime neighbor: 10.0.234.2

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/5

Protocol: pim-dm, UpTime: 00:01:52, Expires:  -

The upstream neighbor of R4 is R3, and R4 is the last-hop router.

Change the IGP cost to control the Assert election result.

Change the OSPF cost of GE0/0/1 on R3 so that the unicast route from R3 to the multicast source address has a higher cost. Consequently, R2 wins the Assert election and becomes the Assert winner.

# On R2 and R3, check the cost of the route to the multicast source address 10.0.1.1.

<R2>display ip routing-table 10.0.1.1

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Table : Public

Summary Count : 1

Destination/Mask   Proto    Pre  Cost       Flags NextHop       Interface

10.0.1.1/32  OSPF    10    1          D   10.0.12.1       GigabitEthernet0/0/3

<R3>display ip routing-table 10.0.1.1

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Table : Public

Summary Count : 1

Destination/Mask   Proto   Pre  Cost       Flags  NextHop          Interface

10.0.1.1/32  OSPF    10   1           D   10.0.13.1       GigabitEthernet0/0/1

The costs of the routes from R2 and R3 to 10.0.1.1 are both 1.

# Change the OSPF cost of GE0/0/1 on R3.

[R3]interface GigabitEthernet0/0/1

[R3-GigabitEthernet0/0/1] ospf cost 2

# On R3, check the cost of the route to the multicast source address 10.0.1.1.

<R3>display ip routing-table 10.0.1.1

Route Flags: R - relay, D - download to fib

------------------------------------------------------------------------------

Routing Table : Public

Summary Count : 2

Destination/Mask			Proto			Pre		Cost								Flags		NextHop							Interface

10.0.1.1/32		OSPF				10			2											D			10.0.13.1							GigabitEthernet0/0/1

OSPF				10			2											D			10.0.234.2						GigabitEthernet0/0/4

The cost of the route from R3 to 10.0.1.1 becomes 2.

# Change the Assert timeout period on GE0/0/4 of R2 and R3 to 10s.

[R2]interface GigabitEthernet0/0/4

[R2-GigabitEthernet0/0/4] pim holdtime assert 10

[R3]interface GigabitEthernet0/0/4

[R3-GigabitEthernet0/0/4] pim holdtime assert 10

# Run the debugging pim join-prune receive command on R1 and then observe the prune process.

<R1>terminal debugging

<R1>terminal monitor

<R1>debugging pim join-prune receive

# Re-trigger multicast traffic on R1.

<R1>ping -a 10.0.1.1 -c 10 239.0.0.12

# Check the PIM routing tables of R2 and R3.

[R2]display pim routing-table

VPN-Instance: public net

Total 0 (*, G) entry; 1 (S, G) entry

(10.0.1.1, 239.0.0.12)

Protocol	: pim-dm, Flag:

UpTime	: 00:00:01

Upstream interface				: GigabitEthernet0/0/3

Upstream neighbor: 10.0.12.1

RPF prime neighbor: 10.0.12.1

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/4

Protocol: pim-dm, UpTime: 00:00:01, Expires: never

[R3]display pim routing-table

VPN-Instance: public net

Total 0 (*, G) entry; 1 (S, G) entry

(10.0.1.1, 239.0.0.12)

Protocol	: pim-dm, Flag:

UpTime	: 00:00:08

Upstream interface				: GigabitEthernet0/01

Upstream neighbor: 10.0.234.2

RPF prime neighbor: 10.0.234.2

Downstream interface(s) information: None

In this case, R3 does not have a downstream interface, and R2 becomes the Assert winner.

# Check the debugging information on R1.

Jul  2 2020 09:49:03.520.1-08:00 R1 PIM/7/JP:(public net): PIM ver 2 JP  receiving 10.0.13.3 -> 224.0.0.13 on GigabitEthernet0/0/1  (P012998)

Jul  2 2020 09:49:03.520.2-08:00 R1 PIM/7/JP:(public net): Upstream 10.0.13.1, Groups 1, Holdtime 180 (P013002)

Jul  2 2020 09:49:03.520.3-08:00 R1 PIM/7/JP:(public net): Group: 239.0.0.12/32 --- 0 join 1 prune (P013011)

Jul  2 2020 09:49:03.520.4-08:00 R1 PIM/7/JP:(public net): Prune: 10.0.1.1/32  (P013021)

Jul  2 2020 09:49:05.790.1-08:00 R1 PIM/7/JP:(public net): PIM ver 2 JP  receiving 10.0.12.2 -> 224.0.0.13 on GigabitEthernet0/0/2  (P012933)

Jul  2 2020 09:49:05.790.2-08:00 R1 PIM/7/JP:(public net): Upstream 10.0.12.1, Groups 1, Holdtime 0 (P012939)

Jul  2 2020 09:49:05.790.3-08:00 R1 PIM/7/JP:(public net): Group: 239.0.0.12/32 --- 1 join 0 prune (P012949)

Jul  2 2020 09:49:05.790.4-08:00 R1 PIM/7/JP:(public net): Join: 10.0.1.1/32  (P012959)

The debugging information shows that R1 received a Prune message from R3 with the group address being 239.0.0.12 and the multicast source address being 10.0.1.1.

Configure IGMP snooping.

To optimize multicast traffic forwarding on S2, enable IGMP snooping on S2 and manually configure a static router port and member port.

# Enable IGMP snooping globally and in VLAN 1.

[S2]igmp-snooping enable

[S2]vlan 1

[S2-vlan1] igmp-snooping enable

[S2-vlan1] quit

# Manually configure GE0/0/4 as a static router port.

[S2]interface GigabitEthernet0/0/4

[S2-GigabitEthernet0/0/4] igmp-snooping static-router-port vlan 1

# Manually configure GE0/0/10 as a static member port of the multicast group 239.0.0.12.

[S2]interface GigabitEthernet0/0/10

[S2-GigabitEthernet0/0/10] l2-multicast static-group group-address 239.0.0.12 vlan 1

[S2-GigabitEthernet0/0/10] quit

# Check the L2 multicast forwarding table on S2.

[S2]display	l2-multicast	forwarding-table	vlan	1

VLAN	ID	:	1,	Forwarding	Mode	:	IP

----------------------------------------------------------------------

(Source,	Group)				Interface													Out-Vlan

----------------------------------------------------------------------

Router-port					GigabitEthernet0/0/4						1

(*,	239.0.0.12)				GigabitEthernet0/0/4						1

GigabitEthernet0/0/10					1

----------------------------------------------------------------------

Total	Group(s)	:	1

GE0/0/4 is a static router port, and GE0/0/10 is a static member port. The static member port must be connected to a device and is up.

----End

Quiz

What are the disadvantages of configuring PIM-DM on a large-sized network?

Configuration Reference

Configuration on R1

#

sysname R1

#

multicast routing-enable

#

interface GigabitEthernet0/0/1

ip address 10.0.13.1 255.255.255.0

pim dm

#

interface GigabitEthernet0/0/2

ip address 10.0.12.1 255.255.255.0

pim dm

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

#

ospf 1 router-id 10.0.1.1

area 0.0.0.0

network 10.0.1.1 0.0.0.0

network 10.0.12.1 0.0.0.0

network 10.0.13.1 0.0.0.0

#

return

Configuration on R2

#

sysname R2

#

multicast routing-enable

#

interface GigabitEthernet0/0/3

ip address 10.0.12.2 255.255.255.0

pim dm

#

interface GigabitEthernet0/0/4

ip address 10.0.234.2 255.255.255.0

pim holdtime assert 10

pim dm

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.12.2 0.0.0.0

network 10.0.234.2 0.0.0.0

#

return

Configuration on R3

#

sysname R3

#

multicast routing-enable

#

interface GigabitEthernet0/0/1

ip address 10.0.13.3 255.255.255.0

pim dm

ospf cost 2

#

interface GigabitEthernet0/0/4

ip address 10.0.234.3 255.255.255.0

pim holdtime assert 10

pim dm

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.13.3 0.0.0.0

network 10.0.234.3 0.0.0.0

#

return

Configuration on R4

#

sysname R4

#

multicast routing-enable

#

interface GigabitEthernet0/0/4

ip address 10.0.234.4 255.255.255.0

pim dm

#

interface GigabitEthernet0/0/5

ip address 192.168.1.1 255.255.255.0

igmp enable

igmp static-group 239.0.0.12

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.255

#

ospf 1 router-id 10.0.4.4

area 0.0.0.0

network 10.0.234.4 0.0.0.0

network 10.0.4.4 0.0.0.0

network 192.168.1.1 0.0.0.0

#

return

Configuration on S2

#

sysname S2

#

igmp-snooping enable

#

vlan 1

igmp-snooping enable

#

interface GigabitEthernet0/0/4

igmp-snooping static-router-port vlan 1

#

interface GigabitEthernet0/0/10

l2-multicast static-group group-address 239.0.0.12 vlan 1

PIM-SM, BSR, and PIM-SSM

Introduction

Objectives

Upon completion of this task, you will be able to:

Know how to use PIM-SM to forward multicast traffic.

Know how to configure a BSR for RP election.

Know how to configure PIM-SM SSM to forward multicast traffic.

Know how to use the ping multicast command to send multicast traffic.

Networking Topology

              - Lab topology for PIM-SM, BSR, and PIM-SSM

OSPF runs on four routers. Loopback0 is created on each router. The IP address of Loopback0 is 10.0.x.x/32, where x is the device number. Use R1 to simulate the source of multicast group 239.0.0.12, and use GE0/0/0 on R4 to simulate a receiver of multicast group 239.0.0.12.

R3 is planned as the RP of the network and is elected as the RP through the BSR mode.

Background

You are a network administrator of a company. PIM-DM has been configured on the company's network. However, when more and more multicast users are dispersed on the network, multicast service quality degrades. To improve multicast reliability and efficiency, you can configure PIM-SM.

In the PIM-SM mode, an RP is required and is used as the root of RPTs.

Lab Configuration

Configuration Roadmap

- Configure IP addresses for the devices.

- Configure OSPF on each Loopback0 interface and the interfaces that connect R1, R2, R3, and R4.

- Enable the multicast routing function on the routers, and enable PIM-SM on involved interfaces.

- Configure Loopback0 on R2 as the BSR and Loopback0 on R3 as the RP.

- Check the PIM-SM routing table on each device. Run the ping multicast command to trigger the RPT-to-SPT switchover. Then check the PIM-SM routing table again.

- Change the IGMP version on GE0/0/0 of R4 to version 3, configure an interface to join multicast group 232.0.0.12 in static mode, and check the PIM-SM SSM routing table.

Configuration Procedure

Configure IP addresses for interconnection interfaces and loopback interfaces.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Configure R1.

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] ip address 10.0.12.1 255.255.255.0

[R1-GigabitEthernet0/0/2] quit

[R1]interface LoopBack0

[R1-LoopBack0] ip address 10.0.1.1 255.255.255.255

[R1-LoopBack0] quit

# Configure R2.

[R2]interface LoopBack0

[R2-LoopBack0] ip address 10.0.2.2 255.255.255.255

[R2-LoopBack0] quit

[R2]interface GigabitEthernet0/0/1

[R2-GigabitEthernet0/0/1] ip address 10.0.24.2 255.255.255.0

[R2-GigabitEthernet0/0/1] quit

[R2]interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] ip address 10.0.23.2 255.255.255.0

[R2-GigabitEthernet0/0/2] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] ip address 10.0.12.2 255.255.255.0

[R2-GigabitEthernet0/0/3] quit

# Configure R3.

[R3]interface LoopBack0

[R3-LoopBack0] ip address 10.0.3.3 255.255.255.255

[R3-LoopBack0] quit

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] ip address 10.0.34.3 255.255.255.0

[R3-GigabitEthernet0/0/2] quit

[R3]interface GigabitEthernet0/0/3

[R3-GigabitEthernet0/0/3] ip address 10.0.23.3 255.255.255.0

[R3-GigabitEthernet0/0/3] quit

# Configure R4.

[R4]interface LoopBack0

[R4-LoopBack0] ip address 10.0.4.4 255.255.255.255

[R4-LoopBack0]quit

[R4]interface GigabitEthernet0/0/1

[R4-GigabitEthernet0/0/1] ip address 10.0.24.4 255.255.255.0

[R4-GigabitEthernet0/0/1]quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] ip address 10.0.34.4 255.255.255.0

[R4-GigabitEthernet0/0/3]quit

# Check the connectivity of interconnection interfaces on R2 and R3.

<R2>ping -c 1 10.0.12.1

PING 10.0.12.1: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.1: bytes=56 Sequence=1 ttl=255 time=40 ms

--- 10.0.12.1 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/40/40 ms

<R2>ping -c 1 10.0.23.3

PING 10.0.23.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.23.3: bytes=56 Sequence=1 ttl=255 time=10 ms

--- 10.0.23.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 10/10/10 ms

<R2>ping -c 1 10.0.24.4

PING 10.0.24.4: 56  data bytes, press CTRL_C to break

Reply from 10.0.24.4: bytes=56 Sequence=1 ttl=255 time=80 ms

--- 10.0.24.4 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 80/80/80 ms

<R3>ping -c 1 10.0.34.4

PING 10.0.34.4: 56  data bytes, press CTRL_C to break

Reply from 10.0.34.4: bytes=56 Sequence=1 ttl=255 time=10 ms

--- 10.0.34.4 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 10/10/10 ms

Configure OSPF on R1, R2, R3, and R4.

Configure R1, R2, R3, and R4 to use their Loopback0 IP addresses as their router ID, and activate OSPF on the interconnected interfaces and Loopback0 interfaces of each device.

# Configure R1.

[R1]ospf 1 router-id 10.0.1.1

[R1-ospf-1] area 0.0.0.0

[R1-ospf-1-area-0.0.0.0] network 10.0.1.1 0.0.0.0

[R1-ospf-1-area-0.0.0.0] network 10.0.12.1 0.0.0.0

# Configure R2.

[R2]ospf 1 router-id 10.0.2.2

[R2-ospf-1]area 0

[R2-ospf-1-area-0.0.0.0] network 10.0.2.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.12.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.23.2 0.0.0.0

[R2-ospf-1-area-0.0.0.0] network 10.0.24.2 0.0.0.0

# Configure R3.

[R3]ospf 1 router-id 10.0.3.3

[R3-ospf-1]area 0

[R3-ospf-1-area-0.0.0.0] network 10.0.3.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.23.3 0.0.0.0

[R3-ospf-1-area-0.0.0.0] network 10.0.34.3 0.0.0.0

# Configure R4.

[R4]ospf 1 router-id 10.0.4.4

[R4-ospf-1]area 0

[R4-ospf-1-area-0.0.0.0] network 10.0.4.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.24.4 0.0.0.0

[R4-ospf-1-area-0.0.0.0] network 10.0.34.4 0.0.0.0

# Check the OSPF neighbor status on R2 and R3.

<R2>display ospf peer brief

OSPF Process 1 with Router ID 10.0.2.2

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/2             10.0.3.3         Full

0.0.0.0          GigabitEthernet0/0/3             10.0.1.1         Full

0.0.0.0          GigabitEthernet0/0/1             10.0.4.4         Full

----------------------------------------------------------------------------

<R3>display ospf peer brief

OSPF Process 1 with Router ID 10.0.3.3

Peer Statistic Information

----------------------------------------------------------------------------

Area Id          Interface                      Neighbor id      State

0.0.0.0          GigabitEthernet0/0/3             10.0.2.2         Full

0.0.0.0          GigabitEthernet0/0/2             10.0.4.4         Full

----------------------------------------------------------------------------

The preceding command outputs show that OSPF neighbor relationships have been established.

# Check the OSPF routing table on R4.

<R4>display	ospf	routing

OSPF	Process	1	with	Router	ID	10.0.4.4

Routing	Tables

Routing	for	Network

Destination							Cost		Type							NextHop						AdvRouter							Area

10.0.4.4/32								0					Stub							10.0.4.4								10.0.4.4								0.0.0.0

10.0.24.0/24							1					Transit					10.0.24.4							10.0.4.4								0.0.0.0

10.0.34.0/24							1					Transit					10.0.34.4							10.0.4.4								0.0.0.0

10.0.1.1/32								2					Stub							10.0.24.2							10.0.1.1								0.0.0.0

10.0.2.2/32								1					Stub							10.0.24.2							10.0.2.2								0.0.0.0

10.0.3.3/32								1					Stub							10.0.34.3							10.0.3.3								0.0.0.0

10.0.12.0/24							2					Transit					10.0.24.2							10.0.1.1								0.0.0.0

10.0.23.0/24							2					Transit					10.0.24.2							10.0.2.2								0.0.0.0

10.0.23.0/24							2					Transit					10.0.34.3							10.0.2.2								0.0.0.0

Total	Nets:	9

Intra	Area:	9		Inter	Area:	0		ASE:	0		NSSA:	0

The preceding command output shows that R4 has learned the routes on the entire network.

Configure PIM-SM.

Enable multicast routing on all routers and enable PIM-SM on involved interfaces.

# Enable multicast routing.

[R1]multicast routing-enable

[R2]multicast routing-enable

[R3]multicast routing-enable

[R4]multicast routing-enable

# Enable PIM-SM on involved interfaces of R1.

[R1]interface LoopBack 0

[R1-LoopBack0] pim sm

[R1-LoopBack0] quit

[R1]interface GigabitEthernet0/0/2

[R1-GigabitEthernet0/0/2] pim sm

[R1-GigabitEthernet0/0/2] quit

# Enable PIM-SM on involved interfaces of R2.

[R2]interface GigabitEthernet0/0/1

[R2-GigabitEthernet0/0/1] pim sm

[R2-GigabitEthernet0/0/1] quit

[R2]interface GigabitEthernet0/0/2

[R2-GigabitEthernet0/0/2] pim sm

[R2-GigabitEthernet0/0/2] quit

[R2]interface GigabitEthernet0/0/3

[R2-GigabitEthernet0/0/3] pim sm

[R2-GigabitEthernet0/0/3] quit

# Enable PIM-SM on involved interfaces of R3.

[R3]interface GigabitEthernet0/0/2

[R3-GigabitEthernet0/0/2] pim sm

[R3-GigabitEthernet0/0/2] quit

[R3]interface GigabitEthernet0/0/3

[R3-GigabitEthernet0/0/3] pim sm

[R3-GigabitEthernet0/0/3] quit

# Enable PIM-SM on involved interfaces of R4.

[R4]interface GigabitEthernet0/0/1

[R4-GigabitEthernet0/0/1] pim sm

[R4-GigabitEthernet0/0/1] quit

[R4]interface GigabitEthernet0/0/3

[R4-GigabitEthernet0/0/3] pim sm

[R4-GigabitEthernet0/0/3] quit

[R4]interface GigabitEthernet0/0/0

[R4-GigabitEthernet0/0/0] pim sm

[R4-GigabitEthernet0/0/0] quit

# Check PIM neighbor relationships on R2 and R3.

<R2>display pim neighbor

VPN-Instance: public net

Total Number of Neighbors = 3

Neighbor       Interface            Uptime  Expires   Dr-Priority  BFD-Session

10.0.24.4       GE0/0/1             00:08:19 00:01:26   1            N

10.0.23.3       GE0/0/2             00:09:09 00:01:37   1            N

10.0.12.1       GE0/0/3             00:10:07 00:01:42   1            N

<R3>display pim neighbor

VPN-Instance: public net

Total Number of Neighbors = 2

Neighbor       Interface            Uptime  Expires   Dr-Priority  BFD-Session

10.0.34.4       GE0/0/2             00:08:35 00:01:39   1            N

10.0.23.2       GE0/0/3             00:09:25 00:01:21   1            N

PIM neighbor relationships have been established between routers.

Deploy a BSR.

Adjust the C-BSR priority of R2 to enable R2 to become the BSR, and configure R3 as a C-RP.

# Adjust the C-BSR priority of R2 to enable Loopback0 of R2 to become the BSR.

[R2]interface LoopBack0

[R2-LoopBack0] pim sm

[R2-LoopBack0] quit

[R2]pim

[R2-pim] c-bsr priority 100

[R2-pim] c-bsr LoopBack0

[R2-pim] quit

Note that PIM-SM must be enabled on Loopback0.

# Configure Loopback0 of R3 as a C-RP and set the multicast group address to 239.0.0.12.

[R3]interface LoopBack 0

[R3-LoopBack0] pim sm

[R3-LoopBack0] quit

[R3]acl 2000

[R3-acl-basic-2000] rule 1 permit source 239.0.0.12 0.0.0.0

[R3-acl-basic-2000] quit

[R3]pim

[R3-pim] c-rp LoopBack 0 group-policy 2000 priority 100

[R3-pim] quit

Note that PIM-SM must be enabled on Loopback0.

# Check information about the BSR and RP on R4.

<R4>display pim bsr-info

VPN-Instance								: public net

Elected AdminScoped BSR Count: 0

Elected BSR Address			: 10.0.2.2

Priority										: 100

Hash mask length	: 30

State												: Accept Preferred

Scope											: Not scoped

Uptime										: 00:03:35

Expires										: 00:02:06

C-RP Count						: 1

<R4>display pim rp-info

VPN-Instance										: public net

PIM-SM BSR RP Number	:2

Group/MaskLen								: 224.0.0.0/4

RP																: 10.0.3.3

Priority												: 100

Uptime												: 00:04:15

Expires												: 00:02:15

Group/MaskLen								: 239.0.0.12/32

RP																: 10.0.3.3

Priority												: 100

Uptime												: 00:00:15

Expires												: 00:02:15

There is only one C-BSR and one C-RP. Therefore, R2 and R3 function as the BSR and RP, respectively. The IP address of the RP corresponding to the multicast group 239.0.0.12 is 10.0.3.3.

Check the PIM routing table.

On R4, use GE0/0/0 to simulate a receiver of multicast group 239.0.0.12 and check the PIM routing tables of R3 and R4. Change the RPT-to-SPT switchover threshold and trigger multicast traffic forwarding again. Then, check the PIM routing table again.

# Enable IGMP on GE0/0/0 of R4 and configure GE0/0/0 to join the multicast group in static mode.

[R4]interface GigabitEthernet0/0/0

[R4-GigabitEthernet0/0/0] ip address 192.168.1.1 24

[R4-GigabitEthernet0/0/0] igmp enable

[R4-GigabitEthernet0/0/0] igmp static-group 239.0.0.12

Note that the interface must be configured with an IP address and be up.

# Check the PIM routing table of R4.

<R4>display pim routing-table

VPN-Instance: public net

Total 1 (*, G) entry; 0 (S, G) entry

(*, 239.0.0.12)

RP						: 10.0.3.3

Protocol	: pim-sm, Flag: WC EXT

UpTime	: 00:01:18

Upstream interface				: GigabitEthernet0/0/3

Upstream neighbor: 10.0.34.3

RPF prime neighbor: 10.0.34.3

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/0

Protocol: static, UpTime: 00:01:29, Expires: -

The outbound interface of the route to the RP (10.0.3.3) on R4 is GE0/0/3. Therefore, R4 uses GE0/0/3 as the upstream interface of (*, 239.0.0.12) and sends PIM Join messages through this interface.

# Check the PIM routing table of R3.

<R3>display pim routing-table

VPN-Instance: public net

Total 1 (*, G) entry; 0 (S, G) entry

(*, 239.0.0.12)

RP						: 10.0.3.3 (local)

Protocol	: pim-sm, Flag: WC

UpTime	: 00:08:05

Upstream interface				: Register

Upstream neighbor: NULL

RPF prime neighbor: NULL

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/2

Protocol: pim-sm, UpTime: 00:08:05, Expires: 00:03:25

R3 is the RP and does not need to send the Join message upstream. Currently, no multicast source has registered with the RP. Therefore, the upstream interface is still null.

# Run the ping multicast command on R1 to simulate the multicast source of the multicast group 239.0.0.12 and send multicast data.

<R1>ping multicast -c 10 239.0.0.12

# After the network becomes stable, check the PIM routing table of R4.

[R4]display pim routing-table

VPN-Instance: public net

Total 1 (*, G) entry; 1 (S, G) entry

(*, 239.0.0.12)

RP						: 10.0.3.3

Protocol	: pim-sm, Flag: WC EXT

UpTime	: 00:03:38

Upstream interface				: GigabitEthernet0/0/3

Upstream neighbor: 10.0.34.3

RPF prime neighbor: 10.0.34.3

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/0

Protocol: static, UpTime: 00:02:27, Expires: -

(10.0.1.1, 239.0.0.12)

RP: 10.0.3.3

Protocol	: pim-sm, Flag: SPT ACT

UpTime	: 00:00:05

Upstream interface				: GigabitEthernet0/0/1

Upstream neighbor: 10.0.24.2

RPF prime neighbor: 10.0.24.2

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/0

Protocol: pim-sm, UpTime: 00:00:03, Expires: -

On R4, the entry with the Flag being SPT ACT is an (S, G) entry, indicating that the (S, G) entry is used to guide multicast packet forwarding. In addition, the upstream interface is GE0/0/1 that is connected to R2, rather than GE0/0/3 that is connected to R3. In this case, the RPT-to-SPT switchover has been performed.

# Change the RPT-to-SPT switchover threshold on R4.

[R4]pim

[R4-pim] spt-switch-threshold infinity

The command configures R4 never to initiate an RPT-to-SPT switchover.

# Run the ping multicast command on R1 to simulate the multicast source of the multicast group 239.0.0.12 and send multicast data.

<R1>ping multicast -c 10 239.0.0.12

# Check the PIM routing table on R4.

<R4>display pim routing-table

VPN-Instance: public net

Total 1 (*, G) entry; 1 (S, G) entry

(*, 239.0.0.12)

RP						: 10.0.3.3

Protocol	: pim-sm, Flag: WC

UpTime	: 00:13:27

Upstream interface				: GigabitEthernet0/0/3

Upstream neighbor: 10.0.34.3

RPF prime neighbor: 10.0.34.3

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/0

Protocol: static, UpTime: 00:13:27, Expires: -

(10.0.1.1, 239.0.0.12)

RP						: 10.0.3.3

Protocol	: pim-sm, Flag: ACT

UpTime	: 00:00:12

Upstream interface				: GigabitEthernet0/0/3

Upstream neighbor: 10.0.34.3

RPF prime neighbor: 10.0.34.3

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/0

Protocol: pim-sm, UpTime: 00:00:12, Expires: -

In this case, the upstream interface of R4 is still GE0/0/3, and the path of the (S, G) entry to the multicast source is still by way of the RP, indicating that no RPT-to-SPT switchover is performed.

Deploy PIM-SSM.

Change the IGMP version on R4's GE0/0/0 to version 3 and configure GE0/0/0 to join SSM group 232.0.0.12 in static mode.

# Modify the configurations of GE0/0/0.

[R4]interface GigabitEthernet0/0/0

[R4-GigabitEthernet0/0/0] igmp version 3

[R4-GigabitEthernet0/0/0] igmp static-group 232.0.0.12 source 10.0.1.1

By default, the address range of multicast groups in an SSM group policy is 232.0.0.0/8. If the address of the multicast group that an interface joins in static mode is not in this range, PIM-SSM entries cannot be generated.

# Check the PIM routing table of R4.

<R4>display pim routing-table

VPN-Instance: public net

Total 1 (*, G) entry; 1 (S, G) entry

...

...

(10.0.1.1, 232.0.0.12)

Protocol:  pim-ssm, Flag: SG_RCVR

UpTime: 00:01:58

Upstream interface: GigabitEthernet0/0/1

Upstream neighbor: 10.0.24.2

RPF prime neighbor: 10.0.24.2

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/0

Protocol: static, UpTime: 00:01:58, Expires: -

The command output on R4 shows that no traffic is triggered, an (S, G) entry is generated, the protocol is PIM-SSM, and the upstream device is R2.

# Check the PIM routing table of R2.

<R2>display pim routing-table

VPN-Instance: public net

Total 0 (*, G) entry; 2 (S, G) entries

...

...

(10.0.1.1, 232.0.0.12)

Protocol:  pim-ssm, Flag:

UpTime: 00:03:30

Upstream interface: GigabitEthernet0/0/3

Upstream neighbor: 10.0.12.1

RPF prime neighbor: 10.0.12.1

Downstream interface(s) information:

Total number of downstreams: 1

1: GigabitEthernet0/0/1

Protocol: pim-ssm, UpTime: 00:03:30, Expires: 00:03:00

The protocol is PIM-SSM, and the upstream device is R1.

----End

Quiz

What are the advantages of PIM-SM over PIM-DM?

Configuration Reference

Configuration on R1

#

sysname R1

#

multicast routing-enable

#

interface GigabitEthernet0/0/2

ip address 10.0.12.1 255.255.255.0

pim sm

#

interface LoopBack0

ip address 10.0.1.1 255.255.255.255

pim sm

#

ospf 1 router-id 10.0.1.1

area 0.0.0.0

network 10.0.1.1 0.0.0.0

network 10.0.12.1 0.0.0.0

#

return

Configuration on R2

#

sysname R2

#

multicast routing-enable

#

interface GigabitEthernet0/0/1

ip address 10.0.24.2 255.255.255.0

pim sm

#

interface GigabitEthernet0/0/2

ip address 10.0.23.2 255.255.255.0

pim sm

#

interface GigabitEthernet0/0/3

ip address 10.0.12.2 255.255.255.0

pim sm

#

interface LoopBack0

ip address 10.0.2.2 255.255.255.255

pim sm

#

ospf 1 router-id 10.0.2.2

area 0.0.0.0

network 10.0.2.2 0.0.0.0

network 10.0.12.2 0.0.0.0

network 10.0.23.2 0.0.0.0

network 10.0.24.2 0.0.0.0

#

pim

c-bsr priority 100

c-bsr LoopBack0

#

return

Configuration on R3

#

sysname R3

#

multicast routing-enable

#

acl number 2000

rule 1 permit source 239.0.0.12 0

#

interface GigabitEthernet0/0/2

ip address 10.0.34.3 255.255.255.0

pim sm

#

interface GigabitEthernet0/0/3

ip address 10.0.23.3 255.255.255.0

pim sm

#

interface LoopBack0

ip address 10.0.3.3 255.255.255.255

pim sm

#

ospf 1 router-id 10.0.3.3

area 0.0.0.0

network 10.0.3.3 0.0.0.0

network 10.0.23.3 0.0.0.0

network 10.0.34.3 0.0.0.0

#

pim

c-rp LoopBack0 group-policy 2000 priority 100

#

return

Configuration on R4

#

sysname R4

#

multicast routing-enable

#

interface GigabitEthernet0/0/0

ip address 192.168.1.1 255.255.255.0

pim sm

igmp enable

igmp version 3

igmp static-group 239.0.0.12

igmp static-group 232.0.0.12 source 10.0.1.1

#

interface GigabitEthernet0/0/1

ip address 10.0.24.4 255.255.255.0

pim sm

#

interface GigabitEthernet0/0/3

ip address 10.0.34.4 255.255.255.0

pim sm

#

interface LoopBack0

ip address 10.0.4.4 255.255.255.255

#

ospf 1 router-id 10.0.4.4

area 0.0.0.0

network 10.0.4.4 0.0.0.0

network 10.0.24.4 0.0.0.0

network 10.0.34.4 0.0.0.0

#

pim

spt-switch-threshold infinity

#

Return

Firewall Technology

Firewall Security Policy

Introduction

Objectives

Upon completion of this task, you will be able to:

Understand how a security policy works

Learn how to configure a security policy on the firewall using the CLI

Observe server mapping entries to understand how NAT ALG works

Networking Topology

              - Firewall security policy

The preceding figure shows how the devices are connected and their IP address planning. Routers R1 and R2 communicate with the firewall FW1 at Layer 3 through switch S1. On S1, its interfaces (GE0/0/1 and GE0/0/2) connected to R1 and R2 are assigned to VLAN 10 and VLAN 20 respectively, and the interfaces (GE0/0/14 and GE0/0/15) connected to FW1 are assigned to VLAN 10 and VLAN 20 respectively.

R1 belongs to the Demilitarized Zone (DMZ), and R2 to the untrusted zone. Configure source NAT on FW1 so that R1 can access the untrusted zone through GE0/0/2 of FW1. Configure NAT Server on FW1, enabling R2 to access the FTP service enabled on R1 through GE0/0/2 of FW1.

In addition, configure security policies on FW1 to restrict the access between R1 and R2 as follows: R1 in the DMZ can access the untrusted zone, but R2 in the untrusted zone can only access the FTP service on R1 in the DMZ.

Background

To protect enterprise network security, you (the enterprise network administrator) decide to deploy a firewall at the border of the enterprise network to prevent external users from proactively accessing the internal network. In addition, as an egress device, the firewall needs to be configured with source NAT (for internal users to access the Internet) and NAT Server (mapping intranet servers to the public network).

The FTP service is provided for external access. FTP is a multi-channel protocol, which requires NAT ALG in addition to security policies to ensure normal communication after NAT is performed on the firewall.

Lab Configuration

Configuration Roadmap

- Complete basic device configurations for connectivity.

Add interfaces to security zones and configure a security policy to allow access from the local zone to the external zones.

Configure source NAT and NAT Server.

Configure a security policy to restrict the access between the untrusted zone and DMZ.

Check the session entries generated for the access traffic between the untrusted zone and DMZ on FW1.

Enable the FTP service on R1. Simulate FTP service access on R1 from R2, and run the dir command to transmit data through the FTP data channel. Then check the server mapping entries on FW1.

Configuration Procedure

Complete basic device configurations for connectivity.

Configure IP addresses for interconnected interfaces, configure VLANs on S1, and configure default routes on R1 and FW1.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Perform basic configurations on S1.

[S1]vlan 10

[S1-vlan10] description DMZ

[S1-vlan10] quit

[S1]interface GigabitEthernet0/0/1

[S1-GigabitEthernet0/0/1] port link-type access

[S1-GigabitEthernet0/0/1] port default vlan 10

[S1-GigabitEthernet0/0/1] quit

[S1]interface GigabitEthernet0/0/14

[S1-GigabitEthernet0/0/14] port link-type access

[S1-GigabitEthernet0/0/14] port default vlan 10

[S1-GigabitEthernet0/0/14] quit

[S1]vlan 20

[S1-vlan20] description Untrust

[S1-vlan20] quit

[S1]interface GigabitEthernet0/0/2

[S1-GigabitEthernet0/0/2] port link-type access

[S1-GigabitEthernet0/0/2] port default vlan 20

[S1-GigabitEthernet0/0/2] quit

[S1]interface GigabitEthernet0/0/15

[S1-GigabitEthernet0/0/15] port link-type access

[S1-GigabitEthernet0/0/15] port default vlan 20

[S1-GigabitEthernet0/0/15] quit

# Configure R1.

[R1]interface GigabitEthernet0/0/3

[R1-GigabitEthernet0/0/3] ip address 10.0.11.11 24

[R1-GigabitEthernet0/0/3] quit

[R1]ip route-static 0.0.0.0 0.0.0.0 10.0.11.1

Configure the default route for accessing the Internet.

# Configure R2.

[R2]interface GigabitEthernet0/0/4

[R2-GigabitEthernet0/0/3] ip address 10.0.12.2 255.255.255.0

[R2-GigabitEthernet0/0/3] quit

# Configure login data for FW1.

Login authentication

Username:admin

Password:

The password needs to be changed. Change now? [Y/N]: Y

Please enter old password:

Please enter new password:

Please confirm new password:

By default, login authentication is enabled for the console port of the firewall. The default user name and password are admin and Admin@123, respectively. After the first login to the firewall, you need to change the password to ensure subsequent successful login.

# Configure interface IP addresses and the default route on FW1.

[FW1]interface GigabitEthernet0/0/1

[FW1-GigabitEthernet0/0/1] ip address 10.0.11.1 255.255.255.0

[FW1-GigabitEthernet0/0/1] quit

[FW1]interface GigabitEthernet0/0/2

[FW1-GigabitEthernet0/0/2] ip address 10.0.12.1 255.255.255.0

[FW1-GigabitEthernet0/0/2] quit

[FW1]ip route-static 0.0.0.0 0.0.0.0 10.0.12.2

# Configure FW1 interfaces to permit ping packets.

[FW1]interface GigabitEthernet0/0/1

[FW1-GigabitEthernet0/0/1] service-manage ping permit

[FW1-GigabitEthernet0/0/1] quit

[FW1]interface GigabitEthernet0/0/2

[FW1-GigabitEthernet0/0/2] service-manage ping permit

[FW1-GigabitEthernet0/0/2] quit

By default, access control is enabled (using the service-manage command) on firewall interfaces, which implements security control at the interface layer and determines whether users can manage or access the firewall through a specific interface (for example, through ping, SSH, Telnet, or SNMP). GE0/0/0 is the NMS interface of the device. By default, the service-manage ping permit and service-manage ssh permit commands are configured on this interface. Therefore, users can manage the firewall through this interface. For other interfaces, the firewall does not allow users to manage or access the firewall through these interfaces by default, unless the service-manage command is manually configured. For example, to allow users to ping GE1/0/1, run the service-manage ping permit command on GE1/0/1. Similarly, to allow users to access GE1/0/1 using SSH, run the service-manage ssh permit command.

Configure a security policy for access from the local zone to other zones.

Add interfaces to security zones and create a security policy named local_to.

- Do not restrict source and destination IP addresses.

Do not restrict the destination security zone.

Do not restrict services.

Set the source security zone to local.

Set the action to permit.

# Add interfaces to security zones.

[FW1]firewall zone dmz

[FW1-zone-dmz] description DMZ

[FW1-zone-dmz] add interface GigabitEthernet0/0/1

[FW1-zone-dmz] quit

[FW1]firewall zone untrust

[FW1-zone-untrust] description Untrust

[FW1-zone-untrust] add interface GigabitEthernet0/0/2

[FW1-zone-untrust] quit

# Create a security policy named local_to.

[FW1]security-policy

[FW1-policy-security] rule name local_to

[FW1-policy-security-rule-local_to] source-zone local

[FW1-policy-security-rule-local_to] action permit

Since the source IP address, destination IP address, destination security zone, and services are not restricted, retain the default setting any for these parameters.

# Test the connectivity between FW1 and R1 interface IP addresses and between FW1 and R2 interface IP addresses.

<FW1>ping -c 1 10.0.11.11

PING 10.0.11.11: 56  data bytes, press CTRL_C to break

Reply from 10.0.11.11: bytes=56 Sequence=1 ttl=255 time=40 ms

--- 10.0.11.11 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/40/40 ms

<FW1>ping -c 1 10.0.12.2

PING 10.0.12.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.2: bytes=56 Sequence=1 ttl=255 time=27 ms

--- 10.0.12.2 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 27/27/27 ms

Configure source NAT and NAT Server.

Configure NAPT for intranet users (R1) to access the Internet and configure NAT Server to map the FTP service of R1 to the public network.

# Configure a NAT address pool and enable port address translation for reuse of public addresses.

[FW1]nat address-group 1

[FW1-address-group-1] mode pat

[FW1-address-group-1] section 0 10.0.12.1 10.0.12.1

[FW1-address-group-1] quit

# Configure a source NAT policy to enable source address translation for intranet users on a specified network segment when they access the Internet.

[FW1]nat-policy

[FW1-policy-nat] rule name 1

[FW1-policy-nat-rule-1] source-zone dmz

[FW1-policy-nat-rule-1] destination-zone untrust

[FW1-policy-nat-rule-1] source-address 10.0.11.0 24

[FW1-policy-nat-rule-1] action source-nat address-group 1

[FW1-policy-nat-rule-1] quit

# Configure NAT Server and create a static mapping to map the FTP service of R1.

[FW1]nat server policy_ftp protocol tcp global 10.0.12.1 ftp inside 10.0.11.11 ftp

# Enable NAT ALG for FTP.

[FW1]firewall zone dmz

[FW1-zone-dmz] detect ftp

[FW1-zone-dmz] quit

[FW1]firewall interzone dmz untrust

[FW1-interzone-dmz-untrust] detect ftp

[FW1-interzone-dmz-untrust] quit

Configure security policies for the DMZ-untrusted interzone.

Configure a security policy named DMZtoUntrust, limit the source address to 10.0.11.0/24, and set the action to permit. Configure a security policy named Untrust_DMZ to allow R2 to access only the FTP service provided by R1.

#Create a security policy named DMZtoUntrust.

[FW1]security-policy

[FW1-policy-security] rule name DMZtoUntrust

[FW1-policy-security-rule-DMZtoUntrust] source-zone dmz

[FW1-policy-security-rule-DMZtoUntrust] destination-zone untrust

[FW1-policy-security-rule-DMZtoUntrust] source-address 10.0.11.0 24

[FW1-policy-security-rule-DMZtoUntrust] action permit

# Create a security policy named Untrust_DMZ.

[FW1]security-policy

[FW1-policy-security-rule] rule name Untrust_DMZ

[FW1-policy-security-rule-Untrust_DMZ] source-zone untrust

[FW1-policy-security-rule-Untrust_DMZ] destination-zone dmz

[FW1-policy-security-rule-Untrust_DMZ] destination-address 10.0.11.11 24

[FW1-policy-security-rule-Untrust_DMZ] service ftp

[FW1-policy-security-rule-Untrust_DMZ] action permit

Note that the destination IP address is the mapped internal address. The security policy processes a packet after NAT Server changes the destination IP address of the packet.

Check sessions on FW1.

Ping R2 from R1 and check detailed session information on FW1.

# Test the access from R1 to R2.

<R1>ping -c 100 10.0.12.2

PING 10.0.12.2: 56  data bytes, press CTRL_C to break

Reply from 10.0.12.2: bytes=56 Sequence=1 ttl=254 time=60 ms

Reply from 10.0.12.2: bytes=56 Sequence=2 ttl=254 time=60 ms

R1 can access R2 through FW1. In this case, you can view detailed information about the related session on FW1.

# Check sessions on FW1.

<FW1>display firewall session table verbose destination global 10.0.12.2

2020-07-01 10:00:22.100

Current Total Sessions : 1

icmp  VPN: public --> public  ID: c487f0653c0805017ce5efc5e84

Zone: dmz --> untrust  TTL: 00:00:20  Left: 00:00:20

Recv Interface: GigabitEthernet0/0/1

Interface: GigabitEthernet0/0/2  NextHop: 10.0.12.2  MAC: 5489-98c8-4a33

<--packets: 80 bytes: 6,720 --> packets: 80 bytes: 6,720

10.0.11.11:52651[10.0.12.1:2048] --> 10.0.12.2:2048 PolicyName: DMZtoUntrust

View details about the session with the destination global IP address of 10.0.12.2. In the command output, you can view the direction of the session regarding the security zone, which is from DMZ to the untrusted zone; the aging time (TTL) of the session is 20s, the interface that receives packets is GigabitEthernet0/0/1, and the interface that sends packets is GigabitEthernet0/0/2. There are a total of 100 packets that match the session, and the total size of the packets is 8400 bytes. The name of the security policy matching the session is DMZtoUntrust.

According to the session, we can learn that the source IP address of the packets is translated from 10.0.11.11 to 10.0.12.1 (IP address of GE0/0/2 on FW1).

Observe the working process of NAT ALG.

Enable the FTP service on R1. Use R2 that serves as the FTP client to access the FTP service of R1 through the IP address mapped by FW1, and run the dir command to view the file list. Check how ASPF of FW1 processes multi-channel protocols.

# Enable the FTP service on R1.

[R1]aaa

[R1-aaa] local-user ftp service-type ftp

[R1-aaa] local-user ftp password cipher ftp@123

[R1-aaa] local-user ftp privilege level 15

[R1-aaa] local-user ftp ftp-directory flash:

[R1-aaa] quit

# Have R2 access the FTP service enabled on R1 through the address mapped by FW1.

<R2>ftp 10.0.12.1

Trying 10.0.12.1 ...

Press CTRL+K to abort

Connected to 10.0.12.1.

220 FTP service ready.

User(10.0.12.1:(none)):ftp

331 Password required for ftp.

Enter password:

230 User logged in.

R2 can access the FTP service enabled on R1 through NAT Server mapping of FW1.

# Check the session table on FW1.

<FW1>display firewall session table verbose protocol tcp destination-port global

21

2020-07-01 10:08:32.300

Current Total Sessions : 1

ftp  VPN: public --> public  ID: c487f0653c081382bee5efc6046

Zone: untrust --> dmz  TTL: 00:20:00  Left: 00:19:54

Recv Interface: GigabitEthernet1/0/2

Interface: GigabitEthernet1/0/1  NextHop: 10.0.11.11  MAC: 5489-98d9-4e30

<--packets: 11 bytes: 558 --> packets: 14 bytes: 598

10.0.12.2:64505 +-> 10.0.12.1:21[10.0.11.11:21] PolicyName: Untrust_DMZ

TCP State: established

The command output shows that the FTP control channel has been established.

# Run the dir command on R2.

[ftp]dir

200 Port command okay.

150 Opening ASCII mode data connection for *.

drwxrwxrwx			1	noone				nogroup									0				Aug	07			2015			src

drwxrwxrwx			1	noone				nogroup									0				Jun	07				16:46		pmdata

drwxrwxrwx			1	noone				nogroup									0				Jun	07				16:46		dhcp

-rwxrwxrwx				1	noone				nogroup									603		Jun	07				18:12		private-data.txt

drwxrwxrwx			1	noone				nogroup									0				Jun	07				17:01		mplstpoam

-rwxrwxrwx				1	noone				nogroup									482		Jun	07				17:51		vrpcfg.zip

226 Transfer complete.

The file list of R1 is displayed. In this case, the FTP transmission channel is used.

# Check the session table on FW1 again.

<FW1>display firewall session table

2020-07-01 10:14:10.310

Current Total Sessions : 1

ftp  VPN: public --> public  10.0.12.2:64505 +-> 10.0.12.1:21[10.0.11.11:21]

Only the FTP control channel session exists, and no transmission channel session exists.

# Check the server mapping entries generated by NAT ALG.

<FW1>display firewall server-map

2020-07-01 10:15:24.830

Current Total Server-map : 2

Type: Nat Server,  ANY -> 10.0.12.1:21[10.0.11.11:21],  Zone:---,  protocol:tcp

Vpn	: public -> public

Type: Nat Server Reverse,  10.0.11.11[10.0.12.1] -> ANY,  Zone:---,  protocol:tcp

Vpn	: public -> public,  counter: 1

The server mapping entry of the FTP data channel is generated on FW1.

Note that you need to run the dir command on R2 to trigger traffic on the transmission channel before checking the server mapping entry.

----End

Quiz

What is the purpose of permitting traffic from the local zone to other zones on the firewall?

Configuration Reference

Configuration on R1

#

sysname R1

#

FTP server enable

#

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user ftp password cipher iA7kS$rR@T=H)H2[EInBK@O#

local-user ftp privilege level 15

local-user ftp ftp-directory flash:

local-user ftp service-type ftp

local-user admin password cipher BJB3#A}[;JZypQCee$t3@bJ#

local-user admin service-type http

#

interface GigabitEthernet0/0/3

ip address 10.0.11.11 255.255.255.0

#

ip route-static 0.0.0.0 0.0.0.0 10.0.11.1

#

return

Configuration on R2

#

sysname R2

#

interface GigabitEthernet0/0/4

ip address 10.0.12.2 255.255.255.0

#

return

Configuration on S1

#

sysname S1

#

vlan batch 10 20

#

vlan 10

description DMZ

vlan 20

description Untrust

#

interface GigabitEthernet0/0/1

port link-type access

port default vlan 10

#

interface GigabitEthernet0/0/2

port link-type access

port default vlan 20

#

interface GigabitEthernet0/0/14

port link-type access

port default vlan 10

#

interface GigabitEthernet0/0/15

port link-type access

port default vlan 20

#

return

Configuration on FW1

#

sysname FW1

#

interface GigabitEthernet0/0/1

undo shutdown

ip address 10.0.11.1 255.255.255.0

service-manage ping permit

#

interface GigabitEthernet0/0/2

undo shutdown

ip address 10.0.12.1 255.255.255.0

service-manage ping permit

#

firewall zone local

set priority 100

#

firewall zone untrust

description Untrust

set priority 5

add interface GigabitEthernet0/0/2

#

firewall zone dmz

description DMZ

set priority 50

add interface GigabitEthernet0/0/1

detect ftp

#

firewall interzone dmz untrust

detect ftp

#

ip route-static 0.0.0.0 0.0.0.0 10.0.12.2

#

nat server policy_ftp protocol tcp global 10.0.12.1 ftp inside 10.0.11.11 ftp

#

nat address-group 1 0

mode pat

route enable

section 0 10.0.12.1 10.0.12.1

#

security-policy

rule name local_to

source-zone local

action permit

rule name DMZtoUntrust

source-zone dmz

destination-zone untrust

source-address 10.0.11.0 mask 255.255.255.0

action permit

rule name Untrust_DMZ

source-zone untrust

destination-zone dmz

destination-address 10.0.11.11 mask 255.255.255.255

service ftp

action permit

#

nat-policy

rule name 1

source-zone dmz

destination-zone untrust

source-address 10.0.11.0 mask 255.255.255.0

action source-nat address-group 1

#

Return

VRRP

Basic VRRP Configurations

Introduction

Objectives

Upon completion of this task, you will be able to:

Deploy VRRP.

Implement collaboration between VRRP and MSTP.

Configure association between BFD and VRRP.

Networking Topology

              - Basic VRRP configurations

Devices are connected as shown in the figure. VLAN 10 and VLAN 20 exist on the network, each with a VRRP group configured. The IDs of the VLANs are used as the VRIDs for their respective VRRP groups. S1 is configured as the master of the VRRP group in VLAN 10, and S2 as the master of the VRRP group in VLAN 20.

In addition, MSTP is deployed on S1, S2, and S3, and instances 1 and 2 are created. VLAN 10 is mapped to MSTI1, and VLAN 20 is mapped to MSTI2. S1 is configured as the primary root bridge of MSTI1 and the secondary root bridge of MSTI2. S2 is configured as the secondary root bridge of MSTI1 and the primary root bridge of MSTI2.

The IP address of each VLANIF interface is 10.0.x.y/24, where x indicates the VRID and y indicates the device ID. The virtual IP address is set to 10.0.x.254/24.

Background

To implement gateway redundancy, you as the network administrator need to deploy VRRP on two aggregation switches. To balance user-to-network traffic of terminal users, you need to deploy a VRRP group in each VLAN. To prevent loops, MSTP is deployed on the switching network and works with VRRP to implement load balancing.

Lab Configuration

Configuration Roadmap

- Create VLANs and configure MSTP on all switches. Manually specify S1 as the primary root bridge of MSTI1 and the secondary root bridge of MSTI2, and specify S2 as the secondary root bridge of MSTI1 and the primary root bridge of MSTI2.

- Create VLANIF 10 and VLANIF 20 and deploy VRRP groups 10 and 20 on both S1 and S2. Manually adjust VRRP priorities so that S1 functions as the master in VRRP group 10 and S2 functions as the master in VRRP group 20.

- Deploy single-hop BFD to detect the connectivity between VLANIF interfaces on S1 and S2. Associate BFD with VRRP to implement fast master/backup VRRP switchovers.

Configuration Procedure

          - Perform basic MSTP configurations.

Create VLANs 10 and 20 on all switches. Configure an MSTP region named hcip, and create two instances Instance 1 and Instance 2. Map VLAN 10 to Instance 1 and VLAN 20 to Instance 2. Plan S1 as the primary root bridge of MSTI1 and secondary root bridge of MSTI2; plan S2 as the primary root bridge of MSTI2 and the secondary root bridge of MSTI1.

# Name the devices.

The configuration details are not provided.

# Disable the interfaces that are not used in this experiment.

The configuration details are not provided.

# Create VLANs.

[S1]vlan batch 10 20

[S2]vlan batch 10 20

[S3]vlan batch 10 20

[S4]vlan batch 10 20

# Configure all interconnection interfaces as trunk interfaces and allow packets from the corresponding VLANs to pass through.

The configuration details are not provided.

# Change the working mode from STP to MSTP.

[S1]stp mode mstp

[S2]stp mode mstp

[S3]stp mode mstp

[S4]stp mode mstp

# Configure MSTP.

[S1]stp region-configuration

[S1-mst-region] region-name hcip

[S1-mst-region] revision-level 1

[S1-mst-region] instance 1 vlan 10

[S1-mst-region] instance 2 vlan 20

[S1-mst-region] active region-configuration

Info: This operation may take a few seconds. Please wait for a moment...done.

[S1-mst-region] quit

[S2]stp region-configuration

[S2-mst-region] region-name hcip

[S2-mst-region] revision-level 1

[S2-mst-region] instance 1 vlan 10

[S2-mst-region] instance 2 vlan 20

[S2-mst-region] active region-configuration

Info: This operation may take a few seconds. Please wait for a moment...done.

[S2-mst-region] quit

[S3]stp region-configuration

[S3-mst-region] region-name hcip

[S3-mst-region] revision-level 1

[S3-mst-region] instance 1 vlan 10

[S3-mst-region] instance 2 vlan 20

[S3-mst-region] active region-configuration

Info: This operation may take a few seconds. Please wait for a moment...done.

[S3-mst-region] quit

[S4]stp region-configuration

[S4-mst-region] region-name hcip

[S4-mst-region] revision-level 1

[S4-mst-region] instance 1 vlan 10

[S4-mst-region] instance 2 vlan 20

[S4-mst-region] active region-configuration

Info: This operation may take a few seconds. Please wait for a moment...done.

[S4-mst-region] quit

# Check the mappings between MSTI instances and VLANs on S1.

[S1]display stp region-configuration

Oper configuration

Format	selector				:0

Region	name						:hcip

Revision	level						:1

Instance   VLANs Mapped

0       1 to 9, 11 to 19, 21 to 29, 31 to 39, 41 to 49, 51 to 59, 61 to

69, 71 to 79, 81 to 4094

1       10,

2       20

# Configure S1 as the primary root bridge of MSTI1 and the secondary root bridge of MSTI2.

[S1]stp instance 1 root primary

[S1]stp instance 2 root secondary

# Configure S2 as the primary root bridge of MSTI2 and the secondary root bridge of MSTI1.

[S2]stp instance 1 root secondary

[S2]stp instance 2 root primary

# Check the status and statistics of MSTI1 on S1.

[S1]display stp instance 1 brief

MSTID Port                        Role  STP State          Protection

1    GigabitEthernet0/0/10       DESI  FORWARDING      NONE

1    GigabitEthernet0/0/11       DESI  FORWARDING      NONE

1    GigabitEthernet0/0/12       DESI  FORWARDING      NONE

All interfaces on S1 are designated interfaces, and S1 is the primary root bridge of MSTI1.

# Check the status and statistics of MSTI2 on S2.

[S2]display stp instance 2 brief

MSTID Port                        Role  STP State          Protection

2    GigabitEthernet0/0/10       DESI  FORWARDING      NONE

2    GigabitEthernet0/0/11       DESI  FORWARDING      NONE

2    GigabitEthernet0/0/13       DESI  FORWARDING      NONE

All interfaces on S2 are designated interfaces, and S2 is the secondary root bridge of MSTI1.

          - Perform basic VRRP configurations.

Create VLANIF 10 and VLANIF 20 on both S1 and S2, and add VLANIF 10 to VRRP group 10 and VLANIF 20 to VRRP group 20. Configure VRRP priorities so that S1 in VLAN 10 and S2 in VLAN 20 both function as the VRRP master.

# Create VLANIF interfaces.

[S1]interface Vlanif10

[S1-Vlanif10] ip address 10.0.10.1 255.255.255.0

[S1-Vlanif10] quit

[S1]interface Vlanif20

[S1-Vlanif20] ip address 10.0.20.1 255.255.255.0

[S1-Vlanif20] quit

[S2]interface Vlanif10

[S2-Vlanif10] ip address 10.0.10.2 255.255.255.0

[S2-Vlanif10] quit

[S2]interface Vlanif20

[S2-Vlanif20] ip address 10.0.20.2 255.255.255.0

[S2-Vlanif20] quit

# Configure VRRP groups on S1.

[S1]interface Vlanif 10

[S1-Vlanif10] vrrp vrid 10 virtual-ip 10.0.10.254

[S1-Vlanif10] vrrp vrid 10 priority 120

[S1-Vlanif10] quit

[S1]interface Vlanif 20

[S1-Vlanif20] vrrp vrid 20 virtual-ip 10.0.20.254

[S1-Vlanif20] quit

Set the VRRP priority to 120 for S1 in VLAN 10, and use the default priority 100 for S1 in VLAN 20.

# Configure VRRP groups on S2.

[S2]interface Vlanif10

[S2-Vlanif10] vrrp vrid 10 virtual-ip 10.0.10.254

[S2-Vlanif10] quit

[S2]interface Vlanif20

[S2-Vlanif20] vrrp vrid 20 virtual-ip 10.0.20.254

[S2-Vlanif20] vrrp vrid 20 priority 120

[S2-Vlanif20] quit

Set the VRRP priority to 120 for S2 in VLAN 20, and use the default priority 100 for S2 in VLAN 10.

# Check the VRRP status.

<S1>display vrrp brief

VRID		State								Interface																Type					Virtual	IP

----------------------------------------------------------------

10				Master							Vlanif10																	Normal			10.0.10.254

20				Backup							Vlanif20																	Normal			10.0.20.254

----------------------------------------------------------------

Total:2     Master:1     Backup:1     Non-active:0

[S2]display vrrp brief

VRID		State								Interface																Type					Virtual	IP

----------------------------------------------------------------

10				Backup							Vlanif10																	Normal			10.0.10.254

20				Master							Vlanif20																	Normal			10.0.20.254

----------------------------------------------------------------

Total:2     Master:1     Backup:1     Non-active:0

The VRRP status is the same as expected.

          - Configure association between VRRP and BFD to implement rapid master/backup VRRP switchovers.

Configure single-hop BFD on S1 and S2 to detect the connectivity between VLANIF interfaces. Associate VRRP with BFD so that the backup device increases its VRRP priority when the BFD session goes down.

# Create BFD sessions on S1.

[S1]bfd

[S1-bfd] quit

[S1]bfd vlanif10 bind peer-ip 10.0.10.2 interface Vlanif10

[S1-bfd-session-vlanif10] discriminator local 1

[S1-bfd-session-vlanif10] discriminator remote 2

[S1-bfd-session-vlanif10] min-tx-interval 100

[S1-bfd-session-vlanif10] min-rx-interval 100

[S1-bfd-session-vlanif10] commit

[S1-bfd-session-vlanif10] quit

[S1]bfd vlanif20 bind peer-ip 10.0.20.2 interface Vlanif20

[S1-bfd-session-vlanif20] discriminator local 11

[S1-bfd-session-vlanif20] discriminator remote 22

[S1-bfd-session-vlanif20] min-tx-interval 100

[S1-bfd-session-vlanif20] min-rx-interval 100

[S1-bfd-session-vlanif20] commit

[S1-bfd-session-vlanif20] quit

# Create BFD sessions on S2.

[S2]bfd

[S2-bfd] quit

[S2]bfd vlanif10 bind peer-ip 10.0.10.1 interface Vlanif10

[S2-bfd-session-vlanif10] discriminator local 2

[S2-bfd-session-vlanif10] discriminator remote 1

[S2-bfd-session-vlanif10] min-tx-interval 100

[S2-bfd-session-vlanif10] min-rx-interval 100

[S2-bfd-session-vlanif10] commit

[S2-bfd-session-vlanif10] quit

[S2]bfd vlanif20 bind peer-ip 10.0.20.1 interface Vlanif20

[S2-bfd-session-vlanif20] discriminator local 22

[S2-bfd-session-vlanif20] discriminator remote 11

[S2-bfd-session-vlanif20] min-tx-interval 100

[S2-bfd-session-vlanif20] min-rx-interval 100

[S2-bfd-session-vlanif20] commit

[S2-bfd-session-vlanif20] quit

# Check the BFD session status.

[S1]display bfd session all

--------------------------------------------------------------------------------

Local Remote     PeerIpAddr      State     Type       InterfaceName

--------------------------------------------------------------------------------

1     2          10.0.10.2       Up        S_IP_IF     Vlanif10

11    22         10.0.20.2       Up        S_IP_IF     Vlanif20

--------------------------------------------------------------------------------

Total UP/DOWN Session Number : 2/0

[S2]display bfd session all

--------------------------------------------------------------------------------

Local Remote     PeerIpAddr      State     Type       InterfaceName

--------------------------------------------------------------------------------

2     1          10.0.10.1       Up        S_IP_IF     Vlanif10

22    11         10.0.20.1       Up        S_IP_IF     Vlanif20

--------------------------------------------------------------------------------

Total UP/DOWN Session Number : 2/0

The BFD sessions on S1 and S2 are in the Up state.

# Configure association between VRRP and BFD.

[S1]interface Vlanif20

[S1-Vlanif20] vrrp vrid 20 track bfd-session 11 increased 30

[S1-Vlanif20] quit

[S2]interface Vlanif10

[S2-Vlanif10] vrrp vrid 10 track bfd-session 2 increased 30

[S2-Vlanif10] quit

Note that bfd-session-id specifies the local discriminator of a BFD session. You only need to configure the backup device to track the BFD session. In this way, the backup device increases its local VRRP priority when the BFD session goes down.

# Shut down all interfaces on S1 to simulate a link fault.

[S1]interface GigabitEthernet0/0/10

[S1-GigabitEthernet0/0/10] shutdown

[S1-GigabitEthernet0/0/10] quit

[S1]interface GigabitEthernet0/0/11

[S1-GigabitEthernet0/0/11] shutdown

[S1-GigabitEthernet0/0/11] quit

[S1]interface GigabitEthernet0/0/12

[S1-GigabitEthernet0/0/12] shutdown

[S1-GigabitEthernet0/0/12] quit

# Check the BFD session status on S2.

<S2>display bfd session all

--------------------------------------------------------------------------------

Local Remote     PeerIpAddr      State     Type       InterfaceName

--------------------------------------------------------------------------------

2     1          10.0.10.1       Down      S_IP_IF     Vlanif10

22    11         10.0.20.1       Down      S_IP_IF     Vlanif20

--------------------------------------------------------------------------------

Total UP/DOWN Session Number : 0/2

The two BFD sessions immediately enter the Down state.

# Check the VRRP status on S2.

<S2>display vrrp brief

VRID  State        Interface                Type    	 Virtual IP

----------------------------------------------------------------

10    Master       Vlanif10                 Normal   10.0.10.254

20    Master       Vlanif20                 Normal   10.0.20.254

----------------------------------------------------------------

Total:2     Master:2     Backup:0     Non-active:0

S2 functions as the master in both VRRP groups 10 and 20.

# Check the VRRP status and configuration parameters on S2.

[S2]display vrrp

Vlanif10 | Virtual Router 10

State										:	Master

Virtual	IP							:	10.0.10.254

Master	IP							:	10.0.10.2

PriorityRun						:	130

PriorityConfig			:	100

MasterPriority			:	130

Preempt								:	YES			Delay	Time	:	0	s

TimerRun							:	1	s

TimerConfig					:	1	s

Auth	type							:	NONE

Virtual	MAC					:	0000-5e00-010a

Check	TTL							:	YES

Config	type					:	normal-vrrp

Track	BFD						:	2		Priority	increased	:	30

BFD-session	state	:	DOWN

Create	time					:	2020-06-05	11:01:54	UTC-08:00

Last	change	time	:	2020-06-05	11:31:15	UTC-08:00

Vlanif20 | Virtual Router 20

State										:	Master

Virtual	IP							:	10.0.20.254

Master	IP							:	10.0.20.2

PriorityRun						:	120

PriorityConfig			:	120

MasterPriority			:	120

Preempt								:	YES			Delay	Time	:	0	s

TimerRun							:	1	s

TimerConfig					:	1	s

Auth	type							:	NONE

Virtual	MAC					:	0000-5e00-0114

Check	TTL							:	YES

Config	type					:	normal-vrrp

Create	time					:	2020-06-05	11:01:54	UTC-08:00

Last	change	time	:	2020-06-05	11:01:55	UTC-08:00

The priority of VRRP group 10 is 130, and the BFD session is in the Down state. The BFD down event triggers S2 to increase the priority of VRRP group 10 by 30.

----End

Quiz

In what situation does a device send VRRP packets carrying a priority of 255?

Configuration Reference

Configuration on S1

#

sysname S1

#

vlan batch 10 20

#

stp instance 1 root primary

stp instance 2 root secondary

#

stp region-configuration

region-name hcip

revision-level 1

instance 1 vlan 10

instance 2 vlan 20

active region-configuration

#

bfd

#

interface Vlanif10

ip address 10.0.10.1 255.255.255.0

vrrp vrid 10 virtual-ip 10.0.10.254

vrrp vrid 10 priority 120

#

interface Vlanif20

ip address 10.0.20.1 255.255.255.0

vrrp vrid 20 virtual-ip 10.0.20.254

vrrp vrid 20 track bfd-session 11 increased 30

#

interface GigabitEthernet0/0/10

shutdown

port link-type trunk

port trunk allow-pass vlan 10 20

#

interface GigabitEthernet0/0/11

shutdown

port link-type trunk

port trunk allow-pass vlan 10 20

#

interface GigabitEthernet0/0/12

shutdown

port link-type trunk

port trunk allow-pass vlan 10 20

#

bfd vlanif10 bind peer-ip 10.0.10.2 interface Vlanif10

discriminator local 1

discriminator remote 2

min-tx-interval 100

min-rx-interval 100

commit

#

bfd vlanif20 bind peer-ip 10.0.20.2 interface Vlanif20

discriminator local 11

discriminator remote 22

min-tx-interval 100

min-rx-interval 100

commit

#

return

Configuration on S2

#

sysname S2

#

vlan batch 10 20

#

stp instance 1 root secondary

stp instance 2 root primary

#

stp region-configuration

region-name hcip

revision-level 1

instance 1 vlan 10

instance 2 vlan 20

active region-configuration

#

bfd

#

interface Vlanif10

ip address 10.0.10.2 255.255.255.0

vrrp vrid 10 virtual-ip 10.0.10.254

vrrp vrid 10 track bfd-session 2 increased 30

#

interface Vlanif20

ip address 10.0.20.2 255.255.255.0

vrrp vrid 20 virtual-ip 10.0.20.254

vrrp vrid 20 priority 120

#

interface GigabitEthernet0/0/10

port link-type trunk

port trunk allow-pass vlan 10 20

#

interface GigabitEthernet0/0/11

port link-type trunk

port trunk allow-pass vlan 10 20

#

interface GigabitEthernet0/0/13

port link-type trunk

port trunk allow-pass vlan 10 20

#

bfd vlanif10 bind peer-ip 10.0.10.1 interface Vlanif10

discriminator local 2

discriminator remote 1

min-tx-interval 100

min-rx-interval 100

commit

#

bfd vlanif20 bind peer-ip 10.0.20.1 interface Vlanif20

discriminator local 22

discriminator remote 11

min-tx-interval 100

min-rx-interval 100

commit

#

return

Configuration on S3

#

sysname S3

#

vlan batch 10 20

#

stp region-configuration

region-name hcip

revision-level 1

instance 1 vlan 10

instance 2 vlan 20

active region-configuration

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 10 20

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 10 20

#

return

DHCP

DHCP Relay Configuration

Introduction

Objectives

Upon completion of this task, you will be able to:

Deploy a DHCP relay agent to enable terminals to dynamically obtain IP addresses.

Configure DHCP static address binding.

Analyze the debugging information of a DHCP relay agent.

Networking Topology

              - DHCP relay configuration

VLANIF 10, VLANIF 20, and VLANIF 30 are created on S4 to simulate DHCP clients. S3 and S1 function as a DHCP relay agent and DHCP server, respectively. A global address pool is created on S1 to allocate IP addresses to the three VLANIF interfaces of S4.

The interfaces between S3 and S4 are configured to work in trunk mode to allow VLANs 10, 20, and 30 to pass through; the interfaces between S1 and S3 are configured to work in access mode. The PVID is set to 40.

Background

You are a network administrator of a company. Because there are a large number of hosts on the network, static address allocation is difficult to manage. Therefore, a DHCP server needs to be deployed.

The core switch S1 functions as a DHCP server, S4 as a DHCP client, and S3 as the gateway of each network segment. DHCPDISCOVER messages are broadcast ones and cannot traverse routers. Therefore, DHCP relay needs to be deployed on S3 to unicast the messages to S1.

In addition, DHCP is required to allocate fixed IP addresses to special clients, such as servers and printers.

Lab Configuration

Configuration Roadmap

- Create VLANs on each switch, configure interfaces to work in the corresponding mode, and allow the corresponding VLANs to pass through.

- Create an address pool on the DHCP server to allocate IP addresses to terminals, and configure static address allocation.

- Configure the IP address of the DHCP server on the DHCP relay agent's interface.

- Enable the DHCP client to obtain an IP address through DHCP.

- Observe the DHCP packet relay process on the DHCP relay agent through debugging.

Configuration Procedure

          - Perform basic configurations.

Create VLANs and VLANIF interfaces on the three switches, and configure interfaces to allow the corresponding VLANs to pass through. The IP address of the VLANIF interface is 10.0.x.y/24, where x and y indicate the VLAN ID and device number, respectively. IP addresses do not need to be configured for the VLANIF interfaces on S4.

# Create VLANs.

[S1]vlan 40

[S3]vlan batch 10 20 30 40

[S4]vlan batch 10 20 30

# Configure interfaces to allow the corresponding VLANs to pass through.

[S4]interface GigabitEthernet0/0/3

[S4-GigabitEthernet0/0/3] port link-type trunk

[S4-GigabitEthernet0/0/3] port trunk allow-pass vlan 10 20 30

[S4-GigabitEthernet0/0/3] quit

[S3]interface GigabitEthernet0/0/1

[S3-GigabitEthernet0/0/1] port link-type access

[S3-GigabitEthernet0/0/1] port default vlan 40

[S3-GigabitEthernet0/0/1] quit

[S3]interface GigabitEthernet0/0/3

[S3-GigabitEthernet0/0/3] port link-type trunk

[S3-GigabitEthernet0/0/3] port trunk allow-pass vlan 10 20 30

[S3-GigabitEthernet0/0/3] quit

[S1]interface GigabitEthernet0/0/12

[S1-GigabitEthernet0/0/12] port link-type access

[S1-GigabitEthernet0/0/12] port default vlan 40

[S1-GigabitEthernet0/0/12] quit

# Configure VLANIF interfaces.

[S4]interface Vlanif 10

[S4-Vlanif10] quit

[S4]interface Vlanif 20

[S4-Vlanif20] quit

[S4]interface Vlanif 30

[S4-Vlanif30] quit

[S3]interface Vlanif 10

[S3-Vlanif10] ip address 10.0.10.3 24

[S3-Vlanif10] quit

[S3]interface Vlanif 20

[S3-Vlanif20] ip address 10.0.20.3 24

[S3-Vlanif20] quit

[S3]interface Vlanif 30

[S3-Vlanif30] ip address 10.0.30.3 24

[S3-Vlanif30] quit

[S3]interface Vlanif 40

[S3-Vlanif40] ip address 10.0.40.3 24

[S3-Vlanif40] quit

[S1]interface Vlanif 40

[S1-Vlanif40] ip address 10.0.40.1 24

[S1-Vlanif40] quit

# Check the connectivity of VLANIF 40 between S1 and S3.

[S1]ping -c 1 10.0.40.3

PING 10.0.40.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.40.3: bytes=56 Sequence=1 ttl=255 time=60 ms

--- 10.0.40.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 60/60/60 ms

The DHCP server and relay agent can communicate with each other.

          - Configure the DHCP server.

Enable the DHCP service, configure a global address pool, and allocate a static IP address to VLANIF 30 on S4.

# Enable the DHCP service.

[S1]dhcp enable

# Create an IP address pool named vlan10 to allocate an IP address to VLANIF 10 of S4.

[S1]ip pool vlan10

[S1-ip-pool-vlan10] gateway-list 10.0.10.3

[S1-ip-pool-vlan10] network 10.0.10.0 mask 255.255.255.0

[S1-ip-pool-vlan10] dns-list 10.0.10.3

[S1-ip-pool-vlan10] quit

# Create an IP address pool named vlan20 to allocate an IP address to VLANIF 20 of S4.

[S1]ip pool vlan20

[S1-ip-pool-vlan20] gateway-list 10.0.20.3

[S1-ip-pool-vlan20] network 10.0.20.0 mask 255.255.255.0

[S1-ip-pool-vlan20] dns-list 10.0.20.3

[S1-ip-pool-vlan20] quit

# Create an IP address pool named vlan30 to allocate an IP address to VLANIF 30 of S4.

[S1]ip pool vlan30

[S1-ip-pool-vlan30] gateway-list 10.0.30.3

[S1-ip-pool-vlan30] network 10.0.30.0 mask 255.255.255.0

[S1-ip-pool-vlan30] dns-list 10.0.30.3

[S1-ip-pool-vlan30] quit

# Check the MAC address of VLANIF 30 on S4.

[S4]display interface Vlanif 30

Vlanif30 current state											: UP

Line protocol current state							: DOWN

Description:

Route Port,The Maximum Transmit Unit is 1500

Internet protocol processing					: disabled

IP Sending Frames' Format is PKTFMT_ETHNT_2, Hardware address is 4c1f-cc49-4c7c

Current system tim3												: 2020-06-05 16:51:20-08:00

Input bandwidth utilization			: --

Output bandwidth utilization	: --

The actual MAC address is subject to the device in the lab environment.

# On S1, configure static address allocation for VLANIF 30 of S4.

[S1]ip pool vlan30

[S1-ip-pool-vlan30] static-bind ip-address 10.0.30.2 mac-address 4c1f-cc49-4c7c

Allocate the fixed IP address 10.0.30.2 to VLANIF 30 of S4.

# Enable the DHCP server function on VLANIF 40.

[S1]interface Vlanif 40

[S1-Vlanif40] dhcp select global

# Check the IP address pool configurations.

[S1]display ip pool name vlan10

Pool-name						:	vlan10

Pool-No								:	0

Lease										:	1	Days	0	Hours	0	Minutes

Domain-name			:	-

DNS-server0				:	10.0.10.3

NBNS-server0			:	-

Netbios-type				:	-

Position								:	Local											Status											:	Unlocked

Gateway-0						:	10.0.10.3

Mask										:	255.255.255.0

VPN instance				: --

-----------------------------------------------------------------------------

Start           End     Total  Used  Idle(Expired)  Conflict  Disable

-----------------------------------------------------------------------------

10.0.10.1     10.0.10.254   253     0        253(0)         0        0

-----------------------------------------------------------------------------

[S1]display ip pool name vlan20

Pool-name						:	vlan20

Pool-No								:	1

Lease										:	1	Days	0	Hours	0	Minutes

Domain-name			:	-

DNS-server0				:	10.0.20.3

NBNS-server0			:	-

Netbios-type				:	-

Position								:	Local											Status											:	Unlocked

Gateway-0						:	10.0.20.3

Mask										:	255.255.255.0

VPN	instance				:	--

-----------------------------------------------------------------------------

Start           End     Total  Used  Idle(Expired)  Conflict  Disable

-----------------------------------------------------------------------------

10.0.20.1     10.0.20.254   253     0        253(0)         0        0

-----------------------------------------------------------------------------

[S1]display ip pool name vlan30

Pool-name						:	vlan30

Pool-No								:	2

Lease										:	1	Days	0	Hours	0	Minutes

Domain-name			:	-

DNS-server0				:	10.0.30.3

NBNS-server0			:	-

Netbios-type				:	-

Position								:	Local											Status											:	Unlocked

Gateway-0						:	10.0.30.3

Mask										:	255.255.255.0

VPN	instance				:	--

-----------------------------------------------------------------------------

Start           End     Total  Used  Idle(Expired)  Conflict  Disable

-----------------------------------------------------------------------------

10.0.30.1     10.0.30.254   253     1        252(0)         0        0

-----------------------------------------------------------------------------

The address pool vlan30 already has a used IP address, which is a static one.

# Configure routes to user network segments.

[S1]ip route-static 10.0.10.0 24 10.0.40.3

[S1]ip route-static 10.0.20.0 24 10.0.40.3

[S1]ip route-static 10.0.30.0 24 10.0.40.3

After receiving a DHCP message from the DHCP client, the DHCP relay agent uses the interface IP address to relay the message to the DHCP server. Therefore, the connectivity between the interface IP address and DHCP server must be ensured.

# Check the connectivity between the DHCP server and relay interface.

<S1>ping -c 1 10.0.10.3

PING 10.0.10.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.10.3: bytes=56 Sequence=1 ttl=255 time=50 ms

--- 10.0.10.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 50/50/50 ms

<S1>ping -c 1 10.0.20.3

PING 10.0.20.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.20.3: bytes=56 Sequence=1 ttl=255 time=40 ms

--- 10.0.20.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 40/40/40 ms

<S1>ping -c 1 10.0.30.3

PING 10.0.30.3: 56  data bytes, press CTRL_C to break

Reply from 10.0.30.3: bytes=56 Sequence=1 ttl=255 time=30 ms

--- 10.0.30.3 ping statistics ---

1 packet(s) transmitted

1 packet(s) received

0.00% packet loss

round-trip min/avg/max = 30/30/30 ms

          - Configure the DHCP relay agent.

Configure DHCP relay on S3.

# Enable the DHCP service.

[S3]dhcp enable

# Configure DHCP relay on interfaces and specify the DHCP server.

[S3]interface Vlanif10

[S3-Vlanif10] dhcp select relay

[S3-Vlanif10] dhcp relay server-ip 10.0.40.1

[S3-Vlanif10] quit

[S3]interface Vlanif20

[S3-Vlanif20] dhcp select relay

[S3-Vlanif20] dhcp relay server-ip 10.0.40.1

[S3-Vlanif20] quit

[S3]interface Vlanif30

[S3-Vlanif30] dhcp select relay

[S3-Vlanif30] dhcp relay server-ip 10.0.40.1

[S3-Vlanif30] quit

# Verify the DHCP relay configuration.

[S3]display dhcp relay all

DHCP relay agent running information of interface Vlanif10 :

Server IP address [01] : 10.0.40.1

Gateway address in use : 10.0.10.3

DHCP relay agent running information of interface Vlanif20 :

Server IP address [01] : 10.0.40.1

Gateway address in use : 10.0.20.3

DHCP relay agent running information of interface Vlanif30 :

Server IP address [01] : 10.0.40.1

Gateway address in use : 10.0.30.3

          - Configure the DHCP client.

Configure VLANIF 10, VLANIF 20, and VLANIF 30 on S4 to obtain IP addresses through DHCP.

# Enable the DHCP service.

[S4]dhcp enable

# Enable the interfaces to obtain IP addresses through DHCP.

[S4]interface Vlanif10

[S4-Vlanif10] ip address dhcp-alloc

[S4-Vlanif10] quit

[S4]interface Vlanif20

[S4-Vlanif20] ip address dhcp-alloc

[S4-Vlanif20] quit

[S4]interface Vlanif30

[S4-Vlanif30] ip address dhcp-alloc

[S4-Vlanif30] quit

# Check the IP address allocated to each interface.

<S4>display interface Vlanif 10

Vlanif10 current state					: UP

Line protocol current state	: UP

Last line protocol up time	: 2020-06-05 17:37:57 UTC-08:00

Description:

Route Port,The Maximum Transmit Unit is 1500

Internet Address is allocated by DHCP, 10.0.10.254/24

[S4]display interface Vlanif 20

Vlanif20 current state					: UP

Line protocol current state	: UP

Last line protocol up time	: 2020-06-05 17:41:23 UTC-08:00

Description:

Route Port,The Maximum Transmit Unit is 1500

Internet Address is allocated by DHCP, 10.0.20.254/24

[S4]display interface Vlanif 30

Vlanif30 current state					: UP

Line protocol current state	: UP

Last line protocol up time	: 2020-06-05 17:43:22 UTC-08:00

Description:

Route Port,The Maximum Transmit Unit is 1500

Internet Address is allocated by DHCP, 10.0.30.2/24

The interfaces have obtained IP addresses through DHCP, and the IP address of VLANIF 30 is the static IP address 10.0.30.2.

          - Observe the DHCP relay process.

Run the debugging dhcp relay info and debugging dhcp relay packet commands on S3. Disable the DHCP client function on VLANIF 30 of S4, and then enable it again. Check the debugging information.

# Enable debugging on S3.

<S3>debugging dhcp relay info

<S3>debugging dhcp relay packet

<S3>terminal debugging

Info: Current terminal debugging is on.

<S3>terminal monitor

Info: Current terminal monitor is on.

# Disable the DHCP client function on VLANIF 30 of S4.

[S4]interface Vlanif 30

[S4-Vlanif30] undo ip address dhcp-alloc

# Check the debugging information on S3.

Jun  5 2020 18:41:41.510.1-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Receives DHCP RELEASE message from interface Vlanif30.

Jun  5 2020 18:41:41.510.2-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:srcip:10.0.30.2 dstip:10.0.40.1 vpnid:0

Jun  5 2020 18:41:41.510.3-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:msgtype:BOOT-REQUEST dhcp msgtype:DHCP RELEASE bflag:uc chaddr:4c1f-cc49-4c7c ciaddr:10.0.30.2 reqip:0.0.0.0 giaddr:0.0.0.0 serverid:10.0.40.1

Jun  5 2020 18:41:41.510.4-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:Select 10.0.30.3 as giaddr.

Jun  5 2020 18:41:41.510.5-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Relay DHCP RELEASE to server 10.0.40.1.

S3 receives a DHCPRELEASE message from VLANIF 30. The source and destination IP addresses of the message are 10.0.30.2 and 10.0.40.1, respectively, and the value of the giaddr field is 0.0.0.0. S3 sets the giaddr field in the message to 10.0.30.3 (IP address of VLANIF 30) and then sends the message to the DHCP server.

# Enable the DHCP client function on VLANIF 30 of S4 again.

[S4]interface Vlanif 30

[S4-Vlanif30] ip address dhcp-alloc

# Check the debugging information on S3.

Jun  5 2020 18:38:42.600.1-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Receives DHCP DISCOVER message from interface Vlanif30.

Jun  5 2020 18:38:42.600.2-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:srcip:0.0.0.0 dstip:255.255.255.255 vpnid:0

Jun  5 2020 18:38:42.600.3-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:msgtype:BOOT-REQUEST dhcp msgtype:DHCP DISCOVER bflag:uc chaddr:4c1f-cc49-4c7c ciaddr:0.0.0.0 reqip:0.0.0.0 giaddr:0.0.0.0 serverid:0.0.0.0

Jun  5 2020 18:38:42.600.4-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:Select 10.0.30.3 as giaddr.

Jun  5 2020 18:38:42.600.5-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Relay DHCP DISCOVER to server 10.0.40.1.

S3 receives a DHCPDISCOVER message from the client. The source and destination IP addresses of the message are 0.0.0.0 and 255.255.255.255, respectively. After setting the giaddr field in the message to 10.0.30.3, S3 unicasts the message to the DHCP server at 10.0.40.1. In this case, the source IP address of the message is 10.0.30.3.

Jun  5 2020 18:38:42.610.1-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Receives DHCP OFFER message from interface Vlanif40.

Jun  5 2020 18:38:42.610.2-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:srcip:10.0.40.1 dstip:10.0.30.3 vpnid:0

Jun  5 2020 18:38:42.610.3-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:msgtype:BOOT-REPLY dhcp msgtype:DHCP OFFER bflag:uc chaddr:4c1f-cc49-4c7c ciaddr:0.0.0.0 reqip:0.0.0.0 giaddr:10.0.30.3 serverid:10.0.40.1

Jun  5 2020 18:38:42.610.4-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Unicast DHCP OFFER to client. (Chaddr=4c1f-cc49-4c7c, Ciaddr=10.0.30.2)

S3 receives a DHCPOFFER message from the DHCP server. The source and destination IP addresses of the message are 10.0.40.1 and 10.0.30.3, respectively, and the message carries the DHCP server ID (DHCP Option 54). S3 then unicasts the message to the client.

Jun  5 2020 18:38:42.650.1-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Receives DHCP REQUEST message from interface Vlanif30.

Jun  5 2020 18:38:42.650.2-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:srcip:0.0.0.0 dstip:255.255.255.255 vpnid:0

Jun  5 2020 18:38:42.650.3-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:msgtype:BOOT-REQUEST dhcp msgtype:DHCP REQUEST bflag:uc chaddr:4c1f-cc49-4c7c ciaddr:0.0.0.0 reqip:10.0.30.2 giaddr:0.0.0.0 serverid:10.0.40.1

Jun  5 2020 18:38:42.650.4-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:Select 10.0.30.3 as giaddr.

Jun  5 2020 18:38:42.650.5-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Relay DHCP REQUEST to server 10.0.40.1.

After receiving a DHCPREQUEST broadcast message from the client, S3 converts the message into a unicast one and then sends it to the DHCP server.

Jun  5 2020 18:38:42.660.1-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Receives DHCP ACK message from interface Vlanif40.

Jun  5 2020 18:38:42.660.2-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:srcip:10.0.40.1 dstip:10.0.30.3 vpnid:0

Jun  5 2020 18:38:42.660.3-08:00 S3 DHCP/7/DEBUG:[dhcpr-info]:msgtype:BOOT-REPLY dhcp msgtype:DHCP ACK bflag:uc chaddr:4c1f-cc49-4c7c ciaddr:0.0.0.0 reqip:0.0.0.0 giaddr:10.0.30.3 serverid:10.0.40.1

Jun  5 2020 18:38:42.660.4-08:00 S3 DHCP/7/DEBUG:[dhcpr-pkt]:Unicast DHCP ACK to client. (Chaddr=4c1f-cc49-4c7c, Ciaddr=10.0.30.2)

After receiving a DHCPACK message with the source IP address 10.0.40.1 and destination IP address 10.0.30.3 from the DHCP server, S3 unicasts the message to the DHCP client.

----End

Quiz

How does a DHCP server select an address pool after receiving a DHCP message from a DHCP relay agent?

Configuration Reference

Configuration on S1

#

sysname S1

#

vlan batch 40

#

dhcp enable

#

ip pool vlan10

gateway-list 10.0.10.3

network 10.0.10.0 mask 255.255.255.0

dns-list 10.0.10.3

#

ip pool vlan20

gateway-list 10.0.20.3

network 10.0.20.0 mask 255.255.255.0

dns-list 10.0.20.3

#

ip pool vlan30

gateway-list 10.0.30.3

network 10.0.30.0 mask 255.255.255.0

static-bind ip-address 10.0.30.2 mac-address 4c1f-cc49-4c7c

dns-list 10.0.30.3

#

interface Vlanif40

ip address 10.0.40.1 255.255.255.0

dhcp select global

#

interface GigabitEthernet0/0/12

port link-type access

port default vlan 40

#

ip route-static 10.0.10.0 255.255.255.0 10.0.40.3

ip route-static 10.0.20.0 255.255.255.0 10.0.40.3

ip route-static 10.0.30.0 255.255.255.0 10.0.40.3

#

return

Configuration on S3

#

sysname S3

#

vlan batch 10 20 30 40

#

dhcp enable

#

interface Vlanif10

ip address 10.0.10.3 255.255.255.0

dhcp select relay

dhcp relay server-ip 10.0.40.1

#

interface Vlanif20

ip address 10.0.20.3 255.255.255.0

dhcp select relay

dhcp relay server-ip 10.0.40.1

#

interface Vlanif30

ip address 10.0.30.3 255.255.255.0

dhcp select relay

dhcp relay server-ip 10.0.40.1

#

interface Vlanif40

ip address 10.0.40.3 255.255.255.0

#

interface GigabitEthernet0/0/1

port link-type access

port default vlan 40

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 10 20 30

#

user-interface con 0

user-interface vty 0 4

#

return

Configuration on S4

#

sysname S4

#

vlan batch 10 20 30

#

dhcp enable

#

interface Vlanif10

ip address dhcp-alloc

#

interface Vlanif20

ip address dhcp-alloc

#

interface Vlanif30

ip address dhcp-alloc

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 10 20 30

#

Return

WLAN

Inter-AC Roaming on a Large-Scale WLAN

Introduction

Objectives

Upon completion of this task, you will be able to:

Implement inter-AC Layer 3 roaming by configuring a mobility group.

Describe how to configure APs to go online.

Know the basic WLAN configuration process.

Networking Topology

              - Inter-AC roaming on a large-scale WLAN

The preceding figure shows the device connection mode. AP1 is managed by AC1, and AP2 is managed by AC2. All APs use the direct forwarding mode.

S4 transparently transmits packets from AP2 at Layer 2. S3 serves as the gateway for APs and STAs.

S3 is enabled with DHCP to allocate IP addresses to AP1, AP2, and STAs associated with them. APs obtain ACs' addresses from Option 43 in DHCP packets.

Background

To meet the increasing STA access requirements, an enterprise needs to deploy a batch of APs. As AC1 is managing APs of its maximum specifications, the enterprise purchases a new AC (AC2) to manage the newly deployed APs. In addition, inter-AC roaming is required to minimize the service interruption time when STAs move between the coverage areas of APs managed by different ACs.

Data Planning

AC's data planning

| Item | Configuration |
| --- | --- |
| Management VLAN for APs | VLANs 10 and 20 |
| Service VLAN for STAs | VLANs 11 and 21 |
| DHCP server | S3 functions as a DHCP server to allocate IP addresses to APs and STAs. |
| IP address pool for APs | 10.0.10.0/24 and 10.0.20.0/24 |
| IP address pool for STAs | 10.0.11.0/24 and 10.0.21.0/24 |
| AC's source interface address | VLANIF 100 (10.0.100.254) and VLANIF 200 (10.0.200.254) |
| AP group | Name: ap-group1 and ap-group2 |
|  | Referenced profile: VAP profile departX |
| Regulatory domain profile | Name: default |
|  | Country code: CN |
| SSID profile | Name: departX |
|  | SSID name: roam |
| Security profile | Name: departX |
|  | Security policy: WPA-WPA2+PSK+AES |
|  | Password: huawei123 |
| VAP profile | Name: departX |
|  | Forwarding mode: direct forwarding |
|  | Service VLANs: VLANs 11 and 21 |
|  | Referenced profiles: SSID profile departX and security profile departX |

X in departX indicates the AC number, that is, depart1 on AC1 and depart2 on AC2.

Lab Configuration

Configuration Roadmap

- Shut down unnecessary ports and enable the PoE function on switches.

Configure wired-side functions so that S3 serves as the gateway for APs and STAs, and AC1 and AC2 communicate with S3 at Layer 3 through VLANIF interfaces.

Configure WLAN services on AC1 and enable AP1 to go online.

Configure WLAN services on AC2 and enable AP2 to go online.

Configure a mobility group on AC1 and AC2 to implement inter-AC roaming.

Configuration Procedure

Complete basic device configurations.

# Name the devices.

The configuration details are not provided.

# Shut down unnecessary ports.

The configuration details are not provided.

# Enable the PoE function on S3 and S4 ports connected to APs.

[S3]interface GigabitEthernet 0/0/4

[S3-GigabitEthernet0/0/4] poe enable

[S4]interface GigabitEthernet 0/0/4

[S4-GigabitEthernet0/0/4] poe enable

The poe enable command enables the PoE function on a port. When a port detects a powered device (PD) connected, the port supplies power to the PD. By default, the PoE function is enabled on a port. Therefore, this command is unnecessary and is provided for demonstration purpose only.

Configure the wired network.

Configure the wired network of the switches and ACs as planned.

# Create VLANs on S3, S4, AC1, and AC2, and assign ports to the VLANs.

[S3]vlan batch 10 11 20 21 100 200

[S3]interface GigabitEthernet0/0/1

[S3-GigabitEthernet0/0/1] port link-type trunk

[S3-GigabitEthernet0/0/1] port trunk allow-pass vlan 100

[S3-GigabitEthernet0/0/1] quit

[S3]interface GigabitEthernet0/0/2

[S3-GigabitEthernet0/0/2] port link-type trunk

[S3-GigabitEthernet0/0/2] port trunk allow-pass vlan 200

[S3-GigabitEthernet0/0/2] quit

[S3]interface GigabitEthernet0/0/3

[S3-GigabitEthernet0/0/3] port link-type trunk

[S3-GigabitEthernet0/0/3] port trunk allow-pass vlan 20 to 21

[S3-GigabitEthernet0/0/3] quit

[S3]interface GigabitEthernet0/0/4

[S3-GigabitEthernet0/0/4] port link-type trunk

[S3-GigabitEthernet0/0/4] port trunk pvid vlan 10

[S3-GigabitEthernet0/0/4] port trunk allow-pass vlan 10 to 11

[S3-GigabitEthernet0/0/4] quit

The PVID of the S3 port connected to AP1 is set to VLAN 10, and packets in the service VLAN and management VLAN from AP2 are allowed to pass on the S3 port connected to S4.

[S4]vlan batch 20 21

Info: This operation may take a few seconds. Please wait for a moment...done.

[S4]interface GigabitEthernet0/0/3

[S4-GigabitEthernet0/0/3] port link-type trunk

[S4-GigabitEthernet0/0/3] port trunk allow-pass vlan 20 to 21

[S4-GigabitEthernet0/0/3] quit

[S4]interface GigabitEthernet0/0/4

[S4-GigabitEthernet0/0/4] port link-type trunk

[S4-GigabitEthernet0/0/4] port trunk pvid vlan 20

[S4-GigabitEthernet0/0/4] port trunk allow-pass vlan 20 to 21

[S4-GigabitEthernet0/0/4] quit

The PVID of the S4 port connected to AP2 is set to VLAN 20, and the uplink port of S4 is configured to transparently transmit packets in VLANs 20 (management VLAN) and 21 (service VLAN).

[AC1]vlan batch 100

[AC1]interface GigabitEthernet0/0/12

[AC1-GigabitEthernet0/0/12] port link-type trunk

[AC1-GigabitEthernet0/0/12] port trunk allow-pass vlan 100

[AC1-GigabitEthernet0/0/12] quit

The interface is configured to allow packets in VLAN 100 to pass through because VLANIF 100 serves as the CAPWAP source interface of AC1.

[AC2]vlan batch 200

[AC2]interface GigabitEthernet0/0/13

[AC2-GigabitEthernet0/0/13] port link-type trunk

[AC2-GigabitEthernet0/0/13] port trunk allow-pass vlan 200

[AC2-GigabitEthernet0/0/13] quit

The interface is configured to allow packets in VLAN 200 to pass through because VLANIF 200 serves as the CAPWAP source interface of AC2.

# Create VLANIF interfaces on S3, AC1, and AC2.

[S3]interface Vlanif10

[S3-Vlanif10] description ap1_mgnt

[S3-Vlanif10] ip address 10.0.10.1 255.255.255.0

[S3-Vlanif10] quit

[S3]interface Vlanif11

[S3-Vlanif11] description ap1_service

[S3-Vlanif11] ip address 10.0.11.1 255.255.255.0

[S3-Vlanif11] quit

[S3]interface Vlanif20

[S3-Vlanif20] description ap2_mgnt

[S3-Vlanif20] ip address 10.0.20.1 255.255.255.0

[S3-Vlanif20] quit

[S3]interface Vlanif21

[S3-Vlanif21] description ap2_service

[S3-Vlanif21] ip address 10.0.21.1 255.255.255.0

[S3-Vlanif21] quit

[S3]interface Vlanif100

[S3-Vlanif100] description to_AC1

[S3-Vlanif100] ip address 10.0.100.1 255.255.255.0

[S3-Vlanif100] quit

[S3]interface Vlanif200

[S3-Vlanif200] description to_AC2

[S3-Vlanif200] ip address 10.0.200.1 255.255.255.0

[S3-Vlanif200] quit

On S3, VLANIF 10 and VLANIF 20 are configured as the management VLAN gateways of AP1 and AP2, respectively; VLANIF 11 and VLANIF 21 are configured as the service VLAN gateways of STAs connected to AP1 and AP2, respectively; and VLANIF 100 and VLANIF 200 are used for Layer 3 communication with AC1 and AC2, respectively.

[AC1]interface Vlanif100

[AC1-Vlanif100] description to_S3_CAPWAP

[AC1-Vlanif100] ip address 10.0.100.254 255.255.255.0

[AC1-Vlanif100] quit

VLANIF 100 is configured as the CAPWAP source interface of AC1.

[AC2]interface Vlanif200

[AC2-Vlanif200] description to_S3_CAPWAP

[AC2-Vlanif200] ip address 10.0.200.254 255.255.255.0

[AC2-Vlanif200] quit

VLANIF 200 is configured as the CAPWAP source interface of AC2.

# Configure routes to the AP management network segments on AC1 and AC2.

[AC1]ip route-static 10.0.10.0 255.255.255.0 10.0.100.1

[AC2]ip route-static 10.0.20.0 255.255.255.0 10.0.200.1

Static routes to the AP management network segments for communication with APs.

# Configure the DHCP service on S3.

[S3]dhcp enable

The DHCP service is enabled.

[S3]ip pool ap1

[S3-ip-pool-ap1] gateway-list 10.0.10.1

[S3-ip-pool-ap1] network 10.0.10.0 mask 255.255.255.0

[S3-ip-pool-ap1] option 43 sub-option 3 ascii 10.0.100.254

[S3-ip-pool-ap1] quit

[S3]ip pool ap2

[S3-ip-pool-ap2] gateway-list 10.0.20.1

[S3-ip-pool-ap2] network 10.0.20.0 mask 255.255.255.0

[S3-ip-pool-ap2] option 43 sub-option 3 ascii 10.0.200.254

[S3-ip-pool-ap2] quit

[S3]ip pool service_a

[S3-ip-pool-service_a] gateway-list 10.0.11.1

[S3-ip-pool-service_a] network 10.0.11.0 mask 255.255.255.0

[S3-ip-pool-service_a] dns-list 10.0.11.1

[S3-ip-pool-service_a] quit

[S3]ip pool service_b

[S3-ip-pool-service_b] gateway-list 10.0.21.1

[S3-ip-pool-service_b] network 10.0.21.0 mask 255.255.255.0

[S3-ip-pool-service_b] dns-list 10.0.21.1

[S3-ip-pool-service_b] quit

The address pools ap1 and ap2 are configured to allocate IP addresses to APs and carry Option 43 specifying the AC's IP address in DHCP packets. The address pools service_a and service_b are configured to allocate IP addresses to STAs on AP1 and AP2, respectively. The gateways for all address pools are set to the addresses of VLANIF interfaces on S3.

[S3]interface Vlanif10

[S3-Vlanif10] dhcp select global

[S3-Vlanif10] quit

[S3]interface Vlanif11

[S3-Vlanif11] dhcp select global

[S3-Vlanif11] quit

[S3]interface Vlanif20

[S3-Vlanif20] dhcp select global

[S3-Vlanif20] quit

[S3]interface Vlanif21

[S3-Vlanif21] dhcp select global

[S3-Vlanif21] quit

The global address pool is selected on the VLANIF interfaces.

Configure AC1.

On AC1, specify VLANIF 100 as the CAPWAP source interface, create the AP group depart1, configure MAC address authentication for APs, name the AP ap1, add it to the AP group depart1, associate parameter profiles with the VAP profile depart1, and bind the VAP profile to the AP group depart1.

# Specify the CAPWAP source interface.

[AC1]capwap source interface vlanif100

# Create an AP group named depart1.

[AC1]wlan

[AC1-wlan-view] ap-group name depart1

[AC1-wlan-ap-group-depart1] quit

# Create a regulatory domain profile and configure the AC's country code in the profile.

[AC1]wlan

[AC1-wlan-view] regulatory-domain-profile name default

[AC1-wlan-regulate-domain-default] country-code cn

Info: The current country code is same with the input country code.

[AC1-wlan-regulate-domain-default] quit

A regulatory domain profile provides configurations of the country code, calibration channel set, and calibration bandwidth for an AP.

By default, the system provides the regulatory domain profile default. Therefore, the default regulatory domain profile is displayed.

A country code identifies the country where AP radios work. Different countries require different AP radio attributes, including the transmit power and supported channels. The correct country code configuration ensures that radio attributes of APs comply with local laws and regulations of countries and regions to which the APs are delivered. By default, the country code CN is configured.

# Bind the regulatory domain profile to the AP group.

[AC1]wlan

[AC1-wlan-view]ap-group name depart1

[AC1-wlan-ap-group- depart1]regulatory-domain-profile default

Warning: Modifying the country code will clear channel, power and antenna gain configurations of the radio and reset the AP. Continue?[Y/N]:y

[AC1-wlan-ap-group- depart1]quit

In the AP group view, the regulatory-domain-profile command binds a regulatory domain profile to an AP or AP group. By default, the regulatory domain profile default is bound to an AP group, but no regulatory domain profile is bound to an AP. In the regulatory domain profile default, the country code is CN, 2.4G calibration channels include channels 1, 6, and 11, and 5G calibration channels include channels 149, 153, 157, 161, and 165. Therefore, you can skip this step and the previous step in actual operations.

# Add an AP.

[AC1]wlan

[AC1-wlan-view] ap auth-mode mac-auth

[AC1-wlan-view] ap-id 0 ap-mac 00e0-fc6e-2890                //Set the AP's MAC address as required in the lab environment.

[AC1-wlan-ap-0] ap-name ap1

[AC1-wlan-ap-0] ap-group depart1

Three AP authentication modes are supported. By default, MAC address authentication is used. The AP is added on the AC before it goes online, named ap1, and added to the AP group depart1.

# Configure parameter profiles.

[AC1]wlan

[AC1-wlan-view] security-profile name depart1

[AC1-wlan-sec-prof-depart1] security wpa2 psk pass-phrase huawei123 aes

[AC1-wlan-sec-prof- depart1] quit

[AC1-wlan-view] ssid-profile name depart1

[AC1-wlan-ssid-prof-depart1] ssid roam

[AC1-wlan-ssid-prof-depart1] quit

[AC1-wlan-view] vap-profile name depart1

[AC1-wlan-vap-prof-depart1] forward-mode direct-forward

[AC1-wlan-vap-prof-depart1] service-vlan vlan-id 11

[AC1-wlan-vap-prof-depart1] ssid-profile depart1

[AC1-wlan-vap-prof-depart1] security-profile depart1

[AC1-wlan-vap-prof-depart1] quit

[AC1-wlan-view] ap-group name depart1

[AC1-wlan-ap-group-depart1] vap-profile depart1 wlan 1 radio all

[AC1-wlan-ap-group-depart1] quit

The security profile depart1 is configured, with the authentication mode of WPA2-PSK and the pre-shared key of huawei123.

The SSID profile depart1 is configured, and the SSID is set to roam.

The VAP profile depart1 is configured, with the direct forwarding mode and the service VLAN 11, and has the SSID profile depart1 and security profile depart1 bound.

The VAP profile depart1 is bound to the AP group depart1.

# Check the AP online status.

<AC1>display	ap	all

Info:	This	operation	may	take	a	few	seconds.	Please	wait	for	a	moment.done.

Total	AP	information:

nor		:	normal										[1]

--------------------------------------------------------------------------------------------

ID			MAC												Name	 		Group			IP														Type												State			STA			Uptime

--------------------------------------------------------------------------------------------

0				00e0-fc6e-2890				ap1		  depart1	  10.0.10.254			  AirEngine5760-10			nor					0						50S

--------------------------------------------------------------------------------------------

Total:	1

Wait for a period of time and check the AP online status. If the AP status is nor, the AP goes online successfully and works properly. The AP has obtained the IP address 10.0.10.254 and has no STA associated.

Configure AC2.

On AC2, specify VLANIF 200 as the CAPWAP source interface, create the AP group depart2, configure MAC address authentication for APs, name the AP ap2, add it to the AP group depart2, associate parameter profiles with the VAP profile depart2, and bind the VAP profile to the AP group depart2.

# Specify the CAPWAP source interface.

[AC2]capwap source interface vlanif200

# Create an AP group named depart2.

[AC2]wlan

[AC2-wlan-view] ap-group name depart2

[AC2-wlan-ap-group-depart2] quit

# Create a regulatory domain profile and configure the AC's country code in the profile.

[AC2]wlan

[AC2-wlan-view] regulatory-domain-profile name default

[AC2-wlan-regulate-domain-default] country-code cn

Info: The current country code is same with the input country code.

[AC2-wlan-regulate-domain-default] quit

# Bind the regulatory domain profile to the AP group.

[AC2]wlan

[AC2-wlan-view] ap-group name depart2

[AC2-wlan-ap-group- depart2] regulatory-domain-profile default

Warning: Modifying the country code will clear channel, power and antenna gain configurations of the radio and reset the AP. Continue?[Y/N]:y

[AC2-wlan-ap-group- depart2] quit

# Add an AP.

[AC2]wlan

[AC2-wlan-view] ap auth-mode mac-auth

[AC2-wlan-view] ap-id 0 ap-mac 00e0-fcde-1990                //Set the AP's MAC address as required in the lab environment.
[AC2-wlan-ap-0] ap-name ap2

[AC2-wlan-ap-0] ap-group depart2

# Configure parameter profiles.

[AC2]wlan

[AC2-wlan-view] security-profile name depart2

[AC2-wlan-sec-prof-depart2] security wpa2 psk pass-phrase huawei123 aes

[AC2-wlan-sec-prof- depart2] quit

[AC2-wlan-view] ssid-profile name depart2

[AC2-wlan-ssid-prof-depart2] ssid roam

[AC2-wlan-ssid-prof-depart2] quit

[AC2-wlan-view] vap-profile name depart2

[AC2-wlan-vap-prof-depart2] forward-mode direct-forward

[AC2-wlan-vap-prof-depart2] service-vlan vlan-id 21

[AC2-wlan-vap-prof-depart2] ssid-profile depart2

[AC2-wlan-vap-prof-depart2] security-profile depart2

[AC2-wlan-vap-prof-depart2] quit

[AC2-wlan-view] ap-group name depart2

[AC2-wlan-ap-group-depart2] vap-profile depart2 wlan 1 radio all

[AC2-wlan-ap-group-depart2] quit

The security profile depart2 is configured, with the authentication mode of WPA2-PSK and the pre-shared key of huawei123.

The SSID profile depart2 is configured, and the SSID is set to roam.

The VAP profile depart2 is configured, with the direct forwarding mode and the service VLAN 21, and has the SSID profile depart2 and security profile depart1 bound.

The VAP profile depart2 is bound to the AP group depart2.

# Check the AP online status.

<AC2>display	ap	all

Info:	This	operation	may	take	a	few	seconds.	Please	wait	for	a	moment.done.

Total	AP	information:

nor		:	normal										[1]

--------------------------------------------------------------------------------------------

ID			MAC												Name	 		Group			IP													Type													State			STA			Uptime

--------------------------------------------------------------------------------------------

0				00e0-fcde-1990				ap2		  depart2  	10.0.20.254			  AirEngine5760-10			nor					0						1M:13S

--------------------------------------------------------------------------------------------

Total:	1

Wait for a period of time and check the AP online status. If the AP status is nor, the AP goes online successfully and works properly. The AP has obtained the IP address 10.0.20.254 and has no STA associated.

Configure Layer 3 roaming.

Configure static routes between the CAPWAP source interfaces on AC1 and AC2. Create the mobility group mobility and add AC1 and AC2 to the mobility group, without specifying a mobility server.

#Configure static routes.

[AC1]ip route-static 10.0.200.0 255.255.255.0 10.0.100.1

[AC2]ip route-static 10.0.100.0 255.255.255.0 10.0.200.1

# Configure AC1.

[AC1]wlan

[AC1 -wlan-view] mobility-group name mobility

[AC1-mc-mg-mobility] member ip-address 10.0.100.254

[AC1-mc-mg-mobility] member ip-address 10.0.200.254

The mobility group mobility is created on AC1, and AC1 and AC2 are added to the mobility group as members.

# Configure AC2.

[AC2]wlan

[AC2 -wlan-view] mobility-group name mobility

[AC2-mc-mg-mobility] member ip-address 10.0.100.254

[AC2-mc-mg-mobility] member ip-address 10.0.200.254

The mobility group mobility is created on AC2, and AC1 and AC2 are added to the mobility group as members.

# Check the mobility group status.

[AC1]display mobility-group name mobility

--------------------------------------------------------------------------------

State        IP address                       Description

--------------------------------------------------------------------------------

normal       10.0.100.254                     -

normal       10.0.200.254                     -

--------------------------------------------------------------------------------

Total: 2

Members AC1 and AC2 in the mobility group are both in normal state.

# Check STA information on AC1.

[AC1]display station ssid roam

Rf/WLAN: Radio ID/WLAN ID

Rx/Tx: link receive rate/link transmit rate(Mbps)

-----------------------------------------------------------------------------------------

STA MAC          			AP ID Ap name  Rf/WLAN Band  Type  Rx/Tx      RSSI  VLAN  IP address

-----------------------------------------------------------------------------------------

5489-986f-73ad        0     ap1      0/1      2.4G  	-     -/-          -     11    10.0.11.254

-----------------------------------------------------------------------------------------

Total: 1 2.4G: 1 5G: 0

After detecting the WLAN with the SSID roam in the coverage area of AP1, the STA is associated with the WLAN through the password huawei123. After the display station ssid roam command is run on AC1 to check STA access information, the command output shows that the STA (MAC address: 5489-986f-73) is associated with AP1. 
Move the STA to the coverage area of AP2 while still associating with AP1 and then power off AP1 to enable the STA to roam to AP2.

# Check the inter-AC roaming track.

<AC2>display	station	roam-track	sta-mac	5489-986f-73ad

Access	SSID:roam

Rx/Tx:	link	receive	rate/link	transmit	rate(Mbps)

z:	Zero	Roam	c:PMK	Cache	Roam	r:802.11r	Roam

------------------------------------------------------------------

L2/L3												AC	IP																			AP	name											Radio	ID

BSSID											TIME																			In	Rx/Tx		RSSI							Out	Rx/Tx			RSSI

------------------------------------------------------------------

10.0.100.254														ap1																		1

00e0-fc6e-2890			2020/06/08	07:27:06								130/130		-44									130/130					-44

L3														10.0.200.254														ap2																		1

00e0-fcde-1990			2020/06/08	07:27:24								130/6				-42										-/-

------------------------------------------------------------------

Number	of	roam	track:	1

The STA with the MAC address 5489-986f-73ad has roamed from AP1 to AP2, which is an inter-AC roaming process.

----End

Quiz

What are the differences in forwarding between inter-AC Layer 2 roaming and inter-AC Layer 3 roaming?

Configuration Reference

Configuration on S3

#

sysname S3

#

vlan batch 10 to 11 20 to 21 100 200

#

dhcp enable

#

ip pool ap1

gateway-list 10.0.10.1

network 10.0.10.0 mask 255.255.255.0

option 43 sub-option 3 ascii 10.0.100.254

#

ip pool ap2

gateway-list 10.0.20.1

network 10.0.20.0 mask 255.255.255.0

option 43 sub-option 3 ascii 10.0.200.254

#

ip pool service_a

gateway-list 10.0.11.1

network 10.0.11.0 mask 255.255.255.0

dns-list 10.0.11.1

#

ip pool service_b

gateway-list 10.0.21.1

network 10.0.21.0 mask 255.255.255.0

dns-list 10.0.21.1

#

interface Vlanif10

description ap1_mgnt

ip address 10.0.10.1 255.255.255.0

dhcp select global

#

interface Vlanif11

description ap1_service

ip address 10.0.11.1 255.255.255.0

dhcp select global

#

interface Vlanif20

description ap2_mgnt

ip address 10.0.20.1 255.255.255.0

dhcp select global

#

interface Vlanif21

description ap2_service

ip address 10.0.21.1 255.255.255.0

dhcp select global

#

interface Vlanif100

description to_AC1

ip address 10.0.100.1 255.255.255.0

#

interface Vlanif200

description to_AC2

ip address 10.0.200.1 255.255.255.0

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 200

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 20 to 21

#

interface GigabitEthernet0/0/4

port link-type trunk

port trunk pvid vlan 10

port trunk allow-pass vlan 10 to 11

#

return

Configuration on S4

#

sysname S4

#

vlan batch 20 to 21

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 20 to 21

#

interface GigabitEthernet0/0/4

port link-type trunk

port trunk pvid vlan 20

port trunk allow-pass vlan 20 to 21

#

return

Configuration on AC1

#

sysname AC1

#

vlan batch 100

#

interface Vlanif100

description to_S3_CAPWAP

ip address 10.0.100.254 255.255.255.0

#

interface GigabitEthernet0/0/12

port link-type trunk

port trunk allow-pass vlan 100

#

ip route-static 10.0.10.0 255.255.255.0 10.0.100.1

ip route-static 10.0.200.0 255.255.255.0 10.0.100.1

#

capwap source interface vlanif100

#

wlan

security-profile name depart1

security wpa2 psk pass-phrase huawei123 aes

aes

ssid-profile name depart1

ssid roam

vap-profile name depart1

service-vlan vlan-id 11

ssid-profile depart1

security-profile depart1

regulatory-domain-profile name default

mobility-group name mobility

member ip-address 10.0.100.254

member ip-address 10.0.200.254

ap-group name depart1

radio 0

vap-profile depart1 wlan 1

radio 1

vap-profile depart1 wlan 1

radio 2

vap-profile depart1 wlan 1

ap-id 0 type-id 56 ap-mac 00e0-fc6e-2890 ap-sn 210235448310F30CF56D

ap-name ap1

ap-group depart1

provision-ap

#

return

Configuration on AC2

#

sysname AC2

#

vlan batch 200

#

interface Vlanif200

description to_S3_CAPWAP

ip address 10.0.200.254 255.255.255.0

#

interface GigabitEthernet0/0/13

port link-type trunk

port trunk allow-pass vlan 200

#

ip route-static 10.0.20.0 255.255.255.0 10.0.200.1

ip route-static 10.0.100.0 255.255.255.0 10.0.200.1

#

capwap source interface vlanif200

#

wlan

security-profile name depart2

security wpa-wpa2 psk pass-phrase huawei123 aes

ssid-profile name default

ssid-profile name depart2

ssid roam

vap-profile name depart2

service-vlan vlan-id 21

ssid-profile depart2

security-profile depart2

regulatory-domain-profile name default

mobility-group name mobility

member ip-address 10.0.100.254

member ip-address 10.0.200.254

ap-group name depart2

radio 0

vap-profile depart2 wlan 1

radio 1

vap-profile depart2 wlan 1

radio 2

vap-profile depart2 wlan 1

ap-id 0 type-id 56 ap-mac 00e0-fcde-1990 ap-sn 210235448310FA145341

ap-name ap2

ap-group depart2

provision-ap

#

Return

VRRP HSB Configuration

Introduction

Objectives

Upon completion of this task, you will be able to:

Improve AC reliability through VRRP HSB.

Configure VRRP HSB.

Describe the technical implementation of VRRP HSB.

Networking Topology

              - VRRP HSB configuration

As shown in the figure, AC1 and AC2 form an HSB group, and VRRP HSB is configured for AC backup. AP1 and AP2 are managed by AC1 and AC2 in active/standby mode, and both use the direct forwarding mode.

S4 transparently transmits packets from AP2 at Layer 2. S3 serves as the gateway for APs and STAs.

S3 is enabled with DHCP to allocate IP addresses to AP1, AP2, and STAs associated with them. APs obtain the ACs' virtual IP address from Option 43 in DHCP packets.

Background

As the number of STAs on the enterprise intranet keeps increasing, to ensure the stability of wireless services, you, a network engineer, decide to purchase a new AC (AC2) to form an HSB group with the existing AC (AC1) and configure the HSB group to work with VRRP to implement AC HSB and improve the reliability of wireless services.

Data Planning

AC's data planning

| Item | Configuration |
| --- | --- |
| Management VLAN for APs | VLAN 10 |
| Service VLAN for STAs | VLAN 11 |
| DHCP server | S3 functions as a DHCP server to allocate IP addresses to APs and STAs. |
| IP address pool for APs | 10.0.10.0/24 |
| IP address pool for STAs | 10.0.11.0/24 |
| AC's source interface address | 10.0.100.254 (VRRP virtual address) |
| AP group | Name: depart |
|  | Referenced profile: VAP profile depart |
| Regulatory domain profile | Name: default |
|  | Country code: CN |
| SSID profile | Name: depart |
|  | SSID name: HSB |
| Security profile | Name: depart |
|  | Security policy: WPA-WPA2+PSK+AES |
|  | Password: huawei123 |
| VAP profile | Name: depart |
|  | Forwarding mode: direct forwarding |
|  | Service VLAN: VLAN 11 |
|  | Referenced profiles: SSID profile depart and security profile depart |
| VRRP group | VRRP group ID: 1 |
|  | Virtual IP address: 10.0.100.254 |
| HSB | IP address and port number of the HSB channel for AC1: 10.0.100.1 and 10241 |
|  | IP address and port number of the HSB channel for AC2: 10.0.100.2 and 10241 |

Lab Configuration

Configuration Roadmap

- Shut down unnecessary ports and enable the PoE function on switches.

Configure wired-side functions so that S3 serves as the gateway for APs and STAs, and AC1 and AC2 communicate with S3 at Layer 3 through VLANIF 100.

Configure WLAN services on AC1 and AC2. Note that you do not need to configure the CAPWAP source address now. Perform this configuration after HSB and VRRP configurations are completed.

Configure HSB on AC1, specify AC1 as the master device in VRRP group 1 and the active device in HSB mode, and set the CAPWAP source address to the VRRP virtual IP address.

Configure HSB on AC1, specify AC1 as the standby device in HSB mode, and set the CAPWAP source address to the VRRP virtual IP address.

Verify the VRRP HSB configuration. Shut down the downlink interface on AC1, and check the states of APs and STAs on AC2.

Configuration Procedure

Complete basic device configurations.

# Name the devices.

The configuration details are not provided.

# Shut down unnecessary ports.

The configuration details are not provided.

# Enable the PoE function on S3 and S4 ports connected to APs.

[S3]interface GigabitEthernet 0/0/4

[S3-GigabitEthernet0/0/4] poe enable

[S4]interface GigabitEthernet 0/0/4

[S4-GigabitEthernet0/0/4] poe enable

The poe enable command enables the PoE function on a port. When a port detects a PD connected, the port supplies power to the PD. By default, the PoE function is enabled on a port. Therefore, this command is unnecessary and is provided for demonstration purpose only.

Configure the wired network.

Configure the wired network of the switches and ACs as planned.

# Create VLANs on S3, S4, AC1, and AC2, and assign ports to the VLANs.

[S3]vlan batch 10 11 100

[S3]interface GigabitEthernet0/0/1

[S3-GigabitEthernet0/0/1] port link-type trunk

[S3-GigabitEthernet0/0/1] port trunk allow-pass vlan 100

[S3-GigabitEthernet0/0/1] quit

[S3]interface GigabitEthernet0/0/2

[S3-GigabitEthernet0/0/2] port link-type trunk

[S3-GigabitEthernet0/0/2] port trunk allow-pass vlan 100

[S3-GigabitEthernet0/0/2] quit

[S3]interface GigabitEthernet0/0/3

[S3-GigabitEthernet0/0/3] port link-type trunk

[S3-GigabitEthernet0/0/3] port trunk allow-pass vlan 10 to 11

[S3-GigabitEthernet0/0/3] quit

[S3]interface GigabitEthernet0/0/4

[S3-GigabitEthernet0/0/4] port link-type trunk

[S3-GigabitEthernet0/0/4] port trunk pvid vlan 10

[S3-GigabitEthernet0/0/4] port trunk allow-pass vlan 10 to 11

[S3-GigabitEthernet0/0/4] quit

The PVID of the S3 port connected to AP1 is set to VLAN 10, packets in the service VLANs and management VLANs are allowed to pass on the S3 port connected to S4, and the S3 ports connected to ACs are configured to allow packets in VLAN 100 to pass through.

[S4]vlan batch 10 11

Info: This operation may take a few seconds. Please wait for a moment...done.

[S4]interface GigabitEthernet0/0/3

[S4-GigabitEthernet0/0/3] port link-type trunk

[S4-GigabitEthernet0/0/3] port trunk allow-pass vlan 10 to 11

[S4-GigabitEthernet0/0/3] quit

[S4]interface GigabitEthernet0/0/4

[S4-GigabitEthernet0/0/4] port link-type trunk

[S4-GigabitEthernet0/0/4] port trunk pvid vlan 10

[S4-GigabitEthernet0/0/4] port trunk allow-pass vlan 10 to 11

[S4-GigabitEthernet0/0/4] quit

The PVID of the S4 port connected to AP2 is set to VLAN 10, and the uplink port of S4 is configured to transparently transmit packets in VLANs 10 (management VLAN) and 11 (service VLAN).

[AC1]vlan batch 100

[AC1]interface GigabitEthernet0/0/12

[AC1-GigabitEthernet0/0/12] port link-type trunk

[AC1-GigabitEthernet0/0/12] port trunk allow-pass vlan 100

[AC1-GigabitEthernet0/0/12] quit

The interface is configured to allow packets in VLAN 100 to pass through.

[AC2]vlan batch 100

[AC2]interface GigabitEthernet0/0/13

[AC2-GigabitEthernet0/0/13] port link-type trunk

[AC2-GigabitEthernet0/0/13] port trunk allow-pass vlan 100

[AC2-GigabitEthernet0/0/13] quit

The interface is configured to allow packets in VLAN 100 to pass through.

# Create VLANIF interfaces on S3, AC1, and AC2.

[S3]interface Vlanif10

[S3-Vlanif10] description ap_mgnt

[S3-Vlanif10] ip address 10.0.10.1 255.255.255.0

[S3-Vlanif10] quit

[S3]interface Vlanif11

[S3-Vlanif11] description ap_service

[S3-Vlanif11] ip address 10.0.11.1 255.255.255.0

[S3-Vlanif11] quit

[S3]interface Vlanif100

[S3-Vlanif100] description to_AC

[S3-Vlanif100] ip address 10.0.100.3 255.255.255.0

[S3-Vlanif100] quit

On S3, VLANIF 10 is configured as the management VLAN gateway of AP1 and AP2; VLANIF 11 is configured as the service VLAN gateway of STAs connected to AP1 and AP2; and VLANIF 100 is used for Layer 3 communication with AC1 and AC2.

[AC1]interface Vlanif100

[AC1-Vlanif100] description to_S3_CAPWAP

[AC1-Vlanif100] ip address 10.0.100.1 255.255.255.0

[AC1-Vlanif100] quit

VLANIF 100 of AC1 is configured as the CAPWAP communication interface (instead of the CAPWAP source interface).

[AC2]interface Vlanif100

[AC2-Vlanif100] description to_S3_CAPWAP

[AC2-Vlanif100] ip address 10.0.100.2 255.255.255.0

[AC2-Vlanif100] quit

VLANIF 100 of AC2 is configured as the CAPWAP communication interface (instead of the CAPWAP source interface).

# Configure routes to the AP management network segments on AC1 and AC2.

[AC1]ip route-static 10.0.10.0 255.255.255.0 10.0.100.3

[AC2]ip route-static 10.0.10.0 255.255.255.0 10.0.100.3

Static routes to the AP management network segments are configured on ACs for CAPWAP communication with APs.

# Configure the DHCP service on S3.

[S3]dhcp enable

The DHCP service is enabled.

[S3]ip pool ap

[S3-ip-pool-ap] gateway-list 10.0.10.1

[S3-ip-pool-ap] network 10.0.10.0 mask 255.255.255.0

[S3-ip-pool-ap] option 43 sub-option 3 ascii 10.0.100.254

[S3-ip-pool-ap] quit

[S3]ip pool service

[S3-ip-pool-service] gateway-list 10.0.11.1

[S3-ip-pool-service] network 10.0.11.0 mask 255.255.255.0

[S3-ip-pool-service] dns-list 10.0.11.1

[S3-ip-pool-service] quit

The address pool ap is configured to allocate IP addresses to APs. Option 43 is configured to specify the AC's IP address (VRRP virtual IP address).

The address pool service is configured to allocate IP addresses to STAs connected to AP1 and AP2. VLANIF interfaces on S3 are configured as the gateways for all address pools.

[S3]interface Vlanif10

[S3-Vlanif10] dhcp select global

[S3-Vlanif10] quit

[S3]interface Vlanif11

[S3-Vlanif11] dhcp select global

[S3-Vlanif11] quit

The global address pool is selected on the VLANIF interfaces.

Configure ACs.

Create the AP group depart, configure MAC address authentication for APs, name the APs ap1 and ap2, add them to the AP group depart, and associate parameter profiles with the VAP profile depart.

The WLAN configurations on AC1 and AC2 are the same. The following uses AC1 as an example.

# Create an AP group named depart.

[AC1]wlan

[AC1-wlan-view] ap-group name depart

[AC1-wlan-ap-group-depart] quit

# Create a regulatory domain profile and configure the AC's country code in the profile.

[AC1]wlan

[AC1-wlan-view] regulatory-domain-profile name default

[AC1-wlan-regulate-domain-default] country-code cn

Info: The current country code is same with the input country code.

[AC1-wlan-regulate-domain-default] quit

A regulatory domain profile provides configurations of the country code, calibration channel set, and calibration bandwidth for an AP.

By default, the system provides the regulatory domain profile default. Therefore, the default regulatory domain profile is displayed.

A country code identifies the country where AP radios work. Different countries require different AP radio attributes, including the transmit power and supported channels. The correct country code configuration ensures that radio attributes of APs comply with local laws and regulations of countries and regions to which the APs are delivered. By default, the country code CN is configured.

# Bind the regulatory domain profile to the AP group.

[AC1]wlan

[AC1-wlan-view]ap-group name depart

[AC1-wlan-ap-group- depart]regulatory-domain-profile default

Warning: Modifying the country code will clear channel, power and antenna gain configurations of the radio and reset the AP. Continue?[Y/N]:y

[AC1-wlan-ap-group- depart]quit

In the AP group view, the regulatory-domain-profile command binds a regulatory domain profile to an AP or AP group. By default, the regulatory domain profile default is bound to an AP group, but no regulatory domain profile is bound to an AP. In the regulatory domain profile default, the country code is CN, 2.4G calibration channels include channels 1, 6, and 11, and 5G calibration channels include channels 149, 153, 157, 161, and 165. Therefore, you can skip this step and the previous step in actual operations.

# Add APs.

[AC1]wlan

[AC1-wlan-view] ap auth-mode mac-auth

[AC1-wlan-view] ap-id 0 ap-mac 00e0-fc6e-2890         //Set the AP's MAC address as required in the lab environment.

[AC1-wlan-ap-0] ap-name ap1

[AC1-wlan-ap-0] ap-group depart

[AC1-wlan-ap-0] quit

[AC1-wlan-view] ap-id 1 ap-mac 00e0-fcde-1990        //Set the AP's MAC address as required in the lab environment.

[AC1-wlan-ap-1] ap-name ap2

[AC1-wlan-ap-1] ap-group depart

[AC1-wlan-ap-1] quit

Three AP authentication modes are supported. By default, MAC address authentication is used. APs are added on the AC before they go online, named ap1 and ap2, and added to the AP group depart.

# Configure parameter profiles.

[AC1]wlan

[AC1-wlan-view] security-profile name depart

[AC1-wlan-sec-prof-depart] security wpa2 psk pass-phrase huawei123 aes

[AC1-wlan-sec-prof- depart] quit

[AC1-wlan-view] ssid-profile name depart

[AC1-wlan-ssid-prof-depart] ssid HSB

[AC1-wlan-ssid-prof-depart] quit

[AC1-wlan-view] vap-profile name depart

[AC1-wlan-vap-prof-depart] forward-mode direct-forward

[AC1-wlan-vap-prof-depart] service-vlan vlan-id 11

[AC1-wlan-vap-prof-depart] ssid-profile depart

[AC1-wlan-vap-prof-depart] security-profile depart

[AC1-wlan-vap-prof-depart] quit

[AC1-wlan-view] ap-group name depart

[AC1-wlan-ap-group-depart] vap-profile depart wlan 1 radio all

[AC1-wlan-ap-group-depart] quit

The security profile depart is configured, with the authentication mode of WPA2-PSK and the pre-shared key of huawei123.

The SSID profile depart is configured, and the SSID is set to HSB.

The VAP profile depart is configured, with the direct forwarding mode and the service VLAN 11, and has the SSID profile depart and security profile depart bound.

The VAP profile depart is bound to the AP group depart.

Configure VRRP HSB on AC1.

Configure AC1 as the master in VRRP group 1. Configure the hot standby (HSB) function on AC1 so that service information on AC1 is backed up to AC2 in real time or in batches, ensuring seamless service switchover.

# Set the recovery delay of the VRRP group to 60 seconds.

[AC1] vrrp recover-delay 60

# Create a management VRRP group on AC1. Set the VRRP priority of AC1 to 120 and the preemption delay to 1800 seconds.

[AC1] interface vlanif 100

[AC1-Vlanif100] vrrp vrid 1 virtual-ip 10.0.100.254

[AC1-Vlanif100] vrrp vrid 1 priority 120

[AC1-Vlanif100] vrrp vrid 1 preempt-mode timer delay 1800

[AC1-Vlanif100] admin-vrrp vrid 1

[AC1-Vlanif100] quit

The VRRP priority of AC1 is adjusted, and AC1 is specified as the master device in VRRP group 1.

# Create HSB service 0 on AC1, and configure the IP addresses and port numbers for establishing an HSB channel. Set the retransmission time and interval of HSB packets.

[AC1] hsb-service 0

[AC1-hsb-service-0] service-ip-port local-ip 10.0.100.1 peer-ip 10.0.100.2 local-data-port 10241 peer-data-port 10241

[AC1-hsb-service-0] service-keep-alive detect retransmit 3 interval 6

[AC1-hsb-service-0] quit

# Create HSB group 0 on AC1, and bind HSB service 0 and the management VRRP group to the HSB group.

[AC1] hsb-group 0

[AC1-hsb-group-0] bind-service 0

[AC1-hsb-group-0] track vrrp vrid 1 interface vlanif 100

[AC1-hsb-group-0] quit

# Bind the NAC service to the HSB group.

[AC1] hsb-service-type access-user hsb-group 0

# Bind the WLAN service to the HSB group.

[AC1] hsb-service-type ap hsb-group 0

# Enable the HSB function.

[AC1]hsb-group 0

[AC1-hsb-group-0] hsb enable

[AC1-hsb-group-0] quit

# Configure the CAPWAP source address of AC1.

[AC1]undo capwap source ip-address

[AC1]capwap source ip-address 10.0.100.254

Note that this IP address must be set to the VRRP virtual IP address.

Configure VRRP HSB on AC2.

Configure AC2 as the backup in VRRP group 1. Configure the HSB function on AC2 to back up service information from AC1, ensuring seamless service switchover.

# Set the recovery delay of the VRRP group to 60 seconds.

[AC2] vrrp recover-delay 60

# Create a management VRRP group on AC2 and set the preemption delay to 1800 seconds.

[AC2] interface vlanif 100

[AC2-Vlanif100] vrrp vrid 1 virtual-ip 10.0.100.254

[AC2-Vlanif100] vrrp vrid 1 preempt-mode timer delay 1800

[AC2-Vlanif100] admin-vrrp vrid 1

[AC2-Vlanif100] quit

# Create HSB service 0 on AC2, and configure the IP addresses and port numbers for establishing an HSB channel. Set the retransmission time and interval of HSB packets.

[AC2] hsb-service 0

[AC2-hsb-service-0] service-ip-port local-ip 10.0.100.2 peer-ip 10.0.100.1 local-data-port 10241 peer-data-port 10241

[AC2-hsb-service-0] service-keep-alive detect retransmit 3 interval 6

[AC2-hsb-service-0] quit

# Create HSB group 0 on AC2, and bind HSB service 0 and the management VRRP group to the HSB group.

[AC2] hsb-group 0

[AC2-hsb-group-0] bind-service 0

[AC2-hsb-group-0] track vrrp vrid 1 interface vlanif 100

[AC2-hsb-group-0] quit

# Bind the NAC service to the HSB group.

[AC2] hsb-service-type access-user hsb-group 0

# Bind the WLAN service to the HSB group.

[AC2] hsb-service-type ap hsb-group 0

# Enable the HSB function.

[AC2]hsb-group 0

[AC2-hsb-group-0] hsb enable

[AC2-hsb-group-0] quit

# Configure the CAPWAP source address of AC2.

[AC2]undo capwap source ip-address

[AC2]capwap source ip-address 10.0.100.254

Note that this IP address must be set to the VRRP virtual IP address.

Verify the configuration.

# Check the VRRP status on AC1 and AC2. The State field of AC1 is Master and that of AC2 is Backup.

<AC1>display vrrp

Vlanif100 | Virtual Router 1

State	:	Master

Virtual	IP						:	10.0.100.254

Master	IP						:	10.0.100.1

PriorityRun					:	120

PriorityConfig		:	120

MasterPriority		:	120

Preempt							:	YES			Delay	Time	:	1800	s

TimerRun						:	1	s

TimerConfig				:	1	s

Auth	type						:	NONE

Virtual	MAC				:	0000-5e00-0101

Check	TTL						:	YES

Config	type				:	admin-vrrp

Backup-forward	:	disabled

Create	time				:	2020-06-12	08:26:33	UTC-05:13

Last	change	time	:	2020-06-12	08:26:36	UTC-05:13

<AC2>display vrrp

Vlanif100 | Virtual Router 1

State : Backup

Virtual	IP						:	10.0.100.254

Master	IP						:	10.0.100.1

PriorityRun					:	100

PriorityConfig		:	100

MasterPriority		:	120

Preempt							:	YES			Delay	Time	:	0	s

TimerRun						:	1	s

TimerConfig				:	1	s

Auth	type						:	NONE

Virtual	MAC				:	0000-5e00-0101

Check	TTL						:	YES

Config	type				:	admin-vrrp

Backup-forward	:	disabled

Create	time				:	2020-06-12	08:41:15	UTC-05:13

Last	change	time	:	2020-06-12	08:41:15	UTC-05:13

AC1 is the owner of the virtual IP address 10.0.100.254.

# Run the display hsb-service 0 command on AC1 and AC2 to check the status of the HSB service.

<AC1>display hsb-service 0

Hot Standby Service Information:

----------------------------------------------------------

Local	IP	Address						:	10.0.100.1

Peer	IP	Address							:	10.0.100.2

Source	Port										:	10241

Destination	Port						:	10241

Keep	Alive	Times							:	3

Keep	Alive	Interval					:	6

Service	State									:	Connected

Service	Batch	Modules		:

----------------------------------------------------------

<AC2>display	hsb-service	0

Hot	Standby	Service	Information:

----------------------------------------------------------

Local	IP	Address						:	10.0.100.2

Peer	IP	Address							:	10.0.100.1

Source	Port										:	10241

Destination	Port						:	10241

Keep	Alive	Times							:	3

Keep	Alive	Interval					:	6

Service	State									:	Connected

Service	Batch	Modules		:

----------------------------------------------------------

The value of the Service State field is Connected, indicating that the HSB channel has been established.

# Run the display hsb-group 0 command on AC1 and AC2 to check the running status of the HSB group.

[AC1] display hsb-group 0

Hot Standby Group Information:

----------------------------------------------------------

HSB-group	ID														:	0

Vrrp	Group	ID													:	1

Vrrp	Interface													:	Vlanif100

Service	Index														:	0

Group	Vrrp	Status									:	Master

Group	Status													:	Active

Group	Backup	Process							:	Realtime

Peer	Group	Device	Name				:	AC2

Peer	Group	Software	Version	:	V200R019C00

Group	Backup	Modules						:	Access-user

AP

----------------------------------------------------------

[AC2]	display	hsb-group	0

Hot	Standby	Group	Information:

----------------------------------------------------------

HSB-group	ID														:	0

Vrrp	Group	ID													:	1

Vrrp	Interface													:	Vlanif100

Service	Index														:	0

Group	Vrrp	Status									:	Backup

Group	Status													:	Inactive

Group	Backup	Process							:	Realtime

Peer	Group	Device	Name				:	AC1

Peer	Group	Software	Version	:	V200R019C00

Group	Backup	Modules						:	Access-user

AP

----------------------------------------------------------

# Check the AP online status on AC1 and AC2.

<AC1>display	ap	all

Info:	This	operation	may	take	a	few	seconds.	Please	wait	for	a	moment.done.

Total	AP	information:

nor		:	normal										[2]

----------------------------------------------------------------------------------------

ID			MAC													Name 	Group			IP										Type												State	STA	Uptime

----------------------------------------------------------------------------------------

0				00e0-fc6e-2890					ap1		 depart	  10.0.10.254		AirEngine5760-10			nor			1			12M:27S

1				00e0-fcde-1990					ap2		 depart	  10.0.10.253		AirEngine5760-10			nor			0			12M:29S

----------------------------------------------------------------------------------------

Total:	2

<AC2>dis	ap	all

Info:	This	operation	may	take	a	few	seconds.	Please	wait	for	a	moment.done.

Total	AP	information:

stdby:	standby									[2]

---------------------------------------------------------------------------------------

ID			MAC												Name 	Group			IP										Type												State	 STA	Uptime

---------------------------------------------------------------------------------------

0				00e0-fc6e-2890				ap1		 depart	  10.0.10.254		AirEngine5760-10		stdby		1		 	-

1				00e0-fcde-1990				ap2		 	depart	  10.0.10.253		AirEngine5760-10		stdby		0			 -

---------------------------------------------------------------------------------------

Total:	2

The AP status is normal on AC1 and standby on AC2. AP information on AC2 is synchronized from the HSB group.

# Enable the STA to search for the WLAN with the SSID HSB and go online. Check STA information on AC1 and AC2.

<AC1>display station all

Rf/WLAN: Radio ID/WLAN ID

Rx/Tx				: link receive rate/link transmit rate(Mbps)

--------------------------------------------------------------------------------------------------

STA MAC          		AP ID Ap name  Rf/WLAN  Band  Type  Rx/Tx      RSSI  VLAN  IP address SSID

--------------------------------------------------------------------------------------------------

5489-986f-73ad        0     ap1      0/1      2.4G     -     -/-        -     11    10.0.11.254 HSB

--------------------------------------------------------------------------------------------------

Total: 1 2.4G: 1 5G: 0

<AC2>display station all

Rf/WLAN: Radio ID/WLAN ID

Rx/Tx				: link receive rate/link transmit rate(Mbps)

-------------------------------------------------------------------------------------------------

STA MAC          		AP ID Ap name  Rf/WLAN  Band  Type  Rx/Tx      RSSI  VLAN  IP address SSID

--------------------------------------------------------------------------------------------------

5489-986f-73ad        0     ap1      0/1      2.4G     -     -/-        -     11    10.0.11.254 HSB

--------------------------------------------------------------------------------------------------

Total: 1 2.4G: 1 5G: 0

STA information is displayed on both AC1 and AC2.

# Shut down the interface on AC1 to simulate an AC fault.

[AC1]interface GigabitEthernet 0/0/12

[AC1-GigabitEthernet0/0/12] shutdown

# Check brief VRRP information on AC2.

<AC2>display	vrrp	brief

Total:1					Master:1					Backup:0					Non-active:0

VRID		State								Interface																Type					Virtual	IP

----------------------------------------------------------------

1					Master							Vlanif100															Admin				10.0.100.254

VLANIF 100 of AC2 is the master in VRRP group 1.

# Check the running status of the HSB group on AC2.

<AC2>display hsb-group 0

Hot Standby Group Information:

----------------------------------------------------------

HSB-group	ID														:	0

Vrrp	Group	ID													:	1

Vrrp	Interface													:	Vlanif100

Service	Index														:	0

Group	Vrrp	Status									:	Master

Group	Status													:	Independent

Group	Backup	Process							:	Independent

Peer	Group	Device	Name				:	AC1

Peer	Group	Software	Version	:	V200R019C00

Group	Backup	Modules						:	Access-user

AP

----------------------------------------------------------

AC2 is the master in VRRP group 1 that is in the independent running state (indicating that the connection to AC1 is interrupted).

# Check AP online information on AC2.

<AC2>display	ap	all

Info:	This	operation	may	take	a	few	seconds.	Please	wait	for	a	moment.done.

Total	AP	information:

nor		:	normal										[2]

----------------------------------------------------------------------------------------

ID			MAC												Name	 Group				IP										Type										 State		 STA	 Uptime

----------------------------------------------------------------------------------------

0				00e0-fc6e-2890				ap1		  depart  10.0.10.254		AirEngine5760-10		nor			1			39M:53S

1				00e0-fcde-1990				ap2		  depart  	10.0.10.253		AirEngine5760-10		nor			0			39M:55S

----------------------------------------------------------------------------------------

Total:	2

The AP status on AC2 is normal, and the Uptime value is not null (-).

# Check STA information on AC2.

<AC2>display station all

Rf/WLAN: Radio ID/WLAN ID

Rx/Tx: link receive rate/link transmit rate(Mbps)

-------------------------------------------------------------------------------------------------

STA MAC          	AP ID Ap name  Rf/WLAN  Band  Type  Rx/Tx      RSSI  VLAN  IP address SSID

--------------------------------------------------------------------------------------------------

5489-986f-73ad        0   ap1      0/1       2.4G      -     -/-        -     11    10.0.11.254 HSB

--------------------------------------------------------------------------------------------------

Total: 1 2.4G: 1 5G: 0

STA information is still displayed on AC2.

----End

Quiz

How many CAPWAP control channels are established between an AP and ACs when VRRP HSB is deployed?

Configuration Reference

Configuration on S3

#

sysname S3

#

vlan batch 10 to 11 100

#

dhcp enable

#

ip pool ap

gateway-list 10.0.10.1

network 10.0.10.0 mask 255.255.255.0

option 43 sub-option 3 ascii 10.0.100.254

#

ip pool service

gateway-list 10.0.11.1

network 10.0.11.0 mask 255.255.255.0

dns-list 10.0.11.1

#

interface Vlanif10

description ap_mgnt

ip address 10.0.10.1 255.255.255.0

dhcp select global

#

interface Vlanif11

description ap_service

ip address 10.0.11.1 255.255.255.0

dhcp select global

#

interface Vlanif100

description to_AC

ip address 10.0.100.3 255.255.255.0

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 10 to 11

#

interface GigabitEthernet0/0/4

port link-type trunk

port trunk pvid vlan 10

port trunk allow-pass vlan 10 to 11

#

return

Configuration on S4

#

sysname S4

#

vlan batch 10 to 11

#

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 10 to 11

#

interface GigabitEthernet0/0/4

port link-type trunk

port trunk pvid vlan 10

port trunk allow-pass vlan 10 to 11

#

return

Configuration on AC1

#

sysname AC1

#

vrrp recover-delay 60

#

vlan batch 100

#

interface Vlanif100

description to_S3_CAPWAP

ip address 10.0.100.1 255.255.255.0

vrrp vrid 1 virtual-ip 10.0.100.254

admin-vrrp vrid 1

vrrp vrid 1 priority 120

vrrp vrid 1 preempt-mode timer delay 1800

#

interface GigabitEthernet0/0/12

port link-type trunk

port trunk allow-pass vlan 100

#

ip route-static 10.0.10.0 255.255.255.0 10.0.100.3

#

capwap source ip-address 10.0.100.254

#

hsb-service 0

service-ip-port local-ip 10.0.100.1 peer-ip 10.0.100.2 local-data-port 10241 pe

er-data-port 10241

service-keep-alive detect retransmit 3 interval 6

#

hsb-group 0

track vrrp vrid 1 interface Vlanif100

bind-service 0

hsb enable

#

hsb-service-type access-user hsb-group 0

#

hsb-service-type ap hsb-group 0

#

wlan

traffic-profile name default

security-profile name depart

security wpa2 psk pass-phrase huawei123 aes

ssid-profile name depart

ssid HSB

vap-profile name depart

service-vlan vlan-id 11

ssid-profile depart

security-profile depart

regulatory-domain-profile name default

ap-group name depart

radio 0

vap-profile depart wlan 1

radio 1

vap-profile depart wlan 1

radio 2

vap-profile depart wlan 1

ap-group name default

ap-group name ap-group1

ap-id 0 type-id 56 ap-mac 00e0-fc6e-2890 ap-sn 210235448310F30CF56D

ap-name ap1

ap-group depart

ap-id 1 type-id 56 ap-mac 00e0-fcde-1990 ap-sn 210235448310FA145341

ap-name ap2

ap-group depart

provision-ap

#

Return

Configuration on AC2

#

sysname AC2

#

vrrp recover-delay 60

#

vlan batch 100

#

interface Vlanif100

description to_S3_CAPWAP

ip address 10.0.100.2 255.255.255.0

vrrp vrid 1 virtual-ip 10.0.100.254

admin-vrrp vrid 1

#

interface GigabitEthernet0/0/13

port link-type trunk

port trunk allow-pass vlan 100

#

ip route-static 10.0.10.0 255.255.255.0 10.0.100.3

#

capwap source ip-address 10.0.100.254

#

hsb-service 0

service-ip-port local-ip 10.0.100.2 peer-ip 10.0.100.1 local-data-port 10241 pe

er-data-port 10241

service-keep-alive detect retransmit 3 interval 6

#

hsb-group 0

track vrrp vrid 1 interface Vlanif100

bind-service 0

hsb enable

#

hsb-service-type access-user hsb-group 0

#

hsb-service-type ap hsb-group 0

#

wlan

traffic-profile name default

security-profile name depart

security wpa2 psk pass-phrase huawei123 aes

ssid-profile name depart

ssid HSB

vap-profile name depart

service-vlan vlan-id 11

ssid-profile depart

security-profile depart

regulatory-domain-profile name default

ap-group name depart

radio 0

vap-profile depart wlan 1

radio 1

vap-profile depart wlan 1

radio 2

vap-profile depart wlan 1

ap-group name default

ap-id 0 type-id 56 ap-mac 00e0-fc6e-2890 ap-sn 210235448310F30CF56D

ap-name ap1

ap-group depart

ap-id 1 type-id 56 ap-mac 00e0-fcde-1990 ap-sn 210235448310FA145341

ap-name ap2

ap-group depart

provision-ap

#

return

Dual-Link Cold Backup Configuration

Introduction

Objectives

Upon completion of this task, you will be able to:

Configure dual-link cold backup.

Improve AC reliability through dual-link cold backup.

Explain the technical implementation of dual-link cold backup.

Specify the IP addresses of the active and standby ACs through DHCP Option 43.

Networking Topology

              - Dual-Link cold backup configuration

As shown in the figure, AC1 is the active AC, and AC2 is the standby AC. APs establish CAPWAP links with the active and standby ACs, and periodically exchange CAPWAP packets with the ACs to monitor the link status. When the AP detects a failure of the link with the active AC, the AP instructs the standby AC to perform an active/standby switchover. The standby AC becomes the active AC and controls STA access, thereby improving WLAN reliability.

S4 transparently transmits packets from AP2 at Layer 2. S3 serves as the gateway for APs and STAs.

S3 is enabled with DHCP to allocate IP addresses to AP1, AP2, and STAs associated with them. APs obtain the IP addresses of AC1 and AC2 from Option 43 in DHCP packets. The direct forwarding mode is configured for all APs.

Background

As the number of STAs on the enterprise intranet keeps increasing, to ensure the stability of wireless services, you, a network engineer, decide to purchase a new AC and deploy dual-link cold backup so that the new AC works as a backup of the existing AC, thereby improving the reliability of wireless services.

Data Planning

AC's data planning

| Item | Configuration |
| --- | --- |
| Management VLAN for APs | VLAN 10 |
| Service VLAN for STAs | VLAN 11 |
| DHCP server | S3 functions as a DHCP server to allocate IP addresses to APs and STAs. |
| IP address pool for APs | 10.0.10.0/24 |
| IP address pool for STAs | 10.0.11.0/24 |
| AC's source interface address | 10.0.100.1 and 10.0.100.2 |
| AP group | Name: depart |
|  | Referenced profile: VAP profile depart |
| Regulatory domain profile | Name: default |
|  | Country code: CN |
| SSID profile | Name: depart |
|  | SSID name: LB |
| Security profile | Name: depart |
|  | Security policy: WPA-WPA2+PSK+AES |
|  | Password: huawei123 |
| VAP profile | Name: depart |
|  | Forwarding mode: direct forwarding |
|  | Service VLAN: VLAN 11 |
|  | Referenced profiles: SSID profile depart and security profile depart |
| Dual-link cold backup | AC1 priority: 0 |
|  | AC2 priority: 1 |

Lab Configuration

Configuration Roadmap

Shut down unnecessary ports and enable the PoE function on switches.

Configure wired-side functions so that S3 serves as the gateway for APs and STAs, and AC1 and AC2 communicate with S3 at Layer 3 through VLANIF 100 working as the CAPWAP source interface.

Configure WLAN services on AC1 and AC2.

Configure dual-link cold backup. Set the priority of AC1 to 0 and that of AC2 to 1 so that AC1 and AC2 become the active and standby ACs, respectively.

Verify dual-link cold backup. Shut down the downlink interface on AC1, and check the states of APs and STAs on AC2.

Configuration Procedure

Complete basic device configurations.

# Name the devices.

The configuration details are not provided.

# Shut down unnecessary ports.

The configuration details are not provided.

# Enable the PoE function on S3 and S4 ports connected to APs.

[S3]interface GigabitEthernet 0/0/4

[S3-GigabitEthernet0/0/4] poe enable

[S4]interface GigabitEthernet 0/0/4

[S4-GigabitEthernet0/0/4] poe enable

The poe enable command enables the PoE function on a port. When a port detects a PD connected, the port supplies power to the PD. By default, the PoE function is enabled on a port. Therefore, this command is unnecessary and is provided for demonstration purpose only.

Configure the wired network.

Configure the wired network of the switches and ACs as planned.

# Create VLANs on S3, S4, AC1, and AC2, and assign ports to the VLANs.

[S3]vlan batch 10 11 100

[S3]interface GigabitEthernet0/0/1

[S3-GigabitEthernet0/0/1] port link-type trunk

[S3-GigabitEthernet0/0/1] port trunk allow-pass vlan 100

[S3-GigabitEthernet0/0/1] quit

[S3]interface GigabitEthernet0/0/2

[S3-GigabitEthernet0/0/2] port link-type trunk

[S3-GigabitEthernet0/0/2] port trunk allow-pass vlan 100

[S3-GigabitEthernet0/0/2] quit

[S3]interface GigabitEthernet0/0/3

[S3-GigabitEthernet0/0/3] port link-type trunk

[S3-GigabitEthernet0/0/3] port trunk allow-pass vlan 10 to 11

[S3-GigabitEthernet0/0/3] quit

[S3]interface GigabitEthernet0/0/4

[S3-GigabitEthernet0/0/4] port link-type trunk

[S3-GigabitEthernet0/0/4] port trunk pvid vlan 10

[S3-GigabitEthernet0/0/4] port trunk allow-pass vlan 10 to 11

[S3-GigabitEthernet0/0/4] quit

The PVID of the S3 port connected to AP1 is set to VLAN 10, packets in the service VLANs and management VLANs are allowed to pass on the S3 port connected to S4, and the S3 ports connected to ACs are configured to allow packets in VLAN 100 to pass through.

[S4]vlan batch 10 11

Info: This operation may take a few seconds. Please wait for a moment...done.

[S4]interface GigabitEthernet0/0/3

[S4-GigabitEthernet0/0/3] port link-type trunk

[S4-GigabitEthernet0/0/3] port trunk allow-pass vlan 10 to 11

[S4-GigabitEthernet0/0/3] quit

[S4]interface GigabitEthernet0/0/4

[S4-GigabitEthernet0/0/4] port link-type trunk

[S4-GigabitEthernet0/0/4] port trunk pvid vlan 10

[S4-GigabitEthernet0/0/4] port trunk allow-pass vlan 10 to 11

[S4-GigabitEthernet0/0/4] quit

The PVID of the S4 port connected to AP2 is set to VLAN 10, and the uplink port of S4 is configured to transparently transmit packets in VLANs 10 (management VLAN) and 11 (service VLAN).

[AC1]vlan batch 100

[AC1]interface GigabitEthernet0/0/12

[AC1-GigabitEthernet0/0/12] port link-type trunk

[AC1-GigabitEthernet0/0/12] port trunk allow-pass vlan 100

[AC1-GigabitEthernet0/0/12] quit

The interface is configured to allow packets in VLAN 100 to pass through.

[AC2]vlan batch 100

[AC2]interface GigabitEthernet0/0/13

[AC2-GigabitEthernet0/0/13] port link-type trunk

[AC2-GigabitEthernet0/0/13] port trunk allow-pass vlan 100

[AC2-GigabitEthernet0/0/13] quit

The interface is configured to allow packets in VLAN 100 to pass through.

# Create VLANIF interfaces on S3, AC1, and AC2.

[S3]interface Vlanif10

[S3-Vlanif10] description ap_mgnt

[S3-Vlanif10] ip address 10.0.10.1 255.255.255.0

[S3-Vlanif10] quit

[S3]interface Vlanif11

[S3-Vlanif11] description ap_service

[S3-Vlanif11] ip address 10.0.11.1 255.255.255.0

[S3-Vlanif11] quit

[S3]interface Vlanif100

[S3-Vlanif100] description to_AC

[S3-Vlanif100] ip address 10.0.100.3 255.255.255.0

[S3-Vlanif100] quit

On S3, VLANIF 10 is configured as the management VLAN gateway of AP1 and AP2; VLANIF 11 is configured as the service VLAN gateway of STAs connected to AP1 and AP2; and VLANIF 100 is used for Layer 3 communication with AC1 and AC2.

[AC1]interface Vlanif100

[AC1-Vlanif100] description to_S3_CAPWAP

[AC1-Vlanif100] ip address 10.0.100.1 255.255.255.0

[AC1-Vlanif100] quit

[AC1] capwap source interface vlanif100

VLANIF 100 is configured as the CAPWAP source interface of AC1.

[AC2]interface Vlanif100

[AC2-Vlanif100] description to_S3_CAPWAP

[AC2-Vlanif100] ip address 10.0.100.2 255.255.255.0

[AC2-Vlanif100] quit

[AC2] capwap source interface vlanif100

VLANIF 100 is configured as the CAPWAP source interface of AC2.

# Configure routes to the AP management network segments on AC1 and AC2.

[AC1]ip route-static 10.0.10.0 255.255.255.0 10.0.100.3

[AC2]ip route-static 10.0.10.0 255.255.255.0 10.0.100.3

Static routes to the AP management network segments are configured on ACs for CAPWAP communication with APs.

# Configure the DHCP service on S3.

[S3]dhcp enable

The DHCP service is enabled.

[S3]ip pool ap

[S3-ip-pool-ap] gateway-list 10.0.10.1

[S3-ip-pool-ap] network 10.0.10.0 mask 255.255.255.0

[S3-ip-pool-ap] option 43 sub-option 2 ip-address 10.0.100.1 10.0.100.2

[S3-ip-pool-ap] quit

[S3]ip pool service

[S3-ip-pool-service] gateway-list 10.0.11.1

[S3-ip-pool-service] network 10.0.11.0 mask 255.255.255.0

[S3-ip-pool-service] dns-list 10.0.11.1

[S3-ip-pool-service] quit

The address pool ap is configured to allocate IP addresses to APs. Option 43 is configured to specify the AC's IP address, and sub-option 2 is configured to specify the IP addresses of the active and standby ACs.

The address pool service is configured to allocate IP addresses to STAs connected to AP1 and AP2. VLANIF interfaces on S3 are configured as the gateways for all address pools.

[S3]interface Vlanif10

[S3-Vlanif10] dhcp select global

[S3-Vlanif10] quit

[S3]interface Vlanif11

[S3-Vlanif11] dhcp select global

[S3-Vlanif11] quit

The global address pool is selected on the VLANIF interfaces.

Configure ACs.

Create the AP group depart, configure MAC address authentication for APs, name the APs ap1 and ap2, add them to the AP group depart, and associate parameter profiles with the VAP profile depart.

The WLAN configurations on AC1 and AC2 are the same. The following uses AC1 as an example.

# Create an AP group named depart.

[AC1]wlan

[AC1-wlan-view] ap-group name depart

[AC1-wlan-ap-group-depart] quit

# Create a regulatory domain profile and configure the AC's country code in the profile.

[AC1]wlan

[AC1-wlan-view] regulatory-domain-profile name default

[AC1-wlan-regulate-domain-default] country-code cn

Info: The current country code is same with the input country code.

[AC1-wlan-regulate-domain-default] quit

A regulatory domain profile provides configurations of the country code, calibration channel set, and calibration bandwidth for an AP.

By default, the system provides the regulatory domain profile default. Therefore, the default regulatory domain profile is displayed.

A country code identifies the country where AP radios work. Different countries require different AP radio attributes, including the transmit power and supported channels. The correct country code configuration ensures that radio attributes of APs comply with local laws and regulations of countries and regions to which the APs are delivered. By default, the country code CN is configured.

# Bind the regulatory domain profile to the AP group.

[AC1]wlan

[AC1-wlan-view]ap-group name depart

[AC1-wlan-ap-group- depart]regulatory-domain-profile default

Warning: Modifying the country code will clear channel, power and antenna gain configurations of the radio and reset the AP. Continue?[Y/N]:y

[AC1-wlan-ap-group- depart]quit

In the AP group view, the regulatory-domain-profile command binds a regulatory domain profile to an AP or AP group. By default, the regulatory domain profile default is bound to an AP group, but no regulatory domain profile is bound to an AP. In the regulatory domain profile default, the country code is CN, 2.4G calibration channels include channels 1, 6, and 11, and 5G calibration channels include channels 149, 153, 157, 161, and 165. Therefore, you can skip this step and the previous step in actual operations.

# Add APs.

[AC1]wlan

[AC1-wlan-view] ap auth-mode mac-auth

[AC1-wlan-view] ap-id 0 ap-mac 00e0-fc6e-2890          //Set the AP's MAC address as required in the lab environment.

[AC1-wlan-ap-0] ap-name ap1

[AC1-wlan-ap-0] ap-group depart

[AC1-wlan-ap-0] quit

[AC1-wlan-view] ap-id 1 ap-mac 00e0-fcde-1990          //Set the AP's MAC address as required in the lab environment.

[AC1-wlan-ap-1] ap-name ap2

[AC1-wlan-ap-1] ap-group depart

[AC1-wlan-ap-1] quit

Three AP authentication modes are supported. By default, MAC address authentication is used. APs are added on the AC before they go online, named ap1 and ap2, and added to the AP group depart.

# Configure parameter profiles.

[AC1]wlan

[AC1-wlan-view] security-profile name depart

[AC1-wlan-sec-prof-depart] security wpa2 psk pass-phrase huawei123 aes

[AC1-wlan-sec-prof- depart] quit

[AC1-wlan-view] ssid-profile name depart

[AC1-wlan-ssid-prof-depart] ssid LB

[AC1-wlan-ssid-prof-depart] quit

[AC1-wlan-view] vap-profile name depart

[AC1-wlan-vap-prof-depart] forward-mode direct-forward

[AC1-wlan-vap-prof-depart] service-vlan vlan-id 11

[AC1-wlan-vap-prof-depart] ssid-profile depart

[AC1-wlan-vap-prof-depart] security-profile depart

[AC1-wlan-vap-prof-depart] quit

[AC1-wlan-view] ap-group name depart

[AC1-wlan-ap-group-depart] vap-profile depart wlan 1 radio all

[AC1-wlan-ap-group-depart] quit

The security profile depart is configured, with the authentication mode of WPA2-PSK and the pre-shared key of huawei123.

The SSID profile depart is configured, and the SSID is set to LB.

The VAP profile depart is configured, with the direct forwarding mode and the service VLAN 11, and has the SSID profile depart and security profile depart bound.

The VAP profile depart is bound to the AP group depart.

Configure dual-link cold backup.

Specify the IP address of the peer AC for APs on the active and standby ACs. Set the priority of AC1 to 0 and that of AC2 to 1 so that AC1 and AC2 become the active and standby ACs, respectively.

# Configure AC1.

[AC1]wlan

[AC1-wlan-view]ac protect protect-ac 10.0.100.2 priority 0

Warning: Operation successful. It will take effect after AP reset.

[AC1-wlan-view]undo ac protect restore disable

Info: Protect restore has already enabled.

[AC1-wlan-view]ac protect enable

Warning: This operation maybe cause AP reset, continue?[Y/N]:y

Info: This operation may take a few seconds. Please wait for a moment.done.

Info: Capwap echo interval has changed to default value 25, capwap echo times to 3.

By default, dual-link backup is disabled. When the ac protect enable command is executed, a message is displayed indicating that all APs will be restarted. After the APs are restarted, the dual-link backup function takes effect.

# Configure AC2.

[AC2]wlan

[AC2-wlan-view]ac protect protect-ac 10.0.100.1 priority 1

Warning: Operation successful. It will take effect after AP reset.

[AC2-wlan-view]undo ac protect restore disable

Info: Protect restore has already enabled.

[AC2-wlan-view]ac protect enable

Warning: This operation maybe cause AP reset, continue?[Y/N]:y

Info: This operation may take a few seconds. Please wait for a moment.done.

Info: Capwap echo interval has changed to default value 25, capwap echo times to 3.

By default, dual-link backup is disabled. When the ac protect enable command is executed, a message is displayed indicating that all APs will be restarted. After the APs are restarted, the dual-link backup function takes effect.

Verify the configuration.

# Run the display ac protect command on AC1 to check the dual-link information and AC priority.

<AC1>display ac protect

------------------------------------------------------------

Protect state             : enable

Protect AC               : 10.0.100.2

Priority                  : 0

Protect restore           : enable

Coldbackup kickoff station: disable

------------------------------------------------------------

The peer IP address is 10.0.100.2, and the local priority is 0.

# Run the display ac protect command on AC2 to check the dual-link information and AC priority.

<AC2>display ac protect

------------------------------------------------------------

Protect state             : enable

Protect AC               : 10.0.100.1

Priority                  : 1

Protect restore           : enable

Coldbackup kickoff station: disable

------------------------------------------------------------

The peer IP address is 10.0.100.1, and the local priority is 1.

# Check the AP online status on AC1 and AC2.

<AC1>display	ap	all

Info:	This	operation	may	take	a	few	seconds.	Please	wait	for	a	moment.done.

Total	AP	information:

nor		:	normal										[2]

----------------------------------------------------------------------------------------

ID			MAC												Name	 Group				IP															Type												  State	 STA 	Uptime

----------------------------------------------------------------------------------------

0				00e0-fc6e-2890				ap1		 depart 	 	10.0.10.254					 	AirEngine5760-10					nor			0			12M:27S

1				00e0-fcde-1990				ap2	  	depart	  10.0.10.253						 AirEngine5760-10					nor			0			12M:29S

----------------------------------------------------------------------------------------

Total:	2

<AC2>dis	ap	all

Info:	This	operation	may	take	a	few	seconds.	Please	wait	for	a	moment.done.

Total	AP	information:

stdby:	standby									[2]

---------------------------------------------------------------------------------------

ID			MAC												Name 	Group			IP														Type												 State	 STA	 Uptime

---------------------------------------------------------------------------------------

0				00e0-fc6e-2890		 	ap1		  depart	  10.0.10.254						AirEngine5760-10				stdby		0			-

1				00e0-fcde-1990		 ap2		  depart	  10.0.10.253						AirEngine5760-10				stdby		0			-

---------------------------------------------------------------------------------------

Total:	2

The AP status is normal on AC1 and standby on AC2. The APs have established CAPWAP tunnels with AC1 and AC2.

# Enable the STA to search for the WLAN with the SSID LB and go online. Check STA information on AC1 and AC2.

<AC1>display station all

Rf/WLAN: Radio ID/WLAN ID

Rx/Tx				: link receive rate/link transmit rate(Mbps)

--------------------------------------------------------------------------------------------------

STA MAC          AP ID Ap name  Rf/WLAN  Band  Type  Rx/Tx      RSSI  VLAN  IP address SSID

--------------------------------------------------------------------------------------------------

5489-986f-73ad        0     ap1      0/1      2.4G      -     -/-        -     11    10.0.11.254 LB

--------------------------------------------------------------------------------------------------

Total: 1 2.4G: 1 5G: 0

<AC2>display station all

Rf/WLAN: Radio ID/WLAN ID

Rx/Tx				: link receive rate/link transmit rate(Mbps)

--------------------------------------------------------------------------------------------------

STA MAC        AP ID Ap name  Rf/WLAN  Band  Type  Rx/Tx      RSSI  VLAN  IP address  SSID

--------------------------------------------------------------------------------------------------

--------------------------------------------------------------------------------------------------

In this case, STA information is displayed only on AC1.

# Shut down the interface on AC1 to simulate an AC fault.

[AC1]interface GigabitEthernet 0/0/12

[AC1-GigabitEthernet0/0/12] shutdown

# After 75 seconds, check the AP online status on AC2.

<AC2>display ap all

Info: This operation may take a few seconds. Please wait for a moment.done.

Total AP information:

nor  : normal          [2]

---------------------------------------------------------------------------------------

ID   MAC            Name Group     IP           Type                 State STA Uptime

---------------------------------------------------------------------------------------

0    00e0-fc6e-2890    ap1  depart 		10.0.10.253   AirEngine5760-10      nor   0   2S

1    00e0-fcde-1990    ap2  depart		 10.0.10.254   AirEngine5760-10      nor   0   2S

---------------------------------------------------------------------------------------

Total: 2

The CAPWAP heartbeat detection time is 25s. If no response is received after three heartbeat timeouts, the peer end is considered faulty. Therefore, the AP status change can be displayed on the standby AC at least 75 seconds later.

The AP status on AC2 changes from standby to normal, but no STA goes online. When an active/standby switchover is implemented between two ACs, STAs using open system authentication remain connected to APs while STAs using other authentication modes are disconnected and need to go online again by default.

In this case, reassociate the STA with the SSID LB.

# Check STA information on AC2.

<AC2>display station all

Rf/WLAN: Radio ID/WLAN ID

Rx/Tx: link receive rate/link transmit rate(Mbps)

--------------------------------------------------------------------------------------------------

STA MAC          AP ID Ap name  Rf/WLAN  Band  Type  Rx/Tx      RSSI  VLAN IP address  SSID

--------------------------------------------------------------------------------------------------

5489-986f-73ad         0     ap1      0/1      2.4G     -     -/-        -      11  10.0.11.254 LB

--------------------------------------------------------------------------------------------------

Total: 1 2.4G: 1 5G: 0

STA information is displayed on AC2.

----End

Quiz

How many CAPWAP control channels are established between an AP and ACs when dual-link cold backup is deployed?

Configuration Reference

Configuration on S3

#

sysname S3

#

vlan batch 10 to 11 100

#

dhcp enable

#

ip pool ap

gateway-list 10.0.10.1

network 10.0.10.0 mask 255.255.255.0

option 43 sub-option 3 ascii 10.0.100.254

#

ip pool service

gateway-list 10.0.11.1

network 10.0.11.0 mask 255.255.255.0

dns-list 10.0.11.1

#

interface Vlanif10

description ap_mgnt

ip address 10.0.10.1 255.255.255.0

dhcp select global

#

interface Vlanif11

description ap_service

ip address 10.0.11.1 255.255.255.0

dhcp select global

#

interface Vlanif100

description to_AC

ip address 10.0.100.3 255.255.255.0

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 100

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 10 to 11

#

interface GigabitEthernet0/0/4

port link-type trunk

port trunk pvid vlan 10

port trunk allow-pass vlan 10 to 11

#

return

Configuration on S4

#

sysname S4

#

vlan batch 10 to 11

#

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 10 to 11

#

interface GigabitEthernet0/0/4

port link-type trunk

port trunk pvid vlan 10

port trunk allow-pass vlan 10 to 11

#

return

Configuration on AC1

#

sysname AC1

#

vlan batch 100

#

interface Vlanif100

description to_S3_CAPWAP

ip address 10.0.100.1 255.255.255.0

#

interface GigabitEthernet0/0/12

port link-type trunk

port trunk allow-pass vlan 100

#

ip route-static 10.0.10.0 255.255.255.0 10.0.100.3

#

capwap source interface vlanif100

#

wlan

ac protect enable protect-ac 10.0.100.2

traffic-profile name default

security-profile name depart

security wpa2 psk pass-phrase huawei123 aes

ssid-profile name depart

ssid LB

vap-profile name depart

service-vlan vlan-id 11

ssid-profile depart

security-profile depart

regulatory-domain-profile name default

ap-group name depart

radio 0

vap-profile depart wlan 1

radio 1

vap-profile depart wlan 1

radio 2

vap-profile depart wlan 1

ap-group name default

ap-group name ap-group1

ap-id 0 type-id 56 ap-mac 00e0-fc6e-2890 ap-sn 210235448310F30CF56D

ap-name ap1

ap-group depart

ap-id 1 type-id 56 ap-mac 00e0-fcde-1990 ap-sn 210235448310FA145341

ap-name ap2

ap-group depart

provision-ap

#

Return

Configuration on AC2

#

sysname AC2

#

vlan batch 100

#

interface Vlanif100

description to_S3_CAPWAP

ip address 10.0.100.2 255.255.255.0

#

interface GigabitEthernet0/0/13

port link-type trunk

port trunk allow-pass vlan 100

#

ip route-static 10.0.10.0 255.255.255.0 10.0.100.3

#

capwap source interface vlanif100

#

wlan

ac protect enable protect-ac 10.0.100.1 priority 1

traffic-profile name default

security-profile name depart

security wpa2 psk pass-phrase huawei123 aes

ssid-profile name depart

ssid LB

vap-profile name depart

service-vlan vlan-id 11

ssid-profile depart

security-profile depart

regulatory-domain-profile name default

ap-group name depart

radio 0

vap-profile depart wlan 1

radio 1

vap-profile depart wlan 1

radio 2

vap-profile depart wlan 1

ap-group name default

ap-id 0 type-id 56 ap-mac 00e0-fc6e-2890 ap-sn 210235448310F30CF56D

ap-name ap1

ap-group depart

ap-id 1 type-id 56 ap-mac 00e0-fcde-1990 ap-sn 210235448310FA145341

ap-name ap2

ap-group depart

provision-ap

#

Return

Reference Answers

Answers to the basic OSPF experiment

- You can configure an interface a silent interface if this interface does not need to establish an OSPF neighbor relationship, which reduces unnecessary OSPF packets sending. In addition, the silent interface does not need to send OSPF packets.

- The cost of a Type 2 external route remains unchanged within the OSPF area and is equal to the cost (the default value is 1) configured when such a route is imported into the OSPF area. The cost of a Type 1 external route within the OSPF area is the sum of the cost of the Type 1 route and the cost of the route from the router to the ASBR in the OSPF area.

- When external routes are imported to the OSPF routing table on an ASBR, a Type 4 LSA is generated by an ABR in the area to which the ASBR belongs. The Type 4 LSA is used to calculate the route destined for the ASBR.

- A non-backbone area on the border of an AS can be configured as an NSSA if this area needs to import external routes and has to avoid resource consumption caused by the external routes.

- A non-backbone area must be connected to backbone area 0. In addition, due to the OSPF loop prevention mechanism, if area 0 is not contiguous, routes between non-backbone areas may be missing, causing communication failures.

Answers to the basic IS-IS experiments

- The following conditions must be met: The routers to which the interfaces belong must be of the same level; the interfaces must be of the same level; the area IDs of the devices must be the same; the IP addresses of the interfaces must be on the same network segment.

Answers to the basic BGP experiments

- Compared with physical interfaces, loopback interfaces are stable. Route redundancy between peers can be used to ensure the reliability of a peer relationship. If a physical interface is faulty, a BGP session is interrupted, even if IP connectivity between peers is normal.

- Summary routes automatically generated using the summarization route command carry only the Aggregator attribute, but not the Atomic-Aggregate attribute. Summary routes manually generated using the aggregate command carry the Atomic-Aggregate attribute.

- No, the Originator_ID and Cluster_List attributes are not carried. They are used to prevent routing loops when RRs exist within an AS. EBGP peers use the AS_Path attribute to prevent routing loops, which is independent of the Originator_ID and Cluster_List attributes.

- A route-policy can be used to implement this function, which is not recommended. Deleting an AS number from the AS_Path attribute may cause routing loops between ASs.

Answers to the routing policy and routing control experiments

- Distance-vector routing protocols generate routes based on routing tables. Filter-policies affect the routes to be accepted from neighbors and the routes to be advertised to neighbors. Link-state routing protocols generate routes based on LSDBs, and routing information is contained in link-state LSAs. The filter-policy, however, cannot filter LSAs to be advertised and accepted. Therefore, the filter-policy affects only the local routing table, but does not affect the LSA and LSDB integrity or the protocol routing table. In addition, only the routes that match the filter-policy can be added to the routing table, and the routes that do not match the filter-policy are not added to the routing table.

Answers to the RSTP and MSTP experiments

- Improvements in RSTP, compared with STP:

  - RSTP defines more port roles to simplify the learning and deployment of STP.

  - RSTP redefines port states.

  - RSTP changes the configuration BPDU format and uses the Flags field to describe port roles.

  - RSTP processes configuration BPDUs differently from STP.

  - RSTP uses the P/A mechanism to implement fast convergence.

  - The protection function is added.

Answers to the multicast experiments

- PIM-DM applies only to small-scale networks where multicast receivers are densely distributed. PIM-DM spreads multicast traffic to the entire network in a harsh way. If leaf nodes do not need multicast traffic, they do not need to use protocol packets to prune themselves from the SPT. Consequently, on a large-scale network where receivers are scattered, a large amount of unnecessary multicast traffic is generated, and multicast protocol packets are frequently exchanged, wasting link bandwidth and device resources.

- PIM-SM uses the "push" mode. Multicast paths are established only when multicast receivers exist. This mode consumes less device resources and link bandwidth.

Answers to the firewall technology experiments

- The local zone defines a device itself, covering interfaces on the device. Adding an interface to another security zone only indicates that the network connected to the interface belongs to the zone. The traffic forwarded by the interface belongs to the security zone that the interface joins. When the interface address of the device is used as the source address to access an extranet, the source security zone is still the local zone.

Answer to the VRRP experiments

- If the actual IP address of an interface is the same as the virtual IP address of a VRRP group, the priority of the VRRP group is 255.

Answer to the DHCP experiments

- After receiving the DHCP Discover message from the DHCP relay agent, the DHCP server selects an address pool on the same network segment as the GIADDR field value in the message and assigns parameters, such as an IP address, to the client.

Answers to the WLAN experiments

- After Layer 2 roaming is performed, STA remains in the original subnet. The FAP/FAC forwards packets of Layer 2 roaming STAs in the same way as it forwards packets of new online STAs. That is, packets of Layer 2 roaming STAs are directly forwarded on the local network of the FAP/FAC and do not need to be forwarded back to the HAP/HAC through the inter-AC tunnel. After Layer 3 roaming is performed, STAs switch to different subnets. To enable the STAs to access the original network after Layer 3 roaming is performed, ensure that user traffic is forwarded to the original subnets over CAPWAP tunnels.

- Only one CAPWAP tunnel exists. No CAPWAP tunnel exists between the AP and the standby AC. Information on the standby AC is synchronized from the HSB.