|  | HCIP-Datacom-Core Technology Lab environment Setup Guide |  |  |  |
| --- | --- | --- | --- | --- |
|  | Applicable to version 1.0 |  |  |  |
|  | Huawei Technologies Co., Ltd. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2020. All rights reserved. All contents of this book are protected by copyright laws. Huawei owns all copyrights, except the contents quoted from other parties. Without prior written permission from Huawei Technologies Co., Ltd., no person or organization shall copy, distribute, reproduce, store, or use any content in the information retrieval system in any manner, or use it for any other commercial purpose. All rights reserved. Trademark Declaration and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks or registered trademarks mentioned in this document are the property of their respective owners.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Headquarters, Bantian, Longgang District, Shenzhen Postcode: 518129 |
| Website: | https://e.huawei.com/en/talent/#/ |
| Email address: | certification@huawei.com |

Update History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 2020-05-21 | New version |

Document Information

| Prepared by | Applicable Time Range | Applicable to |
| --- | --- | --- |
| Talent Ecosystem Development Dept | 2020-05-21~... | All those who want to learn Huawei Datacom certification |

catalogue

1.	Foreword	4

2.	Physical topology of HCIP-Datacom-Core Technology	4

3.	Equipment Procurement	5

4.	Cable Connection	5

5.	Device Configuration	7

6.	Other Precautions	8

7.	Attachment	8

Foreword

This guide identifies the lab equipment and generates a list of the established lab equipment and physical connection diagrams.

This document provides guidance for regional divisions, suppliers, HALPs, and network academies to purchase devices, install modules, connect cables, and configure control devices, and construct a topology. Based on the HCIP-Datacom-Core Technology Lab Guide, complete the experiment of HCIP-Datacom-Core Technology.

Physical topology of HCIP-Datacom-Core Technology

Physical topology of HCIP-Datacom-Core Technology

: This topology is a physical topology that contains all devices in the device purchase list.

Equipment Procurement

For details about the equipment purchase list, see the attachment.

File name: HCIP-Datacom-Core Technology Lab Equipment List V1.0.xlsx

Purchase equipment according to the test requirements. If you have any questions, contact the following email address: certification@huawei.com.

Cable Connection

Connections must follow Figure 1. Table 1-1 lists the mapping between device names and models.

Mapping between device names and models

| Device Name | Device Model |
| --- | --- |
| R1 | AR6120 |
| R2 | AR6120 |
| R3 | AR6120 |
| R4 | AR6120 |
| R5 | AR6120 |
| S1 | S5731-H24T4XC |
| S2 | S5731-H24T4XC |
| S3 | S5731-H24P4XC |
| S4 | S5731-H24P4XC |
| S5 | S5731-H24T4XC |
| FW1 | USG6510E |

After the connection is complete, check the port connection according to the following table.

Interface connection table

| R1 | GE0/0/1 | S5 |
| --- | --- | --- |
|  | GE0/0/2 | R2 |
|  | GE0/0/3 | S1 |
|  | GE0/0/4 | S2 |
| R2 | GE0/0/1 | S5 |
|  | GE0/0/2 | R3 |
|  | GE0/0/3 | R1 |
|  | GE0/0/4 | S1 |
|  | GE0/0/5 | S2 |
| R3 | GE0/0/1 | S5 |
|  | GE0/0/2 | R4 |
|  | GE0/0/3 | R2 |
|  | GE0/0/4 | S1 |
|  | GE0/0/5 | S2 |
| R4 | GE0/0/1 | S5 |
|  | GE0/0/2 | R5 |
|  | GE0/0/3 | R3 |
|  | GE0/0/4 | S1 |
|  | GE0/0/5 | S2 |
| R5 | GE0/0/1 | S5 |
|  | GE0/0/2 | S1 |
|  | GE0/0/3 | R4 |
|  | GE0/0/4 | S2 |
| S1 | GE0/0/1 | R1 |
|  | GE0/0/2 | R2 |
|  | GE0/0/3 | R3 |
|  | GE0/0/4 | R4 |
|  | GE0/0/5 | R5 |
|  | GE0/0/10 | S2 |
|  | GE0/0/11 | S2 |
|  | GE0/0/12 | S3 |
|  | GE0/0/13 | S4 |
|  | GE0/0/14 | FW1 |
|  | GE0/0/15 | FW1 |
| S2 | GE0/0/1 | R1 |
|  | GE0/0/2 | R2 |
|  | GE0/0/3 | R3 |
|  | GE0/0/4 | R4 |
|  | GE0/0/5 | R5 |
|  | GE0/0/10 | S1 |
|  | GE0/0/11 | S1 |
|  | GE0/0/12 | S4 |
|  | GE0/0/13 | S3 |
| S3 | GE0/0/1 | S1 |
|  | GE0/0/2 | S2 |
|  | GE0/0/3 | S4 |
|  | GE0/0/4 | AP1 |
| S4 | GE0/0/1 | S2 |
|  | GE0/0/2 | S1 |
|  | GE0/0/3 | S3 |
|  | GE0/0/4 | AP2 |
| S5 | GE0/0/1 | R1 |
|  | GE0/0/2 | R2 |
|  | GE0/0/3 | R3 |
|  | GE0/0/4 | R4 |
|  | GE0/0/5 | R5 |
| FW1 | GE0/0/1 | S1 |
|  | GE0/0/2 | S1 |
| AP1 | GE0/0/1 | S3 |
| AP2 | GE0/0/1 | S4 |

Device Configuration

GE0/0/0 to GE0/0/7 on the AR6120 are Layer 2 interfaces by default. You can run the ‘undo portswitch’ command to switch the interfaces to Layer 3 interfaces.

Other Precautions

It is recommended that the trainer explain the experiment materials and then the students complete the experiment themselves. If troubleshooting is required, the trainer can participate in the experiment.

To prevent residual configurations from affecting the experiment, you are advised to clear the configuration information saved on the device before shutting down the device after the experiment is complete. At the beginning of the experiment, ensure that the device starts from the empty configuration. Otherwise, clear the configuration and restart the device.

Attachment