Huawei Certification Training

HCIP-Datacom-Network Automation Developer

Network Devices' Open Programmability

Lab Guide

V1.0

Huawei Technologies Co., Ltd.

Copyright © Huawei Technologies Co., Ltd. 2020. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://e.huawei.com/ |

Huawei Certification System

Huawei Certification follows the "platform + ecosystem" development strategy, which is a new collaborative architecture of ICT infrastructure based on "Cloud-Pipe-Terminal". Huawei has set up a complete certification system consisting of three categories: ICT infrastructure certification, platform and service certification, and ICT vertical certification. It is the only certification system that covers all ICT technical fields in the industry. Huawei offers three levels of certification: Huawei Certified ICT Associate (HCIA), Huawei Certified ICT Professional (HCIP), and Huawei Certified ICT Expert (HCIE). Huawei Certification covers all ICT fields and adapts to the industry trend of ICT convergence. With its leading talent development system and certification standards, it is committed to fostering new ICT talent in the digital era, and building a sound ICT talent ecosystem.

Huawei Certified ICT Professional-Datacom (HCIP-Datacom) is designed for professional engineers who are capable of network automation development. The HCIP-Datacom certification will qualify you as professionals in network automation development, who are capable of automatic deployment, development, and O&M of enterprise networks.

The Huawei certification system introduces the industry, fosters innovation, and imparts cutting-edge datacom knowledge.

About This Document

Introduction

This document is an HCIP-Datacom-Network Automation Developer certification training course and is intended for trainees who are going to take the HCIP-Datacom- Network Automation Developer exam or readers who want to understand network automation knowledge and practices.

Experiments

This lab guide contains the following experiments:

SSH experiment

Automatic SNMP configuration experiment

NETCONF configuration experiment

Configuration file comparison experiment

gRPC remote configuration query experiment

Telemetry configuration experiment

OPS experiment

Network flow analysis experiment

Required Background Knowledge

This lab guide is intended for network automation engineers, who are supposed to have the following knowledge and skills:

Python programming basics

Basic network knowledge and Huawei switch configurations

Lab Environment

Description

This lab environment is based on Huawei switch devices of VRP8, development environment of Python 3.8, and compiler of Jupyter Notebook or Pycharm.

For the experiments, either physical devices or devices simulated on a network simulation platform can be used. This lab guide uses CE12800 as an example to how network automation based on Python.

Contents

About This Document	4

1 SSH Experiment	9

1.1 Background	9

1.1.1 Objectives	9

1.1.2 Environment Preparations	9

1.2 Paramiko SSH Login	10

1.2.1 Configuration Roadmap	10

1.2.2 Configurations	10

1.2.3 Complete Code and Code Execution Result	12

1.2.4 Code Explaination	14

1.3 Paramiko SFTP File Transfer	16

1.3.1 Configuration Roadmap	17

1.3.2 Configurations	17

1.3.3 Complete Code and Code Execution Result	18

1.3.4 Code Explaination	19

1.4 Quiz	20

2 Automatic SNMP Configuration Experiment	21

2.1 Background	21

2.1.1 Objectives	21

2.1.2 Environment Preparations	21

2.2 Configuration Roadmap	22

2.3 Complete Code and Code Execution Result	23

2.4 Code Explaination	25

2.5 Quiz	29

3 NETCONF Configuration Experiment	30

3.1 Background	30

3.1.1 Objectives	30

3.1.2 Environment Preparations	30

3.2 Configuration Roadmap	31

3.3 Complete Code and Code Execution Result	32

3.4 Code Explaination	35

3.5 Quiz	39

4 Configuration File Comparison Experiment	40

4.1 Background	40

4.1.1 Objectives	40

4.1.2 Environment Preparations	40

4.2 Complete Code and Code Execution Result	41

4.3 Code Explaination	44

4.4 Quiz	46

5 gRPC Remote Configuration Query Experiment	47

5.1 Background	47

5.1.1 Objectives	47

5.1.2 Environment Preparations	47

5.2 Configuration Roadmap	48

5.3 Configuration Procedure and Complete Code	48

5.3.1 Compiling the .proto File	48

5.3.2 Generating Client and Server Codes	49

5.3.3 Complete Server Code	49

5.3.4 Complete Client Code	50

5.4 Code Explaination	52

5.4.1 Server Code	52

5.4.2 Client Code	54

5.5 Quiz	54

6 Telemetry Configuration Experiment	55

6.1 Background	55

6.1.1 Objectives	55

6.1.2 Environment Preparations	55

6.2 Configuration Roadmap	56

6.3 Configuration Procedure and Complete Code	56

6.3.1 Configuring the Switch	56

6.3.2 Compiling the .proto File	57

6.3.3 Complete Code of Python Server	58

6.4 Code Explaination	61

6.5 Quiz	66

7 OPS Experiment	67

7.1 Background	67

7.1.1 Objectives	67

7.1.2 Environment Preparations	67

7.2 Configuration Roadmap	68

7.3 Configuration Procedure and Complete Code	68

7.3.1 Complete Code	68

7.3.2 Uploading Code	71

7.3.3 Execution Result	72

7.4 Code Explaination	74

7.5 Quiz	80

8 Network Flow Analysis Experiment	81

8.1 Background	81

8.1.1 Objectives	81

8.1.2 Environment Preparations	81

8.2 Configuration Roadmap	82

8.3 Configuration Procedure and Complete Code	82

8.3.1 Complete Code	82

8.3.2 Procedure	91

8.3.3 Execution Result	91

8.4 Code Explaination	92

8.5 Quiz	101

Reference Answers to Quiz	102

SSH Experiment

Background

A company deploys a CE12800 switch whose management IP address is 192.168.56.100/24. You will compile an automation script to capture the current configuration file of the device and upload and download the configuration file.

Objectives

In this experiment, you will learn the common methods of the Paramiko module, and be able to use SSH to log in to a specified device to implement automatic configuration and secure file transfer using SFTP.

Environment Preparations

In this experiment, you need to prepare a network device that communicates with the Python compilation environment at Layer 3. In this example, a local PC will set up the Python compilation environment.

Configure the CE12800 switch.

<HUAWEI>system-view immediately

Enter system view, return user view with return command.

[HUAWEI]sysname CE1

[CE1]interface Vlanif 1

[CE1-Vlanif1]ip add 192.168.56.100 24

[CE1-Vlanif1]quit

Verify connectivity between the switch and local PC.

Open the CMD window on the local PC and test connectivity between the local PC and CE1.

C:\Users\XXX>ping 192.168.56.100

Pinging 192.168.56.100 with 32 bytes of data:

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 2 ms, TTL= 255

Reply from 192.168.56.100: Bytes = 32, time = 4 ms, TTL = 255

Statistics on ping to 192.168.56.100:

Packets: 4 packets transmitted, 4 packets received, 0 packet loss (0.0%)

Estimated RTT (ms):

Minimum = 2 ms, maximum = 4 ms, average = 3 ms

The connection is successful.

Paramiko SSH Login

Paramiko is the most commonly used SSH module in Python. It implements secure remote command execution and file transfer in either password or public key mode.

This section uses RSA user authentication as an example to describe the configuration process that a client logs in to a server using Python Paramiko SSH

Configuration Roadmap

Configure an SSH server.

Configure STelnet. Specially, configure a management IP address, enable the STelnet function, and configure the user interface.

Configure users. Specially, create a local user and an SSH user, and configure service type and authentication mode for the users.

Configure a public key. Specially, add the public key generated by the client and allocate it to the user.

Configure an SSH client.

            - Create a key pair. Specifically, generate a public key and a private key locally.

Compile Python code.

Verify client login to the server.

Configurations

Before using a Python script to log in to a device through SSH, you need to create an SSH account and enable the STelnet function on the device.

Enable STelnet on the SSH server and configure the VTY user interface.

[SSH Server] stelnet server enable

[SSH Server] user-interface vty 0 4

[SSH Server-ui-vty0-4] authentication-mode aaa

[SSH Server-ui-vty0-4] protocol inbound ssh

[SSH Server-ui-vty0-4] user privilege level 3

[SSH Server-ui-vty0-4] quit

Create a local user, python, on the server, add the user to the administrator group, and configure service type for the user.

[SSH Server] aaa

[SSH Server-aaa] local-user python password irreversible-cipher Huawei12#$

[SSH Server-aaa] local-user python user-group manage-ug

[SSH Server-aaa] local-user python service-type ssh

[SSH Server-aaa] quit

Create an SSH user on the SSH server and configure authentication type and service type for the user.

[SSH Server] ssh user python

[SSH Server] ssh user python authentication-type rsa

[SSH Server] ssh user python service-type stelnet

On the client, create an RSA key pair using Git Bash and copy the public key.

In this step, press Enter to retain the default value.

exampleuser@exampleuser MINGW64 ~

$ ssh-keygen -t rsa

Generating public/private rsa key pair.

Enter file in which to save the key (/c/Users/exampleuser/.ssh/id_rsa):

Enter passphrase (empty for no passphrase):

Enter same passphrase again:

Your identification has been saved in /c/Users/exampleuser/.ssh/id_rsa

Your public key has been saved in /c/Users/exampleuser/.ssh/id_rsa.pub

Display the public key.

$ cat /c/Users/exampleuser/.ssh/id_rsa.pub

Add the public key to the SSH server and assign it to the user.

[SSH Server] rsa peer-public-key rsa01 encoding-type openssh

[SSH Server-rsa-public-key] public-key-code begin

[SSH Server-rsa-public-key-rsa-key-code] ssh-rsa AAAAB3NzaC1yc2EAAAADAQABAAABgQDwLRx8MmuNs500dRemhFHdDbBmxco8Bp+wyqwaGuHJZBCjyFQV6AB+ezu5t0eWE3mw57IZfgmvR+MjBcliZv/x3l8oUMLcQKlKslYQDtvfUCZd+za1suXAPB/dyPKMhYPAzSDA7K+xqCWlmU3q06vxHEPLMv4A5IX54rKtBnK92fWjl9ACU+ak0ZlHxbKwOFn1tr0GJBazcInEs9DKGwkTTqJdu9+5hI5NxXTSbM3an53805ZbCU18xPy57g7MZC89vbdsag/uvQmFkLJ3arts/Om2R7fhR92EU/SNPmVy+qDEdwZEVdubdqJInW+8zzVkPGlnb2oH5hwH78Ksklbxb0fEfmGR0mS1ZAi3ZHUGcEEjuFZona3+5Z0Un2OPxfXwvoljVDusbYcugJHo9Ssurz05GzVuamQZlcO2JYY6FhtLUAImtXGQ80MpTjB0lcprkAZCib8agYOtVQNTZ7iB0g2EcBN9UTyMz7sh8RtrBDj445r+XPaDE8LmpDRKHMk=

[SSH Server-rsa-public-key-rsa-key-code] public-key-code end

[SSH Server-key-code] peer-public-key end

[SSH Server] ssh user python assign rsa-key rsa01

Compile and run the Python code to log in to the server in SSH mode.

----End

Complete Code and Code Execution Result

The Python script calls the Paramiko module to log in to CE1, runs the display current-configuration command, and displays the command output.

Complete code:

import paramiko

import time

ssh = paramiko.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname='192.168.56.100',port=22,username='python',key_filename=r'C:\Users\exampleuser\.ssh\id_rsa')

cli = ssh.invoke_shell()

cli.send('screen-length 0 temporary\n')

cli.send('display cu\n')

time.sleep(3)

dis_cu = cli.recv(999999).decode()

print(dis_cu)

ssh.close()

Code execution on the compiler:

Output:

Info: The max number of VTY users is 5, the number of current VTY users online is 1, and total number of terminal users online is 2.

The current login time is 2020-05-12 11:04:56.

<SSH Server>screen-length 0 temporary

Info: The configuration takes effect on the current user terminal interface only.

<SSH Server>display cu

!Software Version V200R005C10SPC607B607

!Last configuration was updated at 2020-05-12 10:46:40+00:00

!Last configuration was saved at 2020-05-12 10:46:42+00:00

#

sysname SSH Server

#

device board 17 board-type CE-MPUB

device board 1 board-type CE-LPUE

#

rsa peer-public-key rsa01 encoding-type openssh

public-key-code begin

ssh-rsa AAAAB3NzaC1yc2EAAAADAQABAAABgQDwLRx8MmuNs500dRemhFHdDbBmxco8Bp+wyqwaGuHJ

ZBCjyFQV6AB+ezu5t0eWE3mw57IZfgmvR+MjBcliZv/x3l8oUMLcQKlKslYQDtvfUCZd+za1suXAPB/d

yPKMhYPAzSDA7K+xqCWlmU3q06vxHEPLMv4A5IX54rKtBnK92fWjl9ACU+ak0ZlHxbKwOFn1tr0GJBaz

cInEs9DKGwkTTqJdu9+5hI5NxXTSbM3an53805ZbCU18xPy57g7MZC89vbdsag/uvQmFkLJ3arts/Om2

R7fhR92EU/SNPmVy+qDEdwZEVdubdqJInW+8zzVkPGlnb2oH5hwH78Ksklbxb0fEfmGR0mS1ZAi3ZHUG

cEEjuFZona3+5Z0Un2OPxfXwvoljVDusbYcugJHo9Ssurz05GzVuamQZlcO2JYY6FhtLUAImtXGQ80Mp

TjB0lcprkAZCib8agYOtVQNTZ7iB0g2EcBN9UTyMz7sh8RtrBDj445r+XPaDE8LmpDRKHMk= rsa-key

public-key-code end

peer-public-key end

#

aaa

local-user python password irreversible-cipher $1c$'&*wWaA2L3$iU"Y!^^Y}VJFkK=_E%)H({;pWU!Zr6]NO<LyLS,0$

local-user python service-type ssh

local-user python user-group manage-ug

#

authentication-scheme default

#

authorization-scheme default

#

accounting-scheme default

#

domain default

#

domain default_admin

#

interface Vlanif1

ip address 192.168.56.100 255.255.255.0

#

interface MEth0/0/0

undo shutdown

#

interface GE1/0/0

undo shutdown

#

interface GE1/0/1

undo shutdown

#

interface GE1/0/2

shutdown

#

interface GE1/0/3

shutdown

#

interface GE1/0/4

shutdown

#

interface GE1/0/5

shutdown

#

interface GE1/0/6

shutdown

#

interface GE1/0/7

shutdown

#

interface GE1/0/8

shutdown

#

interface GE1/0/9

shutdown

#

interface NULL0

#

stelnet server enable

ssh user python

ssh user python authentication-type rsa

ssh user python assign rsa-key rsa01

ssh user python service-type stelnet

ssh authorization-type default root

#

ssh server cipher aes256_gcm aes128_gcm aes256_ctr aes192_ctr aes128_ctr aes256_cbc aes128_cbc 3des_cbc

#

ssh server dh-exchange min-len 1024

#

ssh client cipher aes256_gcm aes128_gcm aes256_ctr aes192_ctr aes128_ctr aes256_cbc aes128_cbc 3des_cbc

#

user-interface con 0

#

user-interface vty 0 4

authentication-mode aaa

user privilege level 3

protocol inbound ssh

#

vm-manager

#

return

<SSH Server>

----End

Code Explaination

Import modules.

import paramiko

import time

Import the Paramiko and time modules required in the code segment. If the modules have not been installed, run the pip install paramiko command to install them.

This section describes the common classes and methods used by Paramiko as a client, for example, SSHClient as a common class and AutoAddPolicy, connect, invoke_shell, and close as methods related to the class. For more information about Paramiko, see http://docs.paramiko.org/.

By default, Python executes all code in sequence without any interval. When Paramiko sends configuration commands to the switch, the SSH response may be not as timely as usual or incomplete command output may be displayed. In this case, you can use the sleep method in the time module to manually suspend the program.

Instantiate an SSH object.

Use Paramiko SSHClient() to instantiate an SSH object. In this example, the value ssh is assigned.

ssh = paramiko.SSHClient()

Allow connections to unknown hosts. That is, you do not need to enter yes or no for confirmation when setting up a new SSH connection.

ssh.set_missing_host_key_policy(paramiko.client.AutoAddPolicy())

Establish an SSH session connection. Specifically, set the IP address of the destination SSH server to 192.168.56.100, user name as python, and key_filename as the local private key file so that the user will be authenticated in key mode.

ssh.connect(hostname='192.168.56.100',username='python',key_filename=r'C:\Users\exampleuser\.ssh\id_rsa')

Open an interactive session.

After logging in to the device through SSH, use the Python script to run commands on the SSH server.

Invoke invoke_shell() to assign a value to cli. invoke_shell() opens an interactive shell session, which is a logical channel established on an SSH session connection.

cli = ssh.invoke_shell()

Run the send() command to continue to enter commands.

Screen-length 0 temporary displays all the command output in one screen instead of split screens. display cu is short for display current-configuration.

time.sleep() sets the wait time (in seconds).

cli.send('screen-length 0 temporary\n')

cli.send('display cu\n')

time.sleep(3)

Capture the channel output information.

A logical channel has been created using invoke_shell(). All input and output information will be saved in this channel. All information in this channel can be captured and displayed on the Python compiler.

dis_cu = cli.recv(999999).decode()

print (dis_cu)

Invoke cli.recv(), use decode() to decode it, and assign a value to dis_cu.

recv(999999) receives data over the channel, 999,999 bytes to the maximum.

decode() decodes the bytes object in the specified encoding format, which is UTF-8 by default.

The decoding function is to facilitate reading, as the result is displayed in a new line on the GUI.

Info: The max number of VTY users is 5, the number of current VTY users online is 2, and total number of terminal users online is 3.

The current login time is 2019-11-05 16:43:26.

The last login time is 2019-11-05 16:43:09 from 192.168.56.1 through SSH.

<CE1>screen-length 0 temporary

Info: The configuration takes effect on the current user terminal interface only.

<CE1>display cu

!Software Version V200R005C10SPC607B607

!Last configuration was updated at 2019-11-05 14:33:28+00:00

#

Otherwise, the following information is displayed:

nInfo: The max number of VTY users is 5, the number of current VTY users online is 2, and total number of terminal users online is 3.\r\n      The current login time is 2019-11-05 16:45:34.\r\n      The last login time is 2019-11-05 16:43:26 from 192.168.56.1 through SSH.\r\n<CE1>screen-length 0 temporary\r\nInfo: The configuration takes effect on the current user terminal interface only.\r\n<CE1>display cu\r\n!Software Version V200R005C10SPC607B607\r\n!Last configuration was updated at 2019-11-05 14:33:28+00:00\r\n#

Close a session connection.

ssh.close()

Invoke close() to close the current session connection. As the number of VTY connections on the device is limited, close the SSH session after the script is executed.

----End

Paramiko SFTP File Transfer

SSH File Transfer Protocol (SFTP) is a secure file transfer protocol based on SSH.

This section uses RSA user authentication as an example to describe how to use Python Paramiko SFTP to upload and download files

Configuration Roadmap

Configure the SSH server.

Configure SFTP for the device. Specifically, configure the management IP address and enable SFTP on the device.

Configure users. Specifically, create an SSH user and configure service type, authentication mode, and SFTP path for the user.

Configure a public key. Specifically, add the public key generated by the client and allocate it to the user.

Verify the result by checking the uploaded files.

Configure the SSH client.

            - Create a key pair. Specifically, generate a public key and a private key locally.

Compile Python code.

Verify the result by checking the downloaded files.

Configurations

Before starting SFTP file transfer by running a Python script, you need to create an SFTP account and enable SFTP on the device.

Enable the SFTP server function.

[SFTP Server] sftp server enable

Create user python and configure authentication type and service type for the user.

[SFTP Server] ssh user python

[SFTP Server] ssh user python authentication-type rsa

[SFTP Server] ssh user python service-type sftp

[SFTP Server] ssh user python sftp-directory cfcard:

[SFTP Server] ssh authorization-type default root

On the client, create an RSA key pair using Git Bash and copy the public key.

exampleuser@exampleuser MINGW64 ~

$ ssh-keygen -t rsa

Generating public/private rsa key pair.

Enter file in which to save the key (/c/Users/exampleuser/.ssh/id_rsa):

Enter passphrase (empty for no passphrase):

Enter same passphrase again:

Your identification has been saved in /c/Users/exampleuser/.ssh/id_rsa

Your public key has been saved in /c/Users/exampleuser/.ssh/id_rsa.pub

Copy the displayed public key.

$ cat /c/Users/exampleuser/.ssh/id_rsa.pub

Add the copied public key to the server and assign the public key to the user.

[SFTP Server] rsa peer-public-key rsa01 encoding-type openssh

[SFTP Server-rsa-public-key] public-key-code begin

[SFTP Server-rsa-public-key-rsa-key-code] ssh-rsa AAAAB3NzaC1yc2EAAAADAQABAAABgQDwLRx8MmuNs500dRemhFHdDbBmxco8Bp+wyqwaGuHJZBCjyFQV6AB+ezu5t0eWE3mw57IZfgmvR+MjBcliZv/x3l8oUMLcQKlKslYQDtvfUCZd+za1suXAPB/dyPKMhYPAzSDA7K+xqCWlmU3q06vxHEPLMv4A5IX54rKtBnK92fWjl9ACU+ak0ZlHxbKwOFn1tr0GJBazcInEs9DKGwkTTqJdu9+5hI5NxXTSbM3an53805ZbCU18xPy57g7MZC89vbdsag/uvQmFkLJ3arts/Om2R7fhR92EU/SNPmVy+qDEdwZEVdubdqJInW+8zzVkPGlnb2oH5hwH78Ksklbxb0fEfmGR0mS1ZAi3ZHUGcEEjuFZona3+5Z0Un2OPxfXwvoljVDusbYcugJHo9Ssurz05GzVuamQZlcO2JYY6FhtLUAImtXGQ80MpTjB0lcprkAZCib8agYOtVQNTZ7iB0g2EcBN9UTyMz7sh8RtrBDj445r+XPaDE8LmpDRKHMk=

[SFTP Server-rsa-public-key-rsa-key-code] public-key-code end

[SFTP Server-key-code] peer-public-key end

[SFTP Server] ssh user python assign rsa-key rsa01

The client compiles and runs Python code, and uses SFTP to upload files to and download files from the server.

----End

Complete Code and Code Execution Result

The Python script invokes the Paramiko module to log in to CE1, downloads the configuration file vrpcfg.cfg, and uploads the configuration file test.cfg.

Complete code:

import paramiko

key=paramiko.RSAKey.from_private_key_file(r'C:\Users\exampleuser\.ssh\id_rsa')

tran = paramiko.Transport(('192.168.56.100', 22))

tran.connect(username=‘python’, pkey=key)

sftp = paramiko.SFTPClient.from_transport(tran)

local_path=r'C:\Users\exampleuser\.ssh\vrptest.cfg'

remote_path= '/vrpcfg.cfg'

sftp.get(remote_path, local_path)

sftp.put(local_path,’/test.cfg’)

tran.close()

Code execution on the compiler:

Result verification:

Get the downloaded file vrptest.cfg from the specified local path (C:\Users\exampleuser\.ssh\vrptest.cfg in this example) on the client.

Run the dir command on the server. The test.cfg file has been uploaded.

----End

Code Explaination

Import the module.

import paramiko

Import the paramiko module to be used in the code segment. If the module has not been installed, run the pip install paramiko command to install it.

SFTP involves the Transport, Key Handling, and SFTPClient classes and their methods.

The Transport class instantiates session channels and establishes session connections.

The Key handling class instantiates key objects.

The SFTPClient class creates an SFTP session connection and performs remote file operations.

Create an RSA key object.

Read the local RSA private key file on the client and assign a value to the key.

key=paramiko.RSAKey.from_private_key_file(r'C:\Users\exampleuser\.ssh\id_rsa')

Instantiate a session channel. The destination SSH server IP address is 192.168.56.100 and the port number is 22.

tran = paramiko.Transport(('192.168.56.100', 22))

Set up an SSH session connection. Set the user name to python, use pkey to specify a key object, and use the key for user authentication.

tran.connect(username=‘python’, pkey=key)

Create an SFTP channel based on the opened session connection and assign a value to sftp.

sftp = paramiko.SFTPClient.from_transport(tran)

Set the local path and remote path

local_path=r'C:\Users\exampleuser\.ssh\vrptest.cfg'

remote_path='/vrpcfg.cfg'

Rename the vrpcfg.cfg file vrptest.cfg.

Download the file.

sftp.get(remote_path, local_path)

Upload the file.

sftp.put(local_path,’/test.cfg’)

Close the session connection.

tran.close()

----End

Quiz

How to use SSH to log in to multiple devices and view their configurations?

SNMP Experiment

Background

A company deploys a CE12800 switch whose management IP address is 192.168.56.100/24. You can remotely log in to the switch using SSH. An SNMP configuration script, snmp.txt, is available. You will compile an automation script to configure SNMP and then collect SNMP information of the device.

Objectives

Master the method of using open to read and write files.

Master the common methods of using the paramiko module as the SSH client.

Master the method of using pysnmp to collect SNMP information of devices.

Environment Preparations

In this experiment, you need to prepare a network device that communicates with the Python compilation environment at Layer 3.

Configure the CE12800 switch.

<HUAWEI>system-view immediately

Enter system view, return user view with return command.

[HUAWEI]sysname CE1

[CE1]interface Vlanif 1

[CE1-Vlanif1]ip add 192.168.56.100 24

[CE1-Vlanif1]quit

Verify connectivity between the switch and local PC.

Open the CMD window on the local PC and test connectivity between the local PC and CE1.

C:\Users\XXX>ping 192.168.56.100

Pinging 192.168.56.100 with 32 bytes of data:

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 2 ms, TTL= 255

Reply from 192.168.56.100: Bytes = 32, time = 4 ms, TTL = 255

Ping statistics for 192.168.56.100:

Packets: 4 packets transmitted, 4 packets received, 0 packet loss (0.0%)

Estimated RTT (ms):

Minimum = 2 ms, maximum = 4 ms, average = 3 ms

The connection is successful.

SSH has been configured on CE1. The user name is python and the password is Huawei12#$. (For details, see section ‎1.2.2 .)

Prepare the SNMP script, that is, snmp.txt file.

In the SNMPv3 configuration script, the SNMP user name is admin and the password is Huawei@123. The complete script is as follows:

snmp-agent usm-user v3 admin group dc-admin

snmp-agen usm v3 admin au sha

Huawei@123

Huawei@123

snmp-a usm-user v3 admin pr aes128

Huawei@123

Huawei@123

snmp-agent trap source ME0/0/0

snmp-agent mib-view included nt iso

snmp-agent mib-view included rd iso

snmp-agent mib-view included wt iso

snmp-agent mib-view included iso-view iso

snmp-agent group v3 dc-admin privacy read-view rd write-view wt notify-view nt

Configuration Roadmap

Prepare the lab environment.

Invoke paramiko to log in to the device.

Invoke open to open the local file snmp.txt and configure the device based on paramiko.

Invoke pysnmp to get the host name based on the device SNMP information.

Complete Code and Code Execution Result

Complete code:

import paramiko

import time

from pysnmp.hlapi import *

# Switch Info

ip = '192.168.56.100'

username='python'

password='Huawei12#$'

# SSH login

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.client.AutoAddPolicy())

ssh.connect(hostname=ip,port=22,username=username,password=password)

print(ip+' login succesfully')

# Open a channel and enter the configuration.

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

# Run the following command to go to the system view:

cli.send('system-view immediately\n')

time.sleep(0.5)

# Read the snmp.txt file in the same local folder line by line and write the file to the SSH channel.

f = open('snmp.txt','r')

snmp_config_list = f.readlines()

for i in snmp_config_list:

cli.send(i)

time.sleep(0.5)

# Set up an SNMP channel.

UdpTransportTarget((ip,161))

g = getCmd(SnmpEngine(),

# Obtain the host name of the device.

UsmUserData('admin','Huawei@123','Huawei@123',authProtocol=usmHMACSHAAuthProtocol,privProtocol=usmAesCfb128Protocol),

UdpTransportTarget((ip, 161)),

ContextData(),

ObjectType(ObjectIdentity('SNMPv2-MIB','sysName',0)))

errorIndication, errorStatus, errorIndex, varBinds =next(g)

for i in varBinds:

print (i)

print (str(i).split('=')[1].strip())

dis_this = cli.recv(999999).decode() # View the script interaction process.

print (dis_this)

# Close the session.

ssh.close()

Code execution on the compiler:

Output:

192.168.56.100 login succesfully

SNMPv2-MIB::sysName.0 = CE1

CE1

Warning: The initial password poses security risks.

The password needs to be changed. Change now? [Y/N]:N

Info: The max number of VTY users is 5, the number of current VTY users online is 1, and total number of terminal users online is 2.

The current login time is 2019-11-05 19:43:51.

The last login time is 2019-11-05 19:43:07 from 192.168.56.1 through SSH.

<CE1>screen-length 0 temporary

Info: The configuration takes effect on the current user terminal interface only.

<CE1>system-view immediately

Enter system view, return user view with return command.

[CE1]snmp-agent usm-user v3 admin group dc-admin

[CE1]snmp-agen usm v3 admin authentication-mode sha

Please configure the authentication password (8-255)

Enter Password:

Confirm Password:

Warning: The privacy and authentication passwords are the same, which is insecure. It is recommended that the privacy and authentication passwords be different.

[CE1]snmp-a usm-user v3 admin privacy-mode aes128

Please configure the privacy password (8-255)

Enter Password:

Confirm Password:

Warning: The privacy and authentication passwords are the same, which is insecure. It is recommended that the privacy and authentication passwords be different.

[CE1]snmp-agent trap source  Vlanif 1

[CE1]snmp-agent mib-view included nt iso

[CE1]snmp-agent mib-view included rd iso

[CE1]snmp-agent mib-view included wt iso

[CE1]snmp-agent mib-view included iso-view iso

[CE1]snmp-agent group v3 dc-admin privacy read-view rd write-view wt notify-view  nt

[CE1]

----End

Code Explaination

Import modules.

import paramiko

import time

from pysnmp.hlapi import *

Import all modules used by this script. If the pysnmp module has not been installed, run the pip install pysnmp command to install it.

The pysnmp module collects SNMP information. This section describes the Get method for synchronizing SNMP to obtain device information. For more information, see http://snmplabs.com/pysnmp/.

Log in to the device.

# Switch Info

ip = '192.168.56.100'

username='python'

password='Huawei12#$'

Create variables ip, username, and password to indicate the SSH host, user name, and password, respectively.

# SSH login

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.client.AutoAddPolicy())

ssh.connect(hostname=ip,port=22,username=username,password=password)

print(ip+' login succesfully')

Invoke paramiko to log in to the device. If the login is successful, the host IP address login successfully is returned.

192.168.56.100 login succesfully

Enable the SSH channel.

Enable the SSH channel and perform basic configurations.

# Open a channel and enter the configuration.

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

# Run the following command to go to the system view:

cli.send('system-view immediately\n')

time.sleep(0.5)

Invoke invoke_shell() to enable the SSH channel. Run the following commands to cancel the split-screen display of command output and go to the system view. The commands are executed at an interval of 0.5 seconds. The CLI interaction process is as follows:

<CE1>screen-length 0 temporary

Info: The configuration takes effect on the current user terminal interface only.

<CE1>system-view immediately

Enter system view, return user view with return command.

Configure SNMP on the device.

Set SNMP parameters. Specifically, set the SNMP user name to admin, password to Huawei@123, authentication mode to SHA, and privacy mode to AES128.

# Read the snmp.txt file in the same local folder line by line and write the file to the SSH channel.

f = open('snmp.txt','r')

snmp_config_list = f.readlines()

for i in snmp_config_list:

cli.send(i)

time.sleep(0.5)

Invoke the built-in open function of Python to read snmp.txt to form a list and assign the value to snmp_config_list.

Use the “for” loop to read each line of command and write the command to the SSH channel.

The CLI interaction process is as follows:

[CE1]snmp-agent usm-user v3 admin group dc-admin

[CE1]snmp-agen usm v3 admin authentication-mode sha

Please configure the authentication password (8-255)

Enter Password:

Confirm Password:

Warning: The privacy and authentication passwords are the same, which is insecure. It is recommended that the privacy and authentication passwords be different.

[CE1]snmp-a usm-user v3 admin privacy-mode aes128

Please configure the privacy password (8-255)

Enter Password:

Confirm Password:

Warning: The privacy and authentication passwords are the same, which is insecure. It is recommended that the privacy and authentication passwords be different.

[CE1]snmp-agent trap source Vlanif 1

[CE1]snmp-agent mib-view included nt iso

[CE1]snmp-agent mib-view included rd iso

[CE1]snmp-agent mib-view included wt iso

[CE1]snmp-agent mib-view included iso-view iso

[CE1]snmp-agent group v3 dc-admin privacy read-view rd write-view wt notify-view  nt

[CE1]

Set up an SNMP connection with the device.

# Set up an SNMP channel.

UdpTransportTarget((ip,161))

Invoke UdpTransportTarget() in pysnmp. The parameters include the destination IP address and port number.

For details about more parameters, see http://snmplabs.com/pysnmp/docs/api-reference.html#synchronous-snmp.

Enable SNMP GET request and response.

The pysnmp module is used to implement the GET operation of SNMP to obtain the device information.

g = getCmd(SnmpEngine(),

UsmUserData('admin','Huawei@123','Huawei@123',authProtocol=usmHMACSHAAuthProtocol,privProtocol=usmAesCfb128Protocol),

UdpTransportTarget((ip, 161)),

ContextData(),

ObjectType(ObjectIdentity('SNMPv2-MIB','sysName',0)))

Invoke getCmd() to implement the GET operation of SNMP and assign the value to g.

UsmUserData indicates SNMP user information, including SNMP user name, password, encryption mode, and authentication mode. The SNMP user information must be consistent with the SNMP configurations on the device.

UdpTransportTarget is the transport layer information.

ContextData is used in asynchronous mode. Leave this parameter empty.

ObjectType indicates the device MIB object to be queried. You can use either the object name or OID. The object name is used here.

You can query the MIB information of Huawei devices at:

https://support.huawei.com/onlinetoolsweb/infoM/index.do?domain=1&lang=en&topicType=mib

The name of the MIB object file to be queried is SNMPv2-MIB.

The object is sysName.

errorIndication, errorStatus, errorIndex, varBinds = next(g)

Create variables errorIndication, errorStatus, errorIndex, and varBinds to obtain the returned information of next(g).

For details about the returned information, see the official document at http://snmplabs.com/pysnmp/docs/hlapi/asyncore/sync/manager/cmdgen/getcmd.html. You can customize the variable names

The varBinds variable is a tuple, including the returned message in an SNMP query.

Obtain the host name of device.

Process SNMP response data to separate the host name.

for i in varBinds:

print (i)

print (str(i).split('=')[1].strip())

The complete information in the varBinds tuple is as follows:

SNMPv2-MIB::sysName.0 = CE1

The returned message contains the MIB file, MIB object, ID, and host name.

Use the split and strip methods to obtain a single host name.

Close the session.

dis_this = cli.recv(999999).decode() # View the script interaction process.

print (dis_this)

# Close the session.

ssh.close()

Check the SSH channel interaction process and close the session.

----End

Quiz

How to encapsulate a host name query into a function for other programs to invoke?

How to improve the SNMP query efficiency if there are a large number of devices on the live network?

NETCONF Experiment

Background

A company deploys a CE12800 switch whose management IP address is 192.168.56.100/24. You can remotely log in to the switch using SSH. As delivering configurations through SSH is inefficient, you will deliver configurations through NETCONF to devices.

Objectives

Master the basic usage of paramiko.

Master the basic methods of ncclient.

Environment Preparations

In this experiment, you need to prepare a network device that communicates with the Python compilation environment at Layer 3.

Configure the CE12800 switch.

<HUAWEI>system-view immediately

Enter system view, return user view with return command.

[HUAWEI]sysname CE1

[CE1]interface Vlanif 1

[CE1-Vlanif1]ip add 192.168.56.100 24

[CE1-Vlanif1]quit

Verify connectivity between the switch and local PC.

Open the CMD window on the local PC and test connectivity between the local PC and CE1.

C:\Users\XXX>ping 192.168.56.100

Pinging 192.168.56.100 with 32 bytes of data:

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 2 ms, TTL= 255

Reply from 192.168.56.100: Bytes = 32, time = 4 ms, TTL = 255

Ping statistics for 192.168.56.100:

Packets: 4 packets transmitted, 4 packets received, 0 packet loss (0.0%)

Estimated RTT (ms):

Minimum = 2 ms, maximum = 4 ms, average = 3 ms

The connection is successful.

SSH has been configured on CE1. The user name is python and the password is Huawei12#$. (For details, see section ‎1.2.2 )

Prepare the NETCONF pre-configuration script, netconf.txt.

Create a NETCONF user with the user name as netconf and password as Huawei12#$. The complete script is as follows:

system-view immediately

aaa

local-user netconf password irreversible-cipher Huawei12#$

local-user netconf service-type ssh

local-user netconf level 3

quit

ssh user netconf authentication-type password

ssh user netconf service-type snetconf

snetconf server enable

netconf

protocol inbound ssh port 830

quit

Configuration Roadmap

Prepare the experiment environment.

Invoke paramiko to log in to the device.

Invoke open to open the local file netconf.txt and configure NETCONF for the device based on paramiko.

Invoke ncclient to configure the G1/0/2 interface through NETCONF. For the interface, the IP address is 192.168.2.1/24 and description is Config by NETCONF.

Complete Code and Code Execution Result

Complete code:

# -*- coding: utf-8 -*-

from ncclient import manager

from ncclient import operations

import paramiko

import time

# Device parameters

ip = '192.168.56.100'

ssh_user = 'python'

ssh_password = 'Huawei12#$'

netconf_port = '830'

netconf_user = 'netconf'

netconf_password = 'Huawei12#$'

filename='netconf.txt'

# Define the SSH class to configure NETCONF on the device.

class ssh():

def ssh_connect(ip,username,password):

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.client.AutoAddPolicy())

ssh.connect(hostname=ip,port=22,username=username,password=password)

print(ip+' login succesfully')

return ssh

def ssh_config(file,ip,username,password):

a = ssh.ssh_connect(ip,username,password)

cli = a.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

f = open(file,'r')

config_list = f.readlines()

for i in config_list:

cli.send(i)

time.sleep(0.5)

dis_this = cli.recv(999999).decode()

print (dis_this)

a.close()

# Define the huawei_connect function to establish a NETCONF connection.

def huawei_connect(host, port, user, password):

return manager.connect(host=host,

port=port,

username=user,

password=password,

hostkey_verify = False,

device_params={'name': "huawei"},

allow_agent = False,

look_for_keys = False)

# Use NETCONF to send XML data and configure an IP address for an interface on the device.

CREATE_INTERFACE = '''<config>

<ethernet xmlns="http://www.huawei.com/netconf/vrp" content-version="1.0" format-version="1.0">

<ethernetIfs>

<ethernetIf operation="merge">

<ifName>GE1/0/2</ifName>

<l2Enable>disable</l2Enable>

</ethernetIf>

</ethernetIfs>

</ethernet>

<ifm xmlns="http://www.huawei.com/netconf/vrp" content-version="1.0" format-version="1.0">

<interfaces>

<interface operation="merge">

<ifName>GE1/0/2</ifName>

<ifDescr>Config by NETCONF</ifDescr>

<ifmAm4>

<am4CfgAddrs>

<am4CfgAddr operation="create">

<subnetMask>255.255.255.0</subnetMask>

<addrType>main</addrType>

<ifIpAddr>192.168.2.1</ifIpAddr>

</am4CfgAddr>

</am4CfgAddrs>

</ifmAm4>

</interface>

</interfaces>

</ifm>

</config>'''

# Execute the main functions in sequence.

if __name__ == '__main__':

ssh.ssh_config(filename,ip,ssh_user,ssh_password)

m = huawei_connect(ip,netconf_port,netconf_user,netconf_password)

m.edit_config(target='running',config=CREATE_INTERFACE)

Code execution on the compiler:

Output:

Compilation result:

192.168.56.100 login succesfully

Warning: The initial password poses security risks.

The password needs to be changed. Change now? [Y/N]:N

Info: The max number of VTY users is 5, the number of current VTY users online is 1, and total number of terminal users online is 2.

The current login time is 2019-11-06 13:08:58.

The last login time is 2019-11-06 11:48:16 from 192.168.56.1 through SSH.

<CE1>screen-length 0 temporary

Info: The configuration takes effect on the current user terminal interface only.

<CE1>system-view immediately

Enter system view, return user view with return command.

[CE1]aaa

[CE1-aaa]local-user netconf password irreversible-cipher Huawei12#$

Info: A new user is added.

[CE1-aaa]local-user netconf service-type ssh

[CE1-aaa]local-user netconf level 3

[CE1-aaa]quit

[CE1]ssh user netconf authentication-type password

[CE1]ssh user netconf service-type snetconf

[CE1]snetconf server enable

Info: The SNETCONF server is already started.

[CE1]netconf

[CE1-netconf]protocol inbound ssh port 830

Info: The ssh port 830 service is already started.

[CE1-netconf]quit

[CE1]

Log in to CE1 to check the configurations:

[CE1]interface GE 1/0/2

[CE1-GE1/0/2]display this

#

interface GE1/0/2

undo portswitch

description Config by NETCONF

shutdown

ip address 192.168.2.1 255.255.255.0

#

return

[CE1-GE1/0/2]

NETCONF configurations are delivered successfully.

----End

Code Explaination

Import modules.

# -*- coding: utf-8 -*-

from ncclient import manager

from ncclient import operations

import paramiko

import time

Import all modules used by this script. If ncclient has not been installed, run the pip install ncclient command to install it.

ncclient is a NETCONF functional module of Python. This section briefly describes the ncclient manager and operation methods to establish and interact with NETCONF. For more information on ncclient, see https://ncclient.readthedocs.io/en/latest/.

Define device parameter variables.

# Device parameters

ip = '192.168.56.100'

ssh_user = 'python'

ssh_password = 'Huawei12#$'

netconf_port = '830'

netconf_user = 'netconf'

netconf_password = 'Huawei12#$'

filename='netconf.txt'

Define parameter variables of the device, including the host IP address, SSH user, SSH password, NETCONF port, NETCONF user name, NETCONF password, and local file name.

Perform NETCONF configurations on the device.

Use paramiko and open to send the local configuration script, netconf.txt, to CE1. In this example, the class encapsulation is used to facilitate invoking by the main function.

# Define the SSH class to configure NETCONF.

class ssh():

Declare the class name as ssh. The class contains two methods (functions), that is, ssh_connect() and ssh_config(), which are used to establish an SSH connection and send configurations, respectively.

def ssh_connect(ip,username,password):

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.client.AutoAddPolicy())

ssh.connect(hostname=ip,port=22,username=username,password=password)

print(ip+' login succesfully')

return ssh

Define the first method ssh_connect(ip,username,password) in the ssh class and enter the IP address, user name, and password of SSH. This function encapsulates the paramiko method. For details, see the preceding sections.

def ssh_config(file,ip,username,password):

a = ssh.ssh_connect(ip,username,password) # invoke ssh_connect to assign a value to a.

cli = a.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

f = open(file,'r')

config_list = f.readlines()

for i in config_list:

cli.send(i)

time.sleep(0.5)

dis_this = cli.recv(999999).decode()

print (dis_this)

a.close()

The second method in the ssh class is ssh_config(file,ip,username,password). Define four parameters, that is, IP address, user name, and password of SSH, and configuration file path.

ssh_config() invokes ssh_connect() to connect to the device and then sends configuration commands. Use the open function to open the local file, netconf.txt, and write data to the SSH channel line by line. Finally, check the process of interaction with the device and close the session.

Establish a NETCONF connection.

# Define the huawei_connect function to establish a NETCONF connection.

def huawei_connect(host, port, user, password):

return manager.connect(host=host,

port=port,

username=user,

password=password,

hostkey_verify = False,

device_params={'name': "huawei"},

allow_agent = False,

look_for_keys = False)

Define the huawei_connect(host, port, user, password) function. The four input parameters of the function are the IP address, port number, user name, and password of the NETCONF host. The function returns the manager.connect method of ncclient.

manager.connect is used to establish a NETCONF connection. The parameters are defined in RFC4741. device_params has two options for Huawei devices. That is, the value can be huawei or huaweiyang, corresponding to the IETF YANG model or Huawei YANG model, respectively.

Construct an XML configuration file.

Construct an XML configuration file by referring to the NETCONF Schema API Reference on the Huawei official website. https://support.huawei.com/enterprise/en/switches/cloudengine-12800-pid-7542409

# Use NETCONF to send XML data and configure an IP address for an interface on the device.

CREATE_INTERFACE = '''<config>

<ethernet xmlns="http://www.huawei.com/netconf/vrp" content-version="1.0" format-version="1.0">

<ethernetIfs>

<ethernetIf operation="merge">

<ifName>GE1/0/2</ifName>

<l2Enable>disable</l2Enable>

</ethernetIf>

</ethernetIfs>

</ethernet>

<ifm xmlns="http://www.huawei.com/netconf/vrp" content-version="1.0" format-version="1.0">

<interfaces>

<interface operation="merge">

<ifName>GE1/0/2</ifName>

<ifDescr>Config by NETCONF</ifDescr>

<ifmAm4>

<am4CfgAddrs>

<am4CfgAddr operation="create">

<subnetMask>255.255.255.0</subnetMask>

<addrType>main</addrType>

<ifIpAddr>192.168.2.1</ifIpAddr>

</am4CfgAddr>

</am4CfgAddrs>

</ifmAm4>

</interface>

</interfaces>

</ifm>

</config>'''

NETCONF uses XML files to send configurations. XML is a commonly used text format that allows you to nest and expand data. A complete NETCONF session consists of the transport layer, message layer, operation layer, and content layer. The current XML configuration file contains only the operation layer and content layer.

Basic NETCONF operations include get-config, get, edit-config, copy-config, delete-config, lock, unlock, close-session, and kill session. In this example, the operation layer information is edit-config, and the operation attribute is merge. This operation edits the target data, which may exist or not in the database. If the target data does not exist, target data is created; if the target data exists, the target data is edited.

The NETCONF content layer is used to edit specific parameters. In this example, disable the Layer 2 function on GE 1/0/2 by running the undo portswitch command, change the description to Config by NETCONF, and set the IP address to 192.168.2.1/24.

Run the main functions.

# Execute the main functions in sequence.

if __name__ == '__main__':

ssh.ssh_config(filename,ip,ssh_user,ssh_password)

m = huawei_connect(ip,netconf_port,netconf_user,netconf_password)

m.edit_config(target='running',config=CREATE_INTERFACE)

Run the main functions.

Run ssh.ssh_config(filename,ip,ssh_user,ssh_password) to invoke the ssh_config method in the ssh class. The input parameters are the variables defined in Step 2.

Then run huawei_connect(ip,netconf_port,netconf_user,netconf_password) to assign a value to m. Enter NETCONF parameters to establish a NETCONF connection.

Finally run m.edit_config(target='running',config=CREATE_INTERFACE). Use the edit_config method to send the XML configuration file constructed in Step 5 to the device. Now you can log in to the device to verify the result. The configurations are successfully delivered.

----End

Quiz

Why if has to be used to build a main function?

What is the difference between the get operation by NETCONF and the query operation by SNMP to obtain device information?

Configuration File Comparison Experiment

Background

A company has multiple network devices (one in this example). To better manage configurations, you need to use code to compare the configurations of each device and output the configuration comparison result.

Objectives

Master the difflib method for text comparison.

Master data processing by regular expressions.

Master device configurations by paramiko.

Environment Preparations

In this experiment, you need to prepare a network device that communicates with the Python compilation environment at Layer 3.

Configure the CE12800 switch.

<HUAWEI>system-view immediately

Enter system view, return user view with return command.

[HUAWEI]sysname CE1

[CE1]interface Vlanif 1

[CE1-Vlanif1]ip add 192.168.56.100 24

[CE1-Vlanif1]quit

Verify connectivity between the switch and local PC.

Open the CMD window on the local PC and test connectivity between the local PC and CE1.

C:\Users\XXX>ping 192.168.56.100

Pinging 192.168.56.100 with 32 bytes of data:

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 2 ms, TTL= 255

Reply from 192.168.56.100: Bytes = 32, time = 4 ms, TTL = 255

Ping statistics for 192.168.56.100:

Packets: 4 packets transmitted, 4 packets received, 0 packet loss (0.0%)

Estimated RTT (ms):

Minimum = 2 ms, maximum = 4 ms, average = 3 ms

The connection is successful.

SSH has been configured on CE1. The user name is python and the password is Huawei12#$. (For details, see section ‎1.2.2 .)

Complete Code and Code Execution Result

Complete code:

# -*- coding: utf-8 -*-

import paramiko

import time

import difflib

import re

# Device information

ip = '192.168.56.100'

username='python'

password='Huawei12#$'

# Define a function to obtain the current configurations.

def get_config(ip,username,password):

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname=ip,port=22,username=username,password=password)

print(ip+' login succesfully')

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

cli.send('display cu\n')

time.sleep(2)

dis_cu = cli.recv(999999).decode()

return (dis_cu)

ssh_client.close()

# Define the ssh_config function to write the script to the device.

def ssh_config(file,ip,username,password):

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname=ip,port=22,username=username,password=password)

print(ip+' login succesfully')

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

f = open(file,'r')

config_list = f.readlines()

for i in config_list:

cli.send(i)

time.sleep(0.5)

dis_this = cli.recv(999999).decode()

#print (dis_this)

ssh.close()

# Invoke get_config to assign a value to output.

output = get_config(ip,username,password)

#print (output)

# Process data and use regular expressions to obtain only configurations.

config = re.findall(r'(<CE1>display cu[\d\D]+<CE1>$)',output)

#print (config)

# Save the configurations to local file1.

with open(r'D:\Config\file1','w') as f:

f.writelines(config[0])

# Invoke ssh_config to write the netconf.txt configurations to the device.

ssh_config('netconf.txt',ip,username,password)

# Read the configurations again and save them to local file2.

output = get_config(ip,username,password)

config = re.findall(r'(<CE1>display cu[\d\D]+<CE1>$)',output)

with open(r'D:\Config\file2','w') as f:

f.writelines(config[0])

# Compare configurations.

d = difflib.HtmlDiff()

# Define a function to read files.

def read_file(filename):

try:

with open(filename,'r') as f:

return f.readlines()

except IOError:

print('%s The file is not found.'% filename)

sys.exit(1)

# Define the compare_files function to compare configurations and save the comparison as result.html.

def compare_files(file1,file2,out_file):

file1_content = read_file(file1)

file2_content = read_file(file2)

d = difflib.HtmlDiff()

result = d.make_file(file1_content,file2_content)

with open(r'D:\Config\result.html','w') as f:

f.writelines(result)

print ()

# Invoke compare_files.

compare_files(r'D:\Config\file1',r'D:\Config\file2',r'D:\Config\result.html')

Code execution on the compiler:

Output:

Compilation result:

192.168.56.100 login successfully

192.168.56.100 login successfully

192.168.56.100 login successfully

You will find the configuration files and comparison file in the directory.

Open the result.html file.

----End

Code Explaination

Import modules.

# -*- coding: utf-8 -*-

import paramiko

import time

import difflib

import re

Import all modules used by this script. If a module does not exist, run pip install [module name] to install the module.

In this example, the difflib module is used to compare texts. The re module is used to process data using regular expressions.

Define device parameter variables.

# Device information

ip = '192.168.56.100'

username='python'

password='Huawei12#$'

Define the parameter variables of the device, that is, host IP address, SSH user, and SSH password.

Define the get_config() function.

Define the get_config() function to obtain the current device configurations.

# Define a function to obtain the current configurations.

def get_config(ip,username,password):

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname=ip,port=22,username=username,password=password)

print(ip+' login succesfully')

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

cli.send('display cu\n')

time.sleep(2)

dis_cu = cli.recv(999999).decode()

return (dis_cu)

ssh_client.close()

In the get_config(ip,username,password) function, enter the IP address, user name, and password. Invoke paramiko in the function to log in to the device and run display current-configuration to collect the command output. Return the command output. For details about the paramiko method, see the experiment of device login by Paramiko.

Define the ssh_config() function.

Define the ssh_config() function to send the configuration script to the device.

# Define the ssh_config function to write the script to the device.

def ssh_config(file,ip,username,password):

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname=ip,port=22,username=username,password=password)

print(ip+' login succesfully')

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

# Read the file and write the content to the device line by line.

f = open(file,'r')

config_list = f.readlines()

for i in config_list:

cli.send(i)

time.sleep(0.5)

dis_this = cli.recv(999999).decode()

#    print (dis_this)

ssh.close()

----End

Quiz

Why is “192.168.56.100 login successfully” displayed for three times in the compilation result?

In this case, some scripts are redundant. Which part can be optimized?

Can the configuration files be saved and compared as scheduled? Is there any other way easier to get configuration files?

gRPC Remote Configuration Query Experiment

Background

A company deploys a CE12800 switch whose management IP address is 192.168.56.100. In a special requirement, the client needs to invoke the server through gRPC to query and return the current device configurations.

Objectives

Master the method of user-defining a .proto file.

Master gRPC server code compilation.

Master gRPC client code compilation.

Environment Preparations

Prepare a simulated environment using a simulator or prepare a real environment. Code is compiled on a local computer for the client and server.

For details about how to set up an environment, see Chapter ‎1 SSH Experiment.

SSH has been configured on CE1. The user name is python and the password is Huawei12#$.

Set the device management address to 192.168.56.100/24.

Create two Python files, one as the client and the other as the server, on Juypter Notebook or any other compiler.

Configuration Roadmap

Compile a .proto file. Define the services and methods of gRPC request and response.

Generate Python code for the client and server based on the .proto file.

Compile the server code.

Compile the client code.

Configuration Procedure and Complete Code

Compiling the .proto File

syntax = "proto3";

package get_config;

// The get_config service definition.

service get_config {

// RFC request and reply

rpc Login_info (Request) returns (Reply) {}

}

// The request message containing the login information.

message Request {

string host = 1;

string username = 2;

string password = 3;

}

// The response message containing the string reply.

message Reply {

string message = 1;

}

Save the file in get_config.proto format.

The get_config service is defined in the .proto file. For the Login_info() method, the input parameter Request contains three string parameters: host, username, and password. For the returned parameter Reply, there is only one string parameter, that is, message.

Generating Client and Server Codes

Install and run grpcio-tools to generate code. For details, see https://grpc.io/docs/tutorials/basic/python/.

Run the following command in the directory where the .proto file is saved:

C:\Users\Richard\HCIP\gRPC>python -m grpc_tools.protoc -I./ --python_out=. --grpc_python_out=. ./get_config.proto

After the command is executed, the client and server code is automatically generated.

Complete Server Code

from concurrent import futures

import time

import grpc

import get_config_pb2

import get_config_pb2_grpc

import paramiko

class Display_Config(get_config_pb2_grpc.get_configServicer):

# Invoke paramiko to log in to the device and obtain the current configurations.

def Login_info(self, request, context):

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname=request.host, port=22, username=request.username, password=request.password)

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

cli.send('display cu\n')

time.sleep(3)

data = cli.recv(999999).decode()

ssh.close()

# Return the configuration information in the command output.

return get_config_pb2.Reply(message=data)

def serve():

# Create the gRPC service.

server = grpc.server(futures.ThreadPoolExecutor(max_workers=10))

# Deploy the gRPC server in the defined service.

get_config_pb2_grpc.add_get_configServicer_to_server(Display_Config(),server)

# Start the server.

server.add_insecure_port('localhost:8080')

server.start()

_ONE_DAY_IN_SECONDS = 60 * 60 * 24

try:

while True:

time.sleep(_ONE_DAY_IN_SECONDS)

except KeyboardInterrupt:

server.stop()

if __name__ == "__main__":

serve()

Run the server and keep listening.

Complete Client Code

import grpc

import get_config_pb2

import get_config_pb2_grpc

def run():

# Instantiate stub on the client.

connect = grpc.insecure_channel('localhost:8080')

stub = get_config_pb2_grpc.get_configStub(channel=connect)

# Invoke the Login_info method of the server through stub.

response = stub.Login_info(get_config_pb2.Request(host='192.168.56.100',username='python',password='Huawei12#$'))

print (response.message)

if __name__ == "__main__":

run()

Run the client and enter the IP address, user name, and port number of the device of login. The server returns the following information:

Warning: The initial password poses security risks.

The password needs to be changed. Change now? [Y/N]:N

Info: The max number of VTY users is 5, the number of current VTY users online is 1, and total number of terminal users online is 2.

The current login time is 2020-01-31 15:58:01.

The last login time is 2020-01-31 15:57:20 from 192.168.56.1 through SSH.

<CE1>screen-length 0 temporary

Info: The configuration takes effect on the current user terminal interface only.

<CE1>display cu

!Software Version V200R005C10SPC607B607

!Last configuration was updated at 2020-01-31 15:31:54+00:00 by SYSTEM automatically

!Last configuration was saved at 2019-11-06 01:39:44+00:00

#

sysname CE1

#

device board 17 board-type CE-MPUB

device board 1 board-type CE-LPUE

#

aaa

local-user python password irreversible-cipher $1c$cYpPW3'A2I$t.+@L>Q;j,igRBE\EurN:4VsP|LeK%jBTK/]~r_>$

local-user python service-type ssh

local-user python level 3

#

authentication-scheme default

#

authorization-scheme default

#

accounting-scheme default

#

domain default

#

domain default_admin

#

interface Vlanif1

ip address 192.168.56.100 255.255.255.0

#

interface MEth0/0/0

undo shutdown

#

interface GE1/0/0

undo shutdown

#

interface GE1/0/1

shutdown

#

interface GE1/0/2

shutdown

#

interface GE1/0/3

shutdown

#

interface GE1/0/4

shutdown

#

interface GE1/0/5

shutdown

#

interface GE1/0/6

shutdown

#

interface GE1/0/7

shutdown

#

interface GE1/0/8

shutdown

#

interface GE1/0/9

shutdown

#

interface NULL0

#

snmp-agent

snmp-agent local-engineid 800007DB03707BE82F1330

#

snmp-agent sys-info version v3

snmp-agent group v3 dc-admin privacy read-view rd write-view wt notify-view nt

#

snmp-agent mib-view included nt iso

snmp-agent mib-view included rd iso

snmp-agent mib-view included wt iso

snmp-agent mib-view included iso-view iso

snmp-agent usm-user v3 admin

snmp-agent usm-user v3 admin group dc-admin

snmp-agent usm-user v3 admin authentication-mode sha cipher %^%#FQ]J>Ba"e*F{#/+rx$UUZ2^:BaG>V/LoKc8No%DE%^%#

snmp-agent usm-user v3 admin privacy-mode aes128 cipher %^%#BEj8L``EE,88ziP^'jFW5A+EB"uqg'wdaS+Y9$F.%^%#

#

snmp-agent trap source Vlanif1

#

stelnet server enable

ssh authorization-type default aaa

#

ssh server cipher aes256_gcm aes128_gcm aes256_ctr aes192_ctr aes128_ctr aes256_cbc aes128_cbc 3des_cbc

#

ssh server dh-exchange min-len 1024

#

ssh client cipher aes256_gcm aes128_gcm aes256_ctr aes192_ctr aes128_ctr aes256_cbc aes128_cbc 3des_cbc

#

user-interface con 0

#

user-interface vty 0 4

authentication-mode aaa

user privilege level 3

#

vm-manager

#

return

<CE1>

Process finished with exit code 0

Code Explaination

Server Code

To compile the gRPC server code, perform the following steps:

Invoke the service defined by .proto. The generated .py file has implemented the service, and the server only needs to invoke the service.

Compile the functional code of the server.

Run the server.

Create the subclass Display_Config(), which inherits the attributes and methods of the get_config_pb2_grpc class.

class Display_Config(get_config_pb2_grpc.get_configServicer):

The Login_info function defines the RPC request and reply.

def Login_info(self, request, context):

The structure of RPC request has been defined in protocol buffers, and it has three attributes, that is, host, username, and password. In this case, the server receives parameters transferred from the client, invoke the paramiko module, and execute the commands for logging in to the device through SSH and querying the current configurations. Finally, data is returned.

# Invoke paramiko to log in to the device and obtain the current configurations.

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname=request.host, port=22, username=request.username, password=request.password)

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

cli.send('screen-length 0 temporary\n')

time.sleep(0.5)

cli.send('display cu\n')

time.sleep(3)

data = cli.recv(999999).decode()

ssh.close()

# Return the configuration information in the command output.

return get_config_pb2.Reply(message=data)

Create the serve() function to instantiate the gRPC service, which is invoked and executed by the main function.

In this case, start port 8080 of the server and listen on this port for a whole day. Wait for the client to invoke the service.

def serve():

# Create the gRPC service.

server = grpc.server(futures.ThreadPoolExecutor(max_workers=10))

# Deploy the gRPC server in the defined service.

get_config_pb2_grpc.add_get_configServicer_to_server(Display_Config(),server)

# Start the server.

server.add_insecure_port('localhost:8080')

server.start()

_ONE_DAY_IN_SECONDS = 60 * 60 * 24

try:

while True:

time.sleep(_ONE_DAY_IN_SECONDS)

except KeyboardInterrupt:

server.stop()

if __name__ == "__main__":

serve()

Client Code

To compile the gRPC client code, perform the following steps:

Create stub to invoke services later.

Invoke service methods through stub.

- Create stub to connect to the server.

def run():

# Instantiate stub on the client.

connect = grpc.insecure_channel('localhost:8080')

stub = get_config_pb2_grpc.get_configStub(channel=connect)

Use stub to invoke the Login_info method of the server.

In this example, enter the IP address, user name, and password of the device for query. Then, the current device configurations are returned.

response = stub.Login_info(get_config_pb2.Request(host='192.168.56.100',username='python',password='Huawei12#$'))

print (response.message)

if __name__ == "__main__":

run()

In this experiment, the configuration query is executed on the server and the query result is returned to the client.

Quiz

What are the benefits of gRPC? Are there any other applications of gRPC in your mind?

Telemetry Experiment

Background

A company deploys a CE12800 switch whose management IP address is 192.168.56.100. To better collect device performance data, it is required that the device push CPU information to the server in Telemetry static subscription mode.

Objectives

Master server code compilation for static collection by Huawei Telemetry.

Master the common methods defined in a Huawei .proto file.

Environment Preparations

Prepare a simulated environment using a simulator or prepare a physical environment. Run the Python script on the local PC as the server (in this example, the IP address as 192.168.56.1). The switch pushes data to the server (in this example, the IP address of CE1 as 192.168.56.100).

Configure the CE12800 switch.

<HUAWEI>system-view immediately

Enter system view, return user view with return command.

[HUAWEI]sysname CE1

[CE1]interface Vlanif 1

[CE1-Vlanif1]ip add 192.168.56.100 24

[CE1-Vlanif1]quit

Verify connectivity between the switch and local PC.

Open the CMD window on the local PC and test connectivity between the local PC and CE1.

C:\Users\XXX>ping 192.168.56.100

Pinging 192.168.56.100 with 32 bytes of data:

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 2 ms, TTL= 255

Reply from 192.168.56.100: Bytes = 32, time = 4 ms, TTL = 255

Ping statistics for 192.168.56.100:

Packets: 4 packets transmitted, 4 packets received, 0 packet loss (0.0%)

Estimated RTT (ms):

Minimum = 2 ms, maximum = 4 ms, average = 3 ms

The connection is successful.

SSH has been configured on CE1. The user name is python and the password is Huawei12#$. (For details, see section ‎1.2.2 .)

Configuration Roadmap

Set up an environment.

Configure Telemetry static subscription on the device, including content to be collected, push object, and push interval.

Download the .proto file of Huawei device from Huawei official website. (Download the .proto file in the same way as you download the system software.) Visit Huawei enterprise technical support website (http://support.huawei.com/enterprise) or Huawei carrier technical support website (http://support.huawei.com/carrier) and search for the device model and version. Go to the software download page and obtain the .proto file of the required version. Compile the .proto file to obtain the method invoked by the server.

Compile the server code to listen on the specified port to obtain data.

Based on the data to be sent, select an appropriate method to decode the data.

Configuration Procedure and Complete Code

Configuring the Switch

Ensure that the switch and server are reachable at Layer 3, and configure Telemetry static subscription on the switch.

Go to the Telemetry view.

<CE1> system-view immediately

Enter system view, return user view with return command.

[CE1] telemetry

[CE1-telemetry]

Configure the push object on the device.

In this example, create a destination group, Dest1. The target IP address is 192.168.56.1 and the port number is 20000.

[CE1-telemetry] destination-group Dest1

[CE1-telemetry-destination-group-Dest1] ipv4-address 192.168.56.1 port 20000 protocol grpc no-tls

Configure sampling data on the device.

When you configure static telemetry subscription to the data sampled, you need to create a sampling sensor group and specify a sampling path.

In this example, the sampling group Sensor1 is created. The sampling path is CPU information.

[CE1-telemetry] sensor-group Sensor1

[CE1-telemetry-sensor-group-Sensor1] sensor-path huawei-devm:devm/cpuInfos/cpuInfo

Create a static subscription.

Create a subscription and associate the configured destination group with the sampling sensor group to send data. In this example, associate the destination group Dest1 with the sensor group Sensor1, and set the sampling interval to 1,000 ms.

[CE1-telemetry]subscription Sub1

[CE1-telemetry-subscription-Sub1]destination-group Dest1

[CE1-telemetry-subscription-Sub1]sensor-group Sensor1 sample-interval 1000

In this way, the device continuously pushes data to the target.

----End

Compiling the .proto File

You need to generate the gRPC client and server interfaces from the .proto service definition. To do this, you can use the protocol buffer compiler(protoc), and a special gRPC Python plug-in. Ensure that you have installed Protoc and the gRPC Python plug-in. For more information, see https://grpc.io/docs/tutorials/basic/python/.

In this example, compile the .proto file using the run_codegen.py script. Place all .proto files in the /protos directory. This script compiles huawei-grpc-dialout.proto, huawei-telemetry.proto, and huawei-devm.proto at a time. If you want to compile more files, you can add them in the same format.

"""Generates protocol messages and gRPC stubs."""

from grpc_tools import protoc

protoc.main(

(

'',

'-I./protos',

'--python_out=.',

'--grpc_python_out=.',

'./protos/huawei-grpc-dialout.proto', # Specify the file path.

)

)

protoc.main(

(

'',

'-I./protos',

'--python_out=.',

'--grpc_python_out=.',

'./protos/huawei-telemetry.proto',

)

)

protoc.main(

(

'',

'-I./protos',

'--python_out=.',

'--grpc_python_out=.',

'./protos/huawei-devm.proto',

)

)

After the compilation is complete, the following Python file is generated in the run_codegen.py directory:

Complete Code of Python Server

Complete code:

from concurrent import futures

import time

import importlib

import grpc #installed by pip.

import huawei_grpc_dialout_pb2_grpc # Generated by run_codegen.py.

import huawei_telemetry_pb2 # Generated by run_codegen.py generation.

_ONE_DAY_IN_SECONDS = 60 * 60 * 24

def serve():

# Create a grpc server object.

server = grpc.server(futures.ThreadPoolExecutor(max_workers=10))

# Register the telemetry data listening service of Huawei.

huawei_grpc_dialout_pb2_grpc.add_gRPCDataserviceServicer_to_server(

Telemetry_CPU_Info(), server)

# Set the socket listening port.

server.add_insecure_port('192.168.56.1:20000')

# Start the grpc server.

server.start()

# Listen in a dead loop.

try:

while True:

time.sleep(_ONE_DAY_IN_SECONDS)

except KeyboardInterrupt:

server.stop(0)

# Create a class that inherits the servicer method in huawei_grpc_dialout_pb2_grpc.

class Telemetry_CPU_Info(huawei_grpc_dialout_pb2_grpc.gRPCDataserviceServicer):

def __init__(self):

return

def dataPublish(self, request_iterator, context):

for i in request_iterator:

print ('############ start ############\n')

telemetry_data = huawei_telemetry_pb2.Telemetry.FromString(i.data)

print (telemetry_data)

for row_data in  telemetry_data.data_gpb.row:

print ('-----------------')

print ('The proto path is :'+telemetry_data.proto_path)

print ('-----------------')

module_name = telemetry_data.proto_path.split('.')[0]

root_class  = telemetry_data.proto_path.split('.')[1]

# Dynamically load the module for obtaining telemetry data. In this example, the module is:

decode_module = importlib.import_module( module_name+'_pb2')

print (decode_module)

# Define the decoding method getattr to obtain the attribute values in the dynamically-loaded module and invoke the decoding method FromString for this attribute.

decode_func = getattr(decode_module,root_class).FromString

print ('----------- content is -----------\n')

# Decode and output the content in row_data using this method.

print (decode_func(row_data.content))

print ('----------- done -----------------')

if __name__ == '__main__':

serve()

Code execution on the compiler:

The server keeps listening unless interrupted.

Output:

Data is pushed at the specified interval. This example uses one output.

############ start ############

node_id_str: "CE1"

subscription_id_str: "Sub1"

sensor_path: "huawei-devm:devm/cpuInfos/cpuInfo"

collection_id: 260

collection_start_time: 1580278055740

msg_timestamp: 1580278055824

data_gpb {

row {

timestamp: 1580278055740

content: "*\022\n\020\"\0011\010\201\200\204\010(\006\030Z0K\020\010"

}

row {

timestamp: 1580278055740

content: "*\023\n\021\"\00217\010\201\200\304\010(\006\030Z0K\020\010"

}

}

collection_end_time: 1580278055740

current_period: 60000

except_desc: "OK"

product_name: "CE12800"

proto_path: "huawei_devm.Devm"

-----------------

The proto path is :huawei_devm.Devm

-----------------

<module 'huawei_devm_pb2' from 'D:\\10 Python Learning\\Telemetry\\huawei_devm_pb2.py'>

----------- content is -----------

cpuInfos {

cpuInfo {

entIndex: 16842753

interval: 8

ovloadThreshold: 90

position: "1"

systemCpuUsage: 6

unovloadThreshold: 75

}

}

----------- done -----------------

-----------------

The proto path is :huawei_devm.Devm

-----------------

<module 'huawei_devm_pb2' from 'D:\\10 Python Learning\\Telemetry\\huawei_devm_pb2.py'>

----------- content is -----------

cpuInfos {

cpuInfo {

entIndex: 17891329

interval: 8

ovloadThreshold: 90

position: "17"

systemCpuUsage: 6

unovloadThreshold: 75

}

}

----------- done -----------------

----End

Code Explaination

The server code invokes the Python file generated by compiling the .proto file. The run_codegen.py script is simple and is not described here. An important part of writing server code is learning to invoke these generated classes and methods.

Import the modules.

from concurrent import futures

import time

import importlib

import grpc # Install GRPC using pip.

import huawei_grpc_dialout_pb2_grpc # Generate code using run_codegen.py.

import huawei_telemetry_pb2 # Generate code using run_codegen.py generation

Import all modules used by this script. If gRPC has not been installed, make the preparations by referring to the following link:

https://grpc.io/docs/tutorials/basic/python/

concurrent.futures implements multiple processes or threads on the server.

importlib dynamically imports modules.

Define the main function.

if __name__ == '__main__':

serve()

The program runs the main function first. The serve() function is invoked in the main function.

Define the serve() function.

As the main part of the server, the serve() function sets threads and listening IP addresses, and start or stop services.

# Define variables for one day in seconds (60 x 60 x 24 seconds).

_ONE_DAY_IN_SECONDS = 60 * 60 * 24

def serve():

# Create a gRPC server object with a maximum of 10 threads.

server = grpc.server(futures.ThreadPoolExecutor(max_workers=10))

# Register with the telemetry data listening service of Huawei.

huawei_grpc_dialout_pb2_grpc.add_gRPCDataserviceServicer_to_server(

Telemetry_CPU_Info(), server)

# Set the socket listening port.

server.add_insecure_port('192.168.56.1:20000')

# Start the grpc server.

server.start()

# Perform listening in an infinite loop.

try:

while True:

time.sleep(_ONE_DAY_IN_SECONDS)

except KeyboardInterrupt:

server.stop(0)

In the serve() function, futures is used to allow the server to receive a maximum of 10 threads and assign a value to the server.

server = grpc.server(futures.ThreadPoolExecutor(max_workers=10))

Then, invoke the add_gRPCDataserviceServicer_to_server method in huawei_grpc_dialout_pb2_grpc. The input Telemetry_CPU_Info() class will be defined later. (You can open the huawei_grpc_dialout_pb2_grpc.py file to view the specific function.)

huawei_grpc_dialout_pb2_grpc.add_gRPCDataserviceServicer_to_server(

Telemetry_CPU_Info(), server)

server() is used to set the listening IP address and port. In this example, the IP address 192.168.56.1 is used for communication with the switch.

server.add_insecure_port('192.168.56.1:20000')

Start the gRPC service. Run the while function continuously within the time specified by time.sleep().

server.start()

# Perform listening in an infinite loop.

try:

while True:

time.sleep(_ONE_DAY_IN_SECONDS)

except KeyboardInterrupt:

server.stop(0)

Obtain Telemetry data.

The serve() function invokes the Telemetry_CPU_Info() class, which obtains and parses Telemetry data. First, let's see how to obtain Telemetry data.

# Create a class that inherits the servicer method in huawei_grpc_dialout_pb2_grpc.

class Telemetry_CPU_Info(huawei_grpc_dialout_pb2_grpc.gRPCDataserviceServicer):

def __init__(self):

return

def dataPublish(self, request_iterator, context):

for i in request_iterator:

print ('############ start ############\n')

telemetry_data = huawei_telemetry_pb2.Telemetry.FromString(i.data)

print (telemetry_data)

Telemetry_CPU_Info () inherits the gRPCDataserviceServicer method in huawei_grpc_dialout_pb2_grpc. For details, see the huawei_grpc_dialout_pb2_grpc.py file.

class Telemetry_CPU_Info(huawei_grpc_dialout_pb2_grpc.gRPCDataserviceServicer):

The subclass inherits the dataPublish(self, request_iterator, context) function of the parent class.

Telemetry data of the device is continuously pushed. Each push is identified by the #####start##### field.

for i in request_iterator:

print ('############ start ############\n')

The obtained telemetry data is expressed by the telemetry_data variable. Note that the FromString method in the huawei_telemetry_pb2.Telemetry class needs to be used, and only the data attribute in request_iterator needs to be entered.

telemetry_data = huawei_telemetry_pb2.Telemetry.FromString(i.data)

print (telemetry_data)

The output of Telemetry_data is as follows:

############ start ############

node_id_str: "CE1"

subscription_id_str: "Sub1"

sensor_path: "huawei-devm:devm/cpuInfos/cpuInfo"

collection_id: 260

collection_start_time: 1580278055740

msg_timestamp: 1580278055824

data_gpb {

row {

timestamp: 1580278055740

content: "*\022\n\020\"\0011\010\201\200\204\010(\006\030Z0K\020\010"

}

row {

timestamp: 1580278055740

content: "*\023\n\021\"\00217\010\201\200\304\010(\006\030Z0K\020\010"

}

}

collection_end_time: 1580278055740

current_period: 60000

except_desc: "OK"

product_name: "CE12800"

proto_path: "huawei_devm.Devm"

The complete Telemetry data contains parameters such as node_id, subscription ID, and time. The data_gpb contains multiple rows, and each row contains the timestamp and content fields. In this case, the data in the content field cannot be identified and needs to be further decoded.

Decode Telemetry data.

In this example, the CPU information to be obtained is contained in the content field and cannot be identified. Therefore, the content field has to be decoded in the .proto file.

As each data push contains multiple rows, data is read and pushed in the for loop.

for row_data in telemetry_data.data_gpb.row:

print ('-----------------')

print ('The proto path is :'+telemetry_data.proto_path)

print ('-----------------')

Check the value of proto_path for each data record. The command output is as follows:

-----------------

The proto path is :huawei_devm.Devm

-----------------

Different modules are dynamically loaded based on the value of proto_path. Use the split() method to split huawei_devm.Devm into huawei_devm and Devm.

module_name = telemetry_data.proto_path.split('.')[0]

root_class = telemetry_data.proto_path.split('.')[1]

# Dynamically load the module for obtaining telemetry data. In this example, the module is...

decode_module = importlib.import_ module( module_name+'_pb2')

print (decode_module)

Use importlib.import_module to dynamically load the module, and output the content. In this example, the module name is huawei_devm_pb2.

<module 'huawei_devm_pb2' from 'D:\\10 Python Learning\\Telemetry\\huawei_devm_pb2.py'>

# Define the decoding method: getattr to obtain the attribute value in the dynamically-loaded module and invoke the decoding method FromString of this attribute.

decode_func = getattr(decode_module,root_class).FromString

Define the decode_func method to decode huawei_devm data.

print ('----------- content is -----------\n')

# Use this method to decode the content in row_data and output the decoded content.

print (decode_func(row_data.content))

print ('----------- done -----------------')

Finally, input the content to be decoded to this method.

The content that cannot be identified...

data_gpb {

row {

timestamp: 1580278055740

content: "*\022\n\020\"\0011\010\201\200\204\010(\006\030Z0K\020\010"

}

row {

timestamp: 1580278055740

content: "*\023\n\021\"\00217\010\201\200\304\010(\006\030Z0K\020\010"

}

}

Is decoded as follows...

cpuInfos {

cpuInfo {

entIndex: 16842753

interval: 8

ovloadThreshold: 90

position: "1"

systemCpuUsage: 6

unovloadThreshold: 75

}

}

and

cpuInfos {

cpuInfo {

entIndex: 17891329

interval: 8

ovloadThreshold: 90

position: "17"

systemCpuUsage: 6

unovloadThreshold: 75

}

}

The display corresponds to the content in two rows, respectively.

----End

Quiz

Why is decoding required instead of transferring complete data in data_gpb?

In this example, one function obtains and decodes data. Is there any way to optimize the function?

OPS Experiment

Background

A company deploys a CE12800 switch whose management IP address is 192.168.56.100. You will test the Open Programmability System (OPS) function of the device, and query and clear the startup configuration file of the device.

Objectives

Understand the common HTTP operations.

Master Huawei OPS development capabilities.

Environment Preparations

The FTP server and software has to be prepared additionally (no FTP server operation is involved in this guide.)

Simulate an environment using a simulator or prepare a physical environment. In this experiment, the local PC runs the FTP server (IP address: 192.168.56.1), and the switch functions as the FTP client (IP address of CE1: 192.168.56.100), which downloads the Python script from the FTP server.

Configure the CE12800 switch.

<HUAWEI>system-view immediately

Enter system view, return user view with return command.

[HUAWEI]sysname CE1

[CE1]interface Vlanif 1

[CE1-Vlanif1]ip add 192.168.56.100 24

[CE1-Vlanif1]quit

Verify connectivity between the switch and local PC.

Open the CMD window on the local PC and test connectivity between the local PC and CE1.

C:\Users\XXX>ping 192.168.56.100

Pinging 192.168.56.100 with 32 bytes of data:

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 3 ms, TTL = 255

Reply from 192.168.56.100: Bytes = 32, time = 2 ms, TTL= 255

Reply from 192.168.56.100: Bytes = 32, time = 4 ms, TTL = 255

Ping statistics for 192.168.56.100:

Packets: 4 packets transmitted, 4 packets received, 0 packet loss (0.0%)

Estimated RTT (ms):

Minimum = 2 ms, maximum = 4 ms, average = 3 ms

Test FTP connectivity.

<CE1>ftp 192.168.56.1

Trying 192.168.56.1 ...

Press CTRL + K to abort

Connected to 192.168.56.1.

User(192.168.56.1:(none)):

The FTP connectivity test is successful.

Configuration Roadmap

Compile the OPS script demo.py on the PC.

Upload the OPS script to the switch.

Run demo.py on the switch.

Configuration Procedure and Complete Code

Complete Code

#!/usr/bin/env python

# -*- coding: utf-8 -*-

import traceback

import httplib

import string

# Define a class for invoking RESTful API. The class contains some methods to perform operations during HTTP connection establishment. This part can be used directly without any modification.

# Invoke this part without any modification.

class OPSConnection(object):

"""Make an OPS connection instance."""

# Initialize a class and create an HTTP connection.

def __init__(self, host, port=80):

self.host = host

self.port = port

self.headers = {

"Content-type": "text/xml",

"Accept": "text/xml"

}

self.conn = None

# Close the HTTP connection.

def close(self):

"""Close the connection"""

self.conn.close()

# Create operations on device resources.

def create(self, uri, req_data):

"""Create operation"""

ret = self.rest_call("POST", uri, req_data)

return ret

# Delete operations on device resources.

def delete(self, uri, req_data):

"""Delete operation"""

ret = self.rest_call("DELETE", uri, req_data)

return ret

# Get operations on device resources.

def get(self, uri, req_data=None):

"""Get operation"""

ret = self.rest_call("GET", uri, req_data)

return ret

# Modify operations on device resources.

def set(self, uri, req_data):

"""Set operation"""

ret = self.rest_call("PUT", uri, req_data)

return ret

# Define methods invoked in a class.

def rest_call(self, method, uri, req_data):

"""REST call"""

print('|---------------------------------- request: ----------------------------------|')

print('%s %s HTTP/1.1\n' % (method, uri))

if req_data == None:

body = ""

else:

body = req_data

print(body)

if self.conn:

self.conn.close()

self.conn = httplib.HTTPConnection(self.host, self.port)

self.conn.request(method, uri, body, self.headers)

response = self.conn.getresponse()

response.status = httplib.OK  # stub code

ret = (response.status, response.reason, response.read())

print('|---------------------------------- response: ---------------------------------|')

print('HTTP/1.1 %s %s\n\n%s' % ret)

print('|------------------------------------------------------------------------------|')

return ret

def clear_startup_info(ops_conn):

# Specify URI of the system startup information. URI is the managed object defined in RESTful API. Different managed objects have different URIs.

# Modify URI as required. For details about the URIs supported by the device, see RESTful API.

uri = "/cfg/clearStartup"

# Specify the request content to be sent. The content of this part corresponds to URIs and different URIs identify different request content.

# Modify the request content based on used URIs. For details on the format of the request content, see RESTful API.

req_data = \

'''<?xml version="1.0" encoding="UTF-8"?>

<clearStartup>

</clearStartup>

'''

# Execute a POST operation request. uri and req_data indicate a URI request and a content request, respectively. ret indicates whether the request is successful. rsp_data indicates the response data of the system after the request is executed. For details about the response data format, see RESTful API.

# Change request type get() as required, for example, to set() or create().

ret, _, rsp_data = ops_conn.create(uri, req_data)

if ret != httplib.OK:

return None

# Define a function to obtain system startup information.

def get_startup_info(ops_conn):

# Specify URI of the system startup information. The URI is the managed object defined in RESTful API, and different managed objects have different URIs.

# Modify URI as required. For details about the URIs supported by the device, see RESTful API.

uri = "/cfg/startupInfos/startupInfo"

# Specify the request content to be sent. The content of this part corresponds to URIs and different URIs identify different request content.

# Modify the request content based on used URIs. For details on the format of the request content, see RESTful API.

req_data = \

'''<?xml version="1.0" encoding="UTF-8"?>

<startupInfo>

</startupInfo>

'''

# Execute a GET operation request. uri and req_data indicate a URI request and a content request, respectively. ret indicates whether the request is successful. rsp_data indicates the response data of the system after the request is executed. For details about the response data format, see RESTful API.

# Change request type get() as required, for example, to set() or create().

ret, _, rsp_data = ops_conn.get(uri, req_data)

if ret != httplib.OK:

return None

return rsp_data

# The main() function defines the operations to be performed by this script. You can modify this function according to service requirements.

def main():

"""The main function."""

# host indicates the loop address. Currently, RESTful API is invoked inside the device, whose value is localhost.

host = "localhost"

try:

# Establish an HTTP connection.

ops_conn = OPSConnection(host)

# Invoke the function to obtain system startup information.

rsp_data = get_startup_info(ops_conn)

rsp_data = clear_startup_info(ops_conn)

rsp_data = get_startup_info(ops_conn)

# Close the HTTP connection.

ops_conn.close()

return

except:

errinfo = traceback.format_exc()

print(errinfo)

return

if __name__ == "__main__":

main()

Uploading Code

The preceding code is saved as demo.py on the FTP server.

The switch functions as an FTP client to download the file.

<CE1>ftp 192.168.56.1

Trying 192.168.56.1 ...

Press CTRL + K to abort

Connected to 192.168.56.1.

220

User(192.168.56.1:(none)):admin

Enter password:

230

[ftp]get demo.py

Warning: The file may not transfer correctly in ASCII mode.

213 5554

200 PORT           .

150 starting

/     0% [           ]

\     100% [***********]

226 Transfer complete.

The file is downloaded successfully.

Execution Result

Multiple methods are available to configure OPS on a switch. In this example, a Python script is manually installed and run for this purpose.

<CE1>ops install file demo.py

<CE1>ops run python demo.py

|---------------------------------- request: ----------------------------------|

GET /cfg/startupInfos/startupInfo HTTP/1.1

<?xml version="1.0" encoding="UTF-8"?>

<startupInfo>

</startupInfo>

|---------------------------------- response: ---------------------------------|

HTTP/1.1 200 OK

<?xml version="1.0" encoding="UTF-8"?>

<rpc-reply>

<data>

<cfg xmlns="http://www.huawei.com/netconf/vrp" format-version="1.0" content-

version="1.0">

<startupInfos>

<startupInfo>

<position>17</position>

<nextStartupFile>cfcard:/vrpcfg.cfg</nextStartupFile>

<configedSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</con

figedSysSoft>

<curSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</curSysSo

ft>

<nextSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</nextSys

Soft>

<curStartupFile>cfcard:/vrpcfg.cfg</curStartupFile>

<curPatchFile>NULL</curPatchFile>

<nextPatchFile>NULL</nextPatchFile>

<boardInfo>101</boardInfo>

<curPafFile>default</curPafFile>

<nextPafFile>default</nextPafFile>

</startupInfo>

</startupInfos>

</cfg>

</data>

</rpc-reply>

|------------------------------------------------------------------------------|

|---------------------------------- request: ----------------------------------|

POST /cfg/clearStartup HTTP/1.1

<?xml version="1.0" encoding="UTF-8"?>

<clearStartup>

</clearStartup>

|---------------------------------- response: ---------------------------------|

HTTP/1.1 200 OK

<?xml version="1.0" encoding="UTF-8"?>

<rpc-reply>

<ok/>

</rpc-reply>

|------------------------------------------------------------------------------|

|---------------------------------- request: ----------------------------------|

GET /cfg/startupInfos/startupInfo HTTP/1.1

<?xml version="1.0" encoding="UTF-8"?>

<startupInfo>

</startupInfo>

|---------------------------------- response: ---------------------------------|

HTTP/1.1 200 OK

<?xml version="1.0" encoding="UTF-8"?>

<rpc-reply>

<data>

<cfg xmlns="http://www.huawei.com/netconf/vrp" format-version="1.0" content-

version="1.0">

<startupInfos>

<startupInfo>

<position>17</position>

<nextStartupFile>NULL</nextStartupFile>

<configedSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</con

figedSysSoft>

<curSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</curSysSo

ft>

<nextSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</nextSys

Soft>

<curStartupFile>NULL</curStartupFile>

<curPatchFile>NULL</curPatchFile>

<nextPatchFile>NULL</nextPatchFile>

<boardInfo>101</boardInfo>

<curPafFile>default</curPafFile>

<nextPafFile>default</nextPafFile>

</startupInfo>

</startupInfos>

</cfg>

</data>

</rpc-reply>

|------------------------------------------------------------------------------|

Code Explaination

Import the modules.

#!/usr/bin/env python

# -*- coding: utf-8 -*-

import traceback

import httplib

import string

Import the modules required by the OPS function. As the code runs on the switch, no module need to be installed on the local PC

Parse the main function.

Parse the main function according to the code execution sequence.

def main():

"""The main function."""

# host indicates the loop address. Currently, RESTful API is invoked inside the device, whose value is localhost.

host = "localhost"

try:

# Establish an HTTP connection.

ops_conn = OPSConnection(host)

# Invoke the function to obtain system startup information.

rsp_data = get_startup_info(ops_conn)

rsp_data = clear_startup_info(ops_conn)

rsp_data = get_startup_info(ops_conn)

# Close the HTTP connection.

ops_conn.close()

return

except:

errinfo = traceback.format_exc()

print(errinfo)

return

if __name__ == "__main__":

main()

The main function defines the local host. Run this script on the switch to invoke the local RESTful interface.

Run the try... except... command to capture exceptions. First, set up an HTTP connection by invoking the OPSConnection(host) class. Then invoke the get_startup_info(ops_conn) function to obtain the configuration information of the current device, the clear_startup_info(ops_conn) function to clear the next startup configuration of the device, and the get_startup_info(ops_conn) function to obtain the configuration information of the current device again for verification.

Invoke ops_conn.close() to close the HTTP connection.

Parse the OPSConnection() class

# Define a class to invoke RESTful API. The class contains some methods to perform operations during HTTP connection establishment. This part can be used directly without any modification.

# Invoke this part without any modification.

class OPSConnection(object):

"""Make an OPS connection instance."""

# Initiate a class to create an HTTP connection.

def __init__(self, host, port=80):

self.host = host

self.port = port

self.headers = {

"Content-type": "text/xml",

"Accept": "text/xml"

}

self.conn = None

# Close the HTTP connection.

def close(self):

"""Close the connection"""

self.conn.close()

# Create operations on device resources.

def create(self, uri, req_data):

"""Create operation"""

ret = self.rest_call("POST", uri, req_data)

return ret

# Delete operations on device resources.

def delete(self, uri, req_data):

"""Delete operation"""

ret = self.rest_call("DELETE", uri, req_data)

return ret

# Get operations on device resources.

def get(self, uri, req_data=None):

"""Get operation"""

ret = self.rest_call("GET", uri, req_data)

return ret

# Modify operations on device resources.

def set(self, uri, req_data):

"""Set operation"""

ret = self.rest_call("PUT", uri, req_data)

return ret

# Define methods invoked in a class.

def rest_call(self, method, uri, req_data):

"""REST call"""

print('|---------------------------------- request: ----------------------------------|')

print('%s %s HTTP/1.1\n' % (method, uri))

if req_data == None:

body = ""

else:

body = req_data

print(body)

if self.conn:

self.conn.close()

self.conn = httplib.HTTPConnection(self.host, self.port)

self.conn.request(method, uri, body, self.headers)

response = self.conn.getresponse()

response.status = httplib.OK  # stub code

ret = (response.status, response.reason, response.read())

print('|---------------------------------- response: ---------------------------------|')

print('HTTP/1.1 %s %s\n\n%s' % ret)

print('|------------------------------------------------------------------------------|')

return ret

OPSConnection() is the class for the device to locally invoke RESTful APIs. This class defines some methods to perform operations (including GET, POST, DELETE, and PUT) when an HTTP connection is set up. This part can be used directly without any modification.

Parse the get_startup_info() function

This function is used to obtain the current startup information of the device.

def get_startup_info(ops_conn):

# Specify URI of the system startup information. The URI is the managed object defined in the RESTful API, and different managed objects have different URIs.

# Modify URI as required. For details about the URIs supported by the device, see RESTful API.

uri = "/cfg/startupInfos/startupInfo"

The URI varies depending on the operation. For details, see RESTful API. In this example, the content is as follows:

The GET operation is performed this time. Therefore, the URI is set to /cfg/startupInfos/startupInfo.

# Specify the request content to be sent. The content of this part corresponds to URIs and different URIs identify different request content.

# Modify the request content based on used URIs. For details on the format of the request content, see RESTful API.

req_data = \

'''<?xml version="1.0" encoding="UTF-8"?>

<startupInfo>

</startupInfo>

'''

Define req_data as the request to be sent. Fill in the request based on the document requirements.

# Execute a GET operation request. uri and req_data indicate a URI request and a content request, respectively. ret indicates whether the request is successful. rsp_data indicates the response data of the system after the request is executed. For details about the response data format, see RESTful API.

# Change request type get() as required, for example, to set() or create().

ret, _, rsp_data = ops_conn.get(uri, req_data)

if ret != httplib.OK:

return None

return rsp_data

After defining the URI and request, invoke ops_conn.get() to perform the GET operation.

When the main function invokes the get_startup_info() function in step 4 for the first time, the following output information is displayed:

|---------------------------------- request: ----------------------------------|

GET /cfg/startupInfos/startupInfo HTTP/1.1

<?xml version="1.0" encoding="UTF-8"?>

<startupInfo>

</startupInfo>

|---------------------------------- response: ---------------------------------|

HTTP/1.1 200 OK

<?xml version="1.0" encoding="UTF-8"?>

<rpc-reply>

<data>

<cfg xmlns="http://www.huawei.com/netconf/vrp" format-version="1.0" content-

version="1.0">

<startupInfos>

<startupInfo>

<position>17</position>

<nextStartupFile>cfcard:/vrpcfg.cfg</nextStartupFile>

<configedSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</con

figedSysSoft>

<curSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</curSysSo

ft>

<nextSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</nextSys

Soft>

<curStartupFile>cfcard:/vrpcfg.cfg</curStartupFile>

<curPatchFile>NULL</curPatchFile>

<nextPatchFile>NULL</nextPatchFile>

<boardInfo>101</boardInfo>

<curPafFile>default</curPafFile>

<nextPafFile>default</nextPafFile>

</startupInfo>

</startupInfos>

</cfg>

</data>

</rpc-reply>

|------------------------------------------------------------------------------|

The current configuration file and the configuration file for the next startup are cfcard:/vrpcfg.cfg.

Parse the clear_startup_info() function.

This function is used to clear the current configuration file of the device.

def clear_startup_info(ops_conn):

# Specify URI of the system startup information. The URI is the managed object defined in the RESTful API, and different managed objects have different URIs.

# Modify the URI as required. For details about the URIs supported by the device, see RESTful API.

uri = "/cfg/clearStartup"

The operation is to clear the configuration file for the next startup. Therefore, the corresponding URI is /cfg/clearStartup.

# Specify the request content to be sent. The content of this part corresponds to URIs and different URIs identify different request content.

# Modify the request content based on used URIs. For details on the format of the request content, see RESTful API.

req_data = \

'''<?xml version="1.0" encoding="UTF-8"?>

<clearStartup>

</clearStartup>

'''

Continue to check the document to obtain the format of the request for clearing the configuration file.

# Execute a POST operation request. uri and req_data indicate a URI request and a content request, respectively. ret indicates whether the request is successful. rsp_data indicates the response data of the system after the request is executed. For details about the response data format, see RESTful API.

# Change request type get() as required, for example, to set() or create().

ret, _, rsp_data = ops_conn.create(uri, req_data)

if ret != httplib.OK:

return None

According to the document, the HTTP operation for clearing the configuration is POST. Check the method in OPSConnection(), which is ops_conn.create.

After the main function executes the clear_startup_info() function, the following information is displayed:

|---------------------------------- request: ----------------------------------|

POST /cfg/clearStartup HTTP/1.1

<?xml version="1.0" encoding="UTF-8"?>

<clearStartup>

</clearStartup>

|---------------------------------- response: ---------------------------------|

HTTP/1.1 200 OK

<?xml version="1.0" encoding="UTF-8"?>

<rpc-reply>

<ok/>

</rpc-reply>

|------------------------------------------------------------------------------|

The HTTP request header is the POST operation, which clears the configuration. The HTTP response is successful.

Run the get_startup_info() function again to view the modifications.

|---------------------------------- request: ----------------------------------|

GET /cfg/startupInfos/startupInfo HTTP/1.1

<?xml version="1.0" encoding="UTF-8"?>

<startupInfo>

</startupInfo>

|---------------------------------- response: ---------------------------------|

HTTP/1.1 200 OK

<?xml version="1.0" encoding="UTF-8"?>

<rpc-reply>

<data>

<cfg xmlns="http://www.huawei.com/netconf/vrp" format-version="1.0" content-

version="1.0">

<startupInfos>

<startupInfo>

<position>17</position>

<nextStartupFile>NULL</nextStartupFile>

<configedSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</con

figedSysSoft>

<curSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</curSysSo

ft>

<nextSysSoft>cfcard:/VRPV200R005C10SPC607B607D0306_ce12800.cc</nextSys

Soft>

<curStartupFile>NULL</curStartupFile>

<curPatchFile>NULL</curPatchFile>

<nextPatchFile>NULL</nextPatchFile>

<boardInfo>101</boardInfo>

<curPafFile>default</curPafFile>

<nextPafFile>default</nextPafFile>

</startupInfo>

</startupInfos>

</cfg>

</data>

</rpc-reply>

|------------------------------------------------------------------------------|

The current configuration file and the configuration file for the next startup are displayed as NULL, indicating that the modification is successful.

----End

Quiz

Switches provide the Zero Touch Provisioning (ZTP) function. ZTP enables a newly delivered or un-configured device to automatically load version files (including the system software, configuration file, license file, patch file, and user-defined file) during startup after the device is powered on. How to use OPS to implement ZTP?

Network Flow Analysis Experiment

Background

In network management requirements, the forwarding path of specific flow on the network needs to be identified. For this purpose, the network administrator will enable remote mirroring on the interfaces of network devices, compile a Python script, and use the Scapy module to capture and analyze mirrored packets, finally to identify the forwarding path.

Objectives

Configure remote mirroring on network devices.

Master the basic usage of the Scapy module.

Environment Preparations

Use the CE12800 to set up the experiment environment. As shown in the following figure, service packets are sent from CE-A to CE-C. Enable remote mirroring on 10GE 1/0/16 and 10GE 1/0/17 of CE-B and CE-D so that mirrored packets are sent to the flow collector (Python code).

Install the remote mirroring plug-in.

The CE12800 of V200R001C00 and later versions provides remote mirroring as a plug-in, instead of a part of system software. For details on how to install a plug-in, see the CloudEngineXXX Plug-in Operation Guide - Remote Mirroring. After the plug-in is installed, the remote mirroring function is available on the CE12800.

Configure remote mirroring on CE-B.

<CE-B>system-view immediately

Enter system view, return user view with return command.

[CE-B]observe-port 2 destination-ip 10.166.231.109 source-ip 10.154.186.102 erspan-id 2

[CE-B]interface 10GE 1/0/17

[CE-B-10GE1/0/17]port-mirroring observe-port 2 both

[CE-B-10GE1/0/17]interface 10GE 1/0/16

[CE-B-10GE1/0/16]port-mirroring observe-port 2 both

[CE-B-10GE1/0/16]quit

In this example, the IP address of CE-B is 10.154.186.102, and the IP address of the flow collector (Python code) is 10.166.231.109.

Configure remote mirroring on CE-D.

<CE-D>system-view immediately

Enter system view, return user view with return command.

[CE-D]observe-port 3 destination-ip 10.166.231.109 source-ip 10.154.186.104 erspan-id 3

[CE-D]interface 10GE 1/0/17

[CE-D-10GE1/0/17]port-mirror observe-port 3 both

[CE-D-10GE1/0/17]interface 10GE 1/0/16

[CE-D-10GE1/0/16]port-mirror observe-port 3 both

[CE-D-10GE1/0/16]quit

In this example, the IP address of CE-D is 10.154.186.104, and the IP address of the flow collector (Python code) is 10.166.231.109.

Configuration Roadmap

Compile a Python script on the PC, use Scapy to capture packets, and analyze the mirrored packets to identify the flow forwarding path.

On CE-A, test ping and telnet connections to CE-C to simulate service packets.

Run the script on the PC when CE-A is testing ping or telnet connections to CE-C.

Configuration Procedure and Complete Code

Complete Code

# -*- coding: utf-8 -*-

from scapy.all import *

from scapy.contrib.erspan import *

import re, paramiko

# Create a dictionary to save networking information about CE-B and CE-D.

interfaceConnect_CE_B = {

"10GE1/0/17": {"CE-A": "10GE1/0/17"},

"10GE1/0/14": {"CE-C": "10GE1/0/14"},

"10GE1/0/16": {"CE-D": "10GE1/0/16"},

}

interfaceConnect_CE_D = {

"10GE1/0/16": {"CE-B": "10GE1/0/16"},

"10GE1/0/17": {"CE-C": "10GE1/0/17"},

}

# Create the NetDevice class to abstract a device and get its information.

class NetDevice:

def __init__(self, deviceName, deviceIp, username, password, interfaceConnect):

self.deviceName = deviceName

self.deviceIp = deviceIp

self.username = username

self.password = password

self.interfaces = []

self.interfacesConnect = {}

self.macUnderInterface = {}

self.interfaceMac = {}

try:

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname=deviceIp, username=username, password=password)

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

# Disable split-screen display temporarily so that the command output can be displayed on one screen.

cli.send('screen-length 0 temporary\n')

time.sleep(1)

except:

print("An exception occurred during SSH login to device {}.".format(deviceIp))

raise

self.get_interfaces_info(cli)

self.get_mac_under_interface(cli)

self.interfacesConnect = interfaceConnect

ssh.close()

# *******************************************************

# Function: to obtain all interfaces and their MAC addresses of the device and assign values to self.interfaces

# and self.interfaceMac.

# Parameters:

# cli - ssh shell object

# *******************************************************

def get_interfaces_info(self, cli):

cli.send('display interface\n')

time.sleep(15)

interfaces_info = cli.recv(999999).decode()

results = re.findall(r'^(\S+) current state.*\(ifindex', interfaces_info, re.M)

if len(results) > 0:

self.interfaces = results

else:

print("No interface is obtained.")

interface_mac = ["", ""]

lines = interfaces_info.splitlines()

for i in lines:

result1 = re.search(r'^(\S+) current state.*\(ifindex', i)

if result1:

interface_mac[0] = result1.group(1)

result2 = re.search(r'Hardware address is (\S{4}-\S{4}-\S{4})', i)

if result2:

interface_mac[1] = result2.group(1)

self.interfaceMac[interface_mac[0]] = interface_mac[1]

# **************************************************************

# Function: to obtain the MAC address learned from the device interface and assign the value to self.macUnderInterface.

# Parameters:

# cli - ssh shell object

# **************************************************************

def get_mac_under_interface(self, cli):

cli.send('display mac-address\n')

time.sleep(3)

mac_address_info = cli.recv(999999).decode()

cli.send('display arp\n')

time.sleep(3)

arp_info = cli.recv(999999).decode()

mac_interface = re.findall(r'^(\S{4}-\S{4}-\S{4}) \S+ +(\S+) ', mac_address_info, re.M)

if len(mac_interface) > 0:

for i in mac_interface:

if i[1] not in self.macUnderInterface.keys():

self.macUnderInterface[i[1]] = []

self.macUnderInterface[i[1]].append(i[0])

else:

if i[0] not in self.macUnderInterface[i[1]]:

self.macUnderInterface[i[1]].append(i[0])

else:

print("No entry in the MAC address table.")

arp_interface = re.findall(r' (\S{4}-\S{4}-\S{4}) .*D.* +(\S+)', arp_info, re.M)

if len(arp_interface) > 0:

for i in arp_interface:

if i[1] not in self.macUnderInterface.keys():

self.macUnderInterface[i[1]] = []

self.macUnderInterface[i[1]].append(i[0])

else:

if i[0] not in self.macUnderInterface[i[1]]:

self.macUnderInterface[i[1]].append(i[0])

else:

print("No dynamic entries in ARP entries.")

# ****************************************************

# Function: to determine whether packets are sent to the device.

# Parameters:

# packet_info - packet information

# Return value:

#   True/False

# *****************************************************

def is_to_device(self, packet_info):

for key in self.interfaceMac.keys():

# The MAC address may be in the 08e8-4f6e-0516 or 08:e8:4f:6e:05:16 format, which has to be converted for a comparison.

if re.sub(r"[-:]", "", packet_info.get("macDst")) == re.sub(r"[-:]", "", self.interfaceMac.get(key)):

return True

return False

# ****************************************************

# Function: to determine whether a packet is sent by the local device.

# Parameters:

# packet_info - packet information

# Returned value:

#   True/False

# *****************************************************

def is_from_device(self, packet_info):

for key in self.interfaceMac.keys():

# The MAC address may be in the 08e8-4f6e-0516 or 08:e8:4f:6e:05:16 format, which has to be converted for a comparison.

if re.sub(r"[-:]", "", packet_info.get("macSrc")) == re.sub(r"[-:]", "", self.interfaceMac.get(key)):

return True

return False

# ********************************************************

# Function: to obtain the inbound interface of the packet and the interface on the peer device, and return the path section.

# Parameters:

# packet_info - packet information

# Returned value:

#   packet_path

#   [{"CE1": "interface1"},

#    {"CE2": "interface2"}]

# Interface 1 on CE1 sends packets to interface 2 on CE2.

# ********************************************************

def in_device_path(self, packet_info):

# The peer device sends packets along the following path:

packet_path_out = {}

# The local device receives packets along the following path:

packet_path_in = {}

interface = ""

for key in self.macUnderInterface.keys():

macs = self.macUnderInterface.get(key)

for i in range(len(macs)):

macs[i] = re.sub(r"[-:]", "", macs[i])

if re.sub(r"[-:]", "", packet_info.get("macSrc")) in macs:

interface = key

break

if interface == "":

raise Exception('No inbound interface on the device is found for the packets. Packet information: \n{}’.format(packet_info))

if interface not in self.interfacesConnect:

raise Exception('The interface on device {} does not have information about the peer device and interface'.format(interface))

for key, value in self.interfacesConnect.get(interface).items():

packet_path_out[key] = value

packet_path_in[self.deviceName] = interface

return [packet_path_out, packet_path_in]

# ********************************************************

#Function: to obtain the outbound interface of the packet and the interface on the peer device, and return the path section.

# Parameters:

# packet_info - packet information

# Returned value:

#   packet_path

#   [{"CE1": "interface1"},

#    {"CE2": "interface2"}]

# Interface 1 on CE1 sends packets to interface 2 on CE2.

# ********************************************************

def out_device_path(self, packet_info):

# The local device sends packets along the following path:

packet_path_out = {}

# The peer device receives packets along the following path:

packet_path_in = {}

interface = ""

for key in self.macUnderInterface.keys():

macs = self.macUnderInterface.get(key)

for i in range(len(macs)):

macs[i] = re.sub(r"[-:]", "", macs[i])

if re.sub(r"[-:]", "", packet_info.get("macDst")) in macs:

interface = key

break

if interface == "":

raise Exception('No outbound interface on the device is found for the packets. Packet information: \n{}’.format(packet_info))

if interface not in self.interfacesConnect:

raise Exception('The interface on device {} does not have information about the peer device and interface'.format(interface))

for key, value in self.interfacesConnect.get(interface).items():

packet_path_in[key] = value

packet_path_out[self.deviceName] = interface

return [packet_path_out, packet_path_in]

# ****************************************************

#Function: to obtain the mirrored packets related to the flow for path calculation.

# Parameters:

# flow_info: flow information for path calculation

# mirror_packets - all mirrored packets

# Returned value:

#   flow_packets - mirrored packets of the flow for path calculation

# *****************************************************

def get_flow_packets(flow_info, mirror_packets):

flow_packets = []

for packet in mirror_packets:

if packet["ipSrc"] != flow_info["ipSrc"]:

continue

elif packet["ipDst"] != flow_info["ipDst"]:

continue

elif packet["ipProto"] != flow_info["ipProto"]:

continue

if flow_info["ipProto"] == "icmp":

flow_packets.append(packet)

continue

elif flow_info["ipProto"] == "tcp" or flow_info["ipProto"] == "udp":

if flow_info["portSrc"] != "any" and packet["portSrc"] != flow_info["portSrc"]:

continue

elif flow_info["portDst"] != "any" and packet["portDst"] != flow_info["portDst"]:

continue

else:

flow_packets.append(packet)

return flow_packets

# *******************************************************

#Function: to determine whether two path sections are equal.

# Parameters:

# path1 - path section 1

# path2 - path section 2

# Returned value:

#   True/False

# *******************************************************

def is_equal_path(path1, path2):

for i in range(len(path1)):

if path1[i] != path2[i]:

return False

return True

# *******************************************************

# Function: to integrate all path sections to form a complete packet path.

# Parameters:

# path_section: the list of path sections of each remote mirroring device that packets pass through

# Returned value:

# path - packet path

# *******************************************************

def integrate_path(path_section):

# Delete duplicate path sections from path_section.

path_section_norepetition = []

path_section_norepetition.append(path_section[0])

for k in range(len(path_section)):

flag = False

for h in path_section_norepetition:

if is_equal_path(h, path_section[k]):

flag = True

break

if not flag:

path_section_norepetition.append(path_section[k])

path = []

path.append(path_section_norepetition[0][0])

path.append(path_section_norepetition[0][1])

del path_section_norepetition[0]

while len(path_section_norepetition) > 0:

last_device = list(path[len(path)-1].keys())[0]

for i in range(len(path_section_norepetition)):

if last_device in path_section_norepetition[i][0]:

path.append(path_section_norepetition[i][0])

path.append(path_section_norepetition[i][1])

del path_section_norepetition[i]

break

if len(path_section_norepetition) == 0:

break

first_device = list(path[0].keys())[0]

for j in range(len(path_section_norepetition)):

if first_device in path_section_norepetition[j][1]:

path.insert(0, path_section_norepetition[j][1])

path.insert(0, path_section_norepetition[j][0])

del path_section_norepetition[j]

break

return path

# *******************************************************

#Function: to print the packet path.

# Parameters:

# path: the list with complete packet path

# Returned value:

#   None

# *******************************************************

def print_path(path):

path_str = ""

i = 0

while i < len(path):

if i == 0:

for key, value in path[i].items():

path_str += key + "(" + value + ")"

elif i == len(path) - 1:

for key, value in path[i].items():

path_str += " ---> (" + value + ")" + key

else:

for key, value in path[i].items():

path_str += " ---> (" + value + ")" + key

if list(path[i].keys())[0] in path[i+1]:

for key, value in path[i+1].items():

path_str += "(" + value + ")"

i += 1

i += 1

print(path_str)

# ****************************************************

# Function: to calculate and display the packet path.

# Parameters:

# flow_info: information about the flow for path calculation

# mirror_packets - all mirrored packets

# devices: all devices with remote mirroring enabled

# Returned value:

#   None

# *****************************************************

def path_calc(flow_info, mirror_packets, devices):

# Save each path section.

path_section = []

flow_packets = get_flow_packets(flow_info, mirror_packets)

for packet in flow_packets:

for device in devices:

if device.is_to_device(packet):

path = device.in_device_path(packet)

path_section.append(path)

elif device.is_from_device(packet):

path = device.out_device_path(packet)

path_section.append(path)

path = integrate_path(path_section)

print_path(path)

# ****************************************************

#Function: to prompt users to enter the 5-tuple information of the flow for path calculation.

# Parameters:

#   None

# Returned value:

# flow_info: flow 5-tuple information in the following format:

#   flow_info = {

#       "ipSrc": "10.1.12.2",

#       "ipDst": "192.168.2.2",

#       "ipProto": "tcp",

#       "portSrc": 23,

#       "portDst": "any",

#   }

# *****************************************************

def input_flow_info():

flow_info = {} # Save the 5-tuple information of flow for path calculation.

input_str = input("Enter the 5-tuple information (source IP address, destination IP address, protocol, source port number, and destination port number) of the flow for path calculation.")

input_list = input_str.split(",")

flow_info["ipSrc"] = input_list[0].strip()

flow_info["ipDst"] = input_list[1].strip()

flow_info["ipProto"] = input_list[2].strip()

flow_info["portSrc"] = ""

flow_info["portDst"] = ""

if flow_info["ipProto"] == "tcp" or flow_info["ipProto"] == "udp":

portSrc = input_list[3].strip()

if portSrc != "any":

flow_info["portSrc"] = int(portSrc)

else:

flow_info["portSrc"] = "any"

portDst = input_list[4].strip()

if portDst != "any":

flow_info["portDst"] = int(portDst)

else:

flow_info["portDst"] = "any"

return flow_info

# ****************************************************

# Function: to use the scapy module to capture packets and extract information about mirrored service packets.

# Parameters:

#   None

# Returned value:

# packInfoList - list, which saves information about mirrored service packets.

# *****************************************************

def packets_capture():

# Use sniff to obtain the mirrored flow. The iface parameter of sniff needs to be modified to be consistent with the actual packet capture interface.

mirrorPackets = sniff(filter = 'ip proto 47', iface = 'Realtek 8822CE Wireless LAN 802.11ac PCI-E NIC', timeout = 30)

# Define the variable packInfoList to save information about mirrored service packets.

packInfoList = []

grePackets = []

for packet in mirrorPackets:

if packet.haslayer(GRE):

# Check whether the GRE passenger protocol is ERSPAN.

if packet[GRE].proto == 0x88be:

packInfo = {

"macSrc": "",

"macDst": "",

"ipSrc": "",

"ipDst": "",

"ipProto": "",

"portSrc": "",

"portDst": "",

}

erspanPack = packet[GRE][1] # Get the ERSPAN packet in the GRE inner layer.

etherPack = erspanPack[1] # Get the Ethernet packets in the inner layer of ERSPAN packets.

macSrc = etherPack.src

macDst = etherPack.dst

if etherPack.type != 0x0800:

continue

ipPack = etherPack[1] # Get the upper-layer IP packets of Ethernet packets.

ipSrc = ipPack.src

ipDst = ipPack.dst

ipProto = ""

# If the packets are ICMP packets...

if ipPack.proto == 1:

ipProto = "icmp"

elif ipPack.proto == 6:

ipProto = "tcp"

packInfo["portDst"] = ipPack[1].dport

packInfo["portSrc"] = ipPack[1].sport

else:

# Only ICMP and TCP packets are processed, and UDP packets will be processed later.

continue

packInfo["macSrc"] = macSrc

packInfo["macDst"] = macDst

packInfo["ipSrc"] = ipSrc

packInfo["ipDst"] = ipDst

packInfo["ipProto"] = ipProto

if packInfo not in packInfoList:

packInfoList.append(packInfo)

return packInfoList

# Main function

def main():

flow_info = input_flow_info()

packInfoList = packets_capture()

devices = []

CE_B = NetDevice("CE-B", "10.154.186.102", "python", "Huawei12#$", interfaceConnect_CE_B)

CE_D = NetDevice("CE-D", "10.154.186.104", "python", "Huawei@1234", interfaceConnect_CE_D)

devices.append(CE_B)

devices.append(CE_D)

path_calc(flow_info, packInfoList, devices)

if __name__ == "__main__":

main()

Procedure

In this experiment, service packets are simulated using a ping or telnet test. IP address of 10GE 1/0/17 on CE-A: 192.168.2.2; IP address of 10GE 1/0/17 on CE-C: 10.1.14.1

On CE-A, start a ping test to CE-C and run the script on the local PC.

On CE-A, start a telnet test to CE-C and run the script on the local PC.

Execution Result

Flow path for the ping test from CE-A from CE-C

icmp echo request packet path:

C:\Users\Anaconda3\python.exe D:/pyworkspace/scapy_expriment/mirrorAnalyse.py

Enter the 5-tuple information (source IP address, destination IP address, protocol, source port number, and destination port number) of the flow for path calculation: 192.168.2.2, 10.1.14.1, icmp.

CE-A(10GE1/0/17) ---> (10GE1/0/17)CE-B(10GE1/0/16) ---> (10GE1/0/16)CE-D(10GE1/0/17) ---> (10GE1/0/17)CE-C

Process finished with exit code 0

icmp echo relpy packet path:

C:\Users\Anaconda3\python.exe D:/pyworkspace/scapy_expriment/mirrorAnalyse.py

Enter the 5-tuple information (source IP address, destination IP address, protocol, source port number, and destination port number) of the flow for path calculation: 10.1.14.1, 192.168.2.2, icmp.

CE-C(10GE1/0/17) ---> (10GE1/0/17)CE-D(10GE1/0/16) ---> (10GE1/0/16)CE-B(10GE1/0/17) ---> (10GE1/0/17)CE-A

Process finished with exit code 0

Flow path for the telnet test from CE-A to CE-C

Client-to-server packet path:

C:\Users\Anaconda3\python.exe D:/pyworkspace/scapy_expriment/mirrorAnalyse.py

Enter the 5-tuple information (source IP address, destination IP address, protocol, source port number, and destination port number) of the flow for path calculation: 192.168.2.2, 10.1.14.1, tcp, any, 23

CE-A(10GE1/0/17) ---> (10GE1/0/17)CE-B(10GE1/0/16) ---> (10GE1/0/16)CE-D(10GE1/0/17) ---> (10GE1/0/17)CE-C

Process finished with exit code 0

Server-to-client packet path:

C:\Users\Anaconda3\python.exe D:/pyworkspace/scapy_expriment/mirrorAnalyse.py

Enter the 5-tuple information (source IP address, destination IP address, protocol, source port number, and destination port number) of the flow for path calculation: 10.1.14.1, 192.168.2.2, tcp, 23, any

CE-C(10GE1/0/17) ---> (10GE1/0/17)CE-D(10GE1/0/16) ---> (10GE1/0/16)CE-B(10GE1/0/17) ---> (10GE1/0/17)CE-A

Process finished with exit code 0

Code Explaination

The overall logic of the script is as follows:

Import the modules.

from scapy.all import *

from scapy.contrib.erspan import *

import re, paramiko

Import the modules required by the script. In the script, you need to log in to the network device in SSH mode to collect and extract related information. Therefore, the re and paramiko modules need to be imported.

Enter networking information.

# Create a dictionary to save networking information about CE-B and CE-D.

interfaceConnect_CE_B = { # The peer of 10GE1/0/17 on CE-B is...

"10GE1/0/17": {"CE-A": "10GE1/0/17"},  # 10GE1/0/17 on CE-A

"10GE1/0/14": {"CE-C": "10GE1/0/14"},

"10GE1/0/16": {"CE-D": "10GE1/0/16"},

}

interfaceConnect_CE_D = {

"10GE1/0/16": {"CE-B": "10GE1/0/16"},

"10GE1/0/17": {"CE-C": "10GE1/0/17"},

}

The networking information about devices needs to be filled in the script in advance for flow path calculation.

Create a NetDevice class.

The NetDevice class defines six device-related methods: get_interfaces_info, get_mac_under_interface, is_to_device, is_from_device, in_device_path, and out_device_path.

NetDevice class. It abstracts network devices.

# Create the NetDevice class to abstract a device and get its information.

class NetDevice:

def __init__(self, deviceName, deviceIp, username, password, interfaceConnect):

self.deviceName = deviceName

self.deviceIp = deviceIp

self.username = username

self.password = password

self.interfaces = []

self.interfacesConnect = {}

self.macUnderInterface = {}

self.interfaceMac = {}

try:

ssh = paramiko.client.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect(hostname=deviceIp, username=username, password=password)

cli = ssh.invoke_shell()

cli.send('N\n')

time.sleep(0.5)

# Disable split-screen display temporarily so that the command output can be displayed on one screen.

cli.send('screen-length 0 temporary\n')

time.sleep(1)

except:

print("An exception occurred during SSH login to device {}.".format(deviceIp))

raise

self.get_interfaces_info(cli)

self.get_mac_under_interface(cli)

self.interfacesConnect = interfaceConnect

ssh.close()

get_interfaces_info method. This method gets the MAC address of a network device interface.

# *******************************************************

# Function: to obtain all interfaces and their MAC addresses of the device and assign values to self.interfaces

# and self.interfaceMac.

# Parameters:

# cli - ssh shell object

# *******************************************************

def get_interfaces_info(self, cli):

cli.send('display interface\n')

time.sleep(15)

interfaces_info = cli.recv(999999).decode()

results = re.findall(r'^(\S+) current state.*\(ifindex', interfaces_info, re.M)

if len(results) > 0:

self.interfaces = results

else:

print("No interface is obtained.")

interface_mac = ["", ""]

lines = interfaces_info.splitlines()

for i in lines:

result1 = re.search(r'^(\S+) current state.*\(ifindex', i)

if result1:

interface_mac[0] = result1.group(1)

result2 = re.search(r'Hardware address is (\S{4}-\S{4}-\S{4})', i)

if result2:

interface_mac[1] = result2.group(1)

self.interfaceMac[interface_mac[0]] = interface_mac[1]

get_mac_under_interface method. This method gets the MAC address learned from the network device interface.

# **************************************************************

# Function: to obtain the MAC address learned from the device interface and assign a value to self.macUnderInterface.

# Parameters:

# cli - ssh shell object

# **************************************************************

def get_mac_under_interface(self, cli):

cli.send('display mac-address\n')

time.sleep(3)

mac_address_info = cli.recv(999999).decode()

cli.send('display arp\n')

time.sleep(3)

arp_info = cli.recv(999999).decode()

mac_interface = re.findall(r'^(\S{4}-\S{4}-\S{4}) \S+ +(\S+) ', mac_address_info, re.M)

if len(mac_interface) > 0:

for i in mac_interface:

if i[1] not in self.macUnderInterface.keys():

self.macUnderInterface[i[1]] = []

self.macUnderInterface[i[1]].append(i[0])

else:

if i[0] not in self.macUnderInterface[i[1]]:

self.macUnderInterface[i[1]].append(i[0])

else:

print("No entry in the MAC address table.")

arp_interface = re.findall(r' (\S{4}-\S{4}-\S{4}) .*D.* +(\S+)', arp_info, re.M)

if len(arp_interface) > 0:

for i in arp_interface:

if i[1] not in self.macUnderInterface.keys():

self.macUnderInterface[i[1]] = []

self.macUnderInterface[i[1]].append(i[0])

else:

if i[0] not in self.macUnderInterface[i[1]]:

self.macUnderInterface[i[1]].append(i[0])

else:

print("No dynamic entries in ARP entries.")

is_to_device method. This method determines whether the service packet is sent to the local network device according to the information collected in the previous method. If the destination MAC address of the service packet is the MAC address of the local network device, the packet is sent to the local network device.

# ****************************************************

# Function: to determine whether packets are sent to the device.

# Parameters:

# packet_info - packet information

# Returned value:

#   True/False

# *****************************************************

def is_to_device(self, packet_info):

for key in self.interfaceMac.keys():

# The MAC address may be in the 08e8-4f6e-0516 or 08:e8:4f:6e:05:16 format, which has to be converted for a comparison.

if re.sub(r"[-:]", "", packet_info.get("macDst")) == re.sub(r"[-:]", "", self.interfaceMac.get(key)):

return True

return False

is_from_device method. This method determines whether the service packet is sent from the local network device according to the information collected in the previous method. If the source MAC address of the service packet is the MAC address of the local network device, the packet is sent from the local network device.

# ****************************************************

# Function: to determine whether a packet is sent by the local device.

# Parameters:

# packet_info - packet information

# Returned value:

#   True/False

# *****************************************************

def is_from_device(self, packet_info):

for key in self.interfaceMac.keys():

# The MAC address may be in the 08e8-4f6e-0516 or 08:e8:4f:6e:05:16 format, which has to be converted for a comparison.

if re.sub(r"[-:]", "", packet_info.get("macSrc")) == re.sub(r"[-:]", "", self.interfaceMac.get(key)):

return True

return False

in_device_path method. This method calculates the path along which a service packet is sent to the local network device. Specifically, this method checks the interface through which the packet enters the network device by searching the MAC address table and ARP table by the source MAC address. Then, it identifies the peer device and interface according to the networking information. If the local network device is CE2 and service packets are sent from GE 1/0/0 on CE1 to GE 1/0/2 on CE2, then the calculated path is CE1(GE1/0/0) ---> (GE1/0/2)CE2.

# ********************************************************

# Function: to obtain the inbound interface of the packet and the interface on the peer device, and return the path section.

# Parameters:

# packet_info - packet information

# Returned value:

#   packet_path

#   [{"CE1": "interface1"},

#    {"CE2": "interface2"}]

# Interface 1 on CE1 sends packets to interface 2 on CE2.

# ********************************************************

def in_device_path(self, packet_info):

# The peer device sends packets along the following path:

packet_path_out = {}

# The local device receives packets along the following path:

packet_path_in = {}

interface = ""

for key in self.macUnderInterface.keys():

macs = self.macUnderInterface.get(key)

for i in range(len(macs)):

macs[i] = re.sub(r"[-:]", "", macs[i])

if re.sub(r"[-:]", "", packet_info.get("macSrc")) in macs:

interface = key

break

if interface == "":

raise Exception('No inbound interface on the device is found for the packets. Packet information: \n{}.format(packet_info))

if interface not in self.interfacesConnect:

raise Exception('The interface on device {} does not have information about the peer device and interface'. format(interface))

for key, value in self.interfacesConnect.get(interface).items():

packet_path_out[key] = value

packet_path_in[self.deviceName] = interface

return [packet_path_out, packet_path_in]

out_device_path method. This method calculates the path of the service packet sent from the local network device. Specifically, this method checks the interface through which the packet is sent by searching the MAC address table and ARP table by the destination MAC address. Then, it identifies the peer device and interface according to the networking information. If the local network device is CE1 and service packets are sent from GE 1/0/0 on CE1 to GE 1/0/2 on CE2, the calculated path is CE1(GE1/0/0) ---> (GE1/0/2)CE2.

# ********************************************************

#Function: to get the outbound interface of the packet and the interface of the peer device, and return the path section.

# Parameters:

# packet_info - packet information

# Returned value:

#   packet_path

#   [{"CE1": "interface1"},

#    {"CE2": "interface2"}]

# Interface 1 on CE1 sends packets to interface 2 on CE2.

# ********************************************************

def out_device_path(self, packet_info):

# The local device sends packets along the following path:

packet_path_out = {}

# The peer device receives packets along the following path:

packet_path_in = {}

interface = ""

for key in self.macUnderInterface.keys():

macs = self.macUnderInterface.get(key)

for i in range(len(macs)):

macs[i] = re.sub(r"[-:]", "", macs[i])

if re.sub(r"[-:]", "", packet_info.get("macDst")) in macs:

interface = key

break

if interface == "":

raise Exception('No outbound interface on the device is found for the packets. Packet information: \n{}.format(packet_info))

if interface not in self.interfacesConnect:

raise Exception('The interface on device {} does not have information about the peer device and interface'. format(interface))

for key, value in self.interfacesConnect.get(interface).items():

packet_path_in[key] = value

packet_path_out[self.deviceName] = interface

return [packet_path_out, packet_path_in]

Enter 5-tuple information of the flow.

The input_flow_info method prompts the user to enter 5-tuple information of the flow for path calculation, saves the 5-tuple information to the flow_info variable, and returns the value.

# ****************************************************

#Function: to prompt users to enter the 5-tuple information of the flow for path calculation.

# Parameters:

#   None

# Returned value:

# flow_info: flow 5-tuple information in the following format:

#   flow_info = {

#       "ipSrc": "10.1.12.2",

#       "ipDst": "192.168.2.2",

#       "ipProto": "tcp",

#       "portSrc": 23,

#       "portDst": "any",

#   }

# *****************************************************

def input_flow_info():

flow_info = {} # Save the 5-tuple information of flow for path calculation.

input_str = input("Enter the 5-tuple information (source IP address, destination IP address, protocol, source port number, and destination port number) of the flow for path calculation."

input_list = input_str.split(",")

flow_info["ipSrc"] = input_list[0].strip()

flow_info["ipDst"] = input_list[1].strip()

flow_info["ipProto"] = input_list[2].strip()

flow_info["portSrc"] = ""

flow_info["portDst"] = ""

if flow_info["ipProto"] == "tcp" or flow_info["ipProto"] == "udp":

portSrc = input_list[3].strip()

if portSrc != "any":

flow_info["portSrc"] = int(portSrc)

else:

flow_info["portSrc"] = "any"

portDst = input_list[4].strip()

if portDst != "any":

flow_info["portDst"] = int(portDst)

else:

flow_info["portDst"] = "any"

return flow_info

Capture packets using Scapy.

The packets_capture method uses the sniff function of the Scapy module to capture mirrored packets, extracts the source and destination MAC addresses, source and destination IP addresses, protocol, and source and destination port numbers from the encapsulated service packets, and saves them to the packInfoList variable.

For details about how to use Scapy, see the official document at https://scapy.readthedocs.io/en/latest/introduction.html.

# ****************************************************

# Function: to use the scapy module to capture packets and extract information about mirrored service packets.

# Parameters:

#   None

# Returned value:

# packInfoList - list, which saves information about mirrored service packets.

# *****************************************************

def packets_capture():

# Use sniff to obtain mirrored flow. The iface parameter of sniff needs to be modified to be consistent with the actual packet capture interface.

mirrorPackets = sniff(filter = 'ip proto 47', iface = 'Realtek 8822CE Wireless LAN 802.11ac PCI-E NIC', timeout = 30)

# Define the variable packInfoList to save information about mirrored service packets.

packInfoList = []

grePackets = []

for packet in mirrorPackets:

if packet.haslayer(GRE):

# Check whether the GRE passenger protocol is ERSPAN.

if packet[GRE].proto == 0x88be:

packInfo = {

"macSrc": "",

"macDst": "",

"ipSrc": "",

"ipDst": "",

"ipProto": "",

"portSrc": "",

"portDst": "",

}

erspanPack = packet[GRE][1] # Get the ERSPAN packet in the GRE inner layer.

etherPack = erspanPack[1] # Get the Ethernet packets in the inner layer of ERSPAN packets.

macSrc = etherPack.src

macDst = etherPack.dst

if etherPack.type != 0x0800:

continue

ipPack = etherPack[1] # Get the upper-layer IP packets of Ethernet packets.

ipSrc = ipPack.src

ipDst = ipPack.dst

ipProto = ""

# If the packets are ICMP packets...

if ipPack.proto == 1:

ipProto = "icmp"

elif ipPack.proto == 6:

ipProto = "tcp"

packInfo["portDst"] = ipPack[1].dport

packInfo["portSrc"] = ipPack[1].sport

else:

# Only ICMP and TCP packets are processed, and UDP packets will be processed later.

continue

packInfo["macSrc"] = macSrc

packInfo["macDst"] = macDst

packInfo["ipSrc"] = ipSrc

packInfo["ipDst"] = ipDst

packInfo["ipProto"] = ipProto

if packInfo not in packInfoList:

packInfoList.append(packInfo)

return packInfoList

Use the main method.

The main method calculates and outputs the flow path. The code prompts the user to enter the 5-tuple information of the flow for path calculation and then uses the Scapy to capture packets. Instantiate each device with remote mirroring enabled on the network and add all NetDevice class instances to the devices variable. The path_calc method uses the 5-tuple flow information, mirrored service packet information, and devices with remote mirroring enabled as parameters to calculate and output the flow path.

def main():

flow_info = input_flow_info()

packInfoList = packets_capture()

devices = []

CE_B = NetDevice("CE-B", "10.154.186.102", "python", "Huawei12#$", interfaceConnect_CE_B)

CE_D = NetDevice("CE-D", "10.154.186.104", "python", "Huawei@1234", interfaceConnect_CE_D)

devices.append(CE_B)

devices.append(CE_D)

path_calc(flow_info, packInfoList, devices)

if __name__ == "__main__":

main()

----End

Quiz

In this experiment, the source and destination MAC addresses of service packets are compared with the MAC addresses of network devices to determine the network devices and interfaces through which the packets pass. However, the inbound and outbound interfaces may be incorrectly identified by searching the MAC address table and ARP table, because the MAC address table and ARP table of network devices are obtained only after packets are captured while they may change in the course. Is there any simpler and more accurate method of determining the network devices and interfaces that service packets pass through?

- Reference Answers to Quiz

Reference answers to quiz in SSH Experiment:

You can use SSH to log in to multiple devices to view configurations in a cyclic manner.

Reference answers to quiz in Automatic SNMP Configuration Experiment:

User-defined function.

Multiple threads can be used. For details, see the Python Programming Basics.

Reference answers to quiz in NETCONF Configuration Experiment:

            - The if __name__ == '__main__' code block is not executed when it is imported as a module.

The NETCONF query object is YANG, and the SNMP query object is MIB.

Reference answers to quiz in Configuration File Comparison Experiment:

            - N/A

Some code for logging in to the device can be encapsulated as a function, which can be invoked by the functions to write and get configurations.

The time module is introduced to implement the timing function, and NETCONF is used to obtain device configuration files more efficiently.

Reference answers to quiz in gRPC Remote Query Configuration Experiment:

N/A

Reference answers to quiz in Telemetry Configuration Experiment:

            - gRPC compression implements more efficient transmission when the data volume is excessively large.

According to the network application requirements, data can be collected and parsed in a synchronous or asynchronous manner. In synchronous manner, data is parsed while being collected; in asynchronous manner, all data is collected and then parsed.

Reference answers to quiz in OPS Experiment:

In the ZTP process, the device (without a configuration file and without a USB flash drive inserted) obtains a temporary IP address and file server address from the DHCP server, obtains the configuration script (boot.py) from the file server, and runs the script. The script implements functions related to device initialization, such as obtaining the version, viewing the configuration file, and configuring the IP address.

Reference answer to quiz in Network Flow Analysis Experiment:

When configuring Layer 3 remote mirroring on a network device, you can specify ERSPAN ID. The command is as follows: observe-port [observe-port-index] [interface interface-type interface-number]
destination-ip dest-ip-address source-ip source-ip-address [dscp dscp-value]
[erspan-id erspan-id].

The erspan-id parameter is encapsulated in the mirrored packet to differentiate mirrored packets, and its value ranges from 0 to 1023. You can use erspan-id to determine the network devices and interfaces through which the mirrored service packets pass. In this experiment, you can configure remote mirroring on CE-B as follows:

<CE-B>system-view immediately

Enter system view, return user view with return command.

[CE-B]observe-port 1 destination-ip 10.166.231.109 source-ip 10.154.186.102 erspan-id 1

[CE-B]observe-port 2 destination-ip 10.166.231.109 source-ip 10.154.186.102 erspan-id 2

[CE-B]observe-port 3 destination-ip 10.166.231.109 source-ip 10.154.186.102 erspan-id 3

[CE-B]observe-port 4 destination-ip 10.166.231.109 source-ip 10.154.186.102 erspan-id 4

[CE-B]interface 10GE 1/0/17

[CE-B-10GE1/0/17] port-mirroring observe-port 1 inbound

[CE-B-10GE1/0/17] port-mirroring observe-port 2 outbound

[CE-B-10GE1/0/17]interface 10GE 1/0/16

[CE-B-10GE1/0/16] port-mirroring observe-port 3 inbound

[CE-B-10GE1/0/16] port-mirroring observe-port 4 outbound

[CE-B-10GE1/0/16]quit

If the value of erspan-id in the remote mirrored packet is 2, service packets are sent out from 10GE1/0/17 on CE-B in the outbound direction.

For CE-D remote mirroring, the value of erspan-id ranges from 5 to 8. The erspan-id values of the two devices are different, which can be used to determine the network devices that packets pass through.